| NPRR Number | XXX | NPRR Title | RTC+B Modifications to RUC Capacity Short Calculations |
| --- | --- | --- | --- |
| Date Posted |  | TBD |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Sections Requiring Revision |  | 5.7.4.1.1, Capacity Shortfall Ratio Share |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) reflects the RTC+B Task Force whitepaper describing the Reliability Unit Commitment (RUC) Capacity Short modifications that was discussed in meetings from February 2024 to April 2024 as part of Task Force issue #17. This NPRR addresses limitations in the current RUC Capacity Short calculations by considering Ancillary Service sub-types and changes to the calculation process involving Regulation Down Service (Reg-Down). This NPRR also addresses changes required to align Protocol language with approved NPRR1204, Considerations of State of Charge with Real-Time Co-Optimization Implementation, as it relates to the RUC process. This NPRR implements an approach that continues the current policy (cost causation) to first proportionally assign RUC Make Whole costs to Qualified Scheduling Entities (QSEs) that are determined to be short of capacity and/or Ancillary Service capability when the RUC decision was made and if necessary, assign the remaining RUC Make Whole costs to QSEs based on Load Ratio Share (LRS). This NPRR continues to follow the TAC approved RTCTF Key Principle 3 (12). |  |
| Reason for Revision |  | Strategic Plan Objective 1 – Be an industry leader for grid reliability and resilience Strategic Plan Objective 2 - Enhance the ERCOT region’s economic competitiveness with respect to trends in wholesale power rates and retail electricity prices to consumers Strategic Plan Objective 3 - Advance ERCOT, Inc. as an independent leading industry expert and an employer of choice by fostering innovation, investing in our people, and emphasizing the importance of our mission General system and/or process improvement(s) Regulatory requirements ERCOT Board/PUCT Directive (please select ONLY ONE – if more than one apply, please select the ONE that is most relevant) |  |
| Justification of Reason for Revision and Market Impacts |  | After the RTC+B Project was restarted, the NPRRs approved while the project was on hold (e.g., NPRR1093, Load Resource Participation in Non-Spinning Reserve) were analyzed to determine if there were any gaps or impacts to the RTC+B Project that needed to be addressed. In this process the following items were identified for the RUC Capacity Short calculation: - Certain limitations in the approved RTC+B Protocols: - Ancillary Service sub-types are not currently considered. For example, a QSE with an RRS-PFR position can cover it with RRS-UFR or RRS-FFR, which should not be allowed. - The logic for Reg-Down has a deficiency where it is possible for the same Resource capacity to be used for Reg-Down as well as any other Up Ancillary Service types/sub-types (Reg-Up, RRS-FFR, RRS-UFR, RRS-PFR, ECRSS, or Non-Spin). - The logic for an Energy Storage Resource (ESR) providing Ancillary Service when charging. - The need to incorporate ESR SOC considerations based on NPRR 1204 and associated discussions with stakeholders in 2023. |  |

| Sponsor |  |
| --- | --- |
| Name |  |
| E-mail Address |  |
| Company | ERCOT |
| Phone Number |  |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name |  |
| E-Mail Address |  |
| Phone Number |  |

Proposed Protocol Language Revision

5.7.4.1.1	Capacity Shortfall Ratio Share

(1)	In calculating the amount short for each QSE, the Wind-powered Generation Resource Production Potential (WGRPP), as described in Section 4.2.2, Wind-Powered Generation Resource Production Potential, for a Wind-powered Generation Resource (WGR), or the PhotoVoltaic Generation Resource Production Potential (PVGRPP), as described in Section 4.2.3, PhotoVoltaic Generation Resource Production Potential, for a PhotoVoltaic Generation Resource (PVGR), at the time of RUC execution, shall be considered the available capacity of the WGR or PVGR when determining responsibility for the corresponding RUC charges, regardless of the Real-Time output of the WGR or PVGR.  Therefore, the HASLSNAP variable used below shall be equal to the WGRPP and PVGRPP described above.

(2)	In calculating the amount short for each QSE, the QSE must be given a capacity credit for non-Intermittent Renewable Resources (IRRs) that were given notice of decommitment within the two hours before the Operating Hour as a result of the RUC process by setting the HASLSNAP and HASLADJ variables used below equal to the HASLSNAP value for the Resource immediately before the decommitment instruction was given.

(3)	In calculating the short amount for each QSE, if the High Ancillary Service Limit (HASL) for a Resource was credited to the QSE during the RUC snapshot but the Resource experiences a Forced Outage within two hours before the start of the Settlement Interval, then the HASL for that Resource is also credited to the QSE in the HASLADJ.

(4)	In calculating the short amount for each QSE, if the DCIMPSNAP was credited to the QSE during the RUC snapshot but the entire Direct Current Tie (DC Tie) experiences a Forced Outage within two hours before the start of the Settlement Interval, then the DCIMPSNAP is also credited to the QSE in the DCIMPADJ.

(5)	For Combined Cycle Generation Resources, if more than one Combined Cycle Generation Resource is shown On-Line in its COP for the same Settlement hour, then the provisions of paragraph (6)(a) of Section 3.9.1, Current Operating Plan (COP) Criteria, apply in the determination of the On-Line Combined Cycle Generation Resource for that Settlement hour.

(6)	The capacity shortfall ratio share of a specific QSE for a particular RUC process is calculated, for a 15-minute Settlement Interval, as follows:

RUCSFRS ruc, i, q	=	RUCSF ruc, i, q / RUCSFTOT ruc, i

Where:

RUCSFTOT ruc, i	=	RUCSF ruc, i, q

(7)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval is:

RUCSF ruc, i, q	=	Max (0, Max (RUCSFSNAP ruc, q, i, RUCSFADJ ruc, q, i) – RUCCAPCREDIT q, i, z)

(8)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at the snapshot, is:

RUCSFSNAP ruc ,q ,i	=	Max (0, ((RTAML q, p, i * 4) – RUCCAPSNAP ruc, q, i))

(9)	The amount of capacity that a QSE had according to the RUC snapshot for a 15-minute Settlement Interval is:

RUCCAPSNAP ruc, q, i =	HASLSNAP q, r, h + (RUCCPSNAP q, h – RUCCSSNAP q, h) + (DAEP q, p, h –DAES q, p, h) + (RTQQEPSNAP q, p, i – RTQQESSNAP q, p, i) +   DCIMPSNAP q, p, i

(10)	The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at Real-Time, but including capacity from IRRs as seen in the RUC snapshot, is:

RUCSFADJ ruc, q, i	=	Max (0, ((RTAML q, p, i) *4) – (HASLSNAP ruc, q, r, h + RUCCAPADJ q, i))

(11)	The amount of capacity that a QSE had in Real-Time for a 15-minute Settlement Interval, excluding capacity from IRRs, is:

RUCCAPADJ q, i =	HASLADJ q, r, h + (RUCCPADJ q, h – RUCCSADJ q, h) + (DAEP q, p, h – DAES q, p, h) + (RTQQEPADJ q, p, i – RTQQESADJ q, p, i) +  DCIMPADJ q, p, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCSFRS ruc, i, q | none | RUC Shortfall Ratio Share—The ratio of the QSE q’s capacity shortfall to the sum of all QSEs’ capacity shortfalls, for the RUC process ruc, for the 15-minute Settlement Interval i. |
| RUCSF ruc, i, q | MW | RUC Shortfall—The QSE q’s capacity shortfall for the RUC process ruc for the 15-minute Settlement Interval i. |
| RUCSFTOT ruc, i | MW | RUC Shortfall Total—The sum of all QSEs’ capacity shortfalls, for a RUC process ruc, for a 15-minute Settlement Interval i. |
| RUCSFSNAP ruc, q, i | MW | RUC Shortfall at Snapshot—The QSE q’s capacity shortfall according to the snapshot for the RUC process ruc for the 15-minute Settlement Interval i. |
| RUCSFADJ ruc, q, i | MW | RUC Shortfall at Adjustment Period—The QSE q’s Adjustment Period capacity shortfall, including capacity from IRRs as seen in the snapshot for the RUC process ruc, for the 15-minute Settlement Interval i. |
| RUCCAPCREDIT q, i, z | MW | RUC Capacity Credit by QSE—The QSE q’s capacity credit resulting from capacity paid through the RUC Capacity-Short Amount for RUC process z for the 15-minute Settlement Interval i. |
| RTAML q, p, i | MWh | Real-Time Adjusted Metered Load—The QSE q’s Adjusted Metered Load (AML) at the Settlement Point p for the 15-minute Settlement Interval i. |
| RUCCAPSNAP ruc, q, i | MW | RUC Capacity Snapshot at time of RUC—The amount of the QSE q’s calculated capacity in the COP and Trades Snapshot for the RUC process ruc for a 15-minute Settlement Interval i. |
| HASLSNAP q, r, h | MW | High Ancillary Services Limit at Snapshot—The HASL of the Resource r represented by the QSE q, according to the COP and Trades Snapshot for the RUC process for the hour h that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| DCIMPADJ q, p, i | MW | DC Import per QSE per Settlement Point—The approved aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p according to the Adjustment Period snapshot, for the 15-minute Settlement Interval i. |
| DCIMPSNAP q, p, i | MW | DC Import per QSE per Settlement Point—The approved aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p, according to the snapshot for the RUC process for the hour that includes the 15-minute Settlement Interval i. |
| RUCCPSNAP q, h | MW | RUC Capacity Purchase at Snapshot—The QSE q’s capacity purchase, according to the COP and Trades Snapshot for the RUC process for the hour h that includes the 15-minute Settlement Interval. |
| RUCCSSNAP q, h | MW | RUC Capacity Sale at Snapshot—The QSE q’s capacity sale, according to the COP and Trades Snapshot for the RUC process for the hour h that includes the 15-minute Settlement Interval. |
| RUCCAPADJ q, i | MW | RUC Capacity Snapshot during Adjustment Period—The amount of the QSE q’s calculated capacity in the RUC according to the COP and Trades Snapshot, excluding capacity for IRRs, at the end of the Adjustment Period for a 15-minute Settlement Interval i. |
| HASLADJ q, r, h | MW | High Ancillary Services Limit at Adjustment Period—The HASL of a non-IRR r represented by the QSE q, according to the Adjustment Period snapshot, for the hour h that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RUCCPADJ q, h | MW | RUC Capacity Purchase at Adjustment Period—The QSE q’s capacity purchase, according to the Adjustment Period COP and Trades Snapshot for the hour h that includes the 15-minute Settlement Interval. |
| RUCCSADJ q, h | MW | RUC Capacity Sale at Adjustment Period—The QSE q’s capacity sale, according to the Adjustment Period COP and Trades Snapshot for the hour h that includes the 15-minute Settlement Interval. |
| DAEP q, p, h | MW | Day-Ahead Energy Purchase—The QSE q’s energy purchased in the DAM at the Settlement Point p for the hour h that includes the 15-minute Settlement Interval. |
| DAES q, p, h | MW | Day-Ahead Energy Sale—The QSE q’s energy sold in the DAM at the Settlement Point p for the hour h that includes the 15-minute Settlement Interval. |
| RTQQEPSNAP q, p, i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval i, in the COP and Trades Snapshot. |
| RTQQESSNAP q, p, i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval i, in the COP and Trades Snapshot. |
| RTQQEPADJ q, p, i | MW | QSE-to-QSE Energy Purchase by QSE by point—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval i, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| RTQQESADJ q, p, i | MW | QSE-to-QSE Energy Sale by QSE by point—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval i, in the last COP and Trades Snapshot at the end of the Adjustment Period for that Settlement Interval. |
| q | none | A QSE. |
| p | none | A Settlement Point. |
| r | none | A Generation Resource that is QSE-committed or planning to operate as a Quick Start Generation Resource (QSGR) for the Settlement Interval as shown by the Resource Status of OFFQS in the COP and Trades Snapshot and/or Adjustment Period snapshot; or RUC-decommitted for the Settlement Interval (subject to paragraphs (1) and (2) above); or a Switchable Generation Resource (SWGR) released by a non-ERCOT Control Area Operator (CAO) to operate in the ERCOT Control Area due to an ERCOT RUC instruction for an actual or anticipated EEA condition. If the Settlement Interval is a RUCAC-Interval, r represents the Combined Cycle Generation Resource that was QSE-committed at the time the RUCAC was issued. |
| z | none | A previous RUC process for the Operating Day. |
| i | none | A 15-minute Settlement Interval. |
| h | none | The hour that includes the Settlement Interval i. |
| ruc | none | The RUC process for which this RUC Shortfall Ratio Share is calculated. |

[NPRR1009, NPRR1014, NPRR1029, NPRR1032, and NPRR1139: Replace applicable portions of Section 5.7.4.1.1 above with the following upon system implementation of the Real-Time Co-Optimization (RTC) project for NPRR1009; or upon system implementation for NPRR1014, NPRR1029, NPRR1032, or NPRR1139:] 5.7.4.1.1 Capacity Shortfall Ratio Share (1) In calculating the shortfall amount for each QSE, the Resource capacity (RCAPSNAP and RCAPADJ) shall be calculated for a Generation Resource that meets any of the following conditions: (a) QSE-committed; (b) Planning to operate as a Quick Start Generation Resource (QSGR) for the Settlement Interval as shown by the COP Status of OFFQS in the RUC Snapshot for the RUC Process and/or Adjustment Period; or (c) A Switchable Generation Resource (SWGR) that is released by a non-ERCOT Control Area Operator (CAO) to operate in the ERCOT Control Area due to an ERCOT RUC instruction for an actual or anticipated EEA condition and that is shown as On-Line in its COP; or (d) If the Settlement Interval is a RUCAC-Interval, the Combined Cycle Generation Resource that was QSE-committed at the time the RUCAC was issued, excluding the condition for SWGRs as describe in paragraph (c) above. (2) In calculating the amount short for each QSE, the available capacity of an IRR when determining responsibility for the corresponding RUC charges shall be the lessor of the HSL value, as reflected in the COP, and the Wind-powered Generation Resource Production Potential (WGRPP), as described in Section 4.2.2, Wind-Powered Generation Resource Production Potential, for a Wind-powered Generation Resource (WGR), or the PhotoVoltaic Generation Resource Production Potential (PVGRPP), as described in Section 4.2.3, PhotoVoltaic Generation Resource Production Potential, for a PhotoVoltaic Generation Resource (PVGR), at the time of RUC execution. For an IRR, the RCAPSNAP variable used below shall be equal to the minimum of the WGRPP or PVGRPP described above and the HSL value as reflected in the QSE’s COP, at the time of the RUC execution. (3) In calculating the amount short for each QSE, the QSE must be given a capacity credit for non-Intermittent Renewable Resources (IRRs) that were given notice of decommitment within the two hours before the Operating Hour as a result of the RUC process by setting the RCAPSNAP and RCAPADJ variables used below set equal to the RCAPSNAP value for the Resource immediately before the decommitment instruction was given. (4) In calculating the short amount for each QSE, if the RCAPSNAP for a non-IRR was credited to the QSE during the RUC Snapshot but the Resource experiences a Forced Outage within two hours before the start of the Settlement Interval, then the RCAPSNAP for that Resource is also credited to the QSE in the RCAPADJ. (5) In calculating the short amount for each QSE, if the DCIMPSNAP was credited to the QSE during the RUC Snapshot but the entire Direct Current Tie (DC Tie) experiences a Forced Outage within two hours before the start of the Settlement Interval, then the DCIMPSNAP is also credited to the QSE in the RTDCIMP. (6) For Combined Cycle Generation Resources, if more than one Combined Cycle Generation Resource is shown On-Line in its COP for the same Settlement hour, then the provisions of paragraph (6)(a) of Section 3.9.1, Current Operating Plan (COP) Criteria, apply in the determination of the On-Line Combined Cycle Generation Resource for that Settlement hour. (7) The QSE Ancillary Service shortfall calculation in MW for each hour in the RUC Snapshot or for the end of the Adjustment Period involves solving an optimization that minimizes any potential Ancillary Service shortfall for a QSE. This is done by determining the optimal utilization of Ancillary Service capabilities within each QSE’s portfolio of Resources to meet its net Ancillary Service position for each Ancillary Service sub-type. A QSE’s Ancillary Service shortfall for an hour, is the difference between the QSE’s net Ancillary Service position and its coverage of Ancillary Services using the outputs of this optimization, based on the QSE’s Resource Ancillary Service capabilities for that hour, as reflected in the COPs submitted by the QSE. (a) For each Ancillary Service sub-type, the Ancillary Service MW capability for each Resource in the QSE’s portfolio for a given hour in the RUC Snapshot or at the end of the Adjustment Period (ASMWCAPSNAP and ASMWCAPADJ), is calculated as the minimum of: (i) HSL minus LSL in the COP if the Resource is On-line. If a Generation Resource COP Resource Status is OFF or OFFQS, only the COP HSL is used; (ii) Submitted Ancillary Service Offer MW quantity for the Ancillary Service type/sub-type; (iii) Submitted COP Ancillary Service MW capability; and (iv) Qualified Ancillary Service MW amount for the Ancillary Service sub-type. For Resources with COP Resource Status of OFFQS, the qualified MW amounts for Reg-Up, Reg-Down, and RRS will be set to zero. For Resources with a COP Resource Status of OFF, the qualified MW amounts for Reg-Up, Reg-Down, RRS, and ECRS will be set to zero. (b) The QSE Ancillary Service shortfall calculation enforces the following constraints for each hour using data from the RUC Snapshot or the end of the Adjustment Period: (i) Ensure that a QSE’s portfolio of Resource capacities are only used to cover that QSE’s net Ancillary Service position by each Ancillary Service sub-type. (ii) A QSE’s Fast Frequency Response Service (FFRS) position can be covered by the QSE’s portfolio of ESRs qualified to provide FFRS, Load Resources having a high set Under Frequency Relay that are qualified for Responsive Reserve Service (RRS) or Controllable Load Resources (CLRs), Generation Resources, and Energy Storage Resources (ESRs) that are qualified to provide RRS as Primary Frequency Response. (iii) A QSE’s RRS position of the type provided by a Load Resources having a high set Under Frequency Relay that are qualified for RRS can be covered by the QSE’s portfolio of Load Resources qualified to provide this type of RRS or Controllable Load Resources (CLRs), Generation Resources, and ESRs that are qualified to provide RRS as Primary Frequency Response. (iv) A QSE’s ERCOT Contingency Reserve Service (ECRS) position of the type that is not SCED dispatchable can be covered by the QSE’s portfolio of Load Resources that are qualified to provide non-SCED dispatchable ECRS, or by CLRs, Generation Resources, and ESRs that are qualified to provide ECRS of the type that is SCED dispatchable. (v) A QSE’s Non-Spin position of the type that is not SCED dispatchable can be covered by the QSE’s portfolios of Load Resources that are qualified to provide non-SCED dispatchable Non-Spin, or by CLRs, Generation Resources, and ESRs that are qualified to provide Non-Spin of the type that is SCED-dispatchable. (vi) For each Resource and Ancillary Service sub-type: (A) Ancillary Service capacity used for each Ancillary Service sub-type cannot exceed that Resource’s Ancillary Service capability for that Ancillary Service sub-type. (B) The sum of all the Ancillary Service capacities used for each Ancillary Service sub-type cannot exceed the COP HSL minus LSL limits. For Generation Resources that have a Resource Status of OFF and the Ancillary Service type is Non-Spin, consider LSL to be zero. Likewise, for Generation Resources that have a Resource Status of OFFQS and the Ancillary Service type is Non-Spin or ECRS, consider LSL to be zero. (C) For ESRs, consider: (1) Duration requirements for each Ancillary Service type and the submitted COP values for Hour Beginning Planned State of Charge (SOC), Minimum SOC (MinSOC) and Maximum SOC (MaxSOC); (2) Ancillary Service deployment factors, duration requirements for different Ancillary Service types or sub-types, and the difference between the submitted COP Hour Beginning Planned SOC for the hour under consideration and the next hour; and (3) The charge or discharge MW required to satisfy the above constraints. (c) The outputs of the optimization for each Resource are: (i) Resource’s MW capacity used to cover its QSEs net Ancillary Service position by Ancillary Service sub-type for a given hour. These values are ASMWCAPUSEDSNAP for a given hour in the RUC Snapshot and ASMWCAPUSEDADJ for the end of the Adjustment Period. (ii) For an ESR, the MW discharge (positive) or charge (negative) required to support the ESR’s calculated Ancillary Service coverage of its QSEs net Ancillary Service position, considering the submitted COP values for MinSOC, MaxSOC, and the difference in the Hour Beginning Planned SOC for the hour under consideration and the next hour. This value will also account for Ancillary Service deployment factors and the duration requirements for energy and different Ancillary Service types. These values are MWSNAP for a given hour in the RUC Snapshot and MWADJ for the end of the Adjustment Period. (8) The capacity shortfall ratio share of a specific QSE for a particular RUC process is calculated, for a 15-minute Settlement Interval, as follows: RUCSFRS ruc, i, q = RUCSF ruc, i, q / RUCSFTOT ruc, i Where: RUCSFTOT ruc, i = RUCSF ruc, i, q (9) The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval is: RUCSF ruc, i, q = Max (0, Max (RUCSFSNAP ruc, q, i, RUCSFADJ ruc, q, i) – RUCCAPCREDIT q, i, z) (10) The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at the RUC Snapshot, is: RUCSFSNAP ruc ,q ,i = Max (RUCOSFSNAP ruc, q, i , RUCASFSNAP ruc, q, i) (11) The overall shortfall in MW that a QSE had according to the RUC Snapshot for a 15-minute Settlement Interval is: RUCOSFSNAP ruc, q, i = Max (0, ((RTAML q, p, i * 4) + ASONPOSSNAP ruc, q, i – RUCCAPSNAP ruc, q, i)) The QSE’s On-Line Ancillary Service Position according to the RUC Snapshot for a 15-minute Settlement Interval is: ASONPOSSNAP ruc, q, i = RUPOSSNAP ruc, q, h + RRPOSSNAP ruc, q, h + Max (0, (ECRPOSSNAP ruc, q, h + NSPOSSNAP ruc, q, h – ASOFFOFRSNAP ruc, q, r, h)) The amount of capacity that a QSE had according to the RUC Snapshot for a 15-minute Settlement Interval is: RUCCAPSNAP ruc, q, i = RCAPSNAP ruc, q, r, h + (RUCCPSNAP ruc, q, h – RUCCSSNAP ruc, q, h) + (DAEP q, p, h –DAES q, p, h) + (RTQQEPSNAP ruc, q, p, i – RTQQESSNAP ruc, q, p, i) + DCIMPSNAP ruc, q, p, i + ASOFRLRSNAP ruc, q, r, h + ESRMWSNAP ruc, q,h – ESRASSNAP ruc, q,h Where: The QSE’s net up Ancillary Service position (Reg-Up + RRS + ECRS + Non-Spin) covered by the QSE’s portfolio of ESRs is: ESRASSNAP ruc, q, h = ΣASSubTypeASMWCAPUSEDSNAP ruc, q, h, ASSubType, r The sum of the QSE’s ESR discharging (positive) or charging (negative) output is: ESRMWSNAP ruc, q, h = MWSNAP ruc, q, h, r (12) The Ancillary Service shortfall in MW that a QSE had according to the RUC Snapshot for a 15-minute Settlement Interval is: RUCASFSNAP ruc, q, i = RUPOSSNAP ruc, q, h + RDPOSSNAP ruc, q, h + RRPOSSNAP ruc, q, h + ECRPOSSNAP ruc, q, h + NSPOSSNAP ruc, q, h – ΣASSubTypeASMWCAPUSEDSNAP ruc, q, h, ASSubType, r Where: RRPOSSNAP ruc, q, h = PFPOSSNAP ruc, q, h + UFPOSSNAP ruc, q, h + FFPOSSNAP ruc, q, h ECRPOSSNAP ruc, q, h = ECSPOSSNAP ruc, q, h + ECMPOSSNAP ruc, q, h NSPOSSNAP ruc, q, h = NSSPOSSNAP ruc, q, h + NSMPOSSNAP ruc, q, h (13) The RUC Shortfall in MW for one QSE for one 15-minute Settlement Interval, as measured at the end of the Adjustment Period, is: RUCSFADJ ruc, q, i = Max (RUCOSFADJ ruc, q, i, RUCASFADJ q, i ) (14) The overall shortfall in MW that a QSE had at the end of the Adjustment Period for a 15-minute Settlement Interval, but including capacity from IRRs as seen in the RUC Snapshot, is: RUCOSFADJ ruc, q, i = Max (0, ((RTAML q, p, i *4) + ASONPOSADJ q, i – (RCAPSNAP ruc, q, r, h + RUCCAPADJ q, i))) Where: The On-Line Ancillary Service Position the QSE had at the end of the Adjustment Period for a 15-minute Settlement Interval is: ASONPOSADJ q ,i = RUPOSADJ q, h + RRPOSADJ q, h + Max (0, (ECRPOSADJ q, h + NSPOSADJ q,h – ASOFFOFRADJ q, r, h )) The amount of capacity that a QSE had at the end of the Adjustment Period for a 15-minute Settlement Interval, excluding capacity from IRRs, is: RUCCAPADJ q, i = RCAPADJ q, r, h + (RUCCPADJ q, h – RUCCSADJ q, h) + (DAEP q, p, h – DAES q, p, h) + (RTQQEPADJ q, p, i – RTQQESADJ q, p, i) + RTDCIMP q, p + ASOFRLRADJ q, r, h + ESRMWADJ q, h – ESRASADJ q, h Where: The QSE’s net up Ancillary Service position (Reg-Up + RRS + ECRS + Non-Spin) covered by the QSE’s portfolio of ESRs is: ESRASADJ q, h = ΣASSubTypeASMWCAPUSEDADJ q, h, ASSubType, r The sum of the QSE’s ESR discharging (positive) or charging (negative) output is: ESRMWADJ q, h = MWADJ q, h, r (15) The Ancillary Service shortfall in MW that a QSE had at the end of the Adjustment Period for a 15-minute Settlement Interval is: RUCASFADJ q, i = RUPOSADJ q, h + RDPOSADJ q, h + RRPOSADJ q, h + ECRPOSADJ q, h + NSPOSADJ q, h – ΣASSubTypeASMWCAPUSEDADJ q, h, ASSubType, r Where: RRPOSADJ q, h = PFPOSADJ q, h + UFPOSADJ q, h + FFPOSADJ q, h ECRPOSADJ q, h = ECSPOSADJ q, h + ECMPOSADJ q, h NSPOSADJ q, h = NSSPOSADJ q, h + NSMPOSADJ q, h The above variables are defined as follows: Variable; Unit; Definition. RUCSFRS ruc, i, q; none; RUC Shortfall Ratio Share—The ratio of the QSE q’s capacity shortfall to the sum of all QSEs’ capacity shortfalls, for the RUC process ruc, for the 15-minute Settlement Interval i.. RUCSF ruc, i, q; MW; RUC Shortfall—The QSE q’s capacity shortfall for the RUC process ruc for the 15-minute Settlement Interval i.. RUCSFTOT ruc, i; MW; RUC Shortfall Total—The sum of all QSEs’ capacity shortfalls, for a RUC process ruc, for a 15-minute Settlement Interval i.. RUCSFSNAP ruc, q, i; MW; RUC Shortfall at Snapshot—The QSE q’s capacity shortfall will be the maximum of the QSE’s overall shortfall or Ancillary Service shortfall, as calculated for the RUC process ruc for the 15-minute Settlement Interval i.. RUCSFADJ ruc, q, i; MW; RUC Shortfall at End of Adjustment Period—The QSE q’s end of Adjustment Period capacity shortfall will be the maximum of the QSE’s overall shortfall or Ancillary Service shortfall, as calculated for the RUC process ruc, for the 15-minute Settlement Interval i.. RUCCAPCREDIT q, i, z; MW; RUC Capacity Credit—The QSE q’s capacity credit resulting from capacity paid through the RUC Capacity-Short Amount for RUC process z for the 15-minute Settlement Interval i.. RUCOSFSNAP ruc, q, i; MW; RUC Overall Shortfall at Snapshot —The QSE q’s overall capacity shortfall according to the RUC Snapshot for the RUC process ruc for the 15-minute Settlement Interval i.. RUCASFSNAP ruc, q, i; MW; RUC Ancillary Service Shortfall at Snapshot —The QSE q’s Ancillary Service capacity shortfall according to the RUC Snapshot for the RUC process ruc for the 15-minute Settlement Interval i.. ASONPOSSNAP ruc ,q ,i; MW; Ancillary Service On-Line Position at Snapshot – The QSE q’s total On-Line Ancillary Service position according to the RUC Snapshot for the RUC process ruc for the 15-minute Settlement Interval i.. RUPOSSNAP ruc, q, h; MW; Regulation Up Position at Snapshot The QSE q’s Real-Time Reg-Up Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. RRPOSSNAP ruc, q, h; MW; Responsive Reserve Service Position at Snapshot The QSE q’s Real-Time RRS Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. ECRPOSSNAP ruc, q, h; MW; ERCOT Contingency Reserve Service Position at Snapshot The QSE q’s Real-Time ECRS Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. NSPOSSNAP ruc, q, h; MW; Non-Spin Reserve Service Position at Snapshot The QSE q’s Real-Time Non-Spin Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. RDPOSSNAP ruc, q, h; MW; Regulation Down Position at Snapshot The QSE q’s Real-Time Regulation Down Service (Reg-Down) Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. ASOFFOFRSNAP ruc, q, r, h; MW; Ancillary Service Offline Offers at Snapshot –The capacity represented by validated Ancillary Service Offers for ECRS and Non-Spin for Resource r represented by QSE q according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. A Resource’s offered capacity is only included in the sum to the extent that the Resource’s COP Status and Ancillary Service Capability indicate it would be capable of providing the Ancillary Service during the hour h.. ASOFRLRSNAP ruc, q, r, h; MW; Ancillary Service Offer per Load Resource at Snapshot – The capacity represented by validated Ancillary Service Offers for Reg-Up, Non-Spin, RRS, and ECRS for the Load Resource r represented by QSE q according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval. A Resource’s offered capacity is only included in the sum to the extent that the Resource’s COP Status and Ancillary Service Capability indicate it would be capable of providing the Ancillary Service during the hour h.. PFPOSSNAP ruc, q, h; MW; Responsive Reserve (Governor Response or Governor-Like Response) Position at Snapshot The QSE q’s Real-Time RRS-PFR Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. UFPOSSNAP ruc, q, h; MW; Responsive Reserve (Under Frequency trigger at 59.7 Hz.) Position at Snapshot The QSE q’s Real-Time RRS-UFR Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. FFPOSSNAP ruc, q, h; MW; Responsive Reserve (Fast Frequency Response) Position at Snapshot The QSE q’s Real-Time RRS-FFR Ancillary Service Position according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. ECSPOSSNAP ruc, q, h; MW; ERCOT Contingency Reserve Service (SCED Dispatchable) Position at Snapshot The QSE q’s ECRS Ancillary Service Position that is SCED dispatchable according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. ECMPOSSNAP ruc, q, h; MW; ERCOT Contingency Reserve Service (Non-SCED Dispatchable) Position at Snapshot The QSE q’s ECRS Ancillary Service Position that is Non-SCED dispatchable according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. NSSPOSSNAP ruc, q, h; MW; Non-Spin Reserve Service (SCED Dispatchable) Position at Snapshot The QSE q’s Non-Spin Ancillary Service Position that is SCED dispatchable according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. NSMPOSSNAP ruc, q, h; MW; Non-Spin Reserve Service (Non-SCED Dispatchable) Position at SnapshotThe QSE q’s Non-Spin Ancillary Service Position that is Non-SCED dispatchable according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. ASMWCAPSNAP ruc, q, h, ASSubtype, r; MW; MW Capability to provide AStype Ancillary Service at Snapshot – The capacity represented by validated ‘ASSubtype’ Ancillary Service Offers for Resource r represented by QSE q according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. A Resource’s offered capacity is only included in the sum to the extent that the Resource’s COP Status and Ancillary Service Capability indicate it would be capable of providing the ‘ASSubtype’ Ancillary Service during the hour h.. ASMWCAPUSEDSNAP ruc, q, h, ASSubtype, r; MW; Calculated MW Capacity used to cover the QSE’s ‘AStype’ Ancillary Service Position at Snapshot– Calculated MW Capacity of a Resource r represented by QSE q that is used to cover its QSE’s “ASSubtype” Ancillary Service Position for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. MWSNAP ruc, q, h, r; MW; Calculated MW required to support ESR’s calculated Ancillary Service coverage at Snapshot -The MW discharge (positive) or charge (negative) required to support the ESR’s calculated Ancillary Service coverage considering the submitted COP values for Hour Beginning Planned SOC, MinSOC, MaxSOC and the difference in the Hour Beginning Planned SOC for the hour under consideration and the next hour while accounting for Ancillary Service deployment factors and the duration requirements for energy and different Ancillary Service types Position for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval... ESRASSNAP ruc, q, h; MW; Calculated Ancillary Service MW Capacity Provided By QSE’s ESR Portfolio at Snapshot – The total ESR MW capacity used to cover the QSE q’s Upward Ancillary Service position for Reg-Up, RRS, ECRS, and Non-Spin in the RUC Snapshot for the RUC process ruc, for the hour h that includes the 15-minute Settlement Interval.. ESRMWSNAP ruc, q, h; MW; Calculated QSE Total ESR MW Discharging or Charging Required To Support Ancillary Service at Snapshot – The total net ESR MW discharging or charging required to cover the QSE q’s Ancillary Service position provided by the QSE ESR portfolio in the RUC Snapshot for the RUC process ruc, for the hour h that includes the 15-minute Settlement Interval, taking into account the COP SOC values from COP.. RUCOSFADJ ruc, q, i; MW; RUC Overall Shortfall at End of Adjustment Period —The QSE q’s overall capacity shortfall at the end of the Adjustment Period, including capacity from IRRs as seen in the RUC Snapshot for the RUC process ruc, for the 15-minute Settlement Interval i.. RUCASFADJ q, i; MW; RUC Ancillary Service Shortfall at End of Adjustment Period —The QSE q’s Ancillary Service capacity shortfall at the end of the Adjustment Period for the 15-minute Settlement Interval i.. ASONPOSADJ q ,i; MW; Ancillary Service On-Line Position at End of Adjustment Period – The QSE q’s total On-Line Ancillary Service position at the end of the Adjustment Period for the 15-minute Settlement Interval i.. RUPOSADJ q, h; MW; Regulation Up Position at End of Adjustment Period The QSE q’s Reg-Up Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. RRPOSADJ q, h; MW; Responsive Reserve Service Position at End of Adjustment Period The QSE q’s RRS Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. ECRPOSADJ q, h; MW; ERCOT Contingency Reserve Service Position at End of Adjustment Period The QSE q’s ECRS Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. NSPOSADJ q, h; MW; Non-Spin Reserve Service Position at End of Adjustment Period The QSE q’s Non-Spin Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. RDPOSADJ q, h; MW; Regulation Down Position at End of Adjustment Period The QSE q’s Reg-Down Ancillary Service Position at the end of the Adjustment period for the hour h that includes the 15-minute Settlement Interval.. ASOFFOFRADJ q, r, h; MW; Ancillary Service Offline Offers at End of Adjustment Period –The capacity represented by validated Ancillary Service Offers for ECRS and Non-Spin for Resource r represented by QSE q at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. A Resource’s offered capacity is only included in the sum to the extent that the Resource’s COP Status and Ancillary Service Capability indicate it would be capable of providing the Ancillary Service during the hour h.. ASOFRLRADJ q, r, h; MW; Ancillary Service Offer per Load Resource at End of Adjustment Period – The capacity represented by validated Ancillary Service Offers for Reg-Up, Non-Spin, RRS, and ECRS for the Load Resource r represented by QSE q at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval. A Resource’s offered capacity is only included in the sum to the extent that the Resource’s COP Status and Ancillary Service Capability indicate it would be capable of providing the Ancillary Service during the hour h.. PFPOSADJ q, h; MW; Responsive Reserve (Governor Response or Governor-Like Response) Position at End of Adjustment Period The QSE q’s RRS-PFR Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. UFPOSADJ q, h; MW; Responsive Reserve (Under Frequency trigger at 59.7 Hz.) Position at End of Adjustment Period The QSE q’s RRS-UFR Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. FFPOSADJ q, h; MW; Responsive Reserve (Fast Frequency Response) Position at End of Adjustment Period The QSE q’s RRS-FFR Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. ECSPOSADJ q, h; MW; ERCOT Contingency Reserve Service (SCED Dispatchable) Position at End of Adjustment Period The QSE q’s ECRS SCED Dispatchable Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. ECMPOSADJ q, h; MW; ERCOT Contingency Reserve Service (Non-SCED Dispatchable) Position at End of Adjustment Period The QSE q’s ECRS non-SCED Dispatchable Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. NSSPOSADJ q, h; MW; Non-Spin Reserve Service (SCED Dispatchable) Position at End of Adjustment Period The QSE q’s Non-Spin SCED Dispatchable Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. NSMPOSADJ q, h; MW; Non-Spin Reserve Service (Non-SCED Dispatchable) Position at End of Adjustment Period The QSE q’s Non-Spin non-SCED Dispatchable Ancillary Service Position at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. ASMWCAPADJ q, h, ASSubtype, r; MW; MW Capability to provide AStype Ancillary Service at at End of Adjustment Period – The capacity represented by validated ‘ASSubtype’ Ancillary Service Offers for Resource r represented by QSE at the end of Adjustment Period for the hour h that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. A Resource’s offered capacity is only included in the sum to the extent that the Resource’s COP Status and Ancillary Service Capability indicate it would be capable of providing the ‘ASSubtype’ Ancillary Service at the end of the Adjustment Period for the hour h that includes the 15-minute Settlement Interval... ASMWCAPUSEDADJ q, h, ASSubtype, r; MW; Calculated MW Capacity used to cover the QSE’s ‘AStype’ Ancillary Service Position at End of Adjustment Period – Calculated MW Capacity of a Resource r represented by QSE q that is used to cover its QSE’s “ASSubtype” Ancillary Service Position at the end of Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. MWADJ q, h, r; MW; Calculated MW discharge (positive) or charge (negative) required to support ESR’s calculated Ancillary Service coverage at End of Adjustment Period -The MW discharge (positive) or charge (negative) required to support the ESR’s calculated Ancillary Service coverage considering the submitted COP values for Hour Beginning Planned SOC, MinSOC, MaxSOC and the difference in the Hour Beginning Planned SOC for the hour under consideration and the next hour while accounting for Ancillary Service deployment factors and the duration requirements for energy and different Ancillary Service types Position at the end of Adjustment Period for the hour h that includes the 15-minute Settlement Interval... ESRASADJ q, h; MW; Calculated Ancillary Service MW Capacity Provided By QSE’s ESR Portfolio at the End of Adjustment Period – The total ESR MW capacity used to cover the QSE q’s Upward Ancillary Service position for Reg-Up, RRS, ECRS, and Non-Spin at the end of Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. ESRMWADJ q, h; MW; Calculated QSE Total ESR MW Discharging or Charging Required To Support Ancillary Service at End of Adjustment Period – The total net ESR MW discharging or charging required to cover the QSE q’s Ancillary Service position provided by the QSE ESR portfolio at the end of Adjustment Period for the hour h that includes the 15-minute Settlement Interval, taking into account the COP SOC values from COP.. RTAML q, p, i; MWh; Real-Time Adjusted Metered Load—The QSE q’s Adjusted Metered Load (AML) at the Settlement Point p for the 15-minute Settlement Interval i.. RUCCAPSNAP ruc, q, i; MW; RUC Capacity Snapshot at time of RUC—The amount of the QSE q’s calculated capacity in the RUC Snapshot for the RUC process ruc for a 15-minute Settlement Interval i.. RCAPSNAP ruc, q, r, h; MW; Resource Capacity at Snapshot—The available capacity of Generation Resource r represented by the QSE q, according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval. For Generation Resources that are not IRRs, the available capacity shall be equal to HSL. For WGRs and PVGRs, the available capacity shall be equal to the WGRPP and the PVGRPP, respectively. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. DCIMPSNAP ruc, q, p, i; MW; DC Import at Snapshot—The approved aggregated DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p, according to the RUC Snapshot for the RUC process ruc for the 15-minute Settlement Interval i.. RTDCIMP q, p; MW; Real-Time DC Import per QSE per Settlement Point—The aggregated final, approved DC Tie Schedule submitted by QSE q as an importer into the ERCOT System through DC Tie p, for the 15-minute Settlement Interval.. RUCCPSNAP ruc, q, h; MW; RUC Capacity Purchase at Snapshot—The QSE q’s capacity purchase, according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. RUCCSSNAP ruc, q, h; MW; RUC Capacity Sale at Snapshot—The QSE q’s capacity sale, according to the RUC Snapshot for the RUC process ruc for the hour h that includes the 15-minute Settlement Interval.. RUCCAPADJ q, i; MW; RUC Capacity at End of Adjustment Period—The amount of the QSE q’s calculated capacity, excluding capacity for IRRs, at the end of the Adjustment Period for a 15-minute Settlement Interval i.. RCAPADJ q, r, h; MW; Resource Capacity at End of Adjustment Period—The HSL of a non-IRR Generation Resource r represented by the QSE q at the end of the Adjustment Period, for the hour h that includes the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train.. RUCCPADJ q, h; MW; RUC Capacity Purchase at End of Adjustment Period—The QSE q’s capacity purchase, at the end of Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. RUCCSADJ q, h; MW; RUC Capacity Sale at End of Adjustment Period—The QSE q’s capacity sale, at the end of Adjustment Period for the hour h that includes the 15-minute Settlement Interval.. DAEP q, p, h; MW; Day-Ahead Energy Purchase—The QSE q’s energy purchased in the DAM at the Settlement Point p for the hour h that includes the 15-minute Settlement Interval.. DAES q, p, h; MW; Day-Ahead Energy Sale—The QSE q’s energy sold in the DAM at the Settlement Point p for the hour h that includes the 15-minute Settlement Interval.. RTQQEPSNAP ruc, q, p, i; MW; Real-Time QSE-to-QSE Energy Purchase at Snapshot—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval i, in the RUC Snapshot for the RUC process ruc.. RTQQESSNAP ruc, q, p, i; MW; Real-Time QSE-to-QSE Energy Sale at Snapshot—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval i, in the RUC Snapshot for the RUC process ruc.. RTQQEPADJ q, p, i; MW; Real-Time QSE-to-QSE Energy Purchase at End of Adjustment Period—The QSE q’s Energy Trades in which the QSE is the buyer at the delivery Settlement Point p for the 15-minute Settlement Interval i, at the end of the Adjustment Period for that Settlement Interval.. RTQQESADJ q, p, i; MW; Real-Time QSE-to-QSE Energy Sale at End of Adjustment Period—The QSE q’s Energy Trades in which the QSE is the seller at the delivery Settlement Point p for the 15-minute Settlement Interval i, at the end of the Adjustment Period for that Settlement Interval.. q; none; A QSE.. p; none; A Settlement Point.. r; none; A Generation Resource, an ESR, or a Load Resource.. ASSubType; none; Ancillary Service Sub-Type: Reg-Up, Reg-Down, RRS provided as Primary Frequency Response, RRS provided via a high-set under-frequency relay, Fast Frequency Response (FFR), ECRS that is SCED dispatchable, ECRS that is non-SCED dispatchable, Non-Spin that is SCED dispatchable, and Non-Spin that is non-SCED dispatchable.. z; none; A previous RUC process for the Operating Day.. i; none; A 15-minute Settlement Interval.. h; none; The hour that includes the Settlement Interval i.. ruc; none; The RUC process for which this RUC Shortfall Ratio Share is calculated..