| NPRR Number | 1219 | NPRR Title | Methodology Revisions and New Definitions for the CDR |
| --- | --- | --- | --- |
| Date |  | June xx, 2024 |  |
| Submitter’s Information |  |  |  |
| Name |  | Pete Warnken |  |
| E-mail Address |  | Pete.Warnken@ercot.com |  |
| Company |  | ERCOT |  |
| Phone Number |  | 512-248-6705 |  |
| Cell Number |  |  |  |
| Market Segment |  | N/A |  |

Comments

Based on feedback from Supply Analysis Working Group participants, ERCOT recommends changes to proposed new Section 3.2.6.2, Effective Load Carrying Capability (ELCC) Studies, and proposed new language in Section 3.2.6.4, Total Capacity Estimates. The changes (1) remove the word “annual” from average ELCC references to clarify that seasonal ELCC values will be used in the Report on Capacity, Demand and Reserves in the ERCOT Region (CDR), and (2) removes the ESR design duration class definitions. This latter change reflects a new ELCC calculation methodology that will not require the specification of such class definitions.

Revised Cover Page Language

None

Revised Proposed Protocol Language

3.2.6.2 	Effective Load Carrying Capability (ELCC) Studies

(1)	ERCOT shall conduct an Effective Load Carrying Capability (ELCC) study every three years or as necessary based on reviews of expected resource penetration and generation technology trends using Generator Interconnection or Modification (GIM) data.  ERCOT shall provide the appropriate WMS working group with a draft ELCC report and subsequent review and comment period before finalizing the ELCC report.  The ELCC reports shall be posted to the ERCOT website.

(2)	The ELCC study shall be based on the Reliability Standard established by the Public Utility Commission of Texas (PUCT).

(3)	ERCOT shall use a Monte Carlo system simulation tool for determining the ELCC values.

(4)	The ELCC study will determine average ELCCs for aggregate WGRs, PVGRs and ESRs by reserve risk period and applicable CDR resource regions as defined in Section 3.2.6.4, Total Capacity Estimates.

(5)	The ELCC study shall produce a range of ELCC values reflecting feasible future mixes of WGRs, PVGRs, ESRs and Load forecasts for the next five future years.  Each CDR will include the ELCCs associated with the resource mix and load forecast for the given forecast year, season, and CDR resource region (in the case of WGRs and PVGRs).

## 3.2.6.4 	Total Capacity Estimates

(1)	Total capacity estimates will be based on generation availability at the time of the forecasted peak Load hour and peak Net Load hour for each future season and year.

(2)	The total capacity estimates shall be determined based on the following equation:

(2)	The total capacity estimates shall be determined based on the following equation:

TOTCAP h, s , i	=	INSTTHERMCAP s, i + PUNCAP p, s, i + WINDCAP p, s, i, wr + HYDROCAP p, s, i + SOLARCAP p, s, i, sr + ESRCAP p, s, i + RMRCAP s, i + DCTIECAP s + PLANDCTIECAP s + SWITCHCAP s, i + MOTHCAP s, i + PLANTHERMCAP s, i + PLANWINDCAP p, s, i, wr + PLANSOLARCAP p, s, i, sr + PLANESRCAP p, s, i  – LTOUTAGE s, i – UNSWITCH s, i – RETCAPNSO s, i – RETCAPUNC s, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| TOTCAP h, s, i | MW | Total Capacity—Estimated total capacity available during the peak Load hour and peak Net Load hour for Season s for the year i. |
| INSTTHERMCAP s, i | MW | Seasonal Net Max Sustainable Rating for each Thermal Generation Resource—The Seasonal net maximum sustainable rating for Season s as reported in the Resource Integration and Ongoing Operations (RIOO) system for each thermal operating Generation Resource for the Year i excluding Resources operating under RMR Agreements, Mothballed Generation Resources, and Generation Resources capable of “switching” from the ERCOT Region to a non-ERCOT Region. For thermal generation resources classified as small generators in accordance with paragraph (3) of Planning Guide Section 5.2.1, Applicability, capacity is considered operational once a Model Ready Date has been assigned to the resource. |
| PUNCAP h, s, i | MW | Private Use Network Capacity—The forecasted generation capacity available to the ERCOT Transmission Grid, net of self-serve load, from Generation Resources and Settlement Only Generators (SOGs) in Private Use Networks for hour h, Season s and year i. The capacity forecasts are developed as follows. First, a base capacity forecast, determined from SCED data, is calculated as the average net generation capacity available to the ERCOT Transmission Grid during the 20 highest system-wide peak Load and peak Net Load hours for each preceding three-year period for Season s and year i. The base capacity forecast is then adjusted by adding the aggregated incremental forecasted annual changes in net generation capacity as of the start of Season s for forecast Year i reported for Private Use Networks pursuant to Section 10.3.2.4, Reporting of Net Generation Capacity. This calculation is limited to Generation Resources and SOGs in Private Use Networks (1) with a Resource Commissioning Date that occurs no later than the start of the most current Season used for the calculation, and (2) that have not been permanently retired by the start of the most current Season used for the calculation. |
| HYDROCAP p, s, i | MW | Hydro Unit Capacity—The average hydro Generation Resource capacity available, as determined from SCED data during the highest 20 peak Load hours for each preceding three-year period for Reserve Risk Period p, Season s and year i. This calculation is limited to hydro Generation Resources (1) with a Resource Commissioning Date that occurs no later than the start of the most current Peak Load Season used for the calculation, and (2) that have not been permanently retired by the start of the most current Peak Load Season used for the calculation. |
| WINDELCC p, s, i, wr | % | Effective Load Carrying Capability (ELCC) for Wind—The average ELCC for all WGRs for Reserve Risk Period p, Season s, year i, and region wr, expressed as a percentage. |
| WINDCAP p, s, i, wr | MW | Existing WGR Capacity—The amount of currently operational WGRs for Reserve Risk Period p, Season s, year i, and region wr, multiplied by WINDELCC p, s, i, wr. Capacity is considered operational if it has an ERCOT Resource Commissioning Date or ERCOT has approved, or expects to approve, the capacity for grid synchronization by the start of Season s for Year i. For wind resources classified as small generators in accordance with paragraph (3) of Planning Guide Section 5.2.1, capacity is considered operational once a Model Ready Date has been assigned to the resource. |
| SOLARELCC p, s, i, sr | % | Effective Load Carrying Capability (ELCC) for Solar—The average ELCC for Reserve Risk Period p, Season s, Year i, and Region sr, expressed as a percentage. |
| SOLARCAP p, s, i, sr | MW | Available PVGR and Small Generator Capacity—The amount of PVGR capacity that is currently operational for Reserve Risk Period p, Season s, Year i, and Region sr, multiplied by SOLARELCC p, s, i, sr. Capacity is considered operational if it has an ERCOT Resource Commissioning Date or ERCOT has approved, or expects to approve, the capacity for grid synchronization by the start of Season s for Year i. For solar resources classified as small generators in accordance with paragraph (3) of Planning Guide Section 5.2.1, capacity is considered operational once a Model Ready Date has been assigned to the resource. |
| ESRELCC p, s, i | % | Effective Load Carrying Capability (ELCC) for Energy Storage Resources (ESRs)—The average ELCC for Reserve Risk Period p, , Season s, and Year i, expressed as a percentage. |
| ESRCAP p, s, i | % | Available ESR Capacity—The amount of ESR capacity by Reserve Risk Period p, Duration Class d, Season s, and Year i that is currently operational, multiplied by ESRELCC p, s, i. Capacity is considered operational if it has an ERCOT Resource Commissioning Date or ERCOT has approved, or expects to approve, the capacity for grid synchronization by the start of Season s for Year i. For ESRs classified as small generators in accordance with paragraph (3) of Planning Guide Section 5.2.1, capacity is considered operational once a Model Ready Date has been assigned to the resource. |
| RMRCAP s, i | MW | Seasonal Net Max Sustainable Rating for Generation Resource providing RMR Service—The Seasonal net maximum sustainable rating for Season s as reported in the RIOO system for each Generation Resource providing RMR Service for the Year i until the approved exit strategy for the RMR Resource is expected to be completed. |
| DCTIEPEAKPCT s | % | Seasonal Net Import Capacity for existing DC Tie Resources as a Percent of Installed DC Tie Capacity—The average net emergency DC Tie imports for Season s, divided by the total installed DC Tie capacity for Season s, expressed as a percentage. The average net emergency DC Tie imports is calculated for the SCED intervals during which ERCOT declared an Energy Emergency Alert (EEA). This calculation is limited to the most recent Seasons in which an EEA was declared. For the spring and fall seasons ERCOT will use the winter and summer values, respectively, if no EEA events have occurred for these seasons. The total installed DC Tie capacity is the capacity amount at the start of the Seasons used for calculating the net DC Tie imports. |
| DCTIECAP s | MW | Expected Existing DC Tie Capacity Available under Emergency Conditions—DCTIEPEAKPCT s multiplied by the installed DC Tie capacity available for Season s, adjusted for any known capacity transfer limitations. |
| PLANDCTIECAP s | MW | Expected Planned DC Tie Capacity Available under Emergency Conditions—DCTIEPEAKPCT s multiplied by the maximum peak import capacity of planned DC Tie projects included in the most recent Steady State Working Group (SSWG) base cases, for Season s. The import capacity may be adjusted to reflect known capacity transfer limitations indicated by transmission studies. |
| SWITCHCAP s, i | MW | Seasonal Net Max Sustainable Rating for Switchable Generation Resources—The Seasonal net maximum sustainable rating for Season s as reported in the RIOO system for each Generation Resource for Year i that can electrically connect (i.e., “switch”) from the ERCOT Region to another power region. |
| MOTHCAP s, i | MW | Seasonal Net Max Sustainable Rating for Mothballed Generation Resource—The Seasonal net maximum sustainable rating for Season s as reported in the RIOO system for each Mothballed Generation Resource for y Year i based on the lead time and probability information furnished by the owners of Mothballed Generation Resources pursuant to Section 3.14.1.9, Generation Resource Status Updates. If the value furnished by the owner of a Mothballed Generation Resource pursuant to Section 3.14.1.9 is greater than or equal to 75%, then use the Seasonal net maximum sustainable rating for Season s as reported in the RIOO system for the Mothballed Generation Resource for Year i. If the value furnished by the owner of a Mothballed Generation Resource pursuant to Section 3.14.1.9 is less than 75%, then exclude that Resource from the Total Capacity Estimate. |
| PLANTHERMCAP s, i | MW | New Thermal Generating Capacity—The amount of new thermal generating capacity available by the start of Season s and Year i that: (a) has a Texas Commission on Environmental Quality (TCEQ)-approved air permit, (b) has a federal Greenhouse Gas permit, if required, (c) has obtained water rights, contracts or groundwater supplies sufficient for the generation of electricity at the Resource, (d) has a signed Standard Generation Interconnection Agreement (SGIA), or a public, financially-binding agreement between the Resource owner and TSP under which generation interconnection facilities would be constructed; or for a Municipally Owned Utility (MOU) or Electric Cooperative (EC), a public commitment letter to construct a new Resource, (e) a written notice from the TSP that the Interconnecting Entity has provided notice to proceed with the construction of the interconnection, and (f) provided the TSP with sufficient financial security to fund the interconnection facilities. New, Thermal generating capacity is excluded if the Generation Interconnection or Modification (GIM) project status in the RIOO interconnection services system is set to “Cancelled” or “Inactive” or if the Resource was previously mothballed or retired and does not have an owner that intends to operate it. For the purposes of this section, ownership of a mothballed or retired Resource for which a new generation interconnection is sought can only be satisfied by proof of site control as described in paragraph (1)(a), (b), or (d) of Planning Guide Section 5.3.2.1, Proof of Site Control. Thermal resources classified as small generators in accordance with paragraph (3) of Planning Guide Section 5.2.1 must have an ERCOT-assigned Model Ready Date. |
| PLANWINDCAP p, s, i, wr |  | New WGR Capacity—For new WGRs, the capacity available by the start of Season s, Reserve Risk Period p, Year i, and region wr, multiplied by WINDELCC for season s for Reserve Risk Period p, year i, and Region wr. New WGRs must have (1) an SGIA or other public, financially binding agreement between the Resource owner and TSP under which generation interconnection facilities would be constructed or, for a MOU or EC, a public commitment letter to construct a new WGR, (2) a written notice from the TSP that the IE has provided notice to proceed with the construction of the interconnection, and (3) provided the TSP with sufficient financial security to fund the interconnection facilities. Wind resources classified as small generators in accordance with paragraph (3) of Planning Guide Section 5.2.1 must have an ERCOT-assigned Model Ready Date. |
| PLANSOLARCAP p, s, i, sr |  | New PVGR Capacity—For new PVGRs, the capacity available by the start of season s for Risk Period p, Year i, and region sr, multiplied by SOLARELCC p, s, i, sr. New PVGRs must have (1) an SGIA or other public, financially binding agreement between the Resource owner and TSP under which generation interconnection facilities would be constructed or, for a MOU or EC, a public commitment letter to construct a new WGR, (2) a written notice from the TSP that the IE has provided notice to proceed with the construction of the interconnection, and (3) provided the TSP with sufficient financial security to fund the interconnection facilities. Solar resources classified as small generators in accordance with paragraph (3) of Planning Guide Section 5.2.1 must have an ERCOT-assigned Model Ready Date. |
| PLANESRCAP p, s, i | MW | Available Energy Storage Resource Capacity—The amount of ESR capacity that ERCOT has approved, or expects to approve, for grid synchronization by the start of season s for Reserve Risk Period p and Year i, multiplied by ERSELCC p, s, i. |
| LTOUTAGE s, i | MW | Forced Outage Capacity Reported in a Notification of Suspension of Operations—For Generation Resources whose operation has been suspended due to a Forced Outage as reported in a Notification of Suspension of Operations (NSO), the sum of Seasonal net maximum sustainable ratings for Season s and Year i, as reported in the NSO forms. For Inverter Based Resources use WINDCAP, SOLARCAP, and ESRCAP rather than ratings reported in NSOs. |
| UNSWITCH s, i | MW | Capacity of Unavailable Switchable Generation Resource—The amount of capacity reported by the owners of a switchable Generation Resource that will be unavailable to ERCOT during Season s and Year i pursuant to paragraph (2) of Section 16.5.4, Maintaining and Updating Resource Entity Information. |
| RETCAPNSO s, i | MW | Capacity Pending Retirement—The amount of capacity in Season s of Year i that is pending retirement based on information submitted on an NSO form (Section 22, Attachment E, Notification of Suspension of Operations) pursuant to Section 3.14.1.11, Budgeting Eligible Costs, but is under review by ERCOT pursuant to Section 3.14.1.2, ERCOT Evaluation Process, that has not otherwise been considered in any of the above defined categories. For Generation Resources and SOGs within Private Use Networks, the retired capacity amount is deducted from PUNCAP. |
| RETCAPUNC s, i | MW | Unconfirmed Planned Retirements—The capacity of Generation Resources for which a public announcement of the intent to permanently shut the unit down has been released, but a Notice of Suspension of Operations for the unit has not been received by ERCOT. To be considered an Unconfirmed Planned Retirement, the Generation Resource must meet the following criteria: (1) a specific retirement date is cited in the announcement, or other timing information is given that indicates the unit will be unavailable as of the start of Season s for Year i, and (2) the announcement, with follow-up inquiry by ERCOT, does not indicate that retirement timing is highly speculative. |
| p | None | Reserve Risk Period. Morning: For the winter season only, Hour Ending 0600 through 0900. Afternoon: For all seasons, Hour Ending 1500 through 1800. Evening: For all seasons, Hour Ending 1900 through 2200. |
| i | None | Year. |
| s | None | Season. Spring (March through May) Summer (June through September) Fall (October through November) Winter (December through February) |
| sr | None | West, Far West, and Other solar regions. PVGRs are classified into regions based on the county that contains their Point of Interconnection Bus (POIB). The West region is defined as the following counties: Archer, Armstrong, Bailey, Baylor, Borden, Briscoe, Callahan, Carson, Castro, Childress, Clay, Cochran, Coke, Coleman, Collingsworth, Concho, Cottle, Crockett, Crosby, Dallam, Dawson, Deaf Smith, Dickens, Donley, Fisher, Floyd, Foard, Garza, Glasscock, Gray, Hale, Hall, Hansford, Hardeman, Hartley, Haskell, Hockley, Howard, Hutchinson, Irion, Jones, Kent, King, Knox, Lamb, Lipscomb, Lubbock, Lynn, Martin, Menard, Mitchell, Moore, Motley, Nolan, Ochiltree, Oldham, Parmer, Potter, Randall, Reagan, Roberts, Runnels, Schleicher, Scurry, Shackelford, Sherman, Sterling, Stonewall, Sutton, Swisher, Taylor, Terry, Throckmorton, Tom Green, Val Verde, Wheeler, Wichita. The Far West region is defined as the following counties: Andrews, Brewster, Crane, Culberson, Ector, El Paso, Gaines, Hudspeth, Jeff Davis, Loving, Midland, Pecos, Presidio, Reeves, Terrell, Upton, Ward, Winkler, Yoakum. The Other solar region consists of all other counties in the ERCOT Region. |
| wr | None | Coastal, Panhandle, and Other wind regions. WGRs are classified into regions based on the county that contains their Point of Interconnection Bus (POIB). The Coastal region is defined as the following counties: Aransas, Brazoria, Calhoun, Cameron, Kenedy, Kleberg, Matagorda, Nueces, Refugio, San Patricio, and Willacy. The Panhandle region is defined as the following counties: Armstrong, Bailey, Briscoe, Carson, Castro, Childress, Cochran, Collingsworth, Crosby, Dallam, Deaf Smith, Dickens, Donley, Floyd, Gray, Hale, Hall, Hansford, Hartley, Hemphill, Hockley, Hutchinson, Lamb, Lipscomb, Lubbock, Moore, Motley, Ochiltree, Oldham, Parmer, Potter, Randall, Roberts, Sherman, Swisher, and Wheeler. The Other region consists of all other counties in the ERCOT Region. |