| NPRR Number | NA | Title | 2026 Ancillary Services Methodology |
| --- | --- | --- | --- |
| Date |  |  |  |
| Submitter’s Information |  |  |  |
| Name |  | Mark Dreyfus, City of Eastland; Rick Arnett, City of Eastland; Cyrus Reed, Sierra Club; Nick Fehrenbach, City of Dallas; Beth Garza, Residential Consumer Representative; Nabaraj Pokharel, OPUC; Mark Smith, CMC Steel; Garret Kent, CMC Steel; Michael Reid, Air Liquide; Preeti Patel, the Dow Chemical Company; John Russ Hubbard, TIEC |  |
| E-mail Address |  | mark@mdenergyconsulting.com, rarnett@lglawfirm.com, cyrus.reed@sierraclub.org, n.fehrenbach@dallas.gov, bethgarza.work@gmail.com, nabaraj.pokharel@opuc.texas.gov, mark@marksmithlawllc.com, garret.kent@cmc.com, Michael.reid@airliguide.com, ppatel@dow.com, jhubbard@omm.com |  |
| Company |  | City of Eastland, Sierra Club, City of Dallas, Residential Consumer Representative, OPUC, CMC Steel Texas, Nucor Steel Texas, Steel Dynamics, Inc., Gerdau Ameristeel, Air Liquide, the Dow Chemical Company, TIEC |  |
| Phone Number |  |  |  |
| Cell Number |  | (512) 632-5872 (Dreyfus) |  |
| Market Segment |  | Consumer Market Segment |  |

Comments

The ERCOT Consumer Market Segment appreciates the opportunity to comment on the proposed 2026 Ancillary Services Methodology.  Though the revised methodology has been discussed since the April 23rd kickoff, the key issues have only crystallized in the last days with the materials released to the WMS and ROS and the discussions in those subcommittees.

ERCOT Consumers also appreciate the participation and thoughtful comments of the Independent Market Monitor for the ERCOT region, which is directed by Commission rules to evaluate the wholesale market and market rules to recommend measures to enhance market efficiency.

ERCOT Consumers’ fundamental question is whether ERCOT’s posture of “conservative operations” best balances reliability and market efficiency.  Our concern is in alignment with comments that the IMM shared with the WMS and ROS.  Consumers agree that over-procurement of Ancillary Services, despite best intentions to assure adequacy in shortage periods, starves the energy market of resources at just the moment that the market is poised to respond to scarcity conditions.  Perceived under-response in the energy market then appears to justify reliance on non-market action—RUC and continued over-procurement of AS—in a never-ending spiral that inhibits the full capability of the energy market to respond to shortage and operate at peak efficiency.  Market efficiency produces the best outcome for consumers, promotes economic incentives for load response, supports self-commitment and ultimately capital investment in new resources.

While ERCOT Consumers accept that in PUC Project No. 55845 the Commission indicated its preference for a conservative approach to Ancillary Services by minimizing the chance of entering an Operational Watch, it is nevertheless the proper role for the Technical Advisory Committee and its subcommittees and working groups to continue to engage in the never-ending dialogue—already underway for more than 25 years—on creating an efficient energy marketplace while assuring system reliability.  The recommendations that result from that dialogue are key inputs to the weighty policy decisions faced by the ERCOT Board and ultimately help to shape electric policy dialogue at the Commission.

Dialogue about conservative operations is especially appropriate at this time because there may be multiple different perspectives on the appropriate parameters of conservative operations.  The Commission’s preference for a conservative approach to setting AS quantities did not dictate duration or time horizons of inputs to the probabilistic model, for example. Those policy parameters should be subject to a full evaluation and debate among the stakeholders prior to a final recommendation to the Board.

Consumer Recommendations

As noted in the comments of the IMM, the draft AS methodology incorporates the most conservative assumptions/parameters for each of the key inputs to the probabilistic model.  Inputs are the drivers of AS quantities.  Those conservative policy choices are stacked one on another, resulting in the model calling for duplicative AS quantities without a demonstration of additional reliability value.  The policy parameters include the following:  objective of the optimization (avoid “watch” criteria), forecast error time horizon, resource outage time horizon, risk credits, and ESR duration.  ERCOT Consumers’ recommendations on each of these are as follows:

- Objective (“watch” criteria)—Consumers have argued consistently that the watch criteria is largely arbitrary, leading to over-procurement of AS and undermining efficient operation of the energy market.  Given the Commission’s determination in PUC Project No. 55845 supporting a posture of minimizing the chance of entering an Operational Watch, however, consumers are not recommending a change for the 2026 AS Methodology.  Consumers note, however, that the redlined document released prior to the meeting of the Reliability and Operations Subcommittee states that the convergence criteria are set to “avoid an Emergency Condition (Physical Responsive Capability (PRC) = 3,000 MW) …”  This statement is in conflict with the ERCOT Protocols, which do not define an Operational Watch or any condition where PRC = 3,000 MW as an emergency condition (Protocol §6.5.9.3.3(1) “A Watch is the third of three levels of communication issued by ERCOT in anticipation of a possible Emergency Condition”).  Note as well that the Commission findings in PUC Project No. 55845 refer to an Operational Watch as a “pre-emergency condition.”  ERCOT Consumers recommend that at a minimum, any reference to a watch or PRC at 3,000 MW as an emergency condition be stricken from the draft document.

- Forecast Error Time Horizon—This policy parameter is the most significant in determining the final quantities of AS resulting from the revised methodology.  ERCOT selected a six-hour time horizon after reviewing the start up times for units receiving RUC instructions for adequacy.  ERCOT Consumers believe that this comparison ignores the dynamism of the energy market.  As noted in the comments of the IMM, in most circumstances, there are sufficient quantities of generation units that are capable of starting up within an hour in response to scarcity pricing signals.  As well, the choice ignores the ongoing expansion of loads capable of responding to price incentives.  By incorporating a shorter forecast error time horizon—and consequently reserving fewer resources for AS—there will be greater depth in the energy market able to respond to scarcity pricing, thus filling the perceived under-commitment gap used as justification for the over-procurement of AS.  ERCOT Consumer recommendation—shorten the forecast error time horizon to three hours.  In the alternative, ERCOT Consumers recommend differentiating the forecast error time horizon in summer from the more extended horizon in the winter.

- Resource Outage Time Horizon—The horizon for resource outages is set at six hours in the proposed methodology for reasons similar to the forecast error time horizon.  ERCOT Consumer recommendation—shorten the resource outage time horizon to three hours.

- Risk Credits—In the probabilistic model, the risk credit policy parameter offsets requirement for additional AS.  The recommended parameters are 60 percent at night and 25 percent during daytime hours.  While it may be understandable that the quantity of available headroom and offline capacity differs by time of day or by season, no analytic basis has been shared for assuming that only 25 percent of headroom will be available when called on.  Particularly in the RTC optimization where a single resource may provide both energy and AS, consumers question the assumption that only 25 percent of those resources will be available. ERCOT Consumer recommendation—raise the Risk Credit value to 50 percent.

- ESR Duration—This issue was discussed at length earlier this year in NPRR1282.  In that dialogue and in the continuing dialogue in consideration of the 2026 AS Methodology, ERCOT Consumers are aligned with the comments of the IMM.  ERCOT Consumers note, however, the commitment of ERCOT staff to the Protocol Revision Subcommittee (May 14, 2025) and Technical Advisory Committee (May 28, 2025) that it will reassess ESR Duration, first, at the conclusion of the RTC + B market trials, and again in the development of the 2027 AS Methodology, once there is experience operating in an RTC+B environment.  ERCOT Consumer recommendation—no change to duration at this time pending the reassessment previously agreed to in the stakeholder process.

Recommendation for the 2027 AS Methodology

ERCOT Consumers appreciate ERCOT’s prior commitment to reevaluate ESR duration in the 2027 AS Methodology to incorporate data and lessons learned from the operation of RTC+B.  ERCOT Consumers recommend that the 2027 AS Methodology provides an appropriate opportunity to review all inputs to the AS Methodology in light of data and lessons learned from RTC+B.

Conclusion

It is a fundamental objective of the ERCOT market structure to reliability deliver power at lowest cost.  Where competitive market solutions are possible, that objective will best be achieved.  ERCOT Consumers support competitive solutions where possible, which will promote both reliability and economically efficient markets, to the benefit of all consumers.  Over-procurement of Ancillary Services, no matter how well intentioned, restricts the energy market, preventing the most economically efficient outcome.  In this chapter of the never-ending dialogue on achieving reliability through competitive market solutions, ERCOT Consumers urge stakeholders to recommend that the ERCOT Board of Directors rebalance scales between the AS and energy markets as recommended in these comments.

See page 8 of the redline to ERCOT Methodologies for Determining Minimum AS Requirements 2026 DRAFT found under Key Documents to the August 7, 2025 meeting of the Reliability and Operations Subcommittee at the linked zip file.

PUC Project No. 55845.  Review of Ancillary Services in the ERCOT Market Commission Findings and Final Report. January 14, 2025 at 4.