March 2026 ERCOT Monthly Operations Report

Reliability and Operations Subcommittee Meeting

May 7, 2026

Table of Contents

1.	Report Highlights	2

2.	Frequency Control	3

2.1.	Frequency Events	3

2.2.	Automatic/Manual Load Resource Deployments	4

3.	Reliability Unit Commitment	5

4.	IRR, Wind, and Solar Generation as a Percent of Load	6

5.	Largest Net-Load Ramps	8

6.	Congestion Analysis	9

6.1.	Notable Constraints	9

6.2.	Generic Transmission Constraint Congestion	22

6.3.	Manual Overrides	22

6.4.	Congestion Costs for Calendar Year 2025	23

7.	System Events	24

7.1.	ERCOT Peak Load	24

7.2.	Load Shed Events	24

7.3.	Stability Events	25

7.4.	Notable PMU Events	25

7.5.	DC Tie Curtailment	25

7.6.	TRE/DOE Reportable Events	25

7.7.	New/Updated Constraint Management Plans	25

7.8.	New/Modified/Removed RAS	25

7.9.	New Procedures/Forms/Operating Bulletins	25

8.	Emergency Conditions	25

8.1.	OCNs	25

8.2.	Advisories	26

8.3.	Watches	26

8.4.	Emergency Notices	26

9.	Application Performance	26

9.1.	TSAT/VSAT Performance Issues	26

9.2.	Communication Issues	26

9.3.	Market System Issues	26

10.	Model Updates	26

Appendix A: Real-Time Constraints	29

# Report Highlights

The unofficial ERCOT peak load for March 2026 was 66,515 MW and occurred on 03/25/2026 during hour ending 18:00, this is 3,591 MW higher than the March 2025 demand of 62,924 MW on 03/25/2025 during hour ending 18:00.

There were 2 frequency events.

There were no events which required automatic/manual deployment of Load Resources that were controlled by an Under-Frequency Relays.

There were 2 OCNs.

There were 2 Advisories.

There was 1 Watch.

There were 0 Emergency Notices.

There were 39 RUC commitments.

A PVGR Generation Record of 33,477 MW was set on 03/21/2026 at 11:20.

A PVGR Penetration Record of 67.27% was set on 03/21/2026 at 10:52.

A Wind Generation Record of 28,751 MW was set on 03/14/2026 at 23:00.

A Renewable Penetration Record of 86.77% was set on 03/18/2026 at 10:30.

The following GTCs saw congestion in March:

| GTC | Days Congestion |
| --- | --- |
| Nelson Sharpe - Rio Hondo | 30 |
| North Edinburg – Lobo | 29 |
| West Texas | 26 |
| Uvalde | 24 |
| South Texas Export Pawnee – Spruce | 19 |
| East Texas | 16 |
| South Texas Export Pawnee – Tango | 15 |
| Panhandle | 14 |
| SAM Switch | 11 |
| Valley Export | 6 |
| Wharton | 6 |
| Kinney | 6 |
| McCamey | 5 |
| North – Houston | 5 |
| South Texas Import Katoen - Lonhill | 3 |
| South Texas Import Pawnee – Spruce | 2 |
| Zapata Starr | 2 |

# Frequency Control

## Frequency Events

The ERCOT Interconnection experienced 2 frequency events.

A summary of the frequency events is provided below. The reported frequency events meet one of the following criteria: Delta Frequency is 60 mHz or greater; the MW loss is 350 MW or greater; resource trip event triggered ECRS deployment. Frequency events that have been identified as Frequency Measurable Events (FME) for purposes of BAL-001-TRE-2 analysis are highlighted in blue. When analyzing frequency events, ERCOT evaluates PMU data according to industry standards. Events with an oscillating frequency of less than 1 Hz are inter-area, while higher frequencies indicate local events. Industry standards specify that damping ratio for inter-area oscillations should be 3.0% or greater. For the frequency event listed below, the ERCOT system met these standards and transitioned well after the disturbance. In the case of negative delta frequency, the MW Loss column could refer to load loss.

| Date and Time | Delta Frequency | Max/Min Frequency | Duration of Event | PMU Data |  | MW Loss | Load | IRR | Inertia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (Hz) | (Hz) |  | Oscillation Mode (Hz) | Damping Ratio |  | (MW) | % | (MW-s) |
| 3/11/2026 8:20:47 | 0.047 | 59.950 | 00:03:15 | 0.54 | 8% | 612 | 49,478 | 35% | 216,865 |
| 3/20/2026 0:29:11 | 0.038 | 59.978 | 00:02:43 | 0.43 | 12% | 467 | 45,595 | 53% | 193,674 |

(Note: All data on this graph encompasses frequency event analysis based on BAL-001-TRE-2.)

## Automatic/Manual Load Resource Deployments

| Resource Location | # of Resources | Operating Day | Total # of Hours Committed | Total MWhs | Reason for Commitment | MWs of ECRS Deployed | MWs of RRS Deployed |
| --- | --- | --- | --- | --- | --- | --- | --- |

There were no events which required automatic/manual deployment of Load Resources that were controlled by an Under-Frequency Relays.

# Reliability Unit Commitment

ERCOT reports on Reliability Unit Commitments (RUC) monthly. Commitments are grouped by operating day and weather zone. The total number of hours committed is the sum of the hours for all the units in the specified region. Additional information on RUC commitments can be found on the MIS secure site at Grid  Generation  Reliability Unit Commitment.

There were 39 RUC commitments.

| Resource Location | # of Resources | Operating Day | Total # of Hours Committed | Total MWhs | Reason for Commitment |
| --- | --- | --- | --- | --- | --- |
| COAST | 1 | March 2, 2026 | 2 | 626.6 | Capacity |
| COAST | 2 | March 3, 2026 | 5 | 1,155.3 | Capacity |
| COAST | 1 | March 4, 2026 | 2 | 672.2 | Capacity |
| NORTH_CENTRAL | 1 | March 13, 2026 | 5 | 1,675.8 | Capacity, Min Run Time |
| COAST, EAST, NORTH_CENTRAL, SOUTH_CENTRAL | 13 | March 16, 2026 | 149 | 32,869.0 | Capacity, E_PASP, SSGVTRC5 |
| COAST, EAST, NORTH, NORTH_CENTRAL | 10 | March 23, 2026 | 35 | 4,070.2 | Capacity |
| COAST, EAST, NORTH_CENTRAL | 8 | March 24, 2026 | 115 | 29,439.0 | Capacity, THWZEN98 |
| NORTH_CENTRAL, SOUTH_CENTRAL | 2 | March 25, 2026 | 34 | 8,365.0 | Capacity, DSALKLN5 |
| NORTH_CENTRAL | 1 | March 26, 2026 | 18 | 2,124.0 | Capacity |

# IRR, Wind, and Solar Generation as a Percent of Load

The graph below shows the maximum, minimum and average aggregate solar, wind and IRR output as a percentage of total ERCOT load when evaluated as 10-minute averaged intervals, over the past 13 months. Current wind and solar generation and penetration records are listed in the footnote below. Maximum IRR penetration for March 2026 was 86.77% on 3/18/2026 interval ending 10:30 and minimum IRR penetration for March 2026 was 8.57% on 3/23/2026 interval ending 20:00.

During the hour of peak load for the month, hourly integrated wind generation was 19,541 MW and solar generation was 24,519 MW. The graph below shows the wind and solar penetration percentage during the hour of the peak load in the last 13 months.

Lastly, the graph below shows the minimum wind, solar, and IRR output during the peak load hour as a percentage of the daily peak load for every day in the month.

# Largest Net-Load Ramps

The net-load ramp is defined as the change in net-load (load minus wind and PVGR generation) during the defined time horizon. Such a variation in net-load needs to be accommodated in grid operations to ensure that the reliability of the grid is satisfactorily maintained. The largest net-load ramps over 5-minute, 10-minute, 15-minute, 30-minute, and 60-minute intervals in March 2026 were 2,119 MW, 3,407 MW, 4,875 MW, 9,515 MW, and 17,942 MW respectively. A comparison with historical values is provided in the table below.

| Month and Year | 5 min | 10 min | 15 min | 30 min | 60 min |
| --- | --- | --- | --- | --- | --- |
| 16-Mar | 979 MW | 1,635 MW | 2,149 MW | 2,967 MW | 5,070 MW |
| 17-Mar | 888 MW | 1,522 MW | 1,838 MW | 3,321 MW | 5,395 MW |
| 18-Mar | 1,375 MW | 1,688 MW | 2,069 MW | 3,576 MW | 5,957 MW |
| 19-Mar | 919 MW | 1,511 MW | 1,932 MW | 3,194 MW | 5,596 MW |
| 20-Mar | 979 MW | 1,406 MW | 1,650 MW | 2,642 MW | 4,660 MW |
| 21-Mar | 926 MW | 1,556 MW | 1,945 MW | 3,282 MW | 6,104 MW |
| 22-Mar | 1,192 MW | 2,155 MW | 3,015 MW | 5,714 MW | 10,750 MW |
| 23-Mar | 1,108 MW | 1,676 MW | 2,204 MW | 4,228 MW | 7,231 MW |
| 24-Mar | 1,621 MW | 2,493 MW | 3,453 MW | 6,685 MW | 12,831 MW |
| 25-Mar | 1,863 MW | 3,497 MW | 4,971 MW | 8,443 MW | 15,835 MW |
| 26-Mar | 2,119 MW | 3,407 MW | 4,875 MW | 9,515 MW | 17,942 MW |
|  | 3/10/2026 | 3/13/2026 | 3/13/2026 | 3/13/2026 | 3/13/2026 |
|  | (IE 15:42) | (IE 18:43) | (IE 18:39) | (IE 18:45) | (IE 19:05) |
| All Months in 2016-2026 | 3,797 MW | 4,431 MW | 6,479 MW | 11,451 MW | 19,909 MW |
|  | 5/28/2025 | 2/15/2026 | 2/15/2026 | 2/5/2026 | 1/18/2026 |
|  | (IE 10:27) | (IE 17:26) | (IE 17:27) | (IE 17:27) | (IE 17:27) |

# Congestion Analysis

## Notable Constraints

Nodal protocol section 3.20 specifies that ERCOT shall identify transmission constraints that are binding in Real-Time three or more Operating Days within a calendar month. As part of this process, ERCOT reports congestion that meets this criterion to ROS. In addition, ERCOT also highlights notable constraints that have an estimated congestion rent exceeding $1,000,000 for a calendar month. These constraints are detailed in the table below, including approved transmission upgrades from TPIT that may provide some congestion relief based on ERCOT’s engineering judgement. Rows highlighted in blue indicate the congestion was affected by one or more outages. For a list of all constraints activated in SCED, please see Appendix A at the end of this report.

| Data Item1 | Contingency Name | Overloaded Element | Contingency Name | Overloaded Element | # of Days Constraint Binding | Congestion Rent | Transmission Project |
| --- | --- | --- | --- | --- | --- | --- | --- |
| KLNSW_STAGE_641__A | DSALKLN5 | 641__A | SALSW TO KLNSW 345 DBLCKT | Killeen Switch - Stagecoach 138kV | 15 | $31,304,486 | Belton - Killeen 138 kV Line via Nolanville (24RPG001) Approved Tier 1 project (MOD 87703) |
| LNGSW_PRLSW_6965__A | DBAKCED5 | 6965__A | BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Longshore Switch - Prairieland Switch 345kV | 25 | $18,409,992 | West Texas 345-kV Infrastructure Rebuild Project ( 23RPG034) Tier 1 project MOD 81268 |
| n/a_n/a_WESTEX | BASE CASE | WESTEX | Basecase | WESTEX GTC | 20 | $18,268,865 | Permian Basin Relibility Plan (765-kV import paths) (The PUCT approved ,helps improve, but not fully exit, the GTC yet.) |
| BR_HOC_BR_HOC09_A | SMDOOAS5 | BR_HOC09_A | MEADOW to OASIS LIN A | Brays - H O Clarke 138kV | 10 | $14,741,585 |  |
| LOBO_LARDVNTH_LARDVN_LOBO2_1 | DFOAVLO5 | LARDVN_LOBO2_1 | FOWLERTON to LOBO & AVANZADA | Lobo - Laredo Vft North 138kV | 21 | $11,602,860 |  |
| n/a_n/a_EASTEX | BASE CASE | EASTEX | Basecase | EASTEX GTC | 14 | $10,030,490 |  |
| HHGT_OMEGA_HHGTOM_1 | BASE CASE | HHGTOM_1 | Basecase | Omega - Horse Hollow Generation Tie 345kV | 25 | $8,416,395 |  |
| ARGYL_LWSVH_587__A | MRNKDHM5 | 587__A | MANUAL RNKSW TO DHMSW 345&KRWSW TO DHMSW 345 DBLCKT | Argyle - Highlands Tnp 138kV | 16 | $7,149,709 |  |
| GN_PZ_GN_PZ_08_A | SMDOOAS5 | GN_PZ_08_A | MEADOW to OASIS LIN A | Grant - Plaza 138kV | 16 | $6,642,244 | Plaza to Grant Ckt.08I Upgrade (MOD 70660) Tier 4 |
| KLNSW_HHSTH_630__B | DSALKLN5 | 630__B | SALSW TO KLNSW 345 DBLCKT | Harker Heights South - Killeen Switch 138kV | 11 | $6,523,623 |  |
| n/a_n/a_NE_LOB | BASE CASE | NE_LOB | Basecase | NE_LOB GTC | 25 | $6,177,757 | San Antonio South Reliability I Project (22RPG048) LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |
| ZEN_THW_THWZEN98_A | DSNG_TB5 | THWZEN98_A | Sng-Tb&Rns 345kV | Th Wharton - Zenith 345kV | 9 | $5,821,851 |  |
| LA_PALMA_HAINE_DR_HAINE__LA_PAL1_1 | MHARNED5 | HAINE__LA_PAL1_1 | Manual dbl ckt for NEDIN-BONILLA 345kV & RIOH-PRIM138kV | Haine Drive - La Palma 138kV | 23 | $4,279,329 |  |
| LOBO_FREER_FREER_LOBO1_1 | DFOAVLO5 | FREER_LOBO1_1 | FOWLERTON to LOBO & AVANZADA | Lobo - Freer 69kV | 23 | $3,966,985 |  |
| SCRCV_KNAPP_6437__F | DMTSCOS5 | 6437__F | DMTSW TO SCOSW 345 DBLCKT | Knapp - Scurry Chevron 138kV | 20 | $3,642,796 |  |
| FVLSW_FTSSW_35065__A | DTCRTHS5 | 35065__A | THSES TO FBRSW & TCRSW 345 DBLCKT | Fort Smith Switch - Files Valley Switch 345kV | 8 | $3,596,281 |  |
| LPLMK_LPLNE_LPLMK_LPLNE_1 | SBWDDBM5 | LPLMK_LPLNE_1 | BLACKWATER DRAW SWITCH to DOUBLE MOUNTAIN SWITCH LIN 1 | Mackenzie Substation - Northeast Substation 115kV | 11 | $3,095,462 |  |
| ELMOT_ELMOT_ELMOT_MR2L | SBOSWHT5 | ELMOT_MR2L | BOSQUE SWITCH to WHITNEY LIN 1 | Elm Mott 138kV | 12 | $2,924,211 |  |
| FAIRDALE_COLLEGE_FAI_COLL_1 | SWALNAA8 | FAI_COLL_1 | NAAMAN to NAAMAN LIN 1 | College - Fairdale 138kV | 1 | $2,873,600 |  |
| SNDSW_AUSTRO_450__A | DSALHUT5 | 450__A | SALSW - HUTTO 345KV | Sandow Switch - Austrop 345kV | 14 | $2,765,844 | Oncor_SE_76650_Walleye Creek 345/138 kV Switch (23RPG030, MOD 76650) |
| PIPECR_MEDILA_255T279_1 | DCAGOMI8 | 255T279_1 | Cagnon-Omicron&Tx Research 138kV | Medina Lake - Pipe Creek 138kV | 3 | $2,604,251 |  |
| n/a_n/a_NELRIO | BASE CASE | NELRIO | Basecase | NELRIO GTC | 29 | $2,446,896 | The Lower Rio Grande Valley (LRGV) System Enhancement Project (21RPG017) |
| FORTMA_YELWJCKT_FORTMA_YELWJC1_1 | SBIGOR45 | FORTMA_YELWJC1_1 | Big HiLL to ORSTED LIN 1 | Yellow Jacket - Fort Mason 138kV | 7 | $2,426,136 |  |
| BOW_BOW_BOW_FMR1 | DRILKRW5 | BOW_FMR1 | RILEY TO KRWSW 345 DBLCKT | Bowie 138kV | 10 | $2,310,531 |  |
| FBRSW_THSES_505__B | STCRTHS5 | 505__B | TIGER CREEK SWITCH to TRADINGHOUSE SES LIN _A | Tradinghouse Ses - Four Brothers Switch 345kV | 9 | $2,232,719 |  |
| PKRSW_BNBSW_6005__A | SBNBPK25 | 6005__A | BENBROOK SWITCH to BENBROOK SWITCH LIN _A | Benbrook Switch - Parker Switch 345kV | 4 | $2,109,832 |  |
| MGSES_CATSW_6945__A | MRGRMGS5 | 6945__A | MAN_SGL_RGRSW-MGSES_345KV | Morgan Creek Ses - Cattleman Switch 345kV | 10 | $1,910,097 |  |
| n/a_n/a_E_PASP | BASE CASE | E_PASP | Basecase | E_PASP GTC | 15 | $1,881,679 | San Antonio South Reliability I Project , 22RPG048 LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |
| n/a_n/a_SAMSW | BASE CASE | SAMSW | Basecase | SAMSW GTC | 9 | $1,847,969 | Rebuild Sam Switch - Venus Switch 345 kV DCKT (24RPG017) Tier 1 (MOD 78369) |
| LA_PALMA_HAINE_DR_HAINE__LA_PAL1_1 | SRAYRI38 | HAINE__LA_PAL1_1 | LAS PULGAS to RAYMONDVILLE 2 LIN 1 | Haine Drive - La Palma 138kV | 8 | $1,805,796 |  |
| EXCSW_RNKSW_109__A | MKRWDHM5 | 109__A | MANUAL KRWSW TO WDENT 345&KRWSW TO DHMSW 345 DBLCKT | Roanoke Switch - Exchange Switch 345kV | 7 | $1,737,292 | Establish Roanoke_Exchange 138 kV Line (21RPG008) MOD 70900F |
| HONDOCK_DEVINESW_HON_DEVI_1 | SHONF38 | HON_DEVI_1 | HONDO to HONDO CREEK SWITCHING STATION LIN 1 | Hondo Creek Switching Station - Devine Switch 69kV | 28 | $1,573,511 |  |
| SAMSW_VENSW_35055__A | DCPSES12 | 35055__A | Comanche Peak 1 & 2 | Sam Switch - Venus Switch 345kV | 1 | $1,572,304 |  |
| HUTTO_HUTTO_HUTTO_MR1L | DHUTLIM5 | HUTTO_MR1L | HUTTO-LIMMER & HUTTO-LIMMER 345kV | Hutto Switch 138kV | 2 | $1,424,397 |  |
| ANGLETON_WCOLLOCL_G138_15_1 | SWCOWCO8 | G138_15_1 | WEST COLUMBIA MAIN TNP to ANGLETON TNP LIN 1 | West Columbia Local Tnp - Angleton Tnp 138kV | 4 | $1,273,182 | Rebuild West Columbia Local-Angleton with 795 ACSS (MOD 60526) |
| ELGISW_BUTLER_203T260_1 | DSALHUT5 | 203T260_1 | SALSW - HUTTO 345KV | Butler Lcra - Elgin Switch 138kV | 3 | $1,234,528 |  |
| LAKENASW_SAMATHIS_LAKENA_SAMATH1_1 | DBAKCED5 | LAKENA_SAMATH1_1 | BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Lake Nasworthy - San Angelo Mathis Field 69kV | 10 | $1,175,630 |  |
| ZEN_THW_THWZEN98_A | DRNS_TB5 | THWZEN98_A | Rns-Rtw & Sng-Tb 345kV | Th Wharton - Zenith 345kV | 1 | $1,006,031 |  |
| GEOWEST_SIGMOR_GEO_SIG_1 | DFOWSMG5 | GEO_SIG_1 | FOWLRTON TO SAN MIGUEL DOUBLE CIRCUIT CONTINGENCY | George West Switching Station - Sigmor 138kV | 12 | $1,003,176 |  |
| FORTMA_YELWJCKT_FORTMA_YELWJC1_1 | DBIGKEN5 | FORTMA_YELWJC1_1 | Bighil-Kendal 345kV | Yellow Jacket - Fort Mason 138kV | 10 | $976,828 |  |
| SAMSW_FVLSW_35045__A | DTCRTHS5 | 35045__A | THSES TO FBRSW & TCRSW 345 DBLCKT | Sam Switch - Files Valley Switch 345kV | 10 | $907,825 |  |
| BLESSING_BLESSING_BLESSING_1382 | DSTPSTA5 | BLESSING_1382 | Double STP to HLJ & Static 345 KV | Blessing 345kV | 3 | $907,530 |  |
| ENWSW_SHKSW_943__B | SENSENW8 | 943__B | ENNIS SOUTH to ENNIS SOUTH LIN _C | Shankle Switch - Ennis West Switch 138kV | 12 | $900,433 |  |
| FCRSW_CDHSW_3180__A | DCDHTVW5 | 3180__A | TVWSW TO CDHSW 345 AND CDHSW TO VENSW 345 DBLCKT | Fish Creek Switch - Cedar Hill Switch 138kV | 3 | $813,706 |  |
| PSA_BCV_BCVPSA03_A | DKG_NB_5 | BCVPSA03_A | TWR(345) JOR-KG97 & JOR-NB99 | Bigvue - Power Systems Arco Cogen 138kV | 7 | $812,219 |  |
| MLB_SLR_QUASAR_MLB_SLR_TLINE_1 | BASE CASE | MLB_SLR_TLINE_1 | Basecase | Quasar - Millers Branch Solar 345kV | 25 | $795,427 |  |
| CMNSW_CMNTP_651__B | SCMNCPS5 | 651__B | COMANCHE SWITCH (Oncor) to COMANCHE PEAK SES LIN _A | Comanche Tap - Comanche Switch (Oncor) 138kV | 20 | $783,454 |  |
| CELANEBI_KLEBERG_CELANE_KLEBER1_1 | SN_SLON5 | CELANE_KLEBER1_1 | LON HILL to NELSON SHARPE LIN 1 | Celanese Bishop - Kleberg Aep 138kV | 7 | $724,824 |  |
| SONR_ATSO_ATSO_SONR1_1 | SCARFRI8 | ATSO_SONR1_1 | Carver to Carver LIN 1 | Atlantic Sonora - Sonora 69kV | 20 | $682,684 |  |
| BRUNI_BRUNI_BRUNI_69_1 | DFOAVLO5 | BRUNI_69_1 | FOWLERTON to LOBO & AVANZADA | Bruni Sub 138kV | 7 | $610,332 |  |
| ASHERTON_CATARINA_ASHERT_CATARI1_1 | SLARLOB8 | ASHERT_CATARI1_1 | LAREDO VFT NORTH to LAREDO VFT NORTH LIN 1 | Asherton - Catarina 138kV | 17 | $596,126 |  |
| n/a_n/a_VALEXP | BASE CASE | VALEXP | Basecase | VALEXP GTC | 5 | $545,938 |  |
| BCKSW_CGRSW_6215__A | DSCOTKW5 | 6215__A | SCOSW TO TKWSW 345 DBLCKT | Bluff Creek Switch - China Grove Switch 138kV | 7 | $528,443 |  |
| FROWS_MCLSW_1025__B | SMCLWNE8 | 1025__B | MCLANE SWITCH to WACO NORTHEAST TAP LIN _A | Mclane Switch - Frow Sub 138kV | 12 | $510,093 |  |
| BR_HOC_BR_HOC09_A | BASE CASE | BR_HOC09_A | Basecase | Brays - H O Clarke 138kV | 6 | $498,077 |  |
| n/a_n/a_PNHNDL | BASE CASE | PNHNDL | Basecase | PNHNDL GTC | 6 | $491,593 |  |
| YELWJCKT_FORTMA_FORTMA_YELWJC1_1 | DBIGKEN5 | FORTMA_YELWJC1_1 | Bighil-Kendal 345kV | Yellow Jacket - Fort Mason 138kV | 10 | $486,816 |  |
| HAMILTON_MAVERICK_HAMILT_MAVERI1_1 | SBRAPIN8 | HAMILT_MAVERI1_1 | BRACKETTVILLE to BRACKETTVILLE LIN 1 | Hamilton Road - Maverick 138kV | 19 | $485,874 |  |
| LCSES_HAL_1020__D | DTPCTHS5 | 1020__D | DOUBLE TEMPLE PECAN CREEK SW TO TRADINGHOUSE SW AND BELFALLS SWITCH 345 KV | Hallsburg Bepc - Lake Creek Ses 138kV | 8 | $446,308 |  |
| PSA_BCV_BCVPSA03_A | SMDOOAS5 | BCVPSA03_A | MEADOW to OASIS LIN A | Bigvue - Power Systems Arco Cogen 138kV | 3 | $396,519 |  |
| ALAZAN_B_DAVIS_ALAZAN_B_DAVI1_1 | SN_SLON5 | ALAZAN_B_DAVI1_1 | LON HILL to NELSON SHARPE LIN 1 | Barney Davis - Alazan 138kV | 6 | $387,183 |  |
| SHKSW_GAMMA_921__F | DBBSRCH5 | 921__F | BBSES TO RCHBR 345 DBLCKT | Shankle Switch - Gamma 138kV | 3 | $343,226 |  |
| FURHMAN_RAYBURN_RAYBUR_FURHMAN_1 | SFURVAN8 | RAYBUR_FURHMAN_1 | FURHMAN to VANDERBILT SWITCHING STATION LIN 1 | Sam Rayburn Switchyd - Furhman 138kV | 20 | $329,864 |  |
| FBRSW_THSES_505__B | SLCSTH25 | 505__B | LAKE CREEK SES to LAKE CREEK SES LIN _A | Tradinghouse Ses - Four Brothers Switch 345kV | 3 | $307,169 |  |
| PSA_BCV_BCVPSA03_A | DDNKPSA8 | BCVPSA03_A | (TWR) 138kV DNK-PSA 08 & PSA-RVR 94 | Bigvue - Power Systems Arco Cogen 138kV | 4 | $290,835 |  |
| TNNALVIN_ALVIN_138_ALV_NAL_1 | SMDOPHR5 | 138_ALV_NAL_1 | MEADOW to PH ROBINSON LIN A | North Alvin Tnp - Alvin Tnp 138kV | 4 | $284,138 |  |
| TAYLR_TAYLR_TAYLR_FMR1 | DSALHUT5 | TAYLR_FMR1 | SALSW - HUTTO 345KV | Taylor (Oncor) 138kV | 4 | $274,387 |  |
| GILLCR_MCNEIL__211T147_1 | SGILTRI8 | 211T147_1 | AEN Gilleland Creek to AEN Gilleland Creek LIN 1 | Gilleland Creek - Mcneil 138kV | 4 | $266,239 |  |
| CARVER_TINSLEY_CARVER_TINSLE1_1 | SILLFTL8 | CARVER_TINSLE1_1 | FORT LANCASTER to FORT LANCASTER LIN 1 | Carver - Tinsley Tap 138kV | 5 | $253,951 |  |
| LCSES_LCSES_LCSES_MR2H | DTPCTHS5 | LCSES_MR2H | DOUBLE TEMPLE PECAN CREEK SW TO TRADINGHOUSE SW AND BELFALLS SWITCH 345 KV | Lake Creek Ses 345kV | 6 | $245,051 |  |
| MADDUX_SAPOWER_MADDUX_SAPOWE1_1 | SMA2SAP8 | MADDUX_SAPOWE1_1 | MADDUX to SAN ANGELO POWER STATION LIN 1 | Maddux - San Angelo Power Station 138kV | 12 | $234,290 |  |
| EXCSW_RNKSW_109__A | DRILKRW5 | 109__A | RILEY TO KRWSW 345 DBLCKT | Roanoke Switch - Exchange Switch 345kV | 5 | $223,805 |  |
| BRTSW_JUNPR_12020__B | MWCSRAM5 | 12020__B | MANUAL WCSWS TO RAMSW 345 DBLCKT TRPLCKT 3 of 3 | Barton Chapel Wind Farm - Juniper Sub 138kV | 3 | $221,836 |  |
| OZONA_MIDW_MIDW_OZONA1_1 | SWOLPAL8 | MIDW_OZONA1_1 | PALOUSE to Wolfcamp LIN 1 | Midway Lane - Ozona 69kV | 3 | $203,002 |  |
| DILLEYSW_DILLEYSW_DILLEYSW_XF1H | DDILCOT8 | DILLEYSW_XF1H | Dilleysw-Sanmgsw&Cotulas 138kV | Dilley Switch Aep 138kV | 8 | $199,628 |  |
| STP_WAP_STPWAP39_1 | DWPWFWP5 | STPWAP39_1 | TWR(345) WAP-WLF64 & WAP-WLY72 | South Texas Project - Wa Parish 345kV | 4 | $153,191 |  |
| NVKSW_ANARN_6840__B | SRICGRS8 | 6840__B | GRAHAM SES to RICE SWITCH LIN _A | Anarene - Navy Kickapoo Switch 69kV | 6 | $151,612 |  |
| HAMILTON_MAVERICK_HAMILT_MAVERI1_1 | SBRAHAM8 | HAMILT_MAVERI1_1 | BRACKETTVILLE to HAMILTON ROAD LIN 1 | Hamilton Road - Maverick 138kV | 9 | $144,874 |  |
| PALOUSE_WOLFCAMP_PALOUS_WOLFCA1_1 | DBIGSCH5 | PALOUS_WOLFCA1_1 | Big Hill - Schneeman Draw & Big Hill - Schneeman Draw 2 | Palouse - Wolfcamp 138kV | 6 | $131,831 |  |
| NLARSW_PILONCIL_NLARSW_PILONC1_1 | DFOAVLO5 | NLARSW_PILONC1_1 | FOWLERTON to LOBO & AVANZADA | North Laredo Switch - Piloncillo 138kV | 3 | $128,538 |  |
| BERGHE_BERGHE_BERGHE_AT1L | DZORHAY5 | BERGHE_AT1L | ZORN - HAYSEN 345KV | Bergheim 138kV | 3 | $120,428 |  |
| PILONCIL_NLARSW_NLARSW_PILONC1_1 | SLARLOB8 | NLARSW_PILONC1_1 | LAREDO VFT NORTH to LAREDO VFT NORTH LIN 1 | North Laredo Switch - Piloncillo 138kV | 4 | $119,600 |  |
| NVKSW_ANARN_6840__B | SSCLWF18 | 6840__B | WINDTHORST SWITCH to RICE SWITCH LIN _C | Anarene - Navy Kickapoo Switch 69kV | 10 | $119,065 |  |
| CRDSW_OLNEY_6830__B | DGRMGRS8 | 6830__B | FIREROCK TO BRNWD 138 AND FIREROCK TO BANGS 69 DBLCKT | Cottonwood Road Switch - Olney Pod 69kV | 8 | $112,326 |  |
| LAMPAS_NARUNA_713T713_1 | DSALHUT5 | 713T713_1 | SALSW - HUTTO 345KV | Naruna - Lampasas 138kV | 4 | $95,961 |  |
| CATARINA_ASHERTON_ASHERT_CATARI1_1 | DFOAVLO5 | ASHERT_CATARI1_1 | FOWLERTON to LOBO & AVANZADA | Asherton - Catarina 138kV | 3 | $89,790 |  |
| SOLSTICE_SOLSTICE_SOLSTICE_PST1 | DBAKSOL5 | SOLSTICE_PST1 | Bakersfield - Solstice line 1 and 2 | Solstice 138kV | 8 | $89,349 |  |
| LON_HILL_CLARK_WD_CLARK__LON_HI1_1 | DLONEQU8 | CLARK__LON_HI1_1 | Lon_Hill - Nueces & Equistar 138 kV | Clarkwood - Lon Hill 138kV | 6 | $70,112 |  |
| LOLITA_BLESSING_BLESSI_LOLITA1_1 | SPAWSAN5 | BLESSI_LOLITA1_1 | SAN MIGUEL 345_138 KV SWITCHYARDS to PAWNEE SWITCHING STATION LIN 1 | Blessing - Lolita 138kV | 3 | $67,349 |  |
| LON_HILL_CLARK_WD_CLARK__LON_HI1_1 | DWELONL8 | CLARK__LON_HI1_1 | LON_HILL - BUNSEN & WEIL_TRC 138 KV | Clarkwood - Lon Hill 138kV | 6 | $67,326 |  |
| PAWNEE_CALAVERS_PAWNEE_SPRUCE_1 | DELMSAN5 | PAWNEE_SPRUCE_1 | Elmcreek-Sanmigl 345kV | Pawnee Switching Station - Calaveras 345kV | 3 | $66,508 |  |
| BRTSW_JUNPR_12020__B | DLONPK25 | 12020__B | DBL_PKRSW-GRSES_PKRSW-LONSW_345KV | Barton Chapel Wind Farm - Juniper Sub 138kV | 4 | $65,300 |  |
| MADDUX_SAPOWER_MADDUX_SAPOWE2_1 | DYETRJU8 | MADDUX_SAPOWE2_1 | Double for YellowJacket - Treadwell & Junction | Maddux - San Angelo Power Station 138kV | 5 | $61,036 |  |
| SACRC_DPCRK_6240__C | DMTSCOS5 | 6240__C | DMTSW TO SCOSW 345 DBLCKT | Sacroc - Deep Creek Sub 138kV | 11 | $60,476 |  |
| n/a_n/a_UVALDE | BASE CASE | UVALDE | Basecase | UVALDE GTC | 20 | $59,347 |  |
| CRDSW_OLNEY_6830__B | SGRMGRS8 | 6830__B | GRAHAM EAST to GRAHAM EAST LIN _A | Cottonwood Road Switch - Olney Pod 69kV | 6 | $45,826 |  |
| FERGUS_WIRTZ_40T189_1 | DFERWIR8 | 40T189_1 | Ferguson-Sherwood Shores & Ferguson-Wirtz 138 kV | Ferguson - Wirtz 138kV | 3 | $44,792 |  |
| SOLSTICE_SOLSTICE_SOLSTICE_PST1 | STARLEO8 | SOLSTICE_PST1 | LEON CREEK TNP to TARBUSH TNP LIN 1 | Solstice 138kV | 4 | $43,548 |  |
| TREADWEL_YELWJCKT_TREADW_YELWJC1_1 | DBIGKEN5 | TREADW_YELWJC1_1 | Bighil-Kendal 345kV | Yellow Jacket - Treadwell 138kV | 4 | $36,843 |  |
| ODESA_ODNTH_6513__A | DODEMOS5 | 6513__A | ODEHV-MOSSW 345&ODEHV-WLFSW 345_DBLCKT | Odessa North - Odessa 138kV | 3 | $32,432 |  |
| BLASW_COFESSRC_BLASCOFE_RC_1 | SEUSWLT8 | BLASCOFE_RC_1 | EUSTACE SOUTHEAST to WALTON LIN 1 | Coffee - Blackburn Switch 138kV | 12 | $30,725 |  |
| YELWJCKT_FORTMA_FORTMA_YELWJC1_1 | DBUCRGP5 | FORTMA_YELWJC1_1 | MANUAL DOUBLE RGPSW - KLNSW 345 KV & BUCSW - KLNSW 345 KV | Yellow Jacket - Fort Mason 138kV | 5 | $28,235 |  |
| COLETO_VICTORIA_COLETO_VICTOR2_1 | SVICCO28 | COLETO_VICTOR2_1 | COLETO CREEK to VICTORIA LIN 1 | Coleto Creek - Victoria 138kV | 3 | $23,818 |  |
| LOBO_FREER_FREER_LOBO1_1 | DDNEDBP5 | FREER_LOBO1_1 | double NORTH EDINBURG to BRISITA & Pomelo | Lobo - Freer 69kV | 4 | $22,885 |  |
| n/a_n/a_I_KALO | BASE CASE | I_KALO | Basecase | I_KALO GTC | 3 | $20,562 |  |
| MAXWELL_WHITING_MAXWEL_WHITIN1_1 | BASE CASE | MAXWEL_WHITIN1_1 | Basecase | Maxwell - Whiting 138kV | 4 | $18,365 |  |
| SNYDR_SNYDR_SNYDR_FMR1 | SSNYCGR8 | SNYDR_FMR1 | SNYDER to CHINA GROVE SWITCH LIN _A | Snyder 138kV | 4 | $17,061 |  |
| COLETO_VICTORIA_COLETO_VICTOR2_1 | SPAWSAN5 | COLETO_VICTOR2_1 | SAN MIGUEL 345_138 KV SWITCHYARDS to PAWNEE SWITCHING STATION LIN 1 | Coleto Creek - Victoria 138kV | 6 | $16,636 |  |
| LEONCRK_FTST_138_FTS_LNC_1 | STARLEO8 | 138_FTS_LNC_1 | LEON CREEK TNP to TARBUSH TNP LIN 1 | Leon Creek Tnp - Fort Stockton Plant 138kV | 5 | $12,958 |  |
| RIESW_GRSBK_1220__G | DTPCTHS5 | 1220__G | DOUBLE TEMPLE PECAN CREEK SW TO TRADINGHOUSE SW AND BELFALLS SWITCH 345 KV | Riesel Switch - Groesbeck (Oncor) 138kV | 3 | $11,157 |  |
| SACRC_DPCRK_6240__C | SSUNRGR8 | 6240__C | SUN SWITCH to RANGER CAMP SWITCH LIN _B | Sacroc - Deep Creek Sub 138kV | 5 | $10,896 |  |
| RAYBURN_RAYBURN_RAYBURN_69_2 | SL_4RAY8 | RAYBURN_69_2 | LOOP 463 SUB to LOOP 463 SUB LIN 1 | Sam Rayburn Switchyd 138kV | 3 | $9,264 |  |
| IH20_TNCOLIET_138_IH2_COT_1 | SSTAWIC8 | 138_IH2_COT_1 | STAGHORN TNP to WICKETT TNP LIN 1 | Ih 20 Tnp - Collie Field Tap Tnp 138kV | 6 | $8,125 |  |
| MEXTP_ITALY_2270__B | SBLURDH8 | 2270__B | BLUM to RED HILL SWITCH LIN _G | Milford Exxon Tap - Italy 69kV | 3 | $6,432 |  |
| n/a_n/a_KINNEY | BASE CASE | KINNEY | Basecase | KINNEY GTC | 4 | $5,842 |  |
| UVALDE_LAPRYOR_LAPRYO_UVALDE1_1 | SW_BUVA8 | LAPRYO_UVALDE1_1 | WEST BATESVILLE to UVALDE AEP LIN 1 | La Pryor - Uvalde Aep 69kV | 5 | $5,760 |  |
| UVALDE_W_BATESV_UVALDE_W_BATE1_1 | SREAUVA8 | UVALDE_W_BATE1_1 | Reading to Reading LIN 1 | Uvalde Aep - West Batesville 138kV | 4 | $4,997 |  |
| MADDUX_SAPOWER_MADDUX_SAPOWE2_1 | STRECFL8 | MADDUX_SAPOWE2_1 | Treadwell to CFLATS SUBSTATION LIN 1 | Maddux - San Angelo Power Station 138kV | 3 | $4,307 |  |
| WLVSW_GAILS_6217__A | SBUZHMP8 | 6217__A | BUZZARD DRAW SWITCH to HARDY MORGAN POI LIN _B | Willow Valley Switch - Gail Sub 138kV | 3 | $3,318 |  |
| LNGSW_PRLSW_6965__A | MCATPRL5 | 6965__A | MAN_SGL_CATSW-PRLSW_345KV | Longshore Switch - Prairieland Switch 345kV | 3 | $3,147 |  |
| TNFXTAIL_FLAT_TOP_138_FLT_FXT_1 | DSLKSOL5 | 138_FLT_FXT_1 | Sand Lake - Solstice line 1 and 2 | Foxtail Tnp - Flat Top Tnp 138kV | 5 | $2,206 |  |
| YELWJCKT_FORTMA_FORTMA_YELWJC1_1 | SBIGOR45 | FORTMA_YELWJC1_1 | Big HiLL to ORSTED LIN 1 | Yellow Jacket - Fort Mason 138kV | 7 | $1,826 |  |
| W_BATESV_UVALDE_UVALDE_W_BATE1_1 | MSUNESC8 | UVALDE_W_BATE1_1 | Construction Manual SUNGLOW-ESCONDID 138kV | Uvalde Aep - West Batesville 138kV | 4 | $1,200 |  |
| LCSES_RIESW_1225__A | DTPCTHS5 | 1225__A | DOUBLE TEMPLE PECAN CREEK SW TO TRADINGHOUSE SW AND BELFALLS SWITCH 345 KV | Lake Creek Ses - Riesel Switch 138kV | 3 | $1,073 |  |
| UVALDE_W_BATESV_UVALDE_W_BATE1_1 | MSUNESC8 | UVALDE_W_BATE1_1 | Construction Manual SUNGLOW-ESCONDID 138kV | Uvalde Aep - West Batesville 138kV | 4 | $688 |  |
| BRONCO_ALPINE_ALPINE_BRONCO1_1 | XFTS89 | ALPINE_BRONCO1_1 | FORT STOCKTON PLANT TRX 69T1 138/69 | Alpine - Bronco 69kV | 4 | $252 |  |

## Generic Transmission Constraint Congestion

| GTC | Days Congestion |
| --- | --- |
| Nelson Sharpe - Rio Hondo | 30 |
| North Edinburg – Lobo | 29 |
| West Texas | 26 |
| Uvalde | 24 |
| South Texas Export Pawnee – Spruce | 19 |
| East Texas | 16 |
| South Texas Export Pawnee – Tango | 15 |
| Panhandle | 14 |
| SAM Switch | 11 |
| Valley Export | 6 |
| Wharton | 6 |
| Kinney | 6 |
| McCamey | 5 |
| North – Houston | 5 |
| South Texas Import Katoen - Lonhill | 3 |
| South Texas Import Pawnee – Spruce | 2 |
| Zapata Starr | 2 |

- There was no activity on the remaining GTCs during the month.

Note: This is how many times a constraint has been activated to avoid exceeding a GTC limit, it does not imply an exceedance of the GTC occurred or that the GTC was binding.

## Manual Overrides

- None

## Congestion Costs for Calendar Year 2025

The following table represents the top twenty active constraints for the calendar year based on the estimated congestion rent attributed to the congestion. ERCOT updates this list on a monthly basis.

| Contingency | Overloaded Element | # of 5-min SCED | Estimated | Transmission |
| --- | --- | --- | --- | --- |
| Basecase | WESTEX GTC | 7,997 | 40,374,553.19 | The PUCT approved Permian Basin Reliability Plan (765-kV import paths) helps improve, but not fully exit, the GTC yet. |
| BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Longshore Switch - Prairieland Switch 345kV | 7,723 | 32,213,444.72 | West Texas 345-kV Infrastructure Rebuild Project (23RPG034, MOD 81268) |
| SALSW TO KLNSW 345 DBLCKT | Killeen Switch - Stagecoach 138kV | 2,796 | 31,304,486.27 |  |
| MANUAL RNKSW TO DHMSW 345&KRWSW TO DHMSW 345 DBLCKT | Argyle - Highlands Tnp 138kV | 5,192 | 30,415,666.38 |  |
| DOUBLE NVARO-LEG & NVARO-OUTSW 345 KV | Big Brown Ses - Jewett 345kV | 504 | 26,003,760.95 |  |
| BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Hargrove - Twin Buttes 138kV | 3,739 | 20,139,945.01 |  |
| BLACKWATER DRAW SWITCH to DOUBLE MOUNTAIN SWITCH LIN 1 | Mackenzie Substation - Northeast Substation 115kV | 3,940 | 16,807,520.07 |  |
| Basecase | E_PASP GTC | 5,103 | 16,259,224.43 | The following RPG-endorsed projects will help improve the GTC but not fully exit the GTC yet: San Antonio South Reliability I Project (22RPG048) LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |
| STP-Elmcreek&WAP 345kV | South Texas Project - Elmcreek 345kV | 355 | 15,792,038.58 |  |
| FOWLERTON to LOBO & AVANZADA | Lobo - Laredo Vft North 138kV | 5,426 | 15,369,961.62 |  |
| MEADOW to OASIS LIN A | Brays - H O Clarke 138kV | 2,124 | 14,775,630.35 |  |
| KENEDSW - TULETA (138) & PETTUS - BAEZ (69) | Coleto Creek - Rosata Tap 138kV | 538 | 14,029,054.48 | Coleto Creek to Kenedy SS 138 kV Transmission Project (20RPG014) approved Tier 3 project (MOD 50870) is supposed to be in service in June 2025, but Grid Geo still reflects original line ratings. |
| Basecase | Omega - Horse Hollow Generation Tie 345kV | 5,433 | 12,167,303.85 |  |
| Basecase | NE_LOB GTC | 9,992 | 10,671,548.78 | San Antonio South Reliability I Project (22RPG048) LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |
| Basecase | EASTEX GTC | 651 | 10,030,862.50 |  |
| DMTSW TO SCOSW 345 DBLCKT | Knapp - Scurry Chevron 138kV | 4,535 | 9,639,432.21 |  |
| TEMPLE PECAN CREEK to TEMPLE SWITCH LIN _A | Temple Pecan Creek - Temple Switch 345kV | 884 | 8797593.041 |  |
| Manual dbl ckt for NEDIN-BONILLA 345kV & RIOH-PRIM138kV | Haine Drive - La Palma 138kV | 6,121 | 8,725,377.43 |  |
| Basecase | PNHNDL GTC | 5,550 | 8,463,517.97 |  |
| FORT SMITH SWITCH to FORT SMITH SWITCH LIN _B | Sam Switch - Venus Switch 345kV | 1,533 | 7,682,999.19 | Venus Switch to Sam Switch 345-kV Line Project 24RPG017 approved Tier 1 project (MOD 78369) |

# System Events

## ERCOT Peak Load

The unofficial ERCOT peak load for March 2026 was 66,515 MW and occurred on 03/25/2026 during hour ending 18:00, this is 3,591 MW higher than the March 2025 demand of 62,924 MW on 03/25/2025 during hour ending 18:00. Instantaneous peak for March 2026 was 66,774 MW. Actual instantaneous peak for the same month last year was 63,523 MW.

## Load Shed Events

- None.

## Stability Events

- None.

## Notable PMU Events

ERCOT analyzes PMU data for any significant system disturbances that do not fall into the Frequency Events category reported in section 2.1. The results are summarized in this section once the analysis has been completed.

There were no PMU events outside of those reported in section 2.1.

## DC Tie Curtailment

- None.

## TRE/DOE Reportable Events

- Geronimo submitted a DOE-417 for 3/17/2026 – Physical threat to its facility that results from actual or suspected intentional human action.

- Tenaska submitted a DOE-417 for 3/26/2026 - Cyber event that could potentially impact electric power system adequacy or reliability.

## New/Updated Constraint Management Plans

MP_2025_42 REV1, MP_2021_03 REV8

## New/Modified/Removed RAS

- None

## New Procedures/Forms/Operating Bulletins

| Date | Subject | Bulletin No. |
| --- | --- | --- |
| 3/31/2026 | Transmission and Security Desk V2 Rev 4 | 2024 |
| 3/31/2026 | Scripts V2 Rev 2 | 2023 |
| 3/31/2026 | Reliability Unit Commitment V2 Rev 3 | 2022 |
| 3/31/2026 | Real Time Desk V2 Rev 2 | 2021 |

# Emergency Conditions

## OCNs

| Date and Time | Message |
| --- | --- |
| Mar 16, 2026 10:24:12 AM | At 10:30 , ERCOT is issuing an OCN due to a potential wildfire risk for Monday, March 16, 2026, for a large portion of South-Central Texas in the ERCOT region until further notice. |
| Mar 16, 2026 6:49:33 PM | ERCOT has issued an OCN for the unplanned outage of ERCOT’s MIS outage. |

## Advisories

| Date and Time | Message |
| --- | --- |
| Mar 20, 2026 6:52:46 PM | At 18:28 The Space Weather Prediction Center has issued a GMD ALERT of K7 until March 21, 2026 at 01:00. |
| Mar 22, 2026 1:22:29 PM | The Space Weather Prediction Center has issued a GMD Alert of [K-7] until [03/22/2026 23:59]. |

## Watches

| Date and Time | Message |
| --- | --- |
| Mar 5, 2026 11:44:47 PM | ERCOT has declared a Watch due to the failure of the SCED process, starting at 23:35 |

## Emergency Notices

- None.

# Application Performance

## TSAT/VSAT Performance Issues

| Date and Time | Message |
| --- | --- |

## Communication Issues

- None.

## Market System Issues

- ERCOT declared a Watch due to SCED failure on March 5, 2026. SCED failure started at approximately 23:28, and SCED became functional at approximately 23:40.

# Model Updates

The Downstream Production Change (DPC) process allows ERCOT to make changes in the one-line Network Operations Model without loading a completely new model.  The purpose of this process is to allow for reliable grid operations as system conditions change between designated Network Operations Model database loads.  The DPC process is limited in scope to just those items listed below, with equipment ratings updates being the most common.  ERCOT has seen a rise in the use of the DPC process to make on-line updates to the Network Operations Model in recent years, instead of through the standard Network Operations Model Change Request process.

- Static Line ratings (Interim Update)

- Dynamic Line ratings (non-Interim Update)

- Autotransformer ratings (non-Interim Update)

- Breaker and Switch Normal status (Interim Update)

- Contingency Definitions (Interim Update)

- RAP and RAS changes or additions (Interim Update)

- Net Dependable and Reactive Capability (NDCRC) values (Interim Update)

- Impedance Updates (non-Interim)

A total of 43 DPCs were implemented in March 2026. 71 DPCs have been implemented year to date. DPCs submitted by TDSPs are mainly updates to transmission element ratings. DPCs submitted by ERCOT are mainly updates to manual contingency definitions.

| Transmission Operator | Number of DPCs |
| --- | --- |
| AEP TEXAS COMPANY (TDSP) | 4 |
| BRAZOS ELECTRIC POWER CO OP INC (TDSP) | 0 |
| BROWNSVILLE PUBLIC UTILITIES BOARD (TDSP) | 0 |
| BRYAN TEXAS UTILITIES (TDSP) | 0 |
| CENTERPOINT ENERGY HOUSTON ELECTRIC LLC (TDSP) | 3 |
| CITY OF AUSTIN DBA AUSTIN ENERGY (TDSP) | 0 |
| CITY OF COLLEGE STATION (TDSP) | 0 |
| CITY OF GARLAND (TDSP) | 0 |
| CPS ENERGY (TDSP) | 0 |
| CROSS TEXAS TRANSMISSION LLC (TSP)) | 0 |
| DENTON MUNICIPAL ELECTRIC (TDSP) | 0 |
| ELECTRIC TRANSMISSION TEXAS LLC (TDSP) | 0 |
| ERCOT | 2 |
| LCRA TRANSMISSION SERVICES CORPORATION (TDSP) | 21 |
| LONE STAR TRANSMISSION LLC (TSP) | 2 |
| ONCOR ELECTRIC DELIVERY COMPANY LLC (TDSP) | 8 |
| PEDERNALES ELECTRIC CO OP INC (TDSP) | 0 |
| RAYBURN COUNTRY CO OP DBA RAYBURN ELECTRIC (TDSP) | 0 |
| SHARYLAND UTILITIES LP (TDSP) | 0 |
| SOUTH TEXAS ELECTRIC CO OP INC (TDSP) | 1 |
| TEXAS MUNICIPAL POWER AGENCY (TDSP) | 0 |
| TEXAS-NEW MEXICO POWER CO (TDSP) | 2 |
| WIND ENERGY TRANSMISSION TEXAS LLC (TSP) | 0 |

# Appendix A: Real-Time Constraints

The following is a complete list of constraints activated in SCED. Full contingency descriptions can be found in the Standard Contingencies List located on the MIS secure site at Grid  Generation  Reliability Unit Commitment.

| Year | Month | Contingency Name | Overloaded Element | From Station | To Station | Count of Days |
| --- | --- | --- | --- | --- | --- | --- |
| 2026 | March | BASE CASE | NELRIO | n/a | n/a | 30 |
| 2026 | March | DBAKCED5 | 6965__A | LNGSW | PRLSW | 29 |
| 2026 | March | BASE CASE | NE_LOB | n/a | n/a | 29 |
| 2026 | March | SFURVAN8 | RAYBUR_FURHMAN_1 | FURHMAN | RAYBURN | 28 |
| 2026 | March | SHONF38 | HON_DEVI_1 | HONDOCK | DEVINESW | 28 |
| 2026 | March | SFURVAN8 | RAYBUR_FURHMAN_1 | RAYBURN | FURHMAN | 28 |
| 2026 | March | BASE CASE | MLB_SLR_TLINE_1 | QUASAR | MLB_SLR | 27 |
| 2026 | March | BASE CASE | MLB_SLR_TLINE_1 | MLB_SLR | QUASAR | 27 |
| 2026 | March | DFOAVLO5 | FREER_LOBO1_1 | LOBO | FREER | 27 |
| 2026 | March | BASE CASE | WESTEX | n/a | n/a | 26 |
| 2026 | March | BASE CASE | HHGTOM_1 | HHGT | OMEGA | 26 |
| 2026 | March | MHARNED5 | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 24 |
| 2026 | March | BASE CASE | UVALDE | n/a | n/a | 24 |
| 2026 | March | SBRAPIN8 | HAMILT_MAVERI1_1 | HAMILTON | MAVERICK | 22 |
| 2026 | March | DFOAVLO5 | LARDVN_LOBO2_1 | LOBO | LARDVNTH | 22 |
| 2026 | March | DMTSCOS5 | 6437__F | SCRCV | KNAPP | 21 |
| 2026 | March | SCARFRI8 | ATSO_SONR1_1 | SONR | ATSO | 20 |
| 2026 | March | SCMNCPS5 | 651__B | CMNSW | CMNTP | 20 |
| 2026 | March | MRNKDHM5 | 587__A | ARGYL | LWSVH | 19 |
| 2026 | March | SEUSWLT8 | BLASCOFE_RC_1 | COFESSRC | BLASW | 19 |
| 2026 | March | BASE CASE | E_PASP | n/a | n/a | 19 |
| 2026 | March | SEUSWLT8 | BLASCOFE_RC_1 | BLASW | COFESSRC | 19 |
| 2026 | March | SLARLOB8 | ASHERT_CATARI1_1 | ASHERTON | CATARINA | 18 |
| 2026 | March | SMDOOAS5 | GN_PZ_08_A | GN | PZ | 17 |
| 2026 | March | DFOWSMG5 | GEO_SIG_1 | GEOWEST | SIGMOR | 17 |
| 2026 | March | BASE CASE | EASTEX | n/a | n/a | 16 |
| 2026 | March | DSALKLN5 | 641__A | KLNSW | STAGE | 15 |
| 2026 | March | BASE CASE | E_PATA | n/a | n/a | 15 |
| 2026 | March | SENSENW8 | 943__B | SHKSW | ENWSW | 15 |
| 2026 | March | SENSENW8 | 943__B | ENWSW | SHKSW | 15 |
| 2026 | March | SMCLWNE8 | 1025__B | FROWS | MCLSW | 14 |
| 2026 | March | BASE CASE | PNHNDL | n/a | n/a | 14 |
| 2026 | March | SBWDDBM5 | LPLMK_LPLNE_1 | LPLMK | LPLNE | 14 |
| 2026 | March | DSALHUT5 | 450__A | SNDSW | AUSTRO | 14 |
| 2026 | March | SBOSWHT5 | ELMOT_MR2L | ELMOT | ELMOT | 14 |
| 2026 | March | DMTSCOS5 | 6240__C | SACRC | DPCRK | 13 |
| 2026 | March | BASE CASE | MAXWEL_WHITIN1_1 | WHITING | MAXWELL | 13 |
| 2026 | March | BASE CASE | MAXWEL_WHITIN1_1 | MAXWELL | WHITING | 13 |
| 2026 | March | SMA2SAP8 | MADDUX_SAPOWE1_1 | SAPOWER | MADDUX | 13 |
| 2026 | March | SMA2SAP8 | MADDUX_SAPOWE1_1 | MADDUX | SAPOWER | 13 |
| 2026 | March | DBIGKEN5 | FORTMA_YELWJC1_1 | FORTMA | YELWJCKT | 12 |
| 2026 | March | DBIGKEN5 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 12 |
| 2026 | March | DBAKCED5 | LAKENA_SAMATH1_1 | LAKENASW | SAMATHIS | 12 |
| 2026 | March | DRILKRW5 | BOW_FMR1 | BOW | BOW | 11 |
| 2026 | March | SSCLWF18 | 6840__B | NVKSW | ANARN | 11 |
| 2026 | March | DSALKLN5 | 630__B | KLNSW | HHSTH | 11 |
| 2026 | March | DDILCOT8 | DILLEYSW_XF1H | DILLEYSW | DILLEYSW | 11 |
| 2026 | March | SN_SLON5 | CELANE_KLEBER1_1 | CELANEBI | KLEBERG | 11 |
| 2026 | March | BASE CASE | SAMSW | n/a | n/a | 11 |
| 2026 | March | MRGRMGS5 | 6945__A | MGSES | CATSW | 11 |
| 2026 | March | SBRAHAM8 | HAMILT_MAVERI1_1 | HAMILTON | MAVERICK | 10 |
| 2026 | March | DTCRTHS5 | 35045__A | SAMSW | FVLSW | 10 |
| 2026 | March | BASE CASE | BR_HOC09_A | BR | HOC | 10 |
| 2026 | March | SMDOOAS5 | BR_HOC09_A | BR | HOC | 10 |
| 2026 | March | DSCOTKW5 | 6215__A | BCKSW | CGRSW | 10 |
| 2026 | March | DSNG_TB5 | THWZEN98_A | ZEN | THW | 9 |
| 2026 | March | MKRWDHM5 | 109__A | EXCSW | RNKSW | 9 |
| 2026 | March | DBAKSOL5 | SOLSTICE_PST1 | SOLSTICE | SOLSTICE | 9 |
| 2026 | March | SBIGOR45 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 9 |
| 2026 | March | DGRMGRS8 | 6830__B | CRDSW | OLNEY | 9 |
| 2026 | March | SBIGOR45 | FORTMA_YELWJC1_1 | FORTMA | YELWJCKT | 9 |
| 2026 | March | SSTAWIC8 | 138_IH2_COT_1 | IH20 | TNCOLIET | 9 |
| 2026 | March | STCRTHS5 | 505__B | FBRSW | THSES | 9 |
| 2026 | March | DTCRTHS5 | 35065__A | FVLSW | FTSSW | 8 |
| 2026 | March | SRAYRI38 | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 8 |
| 2026 | March | SSNYCGR8 | SNYDR_FMR1 | SNYDR | SNYDR | 8 |
| 2026 | March | DTPCTHS5 | LCSES_MR2H | LCSES | LCSES | 8 |
| 2026 | March | DBIGSCH5 | PALOUS_WOLFCA1_1 | PALOUSE | WOLFCAMP | 8 |
| 2026 | March | DTPCTHS5 | 1020__D | LCSES | HAL | 8 |
| 2026 | March | DLONPK25 | 12020__B | BRTSW | JUNPR | 8 |
| 2026 | March | DLONEQU8 | CLARK__LON_HI1_1 | LON_HILL | CLARK_WD | 8 |
| 2026 | March | DFOAVLO5 | BRUNI_69_1 | BRUNI | BRUNI | 8 |
| 2026 | March | DSLKSOL5 | 138_FLT_FXT_1 | TNFXTAIL | FLAT_TOP | 7 |
| 2026 | March | SGRMGRS8 | 6830__B | CRDSW | OLNEY | 7 |
| 2026 | March | DKG_NB_5 | BCVPSA03_A | PSA | BCV | 7 |
| 2026 | March | DDNEDBP5 | FREER_LOBO1_1 | LOBO | FREER | 7 |
| 2026 | March | DWPWFWP5 | STPWAP39_1 | STP | WAP | 7 |
| 2026 | March | SRICGRS8 | 6840__B | NVKSW | ANARN | 7 |
| 2026 | March | DWELONL8 | CLARK__LON_HI1_1 | LON_HILL | CLARK_WD | 7 |
| 2026 | March | SMDOPHR5 | 138_ALV_NAL_1 | TNNALVIN | ALVIN | 6 |
| 2026 | March | BASE CASE | VALEXP | n/a | n/a | 6 |
| 2026 | March | DSALHUT5 | 713T713_1 | LAMPAS | NARUNA | 6 |
| 2026 | March | SPAWSAN5 | COLETO_VICTOR2_1 | COLETO | VICTORIA | 6 |
| 2026 | March | DSALHUT5 | TAYLR_FMR1 | TAYLR | TAYLR | 6 |
| 2026 | March | SN_SLON5 | ALAZAN_B_DAVI1_1 | ALAZAN | B_DAVIS | 6 |
| 2026 | March | SILLFTL8 | CARVER_TINSLE1_1 | CARVER | TINSLEY | 6 |
| 2026 | March | BASE CASE | WHARTN | n/a | n/a | 6 |
| 2026 | March | DRILKRW5 | 109__A | EXCSW | RNKSW | 6 |
| 2026 | March | SGILTRI8 | 211T147_1 | GILLCR | MCNEIL_ | 6 |
| 2026 | March | SSUNRGR8 | 6240__C | SACRC | DPCRK | 6 |
| 2026 | March | SWLTBRT8 | EUSTWLTN_RC_1 | EUSTSERC | WALTSSRC | 6 |
| 2026 | March | SWLTBRT8 | EUSTWLTN_RC_1 | WALTSSRC | EUSTSERC | 6 |
| 2026 | March | BASE CASE | KINNEY | n/a | n/a | 6 |
| 2026 | March | SBUZHMP8 | 6217__A | WLVSW | GAILS | 6 |
| 2026 | March | BASE CASE | MCCAMY | n/a | n/a | 5 |
| 2026 | March | DBUCRGP5 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 5 |
| 2026 | March | MSUNESC8 | UVALDE_W_BATE1_1 | UVALDE | W_BATESV | 5 |
| 2026 | March | STARLEO8 | 138_FTS_LNC_1 | FTST | LEONCRK | 5 |
| 2026 | March | STARLEO8 | 138_FTS_LNC_1 | LEONCRK | FTST | 5 |
| 2026 | March | DYETRJU8 | MADDUX_SAPOWE2_1 | MADDUX | SAPOWER | 5 |
| 2026 | March | MHARNED5 | PANTER_WESMER1_1 | WESMER | PANTERA | 5 |
| 2026 | March | DCDHTVW5 | 3180__A | FCRSW | CDHSW | 5 |
| 2026 | March | SVICCO28 | COLETO_VICTOR2_1 | COLETO | VICTORIA | 5 |
| 2026 | March | SW_BUVA8 | LAPRYO_UVALDE1_1 | UVALDE | LAPRYOR | 5 |
| 2026 | March | DODEMOS5 | 6513__A | ODESA | ODNTH | 5 |
| 2026 | March | SPAWSAN5 | BLESSI_LOLITA1_1 | LOLITA | BLESSING | 5 |
| 2026 | March | MSUNESC8 | UVALDE_W_BATE1_1 | W_BATESV | UVALDE | 5 |
| 2026 | March | MCATPRL5 | 6965__A | LNGSW | PRLSW | 5 |
| 2026 | March | DLOFOAV5 | BRUNI_69_1 | BRUNI | BRUNI | 5 |
| 2026 | March | BASE CASE | N_TO_H | n/a | n/a | 5 |
| 2026 | March | STARLEO8 | SOLSTICE_PST1 | SOLSTICE | SOLSTICE | 5 |
| 2026 | March | DDNKPSA8 | BCVPSA03_A | PSA | BCV | 5 |
| 2026 | March | DELMSAN5 | PAWNEE_SPRUCE_1 | PAWNEE | CALAVERS | 5 |
| 2026 | March | SBNBPK25 | 6005__A | PKRSW | BNBSW | 4 |
| 2026 | March | SLARLOB8 | NLARSW_PILONC1_1 | PILONCIL | NLARSW | 4 |
| 2026 | March | BASE CASE | CTHR_TINSLE1_1 | CTHR | TINSLEY | 4 |
| 2026 | March | SBRTANT8 | EUSTWLTN_RC_1 | EUSTSERC | WALTSSRC | 4 |
| 2026 | March | DHUTLIM5 | HUTTO_MR1L | HUTTO | HUTTO | 4 |
| 2026 | March | DTPCTHS5 | 1225__A | LCSES | RIESW | 4 |
| 2026 | March | SCSDNK8 | BCVPSA03_A | PSA | BCV | 4 |
| 2026 | March | BASE CASE | CTHR_TINSLE1_1 | TINSLEY | CTHR | 4 |
| 2026 | March | DBIGKEN5 | TREADW_YELWJC1_1 | TREADWEL | YELWJCKT | 4 |
| 2026 | March | DSTPSTA5 | BLESSING_1382 | BLESSING | BLESSING | 4 |
| 2026 | March | SWCOWCO8 | G138_15_1 | ANGLETON | WCOLLOCL | 4 |
| 2026 | March | SMOOPEA8 | READIN_UVALDE1_1 | UVALDE | READING | 4 |
| 2026 | March | DCAGCI58 | 255T279_1 | PIPECR | MEDILA | 4 |
| 2026 | March | DTWLCED5 | 6965__A | LNGSW | PRLSW | 4 |
| 2026 | March | SBAKCED5 | BAKRFLD_CEDCAN_1 | CEDACA | BAKESW | 4 |
| 2026 | March | SMDOOAS5 | BCVPSA03_A | PSA | BCV | 4 |
| 2026 | March | SBRTANT8 | EUSTWLTN_RC_1 | WALTSSRC | EUSTSERC | 4 |
| 2026 | March | SHA2MAX8 | MAXWEL_WHITIN1_1 | MAXWELL | WHITING | 4 |
| 2026 | March | SHONMOO8 | BIG_FOOT_69A1 | BIG_FOOT | BIG_FOOT | 4 |
| 2026 | March | STRECFL8 | MADDUX_SAPOWE2_1 | MADDUX | SAPOWER | 4 |
| 2026 | March | SREAUVA8 | UVALDE_W_BATE1_1 | UVALDE | W_BATESV | 4 |
| 2026 | March | DGRSLO25 | 12020__B | BRTSW | JUNPR | 4 |
| 2026 | March | DFERWIR8 | 40T189_1 | FERGUS | WIRTZ | 4 |
| 2026 | March | XFTS89 | ALPINE_BRONCO1_1 | BRONCO | ALPINE | 4 |
| 2026 | March | DZORHAY5 | BERGHE_AT1L | BERGHE | BERGHE | 4 |
| 2026 | March | SWOLPAL8 | MIDW_OZONA1_1 | OZONA | MIDW | 4 |
| 2026 | March | DBBSRCH5 | 921__F | SHKSW | GAMMA | 4 |
| 2026 | March | DCAGOMI8 | 255T279_1 | PIPECR | MEDILA | 3 |
| 2026 | March | SKLNSAL5 | 271__A | KLNSW | SALSW | 3 |
| 2026 | March | DSCOTKW5 | 6474__B | SUNSW | RGRSW | 3 |
| 2026 | March | DGRMGRS8 | 6830__C | OLNEY | GRAHM | 3 |
| 2026 | March | DELMWWE8 | 1020__A | MCTYE | ELMOT | 3 |
| 2026 | March | DSALHUT5 | 203T260_1 | ELGISW | BUTLER | 3 |
| 2026 | March | SBRAPIN8 | GANSO_MAVERI1_1 | MAVERICK | GANSO | 3 |
| 2026 | March | BASE CASE | I_KALO | n/a | n/a | 3 |
| 2026 | March | SLCSTH25 | 505__B | FBRSW | THSES | 3 |
| 2026 | March | MHARNED5 | VERTRE_WESLAU1_1 | WESLAU | VERTREES | 3 |
| 2026 | March | DELMWWE8 | 1020__A | ELMOT | MCTYE | 3 |
| 2026 | March | DTPCTHS5 | 1220__G | RIESW | GRSBK | 3 |
| 2026 | March | DSALHUT5 | 270__A | KNBSW | TMPSW | 3 |
| 2026 | March | SBRAHAM8 | ESCOND_GANSO1_1 | ESCONDID | GANSO | 3 |
| 2026 | March | DFOAVLO5 | NLARSW_PILONC1_1 | NLARSW | PILONCIL | 3 |
| 2026 | March | DCAGCO58 | R0_FAIR_1 | R0 | FAIROA | 3 |
| 2026 | March | DTPCTHS5 | 1020__E | THSTP | MCTYE | 3 |
| 2026 | March | SBLURDH8 | 2270__B | MEXTP | ITALY | 3 |
| 2026 | March | DFOAVLO5 | ASHERT_CATARI1_1 | CATARINA | ASHERTON | 3 |
| 2026 | March | SL_4RAY8 | RAYBURN_69_2 | RAYBURN | RAYBURN | 3 |
| 2026 | March | DTPCTMP5 | TMPCR_MR1H | TMPCR | TMPCR | 3 |
| 2026 | March | DBUCRGP5 | TREADW_YELWJC1_1 | TREADWEL | YELWJCKT | 3 |
| 2026 | March | MWCSRAM5 | 12020__B | BRTSW | JUNPR | 3 |
| 2026 | March | SLAQLOB8 | BRUNI_69_1 | BRUNI | BRUNI | 3 |
| 2026 | March | SFBRTHS5 | 500__B | SAMSW | TCRSW | 3 |
| 2026 | March | SMDOPHR5 | 138_ALV_MNL_1 | ALVIN | MAINLAND | 2 |
| 2026 | March | DCAGCI58 | 584T584_1 | KENDAL | WELFAR | 2 |
| 2026 | March | SDUKNED8 | ADERHO_HEC1_1 | HEC | ADERHOLD | 2 |
| 2026 | March | SJUNYEL9 | HEXT_YELWJC1_1 | YELWJCKT | HEXT | 2 |
| 2026 | March | SCOMHA38 | MAXWEL_WHITIN1_1 | MAXWELL | WHITING | 2 |
| 2026 | March | SN_SLON5 | N_SHARPE_PS3 | N_SHARPE | N_SHARPE | 2 |
| 2026 | March | DSNG_TB5 | THWZEN71_A | ZEN | THW | 2 |
| 2026 | March | BASE CASE | ZAPSTR | n/a | n/a | 2 |
| 2026 | March | DSALHUT5 | 219T219_1 | NARUNA | EBABES | 2 |
| 2026 | March | XCMB58 | 661__A | STNVL | CPSES | 2 |
| 2026 | March | DDILCOT8 | DILLEYSW_XF1L | DILLEYSW | DILLEYSW | 2 |
| 2026 | March | MHAPSTX8 | MAXWEL_WHITIN1_1 | MAXWELL | WHITING | 2 |
| 2026 | March | DSALHUT5 | SNDSW_MR1H | SNDSW | SNDSW | 2 |
| 2026 | March | XST2E89 | 2270__B | MEXTP | ITALY | 2 |
| 2026 | March | SVENFTS5 | 35055__A | SAMSW | VENSW | 2 |
| 2026 | March | SVEAW_L5 | 6217__A | WLVSW | GAILS | 2 |
| 2026 | March | SWCPYT8 | BLESSI_PAVLOV1_1 | PAVLOV | BLESSING | 2 |
| 2026 | March | SLKAWFS8 | BOW_FMR1 | BOW | BOW | 2 |
| 2026 | March | SBRAHAM8 | GANSO_MAVERI1_1 | GANSO | MAVERICK | 2 |
| 2026 | March | BASE CASE | I_PASP | n/a | n/a | 2 |
| 2026 | March | SMV_PAR8 | RIOHND_ERIOHND_1 | MV_RIOHO | RIOHONDO | 2 |
| 2026 | March | DBIGSCH5 | SANTAR_WOLFCA1_1 | WOLFCAMP | SANTARIT | 2 |
| 2026 | March | SEUSWLT8 | 1520__C | ELKTN | TSLVL | 2 |
| 2026 | March | DTCRTHS5 | 35055__A | SAMSW | VENSW | 2 |
| 2026 | March | SNEWGLF8 | BLESSI_PAVLOV1_1 | PAVLOV | BLESSING | 2 |
| 2026 | March | SBRAPIN8 | ESCOND_GANSO1_1 | ESCONDID | GANSO | 2 |
| 2026 | March | BASE CASE | EUSTWLTN_RC_1 | WALTSSRC | EUSTSERC | 2 |
| 2026 | March | SSANPYT8 | LAN_CT_PAVLOV1_1 | LAN_CTY | PAVLOV | 2 |
| 2026 | March | DLOFOAV5 | NLARSW_PILONC1_1 | PILONCIL | NLARSW | 2 |
| 2026 | March | DNAVVEN5 | 20__A | RCHBR | BBSES | 2 |
| 2026 | March | DTHSFBR5 | 35065__A | FVLSW | FTSSW | 2 |
| 2026 | March | SLCSTH25 | 506__A | SAMSW | FBRSW | 2 |
| 2026 | March | DKLNRGP5 | 680__A | BRNSW | GOLDTH | 2 |
| 2026 | March | SESPK8 | DT_PK_91_A | DT | PK | 2 |
| 2026 | March | DSALHUT5 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 2 |
| 2026 | March | SBRAHAM8 | GANSO_MAVERI1_1 | MAVERICK | GANSO | 2 |
| 2026 | March | SNEWGLF8 | LAN_CT_PAVLOV1_1 | LAN_CTY | PAVLOV | 2 |
| 2026 | March | DCMNFUL8 | 660__B | MGPSW | ZEPHYR | 2 |
| 2026 | March | DBAKCED5 | CROSSO_PALOUS1_1 | PALOUSE | CROSSOVE | 2 |
| 2026 | March | SWCPYT8 | LAN_CT_PAVLOV1_1 | LAN_CTY | PAVLOV | 2 |
| 2026 | March | DCAGCI58 | R0_FAIR_1 | R0 | FAIROA | 2 |
| 2026 | March | DLONPK25 | 6375__A | GRSES | GRMES | 2 |
| 2026 | March | MWCSRAM5 | 6375__A | GRSES | GRMES | 2 |
| 2026 | March | SHAYZO25 | 6T227_1 | HAYSEN | ZORN | 2 |
| 2026 | March | DSTEXP12 | BLESSI_LOLITA1_1 | LOLITA | BLESSING | 2 |
| 2026 | March | SEUSWLT8 | BRTNWLTN_RC_1 | BRTNSSRC | WALTSSRC | 2 |
| 2026 | March | BASE CASE | EUSTWLTN_RC_1 | EUSTSERC | WALTSSRC | 2 |
| 2026 | March | DDMTMHO5 | FARMLAND_LONGD_1 | FARMLAND | W_LD_345 | 2 |
| 2026 | March | DBAKCED5 | HARGRO_TWINBU1_1 | TWINBU | HARGROVE | 2 |
| 2026 | March | DHONHON8 | HON_DEVI_1 | HONDOCK | DEVINESW | 2 |
| 2026 | March | SMOOPEA8 | NBA_S_BA_1 | S_BATES | NBATSVS | 2 |
| 2026 | March | SHLSWH28 | 2270__B | ITALY | MEXTP | 2 |
| 2026 | March | SHLSWH28 | 2270__B | MEXTP | ITALY | 2 |
| 2026 | March | SCLCGTN8 | 6635__G | MRVLY | ESTLD | 2 |
| 2026 | March | DLOFOAV5 | ASHERT_CATARI1_1 | ASHERTON | CATARINA | 2 |
| 2026 | March | DWO5_EU8 | BR_HOC09_A | BR | HOC | 2 |
| 2026 | March | DBUCRGP5 | CMNSW_MR1H | CMNSW | CMNSW | 2 |
| 2026 | March | SDOWMOO8 | DOWNIES_AX1H | DOWNIES | DOWNIES | 2 |
| 2026 | March | BASE CASE | EBONY_GENTIE_1 | EBNY_ESS | EBNY_ESS | 2 |
| 2026 | March | DAUSDUN8 | MCN_MCN_1 | MCNEIL_ | MCNEIL | 2 |
| 2026 | March | DBAKCED5 | LVOK_SANTIA1_1 | SANTIAGO | LVOK | 1 |
| 2026 | March | SCARLVO8 | MADDUX_SAPOWE2_1 | MADDUX | SAPOWER | 1 |
| 2026 | March | SFORYEL8 | MADDUX_SAPOWE2_1 | MADDUX | SAPOWER | 1 |
| 2026 | March | DWAP_OB5 | MDOPHR99_A | MDO | PHR | 1 |
| 2026 | March | SSHIMCC8 | NEWM_WAL_1 | NEWMAN | WALNUT1 | 1 |
| 2026 | March | DFRYBE58 | SEA_AAT1 | SEA | SEA | 1 |
| 2026 | March | DRNS_TB5 | THWZEN98_A | ZEN | THW | 1 |
| 2026 | March | SBONRIO5 | VERTRE_WESLAU1_1 | VERTREES | WESLAU | 1 |
| 2026 | March | STHSHLS8 | 1880__E | WCOLO | MRYAV | 1 |
| 2026 | March | SSUTF18 | 42T425_1 | NIXON | NIXOGV | 1 |
| 2026 | March | STCRTHS5 | 506__A | SAMSW | FBRSW | 1 |
| 2026 | March | DCDHTVW5 | 6200__D | SHRSW | PRKRW | 1 |
| 2026 | March | SDUKNED8 | ADERHO_ELSA1_1 | ADERHOLD | ELSA | 1 |
| 2026 | March | SBROALP9 | BELDTA_COCS1_A | BELDTAP | COCS | 1 |
| 2026 | March | SLGDSAP8 | CARVER_TINSLE1_1 | CARVER | TINSLEY | 1 |
| 2026 | March | SCMNCPS5 | CMNSW_MR1H | CMNSW | CMNSW | 1 |
| 2026 | March | SCOMHA38 | CTHR_TINSLE1_1 | CTHR | TINSLEY | 1 |
| 2026 | March | DFOWSMG5 | FWLRTN_TILDEN_1 | FOWLRTON | TILDEN | 1 |
| 2026 | March | DCC1DUKE | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 1 |
| 2026 | March | DHUTLIM5 | HUTTO_MR1H | HUTTO | HUTTO | 1 |
| 2026 | March | DNORCAS8 | LCRANE_RIOPEC1_1 | RIOPECOS | LCRANE | 1 |
| 2026 | March | XLO2N58 | LON_HI_SERDEV2_1 | LON_HILL | LON_HILL | 1 |
| 2026 | March | SGRIRAP5 | MELONC_SEADRF1_1 | SEADRFTC | MELONCRE | 1 |
| 2026 | March | SN_SAJO5 | PANTER_WESMER1_1 | WESMER | PANTERA | 1 |
| 2026 | March | SFURVAN8 | RAY_L_46_1 | RAYBURN | L_463S | 1 |
| 2026 | March | STANPAW5 | RAY_L_46_1 | L_463S | RAYBURN | 1 |
| 2026 | March | SW_BUVA8 | READIN_UVALDE1_1 | UVALDE | READING | 1 |
| 2026 | March | DBCETMP5 | SNDSW_MR1H | SNDSW | SNDSW | 1 |
| 2026 | March | BASE CASE | UVALDE_W_BATE1_1 | UVALDE | W_BATESV | 1 |
| 2026 | March | SN_SAJO5 | VERTRE_WESLAU1_1 | WESLAU | VERTREES | 1 |
| 2026 | March | STNPBC25 | 35045__A | SAMSW | FVLSW | 1 |
| 2026 | March | SSCLWF18 | 6840__A | ANARN | CRDSW | 1 |
| 2026 | March | SBENS_M8 | BENTS_FRTER_1B_1 | FRONTERA | S_MISSIN | 1 |
| 2026 | March | SOZNFRI9 | BIGLAK_PHBL_T1_1 | BIGLAKE | PHBL_TAP | 1 |
| 2026 | March | DFRIILL8 | CARVER_TINSLE1_1 | CARVER | TINSLEY | 1 |
| 2026 | March | DKOCNUE8 | CLARK__LON_HI1_1 | LON_HILL | CLARK_WD | 1 |
| 2026 | March | DWESNUE8 | CLARK__LON_HI1_1 | LON_HILL | CLARK_WD | 1 |
| 2026 | March | SBLACOF8 | EUSTWLTN_RC_1 | EUSTSERC | WALTSSRC | 1 |
| 2026 | March | DCPSES12 | FORTMA_YELWJC1_1 | FORTMA | YELWJCKT | 1 |
| 2026 | March | UMUSBES1 | GALOW_MUSTNGCK_1 | MUSTNGCK | GALOW | 1 |
| 2026 | March | SRAYRI28 | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 1 |
| 2026 | March | SILLFTL8 | HAMILT_MAXWEL1_1 | MAXWELL | HAMILTON | 1 |
| 2026 | March | MSUNESC8 | LAPRYO_UVALDE1_1 | UVALDE | LAPRYOR | 1 |
| 2026 | March | DDILPE89 | LIPTON_W_BATE1_1 | W_BATESV | LIPTON | 1 |
| 2026 | March | DBAKSOL5 | LYNX_TOMBST1_1 | LYNX | TOMBSTNE | 1 |
| 2026 | March | DLYTZOR5 | MCN_MCN_1 | MCNEIL_ | MCNEIL | 1 |
| 2026 | March | DMCEBUT8 | MERK_MKLT1_1 | MKLT | MERK | 1 |
| 2026 | March | SALAN_28 | N_SHARPE_PS3 | N_SHARPE | N_SHARPE | 1 |
| 2026 | March | SLGDSAP8 | PALOUS_WOLFCA1_1 | PALOUSE | WOLFCAMP | 1 |
| 2026 | March | SRVRSLK5 | SLKSW_AX1L | SLKSW | SLKSW | 1 |
| 2026 | March | DKLNRGP5 | TREADW_YELWJC1_1 | TREADWEL | YELWJCKT | 1 |
| 2026 | March | DBLW2JC5 | WAPWLY72_A | WLY | WAP | 1 |
| 2026 | March | DMCLPVN8 | 1020__D | LCSES | HAL | 1 |
| 2026 | March | SPEAX28 | BIG_FOOT_69A1 | BIG_FOOT | BIG_FOOT | 1 |
| 2026 | March | SPAWSAN5 | BLESSING_1382 | BLESSING | BLESSING | 1 |
| 2026 | March | DELMSAN5 | BLESSI_LOLITA1_1 | LOLITA | BLESSING | 1 |
| 2026 | March | SALAN_28 | CELANE_KLEBER1_1 | CELANEBI | KLEBERG | 1 |
| 2026 | March | DAUSLOS5 | CKT_3122_1 | FPPYD2 | HOLMAN | 1 |
| 2026 | March | BASE CASE | CRD_WCTSW_1 | CRD | WLDCATSW | 1 |
| 2026 | March | SWALNAA8 | FAI_COLL_1 | FAIRDALE | COLLEGE | 1 |
| 2026 | March | SBENRAI8 | FRONTE_PALMHR1_1 | FRONTERA | PALMHRTP | 1 |
| 2026 | March | SAEDAB8 | FW_WEB04_A | FW | WEB | 1 |
| 2026 | March | DYETRJU8 | HEXT_MASONS1_1 | HEXT | MASONSW | 1 |
| 2026 | March | XHHG58 | HHGT_T2H | HHGT | HHGT | 1 |
| 2026 | March | SCROPAL8 | MIDW_OZONA1_1 | OZONA | MIDW | 1 |
| 2026 | March | DSALHUT5 | TRIMR_KLNSW_1 | KLNSW | TRIMMIER | 1 |
| 2026 | March | BASE CASE | 208T480_1 | N_MERCED | NWESLACO | 1 |
| 2026 | March | MBONNED5 | 480T480_1 | HARLNSW | N_MERCED | 1 |
| 2026 | March | DCAGMC58 | 584T584_1 | KENDAL | WELFAR | 1 |
| 2026 | March | XCMB58 | 6253__B | CPSES | CMBSW | 1 |
| 2026 | March | SSNYCGR8 | 6695__B | SNYDR | AMOTP | 1 |
| 2026 | March | SRICGRS8 | 6840__A | ANARN | CRDSW | 1 |
| 2026 | March | SAJORI25 | ALAZAN_B_DAVI1_1 | ALAZAN | B_DAVIS | 1 |
| 2026 | March | DBIGSCH5 | MADDUX_SANTIA1_1 | SANTIAGO | MADDUX | 1 |
| 2026 | March | SCARFRI8 | MADDUX_SANTIA1_1 | SANTIAGO | MADDUX | 1 |
| 2026 | March | DLIMGIL5 | MCN_MCN_1 | MCNEIL_ | MCNEIL | 1 |
| 2026 | March | SES2FRI8 | MIDW_OZONA1_1 | OZONA | MIDW | 1 |
| 2026 | March | DBAKSOL5 | NEVILL_BAKESW_1 | BAKESW | NEVILLSW | 1 |
| 2026 | March | DBAKSOL5 | NEVILL_NORTMC_1 | NEVILLSW | NORTMC | 1 |
| 2026 | March | SDOWMOO8 | UVALDE_W_BATE1_1 | UVALDE | W_BATESV | 1 |
| 2026 | March | DVENFTS5 | 235__A | SGRSW | JEWET | 1 |
| 2026 | March | DNAVVEN5 | 30__A | RCHBR | BBSES | 1 |
| 2026 | March | DTHSFBR5 | 35045__A | SAMSW | FVLSW | 1 |
| 2026 | March | DSCOTKW5 | 6240__C | SACRC | DPCRK | 1 |
| 2026 | March | XCMB58 | 6253__B | CMBSW | CPSES | 1 |
| 2026 | March | DCDHTVW5 | 6325__D | LTVSB | GRDPR | 1 |
| 2026 | March | DMOSME25 | 6420__A | HLTSW | TBCSW | 1 |
| 2026 | March | XCMB58 | 651__B | CMNSW | CMNTP | 1 |
| 2026 | March | DLC_PG_8 | ARROZ_EL_CAM1_1 | ARROZ | EL_CAMPO | 1 |
| 2026 | March | SMDOOAS5 | BG_GT_37_A | GT | BG | 1 |
| 2026 | March | DFOWSMG5 | COTULL_REVEIL1_1 | REVEILLE | COTULLA | 1 |
| 2026 | March | SF3PEA8 | DEV_NATA_1 | DEVINESW | NATALISS | 1 |
| 2026 | March | DCOTWR25 | FARMLAND_LONGD_1 | FARMLAND | W_LD_345 | 1 |
| 2026 | March | XLOC89 | LULING_AT1 | LULING | LULING | 1 |
| 2026 | March | BASE CASE | 138_FTS_LNC_1 | FTST | LEONCRK | 1 |
| 2026 | March | STNFTAR8 | 138_FTS_LNC_1 | LEONCRK | FTST | 1 |
| 2026 | March | DSALHUT5 | 13T219_1 | EBABES | BURNET | 1 |
| 2026 | March | SMYPHLS9 | 2270__B | MEXTP | ITALY | 1 |
| 2026 | March | DTHSFBR5 | 35055__A | SAMSW | VENSW | 1 |
| 2026 | March | DSAMFBR5 | 35065__A | FVLSW | FTSSW | 1 |
| 2026 | March | SFMRRYS5 | 400__A | FMRVL | RYSSW | 1 |
| 2026 | March | SSAMFBR5 | 500__B | SAMSW | TCRSW | 1 |
| 2026 | March | MHCKRAM5 | 6375__A | GRSES | GRMES | 1 |
| 2026 | March | DFRYTM58 | 640__A | BLTON | NLNVL | 1 |
| 2026 | March | DNOETWL5 | 6965__A | LNGSW | PRLSW | 1 |
| 2026 | March | SCARFRI8 | ATSO_OZNC1_1 | ATSO | OZNC | 1 |
| 2026 | March | SHONF38 | BIG_FOOT_69A1 | BIG_FOOT | BIG_FOOT | 1 |
| 2026 | March | MWCSRAM5 | BOW_FMR1 | BOW | BOW | 1 |
| 2026 | March | SAJORI25 | CELANE_KLEBER1_1 | CELANEBI | KLEBERG | 1 |
| 2026 | March | SALAN_28 | CLARK__LON_HI1_1 | LON_HILL | CLARK_WD | 1 |
| 2026 | March | SLGDSAP8 | FDR_OZNC_1 | FRIEND_R | OZNC | 1 |
| 2026 | March | SHONMOO8 | FER_DOWNI_1 | DOWNIES | FERRISW | 1 |
| 2026 | March | SMA2SAP8 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 1 |
| 2026 | March | DRESMCL8 | I_DUPS_RESNIK1_1 | I_DUPSW | RESNIK | 1 |
| 2026 | March | DDILPE89 | LAPRYO_UVALDE1_1 | UVALDE | LAPRYOR | 1 |
| 2026 | March | SLONLON8 | LON_HI_SERDEV4_1 | LON_HILL | LON_HILL | 1 |
| 2026 | March | SCARFRI8 | MADDUX_SAPOWE2_1 | MADDUX | SAPOWER | 1 |
| 2026 | March | SFTLMES8 | MIDW_OZONA1_1 | OZONA | MIDW | 1 |
| 2026 | March | DBCETMP5 | SNDSW_MR1L | SNDSW | SNDSW | 1 |
| 2026 | March | DFOWSMG5 | TILDEN_GEOWEST_1 | TILDEN | GEOWEST | 1 |
| 2026 | March | SDIMBEV8 | UVALDE_W_BATE1_1 | W_BATESV | UVALDE | 1 |
| 2026 | March | SFURRAY8 | VAN_VNDB_1 | VANBLTSS | VANBLT69 | 1 |
| 2026 | March | DTRSRCH5 | 1210__B | HUBRD | HAN1 | 1 |
| 2026 | March | SLISBAT8 | GARZA_XF1H | GARZA | GARZA | 1 |
| 2026 | March | SRAYHAR8 | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 1 |
| 2026 | March | DBIGSCH5 | LAKENA_SAMATH1_1 | LAKENASW | SAMATHIS | 1 |
| 2026 | March | SPAWSAN5 | LOOP_VICTORIA_1 | VICTORIA | L_463S | 1 |
| 2026 | March | DYELHE89 | MADDUX_SAPOWE2_1 | MADDUX | SAPOWER | 1 |
| 2026 | March | SMCEABS8 | MKLT_TRNT1_1 | TRNT | MKLT | 1 |
| 2026 | March | SHONF38 | NAT_PRSN_1 | NATALISS | PEARSON | 1 |
| 2026 | March | BASE CASE | N_SHARPE_PS3 | N_SHARPE | N_SHARPE | 1 |
| 2026 | March | SBIGOR45 | TREADW_YELWJC1_1 | TREADWEL | YELWJCKT | 1 |
| 2026 | March | MRNKDHM5 | W_DENT_T2H | W_DENT | W_DENT | 1 |
| 2026 | March | DMCLPVN8 | 1020__D | HAL | LCSES | 1 |
| 2026 | March | DCAGCO58 | 122T122_1 | COMFOR | RAYBAR | 1 |
| 2026 | March | STHSHLS8 | 1880__E | MRYAV | WCOLO | 1 |
| 2026 | March | DCPSES12 | 35055__A | SAMSW | VENSW | 1 |
| 2026 | March | DLEGOUT5 | 50__A | BBSES | JEWET | 1 |
| 2026 | March | MDHMLWS5 | 587__A | ARGYL | LWSVH | 1 |
| 2026 | March | DCPSST58 | 651__B | CMNSW | CMNTP | 1 |
| 2026 | March | DCAGCO58 | 656T656_1 | KENDAL | BERGHE | 1 |
| 2026 | March | SBRKPOS8 | 6635__G | ESTLD | MRVLY | 1 |
| 2026 | March | SGRMGRS8 | 6830__C | OLNEY | GRAHM | 1 |
| 2026 | March | DSTPREF5 | BLESSING_1382 | BLESSING | BLESSING | 1 |
| 2026 | March | DBNTTOW9 | BOW_FMR1 | BOW | BOW | 1 |
| 2026 | March | DKOCHE89 | CLARK__LON_HI1_1 | LON_HILL | CLARK_WD | 1 |
| 2026 | March | SPAWSAN5 | F2_11_1 | KENEDSW | F2 | 1 |
| 2026 | March | SN_SAJO5 | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 1 |
| 2026 | March | MHARNED5 | HARLNS_OLEAND1_1 | OLEANDER | HARLNSW | 1 |
| 2026 | March | SSTEMVH8 | L_MILP_STEWAR1_1 | STEWART | L_MILPAS | 1 |
| 2026 | March | SES2FRI8 | MADDUX_SANTIA1_1 | SANTIAGO | MADDUX | 1 |
| 2026 | March | BASE CASE | MADDUX_SAPOWE2_1 | MADDUX | SAPOWER | 1 |
| 2026 | March | DKG_NB_5 | MDOPHR99_A | MDO | PHR | 1 |
| 2026 | March | MBONMAN5 | PANTER_WESMER1_1 | WESMER | PANTERA | 1 |
| 2026 | March | SWCOASH8 | UVALDE_W_BATE1_1 | UVALDE | W_BATESV | 1 |
| 2026 | March | XWH2T58 | WHTNY_HT1L | WHTNY | WHTNY | 1 |
| 2026 | March | BASE CASE | 207T480_1 | NWESLACO | WESLACO | 1 |
| 2026 | March | DVENFTS5 | 2270__B | MEXTP | ITALY | 1 |

Current Wind Generation Record: 28,751 MW on 03/14/2025 at 20:00 | Current Wind Penetration Record: 69.15% on 04/10/2022 at 01:43

Current Solar Generation Record: 33,477 MW on 03/21/2026 at 11:20 | Current Solar Penetration Record: 67.27% on 03/21/2026 at 10:52