| NPRR Number | 1214 | NPRR Title | Reliability Deployment Price Adder Fix to Provide Locational Price Signals, Reduce Uplift and Risk |
| --- | --- | --- | --- |
| Date of Decision |  | June 10, 2026 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 3.2.5, Publication of Resource and Load Information 3.5.2.1, North 345 kV Hub (North 345) 3.5.2.2, South 345 kV Hub (South 345) 3.5.2.3, Houston 345 kV Hub (Houston 345) 3.5.2.4, West 345 kV Hub (West 345) 3.5.2.5, Panhandle 345 kV Hub (Pan 345) 3.5.2.6, Lower Rio Grande Valley Hub (LRGV 138/345) 3.5.2.7, ERCOT Bus Average 345 kV Hub (ERCOT 345 Bus) 4.4.11, Day-Ahead and Real-Time System-Wide Offer Caps 5.5.2, Reliability Unit Commitment (RUC) Process 5.7.1.3, Revenue Less Cost Above LSL During RUC-Committed Hours 5.7.1.4, Revenue Less Cost During QSE Clawback Intervals 6.3.2, Activities for Real-Time Operations 6.5.7.3, Security Constrained Economic Dispatch 6.5.7.3.1, Determination of Real-Time On-Line Reliability Deployment Price Adder 6.5.9.2, Failure of the SCED Process 6.6.1.1, Real-Time Settlement Point Price for a Resource Node 6.6.1.2, Real-Time Settlement Point Price for a Load Zone 6.6.1.6, Real-Time Market Clearing Prices for Ancillary Services 6.6.1.7, Real-Time Reliability Deployment Prices for Ancillary Services (delete) 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node 6.6.3.6, Real-Time High Dispatch Limit Override Energy Payment 6.6.3.8, Real-Time Payment or Charge for Energy from a Settlement Only Distribution Generator (SODG) or a Settlement Only Transmission Generator (SOTG) 6.6.6.3, RMR Adjustment Charge 6.6.6.10, MRA Variable Payment for Deployment 6.9, Reliability Deployment Indifference Payment and Allocation (new) 6.9.1, Reliability Deployment Indifference Payment (new) 6.9.2, Reliability Deployment Indifference Allocation (new) 9.5.3, Real-Time Market Settlement Charge Types |  |
| Related Documents Requiring Revision/Related Revision Requests |  | ERCOT Nodal ICCP Communication Handbook |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) revises the Real-Time On-Line Reliability Deployment Price Adder (RTRDPA) to: - Send appropriate locational price signals to avoid counterproductive Load and Resource responses to RTRDPA price signals under Real-Time Co-optimization (RTC); - Limit Resource payment to the actual “indifference payment” (consistent with its definition), thereby reducing associated uplift by eliminating the future need for RTRDPA payments to Resources that exacerbate constraints and eliminating payments to available capacity not requiring an indifference payment; - Eliminate any future need for the Ancillary Service Imbalance Payments or Charges (ASIP/C) type of indifference payment associated with RTRDPA, thereby reducing the risk associated with providing Ancillary Services; - Provide Resources an indifference payment under RTC to eliminate the potentially large incentive to ignore Base Point instructions that would likely cause serious reliability issues; and - Provide a stronger locational price signal around Resources committed by the Reliability Unit Commitment (RUC) process or other reliability actions for congestion, thereby reducing RUC Make-Whole Payment-related charges and uplifts and appropriately compensating impacted Qualified Scheduling Entities (QSEs). |  |
| Reason for Revision |  | Strategic Plan Objective 1 – Be an industry leader for grid reliability and resilience Strategic Plan Objective 2 - Enhance the ERCOT region’s economic competitiveness with respect to trends in wholesale power rates and retail electricity prices to consumers Strategic Plan Objective 3 - Advance ERCOT, Inc. as an independent leading industry expert and an employer of choice by fostering innovation, investing in our people, and emphasizing the importance of our mission Administrative Regulatory requirements ERCOT Board and/or PUCT Directive (please select ONLY ONE – if more than one apply, please select the ONE that is most relevant) |  |
| Justification of Reason for Revision and Market Impacts |  | This NPRR fixes RTRDPA implementation by using the locational energy prices and Ancillary Service MCPCs from the SCED Pricing Run. The associated indifference payment is supposed to pay Resources to keep them indifferent from being dispatched at a Base Point from the SCED Dispatch Run while being settled at the prices from the SCED Pricing Run – thereby eliminating any incentive to chase prices and ignore Base Point instructions. Unfortunately, the current implementation of RTRDPA treats RTRDPA as an operating reserve-related price adder, effectively making it the same as the Operating Reserve Demand Curve (ORDC) price adder. Its current application is inconsistent with its definition and results in counter-productive outcomes including compensation to capacity exacerbating a constraint and compensation to capacity with Energy Offer Curves above the resulting energy prices after adding the RTRDPA (both part of RTRDPA-related ASIP Settlement). This inconsistent treatment of RTRDPA resulted in an unnecessary ASIP-related uplift to Load and Direct Current Tie (DC Tie) exports of $10 million on a single Operating Day (4/22/2022) and many tens of millions of dollars in ASIC-related charges to Resources providing Ancillary Services deployed during Winter Storm Uri in February 2021. Without the changes proposed in this NPRR which would implement using the SCED Pricing Run locational price for each Settlement Point rather than the current system-wide price adder, the same issue may arise if the lack of indifference payment under RTC is similarly addressed for post-RTC system-wide RTRDPA. Drs. Hogan and Pope, in their paper “Priorities for the Evolution of an Energy-Only Electricity Market Design in ERCOT” (2017) pointed out that “The Reliability Deployment Price Adder implemented in August 2014 does not attribute local scarcity value to capacity deployments occurring to relieve local reliability problems” and “It does not confer value to reliability actions causing changes in relative locational prices within ERCOT, as measured by changes in the congestion components of LMPs in different locations. A RUC commitment and other reliability deployments may decrease prices in a local area, due to relieving a transmission constraint, for example, yet have little or no effect on prices outside of this local area, so that the estimated change in the system reference price will often be close to zero.” This NPRR elegantly addresses this issue using ERCOT systems that are already in use – the Security-Constrained Economic Dispatch (SCED) Pricing Run. To appropriately reflect the impact of reliability deployments on energy prices, ERCOT reliability actions taken to address localized issues must, by necessity, be reflected in the appropriate locational prices. Otherwise, the post-RTC system-wide RTRDPA for a local issue provides inefficient and inappropriate price signals throughout the market. For example, a RUC commitment required in East Texas due to congestion limiting supply from West Texas could result in a high RTRDPA that is added to prices throughout the system, potentially causing thousands of MW of Large Flexible Loads (LFLs) in West Texas to unnecessarily curtail their consumption in response to this price signal. This counterproductive price response by LFLs would also result in unnecessary curtailment of Wind and Solar generators in West Texas while sending the wrong price signal for investment in West Texas. This is a very inefficient outcome for the market that needs to be addressed urgently due to the dramatic ongoing increase of LFLs in ERCOT. Moreover, locational prices send better congestion price signals throughout the system. Another benefit from more accurate locational prices will be appropriate compensation for QSEs that are adversely impacted by the reliability action. For example, say a QSE purchases energy in the Day-Ahead Market (DAM) at $100/MWh for a DC Tie export. ERCOT curtails the DC Tie export in real-time for local congestion issues, a reliability action that reduces the Locational Marginal Price (LMP) at the corresponding DC Tie Load Zone to $40/MWh. If the resulting system-wide RTRDPA at that time is $5/MWh, then the QSE whose exports were curtailed lost $55/MWh on energy it purchased in the DAM on top of bilateral losses due to the curtailed export. However, the proposed locational price calculated by the SCED Pricing Run for the DC Tie Load Zone could be $1,000/MWh since exports were curtailed. Then, the QSE receives a net payment of $900/MWh for its DAM purchase that the QSE can use to offset its bilateral losses. Even though the current RTRDPA implementation pays Resources much more than the amount required as indifference payment through the ASIP, RTC will eliminate the ASIP. Thus, there will be no indifference payment when SCED Pricing Run is active under RTC. This can be thousands of dollars per MWh during scarcity and thus, absent any indifference payment, Resources will have a strong incentive to generate above their Base Points during scarcity events. This NPRR fixes this misaligned incentive and associated reliability concerns by applying the indifference payment once RTC is implemented. The current 60-minute ramp relaxation in calculating dispatch limits used in the SCED Pricing Run results in meaningless price adders in many cases. Given the many fast ramp rate Resources, this NPRR has the SCED Pricing Run use the same dispatch limits as the SCED Dispatch Run. |  |
| PRS Decision |  | On 1/11/24, PRS voted unanimously to table NPRR1214 and refer the issue to WMS. All Market Segments participated in the vote. On 3/12/25, PRS voted unanimously to recommend approval of NPRR1214 as amended by the 1/28/25 ERCOT comments. All Market Segments participated in the vote. On 4/9/25, PRS voted unanimously to table NPRR1214. All Market Segments participated in the vote. On 5/14/25, PRS voted unanimously to table NPRR1214. All Market Segments participated in the vote. On 6/10/26, PRS voted unanimously to endorse the 5/14/25 PRS Report as amended by the 6/5/26 ERCOT comments for NPRR1214; and to bring the Revised Impact Analysis back to PRS. All Market Segments participated in the vote. |  |
| Summary of PRS Discussion |  | On 1/11/24, the sponsor provided an overview of NPRR1214. Participants requested additional review of NPRR1214 by WMS. On 3/12/25, the sponsor provided an overview of NPRR1214, and participants noted the WMS endorsement of the 1/28/25 ERCOT comments. On 4/9/25, participants noted the 3/31/25 ERCOT comments for an alternative schedule for the Impact Analysis. On 5/14/25, ERCOT Staff reviewed the 5/13/25 Impact Analysis details and requested additional time to develop formal comments to correct and clarify NPRR1214 language. On 6/10/26, participants reviewed the 6/5/26 ERCOT comments, the need for a revised Impact Analysis to address the alternative approach within, and the procedural timeline for NPRR1214 approval. |  |

| Opinions |  |
| --- | --- |
| Credit Review | ERCOT Credit Staff and the Credit Finance Sub Group (CFSG) have reviewed NPRR1214 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |
| Independent Market Monitor Opinion | To be determined |
| ERCOT Opinion | To be determined |
| ERCOT Market Impact Statement | To be determined |

| Sponsor |  |
| --- | --- |
| Name | Shams Siddiqi, Seth Cochran, Don Blackburn, Michael Pohlod |
| E-mail Address | shams@crescentpower.net, cochran@dc-energy.com, dblackburn@huntenergynetwork.com, mpohlod@voltus.co |
| Company | Rainbow Energy Marketing Corporation, DC Energy, Hunt Energy Network, Voltus, Inc. (“Joint Sponsors”) |
| Phone Number | 512-619-3532, 512-971-8767, 214-762-6159, 587-577-9994 |
| Cell Number | 512-619-3532, 512-971-8767, 214-762-6159, 587-577-9994 |
| Market Segment | Independent Power Marketer (IPM) and Consumer |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | Cory.phillips@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| WMS 020824 | Requested PRS continue to table NPRR1214 for further review by the Congestion Management Working Group (CMWG) |
| ERCOT 080924 | Raised concerns with NPRR1214 as submitted, and recommended revisiting this issue after implementation of RTC |
| Joint Sponsors 110424 | Provided additional revisions in response to the 8/9/24 ERCOT comments |
| ERCOT 012825 | Provided additional revisions to the 11/4/24 Joint Sponsors comments clarifying the cover page and including additional sections of Protocols not originally included but are needed to support the concept proposed by NPRR1214 |
| WMS 030625 | Endorsed NPRR1214 as amended by the 1/28/25 ERCOT comments |
| ERCOT 033125 | Proposed an alternative schedule for completion of the Impact Analysis for NPRR1214 prior to the May 14, 2025 PRS meeting |
| Vitol 052826 | Expressed support for NPRR1214 |
| ERCOT 052926 | Proposed extensive revisions to NPRR1214 to streamline Protocol language while maintaining the original intent of NPRR1214 |
| ERCOT 060526 | Proposed additional clarifying edits to the 5/29/26 ERCOT comments |

Market Rules Notes

Please note the baseline Protocol language in the following sections has been updated to reflect the incorporation of the following NPRRs into the Protocols:

- NPRR1007, RTC – NP 3: Management Activities for the ERCOT System (unboxed 12/5/25)

  - Section 3.5.2.1

  - Section 3.5.2.2

  - Section 3.5.2.3

  - Section 3.5.2.4

  - Section 3.5.2.5

  - Section 3.5.2.7

  - Section 6.6.1.1

  - Section 6.6.1.2

- NPRR1010, RTC – NP 6: Adjustment Period and Real-Time Operations (unboxed 12/5/25)

  - Section 6.5.7.3.1

  - Section 6.6.3.1

  - Section 6.6.3.6

  - Section 6.6.3.8

  - Section 6.7.5

  - Section 6.7.6

- NPRR1012, RTC – NP 9: Settlement and Billing (unboxed 12/5/25)

  - Section 9.5.3

- NPRR1014, BESTF-4 Energy Storage Resource Single Model (unboxed 12/5/25)

  - Section 6.5.7.3.1

  - Section 6.6.3.1

  - Section 9.5.3

- NPRR1092, Reduce RUC Offer Floor and Limit RUC Opt-Out Provision (unboxed 1/26/24)

  - Section 6.7.5

- NPRR1131, Controllable Load Resource Participation in Non-Spin (unboxed 8/23/24)

  - Section 6.7.5

- NPRR1149, Implementation of Systematic Ancillary Service Failed Quantity Charges (unboxed 6/28/24)

  - Section 6.7.5

- NPRR1188, Implement Nodal Dispatch and Energy Settlement for Controllable Load Resources (incorporated 12/1/24)

  - Section 6.5.7.3.1

- NPRR1190, High Dispatch Limit Override Provision for Increased Load Serving Entity Costs (incorporated 6/1/25)

  - Section 6.6.3.6

- NPRR1229, Real-Time Constraint Management Plan Cost Recover Payment (incorporated 8/1/25)

  - Section 9.5.3

- NPRR1238, Voluntary Registration of Loads with Curtailable Load Capabilities (incorporated 8/1/25)

  - Section 6.5.7.3.1

- NPRR1245, Additional Clarifying Revisions to Real-Time Co-Optimization (unboxed 12/5/25)

  - Section 6.5.7.3.1

- NPRR1246, Energy Storage Resource Terminology Alignment for the Single-Model Era (unboxed 12/5/25)

  - Section 6.6.3.6

- NPRR1311, Correction to Real-Time Reliability Deployment Price Adders for Ancillary Services under Load Shed for RTC+B (incorporated 4/1/26)

  - Section 6.5.7.3.1

- NPRR1323, Correction to Inadvertent Removal of Real-Time MCPC Capping for NPRR1290 Phase 2 (incorporated 6/1/26)

  - Section 6.5.7.3

Please note that the following NPRR(s) also propose revisions to the following section(s):

- NPRR1296, Residential Demand Response Program

  - Section 9.5.3

- NPRR1309, Board Priority - Dispatchable Reliability Reserve Service Ancillary Service

  - Section 6.5.7.3.1

- NPRR1310, Dispatchable Reliability Reserve Service Plus Energy Storage Resource Participation and Release Factor

  - Section 6.5.7.3.1

- NPRR1328, Establishment of Generation Firming Program

  - Section 9.5.3

Proposed Protocol Language Revision

## 2.1	DEFINITIONS

## SCED Dispatch Run

The two step Security Constrained Economic Dispatch (SCED) process described in Section 6.5.7.3, Security Constrained Economic Dispatch, which does not consider the impact of reliability deployments.

## SCED Pricing Run

The two step Security Constrained Economic Dispatch (SCED) process described in Section 6.5.7.3, Security Constrained Economic Dispatch, with inputs modified to consider the impact of reliability deployments described in paragraph (1) of Section 6.5.7.3.1, SCED Pricing Run.

### 3.2.5	Publication of Resource and Load Information

(1)	Two days after the applicable Operating Day, ERCOT shall post on the ERCOT website for the ERCOT System and, if applicable, for each Disclosure Area, the information derived from each execution of SCED.  The Disclosure Area is the 2003 ERCOT CMZs.  Posting requirements will be applicable to Generation Resources, ESRs, and Controllable Load Resources (CLRs) physically located in the defined Disclosure Area.  This information shall not be posted if the posting of the information would reveal any individual Market Participant’s Protected Information.  The information posted by ERCOT shall include:

(a)	An aggregate energy supply curve based on non-IRR Generation Resources with Energy Offer Curves that are available to SCED.  The energy supply curves will be calculated beginning at the sum of the Low Sustained Limits (LSLs) and ending at the sum of the HSLs for non-IRR Generation Resources with Energy Offer Curves, with the dispatch for each Generation Resource constrained between the Generation Resource’s LSL and HSL.  The result will represent the ERCOT System energy supply curve economic dispatch of the non-IRR Generation Resources with Energy Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(b)	An aggregate energy supply curve based on Wind-powered Generation Resources (WGRs) with Energy Offer Curves that are available to SCED.  The energy supply curves will be calculated beginning at the sum of the LSLs and ending at the sum of the HSLs for WGRs with Energy Offer Curves, with the dispatch for each WGR constrained between the WGR’s LSL and HSL.  The result will represent the ERCOT System energy supply curve economic dispatch of the WGRs with Energy Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(c)	An aggregate energy supply curve based on PhotoVoltaic Generation Resources (PVGRs) with Energy Offer Curves that are available to SCED.  The energy supply curves will be calculated beginning at the sum of the LSLs and ending at the sum of the HSLs for PVGRs with Energy Offer Curves, with the dispatch for each PVGR constrained between the PVGR’s LSL and HSL.  The result will represent the ERCOT System energy supply curve economic dispatch of the PVGRs with Energy Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(d)	An aggregated energy supply and demand curve based on Energy Bid/Offer Curves that are available to SCED.  The curves will be calculated beginning at the sum of the LSLs and ending at the sum of the HSLs for the Energy Bid/Offer Curves, with the dispatch for each Resource constrained between the Resource’s LSL and HSL.  The result will represent the ERCOT System energy supply and demand curve economic dispatch of the ESRs with Energy Bid/Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(e)	The sum of LSLs, sum of Output Schedules, and sum of HSLs for Generation Resources without Energy Offer Curves and ESRs without Energy Bid/Offer Curves;

(f)	The sum of the Base Points of non-IRR Generation Resources with Energy Offer Curves, sum of the Base Points of WGRs with Energy Offer Curves, sum of the Base Points of PVGRs with Energy Offer Curves, sum of the Base Points of ESRs with Energy Bid/Offer Curves, and the sum of the Base Points of all remaining Resources dispatched in SCED;

(g)	The sum of the telemetered Generation Resource net output used in SCED;

(h)	An aggregate energy Demand curve based on the Real-Time Market (RTM) Energy Bid curves available to SCED.  The energy Demand curve will be calculated beginning at the sum of the Low Power Consumptions (LPCs) and ending at the sum of the Maximum Power Consumptions (MPCs), with the dispatch for each CLR constrained between the CLR’s LPC and MPC.  The result will represent the ERCOT System Demand response capability available to SCED of the CLRs with RTM Energy Bids at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

[NPRR1188: Replace paragraph (h) above with the following upon system implementation:] (h) An aggregate energy Demand curve based on the Energy Bid Curves available to SCED. The energy Demand curve will be calculated beginning at the sum of the Low Power Consumptions (LPCs) and ending at the sum of the Maximum Power Consumptions (MPCs), with the dispatch for each CLR constrained between the CLR’s LPC and MPC. The result will represent the ERCOT System Demand response capability available to SCED of the CLRs with Energy Bid Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(i)	The aggregate Ancillary Service Offers (prices and quantities) in the RTM for each type of Ancillary Service regardless of a Resource’s On-Line or Off-Line status.  For RRS, ERCOT shall separately post aggregated offers from Resources providing Primary Frequency Response, Fast Frequency Response (FFR), and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated offers from Resources that are SCED-dispatchable and those that are manually dispatched.  Linked Ancillary Service Offers will be included as non-linked Ancillary Service Offers;

(j)	The sum of the Base Points of ESRs in discharge mode; and

(k)	The sum of the Base Points of ESRs in charge mode.

(2)	Two days after the applicable Operating Day, ERCOT shall post on the ERCOT website for the ERCOT System the following information derived from each execution of SCED:

(a)	The actual ERCOT Load as determined by subtracting the DC Tie Resource actual telemetry from the sum of the telemetered Generation Resource net output as used in SCED.

(3)	Two days after the applicable Operating Day, ERCOT shall post on the ERCOT website the following information for the ERCOT System and, if applicable, for each Disclosure Area from the DAM for each hourly Settlement Interval:

(a)	An aggregate energy supply curve based on all energy offers that are available to the DAM, including the offer portion of Energy Bid/Offer Curves submitted for ESRs, not taking into consideration Resource Startup Offer or Minimum-Energy Offer or any physical limitations of the ERCOT System.  The result will represent the energy supply curve at various pricing points for energy offers available in the DAM;

(b)	Aggregate minimum energy supply curves based on all Minimum-Energy Offers that are available to the DAM;

(c)	An aggregate energy Demand curve based on the DAM Energy Bid curves available to the DAM, not taking into consideration any physical limitations of the ERCOT System;

[NPRR1188: Replace paragraph (c) above with the following upon system implementation:] (c) An aggregate energy Demand curve based on the DAM Energy Bids and Energy Bid Curves from CLRs and including the bid portion of Energy Bid/Offer Curves available to the DAM, not taking into consideration any physical limitations of the ERCOT System;

(d)	The aggregate amount of cleared energy bids and offers including cleared Minimum-Energy Offer quantities;

(e)	The aggregate Ancillary Service Offers (prices and quantities) in the DAM for each type of Ancillary Service regardless of a Resource’s On-Line or Off-Line status and including Ancillary Service Only Offers.  For RRS, ERCOT shall separately post aggregated offers from Resources providing Primary Frequency Response (including Ancillary Service Only Offers), FFR, and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated offers from Resources that are SCED-dispatchable (including Ancillary Service Only Offers) and those that are manually dispatched.  Linked Ancillary Service Offers will be included as non-linked Ancillary Service Offers;

(f)	The aggregate Self-Arranged Ancillary Service Quantity for each type of service by hour.  For RRS, ERCOT shall separately post aggregated Self-Arranged Ancillary Service Quantities from Resources providing Primary Frequency Response, FFR, and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated Self-Arranged Ancillary Service Quantities from Resources that are SCED-dispatchable and those that are manually dispatched;

(g)	The aggregate amount of cleared Resource-Specific Ancillary Service Offers and Ancillary Service Only Offers.  For RRS, ERCOT shall separately post aggregated Ancillary Service Offers from Resources providing Primary Frequency Response (including Ancillary Service Only Offers), FFR, and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated Ancillary Service Offers from Resources that are SCED-dispatchable (including Ancillary Service Only Offers) and those that are manually dispatched; and

(h)	The aggregate Point-to-Point (PTP) Obligation bids (not-to-exceed price and quantities) for the ERCOT System and the aggregate PTP Obligation bids that sink in the Disclosure Area for each Disclosure Area.

(4)	ERCOT shall post on the ERCOT website the following information for each Resource for each execution of SCED 60 days prior to the current Operating Day:

(a)	From the SCED Dispatch Run:

(i)	The Generation Resource name and the Generation Resource’s Energy Offer Curve (prices and quantities):

(A)	As submitted;

(B)	As submitted and extended (or truncated) with proxy Energy Offer Curve logic by ERCOT to fit to the operational HSL and LSL values that are available for dispatch by SCED; and

(C)	As mitigated and extended for use in SCED;

(ii) 	The Resource name and the Resource’s Ancillary Service Offer Curve (prices and quantities) for each type of Ancillary Service:

(A)	As submitted; and

(B)	As submitted and extended with proxy Ancillary Service Offer Curve logic by ERCOT.

(iii)	The Load Resource name and the Load Resource’s bid to buy (prices and quantities);

[NPRR1188: Replace paragraph (iii) above with the following upon system implementation:] (iii) The Load Resource name and the Load Resource’s Energy Bid Curve (prices and quantities);

(iv)	The Generation Resource name and the Generation Resource’s Output Schedule;

(v)	The Generation Resource name and actual metered Generation Resource net output;

(vi)	The self-arranged Ancillary Service by service for each QSE;

(vi)	The following Generation Resource data using a snapshot from each execution of SCED:

(A)	The Generation Resource name;

(B)	The Generation Resource status;

(C)	The Generation Resource HSL, LSL, High Dispatch Limit (HDL), and Low Dispatch Limit (LDL);

(D)	The Generation Resource Base Point from SCED;

(E)	The telemetered Generation Resource net output used in SCED;

(F)	The Ancillary Service Resource awards for each Ancillary Service;

(G)	The Generation Resource Startup Cost and minimum energy cost used in the Reliability Unit Commitment (RUC);

(H) 	The telemetered Normal Ramp Rates;

(I) 	The telemetered Ancillary Service capabilities; and

(vii)	The following Load Resource data using a snapshot from each execution of SCED:

(A)	The Load Resource name;

(B)	The Load Resource status;

(C)	The MPC for a Load Resource;

(D)	The LPC for a Load Resource;

(E)	The Load Resource HDL and LDL, for a CLR that has a Resource Status of ONL;

(F)	The Load Resource Base Point from SCED, for a CLR that has a Resource Status of ONL;

(G)	The telemetered real power consumption;

(H)	The Ancillary Service Resource awards for each Ancillary Service;

(I)	The telemetered self-provided Ancillary Service amount for each Ancillary Service;

(J)	The telemetered Normal Ramp Rates;

(K) 	The telemetered Ancillary Service capabilities; and

(vii)	The ESR name and the ESR’s Energy Bid/Offer Curve (prices and quantities):

(A)	As submitted; and

(B)	As submitted and extended with proxy Energy Offer Curve logic by ERCOT to fit to the operational HSL and LSL values that are available for dispatch by SCED;

(ix)	The following ESR data using a snapshot from each execution of SCED:

(A)	The ESR name;

(B)	The ESR status;

(C)	The ESR HSL, LSL, HDL, and LDL;

(D)	The ESR Base Point from SCED;

(E)	The telemetered ESR net output used in SCED;

(F)	The Ancillary Service Resource awards for each Ancillary Service;

(G) 	The telemetered Normal Ramp Rates;

(H) 	The telemetered Ancillary Service capabilities;

(I)	The telemetered State of Charge in MWh;

(J)	The telemetered Minimum State of Charge (MinSOC) in MWh; and

(K)	The telemetered Maximum State of Charge (MaxSOC) in MWh; and

(b)	From SCED Pricing Run for each SCED execution where reliability deployments are in effect:

(i)	SCED Pricing Run Step 1 offer/bid curve information;

(ii)	SCED Pricing Run Step 2 offer/bid curve information;

(iii)	SCED Pricing Run Step 2 energy and Ancillary Service awards; and

(iv)	Ancillary Service Capabilities, LSL, LDL, HDL, and HSL for each Resource that has any of this data modified for input into the SCED Pricing Run due to a RUC or an ERCOT-directed deployment instruction.

(5)	ERCOT shall post on the ERCOT website for each Resource for each Operating Hour 60 days prior to the current Operating Day a count of the number of times for each Ancillary Service that the Resource’s Ancillary Service Offer quantity or price was updated within the Operating Period.  ERCOT shall post on the ERCOT website for each Resource for each Operating Hour 60 days prior to the current Operating Day, a count of the number of times a Resource’s Energy Offer quantity or price was updated within the Operating Hour, including any reason accompanying the update.

(6)	If any Real-Time Locational Marginal Price (LMP) from the SCED Dispatch Run, exceeds 50 times the Fuel Index Price (FIP) during any SCED interval for the applicable Operating Day, ERCOT shall post on the ERCOT website the portion of any Generation Resource’s as-submitted and as-mitigated and extended Energy Offer Curve or any ESR’s as-submitted and as-mitigated and extended Energy Bid/Offer Curve that is at or above 50 times the FIP for that SCED interval seven days after the applicable Operating Day.

(7)	If any Market Clearing Price for Capacity (MCPC) from the SCED Dispatch Run for an Ancillary Service exceeds 50 times the FIP for any Operating Hour in a DAM or any SCED interval in the RTM for the applicable Operating Day, ERCOT shall post on the ERCOT website the portion on any Resource’s Ancillary Service Offer that is at or above 50 times the FIP for that Ancillary Service for that Operating Hour for the DAM or SCED interval for the RTM seven days after the applicable Operating Day.

(8)	ERCOT shall post on the ERCOT website the offer price from the SCED Dispatch Run and the name of the Entity submitting the offer for the highest-priced offer selected or Dispatched by SCED three days after the end of the applicable Operating Day.  If multiple Entities submitted the highest-priced offers selected, all Entities shall be identified on the ERCOT website.

(9)	ERCOT shall post on the ERCOT website the bid price from the SCED Dispatch Run and the name of the Entity submitting the bid for the highest-priced bid selected or Dispatched by SCED three days after the end of the applicable Operating Day.  If multiple Entities submitted the highest-priced bids selected, all Entities shall be identified on the ERCOT website.

(10)	ERCOT shall post on the ERCOT website the offer price and the name of the Entity submitting the offer for the highest-priced Ancillary Service Offer selected in the DAM or RTM for each Ancillary Service three days after the end of the applicable Operating Day.  If multiple Entities submitted the highest-priced offers selected, all Entities shall be identified on the ERCOT website.  The report shall specify whether the Ancillary Service Offer was selected in a DAM or RTM.

(11)	ERCOT shall post on the ERCOT website for each Operating Day the following information for each Resource:

(a)	The Resource name;

(b)	The name of the Resource Entity;

(c)	Except for Load Resources that are not SCED qualified, the name of the Decision Making Entity (DME) controlling the Resource, as reflected in the Managed Capacity Declaration submitted by the Resource Entity in accordance with Section 3.6.2, Decision Making Entity for a Resource; and

(d)	Flag for Reliability Must-Run (RMR) Resources.

(12)	ERCOT shall post on the ERCOT website the following information from the DAM for each hourly Settlement Interval for the applicable Operating Day 60 days prior to the current Operating Day:

(a)	The Generation Resource name and the Generation Resource’s Three-Part Supply Offer (prices and quantities), including Startup Offer and Minimum-Energy Offer, available for the DAM;

(b)	For each Settlement Point, individual DAM Energy-Only Offer Curves available for the DAM and the name of the QSE submitting the offer;

(c)	The Resource name and the Resource’s Ancillary Service Offers available for the DAM;

(d) 	The Ancillary Service Only Offer for each Ancillary Service and the name of the QSE submitting the offer;

(e)	For each Settlement Point, individual DAM Energy Bids available for the DAM and the name of the QSE submitting the bid;

(f)	For each Settlement Point, individual PTP Obligation bids available to the DAM that sink at the Settlement Point and the QSE submitting the bid;

(g)	The awards for each Ancillary Service from the DAM for each Generation Resource;

(h)	The awards for each Ancillary Service from the DAM for each Load Resource;

(i)	The award for each Three-Part Supply Offer from the DAM and the name of the QSE receiving the award;

(j)	For each Settlement Point, the award of each DAM Energy-Only Offer from the DAM and the name of the QSE receiving the award;

(k)	For each Settlement Point, the award of each DAM Energy Bid from the DAM and the name of the QSE receiving the award;

(l)	For each Settlement Point, the award of each PTP Obligation bid from the DAM that sinks at the Settlement Point, including whether or not the PTP Obligation bid was linked to an Option, and the QSE submitting the bid;

[NPRR1188: Insert items (m) and (n) below upon system implementation and renumber accordingly:] (m) The CLR name and the CLR’s Energy Bid Curve (prices and quantities) available for the DAM; and (n) The award for each CLR’s Energy Bid Curve from the DAM and the name of the QSE receiving the award.

(m)	The ESR name and the ESR’s Energy Bid/Offer Curve (prices and quantities), available for the DAM;

(n)	The awards for each Ancillary Service from the DAM for each ESR; and

(o)	The award for each Energy Bid/Offer Curve from the DAM and the name of the QSE receiving the award.

#### 3.5.2.1	North 345 kV Hub (North 345)

(1)	The North 345 kV Hub is composed of the following Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | ANASW | 345 | NORTH |
| 2 | CN345 | 345 | NORTH |
| 3 | WLSH | 345 | NORTH |
| 4 | FMRVL | 345 | NORTH |
| 5 | LPCCS | 345 | NORTH |
| 6 | MNSES | 345 | NORTH |
| 7 | PRSSW | 345 | NORTH |
| 8 | SSPSW | 345 | NORTH |
| 9 | VLSES | 345 | NORTH |
| 10 | ALNSW | 345 | NORTH |
| 11 | ALLNC | 345 | NORTH |
| 12 | BNDVS | 345 | NORTH |
| 13 | BNBSW | 345 | NORTH |
| 14 | BBSES | 345 | NORTH |
| 15 | BOSQUESW | 345 | NORTH |
| 16 | CDHSW | 345 | NORTH |
| 17 | CNTRY | 345 | NORTH |
| 18 | CRLNW | 345 | NORTH |
| 19 | CMNSW | 345 | NORTH |
| 20 | CNRSW | 345 | NORTH |
| 21 | CRTLD | 345 | NORTH |
| 22 | DCSES | 345 | NORTH |
| 23 | EMSES | 345 | NORTH |
| 24 | ELKTN | 345 | NORTH |
| 25 | ELMOT | 345 | NORTH |
| 26 | EVRSW | 345 | NORTH |
| 27 | KWASS | 345 | NORTH |
| 28 | FGRSW | 345 | NORTH |
| 29 | FORSW | 345 | NORTH |
| 30 | FRNYPP | 345 | NORTH |
| 31 | GIBCRK | 345 | NORTH |
| 32 | HKBRY | 345 | NORTH |
| 33 | VLYRN | 345 | NORTH |
| 34 | JEWET | 345 | NORTH |
| 35 | KNEDL | 345 | NORTH |
| 36 | KLNSW | 345 | NORTH |
| 37 | LCSES | 345 | NORTH |
| 38 | LIGSW | 345 | NORTH |
| 39 | LEG | 345 | NORTH |
| 40 | LFKSW | 345 | NORTH |
| 41 | LWSSW | 345 | NORTH |
| 42 | MLSES | 345 | NORTH |
| 43 | MCCREE | 345 | NORTH |
| 44 | MDANP | 345 | NORTH |
| 45 | ENTPR | 345 | NORTH |
| 46 | NCDSE | 345 | NORTH |
| 47 | NORSW | 345 | NORTH |
| 48 | NUCOR | 345 | NORTH |
| 49 | PKRSW | 345 | NORTH |
| 50 | KMCHI | 345 | NORTH |
| 51 | PTENN | 345 | NORTH |
| 52 | RENSW | 345 | NORTH |
| 53 | RCHBR | 345 | NORTH |
| 54 | RNKSW | 345 | NORTH |
| 55 | RKCRK | 345 | NORTH |
| 56 | RYSSW | 345 | NORTH |
| 57 | SGVSW | 345 | NORTH |
| 58 | SHBSW | 345 | NORTH |
| 59 | SHRSW | 345 | NORTH |
| 60 | SCSES | 345 | NORTH |
| 61 | SYCRK | 345 | NORTH |
| 62 | THSES | 345 | NORTH |
| 63 | TMPSW | 345 | NORTH |
| 64 | TNP_ONE | 345 | NORTH |
| 65 | TRCNR | 345 | NORTH |
| 66 | TRSES | 345 | NORTH |
| 67 | TOKSW | 345 | NORTH |
| 68 | VENSW | 345 | NORTH |
| 69 | WLVEE | 345 | NORTH |
| 70 | W_DENT | 345 | NORTH |
| 71 | WTRML | 345 | NORTH |
| 72 | WCSWS | 345 | NORTH |
| 73 | WEBBS | 345 | NORTH |
| 74 | WHTNY | 345 | NORTH |
| 75 | WCPP | 345 | NORTH |

(2)	The North 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the Day-Ahead Market (DAM) in the Day-Ahead and is the simple average of the time-weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP North345 =	DASL – Σc(DAHUBSF North345, c * DASP c),

if HBBC North345≠0

DASPP North345 =	DASPP ERCOT345Bus, if HBBC North345=0

Where:

DAHUBSF North345, c	=	Σhb(HUBDF hb, North345, c * DAHBSF hb, North345, c)

DAHBSF hb, North345, c	=	Σpb(HBDF pb, hb, North345, c * DASF pb, hb, North345, c)

HUBDF hb, North345, c	=	IF(HB North345, c=0, 0, 1 / HB North345, c)

HBDF pb, hb, North345, c	=	IF(PB hb, North345, c=0, 0, 1 / PB hb, North345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP North345 | $/MWh | Day-Ahead Settlement Point Price¾The DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System Lambda¾The DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraint¾The DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF North345,c | none | Day-Ahead Shift Factor of the Hub ¾The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,North345,c | none | Day-Ahead Shift Factor of the Hub Bus¾The DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,North345,c | none | Day-Ahead Shift Factor of the power flow bus¾The DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, North345,c | none | Hub Distribution Factor per Hub Bus in a constraint¾The distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, North345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraint¾The distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, North345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC North345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB North345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| C | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP North345	=	Max [-$251,

(HUBDF hb, North345 * ((RTHBP hb, North345, y *

TLMP y) / (TLMP y)))], if HB North345≠0

RTSPP North345	=	RTSPP ERCOT345Bus, if HB North345=0

Where:

RNWF y		=	TLMP y / TLMP y

RTHBP hb, North345, y	=	(HBDF b, hb, North345 * RTLMP b, hb, North345, y)

HUBDF hb, North345	=	IF(HB North345=0, 0, 1 / HB North345)

HBDF b, hb, North345	=	IF(B hb, North345=0, 0, 1 / B hb, North345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP North345 | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RTHBP hb, North345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per Security-Constrained Economic Dispatch (SCED) interval¾The Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTLMP b, hb, North345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per interval¾The Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval |
| HUBDF hb, North345 | none | Hub Distribution Factor per Hub Bus¾The distribution factor of Hub Bus hb. |
| HBDF b, hb, North345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub Bus¾The distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, North345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB North345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |

[NPRR1057: Replace paragraph (4) above with the following upon system implementation:] (4) The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows: RTSPP North345 = Max [-$251, (HUBLMP North345, y * RNWF y)] Where: RNWF y = TLMP y / TLMP y The above variables are defined as follows: Variable; Unit; Description. RTSPP North345; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval.. RNWF y; none; Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. HUBLMP North345, y; $/MWh; Hub Locational Marginal Price¾The Hub LMP for the Hub for the SCED Interval y.. TLMP y; second; Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval..

#### 3.5.2.2	South 345 kV Hub (South 345)

(1)	The South 345 kV Hub is composed of the following Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | AUSTRO | 345 | SOUTH |
| 2 | BLESSING | 345 | SOUTH |
| 3 | CAGNON | 345 | SOUTH |
| 4 | COLETO | 345 | SOUTH |
| 5 | CLEASP | 345 | SOUTH |
| 6 | NEDIN | 345 | SOUTH |
| 7 | FAYETT | 345 | SOUTH |
| 8 | FPPYD1 | 345 | SOUTH |
| 9 | FPPYD2 | 345 | SOUTH |
| 10 | GARFIE | 345 | SOUTH |
| 11 | GUADG | 345 | SOUTH |
| 12 | HAYSEN | 345 | SOUTH |
| 13 | HILLCTRY | 345 | SOUTH |
| 14 | HOLMAN | 345 | SOUTH |
| 15 | KENDAL | 345 | SOUTH |
| 16 | LA_PALMA | 345 | SOUTH |
| 17 | LON_HILL | 345 | SOUTH |
| 18 | LOSTPI | 345 | SOUTH |
| 19 | LYTTON_S | 345 | SOUTH |
| 20 | MARION | 345 | SOUTH |
| 21 | PAWNEE | 345 | SOUTH |
| 22 | RIOHONDO | 345 | SOUTH |
| 23 | RIONOG | 345 | SOUTH |
| 24 | SALEM | 345 | SOUTH |
| 25 | SANMIGL | 345 | SOUTH |
| 26 | SKYLINE | 345 | SOUTH |
| 27 | STP | 345 | SOUTH |
| 28 | CALAVERS | 345 | SOUTH |
| 29 | BRAUNIG | 345 | SOUTH |
| 30 | WHITE_PT | 345 | SOUTH |
| 31 | ZORN | 345 | SOUTH |

(2)	The South 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time-weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP South345 =	DASL – Σc(DAHUBSF South345, c * DASP c),

if HBBC South345≠0

DASPP South345 =		DASPP ERCOT345Bus, if HBBC South345=0

Where:

DAHUBSF South345, c	=	Σhb(HUBDF hb, South345, c * DAHBSF hb, South345, c)

DAHBSF hb, South345, c	=	Σpb(HBDF pb, hb, South345, c * DASF pb, hb, South345, c)

HUBDF hb, South345, c	=	IF(HB South345, c=0, 0, 1 / HB South345, c)

HBDF pb, hb, South345, c	=	IF(PB hb, South345, c=0, 0, 1 / PB hb, South345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP South345 | $/MWh | Day-Ahead Settlement Point Price¾The DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System Lambda¾The DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraint¾The DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF South345,c | none | Day-Ahead Shift Factor of the Hub ¾The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,South345,c | none | Day-Ahead Shift Factor of the Hub Bus¾The DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,South345,c | none | Day-Ahead Shift Factor of the power flow bus¾The DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, South345,c | none | Hub Distribution Factor per Hub Bus in a constraint¾The distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, South345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraint¾The distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, South345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC South345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB South345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP South345	=	Max [-$251,

(HUBDF hb, South345 * ((RTHBP hb, South345, y * TLMP y) / (TLMP y)))], if HB South345≠0

RTSPP South345	=	RTSPP ERCOT345Bus, if HB South345=0

Where:

RNWF y		=	TLMP y / TLMP y

RTHBP hb, South345, y	=	(HBDF b, hb, South345 * RTLMP b, hb, South345, y)

HUBDF hb, South345	=	IF(HB South345=0, 0, 1 / HB South345)

HBDF b, hb, South345	=	IF(B hb, South345=0, 0, 1 / B hb, South345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP South345 | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RTHBP hb, South345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED interval¾The Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTLMP b, hb, South345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per interval¾The Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| HUBDF hb, South345 | none | Hub Distribution Factor per Hub Bus¾The distribution factor of Hub Bus hb. |
| HBDF b, hb, South345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub Bus¾The distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, South345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB South345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |

[NPRR1057: Replace paragraph (4) above with the following upon system implementation:] (4) The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows: RTSPP South345 = Max [-$251, (HUBLMP South345, y * RNWF y)] Where: RNWF y = TLMP y / TLMP y The above variables are defined as follows: Variable; Unit; Description. RTSPP South345; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval.. HUBLMP South345, y; $/MWh; Hub Locational Marginal Price¾The Hub LMP for the Hub for the SCED Interval y.. RNWF y; none; Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. TLMP y; second; Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval..

#### 3.5.2.3	Houston 345 kV Hub (Houston 345)

(1)	The Houston 345 kV Hub is composed of the following listed Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | ADK | 345 | HOUSTON |
| 2 | BI | 345 | HOUSTON |
| 3 | CBY | 345 | HOUSTON |
| 4 | CTR | 345 | HOUSTON |
| 5 | CHB | 345 | HOUSTON |
| 6 | DPW | 345 | HOUSTON |
| 7 | DOW | 345 | HOUSTON |
| 8 | RNS | 345 | HOUSTON |
| 9 | GBY | 345 | HOUSTON |
| 10 | JN | 345 | HOUSTON |
| 11 | KG | 345 | HOUSTON |
| 12 | KDL | 345 | HOUSTON |
| 13 | NB | 345 | HOUSTON |
| 14 | OB | 345 | HOUSTON |
| 15 | PHR | 345 | HOUSTON |
| 16 | SDN | 345 | HOUSTON |
| 17 | SMITHERS | 345 | HOUSTON |
| 18 | THW | 345 | HOUSTON |
| 19 | WAP | 345 | HOUSTON |
| 20 | WO | 345 | HOUSTON |

(2)	The Houston 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time-weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP Houston345 =	DASL – Σc(DAHUBSF Houston345, c * DASP c),

if HBBC Houston345≠0

DASPP Houston345 =	DASPP ERCOT345Bus, if HBBC Houston345=0

Where:

DAHUBSF Houston345, c	=	Σhb(HUBDF hb, Houston345, c * DAHBSF hb, Houston345, c)

DAHBSF hb, Houston345, c	=	Σpb(HBDF pb, hb, Houston345, c * DASF pb, hb, Houston345, c)

HUBDF hb, Houston345, c	=	IF(HB Houston345, c=0, 0, 1 / HB Houston345, c)

HBDF pb, hb, Houston345, c	=	IF(PB hb, Houston345, c=0, 0, 1 / PB hb, Houston345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP Houston345 | $/MWh | Day-Ahead Settlement Point Price¾The DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System Lambda¾The DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraint¾The DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF Houston345,c | none | Day-Ahead Shift Factor of the Hub ¾The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,Houston345,c | none | Day-Ahead Shift Factor of the Hub Bus¾The DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,Houston345,c | none | Day-Ahead Shift Factor of the power flow bus¾The DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, Houston345,c | none | Hub Distribution Factor per Hub Bus in a constraint¾The distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, Houston345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraint¾The distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, Houston345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC Houston345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB Houston345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP Houston345	   =	Max [-$251,

(HUBDF hb, Houston345 * ((RTHBP hb, Houston345, y *

TLMP y) / (TLMP y)))], if HB Houston345≠0

RTSPP Houston345   =	RTSPP ERCOT345Bus, if HB Houston345=0

Where:

RNWF y		=	TLMP y / TLMP y

RTHBP hb, Houston345, y	=	(HBDF b, hb, Houston345 * RTLMP b, hb, Houston345, y)

HUBDF hb, Houston345	=	IF(HB Houston345=0, 0, 1 / HB Houston345)

HBDF b, hb, Houston345	=	IF(B hb, Houston345=0, 0, 1 / B hb, Houston345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP Houston345 | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RTHBP hb, Houston345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED interval¾The Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTLMP b, hb, Houston345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per interval¾The Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval |
| HUBDF hb, Houston345 | none | Hub Distribution Factor per Hub Bus¾The distribution factor of Hub Bus hb. |
| HBDF b, hb, Houston345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub Bus¾The distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, Houston345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB Houston345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |

[NPRR1057: Replace paragraph (4) above with the following upon system implementation:] (4) The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows: RTSPP Houston345 = Max [-$251, (HUBLMP Houston345, y * RNWF y )] Where: RNWF y = TLMP y / TLMP y The above variables are defined as follows: Variable; Unit; Description. RTSPP Houston345; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval.. HUBLMP Houston345, y; $/MWh; Hub Locational Marginal Price¾The Hub LMP for the Hub for the SCED Interval y.. RNWF y; none; Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. TLMP y; second; Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval..

#### 3.5.2.4	West 345 kV Hub (West 345)

(1)	The West 345 kV Hub is composed of the following listed Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | MULBERRY | 345 | WEST |
| 2 | BOMSW | 345 | WEST |
| 3 | OECCS | 345 | WEST |
| 4 | BITTCR | 345 | WEST |
| 5 | FSHSW | 345 | WEST |
| 6 | FLCNS | 345 | WEST |
| 7 | GRSES | 345 | WEST |
| 8 | JCKSW | 345 | WEST |
| 9 | MDLNE | 345 | WEST |
| 10 | MOSSW | 345 | WEST |
| 11 | MGSES | 345 | WEST |
| 12 | DCTM | 345 | WEST |
| 13 | ODEHV | 345 | WEST |
| 14 | OKLA | 345 | WEST |
| 15 | REDCREEK | 345 | WEST |
| 16 | SWESW | 345 | WEST |
| 17 | TWINBU | 345 | WEST |

(2)	The West 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP West345 =		DASL – Σc(DAHUBSF West345, c * DASP c),

if HBBC West345≠0

DASPP West345 =		DASPP ERCOT345Bus, if HBBC West345=0

Where:

DAHUBSF West345, c	=	Σhb(HUBDF hb, West345, c * DAHBSF hb, West345, c)

DAHBSF hb, West345, c	=	Σpb(HBDF pb, hb, West345, c * DASF pb, hb, West345, c)

HUBDF hb, West345, c	=	IF(HB West345, c=0, 0, 1 / HB West345, c)

HBDF pb, hb, West345, c	=	IF(PB hb, West345, c=0, 0, 1 / PB hb, West345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP West345 | $/MWh | Day-Ahead Settlement Point Price¾The DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System Lambda¾The DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraint¾The DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF West345,c | none | Day-Ahead Shift Factor of the Hub ¾The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,West345,c | none | Day-Ahead Shift Factor of the Hub Bus¾The DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,West345,c | none | Day-Ahead Shift Factor of the power flow bus¾The DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, West345,c | none | Hub Distribution Factor per Hub Bus in a constraint¾The distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, West345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraint¾The distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, West345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC West345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB West345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP West345	=	Max [-$251,

(HUBDF hb, West345 * ((RTHBP hb, West345, y * TLMP y) / (TLMP y)))], if HB West345≠0

RTSPP West345	=	RTSPP ERCOT345Bus, if HB West345=0

Where:

RNWF y		=	TLMP y / TLMP y

RTHBP hb, West345, y	=	(HBDF b, hb, West345 * RTLMP b, hb, West345, y)

HUBDF hb, West345	=	IF(HB West345=0, 0, 1 / HB West345)

HBDF b, hb, West345	=	IF(B hb, West345=0, 0, 1 / B hb, West345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP West345 | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTHBP hb, West345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED interval¾The Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RTLMP b, hb, West345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per interval¾The Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| HUBDF hb, West345 | none | Hub Distribution Factor per Hub Bus¾The distribution factor of Hub Bus hb. |
| HBDF b, hb, West345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub Bus¾The distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, West345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB West345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |

[NPRR1057: Replace paragraph (4) above with the following upon system implementation:] (4) The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows: RTSPP West345 = Max [-$251, (HUBLMP West345, y * RNWF y)] Where: RNWF y = TLMP y / TLMP y The above variables are defined as follows: Variable; Unit; Description. RTSPP West345; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval.. HUBLMP West345, y; $/MWh; Hub Locational Marginal Price¾The Hub LMP for the Hub for the SCED Interval y.. RNWF y; none; Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. TLMP y; second; Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval..

#### 3.5.2.5	Panhandle 345 kV Hub (Pan 345)

(1)	The Panhandle 345 kV Hub is composed of the following listed Hub Buses:

|  | ERCOT Operations |  |  |
| --- | --- | --- | --- |
| No. | Hub Bus | kV | Hub |
| 1 | ABERNATH | 345 | PAN |
| 2 | AJ_SWOPE | 345 | PAN |
| 3 | ALIBATES | 345 | PAN |
| 4 | CTT_CROS | 345 | PAN |
| 5 | CTT_GRAY | 345 | PAN |
| 6 | OGALLALA | 345 | PAN |
| 7 | RAILHEAD | 345 | PAN |
| 8 | TESLA | 345 | PAN |
| 9 | TULECNYN | 345 | PAN |
| 10 | W_CW_345 | 345 | PAN |
| 11 | WHIT_RVR | 345 | PAN |
| 12 | WINDMILL | 345 | PAN |

(2)	The Panhandle 345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP Pan345 =		DASL – Σc(DAHUBSF Pan345, c * DASP c),

if HBBC Pan345≠0

DASPP Pan345 =		DASPP ERCOT345Bus, if HBBC Pan345=0

Where:

DAHUBSF Pan345, c	=	Σhb(HUBDF hb, Pan345, c * DAHBSF hb, Pan345, c)

DAHBSF hb, Pan345, c	=	Σpb(HBDF pb, hb, Pan345, c * DASF pb, hb, Pan345, c)

HUBDF hb, Pan345, c	=	IF(HB Pan345, c=0, 0, 1 / HB Pan345, c)

HBDF pb, hb, Pan345, c	=	IF(PB hb, Pan345, c=0, 0, 1 / PB hb, Pan345, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP Pan345 | $/MWh | Day-Ahead Settlement Point Price¾The DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System Lambda¾The DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraint¾The DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF Pan345,c | none | Day-Ahead Shift Factor of the Hub ¾The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,Pan345,c | none | Day-Ahead Shift Factor of the Hub Bus¾The DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,Pan345,c | none | Day-Ahead Shift Factor of the power flow bus¾The DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb, Pan345,c | none | Hub Distribution Factor per Hub Bus in a constraint¾The distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, Pan345,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraint¾The distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, Pan345,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c. |
| HBBC Pan345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case. |
| HB Pan345,c | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP Pan345	=	Max [-$251,

(HUBDF hb, Pan345 * ((RTHBP hb, Pan345, y * TLMP y) /           ( TLMP y)))], if HB Pan345≠0

RTSPP Pan345	=	RTSPP ERCOT345Bus, if HB Pan345=0

Where:

RNWF y	=	TLMP y / TLMP y

RTHBP hb, Pan345, y	=	  (HBDF b, hb, Pan345 * RTLMP b, hb, Pan345, y)

HUBDF hb, Pan345	=	IF(HB Pan345=0, 0, 1 / HB Pan345)

HBDF b, hb, Pan345	=	IF(B hb, Pan345=0, 0, 1 / B hb, Pan345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP Pan345 | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub for the 15-minute Settlement Interval. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTHBP hb, Pan345, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED interval¾The Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RTLMP b, hb, Pan345, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per interval¾The Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| HUBDF hb, Pan345 | none | Hub Distribution Factor per Hub Bus¾The distribution factor of Hub Bus hb. |
| HBDF b, hb, Pan345 | none | Hub Bus Distribution Factor per Electrical Bus of Hub Bus¾The distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, Pan345 | none | The total number of energized Electrical Buses in Hub Bus hb. |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB Pan345 | none | The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus. |

[NPRR1057: Replace paragraph (4) above with the following upon system implementation:] (4) The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows: RTSPP Pan345 = Max [-$251, (HUBLMP Pan345, y * RNWF y )] Where: RNWF y = TLMP y / TLMP y The above variables are defined as follows: Variable; Unit; Description. RTSPP Pan345; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub for the 15-minute Settlement Interval.. HUBLMP Pan345, y; $/MWh; Hub Locational Marginal Price¾The Hub LMP for the Hub for the SCED Interval y.. RNWF y; none; Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. TLMP y; second; Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval..

[NPRR941 and NPRR1057: Insert applicable portions of Section 3.5.2.6 below upon system implementation; and renumber accordingly:] #### 3.5.2.6 Lower Rio Grande Valley Hub (LRGV 138/345) (1) The Lower Rio Grande Valley Hub 138/345 kV Hub is composed of the following listed Hub Buses: ERCOT Operations. No.; Hub Bus; kV; Hub. 1; AIRPORT; 138; LRGV. 2; ALBERTA; 138; LRGV. 3; BATES; 138; LRGV. 4; FRONTERA; 138; LRGV. 5; GARZA; 138; LRGV. 6; HARLNSW; 138; LRGV. 7; HEC; 138; LRGV. 8; KEY_SW; 138; LRGV. 9; LA_PALMA_345; 345; LRGV. 10; LA_PALMA_138; 138; LRGV. 11; LASPULGA; 138; LRGV. 12; LISTON; 138; LRGV. 13; LOMA_ALT; 138; LRGV. 14; MARCONI; 138; LRGV. 15; MILHWY; 138; LRGV. 16; MILITARY; 138; LRGV. 17; MV_WEDN4; 138; LRGV. 18; N_MCALLN; 138; LRGV. 19; NEDIN_345; 345; LRGV. 20; NEDIN_138; 138; LRGV. 21; OLEANDER; 138; LRGV. 22; P_ISABEL; 138; LRGV. 23; PALMHRTP; 138; LRGV. 24; PALMITO_345; 345; LRGV. 25; PALMITO_138; 138; LRGV. 26; PAREDES; 138; LRGV. 27; PHARMVEC; 138; LRGV. 28; PHARR; 138; LRGV. 29; PRICE_RD; 138; LRGV. 30; RAILROAD; 138; LRGV. 31; RAYMND2; 138; LRGV. 32; REDTAP; 138; LRGV. 33; RIO_GRAN; 138; LRGV. 34; RIOHONDO_345; 345; LRGV. 35; RIOHONDO_138; 138; LRGV. 36; ROMA_SW; 138; LRGV. 37; S_MCALLN; 138; LRGV. 38; SCARBIDE; 138; LRGV. 39; SILASRAY; 138; LRGV. 40; STEWART; 138; LRGV. 41; WESLACO; 138; LRGV. ΣcΣhbΣpb(2) The Lower Rio Grande Valley 138/345 kV Hub Price uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub. (3) The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows: DASPP LRGV 138/345 = DASL – ΣcΣc(DAHUBSF LRGV 138/345, c * DASP c), if HBBC LRGV138/345≠0 DASPP LRGV138/345 = DASPP ERCOT345Bus, if HBBC LRGV138/345=0 Where: DAHUBSF LRGV138/345, c = Σhb(HUBDF hb, LRGV138/345, c * DAHBSF hb, LRGV138/345, c) DAHBSF hb, LRGV138/345, c = Σpb(HBDF pb, hb, LRGV138/345, c * DASF pb, hb, LRGV138/345, c) HUBDF hb, LRGV138/345, c = IF(HB LRGV138/345, c=0, 0, 1 / HB LRGV138/345, c) HBDF pb, hb, LRGV138/345, c = IF(PB hb, LRGV138/345, c=0, 0, 1 / PB hb, LRGV138/345, c) The above variables are defined as follows: Variable; Unit; Definition. DASPP LRGV138/345; $/MWh; Day-Ahead Settlement Point Price¾The DAM Settlement Point Price at the Hub, for the hour.. DASL; $/MWh; Day-Ahead System Lambda¾The DAM Shadow Price for the system power balance constraint for the hour.. DASP c; $/MWh; Day-Ahead Shadow Price for a binding transmission constraint¾The DAM Shadow Price for the constraint c for the hour.. DAHUBSF LRGV138/345,c; none; Day-Ahead Shift Factor of the Hub ¾The DAM aggregated Shift Factor of a Hub for the constraint c for the hour.. DAHBSF hb, LRGV138/345,c; none; Day-Ahead Shift Factor of the Hub Bus¾The DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour.. DASF pb,hb, LRGV138/345,c; none; Day-Ahead Shift Factor of the power flow bus¾The DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour.. HUBDF hb, LRGV138/345,c; none; Hub Distribution Factor per Hub Bus in a constraint¾The distribution factor of Hub Bus hb for the constraint c for the hour.. HBDF pb, hb, LRGV138/345,c; none; Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraint¾The distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour.. pb; none; An energized power flow bus that is a component of a Hub Bus for the constraint c.. PB hb, LRGV138/345,c; none; The total number of energized power flow buses in Hub Bus hb for the constraint c.. hb; none; A Hub Bus that is a component of the Hub with at least one energized power flow bus for the constraint c.. HBBC LRGV138/345; none; The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus in base case.. HB LRGV138/345,c; none; The total number of Hub Buses in the Hub with at least one energized component in each Hub Bus for the constraint c.. c; none; A DAM binding transmission constraint for the hour caused by either base case or a contingency.. (4) The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows: RTSPP LRGV138/345 = Max [-$251, Σy(HUBLMP LRGV138/345, y * RNWF y)] Where: Σy RNWF y = TLMP y / ΣyTLMP y The above variables are defined as follows: Variable; Unit; Description. RTSPP LRGV138/345kV; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub for the 15-minute Settlement Interval.. HUBLMP LRGV138/345, y; $/MWh; Hub Locational Marginal Price¾The Hub LMP for the Hub for the SCED Interval y.. RNWF y; none; Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. TLMP y; second; Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval..

#### 3.5.2.7	ERCOT Bus Average 345 kV Hub (ERCOT 345 Bus)

(1)	The ERCOT Bus Average 345 kV Hub is composed of the Hub Buses listed in Section 3.5.2.1, North 345 kV Hub (North 345); Section 3.5.2.2, South 345 kV Hub (South 345); Section 3.5.2.3, Houston 345 kV Hub (Houston 345); and Section 3.5.2.4, West 345 kV Hub (West 345).  The Panhandle 345 kV Hub is not included in the ERCOT Bus Average 345 kV Hub price.

[NPRR941: Replace paragraph (1) above upon system implementation:] (1) The ERCOT Bus Average 345 kV Hub is composed of the Hub Buses listed in Section 3.5.2.1, North 345 kV Hub (North 345); Section 3.5.2.2, South 345 kV Hub (South 345); Section 3.5.2.3, Houston 345 kV Hub (Houston 345); and Section 3.5.2.4, West 345 kV Hub (West 345). The Panhandle 345 kV Hub and the Lower Rio Grande Valley 138/345 kV Hub are not included in the ERCOT Bus Average 345 kV Hub price.

(2)	The ERCOT Bus Average 345 kV Hub uses the aggregated Shift Factors of the Hub Buses for each hour of the Settlement Interval of the DAM in the Day-Ahead and is the simple average of the time weighted Hub Bus prices for each 15-minute Settlement Interval in Real-Time, for each Hub Bus included in this Hub.

(3)	The Day-Ahead Settlement Point Price of the Hub for a given Operating Hour is calculated as follows:

DASPP ERCOT345Bus =	DASL – Σc(DAHUBSF ERCOT345Bus, c * DASP c),

if HBBC ERCOT345Bus≠0

DASPP ERCOT345Bus =	0, if HBBC ERCOT345Bus=0

Where:

DAHUBSF ERCOT345Bus, c   =	Σhb(HUBDF hb, ERCOT345Bus, c * DAHBSF hb, ERCOT345Bus, c)

DAHBSF hb, ERCOT345Bus, c  =	Σpb(HBDF pb, hb, ERCOT345Bus, c * DASF pb, hb, ERCOT345Bus, c)

HUBDF hb, ERCOT345Bus, c	=	IF(HB ERCOT345Bus, c=0, 0, 1 / HB ERCOT345Bus, c)

HBDF pb, hb, ERCOT345Bus, c	=	IF(PB hb, ERCOT345Bus, c=0, 0, 1 / PB hb, ERCOT345Bus, c)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| DASPP ERCOT345Bus | $/MWh | Day-Ahead Settlement Point Price¾The DAM Settlement Point Price at the Hub, for the hour. |
| DASL | $/MWh | Day-Ahead System Lambda¾The DAM Shadow Price for the system power balance constraint for the hour. |
| DASP c | $/MWh | Day-Ahead Shadow Price for a binding transmission constraint¾The DAM Shadow Price for the constraint c for the hour. |
| DAHUBSF ERCOT345Bus,c | none | Day-Ahead Shift Factor of the Hub ¾The DAM aggregated Shift Factor of a Hub for the constraint c for the hour. |
| DAHBSF hb,ERCOT345Bus,c | none | Day-Ahead Shift Factor of the Hub Bus¾The DAM aggregated Shift Factor of a Hub Bus hb for the constraint c for the hour. |
| DASF pb,hb,ERCOT345Bus,c | none | Day-Ahead Shift Factor of the power flow bus¾The DAM Shift Factor of a power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| HUBDF hb,ERCOT345Bus,c | none | Hub Distribution Factor per Hub Bus in a constraint¾The distribution factor of Hub Bus hb for the constraint c for the hour. |
| HBDF pb, hb, ERCOT345Bus,c | none | Hub Bus Distribution Factor per power flow bus of Hub Bus in a constraint¾The distribution factor of power flow bus pb that is a component of Hub Bus hb for the constraint c for the hour. |
| pb | none | An energized power flow bus that is a component of a Hub Bus for the constraint c. |
| PB hb, ERCOT345Bus,c | none | The total number of energized power flow buses in Hub Bus hb for the constraint c. |
| hb | none | A Hub Bus that is a component of the ERCOT Bus Average 345 kV Hub (ERCOT 345 Bus) with at least one energized power flow bus for the constraint c. The Hub “ERCOT 345 Bus” includes any Hub Bus defined in the Hub “North 345”, “South 345”, “Houston 345” and “West 345”. |
| HBBC ERCOT345Bus | none | The total number of Hub Buses in the ERCOT Bus Average 345 kV Hub (ERCOT 345 Bus) with at least one energized component in each Hub Bus in base case. The Hub “ERCOT 345 Bus” includes any Hub Bus defined in the Hub “North 345”, “South 345”, “Houston 345” and “West 345”. |
| HB ERCOT345Bus,c | none | The total number of Hub Buses in the ERCOT Bus Average 345 kV Hub (ERCOT 345 Bus) with at least one energized component in each Hub Bus for the constraint c. The Hub “ERCOT 345 Bus” includes any Hub Bus defined in the Hub “North 345”, “South 345”, “Houston 345” and “West 345”. |
| c | none | A DAM binding transmission constraint for the hour caused by either base case or a contingency. |

(4)	The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows:

RTSPP ERCOT345Bus	=	Max [-$251,

(HUBDF hb, ERCOT345Bus * ((RTHBP hb, ERCOT345Bus, y * TLMP y) / (TLMP y)))], if HB ERCOT345Bus ≠0

RTSPP ERCOT345Bus	=	0, if HB ERCOT345Bus =0

Where:

RNWF y		=	TLMP y / TLMP y

RTHBP hb, ERCOT345Bus, y	=	(HBDF b, hb, ERCOT345Bus * RTLMP b, hb, ERCOT345Bus, y)

HUBDF hb, ERCOT345Bus	=	1 / (HB North345 + HB South345 + HB Houston345 + HB West345)

If Electrical Bus b is a component of “North 345”

HBDF b, hb, ERCOT345Bus	=	IF(B hb, North345=0, 0, 1 / B hb, North345)

Otherwise

If Electrical Bus b is a component of “South 345”

HBDF b, hb, ERCOT345Bus	=	IF(B hb, South345=0, 0, 1 / B hb, South345)

Otherwise

If Electrical Bus b is a component of “Houston 345”

HBDF b, hb, ERCOT345Bus	=	IF(B hb, Houston345=0, 0, 1 / B hb, Houston345)

Otherwise

HBDF b, hb, ERCOT345Bus	=	IF(B hb, West345=0, 0, 1 / B hb, West345)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP ERCOT345Bus | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTHBP hb, ERCOT345Bus, y | $/MWh | Real-Time Hub Bus Price at Hub Bus per SCED interval¾The Real-Time energy price at Hub Bus hb for the SCED interval y. |
| RTLMP b, hb, ERCOT345Bus, y | $/MWh | Real-Time Locational Marginal Price at Electrical Bus of Hub Bus per interval¾The Real-Time LMP at Electrical Bus b that is a component of Hub Bus hb, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval. |
| HUBDF hb, ERCOT345Bus | none | Hub Distribution Factor per Hub Bus¾The distribution factor of Hub Bus hb. |
| HBDF b, hb, ERCOT345Bus | none | Hub Bus Distribution Factor per Electrical Bus of Hub Bus¾The distribution factor of Electrical Bus b that is a component of Hub Bus hb. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An energized Electrical Bus that is a component of a Hub Bus. |
| B hb, North345 | none | The total number of energized Electrical Buses in Hub Bus hb that is a component of “North 345.” |
| B hb, South345 | none | The total number of energized Electrical Buses in Hub Bus hb that is a component of “South 345.” |
| B hb, Houston345 | none | The total number of energized Electrical Buses in Hub Bus hb that is a component of “Houston 345.” |
| B hb, West345 | none | The total number of energized Electrical Buses in Hub Bus hb that is a component of “West 345.” |
| hb | none | A Hub Bus that is a component of the Hub. |
| HB North345 | none | The total number of Hub Buses in “North 345.” |
| HB South345 | none | The total number of Hub Buses in “South 345.” |
| HB Houston345 | none | The total number of Hub Buses in “Houston 345.” |
| HB West345 | none | The total number of Hub Buses in “West 345.” |

[NPRR1057: Replace paragraph (4) above with the following upon system implementation:] (4) The Real-Time Settlement Point Price of the Hub for a given 15-minute Settlement Interval is calculated as follows: RTSPP ERCOT345Bus = Max [-$251, (HUBLMPERCOT345Bus,y * RNWF y)] Where: RNWF y = TLMP y / TLMP y The above variables are defined as follows: Variable; Unit; Description. RTSPP ERCOT345Bus; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Hub, for the 15-minute Settlement Interval.. HUBLMP ERCOT345Bus,y; $/MWh; Hub Locational Marginal Price for the ERCOT345Bus¾The Hub LMP for the ERCOT Bus Average 345 kV Hub (ERCOT 345 Bus), for the SCED Interval y.. RNWF y; none; Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval.. TLMP y; second; Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the 15-minute Settlement Interval.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval..

### 4.4.11	Day-Ahead and Real-Time System-Wide Offer Caps

(1)	The DASWCAP and RTSWCAP shall be determined in accordance with the Public Utility Commission of Texas (PUCT) rules.  The methodology for determining the DASWCAP and RTSWCAP is as follows:

(a)	The DASWCAP and RTSWCAP shall be set equal to the respective High System-Wide Offer Cap (HCAP).  Additionally, the Value of Lost Load (VOLL) used to determine the ASDCs for DAM and RTM shall be set to the HCAP for DAM.  These caps shall be maintained at these levels until either of the following criteria are met:

(i)	If the RTM System Lambda is greater than or equal to the HCAP for DAM for a total of 12 hours within a rolling 24-hour period, ERCOT will activate the Emergency Pricing Program (EPP) and the DASWCAP and VOLL used to determine the ASDCs for DAM and RTM will be set to ECAP starting at the beginning of the next Operating Hour that ERCOT can implement the change.  Security-Constrained Economic Dispatch (SCED)-level data, time-weight averaged to a 15-minute Settlement Interval equivalent, will be used to make this determination.  The EPP will remain active until the later of the following, at which point the ECAP Effective Period will end at the beginning of the next Operating Hour:

(A)	24 hours after the initial setting of these values to ECAP; or

(B)	24 hours after ERCOT exits Energy Emergency Alert (EEA) conditions, if ERCOT entered into or remained in EEA while the EPP was active.  If ERCOT reenters EEA conditions within 24 hours, then the ECAP Effective Period will continue for 24 hours after the latest exit from EEA conditions.

(ii)	If the Peaker Net Margin (PNM) exceeds the PNM threshold per MW-year during a year, the DASWCAP and the VOLL used to determine the ASDCs for DAM and RTM shall be set per the schedule in Section 4.4.11.1, Scarcity Pricing Mechanism.

(b)	ERCOT shall issue operations notices when the ECAP Effective Period begins and ends.  Such notices shall respectively state the date and time of the initiation and cessation of the ECAP Effective Period.

(c)	ERCOT will post on the ERCOT website the cumulative number of hours in which the RTM System Lambda has been greater than or equal to the DASWCAP over a rolling 24-hour period.  This calculation of cumulative hours will use the 15-minute Settlement Interval equivalent price referenced in paragraph (1)(a)(i) above.

(d)	Within ten Business Days of the end of the ECAP Effective Period, ERCOT shall file an initial report with the PUCT providing a summary of the event that triggered the EPP and an analysis of the EPP’s performance.

(e)	Within 90 days of the end of the ECAP Effective Period, ERCOT shall file a final report with the PUCT providing a summary of the event that triggered the EPP, an analysis of the EPP’s performance, and any recommendations to modify or improve the EPP.  The report shall also include the number of Resources for which Qualified Scheduling Entities (QSEs) filed for cost recovery and the total dollar amount of costs submitted and costs recovered, including fuel type, MW per hour, and number of Resources associated with the recovered costs.

(f)	For the PNM process described above, ERCOT shall set the PNM threshold at three times the cost of new entry of new generation plants.

The above parameters are defined as follows:

| Parameter | Unit | Current Value* |
| --- | --- | --- |
| ECAP | $/MWh | 2,000 |
| HCAP – DAM (DASWCAP) | $/MWh | 5,000 |
| HCAP – RTM (RTSWCAP) | $/MWh | 2,000 |
| LCAP | $/MWh | 2,000 |
| PNM threshold | $/MW-year | 315,000 |
| * The current value for the parameters referenced in this table above will be recommended by TAC and approved by the ERCOT Board. ERCOT shall update parameter values on the first day of the month following ERCOT Board approval unless otherwise directed by the ERCOT Board. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value. |  |  |

(2)	Any offers submitted that exceed the current respective DASWCAP or RTSWCAP shall be rejected by ERCOT.  The applicable cap will be dependent on the timing of the submission.

### 5.5.2	Reliability Unit Commitment (RUC) Process

(1)	The RUC process recommends commitment of Generation Resources, to match ERCOT’s forecasted Load including Direct Current Tie (DC Tie) Schedules and RUC Ancillary Service Demand Curves (ASDCs), subject to all transmission constraints and Resource performance characteristics.  The RUC process takes into account Resources already committed in the Current Operating Plans (COPs), Resources already committed in previous RUCs, and Off-Line Available Resources having a start-up time of one hour or less.  For On-Line Energy Storage Resources (ESRs), using RUC duration requirements for energy and Ancillary Services, RUC-projected dispatch for energy and Ancillary Service in one interval shall respect the ESR’s minimum and maximum State of Charge (SOC) values from the COP, while incorporating any adjustments under paragraph (20)(d) below.  In addition, using the Ancillary Service Deployment Factors and their respective deployment duration requirements, the SOC required to support these dispatch levels for energy and Ancillary Services will match as closely as possible the difference between the adjusted COP values of the next interval’s Hour Beginning Planned SOC (HBSOC) and the current interval’s HBSOC.  The formulation of the RUC objective function must employ penalty factors on violations of security constraints and violations of ESR COP HBSOC.  The objective of the RUC process is to minimize costs based on the Resource costs described in paragraphs (12) through (16) below.  ESR energy dispatch costs and Ancillary Service Offer costs are not included in the RUC objective function.

(2)	ERCOT shall create an ASDC for each Ancillary Service for use in RUC.  The ASDCs for each Ancillary Service for use in RUC shall be substantively the same as the ASDCs defined in Section 4.4.12, Determination of Ancillary Service Demand Curves for the Day-Ahead Market and Real-Time Market.  Specific to RUC, the ASDC for Non-Spinning Reserve (Non-Spin) shall not extend beyond the Ancillary Service Plan for Non-Spin for the relevant Operating Hour.  ERCOT shall post the ASDCs for RUC to the ERCOT website following each execution of the RUC process.

(3)	ERCOT shall post the following Ancillary Service Deployment Factor data on the ERCOT website:

(a)	Following each execution of RUC, ERCOT shall post the Ancillary Service Deployment Factors used by that RUC process for each hour in the RUC Study Period;

(b)	No later than 0600 in the Day-Ahead for each Operating Day, ERCOT shall post the Ancillary Service Deployments Factors that are projected to be used in the RUC process for that Operating Day; and

(c)	Following each month, ERCOT shall post the average, minimum, and maximum Ancillary Service Deployment Factors used in the RUC process by type of Ancillary Service and hour of the day for the month.

(4)	For all hours of the RUC Study Period within the RUC process, Quick Start Generation Resources (QSGRs) with a COP Resource Status of OFFQS shall be considered as On-Line with Low Sustained Limit (LSL) at zero MW.  QSGRs with a Resource Status of OFFQS shall only be committed by ERCOT through a RUC instruction in instances when a reliability issue would not otherwise be managed through Dispatch Instructions from Security-Constrained Economic Dispatch (SCED).

(5)	In addition to On-Line qualified Generation Resources and ESRs, the RUC engine shall consider a COP Resource status of OFFQS for QSGRs that are qualified for ERCOT Contingency Reserve Service (ECRS), as being eligible to provide ECRS constrained by the Ancillary Service capability in the COP.

(6)	In addition to On-Line qualified Generation Resources and ESRs, the RUC engine shall consider a COP Resource Status of OFFQS for QSGRs that are qualified for Non-Spin, as being eligible to provide Non-Spin constrained by the Ancillary Service Capability in the COP.  The RUC engine shall also consider a COP Resource Status of OFF (Off-Line but available for commitment in the DAM and RUC) for a Resource that is qualified for Non-Spin, as being eligible to provide Non-Spin constrained by the Ancillary Service capability in the COP.

(7)	In addition to On-Line qualified Generation Resources and ESRs, the RUC engine shall consider a COP Resource Status of ONL for Load Resources that are qualified for Ancillary Services, as being eligible to provide Ancillary Services constrained by the Ancillary Service Capability in the COP.  The RUC engine will not consider any Load Resources for dispatch of energy.

(8)       The RUC constraints in the RUC engine shall use 60 minutes as the duration for energy and Ancillary Services, excluding Responsive Reserve (RRS) provided using Fast Frequency Response (FFR), for which duration shall be 15 minutes.  These same duration requirements will be used to enforce a constraint on each ESR’s dispatch for energy and Ancillary Services using Ancillary Service deployment factors for a given hour such that the calculated SOC at the end of that hour is equal to the next hour’s COP value of HBSOC.

(9)	The RUC process can recommend Resource decommitment.  ERCOT may only decommit a Resource to resolve transmission constraints that are otherwise unresolvable.  Qualifying Facilities (QFs) may be decommitted only after all other types of Resources have been assessed for decommitment.  In addition, the HRUC process provides decision support to ERCOT regarding a Resource decommitment requested by a Qualified Scheduling Entity (QSE).

(10)	ERCOT shall review the RUC-recommended Resource commitments and the list of Off-Line Available Resources having a start-up time of one hour or less to assess feasibility and shall make any changes that it considers necessary, in its sole discretion.  During the RUC process, ERCOT may also review and commit, through a RUC instruction, Combined Cycle Generation Resources that are currently planned to be On-Line but are capable of transitioning to a configuration with additional capacity.  ERCOT may deselect Resources recommended in DRUC and in all HRUC processes if in ERCOT’s sole discretion there is enough time to commit those Resources in the future HRUC processes, taking into account the Resources’ start-up times, to meet ERCOT System reliability.  After each RUC run, ERCOT shall post the amount of capacity deselected per hour in the RUC Study Period to the MIS Secure Area.  A Generation Resource shown as On-Line and available for SCED dispatch for an hour in its COP prior to a DRUC or HRUC process execution, according to Section 5.3, ERCOT Security Sequence Responsibilities, will be considered self-committed for that hour.  For purpose of Settlement, snapshot data will be used as specified in paragraph (2) of Section 5.3.

[NPRR1239: Replace paragraph (10) above with the following upon system implementation:] (10) ERCOT shall review the RUC-recommended Resource commitments and the list of Off-Line Available Resources having a start-up time of one hour or less to assess feasibility and shall make any changes that it considers necessary, in its sole discretion. During the RUC process, ERCOT may also review and commit, through a RUC instruction, Combined Cycle Generation Resources that are currently planned to be On-Line but are capable of transitioning to a configuration with additional capacity. ERCOT may deselect Resources recommended in DRUC and in all HRUC processes if in ERCOT’s sole discretion there is enough time to commit those Resources in the future HRUC processes, taking into account the Resources’ start-up times, to meet ERCOT System reliability. After each RUC run, ERCOT shall post the amount of capacity deselected per hour in the RUC Study Period to the ERCOT website. A Generation Resource shown as On-Line and available for SCED dispatch for an hour in its COP prior to a DRUC or HRUC process execution, according to Section 5.3, ERCOT Security Sequence Responsibilities, will be considered self-committed for that hour. For purpose of Settlement, snapshot data will be used as specified in paragraph (2) of Section 5.3.

(11)	ERCOT shall issue RUC instructions to each QSE specifying its Resources that have been committed as a result of the RUC process.  ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the MIS Secure Area any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes.

[NPRR1239: Replace paragraph (11) above with the following upon system implementation:] (11) ERCOT shall issue RUC instructions to each QSE specifying its Resources that have been committed as a result of the RUC process. ERCOT shall, within one day after making any changes to the RUC-recommended commitments, post to the ERCOT website any changes that ERCOT made to the RUC-recommended commitments with an explanation of the changes.

(12)	ERCOT shall use the RUC process to evaluate the need to commit Resources for which a QSE has submitted Three-Part Supply Offers and other available Off-Line Resources in addition to Resources that are planned to be On-Line during the RUC Study Period.  All of the above commitment information must be as specified in the QSE’s COP.  For available Off-Line Resources with a cold start time of one hour or less that have not been removed from special consideration under paragraph (17) below pursuant to paragraph (3) of Section 8.1.2, Current Operating Plan (COP) Performance Requirements, the Startup Offers and Minimum-Energy Offer from a Resource’s Three-Part Supply Offer shall not be used in the RUC process.

(13)	ERCOT shall create Three-Part Supply Offers for all Resources that did not submit a Three-Part Supply Offer, but are specified as available but Off-Line, excluding Resources with a Resource Status of EMR, in a QSE’s COP.  For such Resources, excluding available Off-Line Resources with a cold start time of one hour or less that have not been removed from special consideration under paragraph (16) below pursuant to paragraph (3) of Section 8.1.2, ERCOT shall use in the RUC process 100% of any approved verifiable Startup Cost and verifiable minimum-energy cost or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and the applicable Resource Category Generic Minimum-Energy Offer Cost as described specified in Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, registered with ERCOT.  Also, for Settlement purposes, ERCOT shall use any approved verifiable Startup Costs and verifiable minimum-energy cost for such Resources, or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and Generic Minimum-Energy Offer Cost.

(14)	A QSE shall notify the ERCOT Operator of any physical limitation that impacts its Resource’s ability to start that is not reflected in the Resource’s COP or the Resource’s startup time, minimum On-Line time, or minimum Off-Line time.  The following shall apply:

(a)	If a Resource receives a RUC Dispatch Instruction that it cannot meet due to a physical limitation described in paragraph (5) above, the QSE representing the Resource shall notify the ERCOT Operator of the inability to fully comply with the instruction and shall comply with the instruction to the best of the Resource’s ability.  If the QSE has provided the ERCOT Operator notice of that limitation at least seven days prior to the Operating Day in which the instruction occurs, the QSE shall be excused from complying with the portion of the RUC Dispatch Instruction that it could not meet due to the identified limitation.

(b)	If a QSE provides notice pursuant to paragraph (a) above of a physical limitation that will delay the RUC-committed Resource’s ability to reach its LSL in accordance with a RUC Dispatch Instruction, ERCOT shall extend the RUC Dispatch Instruction so that the Resource’s minimum run time is respected. However, if the Resource will not be available in time to address the issue for which it received the RUC instruction, ERCOT may instead cancel the RUC Dispatch Instruction.

(15)	A QSE shall be excused from complying with any portion of a RUC Dispatch Instruction that it could not meet due to a physical limitation that was reflected, at the time of the RUC Dispatch Instruction, in the Resource’s COP, startup time, minimum On-Line time, or minimum Off-Line time.

(16)	To determine the projected energy output level of each Resource and to project potential congestion patterns for each hour of the RUC, ERCOT shall calculate proxy Energy Offer Curves based on the Mitigated Offer Caps (MOCs) for the type of Resource as specified in Section 4.4.9.4, Mitigated Offer Cap and Mitigated Offer Floor, for use in the RUC.  Proxy Energy Offer Curves are calculated by multiplying the MOC by a constant selected by ERCOT from time to time that is no more than 0.10% and applying the cost for all Generation Resource output between High Sustained Limit (HSL) and LSL.  The intent of this process is to minimize the effect of the proxy Energy Offer Curves on optimization.  For ESRs, energy dispatch costs are not considered in determining projected energy output levels.

(17)	ERCOT shall calculate proxy Ancillary Service Offer Curves for use in RUC based on validated Ancillary Service Offers as specified in Section 4.4.7.2, Ancillary Service Offers.  For all Resources that do not have a valid Ancillary Service Offer but are qualified to provide an Ancillary Service, ERCOT shall create an Ancillary Service Offer Curve for use in RUC as described in Section 6.5.7.3, Security Constrained Economic Dispatch.  Proxy Ancillary Service Offer Curves for use in RUC are calculated by multiplying the Ancillary Service Offer by a constant selected by ERCOT from time to time that is no more than 0.1%, and are extended between the HSL and LSL.  Notwithstanding the presence or absence of a proxy Ancillary Service Offer, Ancillary Service provision in RUC shall be limited by the Resource’s Ancillary Service capabilities as reflected in the COP.  For ESRs, Ancillary Service Offer costs are not considered in determining projected Ancillary Service awards.

(18)	For all available Off-Line Resources having a cold start time of one hour or less and not removed from special consideration pursuant to paragraph (3) of Section 8.1.2, ERCOT shall scale any approved verifiable Startup Cost and verifiable minimum-energy cost or if verifiable costs have not been approved, the applicable Resource Category Generic Startup Offer Cost and the applicable Resource Category Generic Minimum-Energy Offer Cost as specified in Section 4.4.9.2.3 for use in the RUC process.

The above parameter is defined as follows:

| Parameter | Unit | Current Value* |
| --- | --- | --- |
| 1HRLESSCOSTSCALING | Percentage | Maximum value of 100% |
| * The current value for the parameter(s) referenced in this table above will be recommended by the Technical Advisory Committee (TAC) and the ERCOT Board and approved by the Public Utility Commission of Texas (PUCT). ERCOT shall update parameter value(s) on the first day of the month following PUCT approval unless otherwise directed. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value. |  |  |

(19)	Factors included in the RUC process are:

(a)	ERCOT System-wide hourly Load forecast allocated appropriately over Load buses;

(b)	ERCOT’s Ancillary Service Plans in the form of ASDCs;

(c)	Transmission constraints – Transfer limits on energy flows through the electricity network;

(i)	Thermal constraints – protect transmission facilities against thermal overload;

(ii)	Generic constraints – protect the transmission system against transient instability, dynamic instability or voltage collapse;

(d)	Planned transmission topology;

(e)	Energy sufficiency constraints, including RUC duration requirements for energy and Ancillary Services;

(f)	Inputs from the COP, as appropriate;

(g)	Inputs from Resource Parameters, including a list of Off-Line Available Resources having a start-up time of one hour or less, as appropriate;

(h)	Each Generation Resource’s Minimum-Energy Offer and Startup Offer, from its Three-Part Supply Offer;

(i)	Any Generation Resource that is Off-Line and available but does not have a Three-Part Supply Offer;

(j)	Forced Outage information;

(k)	Inputs from the eight-day look ahead planning tool, which may potentially keep a unit On-Line (or start a unit for the next day) so that a unit minimum duration between starts does not limit the availability of the unit (for security reasons); and

(l)	Ancillary Service Deployment Factors.

(20)	The HRUC process and the DRUC process are as follows:

(a)	The HRUC process uses current Resource Status for the initial condition for the first hour of the RUC Study Period.  All HRUC processes use the projected status of transmission breakers and switches starting with current status and updated for each remaining hour in the study as indicated in the COP for Resources and in the Outage Scheduler for transmission elements.

(b)	The DRUC process uses the Day-Ahead forecast of total ERCOT Load including DC Tie Schedules for each hour of the Operating Day.  The HRUC process uses the current hourly forecast of total ERCOT Load including DC Tie Schedules for each hour in the RUC Study Period.

[NPRR1032: Replace paragraph (b) above with the following upon system implementation:] (b) The DRUC process uses the current hourly forecast of total ERCOT Load including DC Tie Schedules up to the physical rating of the DC Tie for each hour of the Operating Day. The HRUC process uses the current hourly forecast of total ERCOT Load including DC Tie Schedules up to the physical rating of the DC Tie for each hour in the RUC Study Period.

(c)	The DRUC process uses the Day-Ahead weather forecast for each hour of the Operating Day.  The HRUC process uses the weather forecast information for each hour of the balance of the RUC Study Period.

(d)	For the HRUC, DRUC, and Weekly Reliability Unit Commitment (WRUC) processes, a feasibility check on the COP submitted HBSOC will be performed.  This check may adjust the HBSOC used in the RUC process.  The feasibility check looks sequentially across all intervals in the RUC Study Period to validate whether a particular interval’s COP HBSOC is achievable from the previous interval.  If it is not feasible, then RUC will adjust the HBSOC to the closest achievable value.

(21)	A QSE with a Resource that is not a Reliability Must-Run (RMR) Unit or has not received an Outage Schedule Adjustment (OSA) that has been committed in a DRUC or HRUC process may opt out of the RUC Settlement (or “buy back” the commitment) by setting the COP status of the RUC-committed Resource to ONOPTOUT for the first hour of a contiguous block of RUC-Committed Hours in the Opt Out Snapshot.  All the configurations of the same Combined Cycle Train shall be treated as the same Resource for the purpose of creating the block of RUC-Committed Hours.  A RUC-committed Combined Cycle Generation Resource may opt out of the RUC Settlement by setting the COP status of any Combined Cycle Generation Resource within the same Combined Cycle Train as the RUC-committed Resource to ONOPTOUT for the first hour of a contiguous block of RUC-Committed Hours in the Opt Out Snapshot.  A Combined Cycle Generation Resource that is RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity may opt out of the RUC Settlement following the same rule for RUC-committed Combined Cycle Generation Resources described above.  A QSE that opts out of RUC Settlement forfeits RUC Settlement for the affected Resource for a given block of RUC Buy-Back Hours.  A QSE that opts out of RUC Settlement treatment must make the Resource available to SCED for all RUC Buy-Back Hours.  All hours in a contiguous block of RUC-Committed Hours that includes the RUC Buy-Back Hour shall be considered RUC Buy-Back Hours.  If a contiguous block of RUC-Committed Hours spans more than one Operating Day and a QSE wishes to opt out of RUC Settlement for the RUC-Committed Hours in the second or subsequent Operating Day, the QSE must set its COP status to ONOPTOUT for the first hour of that the first Operating Day in the Opt Out Snapshot of the first Operating Day.

(22)	ERCOT shall, as soon as practicable, post to the MIS Secure Area a report identifying those hours that were considered RUC Buy-Back Hours, along with the name of each RUC-committed Resource whose QSE opted out of RUC Settlement.

[NPRR1239: Replace paragraph (22) above with the following upon system implementation:] (22) ERCOT shall, as soon as practicable, post to the ERCOT website a report identifying those hours that were considered RUC Buy-Back Hours, along with the name of each RUC-committed Resource whose QSE opted out of RUC Settlement.

(23)	A Resource that has a Three-Part Supply Offer cleared in the Day-Ahead Market (DAM) and subsequently receives a RUC commitment for the Operating Hour for which it was awarded will be treated as if the Resource Status was ONOPTOUT for purposes of Section 6.5.7.3 and Section 6.5.7.3.1, SCED Pricing Run.

(24)	A Resource that has self-committed for an Operating Hour after the RUC Snapshot was taken but before the RUC commitment has been communicated through an XML message for that RUC process and that Operating Hour is included in a block of RUC-committed hours for that RUC process will be treated as if the Resource Status was ONOPTOUT for purposes of Section 6.5.7.3, Section 6.5.7.3.1, and RUC Settlement for the entire block of RUC-committed hours.  A QSE that has a Resource that meets these conditions must make the Resource available to SCED for the entire block of RUC-committed hours.  ERCOT will send the QSE a notification stating the Operating Day and block of hours for which this occurred.

#### 5.7.1.3	Revenue Less Cost Above LSL During RUC-Committed Hours

(1)	The total revenue for a Resource operating above its LSL less the cost based on the Energy Offer Curve Cost Cap (as described in Section 4.4.9.3.3, Energy Offer Curve Cost Caps) during all RUC-Committed Hours of the Operating Day is Revenue Less Cost Above LSL During RUC-Committed Hours.

(2)	The LSL used to calculate Revenue Less Cost Above LSL During RUC-Committed Hours for a Combined Cycle Train is the LSL that corresponds to the Combined Cycle Generation Resource, within the Combined Cycle Train, that is RUC-committed for the hour.

(3)	For each RUC-committed Resource, Revenue Less Cost Above LSL During RUC-Committed Hours is calculated as follows:

RUCEXRR q, r, d   =   Max {0, [RUCEXRR96 q, r, i]}

Where,

RUCEXRR96 q, r, i  =	RTSPP p, i * Max (0, RTMG q, r, i – (LSL q, r, i * (¼)))

+ RTASREV q, r, i

+ (-1) * (VSSVARAMT q, r, i + VSSEAMT q, r, i)

+ (-1) * EMREAMT q, r, i  + (-1) * RDIGA q, r, i

– RTEOCOST q, r, i * Max (0, RTMG q, r, i – (LSL q, r, i * (¼)))]}

Where,

RTASREV q, r, i = RTRUREV q, r, i + RTRDREV q, r, i + RTRRREV q, r, i + RTECRREV q, r, i + RTNSREV q, r, i

[NPRR1140: Replace paragraph (3) above with the following upon system implementation:] (3) For each RUC-committed Resource, Revenue Less Cost Above LSL During RUC-Committed Hours is calculated as follows: If RUCFCA exists: RUCEXRR q, r, d = [RUCEXRR96 q, r, i] Otherwise: RUCEXRR q, r, d = Max {0, [RUCEXRR96 q, r, i]} Where, RUCEXRR96 q, r, i = RTSPP p, i * Max (0, RTMG q, r, i – (LSL q, r, i * (¼))) + RTASREV q, r, i + (-1) * (VSSVARAMT q, r, i + VSSEAMT q, r, i) + (-1) * EMREAMT q, r, i + (-1) * RDIGA q, r, i – (RTEOCOST q, r, i + RUCFCA q, r, i) * Max (0, RTMG q, r, i – (LSL q, r, i * (¼))) Where, RTASREV q, r, i = RTRUREV q, r, i + RTRDREV q, r, i + RTRRREV q, r, i + RTECRREV q, r, i + RTNSREV q, r, i And, RUCFCA q, r, i = Max(0, Volume-weighted average actual fuel price q, r, i * Average heat rate – RTEOCOST q, r, i)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCEXRR q, r, d | $ | Revenue Less Cost Above LSL During RUC-Committed Hours—The sum of the total revenue for Resource r represented by QSE q operating above its LSL less the cost during all RUC-Committed Hours, for the Operating Day d. When one or more Combined Cycle Generation Resources are committed by RUC, revenue less cost above LSL is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCEXRR96 q, r, i | $ | Revenue Less Cost Above LSL During RUC-Committed Hours by interval—The total revenue for Resource r represented by QSE q operating above its LSL less the cost during all RUC-Committed hours, for the Settlement Interval i. When one or more Combined Cycle Generation Resources are committed by RUC, revenue less cost above LSL is calculated for the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Resource Node Settlement Point p for the Settlement Interval i. |
| RTEOCOST q, r, i | $/MWh | Real-Time Energy Offer Curve Cost Cap¾The Energy Offer Curve Cost Cap for Resource r represented by QSE q, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.4.9.3.3. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation—The metered generation of Resource r represented by QSE q for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| [NPRR1140: Insert the variable “RUCFCA q, r, i” below upon system implementation:] RUCFCA q, r, i; $/MWh; Reliability Unit Commitment Fuel Cost Adder—For a QSE that has been granted a fuel dispute per Section 9.14.7, Disputes for RUC Make-Whole Payment for Fuel Costs, the fuel cost adder is calculated as the volume-weighted average actual fuel price times the output-level average heat rate for Resource r represented by QSE q, for the Resource’s generation above LSL, for the Settlement Interval i, minus the RTEOCOST. When one or more Combined Cycle Generation Resources are committed by RUC, RUCFCA is calculated for the Combined Cycle Train for all RUC-Committed Combined Cycle Generation Resources. The average heat rate for the Resource is the Average Heat Rate at the output level at Settlement Interval i, resulting from the input-output coefficients submitted with verifiable costs, if available, otherwise the heat rate value defined in Section 4.4.9.3.3. The volume-weighted average actual fuel price must be proven by the QSE by submitting a dispute per Section 9.14.7... |
| LSL q, r, i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTASREV q, r, i | $ | Real-Time Ancillary Service Revenue—The total Real-Time Ancillary Service revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTRUREV q, r, i | $ | Real-Time Reg-Up Revenue—The Real-Time Reg-Up revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2, Real-Time Ancillary Service Imbalance Payment or Charge. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTRDREV q, r, i | $ | Real-Time Reg-Down Revenue—The Real-Time Reg-Down revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTRRREV q, r, i | $ | Real-Time Responsive Reserve Revenue—The Real-Time RRS revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTNSREV q, r, i | $ | Real-Time Non-Spin Revenue—The Real-Time Non-Spin revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTECRREV q, r, i | $ | Real-Time ERCOT Contingency Reserve Service Revenue—The Real-Time ECRS revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| VSSVARAMT q, r, i | $ | Voltage Support Service VAr Amount—The payment to the QSE q for the Voltage Support Service (VSS) provided by Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. Payment for VSS is made to the Combined Cycle Train. |
| VSSEAMT q, r, i | $ | Voltage Support Service Energy Amount—The lost opportunity payment to the QSE q for ERCOT-directed VSS from the Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1. Payment for emergency energy is made to the Combined Cycle Train. |
| EMREAMT q, r, i | $ | Emergency Energy Amount—The payment to the QSE q as additional compensation for the additional energy or Ancillary Services produced or consumed by the Resource r in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. See Section 6.6.9.1, Payment for Emergency Operations Settlement. Payment for emergency energy is made to the Combined Cycle Train. |
| RDIGA q, r, i | $ | Reliability Deployment Indifference Amount per QSE per Generation Resource—The Reliability Deployment Indifference Payment to QSE q for Generation Resource r for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval within the hour that includes a RUC instruction. |

#### 5.7.1.4	Revenue Less Cost During QSE Clawback Intervals

(1)	The total revenue for a Resource less the cost based on the Energy Offer Curve Cost Cap as described in Section 4.4.9.3.3, Energy Offer Curve Cost Caps, during all QSE Clawback Intervals of the Operating Day is Revenue Less Cost During QSE-Clawback Intervals.

(2)	The MEPR and LSL used to calculate Revenue Less Cost During QSE Clawback Intervals for a Combined Cycle Train is the MEPR and LSL that corresponds to the Combined Cycle Generation Resource, within a Combined Cycle Train, that operates in Real-Time for the QSE Clawback Interval.

(3)	For each QSE Clawback Interval, Revenue Less Cost During QSE Clawback Intervals is calculated as follows:

RUCEXRQC q, r, d		=  Max {0, [(RTSPP p, i * RTMG q, r, i)

+ RTASREVq, r, i

+ (-1) * (VSSVARAMT q, r, i + VSSEAMT q, r, i)

+ (-1) * EMREAMT q, r, i  + (-1) * RDIGA q, r, i

– [MEPR q, r, i * Min (RTMG q, r, i, (LSL q, r, i * (¼)))]

– [RTEOCOST q, r, i * Max (0, RTMG q, r, i – (LSL q, r, i * (¼)))]]}

If the QSE submitted a validated Three-Part Supply Offer for the Resource,

Then, 		MEPR q, r, i	=	Min (MEO q, r, i, MECAP q, r, i)

Otherwise, 	MEPR q, r, i 	= 	MECAP q, r, i

If ERCOT has approved verifiable minimum-energy costs for the Resource,

Then,		MECAP q, r, i	=	verifiable minimum-energy costs q, r, i

Otherwise, 	MECAP q, r, i	= 	RCGMEC i

Where,

RTASREV q, r, i = RTRUREV q, r, i + RTRDREV q, r, i + RTRRREV q, r, i + RTECRREV q, r, i  +  RTNSREV q, r, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RUCEXRQC q, r, d | $ | Revenue Less Cost During QSE-Clawback Intervals—The sum of the total revenue for Resource r less the cost during all QSE-Clawback Intervals for the Operating Day. When one or more Combined Cycle Generation Resources are committed by RUC, Revenue Less Cost During QSE-Clawback Intervals is calculated for the Combined Cycle Train for all Combined Cycle Generation Resources earning revenue in QSE-Clawback Intervals. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price—The Real-Time Settlement Point Price at the Resource’s Settlement Point for the Settlement Interval i. |
| MEPR q, r, i | $/MWh | Minimum-Energy Price—The Settlement price for Resource r for minimum energy for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEO q, r, i | $/MWh | Minimum-Energy Offer—Represents an offer for the costs incurred by Resource r in producing energy at the Resource’s LSL for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MECAP q, r, i | $/MWh | Minimum-Energy Cap—The amount used for Resource r for minimum-energy costs. The minimum cost is the Resource Category Minimum-Energy Generic Cap (RCGMEC) unless ERCOT has approved verifiable unit-specific minimum energy costs for that Resource, in which case the Minimum-Energy Cap is the verifiable unit-specific minimum energy cost. See Section 5.6.1, Verifiable Costs, for more information on verifiable costs. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RCGMEC i | $/MWh | Resource Category Generic Minimum-Energy Cost—The Resource Category Generic Minimum-Energy Cost cap for the category of the Resource, according to Section 4.4.9.2.3, Startup Offer and Minimum-Energy Offer Generic Caps, for the Operating Day. |
| RTEOCOST q, r, i | $/MWh | Real-Time Energy Offer Curve Cost Cap¾The Energy Offer Curve Cost Cap for Resource r represented by QSE q, for the Resource’s generation above the LSL for the Settlement Interval i. See Section 4.4.9.3.3. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTMG q, r, i | MWh | Real-Time Metered Generation—The Resource r’s metered generation for the Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| LSL q, r, i | MW | Low Sustained Limit—The LSL of Generation Resource r represented by QSE q for the hour that includes the Settlement Interval i, as submitted in the COP. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTASREV q, r, i | $ | Real-Time Ancillary Service Revenue—The total Real-Time Ancillary Service revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTRUREV q, r, i | $ | Real-Time Reg-Up Revenue—The Real-Time Reg-Up revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2, Real-Time Ancillary Service Imbalance Payment or Charge. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTRDREV q, r, i | $ | Real-Time Reg-Down Revenue—The Real-Time Reg-Down revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTRRREV q, r, i | $ | Real-Time Responsive Reserve Revenue—The Real-Time RRS revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTNSREV q, r, i | $ | Real-Time Non-Spin Revenue—The Real-Time Non-Spin revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RTECRREV q, r, i | $ | Real-Time ERCOT Contingency Reserve Service Revenue—The Real-Time ECRS revenue for QSE q calculated for Resource r for the 15-minute Settlement Interval i. See Section 6.7.2. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| VSSVARAMT q, r, i | $ | Voltage Support Service VAr Amount—The payment to the QSE for the VSS provided by Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1, Voltage Support Service Payments. Payment for VSS is made to the Combined Cycle Train. |
| VSSEAMT q, r, i | $ | Voltage Support Service Energy Amount—The lost opportunity payment to the QSE for ERCOT-directed VSS from the Generation Resource r for the 15-minute Settlement Interval i. See Section 6.6.7.1. Payment for VSS is made to the Combined Cycle Train. |
| EMREAMT q, r, i | $ | Emergency Energy Amount—The payment to the QSE as additional compensation for the additional energy or Ancillary Services produced or consumed by the Resource r in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. See Section 6.6.9.1, Payment for Emergency Operations Settlement. Payment for emergency energy is made to the Combined Cycle Train. |
| RDIGA q, r, i | $ | Reliability Deployment Indifference Amount per QSE per Generation Resource—The Reliability Deployment Indifference Payment to QSE q for Generation Resource r for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A RUC-committed Generation Resource. |
| d | none | An Operating Day containing the RUC-commitment. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval within the hour that is identified as a QSE-Clawback Interval. |

### 6.3.2	Activities for Real-Time Operations

(1)	Activities for Real-Time operations begin at the end of the Adjustment Period and conclude at the close of the Operating Hour.

(2)	The following table summarizes the timeline for the Operating Period and the activities of QSEs and ERCOT during Real-Time operations where “T” represents any instant within the Operating Hour.  The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section:

| Operating Period | QSE Activities | ERCOT Activities |
| --- | --- | --- |
| During the first hour of the Operating Period |  | Execute the Hour-Ahead Sequence, including HRUC, beginning with the second hour of the Operating Period Review the list of Off-Line Available Resources with a start-up time of one hour or less Review and communicate HRUC commitments and Direct Current Tie (DC Tie) Schedule curtailments Snapshot the Scheduled Power Consumption for Controllable Load Resources (CLRs) |
| SCED run |  | Execute the SCED process. The SCED process involves executing the SCED Dispatch Run and, when reliability deployments are in effect, additionally executing the SCED Pricing Run. The binding Base Points and binding Ancillary Service awards are always from the SCED Dispatch Run. The binding Real-Time LMPs and Real-Time AS MCPCs are from the SCED Dispatch Run when there are no reliability deployments in effect. The binding Real-Time LMPs and Real-Time Ancillary Service MCPCs are from the SCED Pricing Run when reliability deployments are in effect, as described in Protocol 6.5.7.3.1, SCED Pricing Run. |
| During the Operating Hour | Telemeter next Operating Hour Ancillary Service Resource Responsibility for an ESR Acknowledge receipt of Dispatch Instructions Comply with Dispatch Instruction Review Resource Status to assure current state of the Resources is properly telemetered Update COP and telemetry with actual Resource Status and limits and Ancillary Service capabilities Submit and update Ancillary Service Offers Communicate Resource Forced Outages to ERCOT Submit and update Energy Offer Curves and/or RTM Energy Bids | Communicate using Inter-Control Center Communications Protocol (ICCP) or Verbal Dispatch Instructions (VDIs), all binding Base Points, Updated Desired Set Points (UDSPs), binding Real-Time Ancillary Service awards by Ancillary Service sub-type, where applicable, Dispatch Instructions, binding Real-Time LMPs for energy, and binding Real-Time MCPCs for Ancillary Services, and when the SCED Pricing Run is active as described in Section 6.5.7.3.1 SCED Pricing Run, the total Reliability Unit Commitment (RUC)/Reliability Must-Run (RMR) MW relaxed total Load Resource MW deployed that is added to the Demand, total Emergency Response Service (ERS) MW deployed that is added to the Demand, total emergency DC Tie MW that is added to or subtracted from the Demand, total Block Load Transfer (BLT) MW that is added to or subtracted from the Demand, total deployed TDSP standard offer Load Management MW added to demand, total deployed distribution voltage reduction MW added to demand, total deployed offline NSPIN, total deployed VECL MW added to demand, total deployed MW added to demand from load and Settlement-Only Resources under Section 6.5.7.3.1 (1)(j) and (1)(k) respectively . Monitor Resource Status and identify discrepancies between COP and telemetered Resource Status Restart Real-Time Sequence on major change of Resource or Transmission Element Status Monitor ERCOT total system capacity providing Ancillary Services Monitor ESR State of Charge (SOC) information to ensure Ancillary Service Resource Responsibilities can be met Validate COP information Validate Ancillary Service Trades Monitor ERCOT control performance Distribute by ICCP, and post on the ERCOT website, binding Real-Time System Lambda and the binding Real-Time LMPs for each Resource Node, Load Zone and Hub, the binding Real-Time MCPCs for each Ancillary Service, and when the SCED Pricing Run is active as described in Section 6.5.7.3.1 SCED Pricing Run, the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to the Demand, total ERS MW deployed that is added to the Demand, total emergency DC Tie MW that is added to or subtracted from the Demand, total BLT MW that is added to or subtracted from the Demand, total deployed TDSP standard offer Load Management MW added to demand, total deployed distribution voltage reduction MW added to demand, total deployed offline NSPIN, total deployed VECL MW added to demand, total deployed MW added to demand from load and Settlement-Only Resources under Section 6.5.7.3.1 (1)(j) and (1)(k) respectively, for each SCED process. For the posting on the ERCOT website of the binding prices (System Lambda, LMPs and AS MCPCs), there will be a flag indicating whether the binding prices are from the SCED Pricing Run. This data shall be posted immediately subsequent to deployment of Base Points and Ancillary Service awards from SCED with the time stamp the prices are effective Post on the ERCOT website the binding nodal prices for Settlement Only Distribution Generators (SODGs) and Settlement Only Transmission Generator (SOTGs). For the posting of these binding nodal prices on the ERCOT website, there will be a flag indicating whether the binding prices are from the SCED Pricing Run. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post the binding LMPs for each Electrical Bus on the ERCOT website. For the posting of these binding LMPs for each Electrical Bus on the ERCOT website, there will be a flag indicating whether the binding prices are from the SCED Pricing Run. These prices shall be posted immediately subsequent to deployment of Base Points from each binding SCED with the time stamp the prices are effective Post on the ERCOT website the indicative LMPs for each Resource Node, Hub LMPs and Load Zone LMPs, and Real-Time MCPCs for each Ancillary Service created by the SCED process described in paragraph (15) of Protocol 6.5.7.3 (price data is either from the indicative SCED Dispatch Run when reliability deployments are not in effect or from the indicative SCED Pricing Run when reliability deployments are in effect), and, when indicative SCED Pricing Run is in effect , shall include the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to Demand, total emergency DC Tie MW that is added to or subtracted from the Demand, total BLT MW that is added to or subtracted from the Demand, total ERS MW deployed that is added to the Demand, total deployed TDSP standard offer Load Management MW added to demand, total deployed distribution voltage reduction MW added to demand, total deployed Off-Line Non-Spin, total deployed VECL MW added to demand, total deployed MW added to demand from load and Settlement-Only Resources under Section 6.5.7.3.1 (1)(j) and (1)(k) respectively,. These projected indicative prices and pricing run data shall be posted at a frequency of every five minutes from SCED for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections Post on the MIS Certified Area the indicative Base Points and Ancillary Service awards by Ancillary Service sub-type for each Resource created the SCED process described in paragraph (15) of Protocol 6.5.7.3 (output from the indicative SCED Dispatch Run). These indicative Base Points shall be posted at a frequency of every five minutes for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections. Post each hour on the ERCOT website binding SCED Shadow Prices and active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs) For the posting of these binding SCED Shadow Prices and active binding transmission constraints, there will be a flag indicating whether the binding Shadow Prices are from the SCED Pricing Run. Post each hour on the MIS Certified Area, the Shift Factors of all active transmission constraints, including Private Use Network Settlement Points, by Resource Node, Hub, Load Zone, and DC Tie Post on the ERCOT website the Settlement Point Prices for each Settlement Point, the 15- minute Real-Time price for each SODG and SOTG, and Settlement Interval MCPCs for Ancillary Services immediately following the end of each Settlement Interval Post on the ERCOT website, from each SCED Dispatch Run of the SCED process: SCED Dispatch Run System Lambda, SCED Dispatch Run Step 2 LMPs at each Resource Node, Load Zone and Hub, SCED Dispatch Run Step 2 LMPs at each Electrical Bus, SCED Dispatch Run Step 2 Ancillary Service MCPCs, SCED Dispatch Run Step 2 Shadow Prices for active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs), and SCED Dispatch Run Step 2 LMP for SODG and SOTG Post on the MIS Certified Area, from each active SCED Pricing Run of the SCED process when the prices from the SCED Pricing Run are binding: SCED Pricing Run Step 2 Base Point, and SCED Pricing Run Step 2 Ancillary Service awards by sub-type where applicable |

[NPRR829, NPRR904, NPRR995, NPRR1006, NPRR1077, NPRR1226, and NPRR1253: Replace applicable portions of paragraph (2) above with the following upon system implementation:] (2) The following table summarizes the timeline for the Operating Period and the activities of QSEs and ERCOT during Real-Time operations where “T” represents any instant within the Operating Hour. The table is intended to be only a general guide and not controlling language, and any conflict between this table and another section of the Protocols is controlled by the other section: Operating Period; QSE Activities; ERCOT Activities. During the first hour of the Operating Period; Execute the Hour-Ahead Sequence, including HRUC, beginning with the second hour of the Operating Period Review the list of Off-Line Available Resources with a start-up time of one hour or less Review and communicate HRUC commitments and Direct Current Tie (DC Tie) Schedule curtailments Snapshot the Scheduled Power Consumption for Controllable Load Resources. SCED run; Execute the SCED process. The SCED process involves executing the SCED Dispatch Run and, when reliability deployments are in effect, additionally executing the SCED Pricing Run. The binding Base Points and binding Ancillary Service awards are always from the SCED Dispatch Run. The binding Real-Time LMPs and Real-Time AS MCPCs are from the SCED Dispatch Run when there are no reliability deployments in effect. The binding Real-Time LMPs and Real-Time Ancillary Service MCPCs are from the SCED Pricing Run when reliability deployments are in effect, as described in Protocol 6.5.7.3.1, SCED Pricing Run.. During the Operating Hour; Acknowledge receipt of Dispatch Instructions Comply with Dispatch Instruction Review Resource Status to assure current state of the Resources is properly telemetered Update COP and telemetry with actual Resource Status and limits and Ancillary Service capabilities Submit and update Ancillary Service Offers Communicate Resource Forced Outages to ERCOT Submit and update Energy Offer Curves and/or RTM Energy Bids; Communicate using Inter-Control Center Communications Protocol (ICCP) or Verbal Dispatch Instructions (VDIs), all binding Base Points, Updated Desired Set Points (UDSPs), binding Real-Time Ancillary Service awards by Ancillary Service sub-type, where applicable, Dispatch Instructions, binding Real-Time LMPs for energy, binding Real-Time MCPCs for Ancillary Services, and when the SCED Pricing Run is active as described in Section 6.5.7.3.1 SCED Pricing Run, SCED Pricing Run, the total Reliability Unit Commitment (RUC)/Reliability Must-Run (RMR) MW relaxed, total Load Resource MW deployed that is added to the Demand, total Transmission and/or Distribution Service Provider (TDSP) standard offer Load management MW deployed that is added to the Demand, total Emergency Response Service (ERS) MW deployed that is added to the Demand, total ERCOT-directed DC Tie MW that is added to or subtracted from the Demand, total Block Load Transfer (BLT) MW that is added to or subtracted from the Demand, total deployed TDSP standard offer Load Management MW added to demand, total deployed distribution voltage reduction MW added to demand, total deployed Off-Line Non-Spin, total deployed VECL MW added to demand, total deployed MW added to demand from load and Settlement-Only Resources under Section 6.5.7.3.1 (1)(j) and (1)(k) respectively. Monitor Resource Status and identify discrepancies between COP and telemetered Resource Status Restart Real-Time Sequence on major change of Resource or Transmission Element Status Monitor ERCOT total system capacity providing Ancillary Services Validate COP information Validate Ancillary Service Trades Monitor ERCOT control performance Distribute by ICCP, and post on the ERCOT website, binding Real-Time System Lambda and the binding Real-Time LMPs for each Resource Node, Load Zone and Hub, and the binding Real-Time MCPCs for each Ancillary Service, and when the SCED Pricing Run is active as described in Section 6.5.7.3.1 SCED Pricing Run, the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to the Demand, total ERS MW deployed that is added to the Demand, total TDSP standard offer Load management MW deployed that is added to the Demand, total ERCOT-directed DC Tie MW that is added to or subtracted from the Demand, total BLT MW that is added to or subtracted from the Demand, total deployed TDSP standard offer Load Management MW added to demand, total deployed distribution voltage reduction MW added to demand, total deployed Off-Line Non-Spin, total deployed VECL MW added to demand, total deployed MW added to demand from load and Settlement-Only Resources under Section 6.5.7.3.1 (1)(j) and (1)(k) respectively, and total ESR charging created for each SCED Dispatch Run as part of the SCED process, and aggregated data from the estimated Demand response data process as described in Section 6.5.7.1.13, Data Inputs and Outputs for the Real-Time Sequence and SCED. For the posting on the ERCOT website of the binding prices (System Lambda, LMPs and AS MCPCs), there will be a flag indicating whether the binding prices are from the SCED Pricing Run. This data shall be posted immediately subsequent to deployment of Base Points and Ancillary Service awards from SCED with the time stamp the data are effective Post on the ERCOT website the binding nodal prices for Settlement Only Distribution Generators (SODGs), Settlement Only Distribution Energy Storage Systems (SODESSs), Settlement Only Transmission Generators (SOTGs), and Settlement Only Transmission Energy Storage Systems (SOTESSs). For the posting of these binding nodal prices on the ERCOT website, there will be a flag indicating whether the binding prices are from the SCED Pricing Run. These prices shall be posted immediately subsequent to deployment of Base Points from SCED with the time stamp the prices are effective Post binding LMPs for each Electrical Bus on the ERCOT website. For the posting of these binding LMPs for each Electrical Bus on the ERCOT website, there will be a flag indicating whether the binding prices are from the SCED Pricing Run. These prices shall be posted immediately subsequent to deployment of Base Points from each binding SCED with the time stamp the prices are effective Post every 15 minutes on the ERCOT website the aggregate net injection from Settlement Only Generators (SOGs) and Settlement Only Energy Storage Systems (SOESSs) Post on the ERCOT website the indicative LMPs for each Resource Node, Hub LMPs and Load Zone LMPs, and Real-Time MCPCs for each Ancillary Service created by the SCED process described in paragraph (15) of Protocol 6.5.7.3 (price data is either from the indicative SCED Dispatch Run when reliability deployments are not in effect or from the indicative SCED Pricing Run when reliability deployments are in effect), and, when indicative SCED Pricing Run is in effect, shall include the total RUC/RMR MW relaxed, total Load Resource MW deployed that is added to Demand, total TDSP standard offer Load management MW deployed that is added to the Demand, total ERCOT-directed DC Tie MW that is added to or subtracted from the Demand, total BLT MW that is added to or subtracted from the Demand, total ERS MW deployed that are deployed that is added to the Demand, total deployed TDSP standard offer Load Management MW added to demand, total deployed distribution voltage reduction MW added to demand, total deployed Off-Line Non-Spin, total deployed VECL MW added to demand, total deployed MW added to demand from load and Settlement-Only Resources under Section 6.5.7.3.1 (1)(j) and (1)(k) respectively, These projected indicative prices and indicative SCED Pricing Run data described above shall be posted at a frequency of every five minutes from SCED for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections Post on the MIS Certified Area the indicative Base Points and Ancillary Service awards by Ancillary Service sub-type for each Resource created by the SCED process described in paragraph (15) of Section 6.5.7.3 (output from the indicative SCED Dispatch Run). These indicative Base Points shall be posted at a frequency of every five minutes for at least 15 minutes in the future with the time stamp of the SCED process that produced the projections. Post each hour on the ERCOT website binding SCED Shadow Prices and active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs) For the posting of these binding SCED Shadow Prices and active binding transmission constraints, there will be a flag indicating whether the binding Shadow Prices are from the SCED Pricing Run. Post each hour on the MIS Certified Area, the Shift Factors of all active transmission constraints, including Private Use Network Settlement Points, by Resource Node, Hub, Load Zone, and DC Tie Post on the ERCOT website, the Settlement Point Prices for each Settlement Point and the 15-min Real-Time price for each SODG, SODESS, SOTG, and SOTESS immediately following the end of each Settlement Interval Post on the ERCOT website, from each SCED Dispatch Run of the SCED process: SCED Dispatch Run Step 2 System Lambda, SCED Dispatch Run Step 2 LMPs at each Resource Node, Load Zone and Hub, SCED Dispatch Run LMPs at each Electrical Bus, SCED Dispatch Run Step 2 Ancillary Service MCPCs, SCED Dispatch Run Step 2 Shadow Prices for active binding transmission constraints by Transmission Element name (contingency /overloaded element pairs), and SCED Dispatch Run Step 2 LMP for SODG and SOTG Post on the MIS Certified Area, from each active SCED Pricing Run of the SCED process when the prices from the SCED Pricing Run are binding: SCED Pricing Run Step 2 Base Point, and SCED Pricing Run Step 2 Ancillary Service awards by sub-type where applicable.

(3)	At the beginning of each hour, ERCOT shall post on the ERCOT website the following information:

(a)	Changes in ERCOT System conditions that could affect the security and dynamic transmission limits of the ERCOT System, including:

(i)	Changes or expected changes, in the status of Transmission Facilities as recorded in the Outage Scheduler for the remaining hours of the current Operating Day and all hours of the next Operating Day; and

(ii)	Any conditions such as adverse weather conditions as determined from the ERCOT-designated weather service;

(b)	Updated system-wide Mid-Term Load Forecasts (MTLFs) for all forecast models available to ERCOT Operations, as well as an indicator for which forecast was in use by ERCOT at the time of publication;

(c)	The quantities of RMR Services deployed by ERCOT for each previous hour of the current Operating Day; and

(d)	Total ERCOT System Demand, from Real-Time operations, integrated over each Settlement Interval.

(4)	No later than 0600, ERCOT shall post on the ERCOT website the actual system Load by Weather Zone, the actual system Load by Forecast Zone, and the actual system Load by Study Area for each hour of the previous Operating Day.

(5)	ERCOT shall provide notification to the market and post on the ERCOT website Electrical Bus Load distribution factors and other information necessary to forecast Electrical Bus Loads.  This report will be published when updates to the Load distribution factors are made.  Private Use Network net Load will be redacted from this posting.

(6)	After every SCED run, ERCOT shall post to the ERCOT website the total capability of Resources available to provide the following Ancillary Service combinations, based on the Resource telemetry from the QSE and capped by the limits of the Resource and, for ESRs, further capped by Ancillary Service SCED duration requirements and current available State of Charge (SOC), for the most recent SCED execution:

(a)	Capacity to provide Reg-Up, irrespective of whether it is capable of providing any other Ancillary Service;

(b)	Capacity to provide RRS, irrespective of whether it is capable of providing any other Ancillary Service;

(c)	Capacity to provide ECRS, irrespective of whether it is capable of providing any other Ancillary Service;

(d)	Capacity to provide Non-Spin, irrespective of whether it is capable of providing any other Ancillary Service;

(e)	Capacity to provide Reg-Up, RRS, or both, irrespective of whether it is capable of providing ECRS or Non-Spin;

(f)	Capacity to provide Reg-Up, RRS, ECRS, or any combination, irrespective of whether it is capable of providing Non-Spin;

(g)	Capacity to provide Reg-Up, RRS, ECRS, Non-Spin, or any combination; and

(h)	Capacity to provide Reg-Down.

(7)	Each week, ERCOT shall post on the ERCOT website the historical SCED-interval data described in paragraph (6) above.

#### 6.5.7.3	Security Constrained Economic Dispatch

(1)	The SCED process is designed to simultaneously manage energy, Ancillary Services, the system power balance and network congestion through Resource Base Points, Ancillary Service awards, and the calculation of LMPs and Real-Time MCPCs approximately every five minutes, or more frequently if necessary.  The SCED process uses a two-step methodology that applies mitigation to offers for energy prospectively to resolve Non-Competitive Constraints for the current Operating Hour.  The SCED process evaluates Energy Offer Curves, Energy Bid/Offer Curves, Ancillary Service Offers, Output Schedules, and RTM Energy Bids to determine Resource Dispatch Instructions and Ancillary Service awards by maximizing bid-based revenues minus offer-based costs, subject to power balance, Ancillary Service Demand Curves (ASDCs), and network constraints.  The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.  In addition, the SCED process accounts for each ESR’s State of Charge (SOC) and SOC operating limits.  This is to ensure that the SCED process will issue ESR Base Points and Ancillary Services that are feasible taking into account SCED duration requirements for energy and Ancillary Services and also that do not violate the ESR’s Minimum State of Charge (MinSOC) and Maximum State of Charge (MaxSOC) limits. The SCED process involves executing the SCED Dispatch Run and, when reliability deployments are in effect, additionally executing the SCED Pricing Run.

(a)	SCED Dispatch Run:

(i)	Execute the two-step SCED methodology described in this section which does not consider the impact of reliability deployments.

(ii)	The binding Base Points and binding Ancillary Service Awards are always from the SCED Dispatch Run. These binding Base Points and Ancillary Service Awards are from the second step in the two-step SCED process of the SCED Dispatch Run as described in paragraph (14)(b) of Section 6.5.7.3, Security Constrained Economic Dispatch.

(iii)	The SCED Dispatch Run Real-Time LMPs and Real-Time Ancillary Service MCPCs are binding when there are no reliability deployments in effect. When no reliability deployments are in effect, these binding prices for energy and each Ancillary Service (LMP and Ancillary Service MCPCs) are issued from the second step in the two-step SCED process described in paragraph (14)(b) of Section 6.5.7.3.

(b)	SCED Pricing Run:

(i)	Execute the two-step SCED methodology described in this section which considers the impact of reliability deployments as described in Section 6.5.7.3.1, SCED Pricing Run.

(ii)	The SCED Pricing Run Step 2 Real-Time LMPs and Real-Time Ancillary Service MCPCs are binding when reliability deployments are in effect.

[NPRR1188: Replace paragraph (1) above with the following upon system implementation:] (1) The SCED process is designed to simultaneously manage energy, Ancillary Services, the system power balance and network congestion through Resource Base Points, Ancillary Service awards, and the calculation of LMPs and Real-Time MCPCs approximately every five minutes, or more frequently if necessary. The SCED process uses a two-step methodology that applies mitigation to offers for energy prospectively to resolve Non-Competitive Constraints for the current Operating Hour. The SCED process evaluates Energy Offer Curves, Energy Bid/Offer Curves, Ancillary Service Offers, Output Schedules, and Energy Bid Curves to determine Resource Dispatch Instructions and Ancillary Service awards by maximizing bid-based revenues minus offer-based costs, subject to power balance, Ancillary Service Demand Curves (ASDCs), and network constraints. The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP. In addition, the SCED process accounts for each ESR’s State of Charge (SOC) and SOC operating limits. This is to ensure that the SCED process will issue ESR Base Points and Ancillary Services that are feasible taking into account SCED duration requirements for energy and Ancillary Services and also that do not violate the ESR’s Minimum State of Charge (MinSOC) and Maximum State of Charge (MaxSOC) limits. The SCED process involves executing the SCED Dispatch Run and, when reliability deployments are in effect, additionally executing the SCED Pricing Run. (a) SCED Dispatch Run: (i) Execute the two-step SCED methodology described in this section which does not consider the impact of reliability deployments. (ii) The binding Base Points and binding Ancillary Service Awards are always from the SCED Dispatch Run. These binding Base Points and Ancillary Service Awards are from the second step in the two-step SCED process of the SCED Dispatch Run as described in paragraph (14)(b) of Section 6.5.7.3, Security Constrained Economic Dispatch. (iii) The SCED Dispatch Run Real-Time LMPs and Real-Time Ancillary Service MCPCs are binding when there are no reliability deployments in effect. When no reliability deployments are in effect, these binding prices for energy and each Ancillary Service (LMP and Ancillary Service MCPCs) are issued from the second step in the two-step SCED process described in paragraph (14)(b) of Section 6.5.7.3. (b) SCED Pricing Run: (i) Execute the two-step SCED methodology described in this Section which considers the impact of reliability deployments as described in Section 6.5.7.3.1, SCED Pricing Run. (ii) The SCED Pricing Run Step 2 Real-Time LMPs and Real-Time Ancillary Service MCPCs are binding when reliability deployments are in effect.

(2)	The SCED solution must monitor cumulative deployment of Regulation Services and ensure that Regulation Services deployment is minimized over time.

(3)	In the Generation To Be Dispatched (GTBD) determined by LFC, ERCOT shall subtract the sum of the telemetered net real power consumption from all CLRs available to SCED.

(4)	For use as SCED inputs for determining energy dispatch and Ancillary Service awards, ERCOT shall use the available capacity of all committed Generation Resources by creating proxy Energy Offer Curves for certain Resources as follows:

(a)	Non-IRRs without Energy Offer Curves

(i)	ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below for:

(A)	Each non-IRR for which its QSE has submitted an Output Schedule instead of an Energy Offer Curve.

| MW | Price (per MWh) |
| --- | --- |
| HSL | RTSWCAP |
| Output Schedule MW plus 1 MW | RTSWCAP minus $0.01 |
| Output Schedule MW | -$249.99 |
| LSL | -$250.00 |

(b)	Non-IRRs without full-range Energy Offer Curves

(i)	For each non-IRR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the Resource’s available capacity, ERCOT shall create a proxy Energy Offer Curve that extends the submitted Energy Offer Curve to use the entire available capacity of the Resource above the highest point on the Energy Offer Curve to the Resource’s HSL and the offer floor from the lowest point on the Energy Offer Curve to its LSL, using these points:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(c)	IRRs

(i)	For each IRR that has not submitted an Energy Offer Curve, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | $1,500 |
| HSL minus 1 MW | -$249.99 |
| LSL | -$250.00 |

(ii)	For each IRR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the IRR’s available capacity, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with the highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(d)	RUC-committed Resources

(i)        For each RUC-committed Resource that has not submitted an Energy Offer Curve, ERCOT shall create a proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | $250 |
| Zero | $250 |

(ii)       For each RUC-committed Resource that has submitted an Energy Offer Curve, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in Energy Offer Curve) | Greater of $250 or price associated with the highest MW in QSE submitted Energy Offer Curve |
| Energy Offer Curve | Greater of $250 or the QSE submitted Energy Offer Curve |
| Zero | Greater of $250 or the first price point of the QSE submitted Energy Offer Curve |

[NPRR930: Insert paragraph (iii) below upon system implementation and renumber accordingly:] (iii) For each RUC-committed Resource during the time period stated in the Advance Action Notice (AAN) if any Resource received an Outage Schedule Adjustment, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL; $4,500 or the effective Value of Lost Load (VOLL), whichever is less.. Zero; $4,500 or the effective VOLL, whichever is less..

(iii) 	For each Combined Cycle Generation Resource that was RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity, as instructed by ERCOT, that has not submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL of RUC-committed configuration | $250 |
| Zero | $250 |

(iv)	For each Combined Cycle Generation Resource that was RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity, as instructed by ERCOT, that has submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL of RUC-committed configuration (if more than highest MW in Energy Offer Curve) | Greater of $250 or price associated with the highest MW in QSE submitted Energy Offer Curve |
| Energy Offer Curve for MW at and above HSL of QSE-committed configuration | Greater of $250 or the QSE submitted Energy Offer Curve |
| HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve and price associated with highest MW in Energy Offer Curve is less than $250) | $250 |
| HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve) | Price associated with the highest MW in QSE submitted Energy Offer Curve |
| Energy Offer Curve for MW at and below HSL of QSE-committed configuration | The QSE submitted Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

[NPRR1019: Insert paragraphs (v)-(viii) below upon system implementation:] (v) For each RUC-committed Switchable Generation Resource (SWGR) that is not part of a Combined Cycle Train already operating in ERCOT, that has not submitted an Energy Offer Curve, and that has a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of the RUC instruction, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL; $4,500 or the effective Value of Lost Load (VOLL), whichever is less. Zero; $4,500 or the effective VOLL, whichever is less. (vi) For each RUC-committed SWGR that is not part of a Combined Cycle Train already operating in ERCOT, that has submitted an Energy Offer Curve, and that has a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of the RUC instruction, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL (if more than highest MW in Energy Offer Curve); Greater of: $4,500 or the effective VOLL, whichever is less; and the price associated with the highest MW in QSE-submitted Energy Offer Curve. Energy Offer Curve; Greater of: $4,500 or the effective VOLL, whichever is less; and the QSE-submitted Energy Offer Curve. Zero; Greater of: $4,500 or the effective VOLL, whichever is less; and the first price point of the QSE-submitted Energy Offer Curve. (vii) For each Combined Cycle Train configuration that includes at least one SWGR that is operating in a non-ERCOT Control Area as part of a configuration with a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of a RUC instruction requiring the switching of the SWGR into the ERCOT Control Area, if the QSE for the Combined Cycle Train has not submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL of RUC-committed configuration; $4,500 or the effective VOLL, whichever is less. Zero; $4,500 or the effective VOLL, whichever is less. (viii) For each Combined Cycle Train configuration that includes at least one SWGR that is operating in a non-ERCOT Control Area as part of a configuration with a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of a RUC instruction requiring the switching of the SWGR into the ERCOT Control Area, if the QSE for the Combined Cycle Train has submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL of RUC-committed configuration (if more than highest MW in Energy Offer Curve); Greater of: $4,500 or the effective VOLL, whichever is less; and the price associated with the highest MW in QSE-submitted Energy Offer Curve. Energy Offer Curve for MW at and above HSL of QSE-committed configuration; Greater of: $4,500 or the effective VOLL, whichever is less; and the QSE-submitted Energy Offer Curve. HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve and price associated with highest MW in Energy Offer Curve is less than $4,500); $4,500 or the effective VOLL, whichever is less. HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve); Price associated with the highest MW in QSE-submitted Energy Offer Curve. Energy Offer Curve for MW at and below HSL of QSE-committed configuration; The QSE-submitted Energy Offer Curve. 1 MW below lowest MW in Energy Offer Curve (if more than LSL); -$249.99. LSL (if less than lowest MW in Energy Offer Curve); -$250.00.

(5)	For use as SCED inputs for determining energy dispatch and Ancillary Service awards, ERCOT shall use the available Ancillary Service MW capacity of all Resources by creating a proxy Ancillary Service Offer for qualified Resources as follows:

(a)	The proxy Ancillary Service Offer shall be a linked Ancillary Service Offer across all Ancillary Service products for which a Resource is qualified to provide.  For Generation Resources, the proxy Ancillary Service Offer MW shall be equal to the Resource’s telemetered HSL.  For ESRs, the proxy Ancillary Service Offer MW shall be equal to the difference between the Resource’s telemetered HSL and LSL.  For Load Resources, the proxy Ancillary Service Offer MW shall be equal to the Resource’s telemetered Maximum Power Consumption (MPC).

(b)	For Resources that are not RUC-committed, the price in the proxy Ancillary Service Offer shall be set to:

(i)	For Reg-Up and RRS, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for Reg-Up or RRS, respectively;

(B)	The Resource’s highest submitted Ancillary Service Offer price for Reg-Up or RRS, respectively;

(C)	The Resource’s highest Ancillary Service Offer price for ECRS (submitted or proxy); or

(D)	The Resource’s highest Ancillary Service Offer price for Non-Spin (submitted or proxy).

(ii)	For ECRS, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for ECRS;

(B)	The Resource’s highest submitted Ancillary Service Offer price for ECRS; or

(C)	The Resource’s highest Ancillary Service Offer price for Non-Spin (submitted or proxy).

(iii)	For Non-Spin, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for Non-Spin; or

(B)	The Resource’s highest submitted Ancillary Service Offer price for Non-Spin.

(iv)	For Reg-Down, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for Reg-Down; or

(B)	The Resource’s highest submitted Ancillary Service Offer price for Reg-Down.

(c)	The proxy Ancillary Service Offer price floors for each SCED-interval shall be derived from the effective ASDCs and Ancillary Service Plan using the following logic:

(i)        The proxy Ancillary Service Offer price floor for Reg-Up is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for Reg-Up that intersects with a quantity that is 95% of the Ancillary Service Plan for Reg-Up.

(ii)       The proxy Ancillary Service Offer price floor for RRS is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for RRS that intersects with a quantity that is 95% of the Ancillary Service Plan for RRS.

(iii)      The proxy Ancillary Service Offer price floor for ECRS is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for ECRS that intersects with a quantity that is 95% of the Ancillary Service Plan for ECRS.

(iv)      The proxy Ancillary Service Offer price floor for Non-Spin is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for Non-Spin that intersects with a quantity that is 95% of the Ancillary Service Plan for Non-Spin.

(v)       The proxy Ancillary Service Offer price floor for Reg-Down is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for Reg-Down that intersects with a quantity that is 95% of the Ancillary Service Plan for Reg-Down.

(d)	ERCOT systems shall be designed to allow for proxy Ancillary Service Offer price floors to differ when the same Ancillary Service product can be provided by either On-Line or Off-Line Resources, and/or an Ancillary Service product has sub-types.

(e)	For RUC-committed Resources:

(i)	If a RUC-committed Resource does not have an Ancillary Service Offer for an Ancillary Service product that the Resource is qualified to provide, ERCOT shall create an Ancillary Service Offer for that Ancillary Service product at a value of $250 per MWh for the full operating range of the Resource up to its telemetered HSL.

(ii)	For each Ancillary Service product for which a RUC-committed Resource has an Ancillary Service Offer, the Ancillary Service Offer used by SCED for that Ancillary Service product across the full operating range of the Resource up to its telemetered HSL shall be the maximum of:

(A)	The Resource’s highest submitted Ancillary Service Offer price; or

(B)	$250 per MWh.

(6)	For use as SCED inputs for determining energy Dispatch and Ancillary Service awards, ERCOT shall use the available capacity of all On-Line ESRs by creating proxy Energy Bid/Offer Curves for certain Resources as follows:

(a)	For each ESR for which its QSE has submitted an Energy Bid/Offer Curve that does not cover the full offer range (LSL to HSL) of the Resource’s available capacity, ERCOT shall create a proxy Energy Bid/Offer Curve that extends the submitted Energy Bid/Offer Curve to use the entire available capacity of the Resource above the highest MW point on the Energy Bid/Offer Curve to the Resource’s HSL and from the lowest MW point on the Energy Bid/Offer Curve to LSL, using these prices for the corresponding MW segments:

| Scenario | MW Segment | Price (per MWh) |
| --- | --- | --- |
| HSL MW and the highest MW point on the Energy Bid/Offer are both greater than or equal to zero, and, HSL is greater than the highest MW in submitted Energy Bid/Offer Curve | From highest MW point on submitted Energy Bid/Offer Curve to HSL MW | RTSWCAP |
| HSL MW is greater than or equal to zero, and, the highest MW point on the Energy Bid/Offer is less than zero | From highest MW point on submitted Energy Bid/Offer Curve to 0 MW From 0 MW to HSL | Price associated with the highest MW in submitted Energy Bid/Offer Curve RTSWCAP |
| HSL is less than zero and is also greater than the highest MW in submitted Energy Bid/Offer Curve | From highest MW point on submitted Energy Bid/Offer Curve to HSL MW | Price associated with the highest MW in submitted Energy Bid/Offer Curve |
| Energy Bid/Offer Curve |  | Energy Bid/Offer Curve |
| LSL MW and the lowest MW point on the Energy Bid/Offer Curve are both greater than or equal to zero, and, LSL is less than the lowest MW in submitted Energy Bid/Offer Curve | From LSL to lowest MW point on submitted Energy Bid/Offer Curve | Price associated with the lowest MW in submitted Energy Bid/Offer Curve |
| LSL MW is less than zero, and, the lowest MW point on the Energy Bid/Offer Curve is greater than zero | From LSL to 0 MW From 0 MW to lowest MW point on submitted Energy Bid/Offer Curve | -$250.00 Price associated with the lowest MW in submitted Energy Bid/Offer Curve |
| LSL and the lowest MW point on the Energy Bid/Offer Curve are both less than or equal to zero, and, LSL is less than the lowest MW point on the Energy Bid/Offer Curve | From LSL to lowest MW point on submitted Energy Bid/Offer Curve | -$250.00 |

(b)	At the time of SCED execution, if a valid Energy Bid/Offer Curve or Output Schedule does not exist for an ESR that has a status of On-Line, then ERCOT shall notify the QSE and create a proxy Energy Bid/Offer Curve priced at -$250/MWh for the MW portion of the curve less than zero MW, and priced at the RTSWCAP for the MW portion of the curve greater than zero MW.

(c)	At the time of SCED execution, if a QSE representing an ESR has submitted an Output Schedule instead of an Energy Bid/Offer Curve, ERCOT shall create a proxy Energy Bid/Offer Curve priced at -$250 per MWh for the MW portion of the curve from its LSL to the MW amount on the Output Schedule, and priced at the RTSWCAP for the MW portion of the curve from the MW amount on the Output Schedule to its HSL.

(7)	The Entity with decision-making authority, as more fully described in Section 3.19.1, Constraint Competitiveness Test Definitions, over how a Resource or Split Generation Resource is offered or scheduled, shall be responsible for all offers associated with each Resource, including offers represented by a proxy Energy Offer Curve, proxy Energy Bid/Offer Curve, or proxy Ancillary Service Offer.

(8)	For a CLR whose QSE has submitted an RTM Energy Bid that does not cover the full range of the Resource’s available Demand response capability, consistent with the CLR’s telemetered quantities, ERCOT shall create a proxy energy bid as described below:

| MW | Price (per MWh) |
| --- | --- |
| LPC to MPC minus maximum MW of RTM Energy Bid | Price associated with the lowest MW in submitted RTM Energy Bid curve |
| MPC minus maximum MW of RTM Energy Bid to MPC | RTM Energy Bid curve |
| MPC | Right-most point (lowest price) on RTM Energy Bid curve |

[NPRR1188: Replace paragraph (8) above with the following upon system implementation and renumber accordingly:] (8) For a CLR whose QSE has submitted an Energy Bid Curve that does not cover the full range of the Resource’s available Demand response capability, consistent with the CLR’s telemetered quantities, ERCOT shall create a proxy energy bid as described below: MW; Price (per MWh). LPC to MPC minus maximum MW of Energy Bid Curve; Price associated with the lowest MW in submitted Energy Bid Curve. MPC minus maximum MW of Energy Bid Curve to MPC; Energy Bid Curve. MPC; Right-most point (lowest price) on Energy Bid Curve. (9) For a CLR whose QSE has not submitted an Energy Bid Curve, consistent with the CLR’s telemetered quantities, ERCOT shall create a proxy Energy Bid Curve as described below: MW; Price (per MWh). LPC to MPC; Effective Value of Lost Load (VOLL).

(9)	ERCOT shall ensure that any RTM Energy Bid is monotonically non-increasing.  The QSE representing the CLR shall be responsible for all RTM Energy Bids, including bids updated by ERCOT as described above.

[NPRR1188: Replace paragraph (9) above with the following upon system implementation:] (9) ERCOT shall ensure that any Energy Bid Curve is monotonically non-increasing. The QSE representing the CLR shall be responsible for all Energy Bid Curves, including Energy Bid Curves updated by ERCOT as described above.

(10)	If a CLR telemeters a status of OUTL, it is not considered as dispatchable capacity by SCED.  A QSE may use this function to inform ERCOT of instances when the CLR is unable to follow SCED Dispatch Instructions.  Under all telemetered statuses, including OUTL, the remaining telemetry quantities submitted by the QSE shall represent the operating conditions of the CLR that can be verified by ERCOT.  A QSE representing a CLR with a telemetered status of OUTL is still obligated to provide any applicable Ancillary Services awarded to the Resource.  This paragraph does not apply to ESRs.

[NPRR1188: Replace paragraph (10) above with the following upon system implementation:] (10) A CLR may consume energy only when dispatched by SCED to do so. A CLR may telemeter a status of OUTL only if the Resource is Off-Line and unavailable with its energy consumption at zero. In instances when the CLR is unable to follow SCED Dispatch Instructions but still consumes energy, the CLR must submit a Resource Status of ONHOLD. Under all telemetered statuses, including OUTL, the remaining telemetry quantities submitted by the QSE shall represent the operating conditions of the CLR that can be verified by ERCOT. A QSE representing a CLR with a telemetered status of OUTL or ONHOLD is still obligated to provide any applicable Ancillary Services awarded to the Resource. This paragraph does not apply to ESRs.

(11)	Energy Offer Curves that were constructed in whole or in part with proxy Energy Offer Curves shall be so marked in all ERCOT postings or references to the energy offer.

(12)	SCED will enforce Resource-specific Ancillary Service constraints to ensure that Ancillary Service awards are aligned with a Resource’s qualifications and telemetered Ancillary Service capabilities.

(a)	A scaling factor of 5/7 shall be used for Reg-Up award when ensuring that the SCED Base Point plus the product of this scaling factor and the Reg-Up award does not exceed HDL.

(b)	A scaling factor of 5/7 shall be used for Reg-Down award when ensuring that the SCED Base Point minus the product of this scaling factor and the Reg-Down award does not go below LDL.

(13)	Energy Bid/Offer Curves that were constructed in whole or in part with proxy Energy Bid/Offer Curves shall be so marked in all ERCOT postings or references to the energy bid/offer.

(14)	The two-step SCED methodology referenced in paragraph (1) above is:

(a)	The first step is to execute the SCED process to determine Reference LMPs.  In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints in addition to power balance and Ancillary Service constraints.  Energy Offer Curves for all On-Line Generation Resources, Energy Bid/Offer Curves for all On-Line ESRs, and RTM Energy Bids from available CLRs, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

[NPRR1188: Replace paragraph (a) above with the following upon system implementation:] (a) The first step is to execute the SCED process to determine Reference LMPs. In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints in addition to power balance and Ancillary Service constraints. Energy Offer Curves for all On-Line Generation Resources, Energy Bid/Offer Curves for all On-Line ESRs, and Energy Bid Curves from available CLRs, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

(b)	The second step is to execute the SCED process to produce Base Points, Ancillary Service awards, Shadow Prices, Real-Time MCPCs, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints.  The second step must:

(i)	Use Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT.  Each Energy Offer Curve must be bounded at the lesser of the Reference LMP (from Step 1) or the appropriate Mitigated Offer Floor.  In addition, each Energy Offer Curve subject to mitigation under the criteria described in Section 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test, must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node plus a variable not to exceed 0.01 multiplied by the value of the Resource’s Mitigated Offer Cap (MOC) curve at the LSL or the appropriate MOC;

(ii)	Use Energy Bid/Offer Curves for all On-Line ESRs, whether submitted by QSEs or created by ERCOT.  Each Energy Bid/Offer Curve must be bounded at the lesser of the Reference LMP (from Step 1) or the appropriate Mitigated Offer Floor.  The offer portion of each Energy Bid/Offer Curve subject to mitigation under the criteria described in Section 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test, must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node plus a variable not to exceed 0.01 multiplied by the value of the Resource’s MOC curve at the LSL or the appropriate MOC;

(iii)	Use RTM Energy Bids for all available CLRs, whether submitted by QSEs or created by ERCOT.  There is no mitigation of RTM Energy Bids.  An RTM Energy Bid from a CLR represents the bid for energy distributed across all nodes in the Load Zone in which the CLR is located.  For an ESR, an RTM Energy Bid represents a bid for energy at the ESR’s Resource Node;

[NPRR1188: Replace paragraph (iii) above with the following upon system implementation:] (iii) Use Energy Bid Curves for all available CLRs, whether submitted by QSEs or created by ERCOT. There is no mitigation of Energy Bid Curves. An Energy Bid Curve from an Aggregate Load Resource (ALR) represents the bid for energy distributed across all nodes in the Load Zone in which the ALR is located. For an ESR or a CLR that is not an ALR, an Energy Bid Curve represents a bid for energy at the applicable Resource Node;

(iv)	Observe all Competitive and Non-Competitive Constraints; and

(v)	Use Ancillary Service Offers to determine Ancillary Service awards.

(c)	ERCOT shall archive information and provide monthly summaries of security violations and any binding transmission constraints identified in Step 2 of the SCED process.  The summary must describe the limiting element (or identified operator-entered constraint with operator’s comments describing the reason and the Resource-specific impacts for any manual overrides).  ERCOT shall provide the summary to Market Participants on the MIS Secure Area and to the Independent Market Monitor (IMM).

(d)	The System Lambda used to determine LMPs and the Real-Time MCPCs from SCED Step 2 shall be capped at the effective Value of Lost Load (VOLL).  If the following conditions are met for a SCED interval in which the SCED Step 2 System Lambda was capped, a QSE may be eligible for compensation by submitting a Settlement and billing dispute pursuant to paragraph (5) of Section 6.6.9, Emergency Operations Settlement:

(i) 	A Generation Resource or ESR for the QSE received a Base Point greater than the Resource’s LDL for that SCED interval; and

(ii) 	The LMP at the Resource is less than the price on the Resource’s Energy Offer Curve or Energy Bid/Offer Curve, as applicable, with any Resource’s Energy Offer Curve or Energy Bid/Offer Curve capped by the MOC.

[NPRR1290 and NPRR1323: Replace applicable portions of paragraph (d) above with the following upon system implementation:] (d) Any Electrical Bus LMP above the effective VOLL shall be set equal to the greater of the effective VOLL or the initial LMP minus the positive difference between System Lambda and the effective VOLL. All other Electrical Bus LMPs below the effective VOLL remain unchanged. These adjustments shall be applied to Electrical Bus LMPs prior to calculating Real-Time Settlement Point LMPs, Real-Time Settlement Point Prices, and Real-Time prices for energy metered. The System Lambda and the Real-Time MCPCs from SCED Step 2 shall also be capped at the effective VOLL. ERCOT shall post both the capped and uncapped values for Electrical Bus LMPs, System Lambda, and Real-Time MCPCs to the ERCOT website.

(15)	For each SCED process, in addition to the binding Base Points, Ancillary Service awards, Real-Time MCPCs, and LMPs, ERCOT shall calculate indicative projection of the Base Points, Ancillary Service awards, MCPCs, Resource Node LMPs, Hub LMPs, and Load Zone LMPs at a frequency of every five minutes for at least 15 minutes into the future based on the same inputs to the SCED process as described in this Section, except that the Resource’s HDL and LDL and the total generation requirement will be as estimated at future intervals.  The Resource’s HDL and LDL will be calculated for each interval of the projection based on the ramp rate capability over the study period.  ERCOT shall estimate the projected total generation requirement by calculating a Load forecast for the study period.  In lieu of the steps described in Section 6.5.7.3.1, SCED Pricing Run, the indicative projection of Real-Time MCPCs, Resource Node LMPs, and Load Zone LMPs shall be estimated based on GTBD, reliability deployments MWs, offer and bids.  ERCOT shall post the indicative projection of the Base Points and Ancillary Service awards for each Resource for each interval study period on the MIS Certified Area and the indicative projection of the LMPs for Resource Nodes, Real-Time MCPCs, Hub LMPs and Load Zone LMPs on the ERCOT website pursuant to Section 6.3.2, Activities for Real-Time Operations.

(16)	ERCOT may override one or more of a CLR’s parameters in SCED if ERCOT determines that the CLR’s participation is having an adverse impact on the reliability of the ERCOT System.

(17)	The QSE representing an ESR may withdraw energy from the ERCOT System only when dispatched by SCED to do so.  An ESR may telemeter a status of OUT only if the ESR is in Outage status.

6.5.7.3.1	SCED Pricing Run

(1)	The following categories of reliability deployments are considered in the determination of the binding Real-Time LMPs for Energy, and the binding Real-Time MCPCs for Ancillary Services in the SCED Pricing Run:

(a)	RUC-committed Resources, except for those whose QSEs have opted out of RUC Settlement in accordance with paragraph (21) of Section 5.5.2, Reliability Unit Commitment (RUC) Process;

(b)	RMR Resources that are On-Line;

(c)	ERCOT-directed deployment of Ancillary Services from Load Resources other than CLRs;

(d)	ERCOT-directed deployed ERS;

(e)	Real-Time DC Tie imports during an EEA where the total adjustment shall not exceed 1,250 MW in a single interval;

(f)	Real-Time DC Tie exports to address emergency conditions in the receiving electric grid;

[NPRR904: Replace items (e) and (f) above with the following upon system implementation and renumber accordingly:] (e) ERCOT-directed DC Tie imports during an EEA or transmission emergency where the total adjustment shall not exceed 1,250 MW in a single interval; (f) ERCOT-directed curtailment of DC Tie imports below the higher of DC Tie advisory import limit as of 0600 in the Day-Ahead or subsequent advisory import limit to address local transmission system limitations where the total adjustment shall not exceed 1,250 MW in a single interval; (g) ERCOT-directed curtailment of DC Tie imports below the higher of DC Tie advisory import limit as of 0600 in the Day-Ahead or subsequent advisory import limit due to an emergency action by a neighboring system operator during an emergency that is accommodated by ERCOT where the total adjustment shall not exceed 1,250 MW in a single interval; (h) ERCOT-directed DC Tie exports to address emergency conditions in the receiving electric grid where the total adjustment shall not exceed 1,250 MW in a single interval; (i) ERCOT-directed curtailment of DC Tie exports below the DC Tie advisory export limit as of 0600 in the Day-Ahead or subsequent advisory export limit during EEA, a transmission emergency, or to address local transmission system limitations where the total adjustment shall not exceed 1,250 MW in a single interval;

(g)	Energy delivered to ERCOT through registered Block Load Transfers (BLTs) during an EEA;

(h)	Energy delivered from ERCOT to another power pool through registered BLTs during emergency conditions in the receiving electric grid;

[NPRR1006: Insert paragraph (i) below upon system implementation and renumber accordingly:] (i) ERCOT-directed deployment of TDSP standard offer Load management programs.

[NPRR1105: Insert paragraph (j) below upon system implementation and renumber accordingly:] (j) ERCOT-directed deployment of distribution voltage reduction measures;

[NPRR1091: Insert paragraph (k) below upon system implementation and renumber accordingly:] (k) ERCOT-directed deployment of Off-Line Non-Spin;

(i)	ERCOT-directed firm Load shed during EEA Level 3, as described in paragraph (3) of Section 6.5.9.4.2, EEA Levels;

[NPRR1238: Insert paragraph (j) below upon system implementation:] (j) ERCOT-directed deployed Voluntary Early Curtailment Load (VECL) as described in Section 6.5.9.4.1, General Procedures Prior to EEA Operations;

(k)	ERCOT-directed deployment of Load including as part of a Must-Run-Alternative under Section 3.14.4, as capacity procured through Section 6.5.1.1, or as part of a Large Load curtailment under Public Utility Regulatory Act (PURA) § 37.0561;

(l)	ERCOT-directed deployment of SOG including as part of a Must-Run-Alternative under Section 3.14.4 or as capacity procured through Section 6.5.1.1; and

(m)	ERCOT-directed deployment of ESR including as part of a Must-Run-Alternative under Section 3.14.4 or as capacity procured through Section 6.5.1.1.

(2)	The binding Real-Time LMPs for Energy, and binding Real-Time MCPCs for Ancillary Services reflect the energy prices and Real-Time MCPCs taking into consideration the above categories of reliability deployments.  For intervals where there are reliability deployments as described in paragraph (1) above, the binding Real-Time LMPs for Energy and binding Real-Time MCPCs for Ancillary Services are determined as follows:

(a)	For RUC-committed Resources with a telemetered Resource Status of ONRUC and for RMR Resources that are On-Line:

[NPRR1091: Replace paragraph (j) above with the following upon system implementation:] (a) For Off-Line Non-Spin Resources that are brought On-Line by ERCOT deployment instruction, RUC-committed Resources with a telemetered Resource Status of ONRUC and for RMR Resources that are On-Line:

(i)	Set the LSL and LDL to zero;

(ii)	Remove all Ancillary Service Offers; and

(iii)	For the first step of the SCED Pricing Run, administratively set the Energy Offer Curve for the Resource at a value equal to the power balance penalty price for all capacity between 0 MW and the HSL of the Resource.

(b)	Notwithstanding item (a) above, for RUC-committed Combined Cycle Generation Resources with a telemetered Resource Status of ONRUC that were instructed by ERCOT to transition to a different configuration to provide additional capacity:

(i)	Set the LSL and LDL equal to the minimum of their current value and the COP HSL of the QSE-committed configuration for the RUC hour at the snapshot time of the RUC instruction;

(ii)	Set the maximum Ancillary Service capabilities of the Resource equal to the minimum of their current value and COP Ancillary Service capabilities of the QSE-committed configuration for the RUC hour at the snapshot time of the RUC instruction; and

(iii)	For the first step of SCED Pricing Run, administratively set the Energy Offer Curve for the Resource at a value equal to the power balance penalty price for the additional capacity of the Resource, defined as the positive difference between the Resource’s current telemetered HSL and the COP HSL of the QSE-committed configuration for the RUC hour at the snapshot time of the RUC instruction.

(c)	For all other Generation Resources not covered by paragraphs (a) and (b) above, all ESRs, and all CLRs, the LDL and HDL shall be the same values as used in the SCED Dispatch Run.

(d)	Add the ERCOT-deployed MW from Load Resources that are not CLRs and that are providing RRS or ECRS to GTBD linearly ramped over the ten-minute ramp period and add the deployed MW from Load Resources that are not CLRs providing Non-Spin to GTBD linearly ramped over the 30-minute ramp period.  The amount of deployed MW is calculated from the Resource telemetry and from applicable deployment instructions in Extensible Markup Language (XML) messages. ERCOT shall create a pseudo CLR with net real power consumption  equal to the deployed MW added to GTBD for that SCED interval, LPC and LDL equal to zero, HDL equal to the deployed MW added to GTBD for that SCED interval and MPC equal to the deployed MW instruction in the XML, and a linear Energy Bid Curve defined by a price/quantity pair of $700/MWh at zero MW and a price/quantity pair of $300/MWh corresponding to the deployed MW for that SCED interval. The pseudo CLR will be modeled at the closest applicable energized Electrical Bus where the Load Resource MW was deployed. After recall instruction, the restoration period length and amount of MW added to GTBD during the restoration period will be determined by validated telemetry and the type of Ancillary Service deployed from the Resource.  The TAC shall review the validity of the prices for the bid curve at least annually.

[NPRR1238: Insert paragraph (g) below upon system implementation and renumber accordingly:] (g) Add the ERCOT-deployed MW from VECL to GTBD linearly ramped over a 30-minute ramp period. The amount of deployed MW is calculated from the applicable deployment instructions in XML messages. ERCOT shall create a pseudo CLR with net real power consumption equal to the deployed MW added to GTBD for that SCED interval, LPC and LDL equal to zero, HDL equal to the deployed MW added to GTBD for that SCED interval and MPC equal to the deployed MW instruction in the XML, and a linear Energy Bid Curve defined by a price/quantity pair of $700/MWh at zero MW and a price/quantity pair of $300/MWh corresponding to the deployed MW for that SCED interval. Where information on the VECL MW deployed at a particular Electrical Bus is available, the pseudo CLR will be modeled at the closest applicable energized Electrical Bus where the VECL MW was deployed. After recall instruction, GTBD shall be adjusted to reflect restoration on a linear curve over a one-hour restoration period. The TAC shall review the validity of the prices for the bid curve at least annually.

(e)	Add the ERCOT-deployed MW from ERS to GTBD.  The amount of deployed MW is determined from the XML messages and ERS contracted capacities for the ERS Time Periods when ERS is deployed.  After recall, an approximation of the amount of un-restored ERS shall be used.  After ERCOT recalls each group, GTBD shall be adjusted to reflect restoration on a linear curve over the assumed restoration period (“RHours”).

The above parameter is defined as follows:

| Parameter | Unit | Current Value* |
| --- | --- | --- |
| RHours | Hours | 4.5 |
| * Changes to the current value of the parameter(s) referenced in this table above may be recommended by TAC and the ERCOT Board and approved by the Public Utility Commission of Texas (PUCT). ERCOT shall update parameter values on the first day of the month following PUCT approval unless otherwise directed. ERCOT shall provide a Market Notice prior to implementation of a revised parameter value. |  |  |

(f)	Add the MW from Real-Time DC Tie imports during an EEA to GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the ERCOT Operator.

(g)	Subtract the MW from Real-Time DC Tie exports to address emergency conditions in the receiving electric grid from GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the receiving grid operator.

[NPRR904: Replace paragraphs (h) and (i) above with the following upon system implementation and renumber accordingly:] (h) Add the MW from ERCOT-directed DC Tie imports during an EEA or transmission emergency, to address local transmission system limitations, or due to an emergency action by a neighboring system operator during an emergency that is accommodated by ERCOT to GTBD, and create a pseudo CLR with net real power consumption equal to the MW added to GTBD for that SCED interval, LPC and LDL equal to zero, HDL and MPC equal to the MW added to GTBD for that SCED interval, and a linear Energy Bid Curve defined by a price/quantity pair of VOLL at zero MW and a price/quantity pair of VOLL corresponding to the MW added to GTBD for that SCED interval. The pseudo CLR will be modeled at the corresponding DC Tie Resource Node. The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the ERCOT Operator (i) Add the MW from ERCOT-directed DC Tie export curtailments during an EEA or transmission emergency, to address local transmission system limitations, or due to an emergency action by a neighboring system operator during an emergency that is accommodated by ERCOT to GTBD, and create a pseudo CLR with net real power consumption equal to the MW added to GTBD for that SCED interval, LPC and LDL equal to zero, HDL and MPC equal to the MW added to GTBD for that SCED interval, and a linear Energy Bid Curve defined by a price/quantity pair of VOLL at zero MW and a price/quantity pair of VOLL corresponding to the MW added to GTBD for that SCED interval. The pseudo CLR will be modeled at the corresponding DC Tie Resource Node(s) as negative LSL(s) of pseudo Energy Storage Resource(s) with HSL. The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the ERCOT Operator. The MW added to GTBD associated with any individual DC Tie shall not exceed the higher of DC Tie advisory limit for exports on that tie as of 0600 in the Day-Ahead or subsequent advisory export limit minus the aggregate export on the DC Tie that remained scheduled following the Dispatch Instruction from the ERCOT Operator. (j) Model the MW from ERCOT-directed DC Tie exports to address emergency conditions in the receiving electric grid by creating a pseudo Generation Resource with LSL and LDL equal to zero, HSL and HDL equal to the deployed MW and a linear Energy Offer Curve defined by a price/quantity pair of -250$/MWh at zero MW and a price/quantity pair of -250$/MWh corresponding to the deployed MW. This pseudo Generation Resource will be located at the corresponding DC Tie Resource Node. The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the receiving grid operator. (k) Model the MW from ERCOT-directed DC Tie import curtailments to address local transmission system limitations or emergency conditions in the receiving electric grid by creating a pseudo Generation Resource with LSL and LDL equal to zero, HSL and HDL equal to the deployed MW and a linear Energy Offer Curve defined by a price/quantity pair of -250$/MWh at zero MW and a price/quantity pair of -250$/MWh corresponding to the deployed MW. This pseudo Generation Resource will be located at the corresponding DC Tie Resource Node(. The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the receiving grid operator. The MW from ERCOT-directed DC Tie import curtailments to address local transmission system limitations or emergency conditions in the receiving electric grid associated with any individual DC Tie shall not exceed the higher of DC Tie advisory limit for imports on that tie as of 0600 in the Day-Ahead or subsequent advisory import limit minus the aggregate import on the DC Tie that remained scheduled following the Dispatch Instruction from the ERCOT Operator.

(h)	Add the MW from energy delivered to ERCOT through registered BLTs during an EEA to GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the ERCOT Operator.

(i)	Subtract the MW from energy delivered from ERCOT to another power pool through registered BLTs during emergency conditions in the receiving electric grid from GTBD.  The amount of MW is determined from the Dispatch Instruction and should continue over the duration of time specified by the receiving grid operator.

[NPRR1006: Insert paragraph (l) below upon system implementation and renumber accordingly:] (l) Add the ERCOT-deployed MWs from TDSP standard offer Load management programs to GTBD, if ERCOT instructs TDSPs to deploy their standard offer Load management programs. The amount of deployed MW is the value ERCOT provided for all TDSP standard offer Load management programs in the most current May Report on Capacity, Demand and Reserves in the ERCOT Region, unless modified as specified in this paragraph. If ERCOT is informed that all or a portion of a TDSP’s standard offer Load management program has been fully exhausted, or has been expanded as the result of a Public Utility Commission of Texas (PUCT) proceeding, ERCOT will remove the associated MW value of any exhausted capacity from the amount of deployed MW or, in the case of an expansion, ERCOT will request an updated MW value from the relevant TDSPs to use in place of the May Report on Capacity, Demand and Reserves in the ERCOT Region (CDR) value for that year. The initial value ERCOT will use for deployed MW under this paragraph for each calendar year, as well as any subsequent changes to this value, will be communicated to Market Participants in a Market Notice. After recall, an approximation of the amount of un-restored TDSP standard offer Load management programs shall be used. GTBD shall be adjusted to reflect restoration on a linear curve over the assumed restoration period (“RHours”) defined by item (g) above.

(j)	Add the ERCOT-deployed MW from Load to GTBD linearly ramped over the 30-minute ramp period.  The amount of deployed MW is calculated from the applicable deployment instructions in Extensible Markup Language (XML) messages. ERCOT shall create a pseudo CLR with net real power consumption  equal to the deployed MW added to GTBD for that SCED interval, LPC and LDL equal to zero, HDL equal to the deployed MW added to GTBD for that SCED interval and MPC equal to the deployed MW instruction in the XML, and a linear Energy Bid Curve defined by a price/quantity pair of $700/MWh at zero MW and a price/quantity pair of $300/MWh corresponding to the deployed MW for that SCED interval. Where information on the Load MW deployed at a particular Electrical Bus is available, the pseudo CLR will be modeled at the closest applicable energized Electrical Bus where the Load MW was deployed, otherwise this pseudo-CLR will be modeled at the Shift Factor reference bus.  After recall instruction, GTBD shall be adjusted to reflect restoration on a linear curve over a one-hour restoration period.  The TAC shall review the validity of the prices for the bid curve at least annually.

(k)	Add the ERCOT-deployed MW from SOG to GTBD linearly ramped over the ten-minute ramp period.  The amount of deployed MW is calculated from the applicable deployment instructions in Extensible Markup Language (XML) messages.  ERCOT shall create a pseudo Generation Resource with LSL and LDL equal to zero, HSL and HDL equal to the deployed MW and a linear Energy Offer Curve defined by a price/quantity pair of RTSWCAP at zero MW and a price/quantity pair of RTSWCAP corresponding to the deployed MW. Where information on the SOG MW deployed at a particular Electrical Bus is available, the pseudo Generation Resource will be modeled at the closest applicable energized Electrical Bus where the SOG MW was deployed, otherwise this pseudo Generation Resource will be modeled at the Shift Factors reference bus. After recall instruction, GTBD shall be adjusted to reflect restoration on a linear curve over a 30-minute restoration period.  The TAC shall review the validity of the prices for the bid curve at least annually.

(l)	For ERCOT-directed deployment of ESR as part of a Must-Run-Alternative (MRA) under Section 3.14.4 or as capacity procured through Section 6.5.1.1, where the ERCOT-directed deployment instruction is given in Extensible Markup Language (XML) messages:

(i)	Set the LSL and LDL to zero;

(ii)	Remove all Ancillary Service Offers; and

(iii)	For the first step of the SCED Pricing Run, administratively set the offer portion of the Energy Bid/Offer Curve for the Resource at a value equal to the power balance penalty price for all capacity between 0 MW and the HSL of the Resource.

(m)	Perform a SCED as described in Section 6.5.7.3, Security Constrained Economic Dispatch, with changes to the inputs in items (a) through (k) above, considering only Competitive Constraints and the non-mitigated Energy Offer Curves.

(n)	Perform mitigation on the submitted Energy Offer Curves using the LMPs from the previous step as the reference LMP.

(o)	Perform a SCED as described in Section 6.5.7.3, Security Constrained Economic Dispatch, with the changes to the inputs in items (a) through (k) above, considering both Competitive and Non-Competitive Constraints and the mitigated Energy Offer Curves.

(p)	The binding Real-Time LMPs for Energy at each Electrical Bus and Settlement Point is from the second step in the two-step SCED process described in item (o) above.   When ERCOT is directing firm Load shed during EEA Level 3 to either maintain sufficient PRC or stabilize grid frequency, as described in paragraph (3) of Section 6.5.9.4.2, the System Lambda from item (o) shall be replaced by the greater of step 2 System Lambda or VOLL used to determine the Ancillary Service Demand Curves (ASDCs) for the Real-Time Market (RTM) before calculating the LMP for each Electrical Bus and Settlement Point.

(q)	For each individual Ancillary Service, the binding Real-Time MCPC for Ancillary Service is from the two step SCED process in item (o) above, except when ERCOT is directing firm Load shed during EEA Level 3.  When ERCOT is directing firm Load shed during EEA Level 3 to either maintain sufficient PRC or stabilize grid frequency, as described in paragraph (3) of Section 6.5.9.4.2, the Real-Time MCPC for Ancillary Service is the VOLL used to determine the ASDCs for that Ancillary Service.

#### 6.5.9.2	Failure of the SCED Process

(1)	When the SCED process is not able to reach a solution, ERCOT shall issue a Watch.

(2)	For intervals that the SCED process fails to reach a solution, then the LMPs, and Real-Time MCPCs, for the interval for which no solution was reached are equal to the LMPs, and Real-Time MCPCs in the most recently solved interval.  For Settlement Intervals that the Real-Time Settlement Point Prices are identified as erroneous, and ERCOT sets the SCED intervals as failed in accordance with Section 6.3, Adjustment Period and Real-Time Operations Timeline, then the LMPs, and Real-Time MCPCs for the failed SCED intervals are equal to the LMPs, and Real-Time MCPCs in the most recently solved SCED interval that is not set as failed.  ERCOT shall notify the market of the failure by posting on the ERCOT website.  For intervals covering the first 15 minutes of SCED process execution following a failure, ERCOT shall set the LMPs, and Real-Time MCPCs equal to the LMPs, and Real-Time MCPCs in the most recently solved SCED interval prior to the SCED process failure.  ERCOT shall notify the market of this price correction by posting on the ERCOT website.

(3)	In the event that a Market Suspension is declared in accordance with Section 25, Market Suspension and Restart, upon the effective date and time of the Market Suspension, the Market Suspension Settlement methodology set forth in Section 25.5, Market Suspension and Market Restart Settlement, will supersede the provisions set forth in paragraph (2) above.

(4)	Once ERCOT issues a Watch for a SCED process failure, ERCOT may use any of the following measures:

(a)	ERCOT may direct the SCED process to relax the active transmission constraints;

(b)	ERCOT may issue Emergency Base Points for Resources;

(c)	ERCOT may manually issue Emergency Base Points for a Resource and must communicate the Resource name, MW output requested, and start time and duration of the Dispatch Instruction to the QSE representing the Resource;

(d)	ERCOT may issue an instruction to hold the previous interval; and

(e)	A QF, a hydro Generation Resource, or a nuclear-powered Resource may be instructed by ERCOT to operate below its LSL only after all other Resource options have been exhausted.

(5)	The Watch continues until the SCED process can reach a solution without using the measures in paragraph (4) above.

#### 6.6.1.1	Real-Time Settlement Point Price for a Resource Node

(1)	The Real-Time Settlement Point Price for a Resource Node Settlement Point is the time-weighted average of the Real-Time LMPs.  The Real-Time Settlement Point Price for a 15-minute Settlement Interval is calculated as follows:

RTSPP 	=	Max (-$251, ((RNWF y * RTLMP y)))

Where the Resource Node weighting factor is:

RNWF y	=	TLMP y / TLMP y

The above variables are defined as follows:

| Variable | Unit | Description |  |
| --- | --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Settlement Point for the 15-minute Settlement Interval. |  |
| RTLMP y | $/MWh | Real-Time Locational Marginal Price per interval¾The Real-Time LMP at the Settlement Point for the SCED interval y. |  |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Resource Node Settlement Point Price calculation for the portion of the SCED interval y within the Settlement Interval. |  |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the Settlement Interval. |  |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |  |

(2)	The Real-Time Settlement Point Price at the logical Resource Node for a Combined Cycle Train shall be determined in accordance with paragraph (1) above using a Real-Time LMP calculated for the logical Resource Node in each SCED Interval as follows:

(a)	The Real-Time LMP for the logical Resource Node of a Combined Cycle Train for each SCED interval is calculated as follows:

For a Combined Cycle Train that is On-Line in the SCED interval:

RTLMP y = ∑CCGR_PhyR RTLMP CCGR_PhyR, y * RTONCCGRWF CCGR_PhyR

For a Combined Cycle Train that is Off-Line in the SCED interval:

RTLMP y = ∑CCT_PhyR RTLMP CCT_PhyR, y * RTOFFCCGRWF CCT_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTLMP y | $/MWh | Real-Time Locational Marginal Price at a logical Resource Node for a Combined Cycle Train¾The Real-Time LMP at the Combined Cycle Generation Resource logical Resource Node for a SCED interval y. |
| RTLMP CCGR_PhyR, y | $/MWh | Real-Time Locational Marginal Price at a generation unit Resource Node designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource¾The Real-Time LMP at the Resource Node of a generation unit designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource for the SCED interval y. |
| RTLMP CCT_PhyR, y | $/MWh | Real-Time Locational Marginal Price at a generation unit Resource Node registered to the Combined Cycle Train¾The Real-Time LMP at the Resource Node of a generation unit designated in a Combined Cycle Train registration for the SCED interval y. |
| RTONCCGRWF CCGR_PhyR, y | none | Real-Time On-Line Combined Cycle Generation Resource Weighting Factor¾The Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource for the SCED interval y. |
| RTOFFCCGRWF CCT_PhyR, y | none | Real-Time Off-Line Combined Cycle Generation Resource Weighting Factor¾The Real-Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration when the whole Combined Cycle Train is Off-Line for the SCED interval y. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for the On-Line Combined Cycle Generation Resource. |
| CCT_PhyR | none | A generation unit designated in a Combine Cycle Train registration |
| c | none | A binding transmission constraint for the SCED interval y. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

(b)	For an On-Line Combined Cycle Train, the weight factor for each generation unit registered in an On-Line Combined Cycle Generation Resource shall be the Real-Time power output telemetry in each SCED interval for each generation unit registered in the Combined Cycle Generation Resource divided by the total Real-Time power output telemetry for all of the generation units registered in the Combined Cycle Generation Resource.  For an Off-Line Combined Cycle Train, the weight factor for each generation unit designated in a Combined Cycle Train registration shall be its High Reasonability Limit (HRL) divided by the total sum of the HRL for all generation units registered in the Combined Cycle Train.

Where:

RTONCCGRWF CCGR_PhyR, y = TG CCGR_PhyR / ∑CCGR_PhyR TG CCGR_PhyR

RTOFFCCGRWF CCT_PhyR, y = HRL CCT_PhyR / ∑CCT_PhyR HRL CCT_PhyR

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTONCCGRWF CCGR_PhyR, y | none | Real-Time On-Line Combined Cycle Generation Resource Weighting Factor¾The Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource for the SCED interval y. |
| TG CCGR_PhyR, y | MW | Telemetered Generation for a Combined Cycle Generation Resource generation unit¾The telemetered Real-Time power generation for a generation unit designated in a Combined Cycle Train registration for the On-Line Combined Cycle Generation Resource at the time of State Estimator execution for the SCED interval y. |
| RTOFFCCGRWF CCT_PhyR, y | none | Real-Time Off-Line Combined Cycle Generation Resource Weighting Factor¾The Real Time Combined Cycle Generation Resource weighting factor for a generation unit designated in a Combined Cycle Train registration when the whole Combined Cycle Train is Off-Line for the SCED interval y. |
| HRL CCT_PhyR | MW | High Reasonability Limit—The HRL as specified in the ERCOT-approved Resource Registration data for a generation unit designated in a Combined Cycle Train registration. |
| CCGR_PhyR | none | A generation unit designated in a Combine Cycle Train registration for the On-Line Combined Cycle Generation Resource. |
| CCT_PhyR | none | A generation unit designated in a Combine Cycle Train registration. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

#### 6.6.1.2	Real-Time Settlement Point Price for a Load Zone

(1)	The Real-Time Settlement Point Price for a Load Zone Settlement Point is based on the state-estimated Load in MW and the time-weighted average Real-Time LMPs at Electrical Buses that are included in the Load Zone.  The Real-Time Settlement Point Price for a Load Zone Settlement Point for a 15-minute Settlement Interval is calculated as follows:

RTSPP	=	Max (-$251, (TLMP y * LZLMP y) / TLMP y)

For all Load Zones except Direct Current Tie (DC Tie) Load Zones:

LZLMP y	=	 (RTLMP b, y * SEL b, y) / SEL b, y

For a DC Tie Load Zone:

LZLMP y	=	RTLMP b, y

(2)	For all Settlement calculations in which a 15-minute Real-Time Settlement Point Price for a Load Zone is required in order to perform Settlement for a 15-minute quantity that is represented as one value (the integrated value for the 15-minute interval) but varies with each SCED interval within the 15-minute Settlement Interval, an energy-weighted Real-Time Settlement Point Price shall be used and is calculated as follows:

RTSPPEW              =	Max [-$251, (RTLMP b, y * LZWF b, y)]

For all Load Zones except DC Tie Load Zones:

LZWF b, y 	=	(SEL b, y * TLMP y) / [(SEL b, y * TLMP y)]

For a DC Tie Load Zone:

LZWF b, y 	=	(SEL b, y * TLMP y) / [(SEL b, y * TLMP y)]

SEL b, y	=	1

Where:

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTSPP | $/MWh | Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Settlement Point, for the 15-minute Settlement Interval. |
| RTSPPEW | $/MWh | Real-Time Settlement Point Price Energy-Weighted¾The Real-Time Settlement Point Price at the Settlement Point, for the 15-minute Settlement Interval that is weighted by the state-estimated Load of the Load Zone of each SCED interval within the 15-minute Settlement Interval. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per interval¾The Real-Time LMP at Electrical Bus b in the Load Zone, for the SCED interval y. |
| LZWF b, y | none | Load Zone Weighting Factor per bus per interval¾The weight used in the Load Zone Settlement Point Price calculation for Electrical Bus b, for the portion of the SCED interval y within the 15-minute Settlement Interval. |
| LZLMP y | $/MWh | Load Zone Locational Marginal Price¾The Load Zone LMP for the Load Zone for the SCED interval y. |
| SEL b, y | MW | State Estimator Load at bus per interval¾The Load value from State Estimator, including a calculated net Load value at each Private Use Network and adjustments to account for Distribution Generation Resource (DGR) and Distribution Energy Storage Resource (DESR) injections and withdrawals that are settled at a Resource Node, excluding Wholesale Storage Load (WSL) and Non-WSL Energy Storage Resource (ESR) Charging Load, for Electrical Bus b in the Load Zone, for the SCED interval y. [NPRR1188: Replace the description above with the following upon system implementation:] State Estimator Load at bus per interval¾The Load value from State Estimator, including a calculated net Load value at each Private Use Network and adjustments to account for Distribution Generation Resource (DGR) and Distribution Energy Storage Resource (DESR) injections and withdrawals that are settled at a Resource Node, excluding Controllable Load Resource (CLR) Load that is not an Aggregate Load Resource (ALR), Wholesale Storage Load (WSL) and Non-WSL Energy Storage Resource (ESR) Charging Load, for Electrical Bus b in the Load Zone, for the SCED interval y.. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An Electrical Bus in the Load Zone. The summation is over all of the Electrical Buses in the Load Zone. |

#### 6.6.1.6		Real-Time Market Clearing Prices for Ancillary Services

(1)	The Real-Time Market Clearing Price for Capacity (MCPC) for Reg-Up is the time-weighted average of the Real-Time MCPCs for Reg-Up of each SCED interval in the 15-minute Settlement Interval.  The Real-Time MCPC for Reg-Up for a 15-minute Settlement Interval is calculated as follows:

RTMCPCRU  =    (RNWF y * RTMCPCRUS y))

Where:

RNWF y   =  TLMP y / TLMP y

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTMCPCRU | $/MW | Real-Time Market Clearing Price for Capacity for Reg-Up - The Real-Time MCPC for Reg-Up for the 15-minute Settlement Interval. |
| RTMCPCRUS y | $/MW | Real-Time Market Clearing Price for Capacity for Reg-Up per SCED interval - The Real-Time MCPC for Reg-Up for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Ancillary Service Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

(2) 	The Real-Time MCPC for Reg-Down is the time-weighted average of the Real-Time MCPCs for Reg-Down of each SCED interval in the 15-minute Settlement Interval.  The Real-Time MCPC for Reg-Down for a 15-minute Settlement Interval is calculated as follows:

RTMCPCRD  =    (RNWF y * RTMCPCRDS y)

Where:

RNWF y   =  TLMP y / TLMP y

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTMCPCRD | $/MW | Real-Time Market Clearing Price for Capacity for Reg-Down - The Real-Time MCPC for Reg-Down for the 15-minute Settlement Interval. |
| RTMCPCRDS y | $/MW | Real-Time Market Clearing Price for Capacity for Reg-Down per SCED interval - The Real-Time MCPC for Reg-Down for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Ancillary Service Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

(3) 	The Real-Time MCPC for RRS is the time-weighted average of the Real-Time MCPCs for RRS of each SCED interval in the 15-minute Settlement Interval.  The Real-Time MCPC for RRS for a 15-minute Settlement Interval is calculated as follows:

RTMCPCRR  =    (RNWF y * RTMCPCRRS y)

Where:

RNWF y   =  TLMP y / TLMP y

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTMCPCRR | $/MW | Real-Time Market Clearing Price for Capacity for Responsive Reserve - The Real-Time MCPC for RRS for the 15-minute Settlement Interval. |
| RTMCPCRRS y | $/MW | Real-Time Market Clearing Price for Capacity for Responsive Reserve per SCED interval - The Real-Time MCPC for RRS for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Ancillary Service Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

(4) 	The Real-Time MCPC for ECRS is the time-weighted average of the Real-Time MCPC for ECRS of each SCED interval in the 15-minute Settlement Interval.  The Real-Time MCPC for ECRS for a 15-minute Settlement Interval is calculated as follows:

RTMCPCECR  =    (RNWF y * RTMCPCECRS y)

Where:

RNWF y   =  TLMP y / TLMP y

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTMCPCECR | $/MW | Real-Time Market Clearing Price for Capacity for ERCOT Contingency Reserve - The Real-Time MCPC for ECRS for the 15-minute Settlement Interval. |
| RTMCPCECRS y | $/MW | Real-Time Market Clearing Price for Capacity for ERCOT Contingency Reserve per SCED interval - The Real-Time MCPC for ECRS for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Ancillary Service Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

(5) 	The Real-Time MCPC for Non-Spin is the time-weighted average of the Real-Time MCPC for Non-Spin of each SCED interval in the 15-minute Settlement Interval.  The Real-Time MCPC for Non-Spin for a 15-minute Settlement Interval is calculated as follows:

RTMCPCNS  =    (RNWF y * RTMCPCNSS y)

Where:

RNWF y   =  TLMP y / TLMP y

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTMCPCNS | $/MW | Real-Time Market Clearing Price for Capacity for Non-Spin - The Real-Time MCPC for Non-Spin for the 15-minute Settlement Interval. |
| RTMCPCNSS y | $/MW | Real-Time Market Clearing Price for Capacity for Non-Spin per SCED interval - The Real-Time MCPC for Non-Spin for the SCED interval y. |
| RNWF y | none | Resource Node Weighting Factor per interval¾The weight used in the Ancillary Service Price calculation for the portion of the SCED interval y within the Settlement Interval. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the portion of the SCED interval y within the Settlement Interval. |
| y | none | A SCED interval in the 15-minute Settlement Interval. |

#### 6.6.3.1	Real-Time Energy Imbalance Payment or Charge at a Resource Node

(1)	The payment or charge to each QSE for Energy Imbalance Service is calculated based on the Real-Time Settlement Point Price for the following amounts at a particular Resource Node Settlement Point:

(a)	The energy produced by all its Generation Resources, consumed as WSL, or consumed as Non-WSL ESR Charging Load at the Settlement Point; plus

[NPRR1188: Replace item (a) above with the following upon system implementation:] (a) The energy produced or consumed at the Settlement Point by all its Generation Resources, ESR Charging Load with WSL treatment, ESR Charging Load with Non-WSL treatment, or CLRs that are not ALRs; plus

(b)	The amount of its Self-Schedules with sink specified at the Settlement Point; plus

(c)	The amount of its Day-Ahead Market (DAM) Energy Bids cleared in the DAM at the Settlement Point; plus

(d)	The amount of its Energy Trades at the Settlement Point where the QSE is the buyer; minus

(e)	The amount of its Self-Schedules with source specified at the Settlement Point; minus

(f)	The amount of its energy offers cleared in the DAM at the Settlement Point; minus

(g)	The amount of its Energy Trades at the Settlement Point where the QSE is the seller.

(2)	The payment or charge to each QSE for Energy Imbalance Service at a Resource Node Settlement Point for a given 15-minute Settlement Interval is calculated as follows:

RTEIAMT q, p		= (-1) * {((RESREV q, r, gsc, p)) + (WSLAMTTOT q, r, p) + (ESRNWSLAMTTOT q, r, p) + RTSPP p * [(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)]}

[NPRR1188: Replace the formula “RTEIAMT q, p” above with the following upon system implementation:] RTEIAMT q, p = (-1) * {((RESREV q, r, gsc, p)) + (WSLAMTTOT q, r, p) + (CLRAMTTOT q, r, p) + (ESRNWSLAMTTOT q, r, p) + RTSPP p * [(SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)]}

Where:

RESREV q, r, gsc, p	= GSPLITPER q, r, gsc, p * NMSAMTTOT gsc

RESMEB q, r, gsc, p	= GSPLITPER q, r, gsc, p * NMRTETOT gsc

WSLTOT q, p		=  ( MEBL q,r,b)

[NPRR1188: Insert the formula “CLRTOT q, p” below upon system implementation:] CLRTOT q, p = (MEBCL q, r, b)

ESRNWSLTOT q, p	=  ( MEBR q, r, b)

RNIMBAL q, p		= (RESMEB q, r, gsc, p) + WSLTOT q, p + ESRNWSLTOT q, p + (SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)

[NPRR1188: Replace the formula “RNIMBAL q, p” above with the following upon system implementation:] RNIMBAL q, p = (RESMEB q, r, gsc, p) + WSLTOT q, p + CLRTOT q, p + ESRNWSLTOT q, p + (SSSK q, p * ¼) + (DAEP q, p * ¼) + (RTQQEP q, p * ¼) – (SSSR q, p * ¼) – (DAES q, p * ¼) – (RTQQES q, p * ¼)

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| RNIMBAL q, p | MWh | Resource Node Energy Imbalance per QSE per Settlement Point—The Resource Node volumetric imbalance for QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| RTSPP p | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSK q, p | MW | Self-Schedule with Sink at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with sink at Settlement Point p, for the 15-minute Settlement Interval. |
| DAEP q, p | MW | Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s DAM Energy Bids at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. [NPRR1188: Replace the description above with the following upon system implementation:] Day-Ahead Energy Purchase per QSE per Settlement Point—The QSE q’s DAM Energy Bids, Energy Bid Curves, and bid portion of Energy Bid/Offer Curves at Settlement Point p, cleared in the DAM, for the hour that includes the 15-minute Settlement Interval.. |
| RTQQEP q, p | MW | Real-Time QSE-to-QSE Energy Purchase per QSE per Settlement Point¾The amount of MW bought by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| SSSR q, p | MW | Self-Schedule with Source at Settlement Point per QSE per Settlement Point—The QSE q’s Self-Schedule with source at Settlement Point p, for the 15-minute Settlement Interval. |
| DAES q, p | MW | Day-Ahead Energy Sale per QSE per Settlement Point—The QSE q’s energy offers at Settlement Point p cleared in the DAM, for the hour that includes the 15-minute Settlement Interval. |
| RTQQES q, p | MW | Real-Time QSE-to-QSE Energy Sale per QSE per Settlement Point¾The amount of MW sold by QSE q through Energy Trades at Settlement Point p, for the 15-minute Settlement Interval. |
| RESREV q, r, gsc, p | $ | Resource Share Revenue Settlement Payment—The Resource share of the total payment to the entire Facility with a net metering arrangement attributed to Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p. |
| RESMEB q, r, gsc, p | MWh | Resource Share Net Meter Real-Time Energy Total—The Resource share of the net sum for all Settlement Meters attributed to Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p. |
| WSLTOT q, p | MWh | WSL Total—The total WSL energy metered by the Settlement Meters which measure WSL for the QSE q at Settlement Point p. |
| [NPRR1188: Insert the variable “CLRTOT q, p” below upon system implementation:] CLRTOT q, p; MWh; CLR Load Total—The total energy metered by the Settlement Meters which measures CLR Load for the QSE q at Settlement Point p... |
| ESRNWSLTOT q, p | MWh | ESR Non-WSL Total—The total energy metered by the Settlement Meters which measure Non-WSL ESR Charging Load for the QSE q at Settlement Point p. |
| MEBL q,r,b | MWh | Metered Energy for Wholesale Storage Load at bus¾The WSL energy metered by the Settlement Meter which measures WSL for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b. |
| [NPRR1188: Insert the variable “MEBCL q, r, b” below upon system implementation:] MEBCL q, r, b; MWh; Calculated Metered Energy for CLR Load at Bus—The calculated CLR Load, adjusted for Unaccounted For Energy (UFE), for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b... |
| MEBR q, r, b | MWh | Adjusted Metered Energy for Energy Storage Resource Load at Bus - The energy metered by the Settlement Meter which measures Non-WSL ESR Charging Load for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b. [NPRR1188: Replace the description above with the following upon system implementation:] Calculated Metered Energy for Energy Storage Resource Load at Bus - The calculated Non-WSL ESR Charging Load, adjusted for UFE, for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. |
| NMSAMTTOT gsc | $ | Net Metering Settlement—The total payment or charge to a generation site with a net metering arrangement. |
| [NPRR1188: Insert the variable “CLRAMTTOT q, r, p” below upon system implementation:] CLRAMTTOT q, r, p; $; CLR Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for CLR Load for each 15-minute Settlement Interval... |
| WSLAMTTOT q, r, p | $ | Wholesale Storage Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for WSL for each 15-minute Settlement Interval. |
| ESRNWSLAMTTOT q, r, p | $ | Energy Storage Resource Non-WSL Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for Non-WSL ESR Charging Load for each 15-minute Settlement Interval. |
| NMRTETOT gsc | MWh | Net Meter Real-Time Energy Total—The net sum for all Settlement Meters included in generation site code gsc. A positive value indicates an injection of power to the ERCOT System. |
| GSPLITPER q, r, gsc, p | none | Generation Resource SCADA Splitting Percentage—The generation allocation percentage for Resource r that is part of a net metering arrangement. GSPLITPER is calculated by taking the positive Supervisory Control and Data Acquisition (SCADA) values (GSSPLITSCA) for a particular Generation Resource or ESR r that is part of a net metering configuration and dividing by the sum of all positive SCADA values for all Resources that are included in the net metering configuration for each interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |
| r | none | A Generation Resource or ESR that is located at the Facility with net metering. [NPRR1188: Replace the description above with the following upon system implementation:] A Generation Resource, a CLR that is not an ALR, or ESR that is located at the Facility with net metering.. |
| gsc | none | A generation site code. |
| b | none | An Electrical Bus. |

(3)	For a facility with Settlement Meters that measure ESR Load, the total payment or charge for ESR Load is calculated for a QSE, ESR, and Settlement Point for each 15-minute Settlement Interval.

The WSL is settled as follows:

WSLAMTTOT q, r, p	=  (RTRMPRESR b * MEBL q, r, b)

The Non-WSL ESR Charging Load is settled as follows:

ESRNWSLAMTTOT q, r, p	=  (RTRMPRESR b * MEBR q, r, b)

Where the price for Settlement Meter is determined as follows:

RTRMPRESR b 	= Max [-$251, ((RNWFL b, y * RTLMP b, y)]]

Where the weighting factor for the Electrical Bus associated with the meter is:

RNWFL b, y 		= [Max (0.001, ABS( Min(0, BP r, y))) * TLMP y] /

[Max (0.001, ABS(  Min(0, BP r, y))) * TLMP y]

The summation is over all ESR Load r associated to the individual meter.  The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc.

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per interval¾The Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the SCED interval y. |
| MEBL q,r,b | MWh | Metered Energy for Wholesale Storage Load at bus¾The WSL energy metered by the Settlement Meter which measures WSL for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b. |
| MEBR q, r, b | MWh | Metered Energy for Energy Storage Resource Load at Bus - The energy metered by the Settlement Meter which measures Non-WSL ESR Charging Load for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b. |
| WSLAMTTOT q, r, p | $ | Wholesale Storage Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for WSL for each 15-minute Settlement Interval. |
| ESRNWSLAMTTOT q, r, p | $ | Energy Storage Resource Non-WSL Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for Non-WSL ESR Charging Load for each 15-minute Settlement Interval. |
| RNWFL b, y | none | Net meter Weighting Factor per interval for the Energy Metered as Energy Storage Resource LoadThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y, for the ESR Load associated with an ESR. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters. |
| RTRMPRESR b | $/MWh | Real-Time Price for the Energy Metered as Energy Storage Resource Load at bus¾The Real-Time price for the Settlement Meter which measures ESR Load at Electrical Bus b, for the 15-minute Settlement Interval. |
| BP r, y | MW | Base Point per Resource per interval - The Base Point of Resource r, for the SCED interval y. |
| q | none | A QSE. |
| gsc | none | A generation site code. |
| r | none | An ESR. |
| p | none | A Resource Node Settlement Point. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An Electrical Bus. |

[NPRR1188: Replace paragraph (3) above with the following upon system implementation:] (3) For a facility with Settlement Meters that measure CLR (that is not an ALR) or ESR Load, the total payment or charge for CLR (that is not an ALR) or ESR Load is calculated for a QSE, CLR (that is not an ALR) or ESR, and Settlement Point for each 15-minute Settlement Interval. The WSL is settled as follows: WSLAMTTOT q, r, p = (RTRMPRESR b * MEBL q, r, b) The Non-WSL ESR Charging Load is settled as follows: ESRNWSLAMTTOT q, r, p = (RTRMPRESR b * MEBR q, r, b) Where: MEBR q, r, b = MEBRFG q, r, b + MEBRSG q, r, b The total Non-WSL ESR Charging Load is included in the Real-Time Adjusted Meter Load (AML) per QSE. Where the price for Settlement Meter is determined as follows: RTRMPRESR b = Max [-$251, ((RNWFL b, y * RTLMP b, y )] The CLR Load is settled as follows: CLRAMTTOT q, r, p = (RTRMPRCLR b * MEBCL q, r, b) Where: MEBCL q, r, b = MEBCLFG q, r, b + MEBCLSG q, r, b The total CLR Load is included in the Real-Time AML per QSE. Where the price for Settlement Meter is determined as follows: RTRMPRCLR b = Max [-$251, ((RNWFL b, y * RTLMP b, y))] Where the weighting factor for the Electrical Bus associated with the meter is: RNWFL b, y = [Max (0.001, ABS( Min(0, BP r, y))) * TLMP y] / [Max (0.001, ABS( Min(0, BP r, y))) * TLMP y] The summation is over all CLR (that is not an ALR) or ESR Load r associated to the individual meter. The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc. The above variables are defined as follows: Variable; Unit; Description. RTLMP b, y; $/MWh; Real-Time Locational Marginal Price at bus per interval¾The Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y.. TLMP y; second; Duration of SCED interval per interval¾The duration of the SCED interval y.. MEBL q,r,b; MWh; Metered Energy for Wholesale Storage Load at Bus¾The WSL energy metered by the Settlement Meter which measures WSL for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. MEBCL q, r, b; MWh; Calculated Metered Energy for CLR Load at Bus - The calculated CLR Load, adjusted for UFE, for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. MEBCLFG q, r, b; MWh; Adjusted Metered Energy for CLR Load supplied from the grid at Bus (Calculated)—The portion of energy metered by the Settlement Meter which measures CLR Load supplied from the grid that is adjusted for losses, for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. MEBCLSG q, r, b; MWh; Metered Energy for CLR Load supplied from co-located generation with Net Metering arrangement, at Bus (Calculated) —The portion of energy metered by the Settlement Meter which measures CLR Load supplied from the co-located generation with Net Metering arrangement. This is not adjusted for losses, for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. MEBR q, r, b; MWh; Calculated Metered Energy for Energy Storage Resource Load at Bus - The calculated Non-WSL ESR Charging Load, adjusted for UFE, for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. MEBRFG q, r, b; MWh; Adjusted Metered Energy for Energy Storage Resource Load supplied from the grid at Bus (Calculated) —The portion of energy metered by the Settlement Meter which measures Non-WSL ESR Charging Load supplied from the grid that is adjusted for losses, for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. MEBRSG q, r, b; MWh; Metered Energy for Energy Storage Resource Load supplied from co-located generation with Net Metering arrangement, at Bus (Calculated) —The portion of energy metered by the Settlement Meter which measures Non-WSL ESR Charging Load supplied from the co-located generation with Net Metering arrangement. This is not adjusted for losses, for the 15-minute Settlement Interval represented as a negative value, for the QSE q, Resource r, at bus b.. WSLAMTTOT q, r, p; $; Wholesale Storage Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for WSL for each 15-minute Settlement Interval.. CLRAMTTOT q, r, p; $; CLR Load Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for CLR Load for each 15-minute Settlement Interval.. ESRNWSLAMTTOT q, r, p; $; Energy Storage Resource Non-WSL Settlement—The total payment or charge to QSE q, Resource r, at Settlement Point p, for Non-WSL ESR Charging Load for each 15-minute Settlement Interval.. RNWFL b, y; none; Net meter Weighting Factor per interval for the Energy Metered as Energy Storage Resource Load or CLR LoadThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y, for the ESR Load associated with an ESR or for the CLR Load associated with a CLR that is not an ALR. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters.. RTRMPRESR b; $/MWh; Real-Time Price for the Energy Metered as Energy Storage Resource Load at bus¾The Real-Time price for the Settlement Meter which measures ESR Load at Electrical Bus b, for the 15-minute Settlement Interval.. RTRMPRCLR b; $/MWh; Real-Time Price for the CLR Energy Metered at bus¾The Real-Time price for the Settlement Meter which measures CLR Load at Electrical Bus b, for the 15-minute Settlement Interval.. BP r, y; MW; Base Point per Resource per interval - The Base Point of Resource r, for the SCED interval y.. q; none; A QSE.. gsc; none; A generation site code.. r; none; A CLR (that is not an ALR) or an ESR.. p; none; A Resource Node Settlement Point.. y; none; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval.. b; none; An Electrical Bus..

(4)	The total payment or charge to a Facility with a net metering arrangement for each 15-minute Settlement Interval shall be calculated as follows:

NMRTETOT gsc 	= 	Max (0, ( (MEB gsc, b + MEBC gsc, b)))

If NMRTETOT gsc = 0 for a 15-minute Settlement Interval, then

The Load that is not WSL is included in the Real-Time AML per QSE.

Otherwise, when NMRTETOT gsc > 0 for a 15-minute Settlement Interval, then

NMSAMTTOT gsc	=	 [(RTRMPR b * MEB gsc, b) + (RTRMPR b * MEBC gsc, b)]

Where the price for Settlement Meter is determined as follows:

RTRMPR b 		=	Max [-$251, ((RNWF b, y * RTLMP b, y))]

Where the weighting factor for the Electrical Bus associated with the meter is:

RNWF b, y 		= [Max (0.001, Max (0, BP r, y)) * TLMP y] /

[Max (0.001, Max (0, BP r, y)) * TLMP y]

The summation is over all Resources r associated to the individual meter.  The determination of which Resources are associated to an individual meter is static and based on the normal system configuration of the generation site code, gsc.

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| NMRTETOT gsc | MWh | Net Meter Real-Time Energy Total—The net sum for all Settlement Meters included in generation site code gsc. A positive value indicates an injection of power to the ERCOT System. |
| NMSAMTTOT gsc | $ | Net Metering Settlement—The total payment or charge to a generation site with a net metering arrangement. |
| RTRMPR b | $/MWh | Real-Time Price for the Energy Metered for each Resource meter at bus¾The Real-Time price for the Settlement Meter at Electrical Bus b, for the 15-minute Settlement Interval. |
| MEB gsc, b | MWh | Metered Energy at Bus¾The metered energy by the Settlement Meter which is not upstream from another Settlement Meter which measures ESR Load for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy withdrawn. [NPRR1188: Replace the description above with the following upon system implementation:] Metered Energy at Bus¾The metered energy by the Settlement Meter which is not upstream from another Settlement Meter which measures CLR (that is not an ALR) or ESR Load for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy withdrawn.. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per interval¾The Real-Time LMP for the meter at Electrical Bus b, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the SCED interval y. |
| RNWF b, y | none | Net meter Weighting Factor per intervalThe weight factor used in net meter price calculation for meters in Electrical Bus b, for the SCED interval y. The weighting factor used in the net meter price calculation shall not be recalculated after the fact due to revisions in the association of Resources to Settlement Meters. |
| BP r, y | MW | Base Point per Resource per intervalThe Base Point of Resource r, for the SCED interval y. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| MEBC gsc, b | MWh | Metered Energy at Bus (Calculated) ¾ The calculated energy for the 15-minute Settlement Interval for a Settlement Meter which is upstream from another Settlement Meter which measures ESR Load. A positive value represents energy produced, and a negative value represents energy withdrawn. [NPRR1188: Replace the description above with the following upon system implementation:] Metered Energy at Bus (Calculated) ¾ The calculated energy for the 15-minute Settlement Interval for a Settlement Meter which is upstream from another Settlement Meter which measures CLR (that is not an ALR) or ESR Load. A positive value represents energy produced, and a negative value represents energy withdrawn. This is not adjusted for losses and UFE.. |
| gsc | none | A generation site code. |
| r | none | A Generation Resource or ESR that is located at the Facility with net metering. |
| y | none | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| b | none | An Electrical Bus. |

(5)	The Generation Resource or ESR SCADA Splitting Percentage for each Resource within a net metering arrangement for the 15-minute Settlement Interval is calculated as follows:

GSPLITPER q,  r, gsc, p 	= Max(GSSPLITSCA r, 0) / Max(GSSPLITSCA r, 0)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| GSPLITPER q, r, gsc, p | none | Generation Resource SCADA Splitting Percentage—The generation allocation percentage for Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p. GSPLITPER is calculated by taking the positive SCADA values (GSSPLITSCA) for a particular Generation Resource or ESR r that is part of a net metering configuration and dividing by the sum of all positive SCADA values for all Resources that are included in the net metering configuration for each interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| GSSPLITSCA r | MWh | Generation Resource SCADA Net Real Power provided via Telemetry—The positive net real power provided via telemetry per Resource within the net metering arrangement, integrated for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| gsc | none | A generation site code. |
| r | none | A Generation Resource or ESR that is located at the Facility with net metering. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |

(6)	The total net payments and charges to each QSE for Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval is calculated as follows:

RTEIAMTQSETOT q	=	 RTEIAMT q, p

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTEIAMTQSETOT q | $ | Real-Time Energy Imbalance Amount QSE Total per QSE¾The total net payments and charges to QSE q for Real-Time Energy Imbalance Service at all Resource Node Settlement Points for the 15-minute Settlement Interval. |
| RTEIAMT q, p | $ | Real-Time Energy Imbalance Amount per QSE per Settlement Point—The payment or charge to QSE q for Real-Time Energy Imbalance Service at Settlement Point p, for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| p | none | A Resource Node Settlement Point. |

#### 6.6.3.6	Real-Time High Dispatch Limit Override Energy Payment

(1)	If ERCOT directs a Generation Resource or ESR to reduce real power output by employing a manual High Dispatch Limit (HDL) override, or issues a Verbal Dispatch Instruction (VDI) to a Generation Resource or ESR to adjust its operation to produce the same effect, and the reduction causes the QSE to suffer a demonstrable financial loss, the QSE may be eligible for a Real-Time High Dispatch Limit Override Energy Payment, as calculated below, upon providing documented proof of that loss.  In order to qualify for this payment the QSE must:

(a)	Have complied with ERCOT Dispatch Instructions to reduce real power output;

(b)	Have either received a SCED Base Point equal to the Resource’s HDL override value or received a SCED Base Point less than the Resource’s output level at the time of the instruction but greater than or equal to the instructed operating level specified in the VDI, during the 15-minute Settlement Interval;

(c)	Have incurred a demonstrable financial loss (excluding lost opportunity costs) caused by the HDL override associated with one of the following:

(i)	Variable cost components of DAM obligations;

(ii)	QSEs representing only Generation Resources in their portfolio with an HDL override for a Resource with a bilateral contract to sell energy at its Resource Node; or

(iii)	Incremental costs incurred by a QSE in the RTM to serve its Load if the HDL override for a Resource in the same QSE portfolio as the Load, causes the QSE to be short energy compared to its Load; and

(d)	File a timely Settlement and billing dispute in accordance with Section 9.14, Settlement and Billing Dispute Process, including the following items:

(i)	An attestation signed by an officer or executive with authority to bind the QSE;

(ii)	The dollar amount and calculation of the financial loss by Settlement Interval;

(iii)	An explanation of the nature of the loss and how it was attributable to the HDL override or equivalent VDI issued by ERCOT; and

(iv)	Sufficient documentation to support the QSE’s calculation of the amount of the financial loss.

(2)	Notwithstanding the attestation requirement described in paragraph (1)(d) above, for QSEs filing a demonstrable financial loss per paragraph (1)(c)(iii) above, the attestation must also state that the Resource with the HDL override was serving the Load in the same QSE portfolio as the Resource, at the time the HDL override was issued.

(3)        If the total Settlement amount of High Dispatch Limit Override Energy Payments exceeds $3.5 million in a calendar year, ERCOT will report to the Technical Advisory Committee (TAC) the causes of the payments and provide recommendations on how to reduce the costs both operationally and based on the eligible demonstrable financial loss criteria in paragraph (1)(c) above.

(4)	ERCOT may request additional supporting documentation or explanation with respect to the submitted materials within 15 Business Days of receipt.  Additional information requested by ERCOT must be provided by the QSE within 15 Business Days of ERCOT’s request.  ERCOT will provide Notice of its acceptance or rejection of the claim for the High Dispatch Limit Override Energy Payment within 15 Business Days of the updated submission.

(5)	The Energy Offer Curve or Energy Bid/Offer Curve used to calculate the Real-Time High Dispatch Limit Override Energy Payment will be the most recent valid Energy Offer Curve or Energy Bid/Offer Curve received by ERCOT that was effective for the disputed interval(s) when the HDL override or equivalent VDI was issued.  If no curve exists for the interval being disputed, ERCOT will use the most recent valid Energy Offer Curve or Energy Bid/Offer Curve received before the HDL override or equivalent VDI was issued for an interval prior to the disputed interval(s).

(6)	The amount recoverable under this section shall be offset by any Ancillary Service Imbalance revenues received by the QSE that the QSE would not have earned had ERCOT not issued an HDL override.

The payment shall be calculated as follows:

HDLOEAMT q, r, p, i =  	(-1) * Min {HDLOALq,r,p,i, Max(0, ((RTSPPp, i  – RTEOCOSTq,r,i) * HDLOQTY q, r, p, i))}Where:

HDLOQTY q, r, p, i       =  Max(0, (¼ (HDLOBRKP q, r, p, i – AVGHDL q, r, p, i)))

HDLOBRKP q, r, p, i      =  Min(AVGHSL q, r, p, i, HDLOBRKPCP q, r, p, i)

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| HDLOAL q, r, p, i | $ | High Dispatch Limit override attested losses—The financial loss to the Resource r represented by QSE q due to the HDL override as attested by the QSE in accordance with paragraph (1)(d) above. For a combined cycle Resource, r is a Combined Cycle Train. |
| HDLOEAMT q, r, p, i | $ | High Dispatch Limit override energy amount per QSE per Generation Resource—The payment to QSE q for an ERCOT-issued HDL override or equivalent VDI for Resource r at Settlement Point p for the 15-minute Settlement Interval i. For a combined cycle Resource, r is a Combined Cycle Train. |
| HDLOBRKP q, r, p, i | MW | High Dispatch Limit override break point per QSE per Resource—The point on the Energy Offer Curve or Energy Bid/Offer Curve corresponding to the lesser of the AVGHSL or the interception between the RTSPP of the Resource r represented by QSE q minus the Real-Time Reliability Deployment Price for Energy and the Energy Offer Curve Cost Cap of Resource r represented by QSE q, for the 15-minute Settlement Interval i. For a combined cycle Resource, r is a Combined Cycle Train. |
| AVGHDL q, r, p, i | MW | Average High Dispatch Limit per QSE per Settlement Point per Resource—The time-weighted average of all 4-second HDL values calculated by the Resource Limit Calculator, subject to the maximum of the manual HDL override or equivalent VDI and the telemetered output, for the Generation Resource or ESR r represented by QSE q at Settlement Point p within the 15-minute Settlement Interval i. For a Combined Cycle Train, the Resource r is a Combined Cycle Train. |
| AVGHSL q, r, p, i | MW | Average High Sustained Limit per QSE per Settlement Point per Resource—The time-weighted average High Sustained Limit (HSL) for the Generation Resource or ESR r represented by QSE q at Settlement Point p within the 15-minute Settlement Interval i. For a Combined Cycle Train, the Resource r is a Combined Cycle Train. In the case of a VDI that is equivalent to an HDL override, this value is set equal to the HSL of Generation Resource, or ESR r at the time that the VDI is issued to the QSE. |
| HDLOBRKPCP q, r, p, i | MW | High Dispatch Limit override break point at clearing price per QSE per Resource—The MW value on the Energy Offer Curve or Energy Bid/Offer Curve corresponding to the Real-Time Settlement Point Price of Resource r represented by QSE q at Settlement Point p minus the Real-Time Reliability Deployment Price for Energy. For a combined cycle Resource, r is a Combined Cycle Train. |
| RTEOCOST q, r, i | $/MWh | Real-Time Energy Offer Curve Cost Cap—The Energy Offer Curve Cost Cap for Resource r represented by QSE q, for the Resource’s generation above the Low Sustained Limit (LSL) for the Settlement Interval i. See Section 4.4.9.3.3, Energy Offer Curve Cost Caps. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| HDLOQTY q, r, p, i | MWh | High Dispatch Limit override quantity per QSE per Generation Resource—The difference between the HDLOBRKP and the AVGHDL due to an ERCOT-issued HDL override or equivalent VDI for Resource r represented by QSE q at Settlement Point p for the 15-minute Settlement Interval i. For a combined cycle Resource, r is a Combined Cycle Train. |
| RTSPP p, i | $/MWh | Real-Time Settlement Point Price per Settlement Point—The Real-Time Settlement Point Price at Settlement Point p, for the 15-minute Settlement Interval i. |
| q | none | A QSE. |
| r | none | A Generation Resource or ESR. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval. |

(7)	The total compensation to each QSE for an HDL override for the 15-minute Settlement Interval is calculated as follows:

HDLOEAMTQSETOT q, i  =  HDLOEAMT q, r, p, i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| HDLOEAMT q, r, p, i | $ | High Dispatch Limit override energy amount per QSE per Resource—The payment to QSE q for an ERCOT-issued HDL override or equivalent VDI for Resource r at Settlement Point p for the 15-minute Settlement Interval i. For a combined cycle Resource, r is a Combined Cycle Train. |
| HDLOEAMTQSETOT q, i | $ | High Dispatch Limit override energy amount QSE total per QSE—The total of the energy payments to QSE q as compensation for HDL overrides for this QSE for the 15-minute Settlement Interval i. |
| q | none | A QSE. |
| r | none | A Generation Resource or ESR. |
| p | none | A Resource Node Settlement Point. |
| i | none | A 15-minute Settlement Interval. |

#### 6.6.3.8	Real-Time Payment or Charge for Energy from a Settlement Only Distribution Generator (SODG) or a Settlement Only Transmission Generator (SOTG)

(1)	The payment or charge to each QSE for energy from an SODG or an SOTG shall be based on an identified nodal energy price, RTESOGPR, as described in this subsection, with the following exceptions:

(a)	An SODG or SOTG that has opted out of nodal pricing as described in paragraph (5) below; or

(b)	Any site with one or more ESS SODGs or SOTGs where the ESS capacity constitutes more than 50% of the site’s total SOG nameplate capacity.

(2)	For an SODG, the price used as the basis for the 15-minute Real-Time price calculation is the time-weighted price at the Electrical Bus associated with this mapped Load in the Network Operations Model.  For an SOTG, the price used as the basis for the 15-minute Real-Time price calculation is the time-weighted price at the Electrical Bus as determined by ERCOT in review of the meter location of the SOTG in the Network Operations Model.  SODG and SOTG sites will be represented as a single unit in the ERCOT Settlement system.

(3)	For an SODG or an SOTG, the total payment or charge for each 15-minute Settlement Interval shall be calculated as follows:

MEBSOGNET q, gsc =   Max(0, MEBSOG q, gsc, b)

If MEBSOGNET q, gsc = 0 for a 15-minute Settlement Interval, then

The Load is included in the Real-Time AML per QSE and is included in the Real-Time energy imbalance payment or charge at a Load Zone.

Otherwise, when MEBSOGNET q, gsc > 0 for a 15-minute Settlement Interval, then

RTESOGSAMT q, gsc	=	(-1) * [( RTESOGPR b * MEBSOG q, gsc, b)]

Where the price for the SOTG or SODG is determined as follows:

RTESOGPR b 	=	Max [-$251, (SDWF y * RTLMP b, y)]

Where:

SDWF y	=	TLMP y / TLMP y

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| RTESOGSAMT q, gsc | $ | Real-Time Energy for SODG and SOTG Site Amount —The total payment or charge to QSE q for SODG or SOTG site gsc for the 15-minute Settlement Interval. |
| RTESOGPR b | $/MWh | Real-Time Price for the Energy Metered for each SODG or SOTG Site ¾The Real-Time price at Electrical Bus b for the Settlement Meter for the SODG or SOTG site for the 15-minute Settlement Interval. |
| MEBSOGNET q, gsc | MWh | Net Metered energy at gsc for an SODG or SOTG Site ¾The net sum for all Settlement Meters for SODG or SOTG site gsc represented by QSE q. A positive value indicates an injection of power to the ERCOT System. |
| MEBSOG q, gsc, b | MWh | Metered energy at bus for an SODG or SOTG Site ¾The metered energy by the Settlement Meter(s) at Electrical Bus b for SODG or SOTG site gsc represented by QSE q. A positive value represents energy produced, and a negative value represents energy consumed. |
| RTRDPA y | $/MWh | Real-Time Reliability Deployment Price Adder for Energy ¾The Real-Time price adder that captures the impact of reliability deployments on energy prices for the SCED interval y. |
| SDWF y | None | SCED Duration Weighting Factor per interval¾The weight used in the SODG or SOTG price calculation for the portion of the SCED interval y within the Settlement Interval. |
| RTLMP b, y | $/MWh | Real-Time Locational Marginal Price at bus per interval¾The Real-Time LMP at Electrical Bus b, for the SCED interval y. |
| TLMP y | second | Duration of SCED interval per interval¾The duration of the SCED interval y within the Settlement Interval. |
| gsc | none | A generation site code. |
| b | none | An Electrical Bus. |
| y | None | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |

(4)	The total net payments and charges to each QSE for energy from SODGs and SOTGs for the 15-minute Settlement Interval is calculated as follows:

RTESOGAMTQSETOT q	= RTESOGSAMT q, gsc

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RTESOGAMTQSETOT q | $ | Real-Time Energy Payment or Charge per QSE for Energy from SODGs and SOTGs —The payment or charge to QSE q for Real-Time energy from SODGs and SOTGs, for the 15-minute Settlement Interval. |
| RTESOGSAMT q, gsc | $ | Real-Time Energy for SODG and SOTG Site Amount —The total payment or charge to QSE q for an SODG or SOTG site gsc for the 15-minute Settlement Interval. |
| q | none | A QSE. |
| gsc | none | A generation site code. |

(5) 	Notwithstanding anything else in this Section except paragraphs (6) and (7) below, a Resource Entity may opt out of nodal pricing and continue Load Zone Settlement for any SODG or SOTG if, by January 1, 2019, the SODG or SOTG was operational or was subject to a Power Purchase or Tolling Agreement (PPA) or Transmission and/or Distribution Service Provider (TDSP) interconnection agreement, or had an executed agreement with a developer.  By December 31, 2019, the Resource Entity must submit a properly completed Section 23, Form N, Pricing Election for Settlement Only Distribution Generators and Settlement Only Transmission Generators.  Any SODG or SOTG relying on a PPA or TDSP interconnection agreement or agreement with a developer must also have achieved Initial Synchronization for the full Resource capacity before June 1, 2020 to be eligible to opt out of nodal pricing.  A Resource Entity must provide ERCOT documented proof of any PPA, TDSP interconnection agreement, or developer agreement that it relies on as a basis for any election under this paragraph.  This election is valid through the earlier of December 31, 2029 or the date on which the election is revoked pursuant to paragraph (8) of this Section.  On January 1, 2030, all SODGs and SOTGs will be subject to nodal pricing.

(6)	For any SODG or SOTG for which the applicable Resource Entity has elected to opt out of nodal pricing, ERCOT shall settle the output of the SODG or SOTG using the Load Zone Settlement Point Price for the duration of the opt-out period so long as the SODG or SOTG is not physically modified for any purpose, including to increase the capacity of the unit or change the fuel type of the unit, except as necessary for routine maintenance or repairs to address normal wear and tear.

(7)	If at any time ERCOT determines that the SODG or SOTG fails to meet the opt-out conditions in paragraph (6) above, ERCOT shall settle the output of the SODG or SOTG at the applicable nodal price as soon as practicable after providing written notice to the affected Resource Entity.

(8)	A Resource Entity that has opted out of nodal pricing for one or more SODGs or SOTGs pursuant to paragraph (5) of this Section may withdraw that election and begin receiving applicable nodal pricing for one or more such generators by submitting a properly completed election form (Section 23, Form N).  An election of nodal pricing is irrevocable.  ERCOT will effectuate the transition of an SODG or SOTG to nodal pricing in ERCOT Settlement systems as soon as practicable.

[NPRR995: Replace Section 6.6.3.8 above with the following upon system implementation:] #### 6.6.3.8 Real-Time Payment or Charge for Energy from a Settlement Only Distribution Generator (SODG), Settlement Only Transmission Generator (SOTG), Settlement Only Distribution Energy Storage System (SODESS), or Settlement Only Transmission Energy Storage System (SOTESS) (1) The payment or charge to each QSE for energy from an SODG, SOTG, SODESS, or SOTESS shall be based on an identified nodal energy price, RTESOPR, as described in this subsection, with the exception of an SODG or SOTG that has opted out of nodal pricing as described in paragraph (7) below. (2) For an SODG or an SODESS, the price used as the basis for the 15-minute Real-Time price calculation is the time-weighted price at the Electrical Bus associated with this mapped Load in the Network Operations Model. For an SOTG or an SOTESS, the price used as the basis for the 15-minute Real-Time price calculation is the time-weighted price at the Electrical Bus as determined by ERCOT in review of the meter location of the SOTG or SOTESS in the Network Operations Model. Load that is not WSL will be included in the Real-Time AML per QSE. Each SODG, SOTG, SODESS, and SOTESS site will be represented as a single unit in the ERCOT Settlement system. (3) For an SODG, SOTG, SODESS, or SOTESS, the total payment or charge for each 15-minute Settlement Interval shall be calculated as follows: MEBSOGNET q, gsc = Max(0, MEBSOG q, gsc, b) If MEBSOGNET q, gsc = 0 for a 15-minute Settlement Interval, then The Load is included in the Real-Time AML per QSE, excluding WSL. Otherwise, when MEBSOGNET q, gsc > 0 for a 15-minute Settlement Interval, then RTGSOAMT q, gsc = (-1) * [(RTESOPR b * MEBSOG q, gsc, b)] (4) For an SODESS or SOTESS, the total payment or charge for each 15-minute Settlement Interval shall be calculated as follows: RTWSLSOAMT q, gsc = (-1) * [(RTESOPR b * WSOL q, gsc, b)] RTNWSLSOAMT q, gsc = (-1) * [(RTESOPR b * NWSOL q, gsc, b)] (5) The price for the SOTG, SODG, SODESS, or SOTESS is determined as follows: RTESOPR b = Max [-$251, ((SDWF y * RTLMP b,))] Where: SDWF y = TLMP y / TLMP y The above variables are defined as follows: Variable; Unit; Description. RTGSOAMT q, gsc; $; Real-Time Generation for SODG, SOTG, SODESS, or SOTESS Site Amount —The total payment or charge for generation to QSE q for SODG, SOTG, SODESS, or SOTESS site gsc for the 15-minute Settlement Interval.. RTWSLSOAMT q, gsc; $; Real-Time WSL for SODESS or SOTESS Site Amount —The total payment or charge for WSL to QSE q for the SODESS or SOTESS site gsc for the 15-minute Settlement Interval.. RTNWSLSOAMT q, gsc; $; Real-Time Non-WSL for SODESS or SOTESS Site Amount —The total payment or charge for Non-WSL Settlement Only Charging Load to QSE q for the SODESS or SOTESS site gsc for the 15-minute Settlement Interval.. RTESOPR b; $/MWh; Real-Time Price for the Energy Metered for each SODG, SOTG, SODESS, or SOTESS Site ¾The Real-Time price at Electrical Bus b for the Settlement Meter for the SODG, SOTG, SODESS, or SOTESS site for the 15-minute Settlement Interval.. MEBSOGNET q, gsc; MWh; Net Metered energy at gsc for an SODG, SOTG, SODESS or SOTESS Site ¾The net sum for all Settlement Meters for SODG, SOTG, SODESS, or SOTESS site gsc represented by QSE q. A positive value indicates an injection of power to the ERCOT System.. MEBSOG q, gsc, b; MWh; Metered energy at bus for an SODG, SOTG, SODESS, or SOTESS Site ¾The metered energy by the Settlement Meter(s) at Electrical Bus b for SODG, SOTG, SODESS, or SOTESS site gsc represented by QSE q for the 15-minute Settlement Interval. A positive value represents energy produced, and a negative value represents energy consumed.. WSOL q, gsc, b; MWh; WSL for an SODESS or SOTESS Site¾The WSL as measured for an SODESS or SOTESS site gsc at Electrical Bus b, represented by QSE q, represented as a negative value, for the 15-minute Settlement Interval.. NWSOL q, gsc, b; MWh; Non-WSL Settlement Only Charging Load for an SODESS or SOTESS Site¾The Non-WSL Settlement Only Charging Load as measured for an SODESS or SOTESS site gsc at Electrical Bus b, represented by QSE q, represented as a negative value, for the 15-minute Settlement Interval.. SDWF y; None; SCED Duration Weighting Factor per interval ¾The weight used in the SODG, SOTG, SODESS, or SOTESS price calculation for the portion of the SCED interval y within the Settlement Interval.. RTLMP b, y; $/MWh; Real-Time Locational Marginal Price at bus per interval ¾The Real-Time LMP at Electrical Bus b, for the SCED interval y.. TLMP y; second; Duration of SCED interval per interval ¾The duration of the SCED interval y within the Settlement Interval.. gsc; none; A generation site code.. b; none; An Electrical Bus.. y; None; A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval.. (6) The total net payments and charges to each QSE for energy from SODGs, SOTGs, SODESSs, or SOTESSs for the 15-minute Settlement Interval is calculated as follows: RTESOAMTQSETOT q = (RTGSOAMT q, gsc + RTWSLSOAMT q, gsc + RTNWSLSOAMT q, gsc) The above variables are defined as follows: Variable; Unit; Definition. RTESOAMTQSETOT q; $; Real-Time Energy Payment or Charge per QSE for SODGs, SOTGs, SODESSs, or SOTESSs —The payment or charge to QSE q for Real-Time energy from SODGs, SOTGs, SODESSs, or SOTESSs, for the 15-minute Settlement Interval.. RTGSOAMT q, gsc; $; Real-Time Generation for SODG, SOTG, SODESS, or SOTESS Site Amount —The total payment or charge for generation to QSE q for SODG, SOTG, SODESS, or SOTESS site gsc for the 15-minute Settlement Interval.. RTWSLSOAMT q, gsc; $; Real-Time WSL for SODESS or SOTESS Site Amount —The total payment or charge for WSL to QSE q for the SODESS or SOTESS site gsc for the 15-minute Settlement Interval.. RTNWSLSOAMT q, gsc; $; Real-Time Non-WSL for SODESS or SOTESS Site Amount —The total payment or charge for Non-WSL Settlement Only Charging Load to QSE q for the SODESS or SOTESS site gsc for the 15-minute Settlement Interval.. q; none; A QSE.. gsc; none; A generation site code.. (7) Notwithstanding anything else in this Section except paragraphs (8) and (9) below, a Resource Entity may opt out of nodal pricing and continue Load Zone Settlement for any SODG or SOTG if, by January 1, 2019, the SODG or SOTG was operational or was subject to a Power Purchase or Tolling Agreement (PPA) or Transmission and/or Distribution Service Provider (TDSP) interconnection agreement, or had an executed agreement with a developer. By December 31, 2019, the Resource Entity must submit a properly completed Section 23, Form N, Pricing Election for Settlement Only Distribution Generators and Settlement Only Transmission Generators. Any SODG or SOTG relying on a PPA or TDSP interconnection agreement or agreement with a developer must also have achieved Initial Synchronization for the full Resource capacity before June 1, 2020 to be eligible to opt out of nodal pricing. A Resource Entity must provide ERCOT documented proof of any PPA, TDSP interconnection agreement, or developer agreement that it relies on as a basis for any election under this paragraph. This election is valid through the earlier of December 31, 2029 or the date on which the election is revoked pursuant to paragraph (10) of this Section. On January 1, 2030, all SODGs and SOTGs will be subject to nodal pricing. (8) For any SODG or SOTG for which the applicable Resource Entity has elected to opt out of nodal pricing, ERCOT shall settle the output of the SODG or SOTG using the Load Zone Settlement Point Price for the duration of the opt-out period so long as the SODG or SOTG is not physically modified for any purpose, including to increase the capacity of the unit or change the fuel type of the unit, except as necessary for routine maintenance or repairs to address normal wear and tear. (9) If at any time ERCOT determines that the SODG or SOTG fails to meet the opt-out conditions in paragraph (8) above, ERCOT shall settle the output of the SODG or SOTG at the applicable nodal price as soon as practicable after providing written notice to the affected Resource Entity. (10) A Resource Entity that has opted out of nodal pricing for one or more SODGs or SOTGs pursuant to paragraph (7) of this Section may withdraw that election and begin receiving applicable nodal pricing for one or more such generators by submitting a properly completed election form (Section 23, Form N). An election of nodal pricing is irrevocable. ERCOT will effectuate the transition of an SODG or SOTG to nodal pricing in ERCOT Settlement systems as soon as practicable.

#### 6.6.6.3	RMR Adjustment Charge

(1)	Each QSE that represents an RMR Unit shall pay a charge designed to recover the net total revenues from RUC settlements, and from Real-Time settlements received by that QSE for all RMR Units that it represents, except that the charge does not include net revenues received by the QSE for the RMR Standby Payments calculated under Section 6.6.6.1, RMR Standby Payment, and the RMR energy payments calculated under Section 6.6.6.2, RMR Payment for Energy.

(2)	The charge for each QSE representing an RMR Unit for a given Operating Hour is calculated as follows:

RMRAAMT q	=	(-1) * [((-1) * RESREV q, r, gsc, p + EMREAMT q, r, p, i + RUCMWAMT q, r, p + RUCCBAMT q, r, p + RUCDCAMT q, r, p + VSSEAMT q, r, p, i + VSSVARAMT q, r, i +  RDIGA q, r, i)]

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RMRAAMT q | $ | RMR Adjustment Charge per QSE—The adjustment from QSE q Standby Payments and energy payments for all RMR Units represented by this QSE, for the revenues received for the same RMR Units from RUC and Real-Time operations, for the hour. |
| EMREAMT q, r, p, i | $ | Emergency Energy Amount per QSE per Settlement Point per unit per interval—The payment to QSE q as additional compensation for the additional energy or Ancillary Services produced or consumed by Resource r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. Payment for emergency energy is made to the Combined Cycle Train. |
| RESREV q, r, gsc, p | $ | Resource Share Revenue Settlement Payment—The RMR Resource share of the total payment to the entire Facility with a net metering arrangement attributed to Resource r that is part of a generation site code gsc for the QSE q at Settlement Point p. |
| RUCMWAMT q, r, p | $ | RUC Make-Whole Amount per QSE per Settlement Point per unit—The amount calculated for RMR Unit r committed in RUC at Resource Node p to make whole the Startup Cost and minimum-energy cost of this unit, for the hour. See Section 5.7.1, RUC Make-Whole Payment. When one or more Combined Cycle Generation Resources are committed by RUC, payment is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCCBAMT q, r | $ | RUC Clawback Charge per QSE per unit—The RUC Clawback Charge to QSE q for RMR Unit r, for the hour. See Section 5.7.2, RUC Clawback Charge. When one or more Combined Cycle Generation Resources are committed by RUC, a charge is made to the Combined Cycle Train for all RUC-committed Combined Cycle Generation Resources. |
| RUCDCAMT q, r, p | $ | RUC Decommitment Amount per QSE per Settlement Point per unit—The amount calculated for RMR Unit r at Resource Node p represented by QSE q due to ERCOT de-commitment, for the hour. When one or more Combined Cycle Generation Resources are decommitted by RUC, payment is made to the Combined Cycle Train for all RUC-decommitted Combined Cycle Generation Resources. |
| VSSEAMT q, r, p, i | $ | Voltage Support Service Energy Amount per QSE per Settlement Point per unit per interval —The compensation to QSE q for ERCOT-directed power reduction from RMR Unit r at Resource Node p to provide Voltage Support Service (VSS), for the 15-minute Settlement Interval i. Payment for VSS is made to the Combined Cycle Train. |
| VSSVARAMT q, r, i | $ | Voltage Support Service VAr Amount per QSE per Unit—The payment to QSE q for the VSS provided by RMR Unit r, for the 15-minute Settlement Interval i. Payment for VSS is made to the Combined Cycle Train. |
| RDIGA q, r, i | $ | Reliability Deployment Indifference Amount per QSE per Generation Resource—The Reliability Deployment Indifference Payment to QSE q for Generation Resource r for the 15-minute Settlement Interval. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| gsc | none | A generation site code. |
| p | none | A Resource Node Settlement Point. |
| r | none | An RMR Unit. |
| i | none | A 15-minute Settlement Interval in the hour. |

[NPRR885: Insert Section 6.6.6.10 below upon system implementation:] #### 6.6.6.10 MRA Variable Payment for Deployment (1) The variable payment to each QSE representing a Generation Resource MRA: Outside of the MRA Contracted Hours, a Generation Resource MRA shall be treated in Settlements in the same manner as any Generation Resource registered with ERCOT For MRA Contracted Hours with a deployment instruction: MRAVAMT q, r, h = (-1) * (MRAGRCVP q, r, h – MRARTREV q, r, h) For MRA Contracted Hours without a deployment instruction: MRAVAMT q, r, h = (-1) * (Min (MRAGRCVP q, r, h, MRARTREV q, r, h) – MRARTREV q, r, h) Where, MRAGRCVP q, r, h = Max [VPRICE q, r, (FIP + MRACEFA q, r) * MRAPHR q, r] * Min(RTMG q, r, p, i , MRACCAP q, r, m / 4) MRARTREV q,r,h = Max [0, (RESREV q, r, gsc, p, i + (-1) * (EMREAMT q, r, p, i + VSSVARAMT q, r, i + VSSEAMT q, r, i + RDIGA q, r, i))] (2) The variable payment to each QSE representing an Energy Storage Resource (ESR) MRA: (a) ESR MRA will be compensated for energy consumed to re-charge the MRA capability after the hours of injection to the ERCOT system during an event deployment or an ERCOT-required Capacity Test based on the cost of the energy as metered by the meter recording load. The QSE will not be compensated for energy costs incurred during a re-test. An ESR shall only consume energy in hours that are not Contracted Hours, and it must be re-charged to a level sufficient to provide the contracted amount of MRA Service prior to the next start of a block of Contracted Hours. Additionally, the QSE shall use its best efforts to minimize the cost to re-charge and submit bid-to-buy curves taking into consideration estimates of future prices. The cost to re-charge shall be determined by starting with the least expensive consumed energy. (b) Outside of the MRA Contracted Hours, an ESR MRA shall be treated in Settlements in the same manner as any ESR registered with ERCOT. For MRA Contracted Hours with a deployment instruction: MRAVAMT q, r, h = (-1) * (MRAESRCVP q, r, h – MRARTREV q, r, h) For MRA Contracted Hours without a deployment instruction: MRAVAMT q, r, h = (-1) * (Min (MRAESRCVP q, r, h, MRARTREV q, r, h) – MRARTREV q, r, h) Where, MRAESRCVP q, r, h = Max [VPRICE q, r, ESRARCOST q, r] * Min(RTMG q, r, p, i, MRACCAP q, r, m / 4) MRARTREV q, r, h = Max [0, (RESREV q, r, gsc, p, i + (-1) * (EMREAMT q, r, p, i + VSSVARAMT q, r, i + VSSEAMT q, r, i + RDIEA q, r, i))] (3) The variable payment to each QSE representing an Other Generation MRA: For MRA Contracted Hours with a deployment instruction: MRAVAMT q, r, h = (-1) * (MRACVP q, r, h – MRACRTREV q, r, h) For MRA Contracted Hours without a deployment instruction: MRAVAMT q, r,h = (-1) * (Min(MRACVP q, r, h , MRACRTREV q, r, h) –MRACRTREV q, r, h) Where, MRACVP q, r, h = Max[VPRICE q, r, (FIP + MRACEFA q, r) * MRAPHR q, r ] * RTVQ q, r, i MRACRTREV q, r, h = (Max(0, Min(RTVQ q, r, i , MRACCAP q, r, m / 4) * RTSPP p, i)) Where, RTVQ q, r, i = MRAIPF q, r,i * MRACCAP q, r, m / 4 (4) The variable payment to each QSE representing a Demand Response MRA: For MRA Contracted Hours with a deployment instruction: MRAVAMTq, r, h = (-1) * Max[VPRICE q, r, (FIP + MRACEFA q, r) * MRAPHR q, r ] * RTVQ q, r, i Where, RTVQ q, r, i = MRAIPF q, r,i * MRACCAP q, r, m / 4 The above variables are defined as follows: Variable; Unit; Definition. MRAVAMT q, r, h; $; Must-Run Alternative Variable Amount per QSE per Resource by hour—The variable payment to QSE q for MRA r, for the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. MRAGRCVP q, r, h; $; Must-Run Alternative Generation Resource Calculated Variable Payment per QSE per Resource - The variable payment to QSE q for Generation Resource MRA r, for the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. FIP; $/MMBtu; Fuel Index Price—The FIP for the Operating Day.. MRARTREVq, r, h; $; Must-Run Alternative Real-Time Revenues per QSE per Resource by hour—The revenues received in Real-Time for QSE q for MRA r, for the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. MRAESRCVP q, r, h; $; Must-Run Alternative Energy Storage Resource Calculated Variable Payment per QSE per Resource—The variable payment to QSE q for ESR MRA r, for the hour h.. ESRARCOST q, r; $/MWh; Must-Run Alternative Energy Storage Resource Average Recharge Cost per QSE per Resource—The average cost to recharge the ESR MRA r, for QSE q, during the period the ESR is charging to restore its capability to provide the contracted amount of MRA service.. MRACCAP q, r, m; MW; Must-Run Alternative Contract Capacity per QSE per Resource—The capacity of MRA r represented by QSE q as specified in the MRA Agreement, for the month. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. MRAIPF q, r, i; none; Must-Run Alternative Interval Performance Factor per QSE per Resource for the interval— The interval performance factor of the MRA r represented by QSE q, for the 15-minute Settlement Interval i.. MRACVP q, r,h; $; Must-Run Alternative Calculated Variable Payment per QSE per Resource - The variable payment to QSE q for an Other Generation MRA or Demand Response MRA r, for the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. VSSVARAMT q, r, i; $; Voltage Support Service VAr Amount per QSE per Generation Resource - The payment to QSE q for the VSS provided by Generation Resource MRA r, for the 15-minute Settlement Interval i. Where for a combined cycle resource, r is a Combined Cycle Train.. VSSEAMT q, r, i; $; Voltage Support Service Energy Amount per QSE per Generation Resource—The lost opportunity payment to QSE q for ERCOT-directed VSS from Generation Resource MRA r for the 15-minute Settlement Interval. Where for a combined cycle resource, r is a Combined Cycle Train.. RESREV q, r, gsc, p, i; $; Resource Share Revenue Settlement Payment—The Resource share of the total payment to the entire Facility with a net metering arrangement attributed to Generation Resource MRA r that is part of a generation site code gsc for the QSE q at Settlement Point p, for the 15-minute Settlement Interval i.. EMREAMT q, r, p, i; $; Emergency Energy Amount per QSE per Settlement Point per unit per interval—The payment to QSE q as additional compensation for the additional energy or Ancillary Services produced or consumed by Resource MRA r at Resource Node p in Real-Time during the Emergency Condition, for the 15-minute Settlement Interval i. Payment for emergency energy is made to the Combined Cycle Train.. VPRICE q, r; $/MWh; Must-Run Alternative Variable Price per QSE per Resource—The variable price for QSE q for MRA r, as specified in the MRA Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. MRAPHR q, r; MMBtu /MWh; Must-Run Alternative Proxy Heat Rate per QSE per Resource – A proxy heat rate value for MRA r represented by QSE q, as specified in the MRA Agreement. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. MRACRTREV q, r, h; $; Must-Run Alternative Calculated Real-Time Revenues per QSE per Resource —The calculated variable revenue to QSE q for MRA r, for the hour.. RTVQ q, r, i,; MWh; Real-Time Variable Quantity per QSE per Resource by Settlement Interval — The Real-Time variable quantity for MRA r represented by QSE q, for the 15-minute Settlement Interval i.. RTMG q, r, p, i; MWh; Real-Time Metered Generation per QSE per Settlement Point per Generation Resource—The metered generation of Resource r at Resource Node p represented by QSE q in Real-Time for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. MRACEFA q, r; $/MMBtu; Must-Run Alternative Contractual Estimated Fuel Adder—The Estimated Fuel Adder that is contractually agreed upon in Section 22, Attachment N, Standard Form Must-Run Alternative Agreement. Where for a Combined Cycle Train, the Generation Resource r is the Combined Cycle Train.. RTSPP p, i; $/MWh; Real-Time Settlement Point Price¾The Real-Time Settlement Point Price at the Settlement Point p for the 15-minute Settlement Interval i.. RDIGA q, r, i; $; Reliability Deployment Indifference Amount per QSE per Generation Resource—The Reliability Deployment Indifference Payment to QSE q for Generation Resource r for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. RDIEA q, r, i; $; Reliability Deployment Indifference Amount per QSE per Energy Storage Resource—The Reliability Deployment Indifference Payment to QSE q for Energy Storage Resource r for the 15-minute Settlement Interval i.. q; none; A QSE.. r; none; An MRA.. m; none; An MRA Contracted Month.. h; none; An MRA Contracted Hour for the MRA Contracted Month.. i; none; A 15-minute Settlement Interval during the MRA Contracted Hours.. gsc; none; A generation site code.. p; none; A Resource Node Settlement Point.. (5) The total of the variable payments for all MRAs represented by the QSE for a given hour is calculated as follows: MRAVAMTQSETOT q = MRAVAMT q, r, h The above variables are defined as follows: Variable; Unit; Definition. MRAVAMTQSETOT q; $; Must-Run Alternative Variable Amount Total per QSE by hour—The total variable payment for all MRAs r, represented by the QSE q, for the hour.. MRAVAMT q, r, h; $; Must-Run Alternative Variable Amount per QSE per Resource by hour—The variable payment to QSE q representing MRA r for the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train.. q; none; A QSE.. r; none; An MRA.. h; none; An MRA Contracted Hour for the MRA Contracted Month.. (6) The total of the variable payments for a given MRA Contracted Hour is calculated as follows: MRAVAMTTOT = MRAVAMTQSETOT q The above variables are defined as follows: Variable; Unit; Definition. MRAVAMTTOT; $; Must-Run Alternative Variable Amount Total by hour—The total variable payments for the MRA Contracted Hour.. MRAVAMTQSETOT q; $; Must-Run Alternative Variable Amount Total per QSE by hour—The total variable payment for all MRAs, represented by the QSE q, for the MRA Contracted Hour.. q; none; A QSE..

## 6.9	Reliability Deployment Indifference Payments and Charges

### 6.9.1	Reliability Deployment Indifference Payment

(1)	ERCOT shall calculate Reliability Deployment Indifference Payments to each Qualified Scheduling Entity (QSE) for SCED intervals where the binding prices for energy and Ancillary Services are from the SCED Pricing Run.

The Reliability Deployment Indifference Payments to each QSE will make Resources indifferent to any difference between their dispatch levels from the Security-Constrained Economic Dispatch (SCED) Dispatch Run and the SCED Pricing Run.

(2)	The calculation of the Reliability Deployment Indifference Payment to each QSE for each Settlement Interval is calculated based on the following information:

(a)	From the SCED Dispatch Run, the binding Base Point and binding Ancillary Service awards for each Ancillary Service type, including sub-types, for each Resource for all the SCED intervals within a 15-minute Settlement Interval where the SCED Pricing Run is active;

(b)	From the SCED Pricing Run, the binding Market Clearing Price for Capacity (MCPC) for each Ancillary Service, binding Resource Settlement Point Locational Marginal Price (LMP) for each Resource for all SCED intervals within a 15-minute Settlement Interval where the SCED Pricing Run is active;

(c)	From the SCED Pricing Run, the non-binding Base Point and non-binding Ancillary Service awards for each Ancillary Service type, including sub-types, for each Resource for all the SCED intervals within a 15-minute Settlement Interval where the SCED Pricing Run is active;

(d)	From the SCED Pricing Run Step 2, the Energy Offer Curve, Energy Bid/Offer Curve, Energy Bid Curve used;

(e)	From the SCED Pricing Run Step 2, the Ancillary Service Offers, including the proxy Ancillary Service Offer used.

(3)	For a Generation Resource or Energy Storage Resource (ESR), the contribution from energy to its Reliability Deployment Indifference Payment for each SCED interval where the SCED Pricing Run produces the binding LMPs and MCPCs, is calculated as the sum of:

(a)	The product of -1, the SCED interval duration, the Resource Settlement Point LMP floored at -$251, and the difference between the SCED Pricing Run Base Point and SCED Dispatch Run Base Point; and

(b)	The product of the SCED interval duration and the area calculated as the integral (net area) of the Energy Offer Curve for Generation Resource or Energy Bid/Offer Curve for ESR used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Base Point to the SCED Pricing Run Base Point, with respect to the zero-price line.

(4)	For a Controllable Load Resource (CLR), the contribution from energy to its Reliability Deployment Indifference Payment in each SCED interval where the SCED Pricing Run is producing the binding LMPs and MCPCs, is calculated as the sum of:

(a)	The product of -1, the SCED interval duration, the Resource Settlement Point LMP floored at -$251, and the difference between the SCED Dispatch Run Base Point and SCED Pricing Run Base Point; and

(b) 	The product of the SCED interval duration and the area calculated as the integral (net area) of the Energy Bid Curve used in Step 2 of the SCED Pricing Run, evaluated from the SCED Pricing Run Base Point to the SCED Dispatch Run Base Point, with respect to the zero-price line.

(5)	For a Resource, the contribution for a given Ancillary Service type to its Reliability Deployment Indifference Payment in each SCED interval where the SCED Pricing Run is producing the binding LMPs and MCPCs is calculated as the sum of:

(a) 	The product of -1, the SCED interval duration, the Ancillary Service MCPC, and the difference between the SCED Pricing Run Ancillary Service award and SCED Dispatch Run Ancillary Service award; and

(b) 	The product of the SCED interval duration and the area calculated as the integral (net area) of the Ancillary Service Offer used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Ancillary Service award to the SCED Pricing Run Ancillary Service award, with respect to the zero-price line. If the given Ancillary Service type has sub-Types, then this calculation is done for each Ancillary Service sub-type and summed.

(6)	For each SCED interval, the Reliability Deployment Indifference Payment to a Resource is the minimum of zero and the sum of the energy and Ancillary Service components.

(7)	The total Reliability Deployment Indifference Payment to a QSE for a given 15-minute Settlement Interval is calculated as follows:

RDIAMT q, i	=	  (RDIGA q, r, i + RDIEA q, r, i + RDILA q, r, i)

Where:

For a Generation Resource:

RDIGA q, r, i  	=  (Min (0, RDIGE r ,y + RDIAS r, y))

RDIGE r, y  	= (-1) * Max (-$251, PRRTLMP p, r, y) * (PRBP r, y – BP r, y) * TLMP y

+ EOCAREA r, BP to PRBP, y * TLMP y

For an Energy Storage Resource:

RDIEA q, r, i 	=   (Min (0, RDIEE r, y + RDIAS r, y))

RDIEE r, y  	= (-1) * Max (-$251, PRRTLMP p, r, y) * (PRBP r, y – BP r, y) * TLMP y

+ EBOCAREA r, BP to PRBP, y * TLMP y

For a Load Resource:

RDILA q, r, i 	=   (Min (0, RDILE q, r, y + RDIAS q, r, y))

Where:

For a Controllable Load Resource (CLR) that is not an Aggregate Load Resource (ALR):

RDILE r, y  	= (-1) * Max (-$251, PRRTLMP p, r, y) * (BP r, y – PRBP r, y) * TLMP y

+  EBCAREA r, PRBP to BP, y * TLMP y

For a CLR that is an ALR:

RDILE r, y  	= (-1)* Max (-$251, PRLZLMP p, r, y) * (BP r, y – PRBP r, y) * TLMP y + EBCAREA r, PRBP to BP, y * TLMP y

For a Load Resource that is not a CLR:

RDILE r, y  	=  0

And:

For a Generation Resource, Energy Storage Resource (ESR), or Load Resource, the Ancillary Service component of the indifference payment is calculated as:

RDIAS r, y     =  RDIRUS r, y  + RDIRDS r, y + RDIRRS r, y + RDIECRS r, y + RDINSS r, y

Where:

RDIRUS r, y  = (-1) * PRRTMCPCRUS y  * (PRRTRUAWDS r, y –RTRUAWDS r, y) * TLMP y + ASseg (RUSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

RDIRDS r, y  = (-1) * PRRTMCPCRDS y * (PRRTRDAWDS r, y – RTRDAWDS r, y) * TLMP y + ASseg (RDSOAREAr, ASseg, DRsegAwd to PRsegAwd, y ) * TLMP y

RDIRRS r, y  = (-1) * PRRTMCPCRRS y * (PRRTRRAWDS r, y – RTRRAWDS r, y) * TLMP y

+ ASseg (RRPFSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

+ ASseg (RRUFSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

+ ASseg (RRFFSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

RDIECRS r, y  = (-1) * PRRTMCPCECRS y * (PRRTECRAWDS r, y – RTECRAWDS r, y) * TLMP y

+ ASseg (ECRSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

+ ASseg (ECRMSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

RDINSS r, y  = (-1) * PRRTMCPCNSS y * (PRRTNSAWDS r, y – RTNSAWDS r, y) * TLMP y

+ ASseg (NSSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

+ ASseg (NSMSOAREA r, ASseg, DRsegAwd to PRsegAwd, y) * TLMP y

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| RDIAMT q, i | $ | Reliability Deployment Indifference Total Amount per QSE—The total Reliability Deployment Indifference Payment to QSE q for the 15-minute Settlement Interval i. |
| RDIGA q, r, i | $ | Reliability Deployment Indifference Amount per QSE per Generation Resource—The Reliability Deployment Indifference Payment to QSE q for Generation Resource r for the 15-minute Settlement Interval i. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| RDIEA q, r, i | $ | Reliability Deployment Indifference Amount per QSE per Energy Storage Resource—The Reliability Deployment Indifference Payment to QSE q for Energy Storage Resource r for the 15-minute Settlement Interval i. |
| RDILA q, r, i | $ | Reliability Deployment Indifference Amount per QSE per Load Resource—The Reliability Deployment Indifference Payment to QSE q for Load Resource r for the 15-minute Settlement Interval i. |
| RDIAS r, y | $ | Total Ancillary Service Component of Reliability Indifference Amount per Resource per SCED Interval—The total Ancillary Service component of the Reliability Deployment Indifference Payment to Resource r for the SCED Interval y. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Generation Resource within the Combined Cycle Train |
| RDIGE r, y | $ | Energy Component of Reliability Indifference Amount per Generation Resource per SCED Interval—The energy component of the Reliability Deployment Indifference Payment to Generation Resource r for the SCED Interval y. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Generation Resource within the Combined Cycle Train. |
| RDIEE r, y | $ | Energy Component of Reliability Indifference Amount per Energy Storage Resource per SCED Interval—The energy component of the Reliability Deployment Indifference Payment to Energy Storage Resource r for the SCED Interval y. |
| RDILE r, y | $ | Energy Component of Reliability Indifference Amount per Load Resource per SCED Interval—The energy component of the Reliability Deployment Indifference Payment to Load Resource r for the SCED Interval y. |
| PRRTLMP p, r, y | $/MWh | SCED Pricing Run Step 2 Real-Time Locational Marginal Price per interval¾ The SCED Pricing Run Step 2 Real-Time LMP at the Settlement Point p of the Resource r for the SCED interval y. |
| PRLZLMP p, r, y | $/MWh | SCED Pricing Run Step 2 Real-Time Load Zone Locational Marginal Price per interval¾ The SCED Pricing Run Step 2 Real-Time Load Zone LMP at the Settlement Point p of the Resource r for the SCED interval y. |
| PRBP r, y | MW | SCED Pricing Run Base Point per Resource per interval ¾ The Base Point of Resource r, for the SCED interval y that is output from the SCED Pricing Run. |
| BP r, y | MW | SCED Dispatch Run Base Point per Resource per interval ¾ The binding Base Point of Resource r, for the SCED interval y that is output from the SCED Dispatch Run. |
| EOCAREA r, BP to PRBP, y | $ per hour | Area under Generation Resource Energy Offer Curve used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the Energy Offer Curve used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Base Point to the SCED Pricing Run Base Point, with respect to the zero price line, per Generation Resource r for the SCED Interval y. |
| EBOCAREA r, BP to PRBP, y | $ per hour | Area under Energy Storage Resource Energy Bid/Offer curve used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the Energy Bid/Offer Curve used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Base Point to the SCED Pricing Run Base Point, with respect to the zero price line, per Energy Storage Resource r for the SCED Interval y. |
| EBCAREA r, PRBP to BP, y | $ per hour | Area under Controllable Load Resource Energy Bid Curve used in Step 2 SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the Energy Bid Curve used in Step 2 of the SCED Pricing, evaluated from the SCED Pricing Run Base Point to the SCED Dispatch Run Base Point, with respect to the zero price line, per Controllable Load Resource r for the SCED Interval y. |
| RDIRUS r, y | $ | Regulation Up Ancillary Service Component of Reliability Indifference Amount per Resource per SCED Interval—The Regulation Up Ancillary Service component of the Reliability Deployment Indifference Payment to Resource r for the SCED Interval y. |
| RDIRDS r, y | $ | Regulation Down Ancillary Service Component of Reliability Indifference Amount per Resource per SCED Interval—The Regulation Down Ancillary Service component of the Reliability Deployment Indifference Payment to Resource r for the SCED Interval y. |
| RDIRRS r, y | $ | Responsive Reserve Ancillary Service Component of Reliability Indifference Amount per Resource per SCED Interval—The Responsive Reserve Ancillary Service component of the Reliability Deployment Indifference Payment to Resource r for the SCED Interval y. |
| RDIECRS r, y | $ | ERCOT Contingency Reserve Ancillary Service Component of Reliability Indifference Amount per Resource per SCED Interval—The ECRS Ancillary Service component of the Reliability Deployment Indifference Payment to Resource r for the SCED Interval y. |
| RDINSS r, y | $ | Non-Spin Service Component of Reliability Indifference Amount per Resource per SCED Interval—The Non-Spin Ancillary Service component of the Reliability Deployment Indifference Payment to Resource r for the SCED Interval y. |
| PRRTMCPCRUS y | $/MW | SCED Pricing Run Step 2 Real-Time Market Clearing Price for Capacity for Reg-Up per SCED interval—The SCED Pricing Run Step 2 Real-Time MCPC for Reg-Up for the SCED interval y. |
| PRRTMCPCRDS y | $/MW | SCED Pricing Run Step 2 Real-Time Market Clearing Price for Capacity for Reg-Down per SCED interval—The SCED Pricing Run Step 2 Real-Time MCPC for Reg-Down for the SCED interval y. |
| PRRTMCPCRRS y | $/MW | SCED Pricing Run Step 2 Real-Time Market Clearing Price for Capacity for Responsive Reserve per SCED interval—The SCED Pricing Run Step 2 Real-Time MCPC for RRS for the SCED interval y. |
| PRRTMCPCECRS y | $/MW | SCED Pricing Run Step 2 Real-Time Market Clearing Price for Capacity for ECRS per SCED interval—The SCED Pricing Run Step 2 Real-Time MCPC for ECRS for the SCED interval y. |
| PRRTMCPCNSS y | $/MW | SCED Pricing Run Step 2 Real-Time Market Clearing Price for Capacity for Non-Spin per SCED interval—The SCED Pricing Run Step 2 Real-Time MCPC for Non-Spin for the SCED interval y. |
| PRRTRUAWDS r, y | MW | SCED Pricing Run Real-Time Reg-Up Award per Resource per SCED interval—The Reg-Up amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Pricing Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| PRRTRDAWDS r, y | MW | SCED Pricing Run Real-Time Reg-Down Award per Resource per SCED interval—The Reg-Down amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Pricing Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| PRRTRRAWDS r, y | MW | SCED Pricing Run Real-Time RRS Award per Resource per SCED interval—The RRS amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Pricing Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| PRRTECRAWDS r, y | MW | SCED Pricing Run Real-Time ECRS Award per Resource per SCED interval—The ECRS amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Pricing Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| PRRTNSAWDS r, y | MW | SCED Pricing Run Real-Time Non-Spin Award per Resource per SCED interval—The Non-Spin amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Pricing Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTRUAWDS r, y | MW | SCED Dispatch Run Real-Time Reg-Up Award per Resource per SCED interval—The binding Reg-Up amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Dispatch Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTRDAWDS r, y | MW | SCED Dispatch Run Real-Time Reg-Down Award per Resource per QSE per SCED interval—The binding Reg-Down amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Dispatch Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTRRAWDS r, y | MW | SCED Dispatch Run Real-Time RRS Award per Resource per QSE per SCED interval—The binding RRS amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Dispatch Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTECRAWDS r, y | MW | SCED Dispatch Run Real-Time ECRS Award per Resource per QSE per SCED interval—The binding ECRS amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Dispatch Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RTNSAWDS r, y | MW | SCED Dispatch Run Real-Time Non-Spin Award per Resource per QSE per SCED interval—The binding Non-Spin amount awarded to Resource r in Real-Time for the SCED interval y from the SCED Dispatch Run Step 2. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| RUSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource Reg-Up Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the Reg-Up Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Reg-Up award to the SCED Pricing Run Reg-Up award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| RDSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource Reg-Down Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the Reg-Down Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Reg-Down award to the SCED Pricing Run Reg-Down award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| RRPFSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource RRS-PF Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the RRS-PF Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run RRS-PF award to the SCED Pricing Run RRS-PF award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| RRUFSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource RRS-UF Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the RRS-UF Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run RRS-UF award to the SCED Pricing Run RRS-UF award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| RRFFSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource RRS-FF Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the RRS-FF Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run RRS-FF award to the SCED Pricing Run RRS-FF award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| ECRSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource ECRS-SCED dispatchable Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the ECRS-SCED Dispatchable Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run ECRS-SCED Dispatchable award to the SCED Pricing Run ECRS-SCED Dispatchable award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| ECRMSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource ECRS-Non SCED Dispatchable Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the ECRS-Non SCED Dispatchable Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run ECRS-Non SCED Dispatchable award to the SCED Pricing Run ECRS-Non SCED Dispatchable award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| NSSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource Non-Spin-SCED dispatchable Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the Non-Spin-SCED Dispatchable Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Non-Spin-SCED Dispatchable award to the SCED Pricing Run Non-Spin-SCED Dispatchable award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| NSMSOAREA r, ASseg, DRsegAwd to PRsegAwd, y | $ per hour | Area under Resource Non-Spin-Non SCED Dispatchable Ancillary Service Offer segment used in Step 2 of SCED Pricing Run per Resource per interval¾ The area calculated as the integral (net area) of the Non-Spin-Non SCED Dispatchable Ancillary Service Offer segment used in Step 2 of the SCED Pricing Run, evaluated from the SCED Dispatch Run Non-Spin-Non SCED Dispatchable award to the SCED Pricing Run Non-Spin-Non SCED Dispatchable award, with respect to the zero price line, per Resource r for the SCED Interval y. |
| TLMP y | Second | Duration of SCED interval per interval¾ The duration of the portion of the SCED interval y within the Settlement Interval. |
| y | None | A SCED interval in the 15-minute Settlement Interval. The summation is over the total number of SCED runs that cover the 15-minute Settlement Interval. |
| ASseg | None | The Ancillary Service Offer segment for a particular Ancillary Service type or sub-type, including the proxy Ancillary Service Offer. |
| q | None | A QSE. |
| p | None | A Settlement Point. |
| r | None | A Generation Resource, Energy Storage Resource, or Load Resource. |
| i | None | A 15-minute Settlement Interval. |

### 6.9.2	Reliability Deployment Indifference Charge

(1)	ERCOT shall allocate the total Reliability Deployment Indifference Payment to the QSEs representing Load based on Load Ratio Share (LRS).  The charge to each QSE for a given 15-minute Settlement Interval is calculated as follows:

LARDIAMT q, i	=	(-1) * RDIAMTTOT i * LRS q, i

Where:

RDIAMTTOT	i	=	 RDIAMT q,i

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| LARDIAMT q,i | $ | Load-Allocated Reliability Deployment Indifference Amount per QSE—The QSE q’s share of the total charge for the Real-Time Reliability Deployment Indifference amount for the 15-minute Settlement Interval i. |
| RDIAMTTOT i | $ | Reliability Deployment Indifference Total Amount—The total payment to all QSEs for the Reliability Deployment Indifference Payments for the 15-minute Settlement Interval i. |
| RDIAMT q,i | $ | Reliability Deployment Indifference Total Amount per QSE—The total payment to QSE q for the Reliability Deployment Indifference Payments for the 15-minute Settlement Interval i. |
| LRS q,i | none | The LRS calculated for QSE q for the 15-minute Settlement Interval. See Section 6.6.2.2, QSE Load Ratio Share for a 15-Minute Settlement Interval. |
| q | none | A QSE. |
| i | none | A 15-minute Settlement Interval. |

### 9.5.3	Real-Time Market Settlement Charge Types

(1)	ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for each RTM Settlement charge and payment.  The RTM Settlement “Charge Types” are:

(a)	Section 5.7.1, RUC Make-Whole Payment;

(b)	Section 5.7.2, RUC Clawback Charge;

(c)	Section 5.7.3, Payment When ERCOT Decommits a QSE-Committed Resource;

(d)	Section 5.7.4.1, RUC Capacity-Short Charge;

(e)	Section 5.7.4.2, RUC Make-Whole Uplift Charge;

(f)	Section 5.7.5, RUC Clawback Payment;

(g)	Section 5.7.6, RUC Decommitment Charge;

(h)	Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node;

(i)	Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone;

(j)	Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charge at a Hub;

(k)	Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import;

(l)	Section 6.6.3.5, Real-Time Payment for a Block Load Transfer Point;

(m)	Section 6.6.3.6, Real-Time High Dispatch Limit Override Energy Payment;

(n)	Section 6.6.3.7, Real-Time High Dispatch Limit Override Energy Charge;

(o)	Section 6.6.3.8, Real-Time Payment or Charge for Energy from a Settlement Only Distribution Generator (SODG) or a Settlement Only Transmission Generator (SOTG);

(p)	Section 6.6.4, Real-Time Congestion Payment or Charge for Self-Schedules;

(q)	Section 6.6.5.2, Set Point Deviation Charge for Over Generation;

(r)	Section 6.6.5.2.1, Set Point Deviation Charge for Under Generation;

(s)	Section 6.6.5.3, Controllable Load Resource Set Point Deviation Charge for Over Consumption;

(t)	Section 6.6.5.3.1, Controllable Load Resource Set Point Deviation Charge for Under Consumption;

(u)	Section 6.6.5.4, IRR Generation Resource Set Point Deviation Charge;

(v)	Section 6.6.5.7, Set Point Deviation Payment;

(w)	Section 6.6.6.1, RMR Standby Payment;

(x)	Section 6.6.6.2, RMR Payment for Energy;

(y)	Section 6.6.6.3, RMR Adjustment Charge;

(z)	Section 6.6.6.4, RMR Charge for Unexcused Misconduct;

(aa)	Section 6.6.6.5, RMR Service Charge;

(bb)	Section 6.6.6.6, Method for Reconciling RMR Actual Eligible Costs, RMR and MRA Contributed Capital Expenditures, and Miscellaneous RMR Incurred Expenses;

(cc)	Paragraph (3) of Section 6.6.7.1, Voltage Support Service Payments;

(dd)	Paragraph (5) of Section 6.6.7.1;

(ee)	Section 6.6.7.2, Voltage Support Charge;

(ff)	Section 6.6.8.1, Black Start Hourly Standby Fee Payment;

(gg)	Section 6.6.8.2, Black Start Capacity Charge;

(hh)	Section 6.6.9.1, Payment for Emergency Operations Settlement;

(ii)	Section 6.6.9.2, Charge for Emergency Operations Settlement;

(jj)	Section 6.6.10, Real-Time Revenue Neutrality Allocation;

(kk)	Section 6.6.11.1, Emergency Response Service Capacity Payments;

(ll)	Section 6.6.11.2, Emergency Response Service Capacity Charge;

(mm)	Section 6.6.14.2, Firm Fuel Supply Service Hourly Standby Fee Payment and Fuel Replacement Cost Recovery;

(nn)	Section 6.6.14.3, Firm Fuel Supply Service Capacity Charge;

(oo)	Section 6.7.1, Real-Time Settlement for Updated Day-Ahead Market Ancillary Service Obligations;

(pp) 	Section 6.7.2.2, Regulation Up Service Payments and Charges;

(qq) 	Section 6.7.2.3, Regulation Down Service Payments and Charges;

(rr) 	Section 6.7.2.4, Responsive Reserve Payments and Charges;

(ss) 	Section 6.7.2.5	, Non-Spinning Reserve Service Payments and Charges;

(tt) 	Section 6.7.2.6	, ERCOT Contingency Reserve Service Payments and Charges;

(uu) 	Section 6.7.2.7	, Real-Time Derated Ancillary Service Capability Payment;

(vv) 	Section 6.7.2.8	, Real-Time Derated Ancillary Service Capability Charge;

(ww)	Section 6.7.3, Real-Time Ancillary Service Revenue Neutrality Allocation;

(xx) 	Section 6.9.1, Reliability Deployment Indifference Payment;

(yy) 	Section 6.9.2, Reliability Deployment Indifference Charge;

(zz)	Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time; and

(aaa)	Section 9.16.1, ERCOT System Administration Fee.

[NPRR841, NPRR885, NPRR963, NPRR995, NPRR1216, and NPRR1229: Replace applicable portions of paragraph (1) above with the following upon system implementation:] (1) ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for each RTM Settlement charge and payment. The RTM Settlement “Charge Types” are: (a) Section 5.7.1, RUC Make-Whole Payment; (b) Section 5.7.2, RUC Clawback Charge; (c) Section 5.7.3, Payment When ERCOT Decommits a QSE-Committed Resource; (d) Section 5.7.4.1, RUC Capacity-Short Charge; (e) Section 5.7.4.2, RUC Make-Whole Uplift Charge; (f) Section 5.7.5, RUC Clawback Payment; (g) Section 5.7.6, RUC Decommitment Charge; (h) Section 6.6.3.1, Real-Time Energy Imbalance Payment or Charge at a Resource Node; (i) Section 6.6.3.2, Real-Time Energy Imbalance Payment or Charge at a Load Zone; (j) Section 6.6.3.3, Real-Time Energy Imbalance Payment or Charge at a Hub; (k) Section 6.6.3.4, Real-Time Energy Payment for DC Tie Import; (l) Section 6.6.3.5, Real-Time Payment for a Block Load Transfer Point; (m) Section 6.6.3.6, Real-Time High Dispatch Limit Override Energy Payment; (n) Section 6.6.3.7, Real-Time High Dispatch Limit Override Energy Charge; (o) Section 6.6.3.8, Real-Time Payment or Charge for Energy from a Settlement Only Distribution Generator (SODG), Settlement Only Transmission Generator (SOTG), Settlement Only Distribution Energy Storage System (SODESS), or Settlement Only Transmission Energy Storage System (SOTESS); (p) Section 6.6.3.9, Real-Time Constraint Management Plan Cost Recovery Payment; (q) Section 6.6.3.10, Real-Time Constraint Management Plan Cost Recovery Charge; (r) Section 6.6.4, Real-Time Congestion Payment or Charge for Self-Schedules; (s) Section 6.6.5.2, Set Point Deviation Charge for Over Generation; (t) Section 6.6.5.2.1, Set Point Deviation Charge for Under Generation; (u) Section 6.6.5.3, Controllable Load Resource Set Point Deviation Charge for Over Consumption; (v) Section 6.6.5.3.1, Controllable Load Resource Set Point Deviation Charge for Under Consumption; (w) Section 6.6.5.4, IRR Generation Resource Set Point Deviation Charge; (x) Section 6.6.5.7, Set Point Deviation Payment; (y) Section 6.6.5.5, Energy Storage Resource Set Point Deviation Charge for Over Performance; (z) Section 6.6.5.5.1, Energy Storage Resource Set Point Deviation Charge for Under Performance; (aa) Section 6.6.6.1, RMR Standby Payment; (bb) Section 6.6.6.2, RMR Payment for Energy; (cc) Section 6.6.6.3, RMR Adjustment Charge; (dd) Section 6.6.6.4, RMR Charge for Unexcused Misconduct; (ee) Section 6.6.6.5, RMR Service Charge; (ff) Section 6.6.6.6, Method for Reconciling RMR Actual Eligible Costs, RMR and MRA Contributed Capital Expenditures, and Miscellaneous RMR Incurred Expenses; (gg) Section 6.6.6.7, MRA Standby Payment; (hh) Section 6.6.6.8, MRA Contributed Capital Expenditures Payment; (ii) Section 6.6.6.9, MRA Payment for Deployment Event; (jj) Section 6.6.6.10, MRA Variable Payment for Deployment; (kk) Section 6.6.6.11, MRA Charge for Unexcused Misconduct; (ll) Section 6.6.6.12, MRA Service Charge; (mm) Paragraph (3) of Section 6.6.7.1, Voltage Support Service Payments; (nn) Paragraph (5) of Section 6.6.7.1; (oo) Section 6.6.7.2, Voltage Support Charge; (pp) Section 6.6.8.1, Black Start Hourly Standby Fee Payment; (qq) Section 6.6.8.2, Black Start Capacity Charge; (rr) Section 6.6.9.1, Payment for Emergency Operations Settlement; (ss) Section 6.6.9.2, Charge for Emergency Operations Settlement; (tt) Section 6.6.10, Real-Time Revenue Neutrality Allocation; (uu) Section 6.6.11.1, Emergency Response Service Capacity Payments; (vv) Section 6.6.11.2, Emergency Response Service Capacity Charge; (ww) Section 6.6.14.2, Firm Fuel Supply Service Hourly Standby Fee Payment and Fuel Replacement Cost Recovery; (xx) Section 6.6.14.3, Firm Fuel Supply Service Capacity Charge; (yy) Section 6.7.1, Real-Time Settlement for Updated Day-Ahead Market Ancillary Service Obligations; (zz) Section 6.7.2.2, Regulation Up Service Payments and Charges; (aaa) Section 6.7.2.3, Regulation Down Service Payments and Charges; (bbb) Section 6.7.2.4, Responsive Reserve Payments and Charges; (ccc) Section 6.7.2.5 , Non-Spinning Reserve Service Payments and Charges; (ddd) Section 6.7.2.6 , ERCOT Contingency Reserve Service Payments and Charges; (eee) Section 6.7.2.7 , Real-Time Derated Ancillary Service Capability Payment; (fff) Section 6.7.2.8 , Real-Time Derated Ancillary Service Capability Charge; (ggg) Section 6.7.3, Real-Time Ancillary Service Revenue Neutrality Allocation; (hhh) Section 6.8.2, Recovery of Operating Losses During an LCAP or ECAP Effective Period; (iii) Section 6.8.3, Charges for Operating Losses During an LCAP or ECAP Effective Period; (jjj) Section 6.9.1, Reliability Deployment Indifference Payment; (kkk) Section 6.9.2, Reliability Deployment Indifference Charge; (lll) Section 7.9.2.1, Payments and Charges for PTP Obligations Settled in Real-Time; and (mmm) Section 9.16.1, ERCOT System Administration Fee.

(2)	In the event that ERCOT is unable to execute the Day-Ahead Market (DAM), ERCOT shall provide, on each RTM Settlement Statement, the dollar amount for the following RTM Congestion Revenue Right (CRR) Settlement charges and payments:

(a)	Section 7.9.2.4, Payments for FGRs in Real-Time; and

(b)	Section 7.9.2.5, Payments and Charges for PTP Obligations with Refund in Real-Time.