| NPRR Number | 1326 | NPRR Title | Add Energy Storage Resource (ESR) State of Charge (SOC) Information to the Ancillary Services Capacity Monitor |
| --- | --- | --- | --- |
| Date of Decision |  | June 10, 2026 |  |
| Action |  | Tabled |  |
| Timeline |  | Normal |  |
| Estimated Impacts |  | Cost/Budgetary: To be determined Project Duration: To be determined |  |
| Proposed Effective Date |  | To be determined |  |
| Priority and Rank Assigned |  | To be determined |  |
| Nodal Protocol Sections Requiring Revision |  | 6.5.7.5, Ancillary Services Capacity Monitor |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) adds State of Charge (SOC) information for Energy Storage Resources (ESRs) in the ERCOT region to the Ancillary Services Capacity Monitor. |  |
| Reason for Revision |  | Strategic Plan Objective 1 – Be an industry leader for grid reliability and resilience Strategic Plan Objective 2 - Enhance the ERCOT region’s economic competitiveness with respect to trends in wholesale power rates and retail electricity prices to consumers Strategic Plan Objective 3 - Advance ERCOT, Inc. as an independent leading industry expert and an employer of choice by fostering innovation, investing in our people, and emphasizing the importance of our mission General system and/or process improvement(s) Regulatory requirements ERCOT Board/PUCT Directive (please select ONLY ONE – if more than one apply, please select the ONE that is most relevant) |  |
| Justification of Reason for Revision and Market Impacts |  | ESRs are a growing resource in the ERCOT market and provide vital capacity to ensure supply can meet Demand. The fundamentals of the ERCOT market will increasingly be influenced by ESR production or consumption. Therefore, the aggregate amount of SOC for the ESR fleet in ERCOT is essential for Market Participants to understand in order to assess the conditions of the system and respond accordingly. Also, ERCOT has begun adopting SOC as a metric used to deploy Ancillary Services such as Non-Spinning Reserve (Non-Spin). Therefore, increased transparency of aggregate ESR SOC information in Real-Time is important to ensure efficient market response. |  |
| PRS Decision |  | On 4/15/26, PRS voted unanimously to table NPRR1326. All Market Segments participated in the vote. On 5/6/26, PRS voted unanimously to recommend approval of NPRR1326 as submitted. All Market Segments participated in the vote. On 6/10/26, PRS voted unanimously to table NPRR1326. All Market Segments participated in the vote. |  |
| Summary of PRS Discussion |  | On 4/15/26, the sponsor presented NPRR1326. Participants requested additional time to review NPRR1326 and potentially submit comments. On 5/6/26, the sponsor noted that no comments had been filed to NPRR1326. On 6/10/26, participants noted the ERCOT-proposed alternative schedule for the development of an Impact Analysis for NPRR1326. |  |

| Sponsor |  |
| --- | --- |
| Name | Bill Barnes |
| E-mail Address | bill.barnes@nrg.com |
| Company | Reliant Energy Retail Solutions LLC |
| Phone Number | 512-691-6137 |
| Cell Number | 315-885-5925 |
| Market Segment | Independent Retail Electric Provider (IREP) |

| Opinions |  |
| --- | --- |
| Credit Review | To be determined |
| Independent Market Monitor Opinion | To be determined |
| ERCOT Opinion | To be determined |
| ERCOT Market Impact Statement | To be determined |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Brittney Albracht |
| E-Mail Address | Brittney.Albracht@ercot.com |
| Phone Number | 512-225-7027 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 060526 | Proposed an alternative schedule for the development of an Impact Analysis for NPRR1326 |

Market Rules Notes

Please note that the following NPRR(s) also propose revisions to the following section(s):

- NPRR1309, Board Priority - Dispatchable Reliability Reserve Service Ancillary Service

  - Section 6.5.7.5

- NPRR1310, Dispatchable Reliability Reserve Service Plus Energy Storage Resource Participation and Release Factor

  - Section 6.5.7.5

- NPRR1340, Dispatchable Reliability Reserve Service Ancillary Service with Energy Storage Resource Participation

  - Section 6.5.7.5

Proposed Protocol Language Revision

6.5.7.5	Ancillary Services Capacity Monitor

(1)	Every ten seconds, ERCOT shall calculate the following and provide Real-Time summaries to ERCOT Operators and all Market Participants using ICCP and postings on the ERCOT website showing the Real-Time total system amount of:

(a)	RRS capability from:

(i)	Generation Resources and ESRs in the form of PFR that can be sustained for the SCED duration requirements of PFR;

(ii)	Load Resources, excluding CLRs, capable of responding via under-frequency relay;

(iii)	CLRs in the form of PFR;

(iv)	Resources, other than ESRs, capable of Fast Frequency Response (FFR); and

(v)	ESRs, in the form of FFR, that can be sustained for the SCED duration requirements of FFR;

(b)	Ancillary Service Resource awards for RRS to:

(i)	Generation Resources and ESRs in the form of PFR;

(ii)	Load Resources, excluding CLRs, capable of responding by under-frequency relay;

(iii)	CLRs in the form of PFR; and

(iv)	Resources providing FFR;

(c)	ECRS capability from:

(i)	Generation Resources;

(ii)	Load Resources excluding CLRs;

(iii)	CLRs;

(iv)	Quick Start Generation Resources (QSGRs); and

(v) 	ESRs that can be sustained for the SCED duration requirements of ECRS.

(d)	Ancillary Service Resource awards for ECRS to:

(i)	Generation Resources;

(ii)	Load Resources excluding CLRs;

(iii)	CLRs;

(iv)	QSGRs; and

(v) 	ESRs.

(e)	ECRS manually deployed by Resources with a Resource Status of ONSC;

(f)	Non-Spin available from:

(i)	On-Line Generation Resources with Energy Offer Curves;

(ii)	Undeployed Load Resources;

(iii)	Off-Line Generation Resources and On-Line Generation Resources with power augmentation;

(iv)	Resources with Output Schedules; and

(v) 	ESRs that can be sustained for the SCED duration requirements of Non-Spin.

(g)	Ancillary Service Resource awards for Non-Spin to:

(i)	On-Line Generation Resources with Energy Offer Curves;

(ii)	On-Line Generation Resources with Output Schedules;

(iii)	Load Resources;

(iv)	Off-Line Generation Resources excluding Quick Start Generation Resources (QSGRs), including Non-Spin awards on power augmentation capacity that is not active on On-Line Generation Resources;

(v)	QSGRs; and

(vi)	ESRs.

(h)	Reg-Up and Reg-Down capability (for ESRs, the SCED duration requirements of Reg-Up and Reg-Down are considered);

(i)	Undeployed Reg-Up and Reg-Down;

(j)	Ancillary Service Resource awards for Reg-Up and Reg-Down;

(k)	Deployed Reg-Up and Reg-Down;

(l)	Available capacity:

(i)	With Energy Offer Curves in the ERCOT System that can be used to increase Generation Resource Base Points in SCED;

(ii)	With Energy Offer Curves in the ERCOT System that can be used to decrease Generation Resource Base Points in SCED;

(iii)	Without Energy Offer Curves in the ERCOT System that can be used to increase Generation Resource Base Points in SCED;

(iv)	Without Energy Offer Curves in the ERCOT System that can be used to decrease Generation Resource Base Points in SCED;

(v)	With RTM Energy Bid curves from available CLRs in the ERCOT System that can be used to decrease Base Points (energy consumption) in SCED;

[NPRR1188: Replace paragraph (v) above with the following upon system implementation:] (v) With Energy Bid Curves from available CLRs in the ERCOT System that can be used to decrease Base Points (energy consumption) in SCED;

(vi)	With RTM Energy Bid curves from available CLRs in the ERCOT System that can be used to increase Base Points (energy consumption) in SCED;

[NPRR1188: Replace paragraph (vi) above with the following upon system implementation:] (vi) With Energy Bid Curves from available CLRs in the ERCOT System that can be used to increase Base Points (energy consumption) in SCED;

(vii)	From Resources participating in SCED plus the Reg-Up, RRS, and ECRS from Load Resources and the Net Power Consumption minus the Low Power Consumption from Load Resources with a validated Real-Time RRS and ECRS awards;

(viii)	With Energy Bid/Offer Curves for ESRs in the ERCOT System that can be used to increase ESR Base Points in SCED while respecting SCED duration requirements for ESR Base Points in SCED;

(ix)	With Energy Bid/Offer Curves for ESRs in the ERCOT System that can be used to decrease ESR Base Points in SCED while respecting SCED duration requirements for ESR Base Points in SCED;

(x)	Without Energy Bid/Offer Curves for ESRs in the ERCOT System that can be used to increase ESR Base Points in SCED while respecting SCED duration requirements for ESR Base Points in SCED;

(xi)	Without Energy Bid/Offer Curves for ESRs in the ERCOT System that can be used to decrease ESR Base Points in SCED while respecting SCED duration requirements for ESR Base Points in SCED;

(xii)	From Resources included in item (vii) above plus reserves from Resources that could be made available to SCED in 30 minutes;

(xiii) 	In the ERCOT System that can be used to increase Generation Resource Base Points in the next five minutes in SCED; and

(xiv)	In the ERCOT System that can be used to decrease Generation Resource Base Points in the next five minutes in SCED;

(xv)	The total capability of Resources available to provide the following combinations of Ancillary Services, based on the Resource telemetry from the QSE and capped by the limits of the Resource:

(A)	Capacity to provide Reg-Up, RRS, or both, irrespective of whether it is capable of providing ECRS or Non-Spin;

(B)	Capacity to provide Reg-Up, RRS, ECRS, or any combination, irrespective of whether it is capable of providing Non-Spin; and

(C)	Capacity to provide Reg-Up, RRS, ECRS, or Non-Spin, in any combination;

(m)	SOC;

(n)	Maximum SOC (MaxSOC);

(o)	Minimum SOC (MinSOC);

(p)	Aggregate telemetered HSL capacity for Resources with a telemetered Resource Status of EMR;

(q)	Aggregate telemetered HSL capacity for Resources with a telemetered Resource Status of OUT;

(r)	Aggregate net telemetered consumption for Resources with a telemetered Resource Status of OUTL; and

(s)	The ERCOT-wide PRC calculated as follows:

PRC1 =			Min(Max((RDF*FRCHL – FRCO)i , 0.0) , 0.2*RDF*FRCHLi),

where the included On-Line Generation Resources do not include WGRs, nuclear Generation

Resources, or Generation Resources with an output less than or equal to 95% of telemetered LSL or

with a telemetered status of ONTEST, ONHOLD, STARTUP, or SHUTDOWN.





WGRs

online

All

WGR

online

i

PRC2 =			Min(Max((RDFW*HSL – Actual Net Telemetered Output)i , 0.0) , 					0.2*RDFW*HSLi),

where the included On-Line WGRs only include WGRs that are Primary Frequency Response-capable.

PRC3 =	((Synchronous condenser output)i as qualified by item (8) of Operating Guide Section 2.3.1.2, Additional Operational Details for Responsive Reserve and ERCOT Contingency Reserve Service Providers))





resources

load

online

All

resource

load

online

i

PRC4 =	(Min(Max((Actual Net Telemetered Consumption – LPC), 0.0), ECRS and RRS Ancillary Service Resource award * 1.5) from all Load Resources controlled by high-set under-frequency relays with an ECRS and/or RRS Ancillary Service Resource award)i





resources

load

online

All

resource

load

online

i

PRC5 =	Min(Max((LRDF_1*Actual Net Telemetered Consumption – LPC)i, 0.0), (0.2 * LRDF_1 * Actual Net Telemetered Consumption)) from all CLRs active in SCED with an Ancillary Service Resource award

[NPRR1244: Replace the formula “PRC5” above with the following upon system implementation:]   resources load online All resource load online i PRC5 = Min(Max((LRDF_1*Actual Net Telemetered Consumption – LPC)i, 0.0), (0.2 * LRDF_1 * Actual Net Telemetered Consumption)) from all CLRs active in SCED and qualified for Regulation Service and/or RRS with an Ancillary Service Resource award





resources

load

online

All

resource

load

online

i

PRC6 =	Min(Max((LRDF_2 * Actual Net Telemetered Consumption – LPC)i, 0.0), (0.2 * LRDF_2 * Actual Net Telemetered Consumption)) from all CLRs active in SCED without an Ancillary Service Resource award

[NPRR1244: Replace the formula “PRC6” above with the following upon system implementation:]   resources load online All resource load online i PRC6 = Min(Max((LRDF_2 * Actual Net Telemetered Consumption – LPC)i, 0.0), (0.2 * LRDF_2 * Actual Net Telemetered Consumption)) from all CLRs active in SCED and qualified for Regulation Service and/or RRS without an Ancillary Service Resource award





resources

FFR

online

All

resource

FFR

online

i

PRC7 =	(Capacity from Resources capable of providing FFR)i





ESR

online

All

ESR

online

i

PRC8 =	Min(X% of MDRR, HSL-Net MW, the capacity that can be sustained for 45 minutes per the State of Charge

Excludes ESR capacity used to provide FFR.

[NPRR1029: Insert the formula “PRC9” below upon system implementation:]   DC-Coupled Resources online All ESR online i PRC9 = Min(X% of MDRR, HSL-Net MW, the sum of the MW headroom available from the intermittent renewable generation component and the MW capacity that can be sustained for 45 minutes per the ESS State of Charge) Excludes DC-Coupled Resource capacity used to provide FFR.

PRC =	PRC1 + PRC2 + PRC3+ PRC4 + PRC5 + PRC6 + PRC7 + PRC8

[NPRR1029: Replace the formula “PRC” above with the following upon system implementation:] PRC = PRC1 + PRC2 + PRC3+ PRC4 + PRC5 + PRC6 + PRC7 + PRC8 + PRC9

The above variables are defined as follows:

| Variable | Unit | Description |
| --- | --- | --- |
| PRC1 | MW | Generation On-Line greater than 0 MW |
| PRC2 | MW | WGRs On-Line greater than 0 MW |
| PRC3 | MW | Synchronous condenser output |
| PRC4 | MW | Capacity from Load Resources with an ECRS Ancillary Service Resource award |
| PRC5 | MW | Capacity from CLRs active in SCED with an Ancillary Service Resource award [NPRR1244: Replace the description above with the following upon system implementation:] Capacity from CLRs active in SCED and qualified for Regulation Service and/or RRS with an Ancillary Service Resource award. |
| PRC6 | MW | Capacity from CLRs active in SCED without an Ancillary Service Resource award [NPRR1244: Replace the description above with the following upon system implementation:] Capacity from CLRs active in SCED and qualified for Regulation Service and/or RRS without an Ancillary Service Resource award. |
| PRC7 | MW | Capacity from Resources capable of providing FFR |
| PRC8 | MW | ESR capacity capable of providing Primary Frequency Response |
| [NPRR1029: Insert the variable “PRC9” below upon system implementation:] PRC9; MW; Capacity from DC-Coupled Resources capable of providing Primary Frequency Response.. |
| PRC | MW | Physical Responsive Capability |
| X | Percentage | Percent threshold based on the Governor droop setting of ESRs |
| RDF |  | The currently approved Reserve Discount Factor |
| RDFW |  | The currently approved Reserve Discount Factor for WGRs |
| LRDF_1 |  | The currently approved Load Resource Reserve Discount Factor for CLRs awarded an Ancillary Service Resource award |
| LRDF_2 |  | The currently approved Load Resource Reserve Discount Factor for CLRs not awarded an Ancillary Service Resource award |
| FRCHL | MW | Telemetered High limit of the FRC for the Resource |
| FRCO | MW | Telemetered output of FRC portion of the Resource |

(2)	The Load Resource Reserve Discount Factors (RDFs) for CLRs (LRDF_1 and LRDF_2) shall be subject to review and approval by TAC.

(3) 	The RDFs used in the PRC calculation shall be posted to the ERCOT website no later than three Business Days after approval.

(4)	ERCOT shall display on the ERCOT website and update every ten seconds a rolling view of the ERCOT-wide PRC, as defined in paragraph (1)(p) above, for the current Operating Day.