| NPRR Number | 1325 | NPRR Title | Related to PGRR145, Batch Zero Process for Large Load Interconnections |
| --- | --- | --- | --- |
| Date of Decision |  | June 18, 2026 |  |
| Action |  | Approved |  |
| Timeline |  | Urgent |  |
| Estimated Impacts |  | Cost/Budgetary: None Project Duration: No project required |  |
| Effective Date |  | Upon implementation of Planning Guide Revision Request (PGRR) 145, Batch Zero Process for Large Load Interconnections |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Nodal Protocol Sections Requiring Revision |  | 2.1, Definitions 2.2, Acronyms and Abbreviations 3.2.5, Publication of Resource and Load Information 3.9.1, Current Operating Plan 3.10.7.3.1, Withdrawal-Limited Private Use Networks (new) 3.11.4.3, Categorization of Proposed Transmission Projects 4.4.9.4, Mitigated Offer Cap and Mitigated Offer Floor 4.4.9.4.4, Adjusted Bid Caps (new) 6.5.7.3, Security Constrained Economic Dispatch 6.5.7.11, Provisional Controllable Load Resource (PCLR) Ramp Rate Requirements (new) 16.5.5, Provisional Controllable Load Resources (PCLRs) (new) 23W, Declaration of Intent and Commitment to Register as a Provisional Controllable Load Resource (new) 23X, Withdrawal-Limited Private Use Network Designation (new) |  |
| Related Documents Requiring Revision/Related Revision Requests |  | PGRR145 |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) defines essential terminology related to the transitional Large Load interconnection process, the Batch Zero Process, and modifies the Regional Planning Group procedures to allow for a new category of projects that result from the Batch Zero Process. |  |
| Reason for Revision |  | Strategic Plan Objective 1 – Be an industry leader for grid reliability and resilience Strategic Plan Objective 2 - Enhance the ERCOT region’s economic competitiveness with respect to trends in wholesale power rates and retail electricity prices to consumers Strategic Plan Objective 3 - Advance ERCOT, Inc. as an independent leading industry expert and an employer of choice by fostering innovation, investing in our people, and emphasizing the importance of our mission General system and/or process improvement(s) Regulatory requirements ERCOT Board/PUCT Directive (please select ONLY ONE – if more than one apply, please select the ONE that is most relevant) |  |
| Justification of Reason for Revision and Market Impacts |  | ERCOT is implementing a new interconnection process for Large Loads, which will enable the evaluation of one or more Large Load requests on a system-wide basis. As part of this transition, ERCOT is conducting a one-time transitional batch study, Batch Zero, for certain qualifying Large Load interconnection requests. The new definitions are needed to accommodate the transitional interconnection process. In addition, the definition of a Load Commissioning Plan (LCP) is being relocated from the Planning Guide to the Protocols. Finally, ERCOT is revising Section 3.11.4.3 to allow ERCOT to submit to the Regional Planning Group (RPG) a set of Transmission Facilities resulting from the batch study process. |  |
| PRS Decision |  | On 4/15/26, PRS voted unanimously to table NPRR1325. All Market Segments participated in the vote. On 5/6/26, PRS voted unanimously to grant NPRR1325 Urgent status; to recommend approval of NPRR1325 as amended by the 5/2/26 ERCOT comments; and to forward to TAC NPRR1325 and the 3/4/26 Impact Analysis. All Market Segments participated in the vote. |  |
| Summary of PRS Discussion |  | On 4/15/26, ERCOT Staff provided an overview of the numerous Batch Study workshops, upcoming subcommittee meeting schedule, and an outline for additional ERCOT comments to be filed relating to the concepts of Provisional Controllable Load Resources (PCLRs) and “bring your own generation.” On 5/6/26, ERCOT Staff presented on the series of batch study process workshops and the evolution of NPRR1325 and PGRR145 through multiple rounds of ERCOT comments; as well as budgetary impacts associated with the various NPRR1325 and PGRR145 options. |  |
| TAC Decision |  | On 5/13/26, TAC voted unanimously to table NPRR1325. All Market Segments participated in the vote. On 5/19/26, TAC voted unanimously to recommend approval of NPRR1325 as recommended by PRS in the 5/6/26 PRS Report as amended by the 5/15/26 ERCOT comments. All Market Segments participated in the vote. |  |
| Summary of TAC Discussion |  | On 5/13/26, participants reviewed the items below and noted the unanimous PRS vote in support of NPRR1325, but expressed a desire to table NPRR1325 awaiting further formal comments on the related PGRR145. On 5/19/26, there was no additional discussion beyond TAC review of the items below. |  |
| TAC Review/Justification of Recommendation |  | Revision Request ties to Reason for Revision as explained in Justification Impact Analysis reviewed and impacts are justified as explained in Justification Opinions were reviewed and discussed Comments were reviewed and discussed (if applicable) Other: (explain) |  |
| ERCOT Board Decision |  | On 6/2/26, the ERCOT Board voted unanimously to recommend approval of NPRR1325 as recommended by TAC in the 5/19/26 TAC Report. |  |
| PUCT Decision |  | On 6/18/26, the PUCT approved NPRR1325 and accompanying ERCOT Market Impact Statement as presented in Project No. 54445, Review of Protocols Adopted by the Independent Organization. |  |

| Opinions |  |
| --- | --- |
| Credit Review | ERCOT Credit Staff and the Credit Finance Sub Group (CFSG) have reviewed NPRR1325 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |
| Independent Market Monitor Opinion | IMM has no opinion on NPRR1325. |
| ERCOT Opinion | ERCOT supports approval of NPRR1325. |
| ERCOT Market Impact Statement | ERCOT Staff has reviewed NPRR1325 and believes the market impact for NPRR1325, along with PGRR145, effectively and reliably facilitates the transition of the Large Load interconnection process from an individual study-based approach to a batch study-based approach that allocates available transmission capacity for studied and committed Large Loads and, to the extent feasible, results in an actionable transmission plan. |

| Sponsor |  |
| --- | --- |
| Name | Agee Springer |
| E-mail Address | agee.springer@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4508 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | cory.phillips@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 041726 | Proposed additional redlines introducing the Provisional Controllable Load Resource (PCLR) concept |
| ERCOT 050126 | Proposed additional redlines to the 4/17/26 ERCOT comments modifying PCLR-related language and introducing a new Section 16.5.5 |
| ERCOT 050226 | Proposed additional redlines to the 5/1/26 ERCOT comments introducing the Withdrawal-Limited Private Use Network (WLPUN) concept |
| Schaper Energy Consulting 050526 | Proposed additional redlines to the 5/2/26 ERCOT comments to address circumstances in which multiple Transmission Service Providers (TSPs) are involved in interconnecting an Interconnecting Large Load Entity (ILLE) to the ERCOT System |
| ERCOT 051126 | Proposed additional redlines to the 5/2/26 ERCOT comments in response to stakeholder feedback, along with minor corrections |
| ERCOT 051526 | Proposed additional redlines to the 5/11/26 ERCOT comments in response to stakeholder feedback, along with minor corrections |

Market Rules Notes

Please note the baseline Protocol language in the following Section(s) has been updated to reflect the incorporation of the following NPRR(s) into the Protocols:

- NPRR1314, Planning Guide Glossary Transition (incorporated 6/1/26)

  - Section 2.1

  - Section 2.2

- NPRR1323, Correction to Inadvertent Removal of Real-Time MCPC Capping for NPRR1290 Phase 2 (incorporated 6/1/26)

  - Section 6.5.7.3

Please note that the following NPRR(s) also propose revisions to the following section(s):

- NPRR1214, Reliability Deployment Price Adder Fix to Provide Locational Price Signals, Reduce Uplift and Risk

  - Section 3.2.5

  - Section 6.5.7.3

- NPRR1309, Board Priority - Dispatchable Reliability Reserve Service Ancillary Service

  - Section 3.9.1

  - Section 6.5.7.3

- NPRR1310, Dispatchable Reliability Reserve Service Plus Energy Storage Resource Participation and Release Factor

  - Section 3.9.1

  - Section 6.5.7.3

- NPRR1322, 60-Day Disclosure of the Day-Ahead Market (DAM) Ancillary Service Only Offer Awards

  - Section 3.2.5

- NPRR1340, Dispatchable Reliability Reserve Service Ancillary Service with Energy Storage Resource Participation

  - Section 6.5.7.3

Proposed Protocol Language Revision

## 2.1	DEFINITIONS

Batch Zero Process

A transitional interconnection process used to evaluate on a system-wide basis one or more Large Loads that meet the requirements set forth in Planning Guide Section 9.2, General Provisions.

Batch Zero Interconnection Study

A singular, system-wide interconnection study conducted by ERCOT that results in a study report that provides interconnection results for each Large Load evaluated in the study.

Batch Zero Refinement Study

A study conducted by ERCOT after the conclusion of the Batch Zero Interconnection Study that results in a final report that identifies Transmission Facility improvements determined by ERCOT to be necessary to serve reliably the subset of Large Loads that have executed an interconnection agreement that meets the requirements set forth in P.U.C. Subst. R. 25.194.

Interconnecting Distribution Service Provider

The electric utility, municipally owned utility, or electric cooperative that is certificated to provide retail electric delivery service at the location in which the Interconnecting Large Load Entity (ILLE) seeks to interconnect.

Interconnecting Transmission Service Provider

The electric utility, municipally owned utility, or electric cooperative that owns and operates the facilities necessary to interconnect the Interconnecting Large Load Entity (ILLE) to the ERCOT system.

#### Load Commissioning Plan (LCP)

A schedule, initially submitted by the Interconnecting Large Load Entity (ILLE), for connecting a Large Load in increments defined by the ILLE, compiled in the format prescribed by ERCOT, detailing dates and associated cumulative peak Demand amounts.  The LCP shall cover the time period from the Initial Energization date through the date on which the Large Load is expected to energize its final amount of peak Demand.  For Provisional Controllable Load Resources (PCLRs), the LCP will include the maximum Low Power Consumption (LPC) and the Maximum Power Consumption (MPC) values.  For Withdrawal-Limited Private Use Networks (WLPUNs), the LCP will include the MW Withdrawal limit(s).

Private Use Network

An electric network connected to the ERCOT Transmission Grid that contains Load that is not directly metered by ERCOT (i.e., Load that is typically netted with internal generation).

Withdrawal-Limited Private Use Network (WLPUN)

A Private Use Network that contains both generation and at least one Large Load, with an established MW Withdrawal limit at a single Point of Interconnection (POI) determined by a system-wide interconnection study conducted by ERCOT.

## Resource

The term is used to refer to an Energy Storage Resource (ESR), a Generation Resource, or a Load Resource.  The term “Resource” used by itself in these Protocols does not include a Settlement Only Generator (SOG) or an Emergency Response Service (ERS) Resource.

[NPRR995: Replace the above definition “Resource” with the following upon system implementation:] ## Resource The term is used to refer to an Energy Storage Resource (ESR), a Generation Resource, or a Load Resource. The term “Resource” used by itself in these Protocols does not include a Settlement Only Generator (SOG), Settlement Only Energy Storage System (SOESS), or an Emergency Response Service (ERS) Resource.

Energy Storage Resource (ESR)

An Energy Storage System (ESS) registered with ERCOT for the purpose of providing energy and/or Ancillary Service to the ERCOT System.

[NPRR1029: Insert the following definition “DC-Coupled Resource upon system implementation:] DC-Coupled Resource A type of Energy Storage Resource (ESR) in which an Energy Storage System (ESS) is combined with wind and/or solar generation in the same modeled generation station and interconnected at the same Point of Interconnection (POI), and where these technologies are interconnected within the site using direct current (DC) equipment. The combined technologies are then connected to the ERCOT System using the same direct current-to-alternating current (DC-to-AC) inverter(s). To be classified as a DC-Coupled Resource, the generator(s) and ESS(s) at a site must meet the following conditions: (1) The ESS component of the Resource must have a nameplate rating of at least ten MW and ten MWh, or the MW rating must equal or exceed 50% of the nameplate MW rating of the inverter; and (2) All intermittent renewable generators must meet the conditions for aggregation stated in paragraph (13) of Section 3.10.7.2, Modeling of Resources and Transmission Loads, except to the extent any such condition requires the generator to be a Resource.

#### Distribution Energy Storage Resource (DESR)

An Energy Storage Resource (ESR) connected to the Distribution System that is either:

(1)	Greater than ten MW and not registered with the Public Utility Commission of Texas (PUCT) as a self-generator; or

(2)	Greater than one MW that chooses to register as a Resource with ERCOT to participate in the ERCOT markets.

#### Transmission Energy Storage Resource (TESR)

An Energy Storage Resource (ESR) connected to the ERCOT transmission system that is either:

(1)	Greater than ten MW and not registered with the Public Utility Commission of Texas (PUCT) as a self-generator; or

(2)	Greater than one MW that chooses to register as a Resource with ERCOT to participate in the ERCOT markets.

Generation Resource

A generator capable of providing energy or Ancillary Service to the ERCOT System and is registered with ERCOT as a Generation Resource.

#### Distribution Generation Resource (DGR)

A Generation Resource connected to the Distribution System that is either:

(1)	Greater than ten MW and not registered with the Public Utility Commission of Texas (PUCT) as a self-generator; or

(2)	Greater than one MW that chooses to register as a Generation Resource to participate in the ERCOT markets.

#### Transmission Generation Resource (TGR)

A Generation Resource connected to the ERCOT transmission system that is either:

(1)	Greater than ten MW and not registered with the Public Utility Commission of Texas (PUCT) as a self-generator; or

(2)	Greater than one MW that chooses to register as a Generation Resource to participate in the ERCOT markets.

Load Resource

A Load capable of providing Ancillary Service to the ERCOT System and/or energy in the form of Demand response and registered with ERCOT as a Load Resource.

#### Aggregate Load Resource (ALR)

A Controllable Load Resource (CLR) that is an aggregation of individual metered sites, each of which has less than ten MW of Demand response capability and all of which are located within a single Load Zone.

[NPRR1188: Delete the above definition “Aggregate Load Resource (ALR)” upon system implementation.]

#### Controllable Load Resource (CLR)

A Load Resource capable of controllably reducing or increasing consumption under Dispatch control by ERCOT.

[NPRR1188: Insert the definition “Aggregate Load Resource (ALR)” below upon system implementation:] #### Aggregate Load Resource (ALR) A Controllable Load Resource (CLR) that is an aggregation of individual metered sites, each of which has less than ten MW of Demand response capability and all of which are located within a single Load Zone.

#### Provisional Controllable Load Resource (PCLR)

A Controllable Load Resource (CLR) that is required to maintain registration and qualification as a CLR until defined exit or de-registration criteria are met.

[NPRR1188: Insert the definition “Aggregate Load Resource (ALR)” below upon system implementation:] #### Aggregate Load Resource (ALR) A Controllable Load Resource (CLR) that is an aggregation of individual metered sites, each of which has less than ten MW of Demand response capability and all of which are located within a single Load Zone.

Settlement Only Generator (SOG)

A generator that is settled for exported energy only, but may not participate in the Ancillary Services market, Reliability Unit Commitment (RUC), Security-Constrained Economic Dispatch (SCED), or make energy offers.  These units are comprised of:

[NPRR995: Delete the above definition “Settlement Only Generator (SOG)” upon system implementation.]

#### Settlement Only Distribution Generator (SODG)

A generator that is connected to the Distribution System with a rating of:

(1)	One MW or less that chooses to register as an SODG; or

(2)	Greater than one and up to ten MW that is capable of providing a net export to the ERCOT System and does not register as a Distribution Generation Resource (DGR).

SODGs must be registered with ERCOT in accordance with Planning Guide Section 6.8.2, Resource Registration Process, and will be modeled in ERCOT systems for reliability in accordance with Section 3.10.7.2, Modeling of Resources and Transmission Loads.

[NPRR995: Delete the above definition “Settlement Only Distribution Generator (SODG)” upon system implementation.]

#### Settlement Only Transmission Generator (SOTG)

A generator that is connected to the ERCOT transmission system with a rating of ten MW or less and is registered with the Public Utility Commission of Texas (PUCT) as a power generation company.  SOTGs must be registered with ERCOT in accordance with Planning Guide Section 6.8.2, Resource Registration Process, and may be modeled in ERCOT systems for reliability in accordance with Section 3.10.7.2, Modeling of Resources and Transmission Loads.

[NPRR995: Delete the above definition “Settlement Only Transmission Generator (SOTG)” upon system implementation.]

#### Settlement Only Transmission Self-Generator (SOTSG)

A generator that is connected to the ERCOT transmission system with a rating of one MW or more and is registered with the Public Utility Commission of Texas (PUCT) as a self-generator.  SOTSGs must be registered with ERCOT in accordance with Planning Guide Section 6.8.2, Resource Registration Process, and will be modeled in ERCOT systems for reliability in accordance with Section 3.10.7.3, Modeling of Private Use Networks.

[NPRR995: Delete the above definition “Settlement Only Transmission Self-Generator (SOTSG)” upon system implementation.]

## 2.2	ACRONYMS AND ABBREVIATIONS

#### PCLR	Provisional Controllable Load Resource

#### WLPUN	Withdrawal-Limited Private Use Network

### 3.2.5	Publication of Resource and Load Information

(1)	Two days after the applicable Operating Day, ERCOT shall post on the ERCOT website for the ERCOT System and, if applicable, for each Disclosure Area, the information derived from each execution of SCED.  The Disclosure Area is the 2003 ERCOT CMZs.  Posting requirements will be applicable to Generation Resources, ESRs, and Controllable Load Resources (CLRs) physically located in the defined Disclosure Area.  This information shall not be posted if the posting of the information would reveal any individual Market Participant’s Protected Information.  The information posted by ERCOT shall include:

(a)	An aggregate energy supply curve based on non-IRR Generation Resources with Energy Offer Curves that are available to SCED.  The energy supply curves will be calculated beginning at the sum of the Low Sustained Limits (LSLs) and ending at the sum of the HSLs for non-IRR Generation Resources with Energy Offer Curves, with the dispatch for each Generation Resource constrained between the Generation Resource’s LSL and HSL.  The result will represent the ERCOT System energy supply curve economic dispatch of the non-IRR Generation Resources with Energy Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(b)	An aggregate energy supply curve based on Wind-powered Generation Resources (WGRs) with Energy Offer Curves that are available to SCED.  The energy supply curves will be calculated beginning at the sum of the LSLs and ending at the sum of the HSLs for WGRs with Energy Offer Curves, with the dispatch for each WGR constrained between the WGR’s LSL and HSL.  The result will represent the ERCOT System energy supply curve economic dispatch of the WGRs with Energy Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(c)	An aggregate energy supply curve based on PhotoVoltaic Generation Resources (PVGRs) with Energy Offer Curves that are available to SCED.  The energy supply curves will be calculated beginning at the sum of the LSLs and ending at the sum of the HSLs for PVGRs with Energy Offer Curves, with the dispatch for each PVGR constrained between the PVGR’s LSL and HSL.  The result will represent the ERCOT System energy supply curve economic dispatch of the PVGRs with Energy Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(d)	An aggregated energy supply and demand curve based on Energy Bid/Offer Curves that are available to SCED.  The curves will be calculated beginning at the sum of the LSLs and ending at the sum of the HSLs for the Energy Bid/Offer Curves, with the dispatch for each Resource constrained between the Resource’s LSL and HSL.  The result will represent the ERCOT System energy supply and demand curve economic dispatch of the ESRs with Energy Bid/Offer Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(e)	The sum of LSLs, sum of Output Schedules, and sum of HSLs for Generation Resources without Energy Offer Curves and ESRs without Energy Bid/Offer Curves;

(f)	The sum of the Base Points of non-IRR Generation Resources with Energy Offer Curves, sum of the Base Points of WGRs with Energy Offer Curves, sum of the Base Points of PVGRs with Energy Offer Curves, sum of the Base Points of ESRs with Energy Bid/Offer Curves, and the sum of the Base Points of all remaining Resources dispatched in SCED;

(g)	The sum of the telemetered Generation Resource net output used in SCED;

(h)	An aggregate energy Demand curve based on the Real-Time Market (RTM) Energy Bid curves available to SCED.  The energy Demand curve will be calculated beginning at the sum of the Low Power Consumptions (LPCs) and ending at the sum of the Maximum Power Consumptions (MPCs), with the dispatch for each CLR constrained between the CLR’s LPC and MPC.  The result will represent the ERCOT System Demand response capability available to SCED of the CLRs with RTM Energy Bids at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

[NPRR1188: Replace paragraph (h) above with the following upon system implementation:] (h) An aggregate energy Demand curve based on the Energy Bid Curves available to SCED. The energy Demand curve will be calculated beginning at the sum of the Low Power Consumptions (LPCs) and ending at the sum of the Maximum Power Consumptions (MPCs), with the dispatch for each CLR constrained between the CLR’s LPC and MPC. The result will represent the ERCOT System Demand response capability available to SCED of the CLRs with Energy Bid Curves at various pricing points, not taking into consideration any physical limitations of the ERCOT System;

(i)	The aggregate Ancillary Service Offers (prices and quantities) in the RTM for each type of Ancillary Service regardless of a Resource’s On-Line or Off-Line status.  For RRS, ERCOT shall separately post aggregated offers from Resources providing Primary Frequency Response, Fast Frequency Response (FFR), and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated offers from Resources that are SCED-dispatchable and those that are manually dispatched.  Linked Ancillary Service Offers will be included as non-linked Ancillary Service Offers;

(j)	The sum of the Base Points of ESRs in discharge mode; and

(k)	The sum of the Base Points of ESRs in charge mode.

(2)	Two days after the applicable Operating Day, ERCOT shall post on the ERCOT website for the ERCOT System the following information derived from each execution of SCED:

(a)	The actual ERCOT Load as determined by subtracting the DC Tie Resource actual telemetry from the sum of the telemetered Generation Resource net output as used in SCED.

(3)	Two days after the applicable Operating Day, ERCOT shall post on the ERCOT website the following information for the ERCOT System and, if applicable, for each Disclosure Area from the DAM for each hourly Settlement Interval:

(a)	An aggregate energy supply curve based on all energy offers that are available to the DAM, including the offer portion of Energy Bid/Offer Curves submitted for ESRs, not taking into consideration Resource Startup Offer or Minimum-Energy Offer or any physical limitations of the ERCOT System.  The result will represent the energy supply curve at various pricing points for energy offers available in the DAM;

(b)	Aggregate minimum energy supply curves based on all Minimum-Energy Offers that are available to the DAM;

(c)	An aggregate energy Demand curve based on the DAM Energy Bid curves available to the DAM, not taking into consideration any physical limitations of the ERCOT System;

[NPRR1188: Replace paragraph (c) above with the following upon system implementation:] (c) An aggregate energy Demand curve based on the DAM Energy Bids and Energy Bid Curves from CLRs and including the bid portion of Energy Bid/Offer Curves available to the DAM, not taking into consideration any physical limitations of the ERCOT System;

(d)	The aggregate amount of cleared energy bids and offers including cleared Minimum-Energy Offer quantities;

(e)	The aggregate Ancillary Service Offers (prices and quantities) in the DAM for each type of Ancillary Service regardless of a Resource’s On-Line or Off-Line status and including Ancillary Service Only Offers.  For RRS, ERCOT shall separately post aggregated offers from Resources providing Primary Frequency Response (including Ancillary Service Only Offers), FFR, and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated offers from Resources that are SCED-dispatchable (including Ancillary Service Only Offers) and those that are manually dispatched.  Linked Ancillary Service Offers will be included as non-linked Ancillary Service Offers;

(f)	The aggregate Self-Arranged Ancillary Service Quantity for each type of service by hour.  For RRS, ERCOT shall separately post aggregated Self-Arranged Ancillary Service Quantities from Resources providing Primary Frequency Response, FFR, and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated Self-Arranged Ancillary Service Quantities from Resources that are SCED-dispatchable and those that are manually dispatched;

(g)	The aggregate amount of cleared Resource-Specific Ancillary Service Offers and Ancillary Service Only Offers.  For RRS, ERCOT shall separately post aggregated Ancillary Service Offers from Resources providing Primary Frequency Response (including Ancillary Service Only Offers), FFR, and Load Resources controlled by high-set under-frequency relays.  For ECRS and Non-Spin, ERCOT shall separately post aggregated Ancillary Service Offers from Resources that are SCED-dispatchable (including Ancillary Service Only Offers) and those that are manually dispatched; and

(h)	The aggregate Point-to-Point (PTP) Obligation bids (not-to-exceed price and quantities) for the ERCOT System and the aggregate PTP Obligation bids that sink in the Disclosure Area for each Disclosure Area.

(4)	ERCOT shall post on the ERCOT website the following information for each Resource for each execution of SCED 60 days prior to the current Operating Day:

(a)	The Generation Resource name and the Generation Resource’s Energy Offer Curve (prices and quantities):

(i)	As submitted;

(ii)	As submitted and extended (or truncated) with proxy Energy Offer Curve logic by ERCOT to fit to the operational HSL and LSL values that are available for dispatch by SCED; and

(iii)	As mitigated and extended for use in SCED;

(b) 	The Resource name and the Resource’s Ancillary Service Offer Curve (prices and quantities) for each type of Ancillary Service:

(i)	As submitted; and

(ii)	As submitted and extended with proxy Ancillary Service Offer Curve logic by ERCOT.

(c)	The Load Resource name and the Load Resource’s bid to buy (prices and quantities);

[NPRR1188: Replace paragraph (c) above with the following upon system implementation:] (c) The Load Resource name and the Load Resource’s Energy Bid Curve (prices and quantities); (i) As submitted; (ii) As submitted and extended (or truncated) with proxy Energy Bid Curve logic by ERCOT to fit the operational LPC and MPC values that are available for dispatch by SCED; and (iii) As adjusted (pursuant to Section 4.4.9.4.4) and extended for use in SCED (for any Provisional CLRs (PCLRs));

(d)	The Generation Resource name and the Generation Resource’s Output Schedule;

(e)	The Generation Resource name and actual metered Generation Resource net output;

(f)	The self-arranged Ancillary Service by service for each QSE;

(g)	The following Generation Resource data using a snapshot from each execution of SCED:

(i)	The Generation Resource name;

(ii)	The Generation Resource status;

(iii)	The Generation Resource HSL, LSL, High Dispatch Limit (HDL), and Low Dispatch Limit (LDL);

(iv)	The Generation Resource Base Point from SCED;

(v)	The telemetered Generation Resource net output used in SCED;

(vi)	The Ancillary Service Resource awards for each Ancillary Service;

(vii)	The Generation Resource Startup Cost and minimum energy cost used in the Reliability Unit Commitment (RUC);

(viii) 	The telemetered Normal Ramp Rates;

(ix) 	The telemetered Ancillary Service capabilities; and

(h)	The following Load Resource data using a snapshot from each execution of SCED:

(i)	The Load Resource name;

(ii)	The Load Resource status;

(iii)	The MPC for a Load Resource;

(iv)	The LPC for a Load Resource;

(v)	The Load Resource HDL and LDL, for a CLR that has a Resource Status of ONL;

(vi)	The Load Resource Base Point from SCED, for a CLR that has a Resource Status of ONL;

(vii)	The telemetered real power consumption;

(viii)	The Ancillary Service Resource awards for each Ancillary Service;

(ix)	The telemetered self-provided Ancillary Service amount for each Ancillary Service;

(x)	The telemetered Normal Ramp Rates;

(xi) 	The telemetered Ancillary Service capabilities; and

(i)	The ESR name and the ESR’s Energy Bid/Offer Curve (prices and quantities):

(i)	As submitted; and

(ii)	As submitted and extended with proxy Energy Offer Curve logic by ERCOT to fit to the operational HSL and LSL values that are available for dispatch by SCED;

(j)	The following ESR data using a snapshot from each execution of SCED:

(i)	The ESR name;

(ii)	The ESR status;

(iii)	The ESR HSL, LSL, HDL, and LDL;

(iv)	The ESR Base Point from SCED;

(v)	The telemetered ESR net output used in SCED;

(vi)	The Ancillary Service Resource awards for each Ancillary Service;

(vii) 	The telemetered Normal Ramp Rates;

(viii) 	The telemetered Ancillary Service capabilities;

(ix)	The telemetered State of Charge in MWh;

(x)	The telemetered Minimum State of Charge (MinSOC) in MWh; and

(xi)	The telemetered Maximum State of Charge (MaxSOC) in MWh.

(5)	ERCOT shall post on the ERCOT website for each Resource for each Operating Hour 60 days prior to the current Operating Day a count of the number of times for each Ancillary Service that the Resource’s Ancillary Service Offer quantity or price was updated within the Operating Period.  ERCOT shall post on the ERCOT website for each Resource for each Operating Hour 60 days prior to the current Operating Day, a count of the number of times a Resource’s Energy Offer quantity or price was updated within the Operating Hour, including any reason accompanying the update.

(6)	If any Real-Time Locational Marginal Price (LMP) exceeds 50 times the Fuel Index Price (FIP) during any SCED interval for the applicable Operating Day, ERCOT shall post on the ERCOT website the portion of any Generation Resource’s as-submitted and as-mitigated and extended Energy Offer Curve or any ESR’s as-submitted and as-mitigated and extended Energy Bid/Offer Curve that is at or above 50 times the FIP for that SCED interval seven days after the applicable Operating Day.

(7)	If any Market Clearing Price for Capacity (MCPC) for an Ancillary Service exceeds 50 times the FIP for any Operating Hour in a DAM or any SCED interval in the RTM for the applicable Operating Day, ERCOT shall post on the ERCOT website the portion on any Resource’s Ancillary Service Offer that is at or above 50 times the FIP for that Ancillary Service for that Operating Hour for the DAM or SCED interval for the RTM seven days after the applicable Operating Day.

(8)	ERCOT shall post on the ERCOT website the offer price and the name of the Entity submitting the offer for the highest-priced offer selected or Dispatched by SCED three days after the end of the applicable Operating Day.  If multiple Entities submitted the highest-priced offers selected, all Entities shall be identified on the ERCOT website.

(9)	ERCOT shall post on the ERCOT website the bid price and the name of the Entity submitting the bid for the highest-priced bid selected or Dispatched by SCED three days after the end of the applicable Operating Day.  If multiple Entities submitted the highest-priced bids selected, all Entities shall be identified on the ERCOT website.

(10)	ERCOT shall post on the ERCOT website the offer price and the name of the Entity submitting the offer for the highest-priced Ancillary Service Offer selected in the DAM or RTM for each Ancillary Service three days after the end of the applicable Operating Day.  If multiple Entities submitted the highest-priced offers selected, all Entities shall be identified on the ERCOT website.  The report shall specify whether the Ancillary Service Offer was selected in a DAM or RTM.

(11)	ERCOT shall post on the ERCOT website for each Operating Day the following information for each Resource:

(a)	The Resource name;

(b)	The name of the Resource Entity;

(c)	Except for Load Resources that are not SCED qualified, the name of the Decision Making Entity (DME) controlling the Resource, as reflected in the Managed Capacity Declaration submitted by the Resource Entity in accordance with Section 3.6.2, Decision Making Entity for a Resource; and

(d)	Flag for Reliability Must-Run (RMR) Resources.

(12)	ERCOT shall post on the ERCOT website the following information from the DAM for each hourly Settlement Interval for the applicable Operating Day 60 days prior to the current Operating Day:

(a)	The Generation Resource name and the Generation Resource’s Three-Part Supply Offer (prices and quantities), including Startup Offer and Minimum-Energy Offer, available for the DAM;

(b)	For each Settlement Point, individual DAM Energy-Only Offer Curves available for the DAM and the name of the QSE submitting the offer;

(c)	The Resource name and the Resource’s Ancillary Service Offers available for the DAM;

(d) 	The Ancillary Service Only Offer for each Ancillary Service and the name of the QSE submitting the offer;

(e)	For each Settlement Point, individual DAM Energy Bids available for the DAM and the name of the QSE submitting the bid;

(f)	For each Settlement Point, individual PTP Obligation bids available to the DAM that sink at the Settlement Point and the QSE submitting the bid;

(g)	The awards for each Ancillary Service from the DAM for each Generation Resource;

(h)	The awards for each Ancillary Service from the DAM for each Load Resource;

(i)	The award for each Three-Part Supply Offer from the DAM and the name of the QSE receiving the award;

(j)	For each Settlement Point, the award of each DAM Energy-Only Offer from the DAM and the name of the QSE receiving the award;

(k)	For each Settlement Point, the award of each DAM Energy Bid from the DAM and the name of the QSE receiving the award;

(l)	For each Settlement Point, the award of each PTP Obligation bid from the DAM that sinks at the Settlement Point, including whether or not the PTP Obligation bid was linked to an Option, and the QSE submitting the bid;

[NPRR1188: Insert items (m) and (n) below upon system implementation and renumber accordingly:] (m) The CLR name and the CLR’s Energy Bid Curve (prices and quantities) available for the DAM; and (n) The award for each CLR’s Energy Bid Curve from the DAM and the name of the QSE receiving the award.

(m)	The ESR name and the ESR’s Energy Bid/Offer Curve (prices and quantities), available for the DAM;

(n)	The awards for each Ancillary Service from the DAM for each ESR; and

(o)	The award for each Energy Bid/Offer Curve from the DAM and the name of the QSE receiving the award.

### 3.9.1	Current Operating Plan (COP) Criteria

(1)	Each QSE that represents a Resource must submit a COP to ERCOT that reflects expected operating conditions for each Resource for each hour in the next seven Operating Days.

(2)	Each QSE that represents a Resource shall update its COP reflecting changes in availability of any Resource as soon as reasonably practicable, but in no event later than 60 minutes after the event that caused the change.  Each QSE shall timely update its COP unless in the reasonable judgment of the QSE, such compliance would create an undue threat to safety, undue risk of bodily harm, or undue damage to equipment.  The QSE is excused from updating the COP only for so long as the undue threat to safety, undue risk of bodily harm, or undue damage to equipment exists.  The time for updating the COP begins once the undue threat to safety, undue risk of bodily harm, or undue damage to equipment no longer exists.

(3)	Each QSE that represents a Resource shall update its COP to reflect the ability of the Resource to provide each Ancillary Service by product and sub-type.  Additionally, for a COP provided for an ESR, the QSE shall ensure that the Hour Beginning Planned State of Charge (HBSOC) for any two consecutive hours shall be feasible based on the ESR’s maximum rate of charge or discharge.

(4)	Load Resource COP values may be adjusted to reflect Distribution Losses in accordance with Section 8.1.1.2, General Capacity Testing Requirements.

(5)	A COP must include the following for each Resource represented by the QSE:

(a)	The name of the Resource;

(b)	The expected Resource Status:

(i)	Select one of the following for Generation Resources synchronized to the ERCOT System that best describes the Resource’s status.  Unless otherwise provided below, these Resource Statuses are to be used for COP and/or Real-Time telemetry purposes, as appropriate.

(A)	ONRUC – On-Line and the hour is a RUC-Committed Hour;

(B)	ON – On-Line Resource with Energy Offer Curve;

(C)	ONOS – On-Line Resource with Output Schedule;

(D)	ONTEST – On-Line blocked from Security-Constrained Economic Dispatch (SCED) for operations testing (while ONTEST, a Generation Resource may be shown on Outage in the Outage Scheduler);

(E)	ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and High Sustained Limit (HSL) to reflect operating limits);

(F)	ONOPTOUT – On-Line and the hour is a RUC Buy-Back Hour;

(G)	SHUTDOWN – The Resource is On-Line and in a shutdown sequence, and is not eligible for an Ancillary Service award.  This Resource Status is only to be used for Real-Time telemetry purposes;

(H)	STARTUP – The Resource is On-Line and in a start-up sequence and is not eligible for an Ancillary Service award, unless coming On-Line in response to a manual deployment of ERCOT Contingency Reserve Service (ECRS) or Non-Spinning Reserve (Non-Spin).  This Resource Status is only to be used for Real-Time telemetry purposes;

(I)	OFFQS – Off-Line but available for SCED deployment and to provide ECRS and Non-Spin, if qualified and capable.  Only qualified Quick Start Generation Resources (QSGRs) may utilize this status;

(J)	ONSC – Resource is On-Line operating as a synchronous condenser and available to provide Responsive Reserve (RRS) and ECRS, if qualified and capable, and for commitment by RUC, but is unavailable for Dispatch by SCED.  For SCED, Resource Base Points will be set equal to the telemetered net real power of the Resource available at the time of the SCED execution; and

(K)	ONHOLD – Resource is On-Line but temporarily unavailable for Dispatch by SCED or Ancillary Service awards.  This Resource Status is only to be used for Real-Time telemetry purposes.  For SCED, Resource Base Points will be set equal to the telemetered net real power of the Resource available at the time of the SCED execution.

[NPRR1188: Replace item (K) above with the following upon system implementation:] (K) ONHOLD – Resource is On-Line but temporarily unavailable for Dispatch by SCED or Ancillary Service awards due to a valid and verifiable operational reason. This Resource Status is only to be used for Real-Time telemetry purposes. For SCED, Resource Base Points will be set equal to the telemetered net real power of the Resource available at the time of the SCED execution.

(ii)	Select one of the following for Off-Line Generation Resources not synchronized to the ERCOT System that best describes the Resource’s status.  These Resource Statuses are to be used for COP and/or Real-Time telemetry purposes, as appropriate.

- (A)	OUT – Off-Line and unavailable, or not connected to the ERCOT System and operating in a Private Microgrid Island (PMI);

- (B)	OFF – Off-Line but available for commitment in the Day-Ahead Market (DAM), RUC, and providing Non-Spin, if qualified and capable;

- (C)	EMR – Available for commitment as a Resource contracted by ERCOT under Section 3.14.1, Reliability Must Run, or under paragraph (4) of Section 6.5.1.1, ERCOT Control Area Authority, or available for commitment only for ERCOT-declared Emergency Condition events; the QSE may appropriately set LSL and HSL to reflect operating limits;

- (D)	EMRSWGR – Switchable Generation Resource (SWGR) operating in a non-ERCOT Control Area, or in the case of a Combined Cycle Train with one or more SWGRs, a configuration in which one or more of the physical units in that configuration are operating in a non-ERCOT Control Area.

(iii)	Select one of the following for Load Resources.  Unless otherwise provided below, these Resource Statuses are to be used for COP and/or Real-Time telemetry purposes.

[NPRR1188: Insert items (A) and (B) below upon system implementation and renumber accordingly:] (A) ONTEST – On-Line blocked from SCED for operations testing; (B) ONHOLD – CLR is On-Line but temporarily unavailable for Dispatch by SCED or providing Ancillary Service due to a valid and verifiable operational reason. This Resource Status is only to be used for Real-Time telemetry purposes. For SCED, Resource Base Points will be set equal to the telemetered net real power of the Resource available at the time of the SCED execution.

- (A)	OUTL – Not available;

[NPRR1188: Replace item (A) above with the following upon system implementation:] (A) OUTL – Not available. For a CLR that is not an Aggregate Load Resource (ALR), this status can only be used when the Resource is Off-Line and unavailable with its energy consumption at zero;

- (B)	ONL – On-Line and available for Dispatch by SCED or providing Ancillary Services.

(iv)	Select one of the following for ESRs.  Unless otherwise provided below, these Resource Statuses are to be used for COP and Real-Time telemetry purposes:

(A)	ON – On-Line Resource with Energy Bid/Offer Curve;

(B)	ONOS – On-Line Resource with Output Schedule;

(C)	ONTEST – On-Line blocked from SCED for operations testing (while ONTEST, an ESR may be shown on Outage in the Outage Scheduler);

(D)	ONEMR – On-Line EMR (available for commitment or dispatch only for ERCOT-declared Emergency Conditions; the QSE may appropriately set LSL and HSL to reflect operating limits);

(E)	ONHOLD – Resource is On-Line but temporarily unavailable for Dispatch by SCED or Ancillary Service awards.  ESRs shall not be discharging into or charging from the grid.  This Resource Status is only to be used for Real-Time telemetry purposes; and

(F)	OUT – Off-Line and unavailable, or not connected to the ERCOT System and operating in a PMI;

(c)	The HSL;

(i)	For Load Resources other than CLRs, the HSL should equal the expected power consumption;

(ii)	For ESRs, the HSL may be negative;

(d)	The LSL;

(i)	For Load Resources other than CLRs, the LSL should equal the expected Low Power Consumption (LPC);

(ii)	For ESRs, the LSL may be positive;

(e)	The High Emergency Limit (HEL);

(f)	The Low Emergency Limit (LEL);

(g)	Ancillary Service capability in MW for each product and sub-type; and

(h)	For ESRs:

(i)	Minimum State of Charge (MinSOC);

(ii)	Maximum State of Charge (MaxSOC); and

(iii)	HBSOC.

(6)	For Combined Cycle Generation Resources, the above items are required for each operating configuration.  In each hour only one Combined Cycle Generation Resource in a Combined Cycle Train may be assigned one of the On-Line Resource Status codes described above.

(a)	During a RUC study period, if a QSE’s COP reports multiple Combined Cycle Generation Resources in a Combined Cycle Train to be On-Line for any hour, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource with the largest HSL is considered to be On-Line and all other Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.  Furthermore, until the QSE corrects its COP, the Off-Line Combined Cycle Generation Resources as designated through the application of this process are ineligible for RUC commitment or de-commitment Dispatch Instructions.

(b)	For any hour in which QSE-submitted COP entries are used to determine the initial state of a Combined Cycle Generation Resource for a DAM or Day-Ahead Reliability Unit Commitment (DRUC) study and the COP shows multiple Combined Cycle Generation Resources in a Combined Cycle Train to be in an On-Line Resource Status, then until the QSE corrects its COP, the On-Line Combined Cycle Generation Resource that has been On-Line for the longest time from the last recorded start by ERCOT systems, regardless of the reason for the start, combined with the COP Resource Status for the remaining hours of the current Operating Day, is considered to be On-Line at the start of the DRUC study period and all other COP-designated Combined Cycle Generation Resources in the Combined Cycle Train are considered to be Off-Line.

(c)	ERCOT systems shall allow only one Combined Cycle Generation Resource in a Combined Cycle Train to offer Off-Line Non-Spin in the DAM or SCED.

(i)	If there are multiple Non-Spin offers from different Combined Cycle Generation Resources in a Combined Cycle Train, then prior to execution of the DAM, ERCOT shall select the Non-Spin offer from the Combined Cycle Generation Resource with the highest HSL for consideration in the DAM and ignore the other offers.

(ii)	Combined Cycle Generation Resources offering Off-Line Non-Spin must be able to transition from the shutdown state to the offered Combined Cycle Generation Resource On-Line state and be capable of ramping to the full amount of the Non-Spin offered.

(d)	The DAM and RUC shall honor the registered hot, intermediate or cold Startup Costs for each Combined Cycle Generation Resource registered in a Combined Cycle Train when determining the transition costs for a Combined Cycle Generation Resource.  In the DAM and RUC, the Startup Cost for a Combined Cycle Generation Resource shall be determined by the positive transition cost from the On-Line Combined Cycle Generation Resource within the Combine Cycle Train or from a shutdown condition, whichever ERCOT determines to be appropriate.

(7)	ERCOT may accept COPs only from QSEs.

(8)	For the first 168 hours of the COP, ERCOT will update the HSL values for Wind-powered Generation Resources (WGRs) with the most recently updated Short-Term Wind Power Forecast (STWPF), and the HSL values for PhotoVoltaic Generation Resources (PVGRs) with the most recently updated Short-Term PhotoVoltaic Power Forecast (STPPF).  ERCOT will notify the QSE via an Extensible Markup Language (XML) message each time COP HSL values are updated with the forecast values.  A QSE representing a WGR may override the STWPF HSL value but must submit an HSL value that is less than or equal to the amount for that Resource from the most recent STWPF provided by ERCOT; a QSE representing a PVGR may override the STPPF HSL value but must submit an HSL value that is less than or equal to the amount for that Resource from the most recent STPPF provided by ERCOT.

[NPRR1029: Replace paragraph (8) above with the following upon system implementation:] (8) For the first 168 hours of the COP, ERCOT will update the HSL values for Wind-powered Generation Resources (WGRs) with the most recently updated Short-Term Wind Power Forecast (STWPF), and the HSL values for PhotoVoltaic Generation Resources (PVGRs) with the most recently updated Short-Term PhotoVoltaic Power Forecast (STPPF). A QSE representing a DC-Coupled Resource shall provide the capacity value of the Energy Storage System (ESS) that is included in the HSL of the DC-Coupled Resource, and ERCOT will update the DC-Coupled Resource’s HSL with the sum of the forecasts of the intermittent renewable generation component and the QSE-submitted value for the ESS component. ERCOT will notify the QSE via an Extensible Markup Language (XML) message each time COP HSL values are updated with the forecast values. A QSE representing a WGR may override the STWPF HSL value but must submit an HSL value that is less than or equal to the amount for that Resource from the most recent STWPF provided by ERCOT; a QSE representing a PVGR may override the STPPF HSL value but must submit an HSL value that is less than or equal to the amount for that Resource from the most recent STPPF provided by ERCOT. A QSE representing a DC-Coupled Resource may override the COP HSL value with a value that is lower than the ERCOT-populated value, and may override with a value that is higher than the ERCOT-populated value if the ESS component of the DC-Coupled Resource can support the higher value.

(9)	A QSE representing a Generation Resource that is not actively providing Ancillary Services or is providing Off-Line Non-Spin that the Resource will provide following the shutdown, may only use a Resource Status of SHUTDOWN to indicate to ERCOT through telemetry that the Resource is operating in a shutdown sequence or a Resource Status of ONTEST to indicate in the COP and through telemetry that the Generation Resource is performing a test of its operations either manually dispatched by the QSE or by ERCOT as part of the test.  A QSE representing a Generation Resource that is not actively providing Ancillary Services may only use a Resource Status of STARTUP to indicate to ERCOT through telemetry that the Resource is operating in a start-up sequence requiring manual control and is not available for Dispatch.

(10)	If a QSE has not submitted a valid COP for any Generation Resource for any hour in the DAM or RUC Study Period, then the Generation Resource is considered to have a Resource Status as OUT thus not available for DAM awards or RUC commitments for those hours.

(11)	If a COP is not available for any Resource for any hour from the current hour to the start of the DAM period or RUC study, then the Resource Status for those hours are considered equal to the last known Resource Status from a previous hour’s COP or from telemetry as appropriate for that Resource.

(12)	A QSE representing a Resource may only use the Resource Status code of EMR for a Resource whose operation would have impacts that cannot be monetized and reflected through the Resource’s Energy Offer Curve or recovered through the RUC make-whole process or if the Resource has been contracted by ERCOT under Section 3.14.1 or under paragraph (4) of Section 6.5.1.1.  If ERCOT chooses to commit an Off-Line unit with EMR Resource Status that has been contracted by ERCOT under Section 3.14.1 or under paragraph (4) of Section 6.5.1.1, the QSE shall change its Resource Status to ONRUC.  Otherwise, the QSE shall change its Resource Status to ONEMR.

(13)     A QSE representing a Resource may use the Resource Status code of ONEMR for a        Resource that is:

(a)	On-Line, but for equipment problems it must be held at its current output level until repair and/or replacement of equipment can be accomplished; or

(b)	A hydro unit.

(14)	A QSE operating a Resource with a Resource Status code of ONEMR may set the HSL and LSL of the unit to be equal to ensure that SCED does not send Base Points that would move the unit.

(15)	A QSE representing a Resource may use the Resource Status code of EMRSWGR only for an SWGR.

(16)	A QSE representing a Self-Limiting Facility must ensure that the sum of the COP HSL/LSL and the sum of the telemetered HSL/LSL submitted for each Resource within the Self-Limiting Facility do not exceed either the limit on MW Injection or the limit on the MW Withdrawal established for the Self-Limiting Facility.

[NPRR1029: Insert paragraph (17) below upon system implementation and renumber accordingly:] (17) A QSE representing a DC-Coupled Resource shall not submit an HSL that exceeds the inverter rating or the sum of the nameplate ratings of the generation component(s) of the Resource.

(17)	A QSE representing an ESR shall ensure that COP values for a given hour follow the following rules:

(a)	MinSOC is greater than or equal to the nameplate minimum MWh operating SOC limit;

(b)	MaxSOC is less than or equal to the nameplate maximum MWh operating SOC limit; and

(c)	HBSOC is a value between the corresponding COP values of MinSOC and MaxSOC.

(18)	A QSE representing a Resource Entity with one or more Energy Storage Resources (ESRs) in a Withdrawal-Limited Private Use Network (WLPUN) shall ensure that the sum of all of the ESRs’ LSLs reflected in each COP or in telemetry submitted to ERCOT is not lower than the established MW Withdrawal limit for the WLPUN less the sum of the expected consumption of the Large Load and any auxiliary or other load in the WLPUN.

#### 3.10.7.3.1	Withdrawal-Limited Private Use Networks

(1)	The MW Withdrawal limit for a Withdrawal-Limited Private Use Network (WLPUN) shall be determined pursuant to Planning Guide Section 9.3, Batch Zero Interconnection Study.  The established MW Withdrawal limit shall be recorded in the Resource Registration data for the Facility and incorporated in the ERCOT Network Operations Model prior to Initial Energization of the Large Load.

(2)	The QSE for the Resource Entity that represents Generation Resources or Energy Storage Resources (ESRs) in a WLPUN shall ensure that the net Demand at the Point of Interconnection (POI) does not exceed the established MW Withdrawal limit.

(a)	In the event of an exceedance of the established MW Withdrawal limit, the QSE for the Resource Entity that represents the Generation Resources or ESRs in the WLPUN shall ensure the MW Withdrawal is reduced as soon as practicable but not longer than one minute.  If the exceedance is not resolved within one minute, ERCOT may limit or suspend operation of all or part of the WLPUN until mitigation is implemented and agreed to by ERCOT.

(3)	On a monthly basis, ERCOT shall report to the Reliability Monitor any instance where the MW Withdrawal at the POI of a WLPUN exceeded the established MW Withdrawal limit, based on the meter data at the POI or based on telemetry data provided by the QSE representing Resources in the WLPUN.

(4)	The parties participating in a WLPUN may terminate the WLPUN only upon ERCOT’s determination that all necessary Transmission Facilities required for the service of all anticipated load in the WLPUN have been energized and ERCOT’s written confirmation that the WLPUN may be terminated.  The Resource Entity for the generator participating in a WLPUN may terminate such registration on behalf of all parties by submitting written notice of such termination following ERCOT’s written confirmation, as described above.

(5)	If ownership of a Large Load or generator registered as a WLPUN is transferred to another Entity, the acquiring Entity shall be bound by all obligations applicable to the WLPUN under these Protocols and Guides.  The acquiring Entity shall notify ERCOT and submit an updated, executed, and notarized Section 23, Form X, within ten days of the transfer.

#### 3.11.4.3	Categorization of Proposed Transmission Projects

(1)	ERCOT classifies all proposed transmission projects into one of four categories (or Tiers).  Each Tier is defined so that projects with a similar cost and impact on reliability and the ERCOT market are grouped into the same Tier.  For Tier classification, the total estimated cost of the project shall be used which includes costs borne by another party.

(a)	A project shall be classified as Tier 1 if the estimated capital cost is greater than or equal to $200,000,000, unless the project is considered to be a neutral project pursuant to paragraph (f) below.

(b)	A project shall be classified as Tier 2 if the estimated capital cost is less than $200,000,000 and a Certificate of Convenience and Necessity (CCN) is required, unless the project is considered to be a neutral project pursuant to paragraph (f) below.

(c)	A project shall be classified as Tier 3 if any of the following are true:

(i)	The estimated capital cost is less than $200,000,000 and greater than or equal to $50,000,000 and a CCN is not required, unless the project is considered to be a neutral project pursuant to paragraph (f) below; or

(ii)	The estimated capital cost is less than $50,000,000, a CCN is not required, and the project includes 345 kV circuit reconductor of more than one mile, additional 345/138 kV autotransformer capacity, or a new 345 kV substation, unless the project is considered to be a neutral project pursuant to paragraph (f) below.

(d)	A project shall be initially classified as Tier 3 if it meets any of the following conditions and shall subsequently be reclassified as a Tier 4 neutral project once the comment process is complete:

(i)	The estimated capital cost is greater than or equal to $50,000,000, and it is proposed for the purpose of replacing aged infrastructure or for storm hardening; or

(ii)	The estimated capital cost is less than $50,000,000, and it involves the permanent bypass of an existing series capacitor or un-bypassing of a series capacitor that was previously designated as permanently bypassed.  The relevant TSP shall coordinate with ERCOT prior to submission of the project for RPG Project Review, and ERCOT shall perform an economic analysis of the project for inclusion in the RPG project submission.

(e)	A project shall be classified as Tier 4 if it does not meet the requirements to be classified as Tier 1, 2, or 3 or if it is considered a neutral project pursuant to paragraph (f) below.

(f)	A project shall be considered a neutral project if it consists entirely of:

(i)	The addition of or upgrades to radial transmission circuits;

(ii)	The addition of equipment that does not affect the transfer capability of a circuit;

(iii)	Repair and replacement-in-kind projects;

(iv)	Transmission Facilities needed to connect a new Generation Resource, Energy Storage Resource (ESR), or Settlement Only Generator (SOG) to a new or existing substation on the existing ERCOT Transmission Grid, including the substation;

(v)	The addition of static reactive devices;

(vi)	A project to serve a new Load, unless such project would create a new transmission circuit connection between two stations (other than looping an existing circuit into the new Load-serving station);

(vii)	Replacement of failed equipment, even if it results in a ratings and/or impedance change; or

(viii)	Equipment upgrades resulting in only ratings changes.

(2)	ERCOT may use its reasonable judgment to increase the level of review of a proposed project (e.g., from Tier 3 to Tier 2) from that which would be strictly indicated by these criteria, based on stakeholder comments, ERCOT analysis or the system impacts of the project.

(a)	A project with an estimated capital cost greater than or equal to $100,000,000 that requires a CCN shall be reclassified and processed as a Tier 1 project upon request by a Market Participant during the comment period per Planning Guide Section 3.1.5, Regional Planning Group Comment Process.

(3)	Any project that would be built by an Entity that is exempt (e.g., a Municipally Owned Utility (MOU)) from getting a CCN for transmission projects but would require a CCN if it were to be built by a regulated Entity will be treated as if the project would require a CCN for the purpose of defining the Tier of the project.

(4)	If during the course of ERCOT’s independent review of a project, the project scope changes, ERCOT may reclassify the project into the appropriate Tier.

(5)	The set of projects included in the Batch Zero Refinement Study and submitted for RPG Project review as described in Planning Guide Section 9.5, Batch Zero Study Refinement and Delivery of Transmission Plan, shall be reviewed as a single project and classified in accordance with the criteria in this Section considering the sum total of all included projects.

#### 4.4.9.4	Mitigated Offer Cap, Mitigated Offer Floor, and Adjusted Bid Caps

4.4.9.4.4	Adjusted Bid Caps

(1)	For each Provisional Controllable Load Resource (PCLR), SCED shall determine whether an Adjusted Bid Cap (ABC) applies to energy bid curves used in Step 2 of the two-step SCED process under paragraph (14)(b) of Section 6.5.7.3, as follows:

(a)	After each SCED run, SCED shall generate a list of transmission constraint identifiers whose shadow prices were equal to or greater than a pre-defined threshold of their maximum shadow price. The pre-defined thresholds are as follows:

(i)	Base Case: 90%;

(ii)	Contingency: 90%;

(iii)	Interconnection Reliability Operating Limit (IROL): 90%; and

(iv)	Generic Transmission Constraint: 90%.

(b)	SCED shall compare the transmission constraint list generated in paragraph (a) above from the previous SCED run against all active constraints in the current run.

(c)	For any active transmission constraint in the current SCED which matches the list identified in paragraph (a) above, all PCLRs will be considered:

(i)	Where a PCLR has a Shift Factor equal to or less than -2% on the list identified in paragraph (c) above, the constraint contribution shall be calculated by multiplying the maximum Shadow Price of that constraint by the Shift Factor of the PCLR for each constraint identified in paragraph (c) above.

(A)	If no such constraints exist, or where the PCLR has a Shift Factor greater than -2%, no Adjusted Bid Cap (ABC) will be applied.

(B)	Otherwise, the Adjusted Bid Cap (ABC) used in Step 2 of the two-step SCED process as described in paragraph (14)(b)(iv) of Section 6.5.7.3 will be set at the lowest absolute value of these constraint contributions plus the System Lambda of the first step in the two-step SCED process described in paragraph (14)(a) of Section 6.5.7.3 minus $0.01/MWh.

#### 6.5.7.3	Security Constrained Economic Dispatch

(1)	The SCED process is designed to simultaneously manage energy, Ancillary Services, the system power balance and network congestion through Resource Base Points, Ancillary Service awards, and the calculation of LMPs and Real-Time MCPCs approximately every five minutes, or more frequently if necessary.  The SCED process uses a two-step methodology that applies mitigation to offers for energy prospectively to resolve Non-Competitive Constraints for the current Operating Hour.  The SCED process evaluates Energy Offer Curves, Energy Bid/Offer Curves, Ancillary Service Offers, Output Schedules, and RTM Energy Bids to determine Resource Dispatch Instructions and Ancillary Service awards by maximizing bid-based revenues minus offer-based costs, subject to power balance, Ancillary Service Demand Curves (ASDCs), and network constraints.  The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP.  In addition, the SCED process accounts for each ESR’s State of Charge (SOC) and SOC operating limits.  This is to ensure that the SCED process will issue ESR Base Points and Ancillary Services that are feasible taking into account SCED duration requirements for energy and Ancillary Services and also that do not violate the ESR’s Minimum State of Charge (MinSOC) and Maximum State of Charge (MaxSOC) limits.

[NPRR1188: Replace paragraph (1) above with the following upon system implementation:] (1) The SCED process is designed to simultaneously manage energy, Ancillary Services, the system power balance and network congestion through Resource Base Points, Ancillary Service awards, and the calculation of LMPs and Real-Time MCPCs approximately every five minutes, or more frequently if necessary. SCED employs a dynamic capping process for Provisional Controllable Load Resource (PCLR) energy bid curves based on two factors: a comparative list of transmission constraints at risk of being violated and each PCLR's Shift Factor relative to a threshold for resolving those constraints. The SCED process uses a two-step methodology that applies: 1) mitigation to offers for energy prospectively to resolve Non-Competitive Constraints for the current Operating Hour and 2) capping the QSE-submitted Energy Bid Curve of a PCLR to resolve eligible transmission constraints for each SCED run that meets the criteria specified in Section 4.4.9.4.4. The SCED process evaluates Energy Offer Curves, Energy Bid/Offer Curves, Ancillary Service Offers, Output Schedules, and Energy Bid Curves to determine Resource Dispatch Instructions and Ancillary Service awards by maximizing bid-based revenues minus offer-based costs, subject to power balance, Ancillary Service Demand Curves (ASDCs), and network constraints. The SCED process uses the Resource Status provided by SCADA telemetry under Section 6.5.5.2, Operational Data Requirements, and validated by the Real-Time Sequence, instead of the Resource Status provided by the COP. In addition, the SCED process accounts for each ESR’s State of Charge (SOC) and SOC operating limits. This is to ensure that the SCED process will issue ESR Base Points and Ancillary Services that are feasible taking into account SCED duration requirements for energy and Ancillary Services and also that do not violate the ESR’s Minimum State of Charge (MinSOC) and Maximum State of Charge (MaxSOC) limits.

(2)	The SCED solution must monitor cumulative deployment of Regulation Services and ensure that Regulation Services deployment is minimized over time.

(3)	In the Generation To Be Dispatched (GTBD) determined by LFC, ERCOT shall subtract the sum of the telemetered net real power consumption from all CLRs available to SCED.

(4)	For use as SCED inputs for determining energy dispatch and Ancillary Service awards, ERCOT shall use the available capacity of all committed Generation Resources by creating proxy Energy Offer Curves for certain Resources as follows:

(a)	Non-IRRs without Energy Offer Curves

(i)	ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below for:

(A)	Each non-IRR for which its QSE has submitted an Output Schedule instead of an Energy Offer Curve.

| MW | Price (per MWh) |
| --- | --- |
| HSL | RTSWCAP |
| Output Schedule MW plus 1 MW | RTSWCAP minus $0.01 |
| Output Schedule MW | -$249.99 |
| LSL | -$250.00 |

(b)	Non-IRRs without full-range Energy Offer Curves

(i)	For each non-IRR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the Resource’s available capacity, ERCOT shall create a proxy Energy Offer Curve that extends the submitted Energy Offer Curve to use the entire available capacity of the Resource above the highest point on the Energy Offer Curve to the Resource’s HSL and the offer floor from the lowest point on the Energy Offer Curve to its LSL, using these points:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(c)	IRRs

(i)	For each IRR that has not submitted an Energy Offer Curve, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | $1,500 |
| HSL minus 1 MW | -$249.99 |
| LSL | -$250.00 |

(ii)	For each IRR for which its QSE has submitted an Energy Offer Curve that does not cover the full range of the IRR’s available capacity, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in submitted Energy Offer Curve) | Price associated with the highest MW in submitted Energy Offer Curve |
| Energy Offer Curve | Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

(d)	RUC-committed Resources

(i)        For each RUC-committed Resource that has not submitted an Energy Offer Curve, ERCOT shall create a proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL | $250 |
| Zero | $250 |

(ii)       For each RUC-committed Resource that has submitted an Energy Offer Curve, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL (if more than highest MW in Energy Offer Curve) | Greater of $250 or price associated with the highest MW in QSE submitted Energy Offer Curve |
| Energy Offer Curve | Greater of $250 or the QSE submitted Energy Offer Curve |
| Zero | Greater of $250 or the first price point of the QSE submitted Energy Offer Curve |

[NPRR930: Insert paragraph (iii) below upon system implementation and renumber accordingly:] (iii) For each RUC-committed Resource during the time period stated in the Advance Action Notice (AAN) if any Resource received an Outage Schedule Adjustment, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL; $4,500 or the effective Value of Lost Load (VOLL), whichever is less.. Zero; $4,500 or the effective VOLL, whichever is less..

(iii) 	For each Combined Cycle Generation Resource that was RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity, as instructed by ERCOT, that has not submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL of RUC-committed configuration | $250 |
| Zero | $250 |

(iv)	For each Combined Cycle Generation Resource that was RUC-committed from one On-Line configuration in order to transition to a different configuration with additional capacity, as instructed by ERCOT, that has submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a monotonically non-decreasing proxy Energy Offer Curve as described below:

| MW | Price (per MWh) |
| --- | --- |
| HSL of RUC-committed configuration (if more than highest MW in Energy Offer Curve) | Greater of $250 or price associated with the highest MW in QSE submitted Energy Offer Curve |
| Energy Offer Curve for MW at and above HSL of QSE-committed configuration | Greater of $250 or the QSE submitted Energy Offer Curve |
| HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve and price associated with highest MW in Energy Offer Curve is less than $250) | $250 |
| HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve) | Price associated with the highest MW in QSE submitted Energy Offer Curve |
| Energy Offer Curve for MW at and below HSL of QSE-committed configuration | The QSE submitted Energy Offer Curve |
| 1 MW below lowest MW in Energy Offer Curve (if more than LSL) | -$249.99 |
| LSL (if less than lowest MW in Energy Offer Curve) | -$250.00 |

[NPRR1019: Insert paragraphs (v)-(viii) below upon system implementation:] (v) For each RUC-committed Switchable Generation Resource (SWGR) that is not part of a Combined Cycle Train already operating in ERCOT, that has not submitted an Energy Offer Curve, and that has a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of the RUC instruction, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL; $4,500 or the effective Value of Lost Load (VOLL), whichever is less. Zero; $4,500 or the effective VOLL, whichever is less. (vi) For each RUC-committed SWGR that is not part of a Combined Cycle Train already operating in ERCOT, that has submitted an Energy Offer Curve, and that has a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of the RUC instruction, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL (if more than highest MW in Energy Offer Curve); Greater of: $4,500 or the effective VOLL, whichever is less; and the price associated with the highest MW in QSE-submitted Energy Offer Curve. Energy Offer Curve; Greater of: $4,500 or the effective VOLL, whichever is less; and the QSE-submitted Energy Offer Curve. Zero; Greater of: $4,500 or the effective VOLL, whichever is less; and the first price point of the QSE-submitted Energy Offer Curve. (vii) For each Combined Cycle Train configuration that includes at least one SWGR that is operating in a non-ERCOT Control Area as part of a configuration with a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of a RUC instruction requiring the switching of the SWGR into the ERCOT Control Area, if the QSE for the Combined Cycle Train has not submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL of RUC-committed configuration; $4,500 or the effective VOLL, whichever is less. Zero; $4,500 or the effective VOLL, whichever is less. (viii) For each Combined Cycle Train configuration that includes at least one SWGR that is operating in a non-ERCOT Control Area as part of a configuration with a COP Resource Status of EMRSWGR for the instructed Operating Hour at the time of a RUC instruction requiring the switching of the SWGR into the ERCOT Control Area, if the QSE for the Combined Cycle Train has submitted an Energy Offer Curve for the RUC-committed configuration, ERCOT shall create a proxy Energy Offer Curve as described below: MW; Price (per MWh). HSL of RUC-committed configuration (if more than highest MW in Energy Offer Curve); Greater of: $4,500 or the effective VOLL, whichever is less; and the price associated with the highest MW in QSE-submitted Energy Offer Curve. Energy Offer Curve for MW at and above HSL of QSE-committed configuration; Greater of: $4,500 or the effective VOLL, whichever is less; and the QSE-submitted Energy Offer Curve. HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve and price associated with highest MW in Energy Offer Curve is less than $4,500); $4,500 or the effective VOLL, whichever is less. HSL of QSE-committed configuration (if more than highest MW in Energy Offer Curve); Price associated with the highest MW in QSE-submitted Energy Offer Curve. Energy Offer Curve for MW at and below HSL of QSE-committed configuration; The QSE-submitted Energy Offer Curve. 1 MW below lowest MW in Energy Offer Curve (if more than LSL); -$249.99. LSL (if less than lowest MW in Energy Offer Curve); -$250.00.

(5)	For use as SCED inputs for determining energy dispatch and Ancillary Service awards, ERCOT shall use the available Ancillary Service MW capacity of all Resources by creating a proxy Ancillary Service Offer for qualified Resources as follows:

(a)	The proxy Ancillary Service Offer shall be a linked Ancillary Service Offer across all Ancillary Service products for which a Resource is qualified to provide.  For Generation Resources, the proxy Ancillary Service Offer MW shall be equal to the Resource’s telemetered HSL.  For ESRs, the proxy Ancillary Service Offer MW shall be equal to the difference between the Resource’s telemetered HSL and LSL.  For Load Resources, the proxy Ancillary Service Offer MW shall be equal to the Resource’s telemetered Maximum Power Consumption (MPC).

(b)	For Resources that are not RUC-committed, the price in the proxy Ancillary Service Offer shall be set to:

(i)	For Reg-Up and RRS, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for Reg-Up or RRS, respectively;

(B)	The Resource’s highest submitted Ancillary Service Offer price for Reg-Up or RRS, respectively;

(C)	The Resource’s highest Ancillary Service Offer price for ECRS (submitted or proxy); or

(D)	The Resource’s highest Ancillary Service Offer price for Non-Spin (submitted or proxy).

(ii)	For ECRS, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for ECRS;

(B)	The Resource’s highest submitted Ancillary Service Offer price for ECRS; or

(C)	The Resource’s highest Ancillary Service Offer price for Non-Spin (submitted or proxy).

(iii)	For Non-Spin, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for Non-Spin; or

(B)	The Resource’s highest submitted Ancillary Service Offer price for Non-Spin.

(iv)	For Reg-Down, the maximum of:

(A)	The proxy Ancillary Service Offer price floor for Reg-Down; or

(B)	The Resource’s highest submitted Ancillary Service Offer price for Reg-Down.

(c)	The proxy Ancillary Service Offer price floors for each SCED-interval shall be derived from the effective ASDCs and Ancillary Service Plan using the following logic:

(i)        The proxy Ancillary Service Offer price floor for Reg-Up is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for Reg-Up that intersects with a quantity that is 95% of the Ancillary Service Plan for Reg-Up.

(ii)       The proxy Ancillary Service Offer price floor for RRS is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for RRS that intersects with a quantity that is 95% of the Ancillary Service Plan for RRS.

(iii)      The proxy Ancillary Service Offer price floor for ECRS is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for ECRS that intersects with a quantity that is 95% of the Ancillary Service Plan for ECRS.

(iv)      The proxy Ancillary Service Offer price floor for Non-Spin is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for Non-Spin that intersects with a quantity that is 95% of the Ancillary Service Plan for Non-Spin.

(v)       The proxy Ancillary Service Offer price floor for Reg-Down is equal to the lesser of the values below minus $0.01 per MW per hour:

(A)      $2,000 per MW per hour; or

(B)      The point on the ASDC for Reg-Down that intersects with a quantity that is 95% of the Ancillary Service Plan for Reg-Down.

(d)	ERCOT systems shall be designed to allow for proxy Ancillary Service Offer price floors to differ when the same Ancillary Service product can be provided by either On-Line or Off-Line Resources, and/or an Ancillary Service product has sub-types.

(e)	For RUC-committed Resources:

(i)	If a RUC-committed Resource does not have an Ancillary Service Offer for an Ancillary Service product that the Resource is qualified to provide, ERCOT shall create an Ancillary Service Offer for that Ancillary Service product at a value of $250 per MWh for the full operating range of the Resource up to its telemetered HSL.

(ii)	For each Ancillary Service product for which a RUC-committed Resource has an Ancillary Service Offer, the Ancillary Service Offer used by SCED for that Ancillary Service product across the full operating range of the Resource up to its telemetered HSL shall be the maximum of:

(A)	The Resource’s highest submitted Ancillary Service Offer price; or

(B)	$250 per MWh.

(6)	For use as SCED inputs for determining energy Dispatch and Ancillary Service awards, ERCOT shall use the available capacity of all On-Line ESRs by creating proxy Energy Bid/Offer Curves for certain Resources as follows:

(a)	For each ESR for which its QSE has submitted an Energy Bid/Offer Curve that does not cover the full offer range (LSL to HSL) of the Resource’s available capacity, ERCOT shall create a proxy Energy Bid/Offer Curve that extends the submitted Energy Bid/Offer Curve to use the entire available capacity of the Resource above the highest MW point on the Energy Bid/Offer Curve to the Resource’s HSL and from the lowest MW point on the Energy Bid/Offer Curve to LSL, using these prices for the corresponding MW segments:

| Scenario | MW Segment | Price (per MWh) |
| --- | --- | --- |
| HSL MW and the highest MW point on the Energy Bid/Offer are both greater than or equal to zero, and, HSL is greater than the highest MW in submitted Energy Bid/Offer Curve | From highest MW point on submitted Energy Bid/Offer Curve to HSL MW | RTSWCAP |
| HSL MW is greater than or equal to zero, and, the highest MW point on the Energy Bid/Offer is less than zero | From highest MW point on submitted Energy Bid/Offer Curve to 0 MW From 0 MW to HSL | Price associated with the highest MW in submitted Energy Bid/Offer Curve RTSWCAP |
| HSL is less than zero and is also greater than the highest MW in submitted Energy Bid/Offer Curve | From highest MW point on submitted Energy Bid/Offer Curve to HSL MW | Price associated with the highest MW in submitted Energy Bid/Offer Curve |
| Energy Bid/Offer Curve |  | Energy Bid/Offer Curve |
| LSL MW and the lowest MW point on the Energy Bid/Offer Curve are both greater than or equal to zero, and, LSL is less than the lowest MW in submitted Energy Bid/Offer Curve | From LSL to lowest MW point on submitted Energy Bid/Offer Curve | Price associated with the lowest MW in submitted Energy Bid/Offer Curve |
| LSL MW is less than zero, and, the lowest MW point on the Energy Bid/Offer Curve is greater than zero | From LSL to 0 MW From 0 MW to lowest MW point on submitted Energy Bid/Offer Curve | -$250.00 Price associated with the lowest MW in submitted Energy Bid/Offer Curve |
| LSL and the lowest MW point on the Energy Bid/Offer Curve are both less than or equal to zero, and, LSL is less than the lowest MW point on the Energy Bid/Offer Curve | From LSL to lowest MW point on submitted Energy Bid/Offer Curve | -$250.00 |

(b)	At the time of SCED execution, if a valid Energy Bid/Offer Curve or Output Schedule does not exist for an ESR that has a status of On-Line, then ERCOT shall notify the QSE and create a proxy Energy Bid/Offer Curve priced at -$250/MWh for the MW portion of the curve less than zero MW, and priced at the RTSWCAP for the MW portion of the curve greater than zero MW.

(c)	At the time of SCED execution, if a QSE representing an ESR has submitted an Output Schedule instead of an Energy Bid/Offer Curve, ERCOT shall create a proxy Energy Bid/Offer Curve priced at -$250 per MWh for the MW portion of the curve from its LSL to the MW amount on the Output Schedule, and priced at the RTSWCAP for the MW portion of the curve from the MW amount on the Output Schedule to its HSL.

(7)	The Entity with decision-making authority, as more fully described in Section 3.19.1, Constraint Competitiveness Test Definitions, over how a Resource or Split Generation Resource is offered or scheduled, shall be responsible for all offers associated with each Resource, including offers represented by a proxy Energy Offer Curve, proxy Energy Bid/Offer Curve, or proxy Ancillary Service Offer.

(8)	For a CLR whose QSE has submitted an RTM Energy Bid that does not cover the full range of the Resource’s available Demand response capability, consistent with the CLR’s telemetered quantities, ERCOT shall create a proxy energy bid as described below:

| MW | Price (per MWh) |
| --- | --- |
| LPC to MPC minus maximum MW of RTM Energy Bid | Price associated with the lowest MW in submitted RTM Energy Bid curve |
| MPC minus maximum MW of RTM Energy Bid to MPC | RTM Energy Bid curve |
| MPC | Right-most point (lowest price) on RTM Energy Bid curve |

[NPRR1188: Replace paragraph (8) above with the following upon system implementation and renumber accordingly:] (8) For a CLR whose QSE has submitted an Energy Bid Curve that does not cover the full range of the Resource’s available Demand response capability, consistent with the CLR’s telemetered quantities, ERCOT shall create a proxy energy bid as described below: MW; Price (per MWh). LPC to MPC minus maximum MW of Energy Bid Curve; Price associated with the lowest MW in submitted Energy Bid Curve. MPC minus maximum MW of Energy Bid Curve to MPC; Energy Bid Curve. MPC; Right-most point (lowest price) on Energy Bid Curve. (9) For a CLR whose QSE has not submitted an Energy Bid Curve, consistent with the CLR’s telemetered quantities, ERCOT shall create a proxy Energy Bid Curve as described below: MW; Price (per MWh). LPC to MPC; Effective Value of Lost Load (VOLL).

(9)	ERCOT shall ensure that any RTM Energy Bid is monotonically non-increasing.  The QSE representing the CLR shall be responsible for all RTM Energy Bids, including bids updated by ERCOT as described above.

[NPRR1188: Replace paragraph (9) above with the following upon system implementation:] (9) ERCOT shall ensure that any Energy Bid Curve is monotonically non-increasing. The QSE representing the CLR shall be responsible for all Energy Bid Curves, including Energy Bid Curves updated by ERCOT as described above.

(10)	If a CLR telemeters a status of OUTL, it is not considered as dispatchable capacity by SCED.  A QSE may use this function to inform ERCOT of instances when the CLR is unable to follow SCED Dispatch Instructions.  Under all telemetered statuses, including OUTL, the remaining telemetry quantities submitted by the QSE shall represent the operating conditions of the CLR that can be verified by ERCOT.  A QSE representing a CLR with a telemetered status of OUTL is still obligated to provide any applicable Ancillary Services awarded to the Resource.  This paragraph does not apply to ESRs.

[NPRR1188: Replace paragraph (10) above with the following upon system implementation:] (10) A CLR may consume energy only when dispatched by SCED to do so. A CLR may telemeter a status of OUTL only if the Resource is Off-Line and unavailable with its energy consumption at zero. In instances when the CLR is unable to follow SCED Dispatch Instructions but still consumes energy, the CLR must submit a Resource Status of ONHOLD. Under all telemetered statuses, including OUTL, the remaining telemetry quantities submitted by the QSE shall represent the operating conditions of the CLR that can be verified by ERCOT. A QSE representing a CLR with a telemetered status of OUTL or ONHOLD is still obligated to provide any applicable Ancillary Services awarded to the Resource. This paragraph does not apply to ESRs.

(11)	Energy Offer Curves that were constructed in whole or in part with proxy Energy Offer Curves shall be so marked in all ERCOT postings or references to the energy offer.

(12)	SCED will enforce Resource-specific Ancillary Service constraints to ensure that Ancillary Service awards are aligned with a Resource’s qualifications and telemetered Ancillary Service capabilities.

(a)	A scaling factor of 5/7 shall be used for Reg-Up award when ensuring that the SCED Base Point plus the product of this scaling factor and the Reg-Up award does not exceed HDL.

(b)	A scaling factor of 5/7 shall be used for Reg-Down award when ensuring that the SCED Base Point minus the product of this scaling factor and the Reg-Down award does not go below LDL.

(13)	Energy Bid/Offer Curves that were constructed in whole or in part with proxy Energy Bid/Offer Curves shall be so marked in all ERCOT postings or references to the energy bid/offer.

(14)	The two-step SCED methodology referenced in paragraph (1) above is:

(a)	The first step is to execute the SCED process to determine Reference LMPs.  In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints in addition to power balance and Ancillary Service constraints.  Energy Offer Curves for all On-Line Generation Resources, Energy Bid/Offer Curves for all On-Line ESRs, and RTM Energy Bids from available CLRs, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

[NPRR1188: Replace paragraph (a) above with the following upon system implementation:] (a) The first step is to execute the SCED process to determine Reference LMPs. In this step, ERCOT executes SCED using the full Network Operations Model while only observing limits of Competitive Constraints in addition to power balance and Ancillary Service constraints. Energy Offer Curves for all On-Line Generation Resources, Energy Bid/Offer Curves for all On-Line ESRs, and Energy Bid Curves from available CLRs, whether submitted by QSEs or created by ERCOT under this Section, are used in the SCED to determine “Reference LMPs.”

(b)	The second step is to execute the SCED process to produce Base Points, Ancillary Service awards, Shadow Prices, Real-Time MCPCs, and LMPs, subject to security constraints (including Competitive and Non-Competitive Constraints) and other Resource constraints.  The second step must:

(i)	Use Energy Offer Curves for all On-Line Generation Resources, whether submitted by QSEs or created by ERCOT.  Each Energy Offer Curve must be bounded at the lesser of the Reference LMP (from Step 1) or the appropriate Mitigated Offer Floor.  In addition, each Energy Offer Curve subject to mitigation under the criteria described in Section 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test, must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node plus a variable not to exceed 0.01 multiplied by the value of the Resource’s Mitigated Offer Cap (MOC) curve at the LSL or the appropriate MOC;

(ii)	Use Energy Bid/Offer Curves for all On-Line ESRs, whether submitted by QSEs or created by ERCOT.  Each Energy Bid/Offer Curve must be bounded at the lesser of the Reference LMP (from Step 1) or the appropriate Mitigated Offer Floor.  The offer portion of each Energy Bid/Offer Curve subject to mitigation under the criteria described in Section 3.19.4, Security-Constrained Economic Dispatch Constraint Competitiveness Test, must be capped at the greater of the Reference LMP (from Step 1) at the Resource Node plus a variable not to exceed 0.01 multiplied by the value of the Resource’s MOC curve at the LSL or the appropriate MOC;

(iii)	Use RTM Energy Bids for all available CLRs, whether submitted by QSEs or created by ERCOT.  There is no mitigation of RTM Energy Bids.  An RTM Energy Bid from a CLR represents the bid for energy distributed across all nodes in the Load Zone in which the CLR is located.  For an ESR, an RTM Energy Bid represents a bid for energy at the ESR’s Resource Node;

[NPRR1188: Replace paragraph (iii) above with the following upon system implementation:] (iii) Use Energy Bid Curves for all available CLRs, whether submitted by QSEs or created by ERCOT. There is no mitigation of Energy Bid Curves. An Energy Bid Curve from an Aggregate Load Resource (ALR) represents the bid for energy distributed across all nodes in the Load Zone in which the ALR is located. For an ESR or a CLR that is not an ALR, an Energy Bid Curve represents a bid for energy at the applicable Resource Node;

(iv)	Use Energy Bid Curves for all available PCLRs, whether submitted by QSEs or ERCOT, including Adjusted Bid Caps as described in Section 4.4.9.4.4, Adjusted Bid Caps.

(v)	Observe all Competitive and Non-Competitive Constraints; and

(vi)	Use Ancillary Service Offers to determine Ancillary Service awards.

(c)	ERCOT shall archive information and provide monthly summaries of security violations and any binding transmission constraints identified in Step 2 of the SCED process.  The summary must describe the limiting element (or identified operator-entered constraint with operator’s comments describing the reason and the Resource-specific impacts for any manual overrides).  ERCOT shall provide the summary to Market Participants on the MIS Secure Area and to the Independent Market Monitor (IMM).

(d)	The System Lambda used to determine LMPs and the Real-Time MCPCs from SCED Step 2 shall be capped at the effective VOLL.  If the following conditions are met for a SCED interval in which the SCED Step 2 System Lambda was capped, a QSE may be eligible for compensation by submitting a Settlement and billing dispute pursuant to paragraph (5) of Section 6.6.9, Emergency Operations Settlement:

(i) 	A Generation Resource or ESR for the QSE received a Base Point greater than the Resource’s LDL for that SCED interval; and

(ii) 	The LMP at the Resource is less than the price on the Resource’s Energy Offer Curve or Energy Bid/Offer Curve, as applicable, with any Resource’s Energy Offer Curve or Energy Bid/Offer Curve capped by the MOC.

[NPRR1290 and NPRR1323: Replace applicable portions of paragraph (d) above with the following upon system implementation:] (d) Any Electrical Bus LMP above the effective VOLL shall be set equal to the greater of the effective VOLL or the initial LMP minus the positive difference between System Lambda and the effective VOLL. All other Electrical Bus LMPs below the effective VOLL remain unchanged. These adjustments shall be applied to Electrical Bus LMPs prior to calculating Real-Time Settlement Point LMPs, Real-Time Settlement Point Prices, and Real-Time prices for energy metered. The System Lambda and the Real-Time MCPCs from SCED Step 2 shall also be capped at the effective VOLL. ERCOT shall post both the capped and uncapped values for Electrical Bus LMPs, System Lambda, and Real-Time MCPCs to the ERCOT website.

(15)	For each SCED process, in addition to the binding Base Points, Ancillary Service awards, Real-Time MCPCs, and LMPs, ERCOT shall calculate a non-binding projection of the Base Points, Ancillary Service awards, MCPCs, Resource Node LMPs, Real-Time Reliability Deployment Price Adders, Hub LMPs, and Load Zone LMPs at a frequency of every five minutes for at least 15 minutes into the future based on the same inputs to the SCED process as described in this Section, except that the Resource’s HDL and LDL and the total generation requirement will be as estimated at future intervals.  The Resource’s HDL and LDL will be calculated for each interval of the projection based on the ramp rate capability over the study period.  ERCOT shall estimate the projected total generation requirement by calculating a Load forecast for the study period.  In lieu of the steps described in Section 6.5.7.3.1, Determination of Real-Time Reliability Deployment Price Adders, the non-binding projection of Real-Time Reliability Deployment Price Adders shall be estimated based on GTBD, reliability deployments MWs, and aggregated offers.  The Energy Offer Curve and Energy Bid/Offer Curves from SCED Step 2, the virtual offers for Load Resources deployed and the power balance penalty price will be compared against the updated GTBD to get an estimate of the System Lambda from paragraph (2)(m) of Section 6.5.7.3.1.  ERCOT shall post the projected non-binding Base Points and Ancillary Service awards for each Resource for each interval study period on the MIS Certified Area and the projected non-binding LMPs for Resource Nodes, Real-Time MCPCs, Real-Time Reliability Deployment Price Adders, Hub LMPs and Load Zone LMPs on the ERCOT website pursuant to Section 6.3.2, Activities for Real-Time Operations.

(16)	ERCOT may override one or more of a CLR’s parameters in SCED if ERCOT determines that the CLR’s participation is having an adverse impact on the reliability of the ERCOT System.

(17)	The QSE representing an ESR may withdraw energy from the ERCOT System only when dispatched by SCED to do so.  An ESR may telemeter a status of OUT only if the ESR is in Outage status.

#### 6.5.7.11	Provisional Controllable Load Resource (PCLR) Ramp Rate Requirements

(1)	Each Provisional Controllable Load Resource (PCLR) shall provide a down ramp rate that is limited to at least 10% per minute and no greater than 20% per minute of the difference between its Maximum Power Consumption (MPC) and its Low Power Consumption (LPC) at all times when consuming energy.

(2)	Each PCLR shall provide an up ramp rate that is limited to less than or equal to 20% per minute of the difference between its MPC and its LPC at all times when consuming energy.

(3)	PCLRs are exempt from the requirements of paragraphs (1) and (2) above upon receipt of a valid Dispatch Instruction from ERCOT to deviate from the applicable ramp rate limitation when necessary to protect ERCOT System reliability.

(4)	ERCOT shall calculate monthly the number of eligible one-minute intervals where the average up and down ramp rates telemetered by the PCLR met the requirements of paragraphs (1) and (2) above.  PCLRs are compliant with the ramp rate requirements of this Section when the number of eligible one-minute intervals calculated by ERCOT is equal to or greater than 90% of the eligible one-minute intervals in the calendar month.  Intervals where paragraph (3) above applies shall be excluded as eligible intervals for this performance metric.  ERCOT shall initiate a review process with the QSE and Resource Entity for the PCLR in cases where the PCLR’s monthly score is less than 90%.  ERCOT may revoke a PCLR’s qualification in the event of multiple instances of noncompliance.

16.5.5	Provisional Controllable Load Resources (PCLRs)

(1)	A Provisional Controllable Load Resource (PCLR) shall maintain its registration as a PCLR until the earlier of either the end date determined by ERCOT and set forth in Part B of Section 23, Form W, Declaration of Intent and Commitment to Register as a PCLR, or the date that ERCOT approves the Resource Entity election to change the Large Load’s registration status.  After the end date, the PCLR’s existing registration shall remain unchanged until the Resource Entity elects to modify the Resource registration for the Large Load.  A PCLR may convert to another type of Load Resource or to firm Load.

(2)	A Resource Entity that desires to de-register a Large Load as a PCLR prior to its end date shall provide ERCOT no less than 120 days’ written notice.  Upon de-registration, the Large Load shall be limited to the maximum Low Power Consumption (LPC) amounts identified in the Batch Zero Interconnection Study and documented in the Load Commissioning Plan until new interconnection studies have been performed according to applicable ERCOT Protocols and Guides.

(3)	If ownership of a Large Load registered as a PCLR is transferred to another Entity, the acquiring Entity shall be bound by all obligations applicable to the PCLR under these Protocols and Guides.  The acquiring Entity shall notify ERCOT and submit an updated, signed, and notarized Part A or Part B of Section 23, Form W, whichever is applicable, within ten days of the transfer.

ERCOT NODAL PROTOCOLS

SECTION 23

Form W: Declaration of Intent and Commitment to Register as a Provisional Controllable Load Resource

[Date]

Provisional Controllable Load Resource for Batch Zero Interconnection Study Form

An Interconnecting Large Load Entity (ILLE) with a Large Load interconnection request eligible for study in the Batch Zero Process may elect for the Large Load to be evaluated as a Provisional Controllable Load Resource (PCLR) as described in ERCOT Planning Guide Section 9.2.2.1.  A PCLR is a Controllable Load Resource (CLR) that may be approved to energize above the maximum Low Power Consumption determined in the Batch Zero Interconnection Study so long as it remains registered and qualified as a CLR.  The PCLR must remain so registered until a defined end date determined during the Batch Zero Interconnection Study or until it deregisters as provided in Section 16.5.5.  The Low Power Consumption (LPC) of the PCLR must never exceed the maximum LPC determined in the Batch Zero Interconnection Study as set forth in the LCP provided to the ILLE per Planning Guide Section 9.4.1.  PCLRs are not eligible to qualify to provide Ancillary Services to the ERCOT System.

For an ILLE to elect for its designated Large Load to be evaluated as a PCLR in the Batch Zero Interconnection Study defined in Section 9 of the ERCOT Planning Guide, the ILLE’s representative, official, officer, or other authorized person with binding authority over the ILLE must complete Part A of this Form and sign, notarize, and deliver it to the applicable Interconnecting Distribution Service Provider (DSP) or Interconnecting Transmission Service Provider (TSP) on or before July 10, 2026.  The Interconnecting DSP or Interconnecting TSP must provide the completed, signed, and notarized Part A of the Form to ERCOT on or before July 24, 2026.  ERCOT may request additional information as reasonably necessary to support operations under the ERCOT Protocols and Planning Guide.

Following the report summarizing the results of the Batch Zero Interconnection Study, if the ILLE executes an interconnection agreement during the commitment period and still intends for the designated Large Load to be a PCLR, the ILLE must sign, notarize, and deliver Part B to the applicable Interconnecting DSP or Interconnecting TSP and the Interconnecting DSP or Interconnecting TSP must submit it to ERCOT by the commitment deadline in paragraph (2)(c) of Planning Guide Section 9.3.1.

PART A:

Part A ILLE Obligations

By signing and notarizing Part A of this Form, the ILLE identified below confirms the following:

1.	The ILLE declares its intent to register the Large Load designated in Part A below as a PCLR.  The ILLE understands that, by completing Part A of this Form, ERCOT will evaluate the designated Large Load as a PCLR in the Batch Zero Interconnection Study as described in Planning Guide Section 9.3.2.1.  The ILLE must complete, notarize, and sign Part B of this Form after the Batch Zero Interconnection Study.  If ERCOT does not receive the signed and notarized Form W Part B by the date specified in paragraph (2)(c) of Planning Guide Section 9.3.1, ERCOT will exclude that Large Load from the Batch Zero Refinement Study described in Planning Guide Section 9.5.

2.	The ILLE understands it must register the designated Large Load as a PCLR to be approved to energize above the maximum LPC amounts determined in the Batch Zero Interconnection Study if the ILLE executed an interconnection agreement according to paragraph (2) of Planning Guide Section 9.4.

3.	The ILLE understands the steps to register its designated Large Load as a PCLR, which follows the same steps as registering a CLR. The ILLE agrees to comply with all obligations placed on the owner of a CLR and any additional obligations applicable to a PCLR under the ERCOT Protocols or Guides.  These obligations include, but are not limited to, registering with ERCOT as a Resource Entity, registering in the Resource Integration and Ongoing Operations (RIOO) system as a Load Resource, designating a Qualified Scheduling Entity (QSE), and complying with Security-Constrained Economic Dispatch (SCED) Base Point Dispatch Instructions when consuming energy.

4.	The ILLE understands ERCOT does not guarantee that the desired Maximum Power Consumption (MPC) amount can ever be served in Real-Time.  SCED will issue a Base Point corresponding to the amount of load that can be served in Real-Time and the PCLR is obligated to follow that Base Point instruction.

5.	The ILLE understands the ramp rate requirements documented in Section 6.5.7.11 and affirms the designated Large Load will be capable of compliance with these requirements.

6.	The designated Large Load shall remain a PCLR until its ERCOT-determined end date set forth in Part B or the date that ERCOT approves the Resource Entity election to change the Large Load’s registration status as set forth in Section 16.5.5, Provisional Controllable Load Resources (PCLRs).  Once the end date has been reached, the existing registration as a PCLR will remain unchanged unless and until the Resource Entity elects to modify its Resource registration for the Large Load.  The PCLR can convert to another type of Load Resource or become a firm Load.

7.	If the ILLE desires to de-register the Large Load as a PCLR prior to the end date, it may do so with 120 days’ notice to ERCOT as set forth in Section 16.5.5.  Once de-registered as a PCLR, Large Load will be limited to the maximum LPC amounts identified in the Batch Zero Interconnection study and documented in the Load Commissioning Plan (LCP) until new interconnection studies have been performed according to applicable ERCOT Protocols and Guides.

8.	The ILLE understands that failure to comply with Real-Time ERCOT Dispatch and operating instructions may result in the revocation of PCLR status, especially after multiple violations, in addition to any other charges or penalties defined in the ERCOT Protocols and Guides.  In the event PCLR status is revoked, the Large Load will be limited to the maximum LPC amounts identified in the Batch Zero Interconnection study and documented in the LCP until new interconnection studies have been performed according to applicable ERCOT Protocols and Guides.

9.	If ownership of the Large Load is transferred to another Entity, the Entity acquiring the Large Load shall be bound by these obligations.  In accordance with Section 16.5.5, the acquiring Entity must notify ERCOT and submit an updated, signed, and notarized Part A of this Form within ten days of the transfer.

Part A Required Information

Interconnecting Large Load Entity Name:

Name of Large Load Facility:

Large Load Number: LLI-

Address of the Large Load Facility:

Part A Notarization

STATE OF _______________

COUNTY OF _____________

Before me, the undersigned authority, this day appeared ___________________, known by me to be the person whose name is subscribed to the foregoing instrument, who, after first being sworn by me deposed and said:

“I am a representative, official, officer, or other authorized person with binding authority over ______________, I am authorized to sign and submit the foregoing Part A of Form W: Declaration of Intent to Register as a Provisional Controllable Load Resource on behalf of _____________, and the statements contained in such Form are true and correct.”

SWORN TO AND SUBSCRIBED TO BEFORE ME, the undersigned authority on this the _____ day of ____________, 20__.

______________________________

Notary Public, State of ___________

My Commission expires __________

PART B

ERCOT will determine the ILLE’s end date and maximum LPC value by Year in the Batch Zero Interconnection Study and input the numerical fields below.  If the ILLE executes an interconnection agreement, according to paragraph (2) of Planning Guide Section 9.4, and still intends for the designated Large Load to be a PCLR, the ILLE must sign and notarize Part B and return it to the ILLE’s Interconnecting DSP or Interconnecting TSP and the Interconnecting DSP or Interconnecting TSP must submit it to ERCOT on or before the deadline established by paragraph (2)(c) of Planning Guide Section 9.3.1.

Part B Required Information Provided by ERCOT:

Permitted end date:

Part B ILLE Election and Obligations:

The ILLE has reviewed the results and LCP provided by ERCOT per Planning Guide Section 9.4 and makes the following election:

A. The ILLE affirms its intent to register as a PCLR and will accept the LPC and MPC amounts communicated in the LCP with no modifications.

B. The ILLE affirms its intent to register as a PCLR with modifications to the LPC and/or MPC values communicated in the LCP.  The ILLE understands these modified values must be less than or equal to the values communicated by ERCOT.

C. The ILLE withdraws its intent to register as a PCLR but will accept the LPC values communicated in the LCP as allocated peak Demand with no modifications.

D. The ILLE withdraws its intent to register as a PCLR but will accept the LPC values communicated in the LCP as allocated peak Demand with modifications.  The ILLE understands these modified values must be less than or equal to the values communicated by ERCOT.

If option A or B is selected above and the ILLE executed an interconnection agreement according to paragraph (2) of Planning Guide Section 9.4, the ILLE further confirms the following by signing and notarizing Part B of this Form:

1.	By completing Part B of this Form, the ILLE commits to registering the designated Large Load as a PCLR.

2.	The ILLE understands it must register the designated Large Load as a PCLR to be approved to energize above the maximum LPC amounts determined in the Batch Zero Interconnection Study.

3.	The ILLE understands the steps to register its designated Large Load as a PCLR, which follows the same steps as registering a CLR.  The ILLE agrees to comply with all obligations placed on the owner of a CLR and any additional obligations applicable to a PCLR under the ERCOT Protocols or Guides.  These obligations include, but are not limited to, registering with ERCOT as a Resource Entity, registering in Resource Integration and Ongoing Operations (RIOO) system as a Load Resource, designating a Qualified Scheduling Entity (QSE), and complying with Security-Constrained Economic Dispatch (SCED) Base Point Dispatch Instructions when consuming energy.

4.	The ILLE understands ERCOT does not guarantee that the desired MPC amount can ever be served in Real-Time.  SCED will issue a Base Point corresponding to the amount of Load that can be served in Real-Time and the PCLR is obligated to follow that Base Point instruction.

5.	The ILLE understands the ramp rate requirements documented in Section 6.5.7.11 and affirms the designated Large Load will be capable of compliance with these requirements.

6.	The designated Large Load shall remain a PCLR until the ERCOT-determined end date set forth in Part B or the date that ERCOT approves the Resource Entity election to change the Large Load’s registration status as set forth in Protocol Section 16.5.5.  Once the end date has been reached, the existing registration as a PCLR will remain unchanged unless and until the Resource Entity elects to modify its Resource registration for the Large Load.  The PCLR can convert to another type of Load Resource or become a firm Load.

7.	If the ILLE desires to de-register the Large Load as a PCLR prior to the end date, it may do so with 120 days’ notice to ERCOT as set forth in Section 16.5.5.  Once de-registered as a PCLR, Large Load will be limited to the maximum LPC amounts identified in the Batch Zero Interconnection study and documented in the LCP until new interconnection studies have been performed according to applicable ERCOT Protocols and Guides.

8.	The ILLE understands that failure to comply with Real-Time ERCOT Dispatch and operating instructions may result in the revocation of PCLR status, especially after multiple violations, in addition to any other charges or penalties defined in the ERCOT Protocols and Guides.  In the event PCLR status is revoked, the Large Load will be limited to the maximum LPC amounts identified in the Batch Zero Interconnection study and documented in the LCP until new interconnection studies have been performed according to applicable ERCOT Protocols and Guides.

9.	If ownership of the Large Load is transferred to another Entity, the Entity acquiring the Large Load shall be bound by these obligations.  In accordance with Section 16.5.5, the acquiring Entity must notify ERCOT and submit an updated signed and notarized Part B of this Form within ten days of the transfer.

Part B Notarization

STATE OF _______________

COUNTY OF _____________

Before me, the undersigned authority, this day appeared ___________________, known by me to be the person whose name is subscribed to the foregoing instrument, who, after first being sworn by me deposed and said:

“I am a representative, official, officer, or other authorized person with binding authority over ______________, I am authorized to sign and submit the foregoing Part B of Form W: Provisional Controllable Load Resource for Batch Zero Interconnection Study on behalf of ______________, and the statements contained in such Form are true and correct.”

SWORN TO AND SUBSCRIBED TO BEFORE ME, the undersigned authority on this the _____ day of ____________, 20__.

______________________________

Notary Public, State of ___________

My Commission expires __________

ERCOT Nodal Protocols

Section 23

Form X: Withdrawal-Limited Private Use Network Designation

[Date]

Withdrawal-limited private use network Designation

For a Large Load and a generator to be studied and operated as a Withdrawal-Limited Private Use Network (WLPUN) in the Batch Zero Process, the Interconnecting Large Load Entity (ILLE) for the designated Large Load and the Interconnecting Entity (IE) or Resource Entity for the generator must complete the fields below and execute this Form.

If there is more than one Large Load or other load behind the same Point of Interconnection (POI), the ILLE for each additional Large Load or the Customer associated with each such load must be listed below and must execute this Form.

The signer for the ILLE, Customer, or IE designated below must be a representative, official, officer, or other authorized person with binding authority over the ILLE, Customer, or IE.

The signer for the RE must be the Authorized Representative, Backup Authorized Representative, or authorized person with binding authority over the Resource Entity.

The completed and executed Form must be delivered to the applicable Interconnecting Distribution Service Provider (DSP) or Interconnecting Transmission Service Provider (TSP) by July 10, 2026.

If ownership of the Large Load or generator is transferred to another Entity, the Entity acquiring the Large Load shall be bound by the obligations applicable to a WLPUN.  In accordance with Section 3.10.7.3.1, Withdrawal-Limited Private Use Network, the acquiring Entity must notify ERCOT and submit an updated signed and notarized Form within ten days of the transfer.

Large Load Information

Interconnecting Large Load Entity Name:

Name of Large Load Facility:

Large Load Number: LLI-

Address of the Large Load Facility:

By: ___________________________________
Name: _________________________________
Title: _________________________________
Date: _________________________________

STATE OF _______________

COUNTY OF _____________

Before me, the undersigned authority, this day appeared ___________________, known by me to be the person whose name is subscribed to the foregoing instrument, who, after first being sworn by me deposed and said:

“I am a representative, official, officer, or other authorized person with binding authority over ______________, I am authorized to sign and submit the foregoing Section 23, Form X, Withdrawal-Limited Private Use Network Designation on behalf of _____________, and the statements contained in such Form are true and correct.”

SWORN TO AND SUBSCRIBED TO BEFORE ME, the undersigned authority on this the _____ day of ____________, 20__.

______________________________

Notary Public, State of ___________

My Commission expires __________

Generator Information

Applicable Interconnecting Entity or Resource Entity Name:

GINR Number(s):

By: ___________________________________
Name: _________________________________
Title: _________________________________
Date: _________________________________

STATE OF _______________

COUNTY OF _____________

Before me, the undersigned authority, this day appeared ___________________, known by me to be the person whose name is subscribed to the foregoing instrument, who, after first being sworn by me deposed and said:

“I am a representative, official, officer, or other authorized person with binding authority over ______________, I am authorized to sign and submit the foregoing Section 23, Form X, Withdrawal-Limited Private Use Network Designation, on behalf of ______________, and the statements contained in such Form are true and correct.”

SWORN TO AND SUBSCRIBED TO BEFORE ME, the undersigned authority on this the _____ day of ____________, 20__.

______________________________

Notary Public, State of ___________

My Commission expires __________