May 2026 ERCOT Monthly Operations Report

Reliability and Operations Subcommittee Meeting

July 9, 2026

Table of Contents

1.	Report Highlights	2

2.	Frequency Control	3

2.1.	Frequency Events	3

2.2.	Automatic/Manual Load Resource Deployments	4

3.	Reliability Unit Commitment	5

4.	IRR, Wind, and Solar Generation as a Percent of Load	6

5.	Largest Net-Load Ramps	8

6.	Congestion Analysis	9

6.1.	Notable Constraints	9

6.2.	Generic Transmission Constraint Congestion	22

6.3.	Manual Overrides	22

6.4.	Congestion Costs for Calendar Year 2026	23

7.	System Events	24

7.1.	ERCOT Peak Load	24

7.2.	Load Shed Events	24

7.3.	Stability Events	25

7.4.	Notable PMU Events	25

7.5.	DC Tie Curtailment	25

7.6.	TRE/DOE Reportable Events	25

7.7.	New/Updated Constraint Management Plans	25

7.8.	New/Modified/Removed RAS	25

7.9.	New Procedures/Forms/Operating Bulletins	25

8.	Emergency Conditions	25

8.1.	OCNs	25

8.2.	Advisories	26

8.3.	Watches	26

8.4.	Emergency Notices	26

9.	Application Performance	26

9.1.	TSAT/VSAT Performance Issues	26

9.2.	Communication Issues	26

9.3.	Market System Issues	26

10.	Model Updates	26

Appendix A: Real-Time Constraints	29

# Report Highlights

The unofficial ERCOT peak load for May 2026 was 77,822 MW and occurred on 05/29/2026 during hour ending 17:00, this is 577 MW higher than the May 2025 peak demand of 78,399 MW on 05/23/2025 hour ending 18:00.

On 15-minute settlement interval basis, maximum demand of 78,008 MW occurred on 5/29/2026 during interval ending 16:45 and minimum demand of 40,589 MW occurred on 5/02/2026 during interval ending 03:30.

There were 2 frequency events.

There were no events which required automatic/manual deployment of Load Resources that were controlled by an Under-Frequency Relays.

There were 0 OCNs.

There were 0 Advisories.

1 Watch – A transmission watch was issued due to storm damage in the Rio Grande Valley of Harlingen, Brownsville and South Padre Island area from 5/9/26 15:46 to 5/10/26 23:55.

There were 0 Emergency Notices.

There were 76 RUC commitments.

A PVGR Generation Record of 34,498 MW was set on 05/13/2026 at 12:43.

A PVGR Penetration Record of 72.74% was set on 05/02/2026 at 11:28.

A Wind Generation Record of 29,011 MW was set on 05/18/2026 at 00:01.

A Renewable Generation Record of 47,960 was set on 05/14/2026 at 15:20.

The following GTCs saw congestion in May:

| GTC | Days Congestion |
| --- | --- |
| Nelson Sharpe - Rio Hondo | 21 |
| North Edinburg - Lobo | 19 |
| North - Houston | 18 |
| West Texas | 15 |
| South Texas Export Pawnee - Spruce | 14 |
| East Texas | 13 |
| Wharton | 12 |
| Panhandle | 11 |
| South Texas Export Pawnee - Tango | 11 |
| Valley Export | 9 |
| McCamey | 5 |
| Rio Grande Valley Import | 1 |
| Zapata Starr | 1 |
| South Texas Import Katoen - Lonhill | 1 |

# Frequency Control

## Frequency Events

The ERCOT Interconnection experienced 2 frequency events.

A summary of the frequency events is provided below. The reported frequency events meet one of the following criteria: Delta Frequency is 60 mHz or greater; the MW loss is 350 MW or greater; resource trip event triggered ECRS deployment. Frequency events that have been identified as Frequency Measurable Events (FME) for purposes of BAL-001-TRE-2 analysis are highlighted in blue. When analyzing frequency events, ERCOT evaluates PMU data according to industry standards. Events with an oscillating frequency of less than 1 Hz are inter-area, while higher frequencies indicate local events. Industry standards specify that damping ratio for inter-area oscillations should be 3.0% or greater. For the frequency event listed below, the ERCOT system met these standards and transitioned well after the disturbance. In the case of negative delta frequency, the MW Loss column could refer to load loss.

| Date and Time | Delta Frequency | Max/Min Frequency | Duration of Event | PMU Data |  | MW Loss | Load | IRR | Inertia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (Hz) | (Hz) |  | Oscillation Mode (Hz) | Damping Ratio |  | (MW) | % | (MW-s) |
| 5/20/2026 23:17:35 | 0.063 | 59.948 | 00:04:06 | 0.44 | 16% | 780 | 54,420 | 15% | 289,106 |
| 5/27/2026 0:29:39 | 0.048 | 59.954 | 00:03:32 | 0.77 | 6% | 515 | 51,247 | 13% | 277,347 |

(Note: All data on this graph encompasses frequency event analysis based on BAL-001-TRE-2.)

## Automatic/Manual Load Resource Deployments

| Resource Location | # of Resources | Operating Day | Total # of Hours Committed | Total MWhs | Reason for Commitment | MWs of ECRS Deployed | MWs of RRS Deployed |
| --- | --- | --- | --- | --- | --- | --- | --- |

There were no events which required automatic/manual deployment of Load Resources that were controlled by an Under-Frequency Relays.

# Reliability Unit Commitment

ERCOT reports on Reliability Unit Commitments (RUC) monthly. Commitments are reported grouped by operating day and weather zone. The total number of hours committed is the sum of the hours for all the units in the specified region. Additional information on RUC commitments can be found on the MIS secure site at Grid  Generation  Reliability Unit Commitment.

There were 76 RUC commitments.

| Resource Location | # of Resources | Operating Day | Total # of Hours Committed | Total MWhs | Reason for Commitment |
| --- | --- | --- | --- | --- | --- |
| EAST, NORTH, NORTH_CENTRAL | 6 | May 2, 2026 | 27 | 7,592.0 | Capacity |
| COAST, NORTH_CENTRAL | 6 | May 5, 2026 | 44 | 4,829.5 | Capacity, DPHRCTR5, MKRWDHM5 |
| NORTH_CENTRAL | 1 | May 6, 2026 | 4 | 1,740.0 | Capacity |
| NORTH_CENTRAL | 1 | May 7, 2026 | 4 | 956.0 | Capacity |
| NORTH_CENTRAL | 2 | May 8, 2026 | 8 | 2,696.0 | Capacity |
| NORTH_CENTRAL, SOUTHERN | 7 | May 9, 2026 | 36 | 8,088.0 | Capacity |
| COAST, NORTH_CENTRAL | 2 | May 11, 2026 | 10 | 1,616.3 | Capacity |
| COAST, EAST, NORTH, NORTH_CENTRAL | 14 | May 12, 2026 | 69 | 15,983.2 | Capacity |
| EAST, NORTH_CENTRAL, SOUTH_CENTRAL | 10 | May 13, 2026 | 56 | 11,551.2 | Capacity, DCDHTVW5, DWAP_BI5, XCRD58 |
| NORTH, NORTH_CENTRAL | 3 | May 14, 2026 | 22 | 2,542.7 | MKRWDHM5, SHC2EXC5 |
| COAST, NORTH_CENTRAL | 2 | May 19, 2026 | 8 | 2,416.0 | Capacity |
| NORTH_CENTRAL | 1 | May 20, 2026 | 4 | 1,740.0 | Capacity |
| NORTH_CENTRAL | 1 | May 21, 2026 | 4 | 1,740.0 | Capacity |
| NORTH_CENTRAL | 3 | May 24, 2026 | 12 | 4,264.0 | Capacity |
| NORTH_CENTRAL | 2 | May 25, 2026 | 8 | 2,524.0 | Capacity |
| COAST, EAST, NORTH_CENTRAL | 5 | May 26, 2026 | 32 | 3,754.1 | Capacity |
| NORTH_CENTRAL, SOUTHERN | 5 | May 27, 2026 | 30 | 2,586.0 | Capacity |
| NORTH_CENTRAL | 5 | May 28, 2026 | 32 | 1,752.0 | Capacity |

# IRR, Wind, and Solar Generation as a Percent of Load

The graph below shows the maximum, minimum and average aggregate solar, wind and IRR output as a percentage of total ERCOT load when evaluated as 10-minute averaged intervals, over the past 13 months. Current wind and solar generation and penetration records are listed in the footnote below. Maximum IRR penetration for May 2026 was 85.07% on 5/04/2026 interval ending 09:40 and minimum IRR penetration for May 2026 was 6.57% on 5/27/2026 interval ending 20:30.

During the hour of peak load for the month, hourly integrated wind generation was 16,587 MW and solar generation was 23,881 MW. The graph below shows the wind and solar penetration percentage during the hour of the peak load in the last 13 months.

Lastly, the graph below shows the minimum wind, solar, and IRR output during the peak load hour as a percentage of the daily peak load for every day in the month.

# Largest Net-Load Ramps

The net-load ramp is defined as the change in net-load (load minus wind and PVGR generation) during the defined time horizon. Such a variation in net-load needs to be accommodated in grid operations to ensure that the reliability of the grid is satisfactorily maintained. The largest net-load ramps over 5-minute, 10-minute, 15-minute, 30-minute, and 60-minute intervals in May 2026 were 2,438 MW, 3,901 MW, 4,825 MW, 9,111 MW, and 16,825 MW respectively. A comparison with historical values is provided in the table below.

| Month and Year | 5 min | 10 min | 15 min | 30 min | 60 min |
| --- | --- | --- | --- | --- | --- |
| 15-May | 1156 MW | 1770 MW | 2088 MW | 3242 MW | 5318 MW |
| 16-May | 871 MW | 1324 MW | 1804 MW | 2945 MW | 4897 MW |
| 17-May | 1109 MW | 1422 MW | 1883 MW | 3149 MW | 5348 MW |
| 18-May | 1173 MW | 1330 MW | 1845 MW | 3382 MW | 6508 MW |
| 19-May | 1066 MW | 1767 MW | 2483 MW | 4227 MW | 5146 MW |
| 20-May | 988 MW | 1529 MW | 1852 MW | 3104 MW | 5757 MW |
| 21-May | 1414 MW | 1664 MW | 1967 MW | 2874 MW | 4860 MW |
| 22-May | 1,647 MW | 1,663 MW | 2,154 MW | 4,140 MW | 7,012 MW |
| 23-May | 986 MW | 1,590 MW | 2,191 MW | 3,670 MW | 5,981 MW |
| 24-May | 1,771 MW | 2,318 MW | 3,153 MW | 4,974 MW | 8,686 MW |
| 25-May | 3,797 MW | 3,562 MW | 3,985 MW | 6,901 MW | 12,067 MW |
|  | 5/28/2025 | 5/28/2025 | 5/9/2025 | 5/9/2025 | 5/9/2025 |
|  | (IE 10:27) | (IE 10:27) | (IE 19:20) | (IE 19:31) | (IE 19:45) |
| 26-May | 2,438 MW | 3,901 MW | 4,825 MW | 9,111 MW | 16,825 MW |
|  | 5/3/2026 | 5/3/2026 | 5/3/2026 | 5/2/2026 | 5/2/2026 |
|  | (IE 18:39) | (IE 18:40) | (IE 18:48) | (IE 19:27) | (IE 19:26) |
| All Months in 2014-2026 | 3,797 MW | 3,901 MW | 4,825 MW | 9,111 MW | 16,825 MW |
|  | 5/28/2025 | 5/3/2026 | 5/3/2026 | 5/2/2026 | 5/2/2026 |
|  | (IE 10:27) | (IE 18:40) | (IE 18:48) | (IE 19:27) | (IE 19:26) |

# Congestion Analysis

## Notable Constraints

294 reportable events (303 last month)

1096 total congestion events (1128 last month, 1229 two months ago)

Nodal protocol section 3.20 specifies that ERCOT shall identify transmission constraints that are binding in Real-Time three or more Operating Days within a calendar month. As part of this process, ERCOT reports congestion that meets this criterion to ROS. In addition, ERCOT also highlights notable constraints that have an estimated congestion rent exceeding $1,000,000 for a calendar month. These constraints are detailed in the table below, including approved transmission upgrades from TPIT that may provide some congestion relief based on ERCOT’s engineering judgement. Rows highlighted in blue indicate the congestion was affected by one or more outages. For a list of all constraints activated in SCED, please see Appendix A at the end of this report.

| Data Item1 | Contingency Name | Overloaded Element | Contingency Name | Overloaded Element | # of Days Constraint Binding | Congestion Rent | Transmission Project |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n/a_n/a_WESTEX | BASE CASE | WESTEX | Basecase | WESTEX GTC | 13 | $24,980,858.69 | Permian Basin Relibility Plan (765-kV import paths) (The PUCT approved ,helps improve, but not fully exit, the GTC yet.) |  |
| MGSES_CATSW_6945__A | SRGRMGS5 | 6945__A | MORGAN CREEK SES to RANGER CAMP SWITCH LIN _A | Morgan Creek Ses - Cattleman Switch 345kV | 14 | $15,252,131.85 | West Texas 345-kV Infrastructure Rebuild Project (23RPG034) |  |
| LA_PALMA_VCAVAZOS_LA_PAL_VCAVAZ1_1 | SMV_PAR8 | LA_PAL_VCAVAZ1_1 | CENTRAL AVENUE SUB to CENTRAL AVENUE SUB LIN 1 | La Palma - Villa Cavazos 138kV | 1 | $9,254,181.84 |  |  |
| RIOHONDO_MV_RIOHO_RIOHND_ERIOHND_1 | MLAPVCA8 | RIOHND_ERIOHND_1 | manual La Palma to Villa Cavazos | Rio Hondo - East Rio Hondo Sub 138kV | 1 | $6,772,332.43 |  |  |
| LNGSW_PRLSW_6965__A | DBAKCED5 | 6965__A | BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Longshore Switch - Prairieland Switch 345kV | 16 | $6,424,601.03 | Oncor_FW_81268_Longshore – Consavvy 345 kV Double-Circuit Line Rebuild (MOD 81268, 23RPG034) |  |
| STP_WAP_STPWAP39_1 | DWPWFWP5 | STPWAP39_1 | TWR(345) WAP-WLF64 & WAP-WLY72 | South Texas Project - Wa Parish 345kV | 14 | $5,885,974.55 |  |  |
| HHGT_OMEGA_HHGTOM_1 | BASE CASE | HHGTOM_1 | Basecase | Omega - Horse Hollow Generation Tie 345kV | 11 | $3,615,735.67 |  |  |
| CMNSW_CMNTP_651__B | SCMNCPS5 | 651__B | COMANCHE SWITCH (Oncor) to COMANCHE PEAK SES LIN _A | Comanche Tap - Comanche Switch (Oncor) 138kV | 11 | $3,512,611.74 |  |  |
| NLARSW_PILONCIL_NLARSW_PILONC1_1 | DFOAVLO5 | NLARSW_PILONC1_1 | FOWLERTON to LOBO & AVANZADA | North Laredo Switch - Piloncillo 138kV | 20 | $3,470,841.66 |  |  |
| FORTMA_YELWJCKT_FORTMA_YELWJC1_1 | DBIGKEN5 | FORTMA_YELWJC1_1 | Bighil-Kendal 345kV | Yellow Jacket - Fort Mason 138kV | 13 | $3,294,022.32 |  |  |
| ADK_ADK_ADK_AT1 | XTH158 | ADK_AT1 | TH WHARTON TRX THW_AT3 345/138 | Addicks 138kV | 1 | $3,142,420.55 |  |  |
| WHITE_PT_WHITE_PT_WHITE_PT_T3L | SANGNAI5 | WHITE_PT_T3L | Angstrom to NAISMITH LIN 1 | Whitepoint 138kV | 1 | $3,082,040.96 |  |  |
| BCKSW_CGRSW_6215__A | DSCOTKW5 | 6215__A | SCOSW TO TKWSW 345 DBLCKT | Bluff Creek Switch - China Grove Switch 138kV | 12 | $2,909,154.78 |  |  |
| MDO_PHR_MDOPHR99_A | DWAP_OB5 | MDOPHR99_A | TWR (345) OB-WAP98 & OB-WAP99 | Meadow - Ph Robinson 345kV | 15 | $2,826,775.21 | CNP_28TPIT103344_PH_Robinson_to_WA_Parish_Hardening (MOD 103344 Phase 103435, 25RPG028) |  |
| SCRCV_KNAPP_6437__F | DMTSCOS5 | 6437__F | DMTSW TO SCOSW 345 DBLCKT | Knapp - Scurry Chevron 138kV | 18 | $2,705,929.83 | Oncor_FW_102849_Union 138 kV Switch (MOD 102849) |  |
| BCESW_SNDSW_421__A | SSNDBGR5 | 421__A | SANDOW SWITCH to BADGER SWITCH LIN _A | Sandow Switch - Bell County East Switch 345kV | 10 | $2,703,273.07 |  |  |
| HCKSW_EXCSW_107__B | SHC2EXC5 | 107__B | HICKS SWITCH to EXCHANGE SWITCH LIN _C | Hicks Switch - Exchange Switch 345kV | 3 | $2,578,201.34 |  |  |
| ODEHV_ODEHV_ODEHV_MR1H | DODEMOS5 | ODEHV_MR1H | ODEHV-MOSSW 345&ODEHV-WLFSW 345_DBLCKT | Odessa Ehv Switch 345kV | 2 | $2,443,478.09 |  |  |
| BRTSW_JUNPR_12020__B | DLONPK25 | 12020__B | DBL_PKRSW-GRSES_PKRSW-LONSW_345KV | Barton Chapel Wind Farm - Juniper Sub 138kV | 13 | $2,365,762.50 |  |  |
| MDO_PHR_MDOPHR99_A | DPHRCTR5 | MDOPHR99_A | Ctr-Phr&Cby 345kV | Meadow - Ph Robinson 345kV | 12 | $2,268,467.35 | CNP_28TPIT103344_PH_Robinson_to_WA_Parish_Hardening (MOD 103344 Phase 103435, 25RPG028) |  |
| LPLMK_LPLNE_LPLMK_LPLNE_1 | SBWDDBM5 | LPLMK_LPLNE_1 | BLACKWATER DRAW SWITCH to DOUBLE MOUNTAIN SWITCH LIN 1 | Mackenzie Substation - Northeast Substation 115kV | 11 | $2,204,340.41 |  |  |
| GN_PZ_GN_PZ_08_A | SMDOOAS5 | GN_PZ_08_A | MEADOW to OASIS LIN A | Grant - Plaza 138kV | 9 | $2,165,116.44 |  |  |
| SAMSW_VENSW_35055__A | DTCRTHS5 | 35055__A | THSES TO FBRSW & TCRSW 345 DBLCKT | Sam Switch - Venus Switch 345kV | 5 | $2,144,780.71 |  |  |
| GRSES_GRMES_6375__A | DLONPK25 | 6375__A | DBL_PKRSW-GRSES_PKRSW-LONSW_345KV | Graham Ses - Graham East 138kV | 5 | $2,132,286.99 |  |  |
| WLY_WAP_WAPWLY72_A | DBLW2JC5 | WAPWLY72_A | TWR (345) JCK-WAT62 & BLY-JCK57 | Wa Parish - Whaley 345kV | 17 | $2,078,481.88 |  |  |
| n/a_n/a_NELRIO | BASE CASE | NELRIO | Basecase | NELRIO GTC | 19 | $2,051,627.71 | The Lower Rio Grande Valley (LRGV) System Enhancement Project (21RPG017) |  |
| JN_BI_BI_JN_64_A | DWAP_BI5 | BI_JN_64_A | TWR (345) WAP-BI50 & SMITHERS-BI98 | Bellaire - Jeanetta 345kV | 7 | $1,771,892.86 |  |  |
| KLNSW_HHSTH_630__B | DSALKLN5 | 630__B | SALSW TO KLNSW 345 DBLCKT | Harker Heights South - Killeen Switch 138kV | 7 | $1,690,238.63 | ONCOR_SE_87699_Belton - Killeen 138 kV_Line (MOD 87699, 24RPG001) |  |
| SALDS_SONTERRA_1710__A | DSALHUT5 | 1710__A | SALSW - HUTTO 345KV | Salado South - Sonterra 138kV | 8 | $1,628,652.04 |  |  |
| LA_PALMA_VCAVAZOS_LA_PAL_VCAVAZ1_1 | BASE CASE | LA_PAL_VCAVAZ1_1 | Basecase | La Palma - Villa Cavazos 138kV | 1 | $1,561,700.93 |  |  |
| LA_PALMA_VCAVAZOS_LA_PAL_VCAVAZ1_1 | DL_FBR89 | LA_PAL_VCAVAZ1_1 | L_FRESNO - LOMA_ALT (138) & LA_PALMA - BROWNSVL (69) | La Palma - Villa Cavazos 138kV | 3 | $1,478,061.10 |  |  |
| TREADWEL_YELWJCKT_TREADW_YELWJC1_1 | DBIGKEN5 | TREADW_YELWJC1_1 | Bighil-Kendal 345kV | Yellow Jacket - Treadwell 138kV | 5 | $1,408,067.89 |  |  |
| HARLNSW_OLEANDER_HARLNS_OLEAND1_1 | MTGULAP5 | HARLNS_OLEAND1_1 | manual GULCH-LA_PALMA 345 temp | Oleander - Harlingen Switch 138kV | 3 | $1,365,563.92 |  |  |
| n/a_n/a_NE_LOB | BASE CASE | NE_LOB | Basecase | NE_LOB GTC | 12 | $1,348,083.94 | San Antonio South Reliability I Project (22RPG048) LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |  |
| LAKENASW_SAMATHIS_LAKENA_SAMATH1_1 | DBAKCED5 | LAKENA_SAMATH1_1 | BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Lake Nasworthy - San Angelo Mathis Field 69kV | 13 | $1,217,029.89 |  |  |
| SAMSW_VENSW_35055__A | SVENFTS5 | 35055__A | FORT SMITH SWITCH to FORT SMITH SWITCH LIN _B | Sam Switch - Venus Switch 345kV | 5 | $1,134,606.64 |  |  |
| n/a_n/a_EASTEX | BASE CASE | EASTEX | Basecase | EASTEX GTC | 13 | $1,027,408.19 |  |  |
| ALAZAN_B_DAVIS_ALAZAN_B_DAVI1_1 | SN_SLON5 | ALAZAN_B_DAVI1_1 | LON HILL to NELSON SHARPE LIN 1 | Barney Davis - Alazan 138kV | 7 | $1,021,417.42 |  |  |
| BOW_BOW_BOW_FMR1 | DRILKRW5 | BOW_FMR1 | RILEY TO KRWSW 345 DBLCKT | Bowie 138kV | 5 | $994,964.40 |  |  |
| YELWJCKT_FORTMA_FORTMA_YELWJC1_1 | SBIGOR55 | FORTMA_YELWJC1_1 | Big HiLL to ORSTED LIN 1 | Yellow Jacket - Fort Mason 138kV | 9 | $933,413.02 |  |  |
| ZEN_THW_THWZEN71_A | DRNS_TB5 | THWZEN71_A | Rns-Rtw & Sng-Tb 345kV | Th Wharton - Zenith 345kV | 11 | $752,835.84 |  |  |
| CRDSW_OLNEY_6830__B | SGRMGRS8 | 6830__B | GRAHAM EAST to GRAHAM EAST LIN _A | Cottonwood Road Switch - Olney Pod 69kV | 10 | $673,462.03 |  |  |
| MV_HBRG4_MV_WESL4_WESLCO_HIDLBRG_1 | MHARNED5 | WESLCO_HIDLBRG_1 | Manual dbl ckt for NEDIN-BONILLA 345kV & RIOH-PRIM138kV | Heidelburg Sub - Weslaco Sub 138kV | 6 | $624,590.10 |  |  |
| WLY_WAP_WAPWLY72_A | DOASSEB5 | WAPWLY72_A | TWR(345) DOW-OAS 18 & OAS-SEB 27 | Wa Parish - Whaley 345kV | 6 | $616,777.05 |  |  |
| TYPRK_PLYAM_6200__F | DCDHTVW5 | 6200__F | TVWSW TO CDHSW 345 AND CDHSW TO VENSW 345 DBLCKT | Poly-America - Tyre Park Sub 138kV | 3 | $612,707.27 |  |  |
| NORTMC_CROSSOVE_CROSSO_NORTMC1_1 | DBIGSCH5 | CROSSO_NORTMC1_1 | Big Hill - Schneeman Draw & Big Hill - Schneeman Draw 2 | North Mccamey - Crossover 138kV | 5 | $573,570.62 |  |  |
| SEA_SEA_SEA_AAT1 | DFRYTM58 | SEA_AAT1 | DOUBLE FRYSW-TMPSW 138 & KNBSW-SALSW 345 | Seaton 138kV | 4 | $544,289.04 |  |  |
| ARGYL_LWSVH_587__A | MRNKDHM5 | 587__A | MANUAL RNKSW TO DHMSW 345&KRWSW TO DHMSW 345 DBLCKT | Argyle - Highlands Tnp 138kV | 5 | $517,778.82 |  |  |
| PALOUSE_WOLFCAMP_PALOUS_WOLFCA1_1 | DBIGSCH5 | PALOUS_WOLFCA1_1 | Big Hill - Schneeman Draw & Big Hill - Schneeman Draw 2 | Palouse - Wolfcamp 138kV | 9 | $507,181.05 |  |  |
| ESCONDID_GANSO_ESCOND_GANSO1_1 | SBRAHAM8 | ESCOND_GANSO1_1 | BRACKETTVILLE to HAMILTON ROAD LIN 1 | Escondido - Ganso 138kV | 6 | $488,968.29 | Escondido to Hamilton Road 138 kV Line Rebuild Project (22RPG044) |  |
| NUECES_B_WHITE_PT_NUECES_WHITE_2_1 | SMAYWHI8 | NUECES_WHITE_2_1 | GILA to MAYO LIN 1 | Nueces Bay - Whitepoint 138kV | 13 | $463,546.86 |  |  |
| n/a_n/a_WHARTN | BASE CASE | WHARTN | Basecase | WHARTN GTC | 8 | $436,560.03 |  |  |
| BERGHE_BERGHE_BERGHE_AT1H | DZORHAY5 | BERGHE_AT1H | ZORN - HAYSEN 345KV | Bergheim 345kV | 4 | $425,596.72 |  |  |
| FMRVL_RYSSW_400__A | SFMRRYS5 | 400__A | Farmersville Switch to Farmersville Switch LIN _A | Royse Switch - Farmersville Switch 345kV | 11 | $422,738.46 | ONCOR_MNE_99066_Royse Switch – Farmersville Switch 345 kV Double-Circuit Line (MOD 99066, 25RPG004) |  |
| HECKER_RESNIK_HECKER_RESNIK1_1 | DMCDUPS8 | HECKER_RESNIK1_1 | MCCAMPBE - RESNIK & I_DUPSW 138kV | Hecker - Resnik 138kV | 7 | $403,033.00 | Rebuild the Hecker to Resnik 138 kV line 76286 |  |
| ZEN_THW_THWZEN71_A | SZENTH35 | THWZEN71_A | TH WHARTON to Zenith LIN A | Th Wharton - Zenith 345kV | 3 | $385,538.35 |  |  |
| DILLEYSW_DILLEYSW_DILLEYSW_XF1H | DDILCOT8 | DILLEYSW_XF1H | Dilleysw-Sanmgsw&Cotulas 138kV | Dilley Switch Aep 138kV | 10 | $350,269.32 |  |  |
| SGRSW_JEWET_235__A | DVENFTS5 | 235__A | VENSW-FTSSW & VENSW-SAMSW 345 DBLCKT | Jewett - Stranger Switch 345kV | 4 | $348,545.52 |  |  |
| CRDSW_OLNEY_6830__B | DGRMGRS8 | 6830__B | FIREROCK TO BRNWD 138 AND FIREROCK TO BANGS 69 DBLCKT | Cottonwood Road Switch - Olney Pod 69kV | 9 | $333,421.68 |  |  |
| BRUNI_BRUNI_BRUNI_69_1 | DFOAVLO5 | BRUNI_69_1 | FOWLERTON to LOBO & AVANZADA | Bruni Sub 138kV | 4 | $313,645.23 |  |  |
| BALLINGE_HUMBLTAP_BALLIN_HUMBLT1_1 | SCOLBAL8 | BALLIN_HUMBLT1_1 | BALLINGER to COLEMAN LAKE IVIE TAP LIN 1 | Ballinger - Ballinger Humble Tap 69kV | 6 | $305,371.81 |  |  |
| HAMILTON_MAVERICK_HAMILT_MAVERI1_1 | SBRAPIN8 | HAMILT_MAVERI1_1 | BRACKETTVILLE to BRACKETTVILLE LIN 1 | Hamilton Road - Maverick 138kV | 15 | $259,991.98 |  |  |
| THW_NB_NB_THW97_A | DRNS_TB5 | NB_THW97_A | Rns-Rtw & Sng-Tb 345kV | North Belt - Th Wharton 345kV | 8 | $257,729.92 |  |  |
| WOLFCAMP_SANTARIT_SANTAR_WOLFCA1_1 | DBIGSCH5 | SANTAR_WOLFCA1_1 | Big Hill - Schneeman Draw & Big Hill - Schneeman Draw 2 | Santa Rita - Wolfcamp 138kV | 6 | $249,647.26 |  |  |
| DPREA_RCSES_1561__A | XPRS58 | 1561__A | PARIS SWITCH TRX PRSSW_3_1 345/138 | Rivercrest Ses - Deport Rea 138kV | 5 | $246,920.22 |  |  |
| ARGYL_LWSVH_587__A | MDHMLWS5 | 587__A | MANUAL DHMSW TO LWSSW 345 DBLCKT | Argyle - Highlands Tnp 138kV | 3 | $233,662.87 |  |  |
| BLESSING_BLESSING_BLESSING_1382 | DSTPREF5 | BLESSING_1382 | TWR(345) JCK-REF27 & JCK-STP18 | Blessing 345kV | 5 | $232,424.48 |  |  |
| HONDOCK_DEVINESW_HON_DEVI_1 | SHONF38 | HON_DEVI_1 | HONDO to HONDO CREEK SWITCHING STATION LIN 1 | Hondo Creek Switching Station - Devine Switch 69kV | 10 | $227,330.22 |  |  |
| WLVSW_GAILS_6217__A | SVEAW_L5 | 6217__A | wett_long_draw to VEALMOOR - Sharyland Utilities LIN 1 | Willow Valley Switch - Gail Sub 138kV | 8 | $222,443.98 |  |  |
| SONR_ATSO_ATSO_SONR1_1 | SCARFRI8 | ATSO_SONR1_1 | Carver to Carver LIN 1 | Atlantic Sonora - Sonora 69kV | 6 | $211,363.00 |  |  |
| ELKTN_ELKTN_ELKTN_MR3H | DMLSENT5 | ELKTN_MR3H | MARTIN LAKE - SCSES and - ENTERPRISE | Elkton 345kV | 4 | $208,060.35 |  |  |
| FORTMA_YELWJCKT_FORTMA_YELWJC1_1 | SBIGOR55 | FORTMA_YELWJC1_1 | Big HiLL to ORSTED LIN 1 | Yellow Jacket - Fort Mason 138kV | 9 | $199,461.71 |  |  |
| MEXTP_ITALY_2270__B | SBLURDH8 | 2270__B | BLUM to RED HILL SWITCH LIN _G | Milford Exxon Tap - Italy 69kV | 7 | $199,378.21 |  |  |
| MDO_PHR_MDOPHR99_A | DKG_NB_5 | MDOPHR99_A | TWR(345) JOR-KG97 & JOR-NB99 | Meadow - Ph Robinson 345kV | 6 | $199,024.82 | CNP_28TPIT103344_PH_Robinson_to_WA_Parish_Hardening (MOD 103344 Phase 103435, 25RPG028) |  |
| LOBO_LARDVNTH_LARDVN_LOBO2_1 | SLARDUS8 | LARDVN_LOBO2_1 | Dust Devil to Dust Devil LIN 1 | Lobo - Laredo Vft North 138kV | 6 | $194,838.49 |  |  |
| MADDUX_TREADWEL_MADDUX_TREADW1_1 | DBIGKEN5 | MADDUX_TREADW1_1 | Bighil-Kendal 345kV | Maddux - Treadwell 138kV | 4 | $190,049.70 |  |  |
| FLTSW_PRSSW_1545__A | SPRSPAR8 | 1545__A | PARIS SWITCH to PARIS LIN _A | Paris Switch - Flying Tiger Switch 138kV | 9 | $166,796.28 |  |  |
| LNGSW_PRLSW_6965__A | MCATPRL5 | 6965__A | MAN_SGL_CATSW-PRLSW_345KV | Longshore Switch - Prairieland Switch 345kV | 4 | $165,621.36 | Oncor_FW_81268_Longshore – Consavvy 345 kV Double-Circuit Line Rebuild (MOD 81268, 23RPG034) |  |
| SACRC_DPCRK_6240__C | DMTSCOS5 | 6240__C | DMTSW TO SCOSW 345 DBLCKT | Sacroc - Deep Creek Sub 138kV | 3 | $165,501.40 |  |  |
| BLESSING_BLESSING_BLESSING_1382 | DSTPSTA5 | BLESSING_1382 | Double STP to HLJ & Static 345 KV | Blessing 345kV | 6 | $148,867.00 |  |  |
| n/a_n/a_E_PASP | BASE CASE | E_PASP | Basecase | E_PASP GTC | 8 | $148,767.03 | San Antonio South Reliability I Project , 22RPG048 LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |  |
| TWINBU_HARGROVE_HARGRO_TWINBU1_1 | DBAKCED5 | HARGRO_TWINBU1_1 | BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Hargrove - Twin Buttes 138kV | 6 | $147,436.69 |  |  |
| FRONTERA_S_MISSIN_BENTS_FRTER_1B_1 | SSHAFRO8 | BENTS_FRTER_1B_1 | FRONTERA to SHARYLAND LIN 1 | Frontera - South Mission 138kV | 12 | $144,720.98 |  |  |
| FOWLRTON_TILDEN_FWLRTN_TILDEN_1 | DFOWSMG5 | FWLRTN_TILDEN_1 | FOWLRTON TO SAN MIGUEL DOUBLE CIRCUIT CONTINGENCY | Fowlerton - Tilden 138 Sub 138kV | 4 | $144,053.37 |  |  |
| YELWJCKT_FORTMA_FORTMA_YELWJC1_1 | DBIGKEN5 | FORTMA_YELWJC1_1 | Bighil-Kendal 345kV | Yellow Jacket - Fort Mason 138kV | 13 | $143,559.57 |  |  |
| COLETO_VICTORIA_COLETO_VICTOR2_1 | DVICVI89 | COLETO_VICTOR2_1 | VICTORIA - FANNIN 69 & COLETO - VICTORIA 1 138 | Coleto Creek - Victoria 138kV | 4 | $134,337.89 |  |  |
| PALOUSE_CROSSOVE_CROSSO_PALOUS1_1 | DBAKCED5 | CROSSO_PALOUS1_1 | BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Crossover - Palouse 138kV | 5 | $112,500.61 |  |  |
| ELKTN_ELKTN_ELKTN_MR3L | DMLSENT5 | ELKTN_MR3L | MARTIN LAKE - SCSES and - ENTERPRISE | Elkton 138kV | 3 | $107,124.05 |  |  |
| BOW_BOW_BOW_FMR1 | SLKAWFS8 | BOW_FMR1 | ALVORD TAP to ALVORD TAP LIN _F | Bowie 138kV | 5 | $106,185.56 |  |  |
| HAMILTON_MAVERICK_HAMILT_MAVERI1_1 | SBRAHAM8 | HAMILT_MAVERI1_1 | BRACKETTVILLE to HAMILTON ROAD LIN 1 | Hamilton Road - Maverick 138kV | 6 | $103,156.69 |  |  |
| FROWS_MCLSW_1025__B | SMCLWNE8 | 1025__B | MCLANE SWITCH to WACO NORTHEAST TAP LIN _A | Mclane Switch - Frow Sub 138kV | 3 | $93,310.17 |  |  |
| KNBSW_TMPSW_270__A | DSALHUT5 | 270__A | SALSW - HUTTO 345KV | Temple Switch - Knob Creek Switch 345kV | 5 | $91,896.41 |  |  |
| ASHERTON_CATARINA_ASHERT_CATARI1_1 | DLOFOAV5 | ASHERT_CATARI1_1 | Double LOBO - FOWLERTON & AVANZADA | Asherton - Catarina 138kV | 5 | $91,349.46 |  |  |
| BIG_FOOT_PLEASANT_BIG_FO_PLEASA1_1 | SHONMOO8 | BIG_FO_PLEASA1_1 | MOORE SWITCHING STATION to HONDO CREEK SWITCHING STATION LIN 1 | Big Foot - Pleasanton 138kV | 7 | $80,560.44 |  |  |
| COLETO_VICTORIA_COLETO_VICTOR2_1 | SVICCO28 | COLETO_VICTOR2_1 | COLETO CREEK to VICTORIA LIN 1 | Coleto Creek - Victoria 138kV | 4 | $78,050.94 |  |  |
| ENWSW_TMPTN_940__A | SENWSHK8 | 940__A | ENNIS WEST SWITCH to ENNIS WEST SWITCH LIN _B | Ennis West Switch - Templeton 138kV | 3 | $77,302.65 |  |  |
| SNYDR_AMOTP_6695__B | SSNYCGR8 | 6695__B | SNYDER to CHINA GROVE SWITCH LIN _A | Amoco Tap - Snyder 69kV | 4 | $77,198.12 |  |  |
| FROWS_MCLSW_1025__B | SELMTH25 | 1025__B | ELM MOTT to ELM MOTT LIN _A | Mclane Switch - Frow Sub 138kV | 6 | $76,601.49 |  |  |
| TMPSW_FRYSW_1715__B | DSALHUT5 | 1715__B | SALSW - HUTTO 345KV | Temple Switch - Fryers Creek 138kV | 4 | $70,443.39 |  |  |
| NVKSW_ANARN_6840__B | SRICGRS8 | 6840__B | GRAHAM SES to RICE SWITCH LIN _A | Anarene - Navy Kickapoo Switch 69kV | 8 | $67,657.79 |  |  |
| SSPSW_LBRPD_690__C | DCSHSSP5 | 690__C | CSHSW-SSPSW 345&ALNSW-SCRSW 345 DBLCKT | Sulphur Springs Switch - Liberty Pod (Oncor) 138kV | 5 | $66,240.04 |  |  |
| n/a_n/a_VALEXP | BASE CASE | VALEXP | Basecase | VALEXP GTC | 5 | $61,520.31 |  |  |
| CATARINA_ASHERTON_ASHERT_CATARI1_1 | DFOAVLO5 | ASHERT_CATARI1_1 | FOWLERTON to LOBO & AVANZADA | Asherton - Catarina 138kV | 3 | $60,441.63 |  |  |
| ENWSW_TMPTN_940__A | SPEBTRU8 | 940__A | GAMMA to GAMMA LIN _D | Ennis West Switch - Templeton 138kV | 7 | $58,095.87 |  |  |
| NORTMC_CROSSOVE_CROSSO_NORTMC1_1 | SBAKNOR5 | CROSSO_NORTMC1_1 | BAKERSFIELD SWITCHYARD to NORTH McCAMEY LIN 1 | North Mccamey - Crossover 138kV | 3 | $54,538.22 |  |  |
| SOLSTICE_SOLSTICE_SOLSTICE_PST1 | DBAKSOL5 | SOLSTICE_PST1 | Bakersfield - Solstice line 1 and 2 | Solstice 138kV | 3 | $53,488.34 |  |  |
| SAPOWER_MADDUX_MADDUX_SAPOWE1_1 | SMA2SAP8 | MADDUX_SAPOWE1_1 | MADDUX to SAN ANGELO POWER STATION LIN 1 | Maddux - San Angelo Power Station 138kV | 5 | $51,995.26 |  |  |
| FURHMAN_RAYBURN_RAYBUR_FURHMAN_1 | SFURVAN8 | RAYBUR_FURHMAN_1 | FURHMAN to VANDERBILT SWITCHING STATION LIN 1 | Sam Rayburn Switchyd - Furhman 138kV | 21 | $47,204.09 |  |  |
| RS_FB_FB_RS_60_A | MATHEB_8 | FB_RS_60_A | MANUAL ATHENA TO EAST BERNARD LIN A | Fort Bend - Rosenberg 138kV | 3 | $45,014.51 |  |  |
| MV_BURNS_MV_HBRG4_BURNS_HEIDLBRG_1 | MHARNED5 | BURNS_HEIDLBRG_1 | Manual dbl ckt for NEDIN-BONILLA 345kV & RIOH-PRIM138kV | Burns Sub - Heidelburg Sub 138kV | 4 | $40,466.89 | STEC_71926_Burns_Heidelberg_Upgrade (MOD 71926) |  |
| BNDVS_ELMGROVE_BEN_ELMG_1 | SWALWLN8 | BEN_ELMG_1 | WALLACE SWITCH to WALLACE SWITCH LIN 1 | Ben Davis - Elmgrove 138kV | 12 | $36,496.63 |  |  |
| FMRVL_RYSSW_381__A | SFMRRY25 | 381__A | Farmersville Switch to ROYSE SWITCH LIN _A | Farmersville Switch - Royse Switch 345kV | 3 | $34,589.42 |  |  |
| TNFXTAIL_FLAT_TOP_138_FLT_FXT_1 | DSLKSOL5 | 138_FLT_FXT_1 | Sand Lake - Solstice line 1 and 2 | Foxtail Tnp - Flat Top Tnp 138kV | 4 | $22,969.07 |  |  |
| DOWNIES_MOORE_2585_1 | DRAZSA89 | 2585_1 | Double Circuit RAZORBAC to DRYFRIO 138 kV & UVALDE to SABINAL 69 kV | Moore Switching Station - Downie Switching Station 138kV | 5 | $22,218.96 |  |  |
| IH20_TNCOLIET_138_IH2_COT_1 | SSTAWIC8 | 138_IH2_COT_1 | STAGHORN TNP to WICKETT TNP LIN 1 | Ih 20 Tnp - Collie Field Tap Tnp 138kV | 10 | $18,785.36 |  |  |
| SACRC_DPCRK_6240__C | SSUNRGR8 | 6240__C | SUN SWITCH to RANGER CAMP SWITCH LIN _B | Sacroc - Deep Creek Sub 138kV | 4 | $15,199.56 |  |  |
| FTST_LEONCRK_138_FTS_LNC_1 | BASE CASE | 138_FTS_LNC_1 | Basecase | Leon Creek Tnp - Fort Stockton Plant 138kV | 10 | $15,116.91 |  |  |
| HUBRD_HAN1_1210__B | SBLURDH8 | 1210__B | BLUM to RED HILL SWITCH LIN _G | Hubbard - Haney Bepc 138kV | 5 | $15,005.89 |  |  |
| MAXWELL_HAMILTON_HAMILT_MAXWEL1_1 | SILLFTL8 | HAMILT_MAXWEL1_1 | FORT LANCASTER to FORT LANCASTER LIN 1 | Hamilton Road - Maxwell 138kV | 3 | $13,439.87 |  |  |
| PILONCIL_NLARSW_NLARSW_PILONC1_1 | DLOFOAV5 | NLARSW_PILONC1_1 | Double LOBO - FOWLERTON & AVANZADA | North Laredo Switch - Piloncillo 138kV | 3 | $12,937.79 |  |  |
| n/a_n/a_PNHNDL | BASE CASE | PNHNDL | Basecase | PNHNDL GTC | 5 | $12,094.20 |  |  |
| SNYDR_SNYDR_SNYDR_FMR1 | SSNYCGR8 | SNYDR_FMR1 | SNYDER to CHINA GROVE SWITCH LIN _A | Snyder 138kV | 4 | $12,068.82 |  |  |
| BRUNI_BRUNI_BRUNI_69_1 | SLAQLOB8 | BRUNI_69_1 | LAQUINTA to LOBO LIN 1 | Bruni Sub 138kV | 3 | $10,711.74 |  |  |
| MAXWELL_WHITING_MAXWEL_WHITIN1_1 | SHAMMAX8 | MAXWEL_WHITIN1_1 | HAMILTON ROAD TRX PS2 138/138 | Maxwell - Whiting 138kV | 4 | $10,266.13 |  |  |
| FRONTERA_S_MISSIN_BENTS_FRTER_1B_1 | SBENS_M8 | BENTS_FRTER_1B_1 | SOUTH MCALLEN to BENTSEN LIN 1 | Frontera - South Mission 138kV | 15 | $9,796.25 |  |  |
| MADDUX_SAPOWER_MADDUX_SAPOWE1_1 | SMA2SAP8 | MADDUX_SAPOWE1_1 | MADDUX to SAN ANGELO POWER STATION LIN 1 | Maddux - San Angelo Power Station 138kV | 5 | $8,631.04 |  |  |
| LA_PALMA_HAINE_DR_HAINE__LA_PAL1_1 | MHARNED5 | HAINE__LA_PAL1_1 | Manual dbl ckt for NEDIN-BONILLA 345kV & RIOH-PRIM138kV | Haine Drive - La Palma 138kV | 3 | $8,498.84 |  |  |
| CMNSW_CMNTP_651__B | DBUCRGP5 | 651__B | MANUAL DOUBLE RGPSW - KLNSW 345 KV & BUCSW - KLNSW 345 KV | Comanche Tap - Comanche Switch (Oncor) 138kV | 6 | $7,419.56 |  |  |
| TNNALVIN_ALVIN_138_ALV_NAL_1 | SMDOPHR5 | 138_ALV_NAL_1 | MEADOW to PH ROBINSON LIN A | North Alvin Tnp - Alvin Tnp 138kV | 3 | $3,571.10 |  |  |
| NEVADA_SABNCRKT_SABIN_NEVAD_1 | SWALWLN8 | SABIN_NEVAD_1 | WALLACE SWITCH to WALLACE SWITCH LIN 1 | Sabine Creek Tap - Nevada 138kV | 5 | $3,043.83 |  |  |
| NLARSW_PILONCIL_NLARSW_PILONC1_1 | DLOFOAV5 | NLARSW_PILONC1_1 | Double LOBO - FOWLERTON & AVANZADA | North Laredo Switch - Piloncillo 138kV | 3 | $56.59 |  |  |

## Generic Transmission Constraint Congestion

| GTC | Days Congestion |
| --- | --- |
| Nelson Sharpe - Rio Hondo | 21 |
| North Edinburg - Lobo | 19 |
| North - Houston | 18 |
| West Texas | 15 |
| South Texas Export Pawnee - Spruce | 14 |
| East Texas | 13 |
| Wharton | 12 |
| Panhandle | 11 |
| South Texas Export Pawnee - Tango | 11 |
| Valley Export | 9 |
| McCamey | 5 |
| Rio Grande Valley Import | 1 |
| Zapata Starr | 1 |
| South Texas Import Katoen - Lonhill | 1 |

- There was no activity on the remaining GTCs during the month.

Note: This is how many times a constraint has been activated to avoid exceeding a GTC limit, it does not imply an exceedance of the GTC occurred or that the GTC was binding.

## Manual Overrides

- None

## Congestion Costs for Calendar Year 2026

The following table represents the top twenty active constraints for the calendar year based on the estimated congestion rent attributed to the congestion. ERCOT updates this list on a monthly basis.

| Contingency | Overloaded Element | # of 5-min SCED | Estimated | Transmission |
| --- | --- | --- | --- | --- |
| Basecase | WESTEX GTC | 12,178 | 67,874,335.06 | Permian Basin Relibility Plan (765-kV import paths) (The PUCT approved ,helps improve, but not fully exit, the GTC yet.) |
| MANUAL HCKSW-RAMSW 345 kV_2 | Hicks Switch 138kV | 1,363 | 51,703,803.38 | 0 |
| BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Longshore Switch - Prairieland Switch 345kV | 11,303 | 45,788,437.08 | Oncor_FW_81268_Longshore – Consavvy 345 kV Double-Circuit Line Rebuild (MOD 81268, 23RPG034) |
| MANUAL RNKSW TO DHMSW 345&KRWSW TO DHMSW 345 DBLCKT | Argyle - Highlands Tnp 138kV | 6,584 | 34,073,875.92 | 0 |
| Basecase | EASTEX GTC | 4,567 | 32,188,817.64 | 0 |
| SALSW TO KLNSW 345 DBLCKT | Killeen Switch - Stagecoach 138kV | 2,796 | 31,304,486.27 | ONCOR_SE_87703_Belton_Killeen_138 kV Line (via Nolanville) (MOD 87703, 24RPG001) |
| DOUBLE NVARO-LEG & NVARO-OUTSW 345 KV | Big Brown Ses - Jewett 345kV | 504 | 26,003,760.95 | ONCOR_SE_99622_PIN_OAK_SWITCH (MOD 99622, 25RPG004) |
| BLACKWATER DRAW SWITCH to DOUBLE MOUNTAIN SWITCH LIN 1 | Mackenzie Substation - Northeast Substation 115kV | 5,771 | 24,293,502.93 | 0 |
| BAKESW-CEDACA 345kV & BAKESW-CEDACA 345kV | Hargrove - Twin Buttes 138kV | 4,160 | 20,328,157.39 | 0 |
| Basecase | E_PASP GTC | 7,183 | 19,334,223.48 | San Antonio South Reliability I Project , 22RPG048 LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |
| SALSW TO KLNSW 345 DBLCKT | Harker Heights South - Killeen Switch 138kV | 3,535 | 17,440,143.97 | ONCOR_SE_87699_Belton - Killeen 138 kV_Line (MOD 87699, 24RPG001) |
| DMTSW TO SCOSW 345 DBLCKT | Knapp - Scurry Chevron 138kV | 8,424 | 17,067,441.40 | Oncor_FW_102849_Union 138 kV Switch (MOD 102849) |
| Basecase | Omega - Horse Hollow Generation Tie 345kV | 7,311 | 16,645,667.19 | 0 |
| STP-Elmcreek&WAP 345kV | South Texas Project - Elmcreek 345kV | 355 | 15,792,038.58 | 0 |
| MORGAN CREEK SES to RANGER CAMP SWITCH LIN _A | Morgan Creek Ses - Cattleman Switch 345kV | 1,496 | 15,699,076.23 | West Texas 345-kV Infrastructure Rebuild Project (23RPG034) |
| MANUAL SINGLE SANDOW SWITCH to BADGER SWITCH 345 | Sandow Switch - Bell County East Switch 345kV | 2,202 | 15,471,465.60 | 0 |
| FOWLERTON to LOBO & AVANZADA | Lobo - Laredo Vft North 138kV | 5,426 | 15,369,961.62 | 0 |
| MAN_SGL_RGRSW-MGSES_345KV | Morgan Creek Ses - Cattleman Switch 345kV | 2,380 | 14,991,540.47 | West Texas 345-kV Infrastructure Rebuild Project (23RPG034) |
| MEADOW to OASIS LIN A | Brays - H O Clarke 138kV | 2,154 | 14,775,630.35 | 0 |
| Basecase | NE_LOB GTC | 14,219 | 14,698,212.33 | San Antonio South Reliability I Project (22RPG048) LRGV Transmission Improvement Project (21RPG017) San Antonio South Reliability II Project (23RPG032) |

# System Events

## ERCOT Peak Load

The unofficial ERCOT peak load for May 2026 was 77,822 MW and occurred on 05/29/2026 during hour ending 18:00, this is 577 MW lower than the May 2025 demand of 78,399 MW on 05/23/2025 during hour ending 17:00. Instantaneous peak for May 2026 was 78,211 MW. Actual instantaneous peak for the same month last year was 79,475 MW.

## Load Shed Events

- None.

## Stability Events

- None.

## Notable PMU Events

ERCOT analyzes PMU data for any significant system disturbances that do not fall into the Frequency Events category reported in section 2.1. The results are summarized in this section once the analysis has been completed.

There were no PMU events outside of those reported in section 2.1.

## DC Tie Curtailment

| Date | DC Tie | Curtailing Period | # of Tags Curtailed | Initiating Event | Curtailment Reason2,3 |
| --- | --- | --- | --- | --- | --- |
| 5/19/2026 | DC_L | 5/19/26 11:11 – 5/20/26 00:15 | 3 | Faulty equipment | Planned outage extended due to leak |
| 5/27/2026 | DC_E | 00:03 – 02:45 | 3 | Overschedule issues | Bring tie within limit |
| 5/27/2026 | DC_N | 00:04 – 02:44 | 4 | Overschedule issues | Bring tie within limit |

## TRE/DOE Reportable Events

- AEN Submitted a DOE-417 for 5/6/2026 for Complete loss of monitoring or control capability at its staffed Bulk Electric System control center for 30 continuous minutes or more

- Oncor Submitted an EOP-004-4 for 5/13/2026 for Damage of destruction of a Facility and Physical threat to its Facility

- Oncor Submitted a DOE-417 for 5/13/2026 for Damage of destruction of a Facility and Physical threat to its Facility

- Lubbock Power Submitted an EOP-004-4 for 5/26/2026 for Damage of destruction of a Facility and Physical threat to its Facility

## New/Updated Constraint Management Plans

- None

## New/Modified/Removed RAS

- None

## New Procedures/Forms/Operating Bulletins

| Date | Subject | Bulletin No. |
| --- | --- | --- |
| 5/29/2026 | Transmission and Security Desk Operating Procedure V2 Rev 6 | 2029 |
| 5/29/2026 | Real Time Desk V2 Rev 3 | 2028 |

# Emergency Conditions

## OCNs

- None

## Advisories

- None

## Watches

| Date and Time | Message |
| --- | --- |
| May 9, 2026 15:45 | ERCOT is issuing a Transmission Watch due to storm damage in the Rio Grande Valley of Harlingen, Brownsville and South Padre Island area. |

## Emergency Notices

- None

# Application Performance

## TSAT/VSAT Performance Issues

| Date and Time | Message |
| --- | --- |

## Communication Issues

- None.

## Market System Issues

- None.

# Model Updates

The Downstream Production Change (DPC) process allows ERCOT to make changes in the one-line Network Operations Model without loading a completely new model.  The purpose of this process is to allow for reliable grid operations as system conditions change between designated Network Operations Model database loads.  The DPC process is limited in scope to just those items listed below, with equipment ratings updates being the most common.  ERCOT has seen a rise in the use of the DPC process to make on-line updates to the Network Operations Model in recent years, instead of through the standard Network Operations Model Change Request process.

- Static Line ratings (Interim Update)

- Dynamic Line ratings (non-Interim Update)

- Autotransformer ratings (non-Interim Update)

- Breaker and Switch Normal status (Interim Update)

- Contingency Definitions (Interim Update)

- RAP and RAS changes or additions (Interim Update)

- Net Dependable and Reactive Capability (NDCRC) values (Interim Update)

- Impedance Updates (non-Interim)

A total of 82 DPCs were implemented in May 2026. 217 DPCs have been implemented year to date. DPCs submitted by TDSPs are mainly updates to transmission element ratings. DPCs submitted by ERCOT are mainly updates to manual contingency definitions.

| Transmission Operator | Number of DPCs |
| --- | --- |
| AEP TEXAS COMPANY (TDSP) | 11 |
| BRAZOS ELECTRIC POWER CO OP INC (TDSP) | 0 |
| BROWNSVILLE PUBLIC UTILITIES BOARD (TDSP) | 0 |
| BRYAN TEXAS UTILITIES (TDSP) | 0 |
| CENTERPOINT ENERGY HOUSTON ELECTRIC LLC (TDSP) | 11 |
| CITY OF AUSTIN DBA AUSTIN ENERGY (TDSP) | 2 |
| CITY OF COLLEGE STATION (TDSP) | 0 |
| CITY OF GARLAND (TDSP) | 0 |
| CPS ENERGY (TDSP) | 3 |
| CROSS TEXAS TRANSMISSION LLC (TSP)) | 0 |
| DENTON MUNICIPAL ELECTRIC (TDSP) | 0 |
| ELECTRIC TRANSMISSION TEXAS LLC (TDSP) | 1 |
| ERCOT | 0 |
| LCRA TRANSMISSION SERVICES CORPORATION (TDSP) | 27 |
| LONE STAR TRANSMISSION LLC (TSP) | 1 |
| ONCOR ELECTRIC DELIVERY COMPANY LLC (TDSP) | 21 |
| PEDERNALES ELECTRIC CO OP INC (TDSP) | 1 |
| RAYBURN COUNTRY CO OP DBA RAYBURN ELECTRIC (TDSP) | 0 |
| SHARYLAND UTILITIES LP (TDSP) | 0 |
| SOUTH TEXAS ELECTRIC CO OP INC (TDSP) | 3 |
| TEXAS MUNICIPAL POWER AGENCY (TDSP) | 0 |
| TEXAS-NEW MEXICO POWER CO (TDSP) | 2 |
| WIND ENERGY TRANSMISSION TEXAS LLC (TSP) | 1 |

# Appendix A: Real-Time Constraints

The following is a complete list of constraints activated in SCED. Full contingency descriptions can be found in the Standard Contingencies List located on the MIS secure site at Grid  Generation  Reliability Unit Commitment.

| Year | Month | Contingency Name | Overloaded Element | From Station | To Station | Count of Days |
| --- | --- | --- | --- | --- | --- | --- |
| 2026 | May | SFURVAN8 | RAYBUR_FURHMAN_1 | FURHMAN | RAYBURN | 24 |
| 2026 | May | DFOAVLO5 | NLARSW_PILONC1_1 | NLARSW | PILONCIL | 22 |
| 2026 | May | BASE CASE | NELRIO | n/a | n/a | 21 |
| 2026 | May | DMTSCOS5 | 6437__F | SCRCV | KNAPP | 20 |
| 2026 | May | BASE CASE | NE_LOB | n/a | n/a | 19 |
| 2026 | May | BASE CASE | N_TO_H | n/a | n/a | 18 |
| 2026 | May | DBLW2JC5 | WAPWLY72_A | WLY | WAP | 18 |
| 2026 | May | SBRAPIN8 | HAMILT_MAVERI1_1 | HAMILTON | MAVERICK | 17 |
| 2026 | May | SRGRMGS5 | 6945__A | MGSES | CATSW | 17 |
| 2026 | May | DBAKCED5 | LAKENA_SAMATH1_1 | LAKENASW | SAMATHIS | 17 |
| 2026 | May | DWPWFWP5 | STPWAP39_1 | STP | WAP | 17 |
| 2026 | May | DBAKCED5 | 6965__A | LNGSW | PRLSW | 16 |
| 2026 | May | SBENS_M8 | BENTS_FRTER_1B_1 | FRONTERA | S_MISSIN | 16 |
| 2026 | May | DWAP_OB5 | MDOPHR99_A | MDO | PHR | 15 |
| 2026 | May | DDILCOT8 | DILLEYSW_XF1H | DILLEYSW | DILLEYSW | 15 |
| 2026 | May | BASE CASE | WESTEX | n/a | n/a | 15 |
| 2026 | May | SHONF38 | HON_DEVI_1 | HONDOCK | DEVINESW | 15 |
| 2026 | May | SFMRRYS5 | 400__A | FMRVL | RYSSW | 15 |
| 2026 | May | BASE CASE | E_PASP | n/a | n/a | 14 |
| 2026 | May | SSTAWIC8 | 138_IH2_COT_1 | IH20 | TNCOLIET | 14 |
| 2026 | May | SMAYWHI8 | NUECES_WHITE_2_1 | NUECES_B | WHITE_PT | 14 |
| 2026 | May | SBWDDBM5 | LPLMK_LPLNE_1 | LPLMK | LPLNE | 14 |
| 2026 | May | DLONPK25 | 12020__B | BRTSW | JUNPR | 14 |
| 2026 | May | DBIGKEN5 | FORTMA_YELWJC1_1 | FORTMA | YELWJCKT | 14 |
| 2026 | May | DBIGKEN5 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 14 |
| 2026 | May | SSHAFRO8 | BENTS_FRTER_1B_1 | FRONTERA | S_MISSIN | 13 |
| 2026 | May | BASE CASE | EASTEX | n/a | n/a | 13 |
| 2026 | May | DSCOTKW5 | 6215__A | BCKSW | CGRSW | 13 |
| 2026 | May | SWALWLN8 | BEN_ELMG_1 | BNDVS | ELMGROVE | 13 |
| 2026 | May | DPHRCTR5 | MDOPHR99_A | MDO | PHR | 12 |
| 2026 | May | BASE CASE | HHGTOM_1 | HHGT | OMEGA | 12 |
| 2026 | May | SCMNCPS5 | 651__B | CMNSW | CMNTP | 12 |
| 2026 | May | BASE CASE | WHARTN | n/a | n/a | 12 |
| 2026 | May | BASE CASE | HHGTOM_1 | OMEGA | HHGT | 12 |
| 2026 | May | SVEAW_L5 | 6217__A | WLVSW | GAILS | 11 |
| 2026 | May | BASE CASE | 138_FTS_LNC_1 | FTST | LEONCRK | 11 |
| 2026 | May | SPRSPAR8 | 1545__A | FLTSW | PRSSW | 11 |
| 2026 | May | BASE CASE | PNHNDL | n/a | n/a | 11 |
| 2026 | May | SMDOOAS5 | GN_PZ_08_A | GN | PZ | 11 |
| 2026 | May | BASE CASE | E_PATA | n/a | n/a | 11 |
| 2026 | May | DBIGSCH5 | PALOUS_WOLFCA1_1 | PALOUSE | WOLFCAMP | 11 |
| 2026 | May | DRNS_TB5 | THWZEN71_A | ZEN | THW | 11 |
| 2026 | May | SSNDBGR5 | 421__A | BCESW | SNDSW | 10 |
| 2026 | May | DMCDUPS8 | HECKER_RESNIK1_1 | HECKER | RESNIK | 10 |
| 2026 | May | SGRMGRS8 | 6830__B | CRDSW | OLNEY | 10 |
| 2026 | May | SBLURDH8 | 2270__B | MEXTP | ITALY | 10 |
| 2026 | May | SELMTH25 | 1025__B | FROWS | MCLSW | 10 |
| 2026 | May | DBAKCED5 | HARGRO_TWINBU1_1 | TWINBU | HARGROVE | 9 |
| 2026 | May | SMA2SAP8 | MADDUX_SAPOWE1_1 | MADDUX | SAPOWER | 9 |
| 2026 | May | BASE CASE | VALEXP | n/a | n/a | 9 |
| 2026 | May | SBIGOR55 | FORTMA_YELWJC1_1 | FORTMA | YELWJCKT | 9 |
| 2026 | May | SBIGOR55 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 9 |
| 2026 | May | SRICGRS8 | 6840__B | NVKSW | ANARN | 9 |
| 2026 | May | SHONMOO8 | BIG_FO_PLEASA1_1 | BIG_FOOT | PLEASANT | 9 |
| 2026 | May | SMA2SAP8 | MADDUX_SAPOWE1_1 | SAPOWER | MADDUX | 9 |
| 2026 | May | DGRMGRS8 | 6830__B | CRDSW | OLNEY | 9 |
| 2026 | May | DSALKLN5 | 630__B | KLNSW | HHSTH | 8 |
| 2026 | May | DSALHUT5 | 1710__A | SALDS | SONTERRA | 8 |
| 2026 | May | SN_SLON5 | ALAZAN_B_DAVI1_1 | ALAZAN | B_DAVIS | 8 |
| 2026 | May | DSTPSTA5 | BLESSING_1382 | BLESSING | BLESSING | 8 |
| 2026 | May | SSNYCGR8 | SNYDR_FMR1 | SNYDR | SNYDR | 8 |
| 2026 | May | DSALHUT5 | 270__A | KNBSW | TMPSW | 8 |
| 2026 | May | SPEBTRU8 | 940__A | ENWSW | TMPTN | 8 |
| 2026 | May | SLARDUS8 | LARDVN_LOBO2_1 | LOBO | LARDVNTH | 8 |
| 2026 | May | DRNS_TB5 | NB_THW97_A | THW | NB | 8 |
| 2026 | May | SBRAHAM8 | ESCOND_GANSO1_1 | ESCONDID | GANSO | 8 |
| 2026 | May | SBLURDH8 | 1210__B | HUBRD | HAN1 | 8 |
| 2026 | May | SCARFRI8 | ATSO_SONR1_1 | SONR | ATSO | 7 |
| 2026 | May | DWAP_BI5 | BI_JN_64_A | JN | BI | 7 |
| 2026 | May | SMDOPHR5 | 138_ALV_NAL_1 | TNNALVIN | ALVIN | 7 |
| 2026 | May | DRAZSA89 | 2585_1 | DOWNIES | MOORE | 7 |
| 2026 | May | DCSHSSP5 | 690__C | SSPSW | LBRPD | 7 |
| 2026 | May | SILLFTL8 | CARVER_TINSLE1_1 | CARVER | TINSLEY | 7 |
| 2026 | May | DBIGSCH5 | CROSSO_NORTMC1_1 | NORTMC | CROSSOVE | 7 |
| 2026 | May | SBRAHAM8 | HAMILT_MAVERI1_1 | HAMILTON | MAVERICK | 7 |
| 2026 | May | DBIGSCH5 | SANTAR_WOLFCA1_1 | WOLFCAMP | SANTARIT | 7 |
| 2026 | May | XPRS58 | 1561__A | DPREA | RCSES | 7 |
| 2026 | May | SCOLBAL8 | BALLIN_HUMBLT1_1 | BALLINGE | HUMBLTAP | 7 |
| 2026 | May | DSALHUT5 | 1715__B | TMPSW | FRYSW | 6 |
| 2026 | May | DKG_NB_5 | MDOPHR99_A | MDO | PHR | 6 |
| 2026 | May | DSTPREF5 | BLESSING_1382 | BLESSING | BLESSING | 6 |
| 2026 | May | MHARNED5 | WESLCO_HIDLBRG_1 | MV_HBRG4 | MV_WESL4 | 6 |
| 2026 | May | DLONPK25 | 6375__A | GRSES | GRMES | 6 |
| 2026 | May | MCATPRL5 | 6965__A | LNGSW | PRLSW | 6 |
| 2026 | May | DLOFOAV5 | ASHERT_CATARI1_1 | ASHERTON | CATARINA | 6 |
| 2026 | May | SHAMMAX8 | MAXWEL_WHITIN1_1 | MAXWELL | WHITING | 6 |
| 2026 | May | DOASSEB5 | WAPWLY72_A | WLY | WAP | 6 |
| 2026 | May | DBUCRGP5 | 651__B | CMNSW | CMNTP | 6 |
| 2026 | May | SLKAWFS8 | BOW_FMR1 | BOW | BOW | 6 |
| 2026 | May | SSUNRGR8 | 6240__C | SACRC | DPCRK | 6 |
| 2026 | May | DLOFOAV5 | ASHERT_CATARI1_1 | CATARINA | ASHERTON | 6 |
| 2026 | May | DFOAVLO5 | BRUNI_69_1 | BRUNI | BRUNI | 6 |
| 2026 | May | DRILKRW5 | BOW_FMR1 | BOW | BOW | 5 |
| 2026 | May | DBAKCED5 | CROSSO_PALOUS1_1 | PALOUSE | CROSSOVE | 5 |
| 2026 | May | SBLURDH8 | 1275__A | WDBSW | HLSBR | 5 |
| 2026 | May | STARLEO8 | 138_FTS_LNC_1 | LEONCRK | FTST | 5 |
| 2026 | May | DVENFTS5 | 235__A | SGRSW | JEWET | 5 |
| 2026 | May | SSNYCGR8 | 6695__B | SNYDR | AMOTP | 5 |
| 2026 | May | SWALWLN8 | SABIN_NEVAD_1 | NEVADA | SABNCRKT | 5 |
| 2026 | May | BASE CASE | MCCAMY | n/a | n/a | 5 |
| 2026 | May | DBAKSOL5 | SOLSTICE_PST1 | SOLSTICE | SOLSTICE | 5 |
| 2026 | May | DTCRTHS5 | 35055__A | SAMSW | VENSW | 5 |
| 2026 | May | SVENFTS5 | 35055__A | SAMSW | VENSW | 5 |
| 2026 | May | DMTSCOS5 | 6240__C | SACRC | DPCRK | 5 |
| 2026 | May | DFOAVLO5 | ASHERT_CATARI1_1 | CATARINA | ASHERTON | 5 |
| 2026 | May | MRNKDHM5 | 587__A | ARGYL | LWSVH | 5 |
| 2026 | May | SBAKNOR5 | CROSSO_NORTMC1_1 | NORTMC | CROSSOVE | 5 |
| 2026 | May | DSLKSOL5 | 138_FLT_FXT_1 | TNFXTAIL | FLAT_TOP | 5 |
| 2026 | May | DBIGKEN5 | TREADW_YELWJC1_1 | TREADWEL | YELWJCKT | 5 |
| 2026 | May | MLNGPR25 | 16050__B | CRTRVLLE | HILGR | 4 |
| 2026 | May | DBIGKEN5 | MADDUX_TREADW1_1 | MADDUX | TREADWEL | 4 |
| 2026 | May | SLAQLOB8 | BRUNI_69_1 | BRUNI | BRUNI | 4 |
| 2026 | May | MHARNED5 | BURNS_HEIDLBRG_1 | MV_BURNS | MV_HBRG4 | 4 |
| 2026 | May | SENWSHK8 | 940__A | ENWSW | TMPTN | 4 |
| 2026 | May | SVICCO28 | COLETO_VICTOR2_1 | COLETO | VICTORIA | 4 |
| 2026 | May | DFOWSMG5 | FWLRTN_TILDEN_1 | FOWLRTON | TILDEN | 4 |
| 2026 | May | SHC2EXC5 | 107__B | HCKSW | EXCSW | 4 |
| 2026 | May | SPAWCAL5 | COLETO_ROSATA1_1 | COLETO | ROSATA | 4 |
| 2026 | May | SGRMGRS8 | 6830__C | GRAHM | OLNEY | 4 |
| 2026 | May | DMLSENT5 | ELKTN_MR3H | ELKTN | ELKTN | 4 |
| 2026 | May | MATHEB_8 | FB_RS_60_A | RS | FB | 4 |
| 2026 | May | DFRYTM58 | SEA_AAT1 | SEA | SEA | 4 |
| 2026 | May | DZORHAY5 | BERGHE_AT1H | BERGHE | BERGHE | 4 |
| 2026 | May | DVICVI89 | COLETO_VICTOR2_1 | COLETO | VICTORIA | 4 |
| 2026 | May | SGRMGRS8 | 6830__C | OLNEY | GRAHM | 4 |
| 2026 | May | SILLFTL8 | HAMILT_MAXWEL1_1 | MAXWELL | HAMILTON | 3 |
| 2026 | May | DL_FBR89 | LA_PAL_VCAVAZ1_1 | LA_PALMA | VCAVAZOS | 3 |
| 2026 | May | SFMRRY25 | 381__A | FMRVL | RYSSW | 3 |
| 2026 | May | DCDHTVW5 | 6200__F | TYPRK | PLYAM | 3 |
| 2026 | May | DCAGCO58 | 656T656_1 | KENDAL | BERGHE | 3 |
| 2026 | May | SRICGRS8 | 6840__A | ANARN | CRDSW | 3 |
| 2026 | May | SZENTH35 | THWZEN71_A | ZEN | THW | 3 |
| 2026 | May | MDHMLWS5 | 587__A | ARGYL | LWSVH | 3 |
| 2026 | May | SCMNCPS5 | 651__C | CMNTP | SHILO | 3 |
| 2026 | May | MTGULAP5 | HARLNS_OLEAND1_1 | HARLNSW | OLEANDER | 3 |
| 2026 | May | DLOFOAV5 | NLARSW_PILONC1_1 | PILONCIL | NLARSW | 3 |
| 2026 | May | SBONRIO5 | VERTRE_WESLAU1_1 | VERTREES | WESLAU | 3 |
| 2026 | May | SCROPAL8 | SANTAR_WOLFCA1_1 | WOLFCAMP | SANTARIT | 3 |
| 2026 | May | SPRAWAL8 | 155T217_1 | BELLSO | PT | 3 |
| 2026 | May | DVLSPAC5 | 1561__A | DPREA | RCSES | 3 |
| 2026 | May | DMLSENT5 | ELKTN_MR3L | ELKTN | ELKTN | 3 |
| 2026 | May | SBRAPIN8 | ESCOND_GANSO1_1 | ESCONDID | GANSO | 3 |
| 2026 | May | DBUCRGP5 | FORTMA_YELWJC1_1 | YELWJCKT | FORTMA | 3 |
| 2026 | May | SSPUASP8 | SPUR_69_1 | SPUR | SPUR | 3 |
| 2026 | May | DLOFOAV5 | BRUNI_69_1 | BRUNI | BRUNI | 3 |
| 2026 | May | DLOFOAV5 | NLARSW_PILONC1_1 | NLARSW | PILONCIL | 3 |
| 2026 | May | SBNBPK25 | 6005__A | PKRSW | BNBSW | 3 |
| 2026 | May | SGRSPK25 | 6041__B | LONSW | PKRSW | 3 |
| 2026 | May | MHARNED5 | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 3 |
| 2026 | May | SMCLWNE8 | 1025__B | FROWS | MCLSW | 3 |
| 2026 | May | DRILKRW5 | 12020__B | BRTSW | JUNPR | 2 |
| 2026 | May | SWORBRD8 | 138_WIC_STG_1 | WICKETT | STAGHORN | 2 |
| 2026 | May | DFERWIR8 | 40T189_1 | FERGUS | WIRTZ | 2 |
| 2026 | May | SBCESN35 | 455__A | BGRSW | SNDSW | 2 |
| 2026 | May | DFOWSMG5 | COTULL_REVEIL1_1 | REVEILLE | COTULLA | 2 |
| 2026 | May | SDUKNED8 | HEC_NEDIN2_1 | HEC | NEDIN | 2 |
| 2026 | May | DELMSAN5 | PAWNEE_SPRUCE_1 | PAWNEE | CALAVERS | 2 |
| 2026 | May | SFURVAN8 | RAY_L_46_1 | RAYBURN | L_463S | 2 |
| 2026 | May | DELMSAN5 | SIGMOR_THREE_1_1 | SIGMOR | THREE_RI | 2 |
| 2026 | May | DMTSCOS5 | 6437__A | KNAPP | BCKSW | 2 |
| 2026 | May | BASE CASE | BENTS_FRTER_1B_1 | FRONTERA | S_MISSIN | 2 |
| 2026 | May | DELMTEX5 | BLESSING_1382 | BLESSING | BLESSING | 2 |
| 2026 | May | MKRWDHM5 | EXCSW_MR1L | EXCSW | EXCSW | 2 |
| 2026 | May | SWCOWCO8 | G138_15_1 | WCOLLOCL | ANGLETON | 2 |
| 2026 | May | SFTLMES8 | MIDW_OZONA1_1 | OZONA | MIDW | 2 |
| 2026 | May | DCENFRE5 | NLARSW_UNITEC1_1 | NLARSW | UNITEC | 2 |
| 2026 | May | SMOOPEA8 | PALDRO_DILLEY_1 | PALODURO | DILLEYSW | 2 |
| 2026 | May | STARLEO8 | SOLSTICE_PST1 | SOLSTICE | SOLSTICE | 2 |
| 2026 | May | DSTPREF5 | STPWAP39_1 | STP | WAP | 2 |
| 2026 | May | XFTS89 | ALPINE_BRONCO1_1 | BRONCO | ALPINE | 2 |
| 2026 | May | DCE_RIO5 | GARZA_XF1L | GARZA | GARZA | 2 |
| 2026 | May | DFOAVLO5 | LARDVN_LASCRU1_1 | LARDVNTH | LASCRUCE | 2 |
| 2026 | May | DSALHUT5 | 421__A | BCESW | SNDSW | 2 |
| 2026 | May | MLNGPR25 | 5025__A | RGRSW | PRMSW | 2 |
| 2026 | May | DCAGCI58 | 656T656_1 | KENDAL | BERGHE | 2 |
| 2026 | May | DMGSRIC8 | 6840__B | NVKSW | ANARN | 2 |
| 2026 | May | SCRDLOF9 | BOW_FMR1 | BOW | BOW | 2 |
| 2026 | May | DODEMOS5 | ODEHV_MR1H | ODEHV | ODEHV | 2 |
| 2026 | May | MLONOR58 | SND_ORAN_1 | ORNGROV | SNDIEGS | 2 |
| 2026 | May | SNEDSTE5 | WESLCO_HIDLBRG_1 | MV_HBRG4 | MV_WESL4 | 2 |
| 2026 | May | DVENFTS5 | 505__B | FBRSW | THSES | 2 |
| 2026 | May | SCLCGTN8 | 6360__A | BRKTP | BRKRG | 2 |
| 2026 | May | DKLNRGP5 | 651__B | CMNSW | CMNTP | 2 |
| 2026 | May | DRAPMO58 | GARZA_XF1L | GARZA | GARZA | 2 |
| 2026 | May | DDNEDBP5 | NLARSW_UNITEC1_1 | NLARSW | UNITEC | 2 |
| 2026 | May | BASE CASE | RAYBUR_FURHMAN_1 | FURHMAN | RAYBURN | 2 |
| 2026 | May | SGALRIC8 | RAYBUR_FURHMAN_1 | FURHMAN | RAYBURN | 2 |
| 2026 | May | DFOAVLO5 | FREER_LOBO1_1 | LOBO | FREER | 2 |
| 2026 | May | SWCOWCO8 | G138_15_1 | ANGLETON | WCOLLOCL | 2 |
| 2026 | May | SGOOFRO8 | GARZA_XF1H | GARZA | GARZA | 2 |
| 2026 | May | SRIOGUL5 | HARLNS_OLEAND1_1 | HARLNSW | OLEANDER | 2 |
| 2026 | May | DFOWSMG5 | TILDEN_GEOWEST_1 | TILDEN | GEOWEST | 2 |
| 2026 | May | MGTNHTN5 | 235__A | SGRSW | JEWET | 2 |
| 2026 | May | SBOMJC25 | 6085__E | WFSSW | NSTAR | 2 |
| 2026 | May | DSCOTKW5 | 6474__B | SUNSW | RGRSW | 2 |
| 2026 | May | XCB2Y58 | BT_CBY88_A | CBY | BT | 2 |
| 2026 | May | DSTPREF5 | CKT_3124_1 | STP | HLJ | 2 |
| 2026 | May | SHAMMAX8 | CTHR_TINSLE1_1 | CTHR | TINSLEY | 2 |
| 2026 | May | SPAWCAL5 | F2_11_1 | KENEDSW | F2 | 2 |
| 2026 | May | DFOWSMG5 | GEO_SIG_1 | GEOWEST | SIGMOR | 2 |
| 2026 | May | DSTPSTA5 | RAYBUR_FURHMAN_1 | FURHMAN | RAYBURN | 2 |
| 2026 | May | SFORRAY8 | RAY_L_46_1 | RAYBURN | L_463S | 2 |
| 2026 | May | SHCKEXC5 | 106__C | HCKSW | EXCSW | 2 |
| 2026 | May | DSCOTKW5 | 6240__C | SACRC | DPCRK | 2 |
| 2026 | May | DGRMGRS8 | 6830__C | OLNEY | GRAHM | 2 |
| 2026 | May | SP5CAG8 | D5_S1_1 | HOWARD | LEON_CRK | 2 |
| 2026 | May | DWO5_EU8 | DT_PK_91_A | PK | DT | 2 |
| 2026 | May | DFRIILL8 | HAMILT_MAXWEL1_1 | MAXWELL | HAMILTON | 2 |
| 2026 | May | SCISPUT8 | LENSW_PUTN2_1 | PUTN | LENSW | 2 |
| 2026 | May | SKGJOR5 | MDOPHR99_A | MDO | PHR | 2 |
| 2026 | May | SWOLPAL8 | MIDW_OZONA1_1 | OZONA | MIDW | 2 |
| 2026 | May | DCEDERE5 | NLARSW_UNITEC1_1 | NLARSW | UNITEC | 2 |
| 2026 | May | XTH158 | ADK_AT1 | ADK | ADK | 1 |
| 2026 | May | SGRSMNW8 | AGN_WTH_1 | WTHSW | AGNESSW | 1 |
| 2026 | May | SMARALM9 | ALMC_PAIS1_1 | ALMC | PAIS | 1 |
| 2026 | May | SFORFOR8 | BROOKT_JOSLIN1_1 | JOSLIN | BROOKTAP | 1 |
| 2026 | May | XFER85 | CARVER_TINSLE1_1 | CARVER | TINSLEY | 1 |
| 2026 | May | MLNGPR25 | CRTVLE_EINSTEN_1 | EINSTEIN | CRTRVLLE | 1 |
| 2026 | May | SGULLA_5 | HARLNS_OLEAND1_1 | HARLNSW | OLEANDER | 1 |
| 2026 | May | BASE CASE | I_KALO | n/a | n/a | 1 |
| 2026 | May | SMV_PAR8 | LA_PAL_VCAVAZ1_1 | LA_PALMA | VCAVAZOS | 1 |
| 2026 | May | DLOBCEN5 | LON_HI_WWKS_T1_1 | LON_HILL | WWKS_TAP | 1 |
| 2026 | May | UNEDBES1 | NEDNBRG_WEDNBG_1 | NEDIN | MV_WEDN4 | 1 |
| 2026 | May | SWHILON5 | NUECES_WHITE_2_1 | NUECES_B | WHITE_PT | 1 |
| 2026 | May | DHJWFCK5 | STPWAP39_1 | STP | WAP | 1 |
| 2026 | May | SHLSWH28 | 2270__B | MEXTP | ITALY | 1 |
| 2026 | May | DVENFTS5 | 255__A | BALSW | SGRSW | 1 |
| 2026 | May | SRA2D18 | 2585_1 | DOWNIES | MOORE | 1 |
| 2026 | May | STMPTMP5 | 300__A | TMPCR | TMPSW | 1 |
| 2026 | May | DSALHUT5 | 445__A | BLHSW | AUSTRO | 1 |
| 2026 | May | DSALHUT5 | 450__B | WCRSW | AUSTRO | 1 |
| 2026 | May | DJOHMAR5 | 4T250_1 | ZORN | CLEASP | 1 |
| 2026 | May | MLWSWDE5 | 587__A | ARGYL | LWSVH | 1 |
| 2026 | May | DBYRBOW5 | 6011__B | RILEY | FSHSW | 1 |
| 2026 | May | DCDHTVW5 | 6200__E | PRKRW | TYPRK | 1 |
| 2026 | May | DODEMOS5 | 6513__A | ODESA | ODNTH | 1 |
| 2026 | May | SWCAE8 | AE_STR26_A | AE | STR | 1 |
| 2026 | May | SSFB1I8 | BI_SF_37_A | BI | SF | 1 |
| 2026 | May | SPAWCAL5 | BLESSING_1382 | BLESSING | BLESSING | 1 |
| 2026 | May | SHA2MAX8 | CTHR_TINSLE1_1 | CTHR | TINSLEY | 1 |
| 2026 | May | SFURRAY8 | FURHMAN_VANDB_1 | FURHMAN | VANBLTSS | 1 |
| 2026 | May | SJUNYEL9 | HEXT_YELWJC1_1 | YELWJCKT | HEXT | 1 |
| 2026 | May | DBUCRGP5 | MADDUX_TREADW1_1 | MADDUX | TREADWEL | 1 |
| 2026 | May | DRIZACE5 | NLARSW_UNITEC1_1 | NLARSW | UNITEC | 1 |
| 2026 | May | SFTLMES8 | PALOUS_WOLFCA1_1 | PALOUSE | WOLFCAMP | 1 |
| 2026 | May | UCOLCOL1 | RAY_L_46_1 | RAYBURN | L_463S | 1 |
| 2026 | May | MLAPVCA8 | RIOHND_ERIOHND_1 | RIOHONDO | MV_RIOHO | 1 |
| 2026 | May | MCPRJCK8 | SN_STR26_A | SN | STR | 1 |
| 2026 | May | XVIS84 | GREENL_WEAVER1_1 | GREENLK | WEAVERRD | 1 |
| 2026 | May | DBIGKEN5 | HAMILT_MAXWEL1_1 | MAXWELL | HAMILTON | 1 |
| 2026 | May | BASE CASE | LA_PAL_VCAVAZ1_1 | LA_PALMA | VCAVAZOS | 1 |
| 2026 | May | DBAKSOL5 | LYNX_TOMBST1_1 | LYNX | TOMBSTNE | 1 |
| 2026 | May | SCOMHA38 | MAXWEL_WHITIN1_1 | MAXWELL | WHITING | 1 |
| 2026 | May | DMCEBUT8 | MKLT_TRNT1_1 | TRNT | MKLT | 1 |
| 2026 | May | MTGURIO5 | VERTRE_WESLAU1_1 | VERTREES | WESLAU | 1 |
| 2026 | May | SANGNAI5 | WHITE_PT_T3L | WHITE_PT | WHITE_PT | 1 |
| 2026 | May | BASE CASE | ZAPSTR | n/a | n/a | 1 |
| 2026 | May | SHLSWH28 | 1210__B | HUBRD | HAN1 | 1 |
| 2026 | May | DHWKRCS8 | 1545__A | FLTSW | PRSSW | 1 |
| 2026 | May | DCAGCI58 | 255T279_1 | PIPECR | MEDILA | 1 |
| 2026 | May | SBRTGRM8 | 6275__D | BCPSW | AZLES | 1 |
| 2026 | May | DPAIMUR8 | 6635__G | MRVLY | ESTLD | 1 |
| 2026 | May | SSCLWF18 | 6840__B | NVKSW | ANARN | 1 |
| 2026 | May | SZORMAR5 | BERGHE_AT1H | BERGHE | BERGHE | 1 |
| 2026 | May | DWPWFWP5 | BLESSING_1382 | BLESSING | BLESSING | 1 |
| 2026 | May | MGTKBHC8 | BR_HOC09_A | BR | HOC | 1 |
| 2026 | May | DCOTWR25 | FARMLAND_LONGD_1 | FARMLAND | W_LD_345 | 1 |
| 2026 | May | SMDOOAS5 | G138_8A_1 | PHR | HDNLAKES | 1 |
| 2026 | May | DCE_RIO5 | GARZA_XF1H | GARZA | GARZA | 1 |
| 2026 | May | DROMCEN5 | GARZA_XF1H | GARZA | GARZA | 1 |
| 2026 | May | SGOOFRO8 | GARZA_XF1L | GARZA | GARZA | 1 |
| 2026 | May | DYETRJU8 | HEXT_YELWJC1_1 | YELWJCKT | HEXT | 1 |
| 2026 | May | SOXYIN28 | I_DUPP_I_DUPS1_1 | I_DUPP1 | I_DUPSW | 1 |
| 2026 | May | DNOETWL5 | LAKENA_SAMATH1_1 | LAKENASW | SAMATHIS | 1 |
| 2026 | May | SBOSWHT8 | LKW_WHT_1 | LKWHITNY | WHTNY | 1 |
| 2026 | May | DBLBYWF5 | STPWAP39_1 | STP | WAP | 1 |
| 2026 | May | SFURRAY8 | VAN_VNDB_1 | VANBLTSS | VANBLT69 | 1 |
| 2026 | May | MKRWDHM5 | 109__A | EXCSW | RNKSW | 1 |
| 2026 | May | SKLNSAL5 | 271__A | KLNSW | SALSW | 1 |
| 2026 | May | SBOMJC25 | 35020__B | GRVSW | GRSES | 1 |
| 2026 | May | MAUSWCR5 | 445__A | BLHSW | AUSTRO | 1 |
| 2026 | May | SMARALM9 | ALMC_PAIS1_1 | PAIS | ALMC | 1 |
| 2026 | May | DFRYTM58 | BELCNTY_XFMR | BELCNTY | BELCNTY | 1 |
| 2026 | May | DREFSTP5 | BLESSING_1382 | BLESSING | BLESSING | 1 |
| 2026 | May | XJN58 | BR_HOC09_A | BR | HOC | 1 |
| 2026 | May | DFOAVLO5 | CATARI_PILONC1_1 | PILONCIL | CATARINA | 1 |
| 2026 | May | SFTLMES8 | CROSSO_NORTMC1_1 | NORTMC | CROSSOVE | 1 |
| 2026 | May | XDIL89 | DILLEYSW_XF1H | DILLEYSW | DILLEYSW | 1 |
| 2026 | May | DWPWFWP5 | DOWOAS18_A | DOW | OAS | 1 |
| 2026 | May | MBIKBHC8 | DT_PK_91_A | DT | PK | 1 |
| 2026 | May | DSALGA58 | GABRIE_AT1 | GABRIE | GABRIE | 1 |
| 2026 | May | SBRAPIN8 | GANSO_MAVERI1_1 | MAVERICK | GANSO | 1 |
| 2026 | May | MNLFIWO8 | GARZA_XF1L | GARZA | GARZA | 1 |
| 2026 | May | DTWLCED5 | HARGRO_TWINBU1_1 | TWINBU | HARGROVE | 1 |
| 2026 | May | MTGURIO5 | HARLNS_OLEAND1_1 | HARLNSW | OLEANDER | 1 |
| 2026 | May | SCOLBAL8 | HUMBLT_NOVICT1_1 | HUMBLTAP | NOVICTAP | 1 |
| 2026 | May | DBIGKEN5 | MAXWEL_WHITIN1_1 | MAXWELL | WHITING | 1 |
| 2026 | May | BASE CASE | MDOPHR99_A | MDO | PHR | 1 |
| 2026 | May | MPASTNE5 | PANTER_WESMER1_1 | WESMER | PANTERA | 1 |
| 2026 | May | SNEDSTE5 | PANTER_WESMER1_1 | WESMER | PANTERA | 1 |
| 2026 | May | DELMTEX5 | RAYBUR_FURHMAN_1 | FURHMAN | RAYBURN | 1 |
| 2026 | May | DMSSZEN5 | SNGXGC75_1 | GIBCRK | SNG | 1 |
| 2026 | May | DWPWFCK5 | STPWAP39_1 | STP | WAP | 1 |
| 2026 | May | MPASTNE5 | VERTRE_WESLAU1_1 | VERTREES | WESLAU | 1 |
| 2026 | May | DTHSLCS5 | 1020__A | ELMOT | MCTYE | 1 |
| 2026 | May | DWPWFWP5 | 155T217_1 | BELLSO | PT | 1 |
| 2026 | May | SWDDMNS5 | 1561__A | DPREA | RCSES | 1 |
| 2026 | May | DRILKRW5 | 6085__E | WFSSW | NSTAR | 1 |
| 2026 | May | SCMNCPS5 | 651__F | SHILO | HAS | 1 |
| 2026 | May | XCMB58 | 661__A | STNVL | CPSES | 1 |
| 2026 | May | SPEBTRU8 | 940__B | TMPTN | WXHCH | 1 |
| 2026 | May | MDHLYTC5 | 970__C | ELKTN | LLPOI | 1 |
| 2026 | May | SWMMLCS8 | 1025__B | FROWS | MCLSW | 1 |
| 2026 | May | DREDNC85 | 1350__E | NCSTP | LFKSW | 1 |
| 2026 | May | DALASOA8 | 138_FTS_LNC_1 | FTST | LEONCRK | 1 |
| 2026 | May | SPIGTNF8 | 138_FXT_PG2_1 | PIGCREEK | TNFXTAIL | 1 |
| 2026 | May | SBAKCED5 | BAKRFLD_CEDCAN_1 | CEDACA | BAKESW | 1 |
| 2026 | May | DSTEXP12 | BLESSI_LOLITA1_1 | LOLITA | BLESSING | 1 |
| 2026 | May | XCRD58 | CRD_CRD2 | CRD | CRD | 1 |
| 2026 | May | SCISPUT8 | ESTES_SOLANU1_1 | ESTES | SOLANUM | 1 |
| 2026 | May | SV_DVI28 | GREENL_WEAVER1_1 | WEAVERRD | GREENLK | 1 |
| 2026 | May | DBAKCED5 | JERRY_PUMPJA1_1 | PUMPJACK | JERRY | 1 |
| 2026 | May | DRYSALN5 | NEWM_WAL_1 | NEWMAN | WALNUT1 | 1 |
| 2026 | May | DPROPOM5 | NLARSW_UNITEC1_1 | NLARSW | UNITEC | 1 |
| 2026 | May | SL_4RAY8 | RAYBURN_69_2 | RAYBURN | RAYBURN | 1 |
| 2026 | May | SORNLON8 | SND_ORAN_1 | ORNGROV | SNDIEGS | 1 |
| 2026 | May | DBUCRGP5 | TREADW_YELWJC1_1 | TREADWEL | YELWJCKT | 1 |
| 2026 | May | DFVLFTS5 | 2270__B | MEXTP | ITALY | 1 |
| 2026 | May | DVENFTS5 | 2270__B | MEXTP | ITALY | 1 |
| 2026 | May | DSALHUT5 | 455__A | BGRSW | SNDSW | 1 |
| 2026 | May | SBOMJC25 | 6085__B | NSTAR | LKARH | 1 |
| 2026 | May | DGRSLO25 | 6375__B | GRMES | GRMSW | 1 |
| 2026 | May | SFLTTUD8 | 6635__G | MRVLY | ESTLD | 1 |
| 2026 | May | SHLSWDB8 | 810__G | RDHSW | BLUMS | 1 |
| 2026 | May | DBNTTOW9 | BOW_FMR1 | BOW | BOW | 1 |
| 2026 | May | SMDOOAS5 | BR_HOC09_A | BR | HOC | 1 |
| 2026 | May | DHONHON8 | DEV_NATA_1 | DEVINESW | NATALISS | 1 |
| 2026 | May | DHUTGEA8 | GABRIE_AT1 | GABRIE | GABRIE | 1 |
| 2026 | May | SGA2ROM8 | GARZA_XF1H | GARZA | GARZA | 1 |
| 2026 | May | DTWLCED5 | LAKENA_SAMATH1_1 | LAKENASW | SAMATHIS | 1 |
| 2026 | May | SBGLTWI8 | LAKENA_SAMATH1_1 | LAKENASW | SAMATHIS | 1 |
| 2026 | May | DSGTSCH5 | SANTAR_WOLFCA1_1 | WOLFCAMP | SANTARIT | 1 |
| 2026 | May | SGULLA_5 | VERTRE_WESLAU1_1 | VERTREES | WESLAU | 1 |
| 2026 | May | SSOAHOL8 | 138_FTS_LNC_1 | FTST | LEONCRK | 1 |
| 2026 | May | STNFTAR8 | 138_FTS_LNC_1 | FTST | LEONCRK | 1 |
| 2026 | May | DTHSLCS5 | 281__A | THSES | LHLSW | 1 |
| 2026 | May | DCPSES12 | 35055__A | SAMSW | VENSW | 1 |
| 2026 | May | SBCEBGR5 | 421__A | BCESW | SNDSW | 1 |
| 2026 | May | DJCKHND5 | 6041__B | LONSW | PKRSW | 1 |
| 2026 | May | SVEAW_L5 | 6217__B | GAILS | KEYSB | 1 |
| 2026 | May | DVENFTS5 | 810__G | RDHSW | BLUMS | 1 |
| 2026 | May | SPETSNU8 | AE_STR26_A | AE | STR | 1 |
| 2026 | May | SBENRAI8 | BENTS_FRTER_1B_1 | FRONTERA | S_MISSIN | 1 |
| 2026 | May | SE3X28 | BIG_FO_PLEASA1_1 | BIG_FOOT | PLEASANT | 1 |
| 2026 | May | SEBATH8 | FB_RS_60_A | RS | FB | 1 |
| 2026 | May | MNLFIWO8 | GARZA_XF1H | GARZA | GARZA | 1 |
| 2026 | May | SLISBAT8 | GARZA_XF1H | GARZA | GARZA | 1 |
| 2026 | May | SGA2ROM8 | GARZA_XF1L | GARZA | GARZA | 1 |
| 2026 | May | SRAYRI38 | HAINE__LA_PAL1_1 | LA_PALMA | HAINE_DR | 1 |
| 2026 | May | SGRIRAP5 | MELONC_SEADRF1_1 | SEADRFTC | MELONCRE | 1 |
| 2026 | May | SRIOGUL5 | RIOHND_ERIOHND_1 | RIOHONDO | MV_RIOHO | 1 |
| 2026 | May | BASE CASE | VALIMP | n/a | n/a | 1 |
| 2026 | May | DFVLFTS5 | 1210__B | HUBRD | HAN1 | 1 |
| 2026 | May | SW_BLNG5 | 16050__B | CRTRVLLE | HILGR | 1 |
| 2026 | May | DFVLFTS5 | 235__A | SGRSW | JEWET | 1 |
| 2026 | May | DNORCDH5 | 3610__G | PGTP1 | PDSES | 1 |
| 2026 | May | SVEAW_L5 | 6217__D | KEYSB | LMESA | 1 |
| 2026 | May | DMTSCOS5 | 6474__B | SUNSW | RGRSW | 1 |
| 2026 | May | DODEMOS5 | 6520__H | RTRSW | YARBR | 1 |
| 2026 | May | MRGRMGS5 | 6945__A | MGSES | CATSW | 1 |
| 2026 | May | SHLSWDB8 | 810__B | BLUMS | RVSTP | 1 |
| 2026 | May | DFVLFTS5 | 810__G | RDHSW | BLUMS | 1 |
| 2026 | May | BASE CASE | BR_HOC09_A | BR | HOC | 1 |
| 2026 | May | SCITWEI8 | CLARK__LON_HI1_1 | LON_HILL | CLARK_WD | 1 |
| 2026 | May | DLOBCEN5 | FALFUR_RACHAL1_1 | RACHAL | FALFUR | 1 |
| 2026 | May | SAEDAB8 | FW_WEB04_A | FW | WEB | 1 |

Current Wind Generation Record: 29,011 MW on 05/18/2026 at 00:01 | Current Wind Penetration Record: 69.15% on 04/10/2022 at 01:43

Current Solar Generation Record: 33,981 MW on 05/13/2026 at 12:43 | Current Solar Penetration Record: 72.74% on 05/02/2026 at 11:28

All DC Tie Curtailments are posted publicly on the ERCOT Market Information System.  See that posting for additional details for the event(s) in question.

3 See DC Tie Operating Procedure (http://www.ercot.com/mktrules/guides/procedures) for more details.