ERCOT Planning Guide

# Table of Contents

# August 1, 2026

1	Overview	1-1

1.1	Purpose	1

1.2	Process for Planning Guide Revision	1

1.2.1	Introduction	1

1.2.2	Submission of a Planning Guide Revision Request	2

1.2.3	Planning Guide Revision Procedure	3

1.2.3.1	Review and Posting of Planning Guide Revision Requests	3

1.2.3.2	Withdrawal of aPlanning Guide Revision Request	4

1.2.3.3	ROS Review and Action	4

1.2.3.4	Comments to the ROS Report	5

1.2.3.5	Planning Guide Revision Request Impact Analysis	5

1.2.3.6	ROS Review of Impact Analysis	6

1.2.3.7	Wholesale Market Subcommittee Review	7

1.2.3.8	ERCOT Impact Analysis Based on ROS Report	7

1.2.3.9	PRS Review of Project Prioritization	7

1.2.3.10	Technical Advisory Committee Vote	7

1.2.3.11	ERCOT Impact Analysis Based on Technical Advisory Committee Report	8

1.2.3.12	ERCOT Board Vote	9

1.2.3.13	PUCT Approval of Revision Requests	9

1.2.3.14	Appeal of Action	9

1.2.5	Urgent Requests	10

1.2.6	Planning Guide Revision Implementation	11

2	Definitions and Acronyms	2-1

3	Regional Planning	3-1

3.1	COMMUNICATIONS	3-1

3.1.1 Overview of Major Transmission Planning Activities	3-1

3.1.1.1	Long-Term System Assessment	3-1

3.1.1.2	Regional Transmission Plan	3-2

3.1.1.3	Regional Planning Group Project Reviews	3-2

3.1.1.4	Generation Interconnection Process	3-3

3.1.1.5	Geomagnetic Disturbance (GMD) Vulnerability Assessment	3-3

3.1.1.6	Grid Reliability and Resiliency Assessment (GRRA)	3-3

3.1.2	Regional Planning Group Project Submission	3-5

3.1.2.1	All Projects	3-5

3.1.3	Project Evaluation	3-6

3.1.3.1	Definitions of Reliability-Driven and Economic-Driven Projects	3-8

3.1.3.2	Reliability-Driven Project Evaluation	3-8

3.1.4	Regional Transmission Plan Development Process	3-8

3.1.4.1	Development of Regional Transmission Plan	3-9

3.1.4.1.1	Regional Transmission Plan Cases	3-9

3.1.4.2	Use of Regional Transmission Plan	3-11

3.1.5	Regional Planning Group Comment Process	3-11

3.1.6	Notify PUCT of Recommended Transmission Projects	3-12

3.1.7	Steady State Transmission Planning Load Forecast	3-12

3.1.8	Planning Geomagnetic Disturbance (GMD) Activities	3-13

3.1.9	Transmission Interconnection Study	3-15

4	Transmission Planning Criteria	4-1

4.1	Introduction	1

4.1.1	Reliability Criteria	2

4.1.1.1	Planning Assumptions	2

4.1.1.2	Reliability Performance Criteria	3

4.1.1.3	Voltage Stability Margin	8

4.1.1.4	Steady State Voltage Response Criteria	8

4.1.1.5	Transient Voltage Response Criteria	9

4.1.1.6	Damping Criteria	9

4.1.1.7	Minimum Deliverability Criteria	9

4.1.1.8	Maintenance Outage Reliability Criteria	10

4.1.2	Resiliency Criteria	11

5	Generator Interconnection or Modification	5-1

5.1	INTRODUCTION	5-1

5.2	GENERAL PROVISIONS	5-1

5.2.1	Applicability	5-1

5.2.2	Initiation of Generator Interconnection or Modification	5-3

5.2.3	Confidentiality	5-5

5.2.4	Duty to Update Project Information and Respond to ERCOT and TDSP Requests for Information	5-6

5.2.5	Inactive Status	5-7

5.2.6	Project Cancellation Due to Failure to Comply with Requirements	5-9

5.2.7	Voluntary Project Cancellation	5-9

5.2.8	Interconnection Agreements and Procedures	5-10

5.2.8.1	Standard Generation Interconnection Agreement for Transmission-Connected Generators	5-10

5.2.8.2	Interconnection Agreement for Distribution-Connected Generators	5-10

5.2.8.3	Provisions for Municipally Owned Utilities and Cooperatives	5-11

5.2.8.4	Interconnection Cost Reporting for Transmission-Connected Generators	5-11

5.2.9	Self-Limiting Facilities	5-11

5.2.9	Self-Limiting Facilities	5-12

5.2.10	Subsynchronous Oscillation (SSO) Risk Reduction	5-12

5.2.11	Required Interconnection Equipment	5-13

5.3	INTERCONNECTION STUDY PROCEDURES FOR LARGE GENERATORS	5-13

5.3.1	Security Screening Study	5-13

5.3.2	Full Interconnection Study	5-15

5.3.2.1	Proof of Site Control	5-17

5.3.2.2	Full Interconnection Study Scoping Process	5-18

5.3.2.3	Full Interconnection Study Description and Methodology	5-20

5.3.2.4	Full Interconnection Study Elements	5-20

5.3.2.4.1	Steady-State Analysis	5-20

5.3.2.4.2	System Protection (Short-Circuit) Analysis	5-21

5.3.2.4.3	Dynamic and Transient Stability (Unit Stability, Voltage) Analysis	5-21

5.3.2.4.4	Facility Study	5-23

5.3.2.5	FIS Report and Follow-up	5-23

5.3.3	ERCOT Economic Study	5-25

5.3.4	Reactive Study	5-25

5.3.5	ERCOT Quarterly Stability Assessment	5-26

5.4	INTERCONNECTION PROCEDURES FOR SMALL GENERATORS	5-30

5.4.1	Small Generator Review Meetings	5-30

5.4.2	Submission of Interconnection Agreement and TSP and/or DSP Studies and Technical Requirements	5-30

5.4.3	Reviews and Approval to Submit Model Information	5-30

5.4.4	Transmission System Reliability Impact	5-31

5.5	GENERATOR COMMISSIONING AND CONTINUING OPERATIONS	5-3

6	Data/Modeling	6-2

6.1	Steady-State Model Development	6-2

6.2	Dynamics Model Development	6-4

6.2.1	Dynamics Data Requirements for Generation Resources, Energy Storage Resources, and Settlement Only Generators	6-8

6.2.2	Dynamics Data Requirements for Load Resources	6-8

6.2.3	Dynamics Data Requirements for Transmission and/or Distribution Service Providers	6-9

6.2.3	Dynamics Data Requirements for Transmission and/or Distribution Service Providers, Direct Current Tie Facility Owners, and Direct Current Tie Operators	6-9

6.2.4	Dynamics Data Screening and Maintenance	6-10

6.3	Process for Developing Short Circuit Cases	6-10

6.4	Transmission Project and Information Tracking Report and Data Requirements	6-11

6.4.1	Transmission Project and Information Tracking Report	6-11

6.4.2	ERCOT Responsibilities	6-12

6.4.3	TSP Responsibilities	6-12

6.4.4	Regional Transmission Plan Projects in Transmission Project and Information Tracking Report	6-12

6.4.5	Content of the Transmission Project and Information Tracking Report	6-12

6.5	Annual Load Data Request	6-13

6.6	Modeling of Large Loads	6-14

6.6.1	Modeling of Large Loads Not Co-Located with a Generation Resource, Energy Storage Resource (ESR), or Settlement Only Generator (SOG)	6-14

6.6.2	Modeling of Large Loads Co-Located with an Existing Generation Resource, Energy Storage Resource (ESR), or Settlement Only Generator (SOG)	6-15

6.6.2.1 	Modeling of Large Loads within a Withdrawal-Limited Private Use Network	6-16

6.6.3	Modeling of Large Loads Co-Located with a Proposed Generation Resource, Energy Storage Resource (ESR), or Settlement Only Generator (SOG)	6-16

6.7	Data Dictionary	6-16

6.8	Resource Registration Procedures	6-16

6.8.1	Resource Registration	6-17

6.8.2	Resource Registration Process	6-17

6.9	Addition of Proposed Generation to the Planning Models	6-18

6.10	Contingency Filing Requirements	6-21

6.11	Process for Developing Geomagnetically-Induced Current (GIC) System Models	6-21

6.12	Addition of a Proposed DC Tie to the Planning Models	6-23

7	Market Data Transparency	7-1

7.1	Planning Data and Information	1

8	Attachments	8-1

Declaration of Resource Data Accuracy	8(A)

Declaration of Adequate Water Supplies	8(B)

Declaration of Department of Defense Notification	8(C)

Attestation Regarding Compliance with the Lone Star Infrastructure Protection Act	8(D)

9	Large Load Additions At New Or Modification Of Existing Load Interconnection(s)	9-1

9.1	INTRODUCTION	1

9.2	GENERAL PROVISIONS	2

9.2.1	Applicability of the Batch Zero Process	2

9.2.1.1	Eligibility Criteria for Inclusion of a Large Load as Base Load not Subject to Additional Study in the Batch Zero Process	3

9.2.1.2	Eligibility Criteria for Inclusion as Load to be Studied and Allocated in Batch Zero	14

9.2.1.3	Load Not Included in Batch Zero	16

9.2.1.4	Evaluation of Existing Interconnection Studies for Large Loads	16

9.2.2	Submission of Large Load Information for Batch Zero Process	19

9.2.2.1	Additional Information Required for Provisional Controllable Load Resources (PCLRs)	21

9.2.2.2	Additional Information Required for Withdrawal-Limited Private Use Networks (WLPUNs)	21

9.2.3	Modification of Large Load Information	22

9.2.4	Load Commissioning Plan	22

9.2.5	Required Interconnection Equipment	23

9.3	BATCH ZERO INTERCONNECTION STUDY	24

9.3.1	Batch Zero Process Overview and Timelines	24

9.3.2	Batch Zero Interconnection Study Methodology	25

9.3.2.1	Treatment of Provisional Controllable Load Resources (PCLRs) in the Batch Zero Interconnection Study	27

9.3.2.2	Treatment of Withdrawal-Limited Private Use Networks (WLPUNs) in the Batch Zero Interconnection Study	28

9.4	BATCH ZERO REPORT AND INTERCONNECTING LARGE LOAD ENTITY (ILLE) COMMITMENT	28

9.4.1	Additional Commitments for Provisional Controllable Load Resources (PCLRs)	30

9.4.2	Additional Commitments for Withdrawal-Limited Private Use Networks (WLPUNs)	31

9.5	BATCH ZERO STUDY REFINEMENT AND DELIVERY OF TRANSMISSION PLAN	32

9.5.1	ERCOT Activities During the Batch Zero Refinement Period	33

9.5.2	System Protection (Short-Circuit) Analysis	34

9.5.3	Treatment of Provisional Controllable Load Resources (PCLRs) in the Batch Zero Refinement Study	34

9.5.4	Treatment of Withdrawal-Limited Private Use Networks (WLPUNs) in the Batch Zero Refinement Study	34

9.6	INITIAL ENERGIZATION AND CONTINUING OPERATIONS FOR LARGE LOADS	34

9.6.1	Additional Energization and Operation Requirements for Provisional Controllable Load Resources (PCLRs)	35

9.6.2	Additional Energization and Operation Requirements for Withdrawal-Limited Private Use Networks (WLPUNs)	36

9.7	REQUIRED DISCLOSURES	37

9.8	LEGACY INTERCONNECTION STUDY PROCEDURES FOR LARGE LOADS	39

9.8.1	Legacy Large Load Interconnection Study (LLIS)	39

9.8.2	Legacy Large Load Interconnection Study Scoping Process	40

9.8.3	Legacy Large Load Interconnection Study Description and Methodology	41

9.8.4.1	Legacy Steady-State Analysis	42

9.8.4.2	Legacy System Protection (Short-Circuit) Analysis	42

9.8.4.3	Legacy Dynamic and Transient Stability Analysis	43

9.9	LEGACY LLIS REPORT AND FOLLOW-UP	44

9.10	LEGACY INTERCONNECTION AGREEMENTS AND RESPONSIBILITIES	46