| NPRR Number | 1335 | NPRR Title | Implementation of PUCT Changes to Firm Fuel Supply Service for Phase 3 |
| --- | --- | --- | --- |
| Date of Decision |  | September 15, 2026 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Urgent |  |
| Estimated Impacts |  | Cost/Budgetary: Less than $10K to Operations & Maintenance (O&M) Project Duration: No project required |  |
| Proposed Effective Date |  | The day following Public Utility Commission of Texas (PUCT) approval |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Nodal Protocol Sections Requiring Revision |  | 1.3.1.2, Items Not Considered Protected Information 2.1, Definitions 3.14, Contracts for Reliability Resources and Emergency Response Service Resources 3.14.5, Firm Fuel Supply Service 6.6.14.1, Firm Fuel Supply Service Fuel Replacement Costs Recovery 6.6.14.2, Firm Fuel Supply Hourly Standby Fee Payment and Fuel Replacement Cost Recovery 8.1.1.2.1.6 Firm Fuel Supply Service Resource Qualification, Testing, Decertification, and Recertification 22, Attachment F, Firm Fuel Supply Service Agreement (new) |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) aligns the Protocols with the PUCT’s recent adoption of a rule to govern Firm Fuel Supply Service (FFSS): 16 Texas Administrative Code (TAC) § 25.520. In some cases, the PUCT’s rule codifies existing FFSS practices and therefore no changes to the Protocols are needed. In other cases though, the PUCT’s rule institutes Phase 3 of FFSS’s development, namely to permit additional Resource types to participate in FFSS while reserving a portion of the budget for certain other Resource types. Accordingly, this NPRR updates the criteria for a Resource to participate in the FFSS program. The rule requires ERCOT to procure FFSS prior to each winter season to help maintain reliability during, or in preparation for, a natural gas curtailment or other fuel supply disruption. ERCOT may spend a maximum of $54 million in standby payments to procure FFSS during a single winter season, not including the incremental cost of restocking fuel (i.e., FFSS Non-Procurement Costs), as approved by ERCOT. Unless insufficient offers are received, ERCOT must ensure that at least 70% of the $54 million budget for FFSS Procurement Costs is allocated to the following types of Resources with their own offer cap: - On-site FFSS category which successfully demonstrates dual fuel capability, the ability to establish and burn an alternative on-site stored fuel, and has on-site fuel storage capability; and - Resource-controlled FFSS category which must have an on-site natural gas or fuel oil storage capability or off-site natural gas storage in which the Resource Entity or Qualified Scheduling Entity (QSE) owns and controls both the natural gas storage and the transportation pipeline to deliver the required amount of reserved natural gas to the Generation Resource from the storage facility in an amount that satisfies the minimum FFSS capability requirements. Any remaining budget amounts should be allocated to contractual off-site FFSS category. The NPRR also describes how ERCOT must calculate the FFSS Offer Caps as a function of maximum hours per deployment, heat rate, and fuel price. The NPRR includes an ERCOT requirement to establish a single market clearing price mechanism for Resources eligible to provide FFSS under the on-site FFSS category and the Resource-controlled FFSS category. ERCOT must establish a separate market clearing price mechanism for Resources eligible to provide FFSS under the contractual off-site FFSS category. Finally, this NPRR establishes an FFSS Agreement, as directed by 16 TAC § 25.520, and adds that the Agreement itself is not considered Protected Information under the Protocols. |  |
| Reason for Revision |  | Strategic Plan Objective 1 – Be an industry leader for grid reliability and resilience Strategic Plan Objective 2 - Enhance the ERCOT region’s economic competitiveness with respect to trends in wholesale power rates and retail electricity prices to consumers Strategic Plan Objective 3 - Advance ERCOT, Inc. as an independent leading industry expert and an employer of choice by fostering innovation, investing in our people, and emphasizing the importance of our mission General system and/or process improvement(s) Regulatory requirements ERCOT Board/PUCT Directive (please select ONLY ONE – if more than one apply, please select the ONE that is most relevant) |  |
| Justification of Reason for Revision and Market Impacts |  | ERCOT is required by 16 TAC § 25.520(j) to develop Revision Requests to ensure that the Protocols are consistent with the PUCT’s FFSS rule. |  |
| PRS Decision |  | On 6/10/26, PRS voted unanimously to table NPRR1335 and refer the issue to WMS. All Market Segments participated in the vote. On 8/12/26, PRS voted unanimously to grant NPRR1335 Urgent status; to recommend approval of NPRR1335 as amended by the 7/1/26 ERCOT comments; and to forward to TAC NPRR1335 and the 5/26/26 Impact Analysis with a recommended effective date of the day after PUCT approval. All Market Segments participated in the vote. |  |
| Summary of PRS Discussion |  | On 6/10/26, ERCOT Staff provided an overview of NPRR1335. Participants requested additional review by WMS. On 8/12/26, participants noted the WMS endorsement of the 7/1/26 ERCOT comments, and discussed the request for Urgent status outlined in the 8/5/26 ERCOT comments. |  |
| TAC Decision |  | On 8/26/26, TAC voted unanimously to recommend approval of NPRR1335 as recommended by PRS in the 8/12/26 PRS Report. All Market Segments participated in the vote. |  |
| Summary of TAC Discussion |  | On 8/26/26, there was no additional discussion beyond TAC review of the items below. |  |
| TAC Review/Justification of Recommendation |  | Revision Request ties to Reason for Revision as explained in Justification Impact Analysis reviewed and impacts are justified as explained in Justification Opinions were reviewed and discussed Comments were reviewed and discussed (if applicable) Other: (explain) |  |
| ERCOT Board Decision |  | On 9/15/26, the ERCOT Board voted unanimously to recommend approval of NPRR1335 as recommended by TAC in the 8/26/26 TAC Report. |  |

| Opinions |  |
| --- | --- |
| Credit Review | ERCOT Credit Staff and the Credit Finance Sub Group (CFSG) have reviewed NPRR1335 and do not believe that it requires changes to credit monitoring activity or the calculation of liability. |
| Independent Market Monitor Opinion | IMM has no opinion on NPRR1335. |
| ERCOT Opinion | ERCOT supports approval of NPRR1335. |
| ERCOT Market Impact Statement | ERCOT Staff has reviewed NPRR1335 and believes the market impact for NPRR1335 ensures that the Protocols are consistent with the PUCT’s FFSS rule as required by 16 TAC § 25.520(j). |

| Sponsor |  |
| --- | --- |
| Name | Ino Gonzalez / Mark Patterson |
| E-mail Address | Ino.Gonzalez@ercot.com / Mark.Patterson@ercot.com |
| Company | ERCOT |
| Phone Number | 512-632-7927 / 512-569-5539 |
| Cell Number |  |
| Market Segment | Not applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Cory Phillips |
| E-Mail Address | cory.phillips@ercot.com |
| Phone Number | 512-248-6464 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 060526 | Proposed additional redlines to correct errors within Section 22, Attachment F |
| ERCOT 062926 | Proposed additional redlines to the 6/5/26 ERCOT comments to specify that the value of the Henry Hub Natural Gas Futures natural gas price is the price published by July 15 for the upcoming November of the current year |
| ERCOT 070126 | Proposed additional redlines to the 6/29/26 ERCOT comments to clarify that for fuel oil, the price will be based on the US Gulf Coast NYMEX Heating Oil for November, or equivalent, published by July 15 of the current year |
| WMS 070826 | Requested PRS continue to table NPRR1335 |
| ERCOT 080526 | Requested PRS grant NPRR1335 Urgent status to allow Market Participants to prepare for the upcoming FFSS obligation period |
| WMS 080626 | Endorsed NPRR1335 as amended by the 7/1/26 ERCOT comments |

Market Rules Notes

Please note the baseline Protocol language in the following sections has been updated to reflect the incorporation of the following NPRR(s) into the Protocols:

- NPRR1327, As-Built for NPRR1281, Improvements to Alternate FFSS Resource Designation (incorporated 8/1/26)

  - Section 6.6.14.2

  - Section 8.1.1.2.1.6

Please note that the following NPRR(s) also propose revisions to the following section(s):

- NPRR1278, Establishing Advanced Grid Support Service as an Ancillary Service

  - Section 3.14

- NPRR1320, Reserve Margin Reporting and Miscellaneous Changes for the Report on Capacity, Demand, and Reserves in the ERCOT Region (CDR)

  - Section 1.3.1.2

Proposed Protocol Language Revision

1.3.1.2	Items Not Considered Protected Information

(1)	Notwithstanding the definition of “Protected Information” in Section 1.3.1.1, Items Considered Protected Information, the following items are not Protected Information even if so designated:

(a)	Data comprising Load flow cases, which may include estimated peak and off-peak Demand of any Load;

(b)	Existence of Power System Stabilizers (PSSs) at each interconnected Generation Resource or ESR, and PSS status (in service or out of service);

(c)	Reliability Must-Run (RMR) Agreements;

(d)	Studies, reports and data used in ERCOT’s assessment of whether an RMR Unit satisfies ERCOT’s criteria for operational necessity to support ERCOT System reliability but only if they have been redacted to exclude Protected Information under Section 1.3.1.1;

(e)	Status of RMR Units;

(f)	Black Start Agreements;

(g)	FFSS awards and Agreements;

(h)	RMR Settlement charges and payments;

[NPRR885: Insert items (i) and (j) below upon system implementation and renumber accordingly:] (i) Must-Run Alternative (MRA) Agreements; (j) Settlement charges and payments for MRA Service;

(i)	Within two Business Days of a request from a potential generating Facility for a full resource interconnection study, the county in which the Facility is located, Facility fuel type(s), Facility nameplate capacity, and anticipated Commercial Operations Date(s) and signed generation interconnection agreements; and

(j)	Any other information specifically designated in these Protocols or in the PUCT Substantive Rules as information to be posted to the ERCOT website or Market Information System (MIS) Secure Area that is not specified as information that is subject to the requirements of Section 1.3, Confidentiality.

(2)	Protected Information that Receiving Party is permitted or required to disclose or use under the Protocols or under an agreement between Receiving Party and a Disclosing Party does not cease to be regarded as Protected Information in all other circumstances not encompassed by these Protocols or such agreement by virtue of the permitted or required disclosure or use under these Protocols or such agreement.

## 2.1	DEFINITIONS

Firm Fuel Supply Service (FFSS) Obligation Period

The period from November 15 through March 15 for which a procured Resource is obligated to provide FFSS.

Firm Fuel Supply Service (FFSS) Market Clearing Price

The dollar amount per megawatt (MW) that is awarded for an FFSS Resource (FFSSR) procured for the FFSS Obligation Period.

Firm Fuel Supply Service (FFSS) Offer Cap

The maximum dollar amount per MW that a Qualified Scheduling Entity (QSE) representing a Resource may offer into FFSS.

Firm Fuel Supply Service (FFSS) Non-Procurement Costs

The fuel restocking payments to FFSS Resources following a deployment during the FFSS Obligation Period.

Firm Fuel Supply Service (FFSS) Procurement Costs

The standby payments to FFSS Resources for an FFSS Obligation Period.

3.14	Contracts for Reliability Resources and Emergency Response Service Resources

(1)	ERCOT shall procure Reliability Must-Run (RMR) Service, Black Start Service (BSS), Firm Fuel Supply Service (FFSS), or Emergency Response Service (ERS) through Agreements.

[NPRR885: Replace paragraph (1) above with the following upon system implementation:] (1) ERCOT shall procure Reliability Must-Run (RMR) Service, Must-Run Alternative (MRA) Service, Black Start Service (BSS), Firm Fuel Supply Service (FFSS), or Emergency Response Service (ERS) through Agreements.

### 3.14.5	Firm Fuel Supply Service

(1)	Each Generation Resource providing or offering to provide Firm Fuel Supply Service (FFSS), including the primary and any alternate Generation Resources identified in the FFSS Offer Submission Form, must meet technical requirements specified in Section 8.1.1, QSE Ancillary Service Performance Standards, and Section 8.1.1.1, Ancillary Service Qualification and Testing.

(2)	ERCOT shall issue an RFP by August 1 of each year soliciting offers from QSEs for Generation Resources to provide FFSS.  The RFP shall require offers to be submitted on or before September 1 of each year. Generation Resources eligible to provide FFSS include Resources under the on-site FFSS category, resource-controlled FFSS category, and contractual off-site FFSS category, and as defined in Section 8.1.1.2.1.6, Firm Fuel Supply Service Resource Qualification, Testing, Decertification, and Recertification.

(3)	ERCOT must procure FFSS ahead of each FFSS Obligation Period to help maintain reliability during, or in preparation for, a natural gas curtailment or other fuel supply disruption.  ERCOT may spend a maximum of $54 million in FFSS Procurement Costs during a single FFSS Obligation Period.  ERCOT may reject an offer that a QSE submits on behalf of a Generation Resource if ERCOT determines that:

(a) 	The offer is unreasonable;

(b) 	The offer is an outlier when evaluating the parameters of an acceptable offer;

(c) 	The offer exceeds the applicable FFSS Offer Cap;

(d) 	ERCOT lacks a sufficient basis to verify whether the Generation Resource complied with ERCOT established performance standards in an event in which the Generation Resource was deployed by ERCOT during the preceding FFSS Obligation Period;

(e)	The QSE representing the Generation Resource fails to reserve sufficient fuel for the first deployment for the FFSS Obligation Period; or

(f) 	The QSE representing the Generation Resource fails to reserve sufficient emissions allowances or credits to meet at least three deployments for the FFSS Obligation Period.

(4)	ERCOT must allocate a combined amount of at least 70% of the $54 million budget to procure Generation Resources under the on-site FFSS category and the resource-controlled FFSS category, unless insufficient offers were submitted for Generation Resources under those categories.  If insufficient offers were submitted for Generation Resources under the on-site FFSS category and the resource-controlled FFSS category to allocate 70% of the budget to those resources, then ERCOT may reallocate the remainder of that portion of the budget to resources under the contractual off-site FFSS category.

(5)	Within seven days of being notified of a FFSS offer being accepted, the QSE for an awarded Resource must email and executed Section 22, Attachment F, Firm Fuel Supply Service Agreement to FFSS@ercot.com.  Failure to do so will result in reallocation of FFSS awards.

(6)	Before the start of an FFSS Obligation Period, ERCOT must administratively set the FFSS Offer Cap for each category of eligible resources.  The FFSS Offer Cap must be calculated as a function of maximum hours per deployment (hours), heat rate (MMBtu/MWh), and fuel price ($/MMBtu), using the following equation:

FFSSOCAP c, p =  FFSSDH * FFSSPFP c, p * FFSSHR c, p

(a)	The FFSS projected fuel price (FFSSPFP) for Resources under the on-site FFSS category must be based on the projected index price of fuel oil for the upcoming FFSS Obligation Period, as described in paragraph (11) below, plus a fuel adder of $0.05 per gallon.

(b)	The FFSS projected fuel price (FFSSPFP) for Resources under the resource-controlled FFSS category must be based on the projected index price of fuel oil for the upcoming FFSS Obligation Period, as described in paragraph (11) below, which does not include a fuel adder.

(c)	The FFSS projected fuel price (FFSSPFP) for Resources under the contractual off-site FFSS category must be based on the projected index price of natural gas for the upcoming FFSS Obligation Period, and as described in paragraph (11) below, plus a fuel adder of $0.5/MMBtu.

(d)	The heat rate for each of the three categories of Resources, on-site FFSS category, Resource-controlled FFSS category, and off-site FFSS category must be based on the characteristics of the resources that are eligible to provide FFSS under each of these categories, and as further defined in paragraphs (9), (10) and (11) below.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FFSSOCAPc,p | $/MW | Firm Fuel Supply Service Offer Cap per Resource Category per Obligation Period—The FFSS Offer Cap applicable to the FFSS Resource category c, for the FFSS Obligation Period p. |
| FFSSDH | hrs | Firm Fuel Supply Service Deployment Hours—The maximum deployment hours Generation Resources contracted under FFSS must be able to generate at the FFSS awarded capacity. The deployment hours shall be set at 48 hours per deployment. |
| FFSSHRc,p | MMBtu/MWh | Firm Fuel Supply Service Resource Category Heat Rate— The heat rate applicable to the FFSS Resource category c, for the FFSS Obligation Period p. |
| FFSSPFPc,p | $/MMBtu | Firm Fuel Supply Service Projected Fuel Price—The projected fuel price for the Resource category c, applicable for the FFSS Obligation Period p. |
| c | none | Firm Fuel Supply Service Resource category. |
| p | none | FFSS Obligation Period. |

(7)	ERCOT must establish a single market clearing price mechanism for Resources eligible to provide FFSS under the on-site FFSS category and the Resource controlled FFSS category.  ERCOT must establish a separate market clearing price mechanism for Resources eligible to provide FFSS under the contractual off-site FFSS category.

(8)	Prior to the start of each FFSS Obligation Period, ERCOT must publicly report the number and category of FFSS Resources (FFSSRs) providing the service, the market clearing prices, the amount of reserved fuel associated with each FFSS award, the highest and lowest offers, the number of MW associated with each FFSS award, and the projected total cost to procure FFSS for that Obligation Period.

(9)	The following represent the minimum FFSS program requirements:

(a)	QSEs may submit offers individually for one or more Generation Resources to provide FFSS using the FFSS Offer Submission Form posted on the ERCOT website.

(b)	A QSE may not submit an offer for a given Generation Resource unless it is the QSE designated by the Resource Entity associated with that Generation Resource.

(c)	ERCOT must evaluate offers using criteria identified in an appendix to the RFP.

(d)	ERCOT will issue FFSS awards by September 30 and will post the awards to the MIS Certified Area for each QSE that is awarded an FFSS obligation.

(e)	The posting will include information such as, but not limited to, the identity of the primary Generation Resource and any alternate Generation Resource(s), the FFSS clearing price, the amount of reserved fuel associated with the FFSS award, the MW amount awarded, and the Generation Resource’s initial minimum LSL when providing FFSS.

(f)	The RFP awards shall cover a period beginning November 15 of the year in which the RFP is issued and ending on March 15 of the year after the year in which the RFP is issued.

(g)	A QSE may submit an offer for one or more Generation Resources to provide FFSS beginning in the same year the RFP is issued or as otherwise specified in the RFP.

(h)	An FFSS Resource (FFSSR) shall be considered an FFSSR and is required to provide FFSS from November 15 through March 15 for each year of the awarded FFSS Obligation Period.

(i)	ERCOT shall ensure FFSSRs are procured and deployed as necessary to maintain ERCOT System reliability during, or in preparation for, a natural gas curtailment or other fuel supply disruption.  In addition, the following requirements apply:

(i)	On the FFSS Offer Submission Form, the QSE shall disclose information including, but not limited to, the Generation Resource and any alternate Generation Resource(s), the amount of reserved fuel offered, the MW available from the capacity offered, an estimate of the time to restock fuel reserves, and each limitation of the offered Generation Resource that could affect the Generation Resource’s ability to provide FFSS.

(ii)	If the QSE offers a Generation Resource as meeting the qualification requirements in paragraph (1)(c) of Section 8.1.1.2.1.6, Firm Fuel Supply Service Resource Qualification, Testing, Decertification, and Recertification, the QSE must submit as part of its offer a certification for the offered Generation Resource.  The certification must include:

(A)	Certification that the Generation Entity for the Generation Resource (or an Affiliate) has a Firm Transportation Agreement, firm natural gas supply, and contracted or owned storage capacity meeting the qualification requirements in paragraph (1)(c) of Section 8.1.1.2.1.6;

(B)	The following information regarding the Firm Transportation Agreement:

(1)	FFSS Qualifying Pipeline name;

(2)	Term;

(3)	Primary points of receipt and delivery;

(4)	Maximum daily contract quantity (in MMBtu);

(5)	Shipper of record; and

(6)	Whether the Firm Transportation Agreement provides for ratable receipts and deliveries; and

(C)	The following information regarding the storage arrangements:

(1)	Storage facility name;

(2)	Term of the Firm Gas Storage Agreement (if applicable);

(3)	Maximum storage quantity owned or contracted under the Firm Gas Storage Agreement (in MMBtu); and

(4)	Maximum daily withdrawal quantity (in MMBtu).

(iii)	For a Generation Resource to be eligible to receive an FFSS award, the primary Generation Resource and any alternate Generation Resource(s) identified in the FFSS Offer Submission Form shall complete all applicable testing requirements as specified in Section 8.1.1.2.1.6.  A QSE representing an FFSSR is allowed to provide the FFSS with an alternate Resource previously approved by ERCOT to replace the FFSSR.

(iv)	An offer to provide FFSS is an offer to supply an awarded amount of capacity, maintain a sufficient amount of reserved fuel to meet at least one deployment at the awarded level for the duration requirement specified in the RFP, and to designate a specific number of emissions hours that will be reserved for the awarded FFSSR in meeting its obligation to perform for at least three deployments  during the FFSS Obligation Period, while operating at awarded MW level.  Reserved fuel, emissions hours, and other attributes, in excess of what is needed to meet the FFSS obligation can be used at the discretion of the QSE as long as sufficient fuel reserves and emissions hours are maintained for the purposes of ERCOT deployment of FFSS.

(v)	Within ten Business Days of issuing FFSS awards and ERCOT and QSEs representing FFSS Resources executing all FFSS Agreements, ERCOT will post on the ERCOT website the identity of all Generation Resources that were offered as primary Generation Resources or alternate Generation Resources to provide FFSS for the most recent procurement period, including prices and quantities offered.

(10)	A QSE or Resource Entity representing a thermal FFSSR that submits an offer to provide FFSS must submit the heat rate curve (input-output equation) for the alternate fuel for the winter season by the deadline for submitting the FFSS offer.  The QSE or Resource Entity must email NodalVerifiableCost@ercot.com the heat rate curve and data used to derive it.  This requirement is not applicable for thermal Generation Resources that have submitted their heat rate curves for the alternate fuel for the winter season within five years prior to the deadline for submitting the FFSS offer.

(11)	When calculating average heat rates for the next FFSS Obligation Period ERCOT may take into consideration one or more of the following Resource-specific data sources:

(a)	Winter season heat rates as described in paragraph (10) above;

(b)	Fuel heat content (MMBtu) and generation (MWh) values submitted to the U.S. Environmental Protection Agency (EPA) for hours when the Resource burned alternate fuel during the period(s) corresponding to the FFSS Obligation Period; or

(c)	Actual data submitted via fuel disputes when requesting recovery of fuel re-stocking costs;

(12)	The FFSS index prices described in paragraph (7) above will be determined as follows:

(a)	Oil price index source(s):

Nymex Heating Oil Futures for November, US Gulf Coast (USGC) or equivalent, published by July 15 of the current year.

(b)	Natural gas price index source(s):

Henry Hub Natural Gas Futures for November, or equivalent, published by July 15 of the current year.

(13)	The QSE for an FFSSR shall ensure that the Resource is prepared and able to come On-Line or remain On-Line in order to maintain Resource availability in the event of a natural gas curtailment or other fuel supply disruption.

(a)	When ERCOT issues a Watch for winter weather, ERCOT will notify all Market Participants, including all QSEs representing FFSSRs, to begin preparation for potential FFSS deployment.  Such preparation may include, but is not limited to, circulation of alternate fuel to its facilities, if applicable; heat fuel oil to appropriate temperatures, if applicable; call out additional personnel as necessary, and be ready to receive a Dispatch Instruction to provide FFSS.  An FFSSR may begin consuming a minimum amount of alternate fuel to validate it is ready for an FFSS deployment.

(b)	In anticipation of or in the event of a natural gas curtailment or other fuel supply disruption to an FFSSR, the QSE shall notify ERCOT as soon as practicable and may request approval to deploy FFSS to generate electricity.  ERCOT shall evaluate system conditions and may approve the QSE’s request.  The QSE shall not deploy the FFSS unless approved by ERCOT.  Upon approval to deploy FFSS, ERCOT shall issue an FFSS Verbal Dispatch Instruction (VDI) to the QSE.  ERCOT may issue separate VDIs for each Operating Day for each FFSSR that is deployed for FFSS.

(c)	In conjunction with a QSE notification under paragraph (b) above, the QSE shall also report to ERCOT any environmental limitations that would impair the ability of the FFSSR to provide FFSS for the required duration of the FFSS award.

(d)	ERCOT may issue an FFSS VDI without a request from the QSE, however ERCOT shall not issue an FFSS VDI without evidence of an impending or actual fuel supply disruption affecting the FFSSR.

(e)	If the FFSSR is generating at a level above the FFSS MW awarded amount and that level of output cannot be sustained for the required duration of the FFSS award, ERCOT may use a manual High Dispatch Limit (HDL) override to ensure the FFSSR can continue to generate at the FFSS MW award level for the entire FFSS duration requirement specified in the RFP.

(f)	The FFSSR shall continuously deploy FFSS to generate electricity until the earlier of (i) the exhaustion of the fuel reserved to generate at the FFSS MW award level for the duration requirement specified in the RFP, including any fuel that was restocked following approval or instruction from ERCOT, (ii) the fuel supply disruption no longer exists, or (iii) ERCOT determines the FFSS deployment is no longer needed.  Upon satisfying one of these qualifications, ERCOT shall terminate the VDI.  In the event of (i), the FFSSR shall not be obligated to continue being available for FFSS deployment for the remainder of the Watch.  In the event of (ii) or (iii), the FFSSR shall continue being available for FFSS deployment for the remainder of the Watch.

(g)	The QSE for the FFSSR is responsible for communicating with the ERCOT control room the anticipated exhaustion of the reserved fuel at least six hours before that anticipated exhaustion and upon the exhaustion of that fuel.

(h)	A QSE shall notify the ERCOT control room of the anticipated exhaustion of emissions credits or permit allowances at least six hours before the exhaustion of those credits or allowances.  Upon receiving such notification, ERCOT shall modify the VDI so the FFSS deployment is terminated upon exhaustion of those credits or allowances.

(i)	Upon deployment or recall of FFSS, ERCOT shall notify all Market Participants that such deployment or recall has been made, including the MW capacity of service deployed or recalled.

(14)	Following each deployment of FFSS, the QSE for an FFSSR may request approval from ERCOT via email to FFSS@ercot.com , or ERCOT may instruct the QSE to restock their fuel reserve to restore their ability to generate at the FFSS MW award level for the duration requirement specified in the RFP as follows:

(a)	The QSE requests preliminary approval from ERCOT control room, or ERCOT provides preliminary instruction, to restock and provide ERCOT an initial estimated timeline to complete the refueling.

(b)	After receiving preliminary approval or instruction from ERCOT, the QSE shall:

(i)	Immediately provide a final estimate for completing the restocking of fuel; or

(ii)	Within 24 hours, notify the ERCOT control room with an updated estimated timeline to complete the restocking of the fuel.

(c)	Based on the most recent expected time needed to restock the fuel, the ERCOT control room may or may not provide final approval for restocking of the fuel.

(d)	If ERCOT makes final approval to restock the fuel, the QSE representing the FFSSR shall inform the ERCOT control room immediately when restocking is complete.

(15)	Following final approval from ERCOT, a QSE must restock their fuel reserve, using existing fuel inventories or new fuel purchases, to restore their ability to generate at the FFSS MW award level for the specified duration requirement.  In the event ERCOT does not receive the request to restock from a QSE representing an FFSSR, as described in paragraph (13) above, but the QSE no longer has sufficient reserved fuel to generate at the FFSS MW award level for the specified duration requirement, the QSE shall communicate to the ERCOT control room this reduced capability and ERCOT may instruct the QSE to restock the fuel reserve as described in paragraph (14) above.

(16)	For a Resource to be considered as an alternate for providing FFSS, the following requirements must be met.  The alternate Resource must:

(a)	Be able to provide net real power sufficient to generate at the same FFSS MW award level as the primary Resource for the duration requirement specified in the RFP;

(b)	Be a single Generation Resource, as registered with ERCOT; and

(c)	Use the same source of fuel reserve for providing FFSS as the primary Resource.

(17)   An FFSS Offer Submission Form may have up to three alternate Generation Resources per primary Resource offering to provide FFSS.

(18)	For FFSSRs with approved alternate Generation Resources if the FFSSR becomes unavailable, the QSE must:

(a)	As soon as practicable, notify ERCOT via email to FFSS@ercot.com and inform ERCOT that the FFSSR will be replaced by one of the alternate Generation Resources, specify which alternate Generation Resource (if multiple alternate Generation Resources have been designated), and provide an estimate of how long the replacement will be in effect;

(b)	Update the Availability Plans for these Generation Resources to reflect current operating conditions within 60 minutes after identifying the change in availability of the FFSSR; and

(c)	Update the COPs for these Generation Resources within 60 minutes after identifying the change in availability of the FFSSR.

(19)	For FFSSRs that were replaced by one of their approved alternate Generation Resources, when the primary Resource is once again the FFSSR, the QSE must notify ERCOT of the change via email to the email address provided in paragraph (18)(a) above as soon as practicable.

(20)	An FFSSR providing BSS must have sufficient fuel reserved to generate at the FFSS MW award level for the duration requirement specified in the RFP in addition to any fuel required for the Generation Resource to meet the contracted BSS obligation.  Any remaining fuel reserve in addition to that required for meeting FFSS and BSS obligations can be used at the QSE’s discretion.

(21)	If ERCOT issues an FFSS VDI to an FFSSR for the same Operating Hour where a RUC instruction was issued, then for Settlement purposes ERCOT will consider the RUC instruction as cancelled.

(22)	If FFSS is deployed, then ERCOT will provide a report to the TAC or its designated subcommittee within 45 days of the end of the FFSS Obligation Period.  The report must include the Resources deployed and the reason for any deployments.

(23)	Any QSE that submits an offer or receives an award for a SWGR to provide FFSS, and the Resource Entity that owns or controls that SWGR, shall:

(a)	Not nominate the SWGR to satisfy supply adequacy or capacity planning requirements in any Control Area other than the ERCOT Region during the period of the FFSS obligation; and

(b)	Take any further action requested by ERCOT to ensure that ERCOT will be classified as the “Primary Party” for the SWGR under any agreement between ERCOT and another CAO during the period of the FFSS obligation.

(24)	On an annual basis after the FFSS season, ERCOT will provide a report separately for the total amounts from Section 6.6.14.1, Firm Fuel Supply Service Fuel Replacement Costs Recovery, and Section 6.6.14.2, Firm Fuel Supply Service Hourly Standby Fee Payment and Fuel Replacement Cost Recovery, to the TAC or its designated subcommittee.

6.6.14.1	Firm Fuel Supply Service Fuel Replacement Costs Recovery

(1)	If ERCOT approves a Firm Fuel Supply Service Resource (FFSSR) to switch to consume the reserved fuel and directs or approves a restocking pursuant to paragraph (14) of Section 3.14.5, Firm Fuel Supply Service, ERCOT shall pay the QSE representing the FFSSR for the replacement of burned fuel, if the QSE has:

(a)	Complied with the Firm Fuel Supply Service (FFSS) instruction to switch to the reserved fuel;

(b)	Submitted a Settlement and billing dispute consistent with the dispute process described in Section 9.14, Settlement and Billing Dispute Process;

(c)	Submitted the following within 90 days of the issuance of a RTM Initial Statement for the Operating Day on which the FFSS instruction was issued:

(i)	An attestation signed by an officer or executive with authority to bind the QSE stating that the information contained in the dispute is accurate;

(ii)	For each deployment of FFSS, the quantity of total fuel consumed for the hours in each instance when FFSS was deployed;

(iii)	For thermal units, the input-output equation,  or other documentation that allows for verification of fuel consumption for the hours when FFSS was deployed;

(iv)	The heat content of the fuel, in terms of MMBtu/gal or similar units of measurement;

(v)	The dollar amount and quantity of fuel purchased to replace the consumed fuel;

(vi)	Sufficient documentation to support the QSE’s determination of the amount and cost of replaced fuel; and

(vii)	Any other technical documentation within the possession of the QSE or Resource Entity which ERCOT finds reasonably necessary to verify paragraphs (i) through (vi) above.  Any additional request from ERCOT for documentation or clarification of previously submitted documentation must be honored within 15 Business Days.

(2)	In addition to the requirements under paragraph (1)(c) above, for a Generation Resource that was awarded FFSS using reserved fuel based on a Firm Gas Storage Agreement and is requesting compensation for the cost of the replaced fuel, the QSE or Resource Entity representing the FFSSR must show proof that it purchased and nominated fuel in sufficient quantities (in MMBtu) that was consumed during the FFSS deployment and that its actual receipts and deliveries of such replacement fuel conformed with its nominated quantities.

(3)	The Firm Fuel Supply Service Fuel Replacement Cost shall only represent the replacement fuel costs not recovered during the FFSS deployment period through Day-Ahead energy sales and Real-Time energy imbalance settlement revenues related to the Resource with the FFSS award.  In addition, the Firm Fuel Supply Service Fuel Replacement Cost shall only include commodity and variable transportation costs directly attributable to the replenishment of fuel for the FFSSR.

(4) 	If, after ERCOT approval, the fuel that was burned during FFSS deployment is replaced using existing fuel inventories, the Firm Fuel Supply Service Fuel Replacement Cost may be based on:

(a)	New fuel purchases made within 30 days after ERCOT approves the restock of the burned fuel as described in paragraph (15) of Section 3.14.5; or

(b)	The Fuel Oil Index Price (FOP) for the Operating Day the QSE received approval to restock fuel, which includes $0.05/gallon to cover the cost of transportation.

(5)	ERCOT shall allocate any approved fuel replacement costs to the hours of the corresponding FFSS deployment period when the fuel was consumed following ERCOT’s approval to switch to utilize the awarded FFSS.

6.6.14.2	Firm Fuel Supply Service Hourly Standby Fee Payment and Fuel Replacement Cost Recovery

(1)	ERCOT shall pay the FFSS Hourly Standby Fee to the QSE representing the primary Generation Resource.  This standby fee is determined through a competitive bidding process, with an adjustment for reliability based on an Hourly Rolling Equivalent Availability Factor, as well as adjustments for capacity and deployment.

(2)	The FFSSR will be considered available when calculating the FFSS Hourly Rolling Equivalent Availability Factor:

(a)	During each non-FFSS deployment hour for which the FFSSR shows available in its Availability Plan;

(b)	During any successful FFSS deployment of the FFSSR in which the FFSSR shows available in its Availability Plan;

(c)	If the reserved fuel was exhausted during an FFSS deployment, starting the hour after the FFSSR has consumed all the fuel reserved to provide FFSS, through the approved hours when reserved fuel for FFSS is being restocked following a final approval from ERCOT to do so, per paragraph (15) of Section 3.14.5, Firm Fuel Supply Service;

(d)	In the event the FFSSR has consumed all the fuel reserved to provide FFSS and ERCOT does not issue an instruction or approval to restore FFSS capability, the FFSSR shall be considered to be available for the remainder of the FFSS Obligation Period in progress;

(e)	If the FFSSR was deployed to provide FFSS and, as a result, has exhausted its emission hours allocated for the FFSSR, as specified in the FFSS Offer Submission Form.

[NPRR1327: Insert paragraph (3) below upon system implementation of NPRR1281 and renumber accordingly:] (3) For the calculation of the Available Capacity (AVCAP), if multiple alternate Generation Resources show available in their respective Availability Plan when the primary Generation Resource is unavailable, ERCOT will use the HSL of the alternate Generation Resource with the lowest value. For a Combined Cycle Train, the HSL will be the HSL of the largest Combined Cycle Generation Resource that is available.

(3)	The FFSS Hourly Standby Fee is subject to reduction and claw-back provisions as described in Section 8.1.1.2.1.6, Firm Fuel Supply Service Resource Qualification, Testing, Decertification, and Recertification.

(4)	ERCOT shall pay an FFSS payment to each QSE for each FFSSR.  The FFSS payment for each hour of November 15, through March 15, i.e., during the FFSS Obligation Period, is calculated as follows:

FFSSAMT q, r, h	=	(-1) * (FFSSSBF q, r, h + FFSSFRC q, r, h )

Where:

FFSSSBF q, r, h	=	 FFSSAWARD q, r, h * FFSSCRF q, r, h * FFSSARF q, r, h * (1 - FFSSDRP q, r, h)

FFSSAWARD q, r, h = FFSSPR q, r, h * FFSSACAP q, r, h

And:

FFSS Capacity Reduction Factor

If (FFSSTCAP q, r, h ≥ FFSSACAP q, r, h)

Then: 		FFSSCRF q, r, h  = 1

Otherwise:	FFSSCRF q, r, h = Max (0, 1 – 2 * (FFSSACAP q, r, h – FFSSTCAP q, r, h) /

FFSSACAP q, r, h)

FFSS Availability Reduction Factor

If (FFSSHREAF q, r, h  0.90)

Then:		FFSSARF q, r, h 	= 1

Otherwise:	FFSSARF q, r, h 	= Max (0, 1 - (0.90 - FFSSHREAF q, r, h) * 2)

FFSS Hourly Rolling Equivalent Availability Factor

FFSSHREAF q, r, h  = hr=h-1451h(max(AVCAP q, r, hr)) / hr=h-1451h(FFSSACAP q, r, hr)

[NPRR1327: Replace the formula “FFSSHREAF q, r, h” above with the following upon system implementation of NPRR1281:] FFSSHREAF q, r, h = hr=h-1451h(AVCAP q, r, hr) / hr=h-1451h(FFSSACAP

Where,

If the Resource is a Combined Cycle Train:

AVCAPq, r, hr  = maxtrain,hr (max(FFSEDFLAG q, train, hr, FFSSAFLAG q, ccgr, hr) * min(HSL q, ccgr, hr, FFSSACAPq, train, hr))

Otherwise:

AVCAP q, r, hr = max(FFSEDFLAG q, r, hr, FFSSAFLAG q, r, hr) * min(HSL q, r, hr, FFSSACAP q, r, hr)

Availability for a Combined Cycle Train will be determined pursuant to terms set forth in the RFP but no more than once per hour.

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FFSSAMT q, r, h | $ | Firm Fuel Supply Service Amount per QSE per Resource by hour—The payment to QSE q assigned to the FFSS for the primary Generation Resource r, for the hour, calculated each hour of November 15 through March 15 during the awarded FFSS Obligation Period. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSAWARD q, r, h | $ | Firm Fuel Supply Service Award Amount per QSE by hour—The payment to the QSE q for the FFSS awarded to the primary Generation Resource r for each hour h, during the awarded FFSS Obligation Period. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSPR q, r, h | $/MW per hour | Firm Fuel Supply Service Price per QSE per Resource by hour—The standby price of the primary Generation Resource r represented by QSE q, as specified in the FFSS award. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSCRF q, r, h | none | Firm Fuel Supply Service Capacity Reduction Factor per QSE per Resource by hour—The capacity reduction factor assigned to the primary Generation Resource r, represented by QSE q, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| HSL q, r, hr | MW | High Sustained Limit—The HSL of the primary Generation Resource or the alternate Generation Resource r represented by QSE q as submitted in the COP, for the hour h. Where for a combined cycle Resource r is a Combined Cycle Generation Resource. |
| FFSSFRC q, r, h | $ per hour | Firm Fuel Supply Service Fuel Replacement Cost—The fuel costs and fees to replace the burned fuel by the FFSSR, not recovered during the FFSS deployment period, paid to the primary Generation Resource r represented by QSE q for each FFSS instructed hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSDRP q, r, h | none | Firm Fuel Supply Service Deployment Reduction Percentage—The percentage of the Firm Fuel Supply Service Standby Fee subject to clawback per paragraphs (9) through (16) of Section 8.1.1.2.1.6, Firm Fuel Supply Service Resource Qualification, Testing, Decertification, and Recertification, for the QSE q, assigned to the primary Generation Resource r, for the hour h. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSSBF q, r, h | $ | Firm Fuel Supply Service Standby Fee per QSE per Resource by hour—The standby fee to QSE q for the FFSS assigned to the primary Generation Resource r, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSTCAP q, r, h | MW | Firm Fuel Supply Service Testing Capacity per QSE per Resource—The tested capacity of the primary Generation Resource r, represented by QSE q, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSACAP q, r, hr | MW | Firm Fuel Supply Service Awarded Capacity per QSE per Resource—The awarded FFSS capacity of the primary Generation Resource r, represented by QSE q as specified in the FFSS award, applicable to each hour of November 15 through March 15 during the awarded FFSS Obligation Period. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSARF q, r, h | none | Firm Fuel Supply Service Availability Reduction Factor per QSE per Resource by hour—The availability reduction factor assigned to the primary Generation Resource r represented by QSE q for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSHREAF q, r, h | none | Firm Fuel Supply Service Hourly Rolling Equivalent Availability Factor per QSE per Resource by hour—The equivalent availability factor assigned to the primary Generation Resource r represented by QSE q over 1,452 hours, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSAFLAG q, r, hr | none | Firm Fuel Supply Service Availability Flag per QSE per Resource by hour—The flag of the availability assigned to the primary Generation Resource or the alternate Generation Resource r represented by QSE q, 1 for available and 0 for unavailable, for the hour. Where for a Combined Cycle Train, the Resource r is a Combined Cycle Generation Resource within the Combined Cycle Train. |
| FFSEDFLAG q, r, hr | none | Firm Fuel Supply Event Deployment Flag per QSE per Resource by hour—The flag assigned to the primary Generation Resource r, represented by QSE q, that is used to determine if the FFSSR is considered available, as described in paragraph (2)(c) through (2)(e) above, 1 for available and 0 for unavailable, for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| AVCAP q, r, hr | MW | Available Capacity per Resource by hour—The available capacity assigned to the primary Generation Resource r represented by QSE q as calculated for the hour. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A primary or alternate Generation Resource approved by ERCOT to provide FFSS. |
| hr | none | The index of a given hour and the previous 1,451 hours counted only during each hour of November 15 through March 15 during the awarded FFSS Obligation Period. |
| h | none | The Operating Hour. |
| train | none | A Combined Cycle Train or an alternate Combined Cycle Train approved by ERCOT. |
| ccgr | none | A Combined Cycle Generation Resource within the Combined Cycle Train. |

(5)	The total of the payments to each QSE for all FFSSRs represented by this QSE for a given hour is calculated as follows:

FFSSAMTQSETOT q	=	FFSSAMT q, r

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FFSSAMTQSETOT q | $ | Firm Fuel Supply Service Amount QSE Total per QSEThe total of the payments to QSE q for FFSS provided by all the FFSS Resources represented by this QSE for the hour. |
| FFSSAMT q, r | $ | Firm Fuel Supply Service Amount per QSE per Resource—The payment to QSE q for the FFSS assigned to the primary Generation Resource r, for the hour, calculated each hour of November 15 through March 15 during the awarded FFSS Obligation Period. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A primary or alternate Generation Resource approved by ERCOT to provide FFSS. |

8.1.1.2.1.6	Firm Fuel Supply Service Resource Qualification, Testing, Decertification, and Recertification

(1)	Generation Resources that meet one of the three following categories under this subsection  are eligible to provide Firm Fuel Supply Service (FFSS) and may be selected in the procurement process for FFSS for an FFSS Obligation Period.  Both the primary Generation Resource and any alternate Generation Resources, as specified in the FFSS Offer Submission Form, must meet the following requirements prior to submitting an FFSS Offer Submission Form:

(a)	On-site FFSS category.  An FFSS Resource under this category must successfully demonstrates dual fuel capability, have the ability to establish and burn an alternative on-site stored fuel, and has onsite fuel storage capability in an amount that satisfies the minimum FFSS capability requirements, as described in paragraph (2) below;

(b)	Resource-controlled FFSS category.  An FFSS Resource under this category  must have an on-site natural gas or fuel oil storage capability or off-site natural gas storage where the Resource Entity and/or QSE owns and controls the natural gas storage and pipeline to deliver the required amount of reserve natural gas to the Generation Resource from the storage facility in an amount that satisfies the minimum FFSS capability requirements, as defined in paragraph (2) below; or

(c)	Contractual off-site FFSS category.  An FFSS Resource under this category must have a firm gas storage agreement with a storage provider for firm storage of the natural gas at the storage facility and have a firm transportation agreement with a natural gas pipeline that is a critical natural gas facility, as defined in 16 Texas Administrative Code (TAC) § 25.52 for firm transportation of the natural gas from the storage facility to the FFSS resource.  In addition, contractual off-site FFSS Resources must meet the following requirements:

(i)	The Generation Entity for the Generation Resource (or an Affiliate of such Generation Entity) either owns a storage facility with, or has a Firm Gas Storage Agreement for, sufficient natural gas storage capacity for the offered Generation Resource to deliver the offered MW for the duration requirement specified in the request for proposal (RFP);

(ii)	The Generation Entity for the Generation Resource (or an Affiliate of such Generation Entity) must own and have good title to sufficient natural gas in the storage facility for the offered Generation Resource to deliver the offered MW for at least the duration requirement specified in the RFP, and must commit to maintain such quantity of natural gas in storage at all times during the FFSS Obligation Period; and

(iii)	The Generation Entity for the Generation Resource (or an Affiliate of such Generation Entity) must have entered into a Firm Transportation Agreement on an FFSS Qualifying Pipeline, or multiple Firm Transportation Agreements on multiple Qualifying Pipelines, and:

(A)	Each Firm Transportation Agreement must have a maximum daily contract quantity sufficient to transport the quantity of natural gas described above from the storage facility to the Generation Resource in a quantity that is sufficient to allow generation of the offered FFSS MW for at least the duration requirement specified in the RFP;

(B)	At least one of the Firm Transportation Agreements must contain a primary receipt point that is the point of withdrawal for the storage facility used to comply with paragraph (i) above;

(C)	At least one of the Firm Transportation Agreements must contain a primary delivery point that permits delivery of the natural gas directly to the Generation Resource (including through a plant line or other dedicated lateral);

(D)	Each Firm Transportation Agreement must have a term that includes each hour of November 15 through March 15, i.e., during the FFSS Obligation Period; and

(E)	If multiple Firm Transportation Agreements will be used, the point of delivery for each Firm Transportation Agreement, other than the Firm Transportation Agreement that satisfies the requirements set forth in paragraph (C) above, must be a primary receipt point under another Firm Transportation Agreement such that there is a complete path for firm transportation service from the storage facility to the Generation Facility.

(iv)	If the Generation Entity will utilize a contractual right to firm gas storage capacity on a third-party system under a Firm Gas Storage Agreement to comply with paragraph (i) above rather than a self-owned physical gas storage facility to qualify, then the Firm Gas Storage Agreement must have:

(A)	A term that includes each hour of November 15 through March 15, i.e., during the FFSS Obligation Period;

(B)	A maximum storage quantity not less than the amount of natural gas needed to allow the Generation Resource to deliver the offered MW for the duration requirement specified in the RFP;

(C)	A maximum daily withdrawal quantity that permits the Generation Entity (or an Affiliate) to withdraw from storage a daily quantity of natural gas sufficient to allow the Generation Resource to deliver the offered MW for the duration requirement specified in the RFP; and

(D)	A point of withdrawal that is a primary receipt point under its Firm Transportation Agreement.

(v)	If the Generation Entity will utilize storage owned by it or an Affiliate to comply with paragraph (i) above, then the Generation Entity must certify that for the entire FFSS Obligation Period it or its Affiliate, as applicable, retains the rights to:

(A)	Sufficient storage capacity in its facility to store not less than the amount of natural gas needed to allow the Generation Resource to deliver the offered MW for the duration requirement specified in the RFP;

(B)	Withdraw from its storage a daily quantity of natural gas sufficient to allow the Generation Resource to deliver the offered MW for the duration requirement specified in the RFP; and

(C)	Withdraw from its storage facility at a point of withdrawal that is a primary receipt point under its Firm Transportation Agreement.

(vi)	The MW offered by the QSE for the Generation Resource may not be less than the Generation Resource’s LSL.

(vii)	The Generation Entity for the Generation Resource may satisfy the requirements set forth in paragraphs (i) through (v) above through use of a single, bundled agreement providing for gas supply, storage, and transportation service, as long as the bundled agreement satisfies the requirements of the definitions of Firm Transportation Agreement and Firm Gas Storage Agreement, the requirements in paragraphs (ii), (iii)(A), (iii)(D), (iv)(A), (iv)(B), and (iv)(C) above, and has a primary delivery point that permits delivery of the gas directly to the Generation Resource (including through a plant line or other dedicated lateral).

(d)	A Generation Resource may participate as a Firm Fuel Supply Service Resource (FFSSR) under only one of paragraphs (a), (b), or (c) above.

(e)	Successfully demonstrates the ability to provide FFSS in order to maintain Resource availability in the event of a natural gas curtailment or other fuel supply disruption consistent with qualifying technologies identified by the Public Utility Commission of Texas (PUCT).

(2)	The minimum FFSS capability requirement is the volume of fuel necessary to operate the Generation Resource at the FFSS MW award level for the duration requirement specified in the RFP.  This MW value must be greater than or equal to the Generation Resource’s LSL and is a limit on the MW quantity of FFSS that can be offered for the Generation Resource in the FFSS Offer Submission Form.

(3)	A Generation Resource will not be considered qualified to provide FFSS if, in a prior FFSS Obligation Period, the Generation Resource was decertified per paragraph (18) below.  However, such Generation Resource may nevertheless be considered qualified to provide FFSS if the Generation Resource:

(a)	Has subsequently been recertified, as provided in paragraph (22) below; or

(b)	The QSE representing the Generation Resource submits a corrective action plan to ERCOT and has agreement with ERCOT on that plan.

(4)	A Generation Entity may, but is not required to, submit in writing a proposed form of Firm Gas Storage Agreement or Firm Transportation Agreement (whether to be entered into by the Generation Entity or an Affiliate thereof) to ERCOT for review to be certified as an FFSS Qualified Contract in accordance with such policies and procedures as ERCOT may develop or require from time to time consistent with the requirements of the ERCOT Protocols.

(a)	ERCOT may, but is not obligated to, undertake a review of such agreement and, if acceptable, certify in writing such agreement as an FFSS Qualified Contract.  The decision whether to certify such agreement as an FFSS Qualified Contract shall be in ERCOT’s sole discretion.

(b)	To the extent that any such agreement is so certified by ERCOT, it shall constitute an FFSS Qualified Contract, and a Generation Entity may rely upon such certification for purposes of qualifying as an FFSSR under paragraph (1)(c) above.  Any material change to the ERCOT certified form of an existing FFSS Qualified Contract that affects the requirements of a firm natural gas FFSSR shall require a re-certification by ERCOT.  For the avoidance of doubt, a Firm Gas Storage Agreement or Firm Transportation Agreement meeting the requirements of the natural gas FFSSR is not required to be certified as an FFSS Qualified Contract.

(5)	A QSE representing a Generation Resource that will be offered to provide FFSS as a primary Generation Resource or an alternate Generation Resource must annually demonstrate each offered Generation Resource’s capability to use reserved fuel sources identified in paragraphs (1)(a) through (1)(c) above and sustain its output for 60 minutes at the MW value equal to the QSE’s desired level of FFSS qualification for the Resource.  The maximum MW of FFSS that can be offered for the designated Resource by the QSE must be limited to the average Real-Time net real power (in MW) telemetered for the Resource during the demonstration period.  Each QSE representing an FFSSR or prospective FFSSR must annually complete the test or successfully deploy at the maximum awarded MW amount for at least the demonstration period and inform ERCOT by August 15 of each year.  In order to complete this annual process, the QSE representing the Generation Resource(s) shall:

(a)	If qualifying by a self-test, coordinate the test with the ERCOT control room and show the Resource as having a Resource Status of “ONTEST” in its COP and through its Real-Time telemetry for the duration of the demonstration; and

(b)	Submit a Resource FFSS qualification form with the date and time of the self-test or the successful deployment that the QSE would like considered for qualification.

(6)	A QSE representing an FFSSR must ensure the full awarded FFSS capability is available by November 15 of each year awarded in the RFP.

(7)	A QSE representing an FFSSR shall update the Availability Plan for a Generation Resource to show it is unavailable to provide FFSS if it is not available to come On-Line or generate using reserved fuel.  The QSE representing an FFSSR must submit an Availability Plan for any alternate Generation Resource that were designated in the FFSS Offer Submission Form.  The QSE shall continue to show the Generation Resource is unavailable to provide FFSS in the Availability Plan until it can successfully come On-Line or generate using the reserved fuel.

[NPRR1327: Replace paragraph (7) above with the following upon system implementation of NPRR1281:] (7) A QSE representing an FFSSR shall update the Availability Plan for a Generation Resource to show it is unavailable to provide FFSS if it is not available to come On-Line or generate using reserved fuel. The QSE representing an FFSSR must submit an Availability Plan for only the alternate Generation Resource that was designated in the FFSS Offer Submission Form and is replacing the primary Generation Resource. The QSE shall continue to show that the Generation Resource is unavailable to provide FFSS in the Availability Plan until it can successfully come On-Line or generate using the reserved fuel.

(8)	An FFSSR that is not available to come On-Line shall inform the ERCOT control room as soon as practicable and update the FFSSR Availability Plan within 60 minutes of identifying the unavailability.

(9)	If the FFSSR is not available for the hours for which ERCOT has issued a Watch for winter weather, ERCOT shall claw back and/or withhold the FFSS Hourly Standby Fee for a number of days as calculated below, unless the FFSSR exhausted the fuel reserved to generate at the FFSS MW award level for the duration requirement specified in the RFP, including any fuel that was restocked following final approval or instruction from ERCOT, or the FFSSR exhausted emission hours allocated for the FFSSR, as specified in the FFSS Offer Submission Form.  Evidence of an FFSSR not being available includes, but is not limited to, an Availability Plan submission of unavailable or other communications to the ERCOT control room indicating the FFSSR is not available during the Watch.  The number of days subject to claw back and/or withholding is calculated as follows:

FFSSDCB q, r = Min(FFSSUFDW q, r * 2, 1) * 90

Where:

FFSSUFDW q, r = FFSSUHDW q, r / FFSSDW

The above variables are defined as follows:

| Variable | Unit | Definition |
| --- | --- | --- |
| FFSSUFDW q, r | none | Firm Fuel Supply Service Unavailability Factor per QSE per Resource—The unavailability factor of Resource r represented by QSE q during a Watch for winter weather. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSUHDW q, r | hour | Firm Fuel Supply Service Unavailable Hours per QSE per Resource—The number of hours that the Resource r represented by QSE q was not available during a Watch for winter weather. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| FFSSDW | hour | Firm Fuel Supply Service Duration of a Watch for Winter Weather—The duration of a Watch for winter weather that occurs during a FFSS Obligation Period. |
| FFSSDCB q, r | none | Firm Fuel Supply Service Days to Claw Back—The number of days subject to claw back for Resource r represented by QSE q, rounded to the nearest whole number. Where for a Combined Cycle Train, the Resource r is the Combined Cycle Train. |
| q | none | A QSE. |
| r | none | A primary or alternate Generation Resource approved by ERCOT to provide FFSS. |

(10)	If the FFSSR fails to come On-Line or stay On-Line during an FFSS deployment due to a fuel-related issue, ERCOT shall claw back and/or withhold the FFSS Hourly Standby Fee for 90 days.  A QSE representing an FFSSR may coordinate with ERCOT and seek approval to take the FFSSR Off-Line for no more than four hours to perform critical maintenance associated with consuming the reserved fuel.  If the QSE coordinates with ERCOT and receives approval to take the FFSSR unit Off-Line and brings the FFSSR back On-Line within four hours or less, this shall not count as failure to stay On-Line for the purpose of this paragraph.

(11)	If the FFSSR comes On-Line or continues generating using reserved fuel during an FFSS deployment, but fails to telemeter on average an HSL equal to or greater than 95% of the awarded FFSS MW value due to a fuel-related issue, ERCOT shall claw back and/or withhold the FFSS Hourly Standby Fee for 90 days, in proportion to the difference between the awarded MW value and the average telemetered HSL over the FFSS deployment period.

(12)	If the FFSSR comes On-Line or continues generating using reserved fuel during an FFSS deployment but fails to generate on average at the minimum of either 95% of the MW level instructed by ERCOT or 95% of the awarded FFSS MW value due to a fuel-related issue, ERCOT shall claw back and/or withhold the FFSS Hourly Standby Fee for 90 days, in proportion to the difference between the average MW level instructed by ERCOT over the FFSS deployment period and the corresponding average generation of the FFSSR.

(13)	If the FFSSR fails to come On-Line or stay On-Line during an FFSS deployment due to a non-fuel related issue, ERCOT shall claw back and/or withhold the FFSS Hourly Standby Fee for 15 days.

(14)	If the FFSSR comes On-Line or continues generating using reserved fuel during an FFSS deployment but fails to telemeter on average an HSL equal to or greater than 95% of the awarded FFSS MW value due to a non-fuel related issue, ERCOT shall claw back and/or withhold the FFSS Hourly Standby Fee for 15 days, in proportion to the difference between the awarded MW value and the average telemetered HSL over the FFSS deployment period.

(15)	If the FFSSR comes On-Line or continues generating using reserved fuel during an FFSS deployment but fails to generate on average at the minimum of either 95% of the MW level instructed by ERCOT or 95% of the awarded FFSS MW value due to a non-fuel related issue, ERCOT shall claw back and/or withhold the FFSS Hourly Standby Fee for 15 days, in proportion to the difference between the average MW level instructed by ERCOT over the FFSS deployment period and the corresponding average generation of the FFSSR.

(16)	Notwithstanding paragraphs (9) through (15) above, if the FFSSR is otherwise available but fails to come On-Line or is forced Off-Line due to a transmission system outage or transmission system limitation that would prevent the unit from being deployed to LSL, ERCOT shall not claw back the FFSS Hourly Standby Fee.

(17)	If conditions described in paragraphs (11) and (12) occur for the same deployment period, ERCOT shall only claw back the larger amount calculated in paragraph (11) or (12).  If conditions described in paragraphs (14) and (15) occur for the same deployment period, ERCOT shall only claw back the larger amount calculated in paragraph (14) or (15).

(18)	ERCOT shall decertify a primary Generation Resource or any alternate Generation Resource that was an FFSSR for any of the following:

(a)	Failure to come On-Line or stay On-Line during an FFSS deployment due to a fuel-related issue for two or more deployments;

(b)	If the FFSSR comes On-Line or continues generating using reserved fuel during an FFSS deployment, failure to generate on average at the minimum of either 95% of the MW level instructed by ERCOT or 95% of the awarded FFSS MW value due to a fuel-related issue for two or more deployments; or

(c)	Failure to maintain an Hourly Rolling Equivalent Availability Factor greater than or equal to 50%.

(19)	If ERCOT decertifies a primary Generation Resource, the QSE shall designate an alternate Generation Resource that was awarded through the FFSS procurement process to replace the decertified Generation Resource and continue to provide FFSS.  The designated alternate Generation Resource shall satisfy all of the requirements in paragraph (9) of Section 3.14.5, Firm Fuel Supply Service.  The designated alternate Generation Resource may no longer be an alternate for another primary Generation Resource.

[NPRR1281: Replace paragraph (19) above with the following upon system implementation:] (19) If ERCOT decertifies a primary Generation Resource, the QSE shall designate an alternate Generation Resource that was awarded through the FFSS procurement process to replace the decertified Generation Resource and continue to provide FFSS. The designated alternate Generation Resource shall satisfy all of the requirements in paragraph (10) of Section 3.14.5, Firm Fuel Supply Service.

(20)	If ERCOT decertifies an FFSSR that does not have any alternate Generation Resources that were awarded through the FFSS procurement process, ERCOT will cease payments to the QSE under Section 6.6.14.2, Firm Fuel Supply Service Hourly Standby Fee Payment and Fuel Replacement Cost Recovery, until the FFSSR is recertified by ERCOT.  ERCOT may issue one or more RFPs to replace the decertified FFSSR’s capacity for the remainder of the FFSS Obligation Period.

(21)	If ERCOT has not replaced a decertified Generation Resource’s FFSSR capacity, the QSE of a decertified Generation Resource may request to reestablish its FFSSR certification by submitting a corrective action plan to ERCOT that identifies actions taken to correct performance deficiencies and by successfully passing a new test, as described in paragraph (5) above.  ERCOT shall, in its sole discretion, determine whether a Generation Resource shall be recertified.

(22)	A decertified Generation Resource that has not been recertified by ERCOT must submit a corrective action plan to ERCOT and have agreement with ERCOT on that plan in order to be considered qualified to provide FFSS and be selected in the procurement process for any future FFSS Obligation Period.

(23)	If an FFSSR is unavailable or fails to continuously deploy due to a Force Majeure Event, the Generation Entity for such Generation Resource must provide a report to ERCOT containing certain additional information, including:

(a)	If the basis of the non-performance is a Force Majeure Event affecting the FFSSR, a description of the Force Majeure Event giving rise to the non-performance, with reasonably full details of such Force Majeure Event;

(b)	If the basis of the non-performance is the unavailability of the FFSSR’s FFSS Qualifying Pipeline or natural gas storage facility:

(i)	A copy of the relevant Firm Transportation Agreement and/or Firm Gas Storage Agreement;

(ii)	A copy of the nominations submitted or a detailed accounting of no notices volumes delivered for the gas day prior to the Force Majeure Event until the gas day after the Force Majeure Event;

(iii)	The applicable storage inventory level for the gas day prior to the Force Majeure Event until the gas day after the Force Majeure Event;

(iv)	A copy of the force majeure notice from the FFSS Qualifying Pipeline operator or storage provider; and

(v)	The capacity and flow data from the FFSS Qualifying Pipeline or storage facility for the gas day prior to the Force Majeure Event until the gas day after the Force Majeure Event;

(c)	To the best of its knowledge, how, why, and to what extent the Force Majeure Event actually and directly affected the FFSSR’s ability to perform;

(d)	The FFSSR’s heat rate;

(e)	The applicable nominations, and if applicable, no-notice delivered, on the FFSS Qualifying Pipeline from the gas day prior to the Force Majeure Event until the day after the Force Majeure Event; and

(f)	ERCOT will have the right to request that the Generation Entity provide, or cause to be provided, any additional information ERCOT deems necessary, and the Generation Entity must provide such requested information to the extent reasonably within its possession or control.  If the information is not in the possession of the Generation Entity (or its Affiliate) but may be in the possession of the FFSS Qualifying Pipeline operator or storage provider, the Generation Entity will exercise any contractual rights it has to request such information from the FFSS Qualifying Pipeline operator or storage provider, as applicable.

(24)	Unless the agreement is a certified contract, if the relevant Firm Transportation Agreement and/or Firm Gas Storage Agreement does not ensure firmness in the manner required by the ERCOT Protocols, ERCOT shall revoke the award and claw back and/or withhold all of the FFSS Hourly Standby Fees for all of the days of the FFSS Obligation Period.

(25)	For an FFSSR, a Force Majeure Event will be treated the same as any other cause for unavailability for the purposes of calculating the FFSSR’s FFSS Hourly Rolling Equivalent Availability Factor and for paragraphs (9) through (15) above.

(26)	It will constitute a material change under the ERCOT Protocols if a primary Generation Resource or any alternate Generation Resource that qualified to provide FFSS under paragraph (1)(c) above ceases to satisfy any of the requirements to qualify as an FFSSR under paragraph (1)(c) above (for example, but not limited to, if the Firm Transportation Agreement is terminated or if the FFSS Qualifying Pipeline no longer qualifies as an FFSS Qualifying Pipeline).

(a)	The QSE of such Generation Resource will be required to notify ERCOT within two Business Days of such a material change.

(b)	ERCOT may decertify a primary Generation Resource or alternate Generation Resource if such material change is, in ERCOT’s sole opinion, an adverse change (for example, but not limited to, if a Firm Transportation Agreement is terminated and not replaced with a comparable, qualifying Firm Transportation Agreement).

ERCOT Nodal Protocols

Section 22

Attachment F:  Firm Fuel Supply Service Agreement

# TBD

Standard Form Firm Fuel Supply Service Agreement

Between

Insert Participant

and

Electric Reliability Council of Texas, Inc.

This Firm Fuel Supply Service Agreement (“Agreement”), effective as of _________ of _____________, ___________ (“Effective Date”), is entered into by and between Insert Participant, a Qualified Scheduling Entity (QSE)  (“Participant”) and Electric Reliability Council of Texas, Inc., a Texas non-profit corporation (“ERCOT”).

Recitals

WHEREAS:

A.	Participant is a Qualified Scheduling Entity (QSE) of at least one Generation Resource, as those terms are defined in the ERCOT Protocols, and Participant is eligible to and intends to provide Firm Fuel Supply Service (FFSS);

B.	ERCOT is the Independent Organization certified under the Public Utility Regulatory Act, Tex. Util. Code Ann. § 39.151 (West) (PURA) for the ERCOT Region; and

C.	The Parties enter into this Agreement in order to establish the terms and conditions by which ERCOT and Participant will discharge their respective duties and responsibilities under the ERCOT Protocols.

Agreements

NOW, THEREFORE, in consideration of the mutual covenants and promises contained herein, ERCOT and Participant (the “Parties”) hereby agree as follows:

Section 1. Resource-Specific Terms.

A.	Start Date: November 15, ______

B.	End Date: March 15, ________

C. 	Firm Fuel Supply Service Resources Offer Description.

| Resource Name | Unit Code / Mnemonic | Resource Entity | Awarded Capacity (MW) | Clearing Price ($/MW) | Reserve Fuel Type | Resource Category |
| --- | --- | --- | --- | --- | --- | --- |

D.	Notice. All notices required to be given under this Agreement shall be in writing, and shall be deemed delivered upon being emailed to FFSS@ercot.com or to the Participant email listed below.  Either Party may change its email address for such notices by delivering to the other Party a written notice referring specifically to this Agreement

If to ERCOT:

Electric Reliability Council of Texas, Inc.

8000 Metropolis Drive (Building E), Suite 100

Austin, Texas 78744

FFSS@ercot.com

Tel No. (512) 225-7000

If to Participant:

[Insert Participant Name]

[Insert Contact Person/Dept.]

[Insert Street Address]

[Insert City, State Zip]

[Insert Telephone]

[Insert Email Address]

Section 2. Definitions.

A.	Unless herein defined, all definitions and acronyms found in the ERCOT Protocols shall be incorporated by reference into this Agreement.

B.	“ERCOT Protocols” shall mean the document adopted by ERCOT, including any attachments or exhibits referenced in that document, as amended from time to time, that contains the scheduling, operating, planning, reliability, and Settlement (including Customer registration) policies, rules, guidelines, procedures, standards, and criteria of ERCOT.  For the purposes of determining responsibilities and rights at a given time, the ERCOT Protocols, as amended in accordance with the change procedure(s) described in the ERCOT Protocols, in effect at the time of the performance or non-performance of an action, shall govern with respect to that action.

Section 3. Term and Termination.

A.	Term.

(1)	This Agreement is effective beginning on the Effective Date.

(2)	The full term (“Full Term”) of this Agreement begins on the November 15, ____ through March 15, ____.

B.	Termination by Participant. Participant may, at its option, terminate this Agreement immediately upon the failure of ERCOT to continue to be certified by the Public Utility Commission of Texas (PUCT) as the Independent Organization under PURA §39.151 without the immediate certification of another Independent Organization under PURA §39.151.

C.	Effect of Termination and Survival of Terms. If this Agreement is terminated by a Party pursuant to the terms hereof, the rights and obligations of the Parties hereunder shall terminate, except that the rights and obligations of the Parties that have accrued under this Agreement prior to the date of termination shall survive.

Section 4. Representations, Warranties, and Covenants.

A.	Participant represents, warrants, and covenants that:

(1)	Participant is duly organized, validly existing, and in good standing under the laws of the jurisdiction under which it is organized, and is authorized to do business in Texas;

(2)	Participant has full power and authority to enter into this Agreement and perform all of Participant’s obligations, representations, warranties, and covenants under this Agreement;

(3)	Participant’s past, present, and future agreements or Participant’s organizational charter or bylaws, if any, or any provision of any indenture, mortgage, lien, lease, agreement, order, judgment, or decree to which Participant is a party or by which its assets or properties are bound do not materially affect performance of Participant’s obligations under this Agreement;

(4)	The execution, delivery, and performance of this Agreement by Participant have been duly authorized by all requisite action of its governing body;

(5)	Except as set out in an exhibit (if any) to this Agreement, ERCOT has not, within the 24 months preceding the Effective Date, terminated for Default any Prior Agreement with Participant, any company of which Participant is a successor in interest, or any Affiliate of Participant;

(6)	If any Defaults are disclosed on any such exhibit mentioned in subsection 4(A)(5), either (a) ERCOT has been paid, before execution of this Agreement, all sums due to it in relation to such Prior Agreement, or (b) ERCOT, in its reasonable judgment, has determined that this Agreement is necessary for system reliability, and Participant has made alternate arrangements satisfactory to ERCOT for the resolution of the Default under the Prior Agreement;

(7)	Participant has obtained, or will obtain prior to beginning performance under this Agreement, all licenses, registrations, certifications, permits, and other authorizations and has taken, or will take prior to beginning performance under this Agreement, all actions required by applicable laws or governmental regulations except licenses, registrations, certifications, permits or other authorizations that do not materially affect performance under this Agreement;

(8)	Participant is not in violation of any laws, ordinances, or governmental rules, regulations, or order of any Governmental Authority or arbitration board materially affecting performance of this Agreement and to which it is subject;

(9)	Participant is not Bankrupt, does not contemplate becoming Bankrupt nor, to its knowledge, will become Bankrupt;

(10)	Participant acknowledges that it has received and is familiar with the ERCOT Protocols; and

(11)	Participant acknowledges and affirms that the foregoing representations, warranties, and covenants are continuing in nature throughout the term of this Agreement.  For purposes of this Section, “materially affecting performance” means resulting in a materially adverse effect on Participant’s performance of its obligations under this Agreement.

B.	ERCOT represents, warrants, and covenants that:

(1)	ERCOT is the Independent Organization certified under PURA §39.151 for the ERCOT Region;

(2)	ERCOT is duly organized, validly existing, and in good standing under the laws of Texas, and is authorized to do business in Texas;

(3)	ERCOT has full power and authority to enter into this Agreement and perform all of ERCOT’s obligations, representations, warranties and covenants under this Agreement;

(4)	ERCOT’s past, present and future agreements or ERCOT’s organizational charter or bylaws, if any, or any provision of any indenture, mortgage, lien, lease, agreement, order, judgment, or decree to which ERCOT is a party or by which its assets or properties are bound do not materially affect performance of ERCOT’s obligations under this Agreement;

(5)	The execution, delivery, and performance of this Agreement by ERCOT have been duly authorized by all requisite action of its governing body;

(6)	ERCOT has obtained, or will obtain prior to beginning performance under this Agreement, all licenses, registrations, certifications, permits and other authorizations and has taken, or will take prior to beginning performance under this Agreement, all actions required by applicable laws or governmental regulations except licenses, registrations, certifications, permits or other authorizations that do not materially affect performance under this Agreement;

(7)	ERCOT is not in violation of any laws, ordinances, or governmental rules, regulations or order of any Governmental Authority or arbitration board materially affecting performance of this Agreement and to which it is subject;

(8)	ERCOT is not Bankrupt, does not contemplate becoming Bankrupt nor, to its knowledge, will become Bankrupt; and

(9)	ERCOT acknowledges and affirms that the foregoing representations, warranties, and covenants are continuing in nature throughout the term of this Agreement.  For purposes of this Section, “materially affecting performance” means resulting in a materially adverse effect on ERCOT’s performance of its obligations under this Agreement.

Section 5. Participant Obligations.

A.	Participant shall comply with, and be bound by, all ERCOT Protocols, ERCOT Operating Guides, and the North American Electric Reliability Corporation (NERC) Reliability Standards as they pertain to operation of a Firm Fuel Supply Service Resource by a Resource Entity.

B.	Participant shall not take any action, without first providing written notice to ERCOT and reasonable time for ERCOT and Market Participants to respond, that would cause a Market Participant within the ERCOT Region that is not a “public utility” under the Federal Power Act, 16 U.S.C. § 824(e)(2005), or ERCOT itself to become a “public utility” under the Federal Power Act or become subject to the plenary jurisdiction of the Federal Energy Regulatory Commission (FERC).

C. 	For all Contractual off-site FFSS category Resources listed above under Section 1, Part C, the Participant has a firm transportation agreement with a natural gas pipeline that is a critical natural gas facility as that is defined in 16 Texas Administrative Code 25.52(c)(2).

Section 6. ERCOT Obligations.

A.	ERCOT shall comply with, and be bound by, all ERCOT Protocols.

B.	ERCOT shall not take any action, without first providing written notice to Participant and reasonable time for Participant and other Market Participants to respond, that would cause Participant if Participant is not a “public utility” under the Federal Power Act, or ERCOT itself to become a “public utility” under the Federal Power Act or become subject to the plenary jurisdiction of the FERC.  If ERCOT receives any notice similar to that described in Section 5(B) from any Market Participant, ERCOT shall provide notice of same to Participant.

Section 7. Firm Fuel Supply Service Decertification.

If a Firm Fuel Supply Service Resource does not remain certified, or if Participant is in default as described in the Standard Form Market Participant Agreement referenced in Section 10(A) during the term of this Agreement, then the Firm Fuel Supply Service Hourly Standby Fee for each applicable Resource is reduced to zero for the remainder of the Full Term, except in the case of a decertified Resource being re-certified. Participant may be required to refund to ERCOT certain amounts paid by ERCOT under this Agreement during the Full Term as described in the ERCOT Protocols.

Section 8. Operation.

A.	Firm Fuel Supply Service Resource Maintenance. No Planned Outages for inspection, repair, maintenance, and overhaul of the Firm Fuel Supply Service Resource may occur during the term of this Agreement. Dec 1 – March 1

B.	Planning Data.

Participant shall timely report to ERCOT those items and conditions necessary for ERCOT’s internal planning and compliance with ERCOT’s guidelines in effect from time to time.  The information supplied must include, without limitation, the following:

(1)	Availability Plan for each hour of the next Operating Day submitted by 0600 of the preceding day; and

(2)	Revised Availability Plan reflecting changes in hourly availability of Firm Fuel Supply Service Capacity status as indicated in a revised Availability Plan as soon as reasonably practical, but in no event later than 60 minutes after the event that caused the change.

Section 9. Payment.

A.	Firm Fuel Supply Service Hourly Standby Fee Payments. ERCOT shall pay Participant the Hourly Standby Fee as described in Section 6.6.14.2, Firm Fuel Supply Service Hourly Standby Fee Payment and Fuel Replacement Cost Recovery, of the ERCOT Protocols, except as specified otherwise in Section 7 above.

Section 10. Standard Form Market Participant Agreement Applicability.

A.	If not otherwise expressly covered by this Agreement, the terms of the Section 22, Attachment A: Standard Form Market Participant Agreement control.

SIGNED, ACCEPTED, AND AGREED TO by each undersigned signatory who, by signature hereto, represents and warrants that he or she has full power and authority to execute this Agreement.

### Electric Reliability Council of Texas, Inc.:

By: ______________________________

Name: ____________________________

Title: _____________________________

Date: _____________________________

### Participant:

By: ______________________________

Name:

Title:

Date:

Market Participant Name:

Market Participant DUNS:

Oil or Natural Gas

On-site FFSS category, Resource-controlled FFSS category, or Contractual off-site FFSS category