| NOGRR Number | 287 | NOGRR Title | Load Shed Provisions to Identify and Minimize Overlap of Essential Loads |
| --- | --- | --- | --- |
| Date of Decision |  | September 15, 2026 |  |
| Action |  | Recommended Approval |  |
| Timeline |  | Normal |  |
| Estimated Impacts |  | Cost/Budgetary: None Project Duration: No project required |  |
| Proposed Effective Date |  | April 1, 2027 |  |
| Priority and Rank Assigned |  | Not applicable |  |
| Nodal Operating Guide Sections Requiring Revision |  | 4.5.3.4, Load Shed Obligation 8L, Emergency Operations Plan |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Operating Guide Revision Request (NOGRR) establishes guidelines for Transmission Operators (TOs) and Transmission and/or Distribution Service Providers (TDSPs) to collaborate in identifying and prioritizing critical natural gas infrastructure Loads essential for the reliability of the ERCOT System. Furthermore, this NOGRR outlines provisions for TOs and TDSPs to minimize overlaps between circuits designated for manual Load shedding, Under-Voltage Load Shedding (UVLS), and Under-Frequency Load Shedding (UFLS), and those circuits identified as critical Loads necessary for maintaining reliability of the ERCOT System. |  |
| Reason for Revision |  | Strategic Plan Objective 1 – Be an industry leader for grid reliability and resilience Strategic Plan Objective 2 - Enhance the ERCOT region’s economic competitiveness with respect to trends in wholesale power rates and retail electricity prices to consumers Strategic Plan Objective 3 - Advance ERCOT, Inc. as an independent leading industry expert and an employer of choice by fostering innovation, investing in our people, and emphasizing the importance of our mission General system and/or process improvement(s) Regulatory requirements ERCOT Board/PUCT Directive (please select ONLY ONE – if more than one apply, please select the ONE that is most relevant) |  |
| Justification of Reason for Revision and Market Impacts |  | Provisions of North American Electric Reliability Corporation (NERC) Reliability Standard EOP-011-4, that go into effect on April 1, 2027, require ERCOT, as the balancing authority, to develop and implement operating plan(s) to mitigate Capacity Emergencies and Energy Emergencies, as those terms are defined in NERC’s Glossary of Terms Used in NERC Reliability Standards. The plan must include provisions for Transmission Operators to implement operator-controlled manual Load shed, UVLS, or UFLS during an Emergency that includes 1) provisions to minimize the overlap of circuits that are designated for manual Load shed, UVLS, or UFLS, and circuits that serve designated critical loads that are essential to the reliability of the Bulk Electric System and 2) provisions for the identification and prioritization of designated critical natural gas infrastructure loads that are essential to the reliability of the Bulk Electric System, as defined by the Applicable Entity. The revisions in this NOGRR are necessary to align ERCOT with TOs and align TOs with TDSPs before and during an Energy Emergency Alert (EEA) Level 3 Load shed event to ensure compliance with the new EOP-011-4 requirements and clarify each entity’s responsibilities concerning coordination. |  |
| ROS Decision |  | On 5/8/26, ROS voted unanimously to recommend approval of NOGRR287 with a proposed effective date of no earlier than 4/1/27. All Market Segments participated in the vote. On 6/4/26, ROS voted unanimously to endorse and forward to TAC the 5/7/26 ROS Report, as amended by the 6/1/26 ERCOT comments, and the 4/22/26 Impact Analysis for NOGRR287. All Market Segments participated in the vote. |  |
| Summary of ROS Decision |  | On 5/8/26, ERCOT Staff presented NOGRR287 and the regulatory alignments of the NOGRR, and suggested that the language remain grayboxed until April 1, 2027. Participants discussed differences on the submitted language versus draft language, the need to reinsert the inadvertently omitted “as defined by TO,” and determined to advance NOGRR287 as-is and adopt correcting comments when the Impact Analysis is considered. On 6/4/26, participants reviewed the 6/1/26 ERCOT comments reinserting the inadvertently omitted “as defined by TO.” |  |
| TAC Decision |  | On 7/29/26, TAC voted unanimously to recommend approval of NOGRR287 as recommended by ROS in the 6/4/26 ROS Report. All Market Segments participated in the vote. |  |
| Summary of TAC Discussion |  | On 7/29/26, there was no additional discussion beyond TAC review of the items below. |  |
| TAC Review/Justification of Recommendation |  | Revision Request ties to Reason for Revision as explained in Justification Impact Analysis reviewed and impacts are justified as explained in Justification Opinions were reviewed and discussed Comments were reviewed and discussed (if applicable) Other: (explain) |  |
| ERCOT Board Decision |  | On 9/15/26, the ERCOT Board voted unanimously to recommend approval of NOGRR287 as recommended by TAC in the 7/29/26 TAC Report. |  |

| Opinions |  |
| --- | --- |
| Credit Review | Not applicable |
| Independent Market Monitor Opinion | IMM has no opinion on NOGRR287. |
| ERCOT Opinion | ERCOT supports approval of NOGRR287. |
| ERCOT Market Impact Statement | ERCOT Staff believe NOGRR287 is necessary to establish provisions requiring TOs and TDSPs to collaborate in identifying and prioritizing critical natural gas infrastructure Loads that are essential to the reliability of the ERCOT System. The proposed revisions align with the EOP-011-4 requirements by ensuring that NERC-registered entities have clear provisions applicable to the ERCOT System and by clarifying each entity’s responsibilities for coordinating the identification of critical natural gas Loads and implementing the prioritization of those Loads during EEA Level 3 Load shed event. |

| Sponsor |  |
| --- | --- |
| Name | Vamsi Madam |
| E-mail Address | Vamsi.madam@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4275 |
| Cell Number |  |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Elizabeth Morales |
| E-Mail Address | Elizabeth.morales@ercot.com |
| Phone Number | 210-420-1722 |

| Comments Received |  |
| --- | --- |
| Comment Author | Comment Summary |
| ERCOT 052726 | Reinstated the inadvertently omitted “as defined by TO” |

Proposed Guide Language Revision

### 4.5.3.4	Load Shed Obligation

[NOGRR265: Replace the title for Section 4.5.3.4 above with the following upon system implementation of NPRR1238:] 4.5.3.5 Transmission Operator Load Shed Obligation

(1)	Each TO shall take and direct actions to ensure that ERCOT Load shed instructions are effectuated.  Each DSP shall comply with any reasonable instruction given by its TO to effectuate Load shed obligations.

(2)	Load shed obligation percentages for ERCOT EEA Level 3 Load shedding will be determined by calculating each TO’s Load as a percentage of the ERCOT System summer and winter peak 15 minute Demand interval.  For the purposes of this paragraph, TO Load will be the amount of Load being served by all of the TDSPs that the TO represents.  The calculations for summer and winter Load shed obligation percentage are as follows:

[NOGRR265: Replace paragraph (2) above with the following upon system implementation of NPRR1238:] (2) Load shed obligation percentages for ERCOT EEA Level 3 Load shedding will be determined by calculating each TO’s Load as a percentage of the ERCOT System summer and winter peak 15 minute Demand interval. For the purposes of this paragraph, TO Load, with the exception of VECLs, will be the amount of Load being served by all of the Transmission and/or Distribution Service Providers (TDSPs) that the TO represents. The calculations for summer and winter Load shed obligation percentage are as follows:

(a)	The calculated Load shed obligation percentage for the summer Season will be based on the single highest coincident ERCOT System peak 15 minute Demand interval for the summer months of June through September as reflected in the 4-Coincident Peak (4-CP) data submitted by ERCOT to the Public Utility Commission of Texas (PUCT) for that year.  Anticipated revisions to the summer Load shed table shall be posted as described in paragraph (4) below no later than March 31st of each year based on data from the previous calendar year.

(b)	The calculated Load shed obligation percentage for the winter Season will be based on the single highest coincident ERCOT System peak 15 minute Demand interval for the winter months of December through February as reflected at the time that ERCOT extracts the Load data for the winter Season from its settlement system.  Anticipated revisions to the winter Load shed table shall be posted as described in paragraph (4) below no later than August 31st of each year based on data from December of the previous calendar year and January through February of the current year.

(3)	The summer Load shed table will be used during a hot weather Load shed event and the winter Load shed table will be used during a cold weather Load shed event.  ERCOT will determine, in its sole discretion, whether an EEA event will be treated as a hot weather or cold weather Load shed event based on the weather conditions.  The summer and winter Load shed time periods will be published annually with the updated obligation tables in paragraph (2) above.  In addition, if ERCOT issues an Operating Condition Notice (OCN), it will notify Market Participants which Load shed table would apply to the potential Load shed event.  When ERCOT directs TOs to shed Load, it will specify which Load shed table applies for the Load shed event.  ERCOT shall use the same Load shed table for the duration of a Load shed event.

(4)	ERCOT shall maintain the Seasonal Load shed tables reflecting each TO’s total Load shed obligation on the ERCOT website.  The Load shed obligation percentages will be reviewed by ERCOT and revised as described above, or as otherwise deemed appropriate by ERCOT, to reflect any new or changed TO designation by a DSP.  Adjustments to the Load shed obligations due to changes in TO designations will be performed using the same Load data upon which the table was based.  Following ERCOT’s Seasonal peak Load reviews or ERCOT’s receipt of any new or changed TO designation, ERCOT shall post any anticipated revisions to the Load shed tables on the ERCOT website.  ERCOT shall issue a Market Notice announcing the posting of the revisions at least ten days prior to the effective date of the revisions or as soon as practicable if ERCOT determines there is a need to correct the Market Notice less than ten days before the effective date.

[NOGRR265: Replace paragraph (4) above with the following upon system implementation of NPRR1238:] (4) ERCOT shall maintain the Seasonal Load shed tables reflecting each TO’s total Load shed obligation on the ERCOT website. The Load shed obligation percentages will be reviewed by ERCOT and revised as described above, or as otherwise deemed appropriate by ERCOT, to reflect any new or changed TO designation by a DSP or changes in the VECL registration. Adjustments to the Load shed obligations due to changes in TO designations will be performed using the same Load data upon which the table was based. Following ERCOT’s Seasonal peak Load reviews or ERCOT’s receipt of any new or changed TO designation, ERCOT shall post any anticipated revisions to the Load shed tables on the ERCOT website. ERCOT shall issue a Market Notice announcing the posting of the revisions at least ten days prior to the effective date of the revisions or as soon as practicable if ERCOT determines there is a need to correct the Market Notice less than ten days before the effective date.

(5)	Each TO shall coordinate with each TDSP it represents to:

(a) 	Identify and prioritize designated critical natural gas infrastructure Loads in its area that are essential to the reliability of the ERCOT System as defined by the TO;

(b)	Minimize overlap of circuits that are designated for manual  Load shed, UFLS, or Under-Voltage Load Shed (UVLS) and circuits that serve designated critical Loads in its TO area that are essential to the reliability of the ERCOT System as defined by the TO; and

(c)	Minimize overlap of circuits that are designated for manual  Load shed with circuits that are utilized for UFLS  or UVLS.

ERCOT Nodal Operating Guides

Section 8

Attachment L

Emergency Operations Plan

TBD

This attachment provides a template to be used by each Transmission Operator (TO) for the development of its emergency operations plan to mitigate operating emergencies, as required by the applicable North American Electric Reliability Corporation (NERC) Reliability Standard.  The emergency operations plan can be made up of multiple parts and does not need to be a single document.  When multiple parts are used, the TO shall include documentation describing the location of each element required by the applicable NERC Reliability Standard.  Each plan should include each of the elements listed below:

I.	PURPOSE – The purpose statement will address the TO’s operations plan to mitigate operating emergencies.

II.	SCOPE – The scope statement shall provide, in a brief summary, the boundaries of the emergency operations plan and to whom the emergency operations plan applies.

III.	DEFINITIONS – Definitions of terms that are used in the TO emergency operations plan that are not common to the ERCOT Region.  Define what is considered an operating emergency.

IV.	KEY PERSONNEL ROLES AND RESPONSIBILITIES – Identify roles and responsibilities of key personnel that are responsible for activating the plan.

V.	PROCESSES TO PREPARE FOR AND MITIGATE EMERGENCIES – Include the following:

A.	Notification to ERCOT to include current and known projected Real-Time conditions, when experiencing an operating emergency;

B.	Cancellation of Transmission Facility Outages;

C.	Transmission system reconfiguration;

D.	Operator-controlled manual Load shed, Under-Voltage Load Shed (UVLS), or Under-Frequency Load Shed (UFLS) during an Emergency Condition that accounts for each of the following:

1.	Provisions for manual Load shed capable of being implemented in a timeframe adequate for mitigating the emergency;

2.	Provisions to minimize the overlap of circuits that are designated for manual Load shed, UVLS, or UFLS and circuits that serve designated critical loads in its TO area that are essential to the reliability of the ERCOT System as defined by the TO;

3.	Provisions to minimize the overlap of circuits that are designated for manual Load shed and circuits that are utilized for UFLS or UVLS;

4.	Provisions to limit the utilization of UFLS or UVLS circuits for manual Load shed to situations where such use is consistent with the ERCOT Nodal Protocols and ERCOT Nodal Operating Guides and is warranted by system conditions; and

5. 	Provisions for the identification and prioritization of designated critical natural gas infrastructure in its TO area that are essential to the reliability of the ERCOT System as defined by the TO.

E.	Provisions to determine reliability impacts of:

1.	Cold weather conditions; and

2.	Extreme weather conditions.