| NPRR Number | 1349 | NPRR Title | 765kV Transmission Outage Priority and Modeling Timelines |
| --- | --- | --- | --- |
| Date Posted |  | September 29, 2026 |  |
| Requested Resolution |  | Normal |  |
| Nodal Protocol Sections Requiring Revision |  | 3.1.5.8 Priority of Approved Planned, Maintenance, and Rescheduled Outages 3.10.1, Time Line for Network Operations Model Changes |  |
| Related Documents Requiring Revision/Related Revision Requests |  | None |  |
| Revision Description |  | This Nodal Protocol Revision Request (NPRR) establishes timelines for the modeling of new 765kV and its supporting transmission in the ERCOT Network Operations Model and allows ERCOT to give priority to transmission Outages in support of 765kV transmission construction, commissioning, and energization. |  |
| Reason for Revision |  | Strategic Plan Objective 1 – Be an industry leader for grid reliability and resilience Strategic Plan Objective 2 - Enhance the ERCOT region’s economic competitiveness with respect to trends in wholesale power rates and retail electricity prices to consumers Strategic Plan Objective 3 - Advance ERCOT, Inc. as an independent leading industry expert and an employer of choice by fostering innovation, investing in our people, and emphasizing the importance of our mission General system and/or process improvement(s) Regulatory requirements ERCOT Board/PUCT Directive (please select ONLY ONE – if more than one apply, please select the ONE that is most relevant) |  |
| Justification of Reason for Revision and Market Impacts |  | As 765kV transmission is developed in ERCOT, long lead time Outages may be necessary to support its construction, commissioning, and energization. To ensure that the equipment is available for Transmission Service Providers (TSPs) to submit and prioritize a related Outage, this NPRR establishes the ability to model new 765kV and its supporting transmission greater than a year of the required submission date. In addition, currently per paragraph (1) of Section 3.1.5.8, the only mechanism to give priority to a planned, maintenance, or rescheduled Outage is by the order in which the outage was received. Outages in support of the 765kV transmission construction should be submitted with enough lead time that they would already be given priority per existing language. For unforeseen circumstances, ERCOT in coordination with ERCOT TSPs and PUCT Staff submits this revision to address situations where another Outage with submission priority, unrelated to the 765kV construction, may be in conflict with a supporting 765kV Outage that may not be able to be rescheduled due the complex and interrelated nature of the Outages that will be necessary to complete a very large project. This revision gives ERCOT, at its sole discretion, the ability to consider giving priority to a transmission Outage that is in support of the 765kV transmission construction, commissioning, and energization. This revision does not guarantee that a supporting 765kV Outage will be given priority. There may be instances where another high impact Outage that already maintains priority preserves that priority. ERCOT appreciates consideration of these revisions to the Nodal Protocols. |  |

| Sponsor |  |
| --- | --- |
| Name | Freddy Garcia |
| E-mail Address | Freddy.Garcia@ercot.com |
| Company | ERCOT |
| Phone Number | 512-248-4245 |
| Cell Number |  |
| Market Segment | Not Applicable |

| Market Rules Staff Contact |  |
| --- | --- |
| Name | Brittney Albracht |
| E-Mail Address | Brittney.Albracht@ercot.com |
| Phone Number | 512-225-7027 |

Proposed Protocol Language Revision

3.1.5.8	Priority of Approved Planned, Maintenance, and Rescheduled Outages

(1)	In considering TSP requests, ERCOT shall give priority to Planned Outages, Maintenance Outages, and Rescheduled Outages in the order of receipt.

[NPRR857: Replace paragraph (1) above with the following upon system implementation and satisfying the following conditions: (1) Southern Cross provides ERCOT with funds to cover the entire estimated cost of the project; and (2) Southern Cross has signed an interconnection agreement with a TSP and the TSP gives ERCOT written notice that Southern Cross has provided it with: (a) Notice to proceed with the construction of the interconnection; and (b) The financial security required to fund the interconnection facilities:] (1) In considering TSP or DCTO requests, ERCOT shall give priority to Planned Outages, Maintenance Outages, and Rescheduled Outages in the order of receipt.

(2)	ERCOT, at its sole discretion, may give priority to transmission Outages in support of 765 kV transmission construction, commissioning, and energization.

3.10.1	Time Line for Network Operations Model Changes

(1)	ERCOT shall perform periodic updates to the Network Operations Model.  Market Participants may provide Network Operations Model updates to ERCOT to implement planned transmission and Resource construction up to one year before the required submittal date below.  Updates may not be submitted more than one year before the required submitted date unless they are submitted per paragraph (5) below.   ERCOT shall have the discretion to accept changes further than one year out if ERCOT deems it necessary.  TSPs and Resource Entities must timely submit Network Operations Model changes pursuant to the schedule in this Section to be included in the updates.

[NPRR857: Replace paragraph (1) above with the following upon system implementation and satisfying the following conditions: (1) Southern Cross provides ERCOT with funds to cover the entire estimated cost of the project; and (2) Southern Cross has signed an interconnection agreement with a TSP and the TSP gives ERCOT written notice that Southern Cross has provided it with: (a) Notice to proceed with the construction of the interconnection; and (b) The financial security required to fund the interconnection facilities:] (1) ERCOT shall perform periodic updates to the Network Operations Model. Market Participants may provide Network Operations Model updates to ERCOT to implement planned transmission and Resource construction one year before the required submittal date below. TSPs, DCTOs, and Resource Entities must timely submit Network Operations Model changes pursuant to the schedule in this Section to be included in the updates.

(2)	For a facility addition, revision, or deletion to be included in any Network Operations Model update, all technical modeling information must be submitted to ERCOT pursuant to the ERCOT NOMCR process or the applicable Resource Registration process for Resource Entities.  If a Resource Entity is required to follow the generation interconnection process for a new Generation Resource, Energy Storage Resource (ESR), or Settlement Only Generator (SOG) as described in Planning Guide Section 5, Generator Interconnection or Modification, it must meet the conditions of Planning Guide Section 6.9, Addition of Proposed Generation to the Planning Models, before submitting a change to the Network Operations Model to reflect the new Generation Resource, ESR, or SOG.

[NPRR995: Replace paragraph (2) above with the following upon system implementation:] (2) For a facility addition, revision, or deletion to be included in any Network Operations Model update, all technical modeling information must be submitted to ERCOT pursuant to the ERCOT NOMCR process or the applicable Resource Registration process for Resource Entities. If a Resource Entity is required to follow the generation interconnection process for a new Generation Resource, Energy Storage Resource (ESR), Settlement Only Generator (SOG), or Settlement Only Energy Storage System (SOESS) as described in Planning Guide Section 5, Generator Interconnection or Modification, it must meet the conditions of Planning Guide Section 6.9, Addition of Proposed Generation to the Planning Models, before submitting a change to the Network Operations Model to reflect the new Generation Resource, ESR, SOG, or SOESS.

(3)	TSPs and Resource Entities shall submit all Network Operations Model updates that are not subject to the requirements of paragraph (4) or (5) below by the applicable deadline to submit information to ERCOT for the target date of inclusion in the production model detailed in the table below.  ERCOT shall update the Network Operations Model according to the following table:

| Deadline to Submit Information to ERCOT Note 1 | Model Complete and Available for Test Note 2 | Updated Network Operations Model Testing Complete Note 3 Paragraph (6) | Update Network Operations Model Production Environment Notes 4, 5 |
| --- | --- | --- | --- |
| Jan 1 | Feb 15 | March 15 | Month of April |
| Feb 1 | March 15 | April 15 | Month of May |
| March 1 | April 15 | May 15 | Month of June |
| April 1 | May 15 | June 15 | Month of July |
| May 1 | June 15 | July 15 | Month of August |
| June 1 | July 15 | August 15 | Month of September |
| July 1 | August 15 | September 15 | Month of October |
| August 1 | September 15 | October 15 | Month of November |
| September 1 | October 15 | November 15 | Month of December |
| October 1 | November 15 | December 15 | Month of January (the next year) |
| November 1 | December 15 | January 15 | Month of February (the next year) |
| December 1 | January 15 | February 15 | Month of March (the next year) |

Notes:

1.	TSP and Resource Entity data submissions complete per the NOMCR process or other ERCOT-prescribed process applicable to Resource Entities for inclusion in next update period.

2.	Network Operations Model data changes and preliminary fidelity test complete by using the Network Operations Model test facility described in paragraph (3) of Section 3.10.4, ERCOT Responsibilities.  A test version of the Redacted Network Operations Model will be posted to the MIS Secure Area for Market Participants and Network Operations Model to the MIS Certified Area for TSPs as described in paragraph (9) of Section 3.10.4, for market review and further testing by Market Participants.

3.	Testing of the Redacted Network Operations Model by Market Participants and Network Operations Model by TSPs is complete and ERCOT begins the Energy Management System (EMS) testing prior to placing the new model into the production environment.

4.	Updates include changes starting at this date and ending within the same month.  The schedule for Operations Model load dates will be published by ERCOT on the ERCOT website.  This includes all other Network Operations Model modeling submissions and the production load dates for operational model loads.

5.	Changes in the production load dates can be delayed or moved when operating conditions, system application issues, or legal requirements will not allow for disruptions.  In such cases ERCOT shall issue a Market Notice to inform Market Participants of changes to production load schedules.

[NPRR857: Replace paragraph (3) above with the following upon system implementation and satisfying the following conditions: (1) Southern Cross provides ERCOT with funds to cover the entire estimated cost of the project; and (2) Southern Cross has signed an interconnection agreement with a TSP and the TSP gives ERCOT written notice that Southern Cross has provided it with: (a) Notice to proceed with the construction of the interconnection; and (b) The financial security required to fund the interconnection facilities for NPRR857:] (3) TSPs, DCTOs, and Resource Entities shall submit all Network Operations Model updates that are not subject to the requirements of paragraph (4) below by the applicable deadline to submit information to ERCOT for the target date of inclusion in the production model detailed in the table below. ERCOT shall update the Network Operations Model according to the following table: Deadline to Submit Information to ERCOT Note 1; Model Complete and Available for Test Note 2; Updated Network Operations Model Testing Complete Note 3 Paragraph (6); Update Network Operations Model Production Environment Notes 4, 5. January 1; February 15; March 15; Month of April. February 1; March 15; April 15; Month of May. March 1; April 15; May 15; Month of June. April 1; May 15; June 15; Month of July. May 1; June 15; July 15; Month of August. June 1; July 15; August 15; Month of September. July 1; August 15; September 15; Month of October. August 1; September 15; October 15; Month of November. September 1; October 15; November 15; Month of December. October 1; November 15; December 15; Month of January (the next year). November 1; December 15; January 15; Month of February (the next year). December 1; January 15; February 15; Month of March (the next year). Notes: 1. TSP, DCTO, and Resource Entity data submissions complete per the NOMCR process or other ERCOT-prescribed process applicable to Resource Entities for inclusion in next update period. 2. Network Operations Model data changes and preliminary fidelity test complete by using the Network Operations Model test facility described in paragraph (3) of Section 3.10.4, ERCOT Responsibilities. A test version of the Redacted Network Operations Model will be posted to the MIS Secure Area for Market Participants and Network Operations Model to the MIS Certified Area for TSPs as described in paragraph (9) of Section 3.10.4, for market review and further testing by Market Participants. 3. Testing of the Redacted Network Operations Model by Market Participants and Network Operations Model by TSPs is complete and ERCOT begins the Energy Management System (EMS) testing prior to placing the new model into the production environment. 4. Updates include changes starting at this date and ending within the same month. The schedule for Operations Model load dates will be published by ERCOT on the ERCOT website. This includes all other Network Operations Model modeling submissions and the production load dates for operational model loads. 5. Changes in the production load dates can be delayed or moved when operating conditions, system application issues, or legal requirements will not allow for disruptions. In such cases ERCOT shall issue a Market Notice to inform Market Participants of changes to production load schedules.

(4)	Resource Entities shall submit complete initial Resource Registration data for inclusion in the ERCOT Network Operations Model as described in paragraph (6) of Planning Guide Section 6.8.1, Resource Registration, by the applicable deadline for the Resource Entity to submit complete information to ERCOT for the target date of inclusion in the production model detailed in the table below.  ERCOT shall update the Network Operations Model according to the following table:

| Deadline for Resource Entity to Submit Complete Information to ERCOT Note 1 | Deadline for Resource Registration Data to Meet Criteria for ERCOT Acceptance Note 2 | Model Complete and Available for Test Note 3 | Updated Network Operations Model Testing Complete Note 4 Paragraph (6) | Update Network Operations Model Production Environment Notes 5, 6 |
| --- | --- | --- | --- | --- |
| December 1 | January 1 | February 15 | March 15 | Month of April |
| January 1 | February 1 | March 15 | April 15 | Month of May |
| February 1 | March 1 | April 15 | May 15 | Month of June |
| March 1 | April 1 | May 15 | June 15 | Month of July |
| April 1 | May 1 | June 15 | July 15 | Month of August |
| May 1 | June 1 | July 15 | August 15 | Month of September |
| June 1 | July 1 | August 15 | September 15 | Month of October |
| July 1 | August 1 | September 15 | October 15 | Month of November |
| August 1 | September 1 | October 15 | November 15 | Month of December |
| September 1 | October 1 | November 15 | December 15 | Month of January (the next year) |
| October 1 | November 1 | December 15 | January 15 | Month of February (the next year) |
| November 1 | December 1 | January 15 | February 15 | Month of March (the next year) |

Notes:

1.	The date listed in this column shall serve as the deadline for initial submission of complete Resource Registration data to ERCOT, as described in paragraph (2) of Planning Guide Section 6.8.2, Resource Registration Process.  ERCOT may work with the Resource Entity to resolve any data quality issues found in the Resource Registration data for up to one month after the submission date that corresponds to the date listed in this column.  If ERCOT determines any Resource Registration data deficiencies are not sufficiently resolved by the end of the one-month period, then that submission shall be treated as a new initial submission for the following month.

2.	Resource Entity data submission must be deemed complete by ERCOT with all data deficiencies resolved per the process described in Planning Guide Section 6.8.2 by this date.

3.	Network Operations Model data changes and preliminary fidelity test complete by using the Network Operations Model test facility described in paragraph (3) of Section 3.10.4.  A test version of the Redacted Network Operations Model will be posted to the MIS Secure Area for Market Participants and Network Operations Model to the MIS Certified Area for TSPs as described in paragraph (9) of Section 3.10.4, for market review and further testing by Market Participants.

4.	Testing of the Redacted Network Operations Model by Market Participants and Network Operations Model by TSPs is complete and ERCOT begins the EMS testing prior to placing the new model into the production environment.

5.	Updates include changes starting at this date and ending within the same month.  The schedule for Operations Model load dates will be published by ERCOT on the ERCOT website.

6.	Changes in the production load dates can be delayed or moved when operating conditions, system application issues, or legal requirements will not allow for disruptions.  In such cases ERCOT shall issue a Market Notice to inform Market Participants of changes to production load schedules.

(5)	All new 765kV and supporting transmission facilities not existing in the Network Operations Model shall be submitted to the Network Operations Model as soon as practicable and may be submitted one year or more prior to energization.

(6)	ERCOT shall only approve energization requests when the Transmission Element is satisfactorily modeled in the Network Operations Model.

(7)	Changes to an existing NOMCR that modify only Inter-Control Center Communications Protocol (ICCP) data object names shall be provided 15 days prior to the Network Operations Model load date.  NOMCR modifications containing only ICCP data object names shall not be subject to interim update reporting to the Independent Market Monitor (IMM) and Public Utility Commission of Texas (PUCT) (reference Section 3.10.4), according to the following:

| NOMCR that contains ICCP Data and is submitted … | ERCOT shall … | Subject to IMM & PUC Reporting |
| --- | --- | --- |
| Beyond 90 days of the energization date. | Allow modification of only ICCP data for an existing NOMCR | No |
| Between 90 and 15 days prior to the scheduled database load. | Allow modification of only ICCP data for an existing NOMCR | No |
| Less than 15 days before scheduled database load. | Require a new NOMCR to be submitted containing the ICCP data | Yes |