# Power Operations Bulletin #134

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes include new step six in section 2.2.2 “OC1 Limits for Outage Support.”

| ### 2.2.2 OC1 Limits for Outage Support |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | If step 1.2 is required, use the MIS and post that ERCOT is controlling to a limit other than the one posted. Typical Script: The “CSC Name” limit will change to “MW Amount” at “Time”. ERCOT will start controlling to this limit in accordance with Section 2.2.2 of the Transmission Security Desk procedures. |
| 1 | Adjust OC1 limits as required to support an outage. 1.1 If flow across a CSC is already below the limit required to support an outage, then it is permissible to wait and adjust the OC1 limit at the scheduled start time of the outage. 1.2 IF flow across a CSC is above the limit required to support an outage, then adjust the OC1 limits for intervals preceding the scheduled outage start time such that flow across the constraint is gradually reduced to at or below the limit required to support the specific outage. - While this action might result in over-constraining for a brief period, it is preferable to the negative effects of a sudden reduction in the OC1 limit. |
| 2 | When a CSC limit changes, immediately change the limit in RTMONI. Record this event in the Transmission Security log. |