# Power Operations Bulletin #136

CN#191 – South DC Tie

## Extending CN#160

## Effective Date:  January 17, 2005

## Expiration Date:  June 30, 2005

# Frequency Control Desk

On August 1, 2004 at 0001, ERCOT will begin the accounting process for MW that flow across the South HVDC tie.  There are 88 MWs of accumulated inadvertent on the South HVDC tie at this time.  This is to be paid to ERCOT from past testing.  ERCOT will start with a balance of 88 MWs in the “Carryover from Inadvertent Account” cell on the South DC Tie Inadvertent Accounting spreadsheet.

The procedure for tracking inadvertent on this tie is as follows:

- At midnight on July 31, 2004, ERCOT will call the AEP Corpus Christi TDSP and initiate the tracking of inadvertent on the South DC tie.

- At this time, no MWs will be scheduled on the South DC tie.  All the power that flows into or out of ERCOT will be treated as inadvertent.

- ERCOT will call the AEP Corpus Christi TDSP for actual hourly flows and check out for each hour of the day.  The hourly values will be entered in the “Actual Flow In” or the “Actual Flow Out” depending on the direction of flow.  The hourly flow will be monitored by ERCOT on the “Generation Tie Line Status” page in the POS.

- At midnight of each day the “Inadvertent on DC-S Tie” total will be compared with AEP Corpus Christi TDSP Operator.  After the total inadvertent for the day is agreed upon, the value in the “Inadvertent Account Total” will be carried over to the “Carryover from Inadvertent Account” cell in the “DC Tie Inadvertent Accounting” spreadsheet for the next day. (It is important that the sign convention of this value is carried from day to day, e.g.. If the number is a negative number it must be carried over as a negative, if it is a positive number then carry over as a positive.)

- ERCOT will at this time verify planned flows for the next day, on the South DC tie, with the AEP Corpus Christi TDSP Operator.