# ERCOT Operations Bulletin 137

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

This change is a temporary change and expires on June 30, 2005.

# CN # 193 Replace RRS Through a Bilateral Trade

## Expiration Date:  June 30, 2005

When a Resource (either LAAR or GEN) that is supplying RRS becomes unavailable to a QSE (QSE–A) and needs to be replaced through a Bilateral Trade with another QSE (QSE–B), then, QSE-A is expected to inform ERCOT.

- Obtain the following information from QSE-A:  Intervals affected and the name of the QSE with whom the bilateral trade was made.

- Log this conversation and the information provided and forward the information by email to Mark Henry (Compliance) and Mark Patterson (Market Operations Support).

- IF the resource that became unavailable was a LAAR, then:

- Ensure that QSE-B maintains an amount of Responsive Reserve Service equal to their own obligations plus the amount of the bilateral agreement with QSE-A for the affected intervals.