# ERCOT Operations Bulletin 140

ERCOT has revised the Day Ahead Desk section of the Operating Procedure Manual.

Changes include updating Section 2.8 step 3.

| ## 2.8 Review the Ancillary Services Market Clearing Data for the ERCOT System |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | On the “Ancillary Service Markets” – “Ancillary Service” tab: REVIEW the capacity value of each ancillary service procured by ERCOT for each hour of the operating day. |
| NOTE: | If the capacity of any ancillary service required by ERCOT for any hour exceeds the capacity available for that ancillary service for that hour, the values displayed for that service for that hour will be displayed in red on the “Ancillary Services” tab of the “Ancillary Service Markets” display, as an alarm indication. |
| 2 | Check for any significant capacity shortage for any hour for DRS, URS, RRS, and NSRS. A capacity shortage is significant if it cannot be accounted for by system rounding errors or known problems in ERCOT or QSE systems. |
| 3 | If a capacity shortage exists for any ancillary service for any hour, THEN CONTINUE; OTHERWISE, GO TO Step 4. PU 196 01/03/05 JEB CF REQUEST that the Transmission and Security Operator ISSUE a Market Insufficiency Alert including a statement that Ancillary Service bids were insufficient AND NOTIFY all positions. - POST the following message to the MIS: “A Market Insufficiency Alert has been issued due to insufficient <Ancillary Service> bids for <MM/DD/YYYY>, hours ending <HH>.” - POST the following message on the Emergency Notification System: “Alert- Insuf AS bids, see MIS” - IF additional capacity is arranged to resolve the market insufficiency and the additional capacity is scheduled, THEN REQUEST that the Transmission and Security Operator TERMINATE the Market Insufficiency Alert. OTHERWISE, CONTACT the Shift Supervisor for resolution. |

A copy of the updated procedure, Day Ahead Desk V3R17, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm