# ERCOT Operations Bulletin 143

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes include updating Section 2.6.3 step1, adding a third bullet to Step F.

| ### 2.6.3 Issue an Operating Condition Notice (OCN) |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | General guidelines: Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notification Messaging System. |
| 1 | ISSUE an Operating Condition Notice per applicable text in Appendix 5 IF: - There is a hurricane in the Gulf with >20% chance of landfall in Texas. - A cold front is approaching with temperatures anticipated to be in mid to low 20o F range and maximum temperature expected to remain near or below freezing impacting 50% or more of major metropolitan areas. - There is the potential for icing conditions to affect major transmission systems. - Hot weather expected to result in either capacity shortages or transmission security criteria violations. - Projected fuel limitations or emission limitations expected to lead to insecure operation. - A situation is emerging that has the reasonable expectation of leading ERCOT into an insecure state; including: - Forced outages which may lead to operating in an insecure state - Expectation of operating in a single contingency insecure state for an established list of contingencies - Insufficient Balancing Energy available to clear the next hour Balancing Energy requirement as shown on the MAI (if the load forecast is reasonable) |

A copy of the procedure, Frequency Control Desk V3R20, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.