# ERCOT Operations Bulletin #64

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual to reflect PRR 470.

The update is located in section 2.6.2 on page 26.

V2R10

| ### 2.6.2 Issue an Emergency Notice |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
|  | IF an immediate Short Supply condition has been identified that will result in unreliable conditions THEN REQUEST the Transmission/Security Desk issue a “Short Supply” Emergency Notice AND PROVIDE the condition causing the emergency, the intended ERCOT action and the timeframe for the action. |
| NOTE: | General guidelines for issuance of emergency notification Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notifications. |
| 1 | ISSUE an Emergency Notice IF - A transmission condition has been identified causing unreliable operation or overloaded elements. - A short supply, congestion condition is affecting voltage and/or frequency with risk of grid area failure(s) and no market alternatives exist to relieve the condition. - A severe single contingency event presents the threat of uncontrolled separation or cascading outages, large-scale service disruption to load, and/or overload of critical transmission elements and no timely solution is available from the market. - ERCOT has determined a fuel shortage exists |

V2R9

| ### Issue an Emergency Notice |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
|  | IF an immediate Short Supply condition has been identified that will result in unreliable conditions THEN REQUEST the Transmission/Security Desk issue a “Short Supply” Emergency Notice AND PROVIDE the condition causing the emergency, the intended ERCOT action and the timeframe for the action. |
| NOTE: | General guidelines for issuance of emergency notification Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notifications. |
| 1 | ISSUE an Emergency Notice IF - A transmission condition has been identified causing unreliable operation or overloaded elements. - A short supply, congestion condition is affecting voltage and/or frequency with risk of grid area failure(s) and no market alternatives exist to relieve the condition. - A severe single contingency event presents the threat of uncontrolled separation or cascading outages, large-scale service disruption to load, and/or overload of critical transmission elements and no timely solution is available from the market. |

A copy of the procedure, Frequency Control Desk V2R10, is attached.