# ERCOT Operations Bulletin #65

ERCOT has revised the Transmission & Security section of the Operating Procedure Manual.

The update is located in section 2.5.2 and 2.5.4, located on pages 20 and 23.

V2R16

| ### 2.5.2 Issue an Advisory |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | Send to TDSPs and QSEs for information purposes however ERCOT may increase ancillary service requirements or increase the Day Ahead market to two days ahead. Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notification Messaging System. |
| 1 | ISSUE an Advisory per applicable text in Appendix 4 IF: System Advisories: a. When ERCOT recognizes that conditions are developing or have changed and more Ancillary Services will be needed to maintain current or near-term operating reliability. b. When weather or ERCOT System conditions require more lead-time than the normal Day Ahead market allows. c. Transmission system conditions indicate ERCOT is likely to exceed established limits that would ensure reliability in the occurrence of a single event (ERCOT is likely to exceed first Contingency Criteria). (Appendix 1: First Contingency Criteria) d. There is a hurricane in the Gulf with expected landfall in Texas. e. A cold front is approaching with temperatures anticipated to be in mid to low 20o F range and maximum temperature expected to remain near or below freezing impacting 50% or more of major metropolitan areas. f. There is the probability for icing conditions to effect major transmission systems. g. A situation is emerging that has the reasonable expectation of leading ERCOT into an insecure state. h. Loss of communications or control condition is anticipated or significantly limited. i. If ERCOT is unable to comply with any of the market timing deadlines in the Day Ahead or Adjustment Period, or omits one or more procedures in the scheduling process. (Protocols 4.8) j. When notified by a Market Participant that fuel curtailment has occurred. |

V2R15

| ### 2.5.2 Issue an Advisory |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | Send to TDSPs and QSEs for information purposes however ERCOT may increase ancillary service requirements or increase the Day Ahead market to two days ahead. Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notification Messaging System. |
| 1 | ISSUE an Advisory per applicable text in Appendix 4 IF: System Advisories: a. When ERCOT recognizes that conditions are developing or have changed and more Ancillary Services will be needed to maintain current or near-term operating reliability. b. When weather or ERCOT System conditions require more lead-time than the normal Day Ahead market allows. c. Transmission system conditions indicate ERCOT is likely to exceed established limits that would ensure reliability in the occurrence of a single event (ERCOT is likely to exceed first Contingency Criteria). (Appendix 1: First Contingency Criteria) d. There is a hurricane in the Gulf with expected landfall in Texas. e. A cold front is approaching with temperatures anticipated to be in mid to low 20o F range and maximum temperature expected to remain near or below freezing impacting 50% or more of major metropolitan areas. f. There is the probability for icing conditions to effect major transmission systems. g. A situation is emerging that has the reasonable expectation of leading ERCOT into an insecure state. h. Loss of communications or control condition is anticipated or significantly limited. i. If ERCOT is unable to comply with any of the market timing deadlines in the Day Ahead or Adjustment Period, or omits one or more procedures in the scheduling process. (Protocols 4.8) |

V2R16

| ### 2.5.4 Issue an Emergency Notice |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | General guidelines for issuance of emergency notification. Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notification Messaging System. |
| 1 | ISSUE an Emergency Notice per applicable text in Appendix 4 IF: - ERCOT cannot maintain minimum reliability standards during the Operating Period using every Resource practicably obtainable from the market, - ERCOT is in an unreliable condition, as defined below, - Immediate action must be taken to avoid or relieve an overloaded transmission element, or - ERCOT varies from timing requirements or omits one or more scheduling procedures, as described in Section 4.8, Temporary Deviations from Scheduling Procedures. - ERCOT has determined a fuel shortage exists |

V2R15

| ### 2.5.4 Issue an Emergency Notice |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | General guidelines for issuance of emergency notification. Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notification Messaging System. |
| 1 | ISSUE an Emergency Notice per applicable text in Appendix 4 IF: 1. ERCOT cannot maintain minimum reliability standards during the Operating Period using every Resource practicably obtainable from the market, 2. ERCOT is in an unreliable condition, as defined below, 3. Immediate action must be taken to avoid or relieve an overloaded transmission element, or 4. ERCOT varies from timing requirements or omits one or more scheduling procedures, as described in Section 4.8, Temporary Deviations from Scheduling Procedures. |

A copy of the procedure, Transmission & Security Desk V2R16, is attached.