# ERCOT Operations Bulletin 154

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes include updating section 2.1.3 “Issue VDE”.

| ### 2.1.3 Issue Verbal Dispatch Exception (VDE) |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | A VDE shall be granted to a QSE for the intervals where the OOME instruction was not included in the MCE execution if the following conditions are met: - A QSE must request a VDE. - Accept a VDE request received before or during the first interval of the OOME instruction. - A VDE shall not include any interval prior to the requested interval. |
| 2 | Issue a VDE when a VDI has been issued if the following conditions are met: - A QSE must request a VDE. - Accept a VDE request received before or during the first interval of the VDI. - A VDE shall not include any interval prior to the requested interval. - If a VDI was used to issue an OOMC instruction, accept a VDE request received 30 minutes prior to the first Operating Period or during the Operating Period of the OOMC instruction. |
| 3 | A VDE shall be granted to a QSE where an OOMC was issued under the following conditions: - Accept a VDE request received 30 minutes prior to the first Operating Period or during the Operating Period of the OOMC instruction. - A QSE must request a VDE. - A VDE shall not include any interval prior to the requested interval. The VDE should include the time the OOMC was issued. |
| 4 | Document the Exception on the Verbal Dispatch Exception form. (See Appendix 4) |
| 5 | Ensure a copy of the VDE form, with appendices, is forwarded to Settlements and Market Operations. File the original in the control room. |