# Power Operations Bulletin #160

ERCOT has revised the Operating Period Desk section of the Operating Procedure Manual.

Changes include updating section 2.1.1 “Execute the HA Study”.

| ### 2.1.1 Execute the Hour-Ahead Study |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | An Hour Ahead Study is performed each hour to determine availability of resources to provide load balancing and control of CSC and OC congestion in the next operating hour. Determination is made as to whether NSRS and/or OOM resources need to be manually deployed if there are insufficient bids to resolve capacity insufficiency or congestion. |
| NOTE: | Don’t execute the Hour-Ahead Study for the first three minutes at the start of the interval. This is to avoid the time period that real-time balancing automatically executes. |
|  | EXECUTE CASE |
| 1 | EXECUTE a Case. Refer to APPENDIX 2: MAKING A SET-UP CASE, SAVE CASE, AND EXECUTING AN HOUR AHEAD STUDY |
| NOTE: | If study fails to execute, FOLLOW steps 3 through 6. |
| NOTE: | There is no point in attempting a study more than three times. If a third failure occurs, contact Shift Supervisor for engineering support. |
| 2 | If case is slow to complete, SELECT “MESSAGES” to verify process is running and REVIEW any relevant information or messages generated by the study process. (Select the messages tab on the studies page). |
|  | HALT CASE |
| 3 | SELECT “STOP.” |
| 4 | MAKE any corrections needed. Select SFT as appropriate. Ensure the start and end times are correct. |
| 5 | CONTINUE the study by selecting “EXECUTE.” When the study indicates it has finished processing, “complete” will appear on the Studies display. |
|  | ARCHIVE |
| 6 | CLICK on “ARCHIVE.” |

A copy of the procedure, Operating Period Desk V2R9, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.