# ERCOT Operations Bulletin #76

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

The following updates were made on this revision:  Section 2.3.2 combined steps 1 and 2; added guidance for creating an estimated end time for a forced outage; added the PRR425 allowance for the ERCOT operator to negotiate start times for Level 1 Maintenance Outages; Section 2.3.4 new section from PRR 425; Section 2.3.5 new section from PRR 425; changed all references to “Level 1 Outages” to “Level 1 Maintenance Outages.”

V2R21

| ### 2.3.2 Manage Forced & Level 1 Maintenance Outages |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | General Requirements: - VERIFY information describing the Forced or Level 1 Maintenance Outage on the outage form, contains the cause of the outage, and is understandable. - NOTIFY the Transmission Operator or QSE if the Forced or Level 1 Maintenance Outage does not appear to comply with the Protocols. Note this notification on the outage form. |
| 1 | After receiving verbal information from the Transmission Operator or QSE on Forced & Level 1 Maintenance Outages, ENTER the Forced or Level 1 Maintenance Outage into the Outage Scheduler. - If the Transmission Operator or QSE will not provide an estimated end date/time, then enter the end date/time to be 24 hours after the start time. - In the case of Level 1 Maintenance Outages of Transmission Facilities or Resources, coordinate the start of the outage if necessary to ensure system reliability as long as the outage is allowed to start within 24 hours. |

| ### 2.3.4 Unavoidable Extensions of Planned or Maintenance Outages |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| Definition | A Planned or Maintenance Outage is not completed with the ERCOT timeframe and the Transmission Facility or Resource cannot be restored at the outage completion date. |
| 1 | When notified (either verbally or when the Forced Outage [Unavoidable Extension] Form is received), then refer to Section 2.3.2 of this procedure and handle the same as any Forced Outage. |

| ### 2.3.5 Opportunity Outages (Resources) |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| Definition | A forced outage occurs on a Resource that has an approved or accepted outage scheduled within the following eight (8) days. The resource may remain off-line and start the accepted/approved outage earlier than scheduled. |
| 1 | When notified by the QSE of this situation, then refer to Section 2.3.2 of this procedure and handle the same as any Forced Outage. |

A copy of the procedure, Transmission & Security Desk V2R21, is attached.