# ERCOT Operations Bulletin #79

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes to this procedure update include:  Added step four to current section 2.1.2 Reporting Potential IROL violations and added new step 2.2.5 Managing Line Rating Changes..

V2R22

### Procedure for Control of Transmission System Loading Within N-1 Thermal Contingency Criteria Using Real Time Contingency Analysis

- Any RTCA results that predict a Facility Rating to exceed, post-contingency, 125% or greater of the facility’s Emergency Rating for more than 30 minutes is considered to be a potential IROL violation. Report such results via e-mail to the on-call Operations Support Engineer for assessment and reporting.

| ### 2.2.5 Managing Line Rating Changes |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE | If the rating received is a dynamic rating, the Transmission Operator must send it by e-mail or fax. |
| 1 | When notified by a Transmission Operator of a line rating that is different from the ratings currently in RTNET, obtain the following information from the Transmission Operator: - Equipment ID Name - MVA ratings (Continuous, Emergency and 15 Minutes) - Name of Operator on duty - Name of Transmission Operator - Name of Transmission Operator’s authorizing Engineer - Authorizing Engineer’s contact phone number. |
| 2 | Provide this information to the Shift Supervisor. |
| 3 | Enter this information in the daily log, using the “Line Rating Changes” log type. |
| 4 | Continue to operate according to RTNET ratings. |