# ERCOT Operations Bulletin 165

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes include updating section 2.5.2, step 1, adding step j.

| ### 2.5.2 Issue an Advisory |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | ISSUE an Advisory per applicable text in Appendix 4 IF: System Advisories: - When ERCOT recognizes that conditions are developing or have changed and more Ancillary Services will be needed to maintain current or near-term operating reliability. - When weather or ERCOT System conditions require more lead-time than the normal Day Ahead market allows. - Transmission system conditions indicate ERCOT is likely to exceed established limits that would ensure reliability in the occurrence of a single event (ERCOT is likely to exceed first Contingency Criteria). (Appendix 1: First Contingency Criteria) - There is a hurricane in the Gulf with expected landfall in Texas. - A cold front is approaching with temperatures anticipated to be in mid to low 20oF range and maximum temperature expected to remain near or below freezing impacting 50% or more of major metropolitan areas. - There is the probability for icing conditions to affect major transmission systems. - A situation is emerging that has the reasonable expectation of leading ERCOT into an insecure state. - Loss of communications or control condition is anticipated or significantly limited. - If ERCOT is unable to comply with any of the market timing deadlines in the Day Ahead or Adjustment Period, or omits one or more procedures in the scheduling process. (Protocols 4.8) - When long-range notice (approximately 48 hours) is needed to ensure the secure operation of the ERCOT system, alert the Market Participants to a potential threat to reliable operations with the following: - Lead time for dual fuel generating units to prepare for fuel supply problems in the event of anticipated severe cold weather - Any other operational concerns |

A copy of the updated procedure, Transmission & Security Desk V3R39, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.