# ERCOT Operations Bulletin 166

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes include updating section 2.8.1.

| ### 2.8.1 Approve NERC Tags for DC Ties |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| OVERVIEW: | A NERC Tag received in OATI regarding DC Tie flows into or out of ERCOT must be reviewed for correctness, and should be in agreement with the Market Operating System (MOS) except for changes made during the Operating Period such as curtailments. |
| 1 | When a new or updated tag is received in the OATI system, compare the tag data to the schedule in the MOS. - If the data doesn’t match and the reason for a tag IS NOT a curtailment, the operator may deny the tag. - If the data does match, then continue. |
| NOTE: | Data from the OATI tag may be entered into the Operations Spreadsheet at this time for purposes of comparison. |
| NOTE: | While the same entity is usually expected to submit all data and tags, a temporary exception has been made for TENASKA, AEP, and OMPA concerning which one submits specific documentation regarding the North DC Tie. This means names of data submitters may not match in these cases. |
| NOTE: | Prior to the start of the Operating Period, tag totals should match the schedules in the MOS. |
| 2 | APPROVE the tag by clicking on “APPROVE.” - The tag status should change to “Implemented” after all entities have approved - If NOT, then check if the status is either, “Denied” or “Withdrawn” or if the tag has gone “dead.” IF this is the case, then no action is required until the status changes to “Implemented” - If YES, then continue. |
| NOTE: | If no action is taken and all other approving entities have approved the tag, at the expiration of the time limit its status will default to “Implemented”. This is, in effect, an approval. If more than two approving entities take no action on a tag, the tag will “go dead”. |
| 3 | Print the tag |
| 4 | Ensure the Operations Spreadsheet data matches the schedule data on the NERC Tag, either by verifying previously entered data against the NERC Tag, or by entering the Tag data into Operations Spreadsheet. |
| 5 | File the Tag and any attachments in the assigned folder. |

A copy of the updated procedure, Frequency Control Desk V3R25, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.