# Power Operations Bulletin #82

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes to this procedure update include modifying section 2.9 to include handling Inadvertent Payback.

| ## Managing and Documenting Daily Inadvertent and Payback |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| OVERVIEW: | Inadvertent Energy flows across the DC Ties need to be monitored frequently enough during the day to ensure that large Inadvertent flows don’t accumulate |
| NOTE: | When the data from the NERC tag or Schedule Profile Fax is entered, the “Total Scheduled” flow value on the Operations Spreadsheet is automatically updated. |
| NOTE: | All references to Inadvertent Payback are limited to payback for 2001 through 2003 Inadvertent Energy, which will always be received by ERCOT. Initially, Inadvertent will only be paid back on the East Tie using a 20 MW Bias. Payback will only be scheduled when there is room on the tie. Power schedules of market participants will supercede any inadvertent payback schedule. |
| 1 | Contact the AEP DC Tie Operator and develop a schedule for inadvertent payback for the next day. - Ignore on-peak or off-peak considerations - Limit payback to 20 MW per hour |
| 2 | In the “DC Ties Totals” page of the Operations Spreadsheet, enter the hourly amounts of planned inadvertent payback in the “TO ERCOT” column. - The spreadsheet will adjust the “Total Scheduled” column depending on which direction the tie is flowing. |
| 3 | Periodically, call AEP and obtain values for the hourly accumulated totals (also known as “Actual Flows”) and verify the payback schedule. |
| 4 | Enter the data into the Operations Spreadsheet. |
| 5 | Prior to 2400 confirm the next day’s DC Tie schedules. |
| 6 | During the last update call to AEP for the day: - Determine if the daily “Total Actual Flow” values on the spreadsheet agree with the AEP total. If, not, then correct the spreadsheet as necessary. - Verify the values in the “Inadvertent on DC Tie” and “Inadvertent Payback” columns. Correct as necessary. |
| 7 | Transfer the current day’s “Inadvertent Account Total” to the “Carryover From Inadvertent Account” cell in the next day’s spreadsheet. |
| 8 | Print the completed Operations Spreadsheet. - Provide a copy to the Shift Supervisor. - File the Operations Spreadsheet in the Monthly DC Tie folder. |

A copy of the procedure, Frequency Control Desk V2R13, is attached.