# Power Operations Bulletin #83

ERCOT has revised the Day Ahead Desk section of the Operating Procedure Manual.

Changes to this procedure update include updating Appendix 4.

###### 3.5	Appendix 4:  Methodology for Determining Non-Spin Procurement

For hours of the operating day that the following conditions are met, NSRS requirements = MW value of the largest unit on-line, for all other hours, NSRS requirements = 0 MW.

- During the months of April, May or October for hours that the forecasted load will be within 85% of forecasted peak load for the operating day:

- For hours with forecast temperatures greater than 92 degrees:

- For hours with forecast temperatures less than 30 degrees

- For hours in which current day Operations Plan indicates less than 2300 MWs of spinning reserve will be on-line:

A copy of the procedure, Day Ahead Desk V2R14, is attached.