# Power Operations Bulletin #87

ERCOT has revised the Transmission & Security section of the Operating Procedure Manual.

Changes include putting section 2.2.1 back into procedure, and updated section 2.2.2.

| ### 2.2.1 Monitoring Outages |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | Review Daily Outage notes and Engineering Support notes to determine which outages need monitoring. |
| 2 | Verify each requirement has been met. |
| 3 | Mark the required outages in the Outage Scheduler Real-Time Monitor. |
| 4 | Verify that the outage is being monitored by change in color to red. |
| 5 | Monitor outages as required. |

| CN 123 5/18/2004 KG DP 2.2.2 OC1 Limits for Outage Support |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | Adjust OC1 limits as required to support an outage. 2.1 If flow across a CSC is already below the limit required to support an outage, then it is permissible to wait and adjust the OC1 limit at the scheduled start time of the outage. 2.2 IF flow across a CSC is above the limit required to support an outage, then adjust the OC1 limits for intervals preceding the scheduled outage start time such that flow across the constraint is gradually reduced to at or below the limit required to support the specific outage. - While this action might result in over-constraining for a brief period, it is preferable to the negative effects of a sudden reduction in the OC1 limit. |

A copy of the procedure, Transmission & Security V2R23, is attached.