# Power Operations Bulletin #90

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes include inserting new section 2.5.5 Notification of Condition Termination.

| ### 2.5.5 Notification of Condition Termination |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE | CN #124 5/18/2004 JEB DP If a condition increases in severity, then the posting of the corresponding Alert, Advisory, or Emergency Notice will constitute cancellation of the previous notification. |
| 1 | When the situation requiring the notification is relieved, NOTIFY Market Participants and Transmission Operators that the condition has been terminated. OCNs do not require “termination” notification. - USE the Emergency Notification System AND ISSUE Termination Notice. (Short version – 32 characters) - POST the long version of the Termination Notice on the Market Information System (MIS). - Initiate a hotline call to all Transmission Operators and QSEs (or see that another operator does so). - Make a printout of the hotline log and save it in the daily log file. |

A copy of the procedure, Transmission & Security Desk V2R23, is attached.