# ERCOT Operations Bulletin 175

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes include updating section 2.1.2 “Verbal Dispatch Instructions.”

| ### 2.1.2 Verbal Dispatch Instructions (VDI) |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | A VDI number should be included on all VDIs. The number will indicate the desk by use of the following: Day Ahead use – D Transmission and Security use – T Frequency Control use – F Operating Period use – OP Shift Supervisor use – S EXAMPLE: For a 4th VDI issued by the Frequency Control Desk on July 22, 2002 the number would appear as follows: F-072202-4. * As time permits, complete the following: - The VDI form - Communicate all required information to the respective recipient. - Log all relevant information regarding the VDI in the Frequency Log. *This flexibility gives the system operator the latitude to dispatch generation quickly in order to facilitate ERCOT’s response to a potential insecure state. ISSUE a VDI to a QSE specifying changes to ERCOT generation. VDI may be initiated as indicated by other steps in this, or other, ERCOT Operating Procedures. |
| 1 | HOLD LAST INTERVAL IF market control mechanisms have failed, or ERCOT has issued erroneous instructions THEN, using the Hotline, issue an instruction to: Typical instruction: “Hold the [interval ending time] interval Balancing Energy Service deployment until further notice.” Complete the VDI form (Appendix 3) and process. - Issued a Verbal Dispatch Instruction to – write in “All QSEs” - The Instruction was given due to the following reasons: “Hold interval ending <time>, resume with interval ending <time>.” All fields are to have either pertinent data or N/A entered. Leave no field(s) blank. Print Hotline Log and enter date and time on printout. |
| NOTE: | When issuing a VDI, all information on the VDI form must be communicated to the recipient or ERCOT will be in violation of Protocol Section 5.4.2. |
| NOTE: | VDI’s should not re-iterate an instruction already given via XML. Such a communication is not a VDI but simply verification. If a VDI form is filled out which simply re-iterates an instruction already given via the XML, note this on the VDI form in the space titled “The Instruction was as follows.” |
| 2 | The following are required fields on all VDIs: - VDI # - Operating Day - ERCOT System Operator - Receiving Entity - Receiving Entity’s Operator - Time & Date - Instruction - Reason for Instructions - Intervals - ERCOT Operator’s signature Blank fields are to have N/A entered. |
| 3 | As time permits, forward the VDI to the Shift Supervisor. |
| 4 | Log the VDI information and information relevant to the decision for issuing the VDI in the Frequency Log. |