# ERCOT Operations Bulletin 184

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes include incorporating new section 2.3.7 “Guidelines for Withdrawal of an Outage.”

| ### 2.3.7 Guidelines for Withdrawal of an Outage |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | This procedure applies to any Forced or Maintenance Outage. The system operator may WITHDRAW any: - approved or ACTIVE outage IF the security analysis and operator experience, or operator experience alone indicates the outage may have an adverse impact on system reliability. |
| 1 | IF an outage is withdrawn, ENTER an explanation in the “Reviewers Notes” listing the reliability concerns that caused the withdrawal. As soon as possible, NOTIFY by e-mail: - 1 ERCOT Outage Coordination - 1 ERCOT OPS SUPPORT ENGINEERING Include in the e-mail: - From Station - To Stations (if applicable), - The Planned Start and Planned End Date of the outage |
| 2 | IF a TO or QSE reports that the outage in question has progressed to a point that withdrawal is not practicable within the outages designated restoration time, the System Operator shall: - Record this in the “Outage Scheduler Review’s Notes” - Log it in the Transmission Security Log. - INFORM the Shift Supervisor and continue monitoring system security. |

A copy of the updated procedure, Transmission & Security Desk V3R48, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.