# Power Operations Bulletin #95

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Updated section 2.1.2 “ERCOT Procedure to Allow Unit Testing When a Unit is Involved in Managing Congestion

V2R8 – CN#132

| ### ERCOT Procedure to Allow Unit Testing When a Unit is Involved in Managing Congestion |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | ERCOT will make every reasonable effort to accommodate this testing. |
| 1 | In the event that a facility requires a specific unit output level for required testing, equipment testing, ERCOT required capability testing, and other mandatory testing; ERCOT shall make every reasonable effort to accommodate this testing, including constraining other resources prior to requesting the testing unit to halt their test. |
| NOTE: | The specific protocol allowing such treatment is section 5.4.3: - All Dispatch Instructions to Resources – whether for dispatch of Ancillary Services, System Emergencies or any other reason – shall be directed to the QSE responsible for the affected Resource. ERCOT shall refrain from issuing dispatch instructions to generation units and other Resources undergoing testing with the exception of: - Dispatch instructions that are a part of the testing; or - During conditions when the generating unit is the only alternative for solving a transmission constraint; or - During Force Majeure Events that threaten the reliability of the system. |
|  | Accommodation shall include: - Removing the unit from the MCE by placing it in the Test mode in the MOI. - OOME other units to control local congestion in place of the testing unit. |
| NOTE: | This includes NDC on wind generation. |
| 2 | If ERCOT is incapable of controlling congestion without deployment of the testing unit; ERCOT will direct that the test be canceled and the unit deployed to control congestion. |
| 3 | QSEs or resources desiring such accommodation shall request the special treatment via e-mail to shiftsupv@ercot.com or fax to (512) 248-6858, using the form/affidavit as shown in “Attachment A: Request for Unit Testing,” (located in the Appendices). |
| 4 | ERCOT will reply with one of the following responses (either step 5 or step 6). |
| 5 | If ERCOT believes that it can accommodate the request; it will respond via EMAIL or FAX as follows: ERCOT recognizes the need of ___________ to test its unit on _________ and (company name) (date/time) intends to make system adjustments to accommodate this testing to the extent possible consistent with system security. This intent is NOT a guarantee of accommodation. __________ ______ (Approving supervisor) __________________ Date/time |
| 6 | If ERCOT believes that the request cannot be accommodated for one of the following reasons it will refuse with the script below: - No combination of other units can maintain transmission security - The request exceeds 14 days - A pattern of repeated requests for the same unit(s) indicate to ERCOT that abuse of the testing privilege may be taking place. If ERCOT refuses any request for this reason it will notify ROS and the Market Oversight Section of the Public Utility Commission. ERCOT cannot accommodate the request of ____________________ for a unit test on (company) the dates __________________ for the following reason: Where the reason is one of the three listed below. - No combination of other units can maintain transmission security - The request exceeds 14 days. - A pattern of repeated request for the same unit(s) indicate(s) to ERCOT that abuse of the testing privilege may be taking place. |
| 7 | One of these two responses shall be emailed or faxed to the requesting party. |
| 8 | Direct the Transmission and Security Desk Operator to enter the information regarding the testing of this resource into the MOI. |