# ERCOT Operations Bulletin 193

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Changes include incorporating new section 2.2.13 “Procurement of Additional RRS.”

| ### Procurement of Additional RRS |  |
| --- | --- |
|  | ###### Procedural Steps |
| NOTE: | The order of steps in this procedure is a recommendation. The system operator will determine the sequence of steps, or any additional actions necessary to ensure system security. |
| 1 | If Responsive Reserve Service (RRS) is less than 2300 but greater than 2200, ISSUE an ALERT. Typical Script: “Responsive Reserve Service is less than 2300 MWs which is required by ERCOT. The current Responsive Reserve Service is at < MWs >. Should Responsive Reserve Service continue to become depleted, ERCOT will Implement EECP Step 1 and open a second market to procure the deficient amount of Responsive Reserve Service.” |
| 2 | If RRS reaches 2200, IMPLEMENT EECP Step 1. |
| 3 | DIRECT the Day Ahead operator to open a second market and procure sufficient RRS to restore the 2300 MW requirement. |

A copy of the updated procedure, Shift Supervisor Desk V3R16, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.