# Power Operations Bulletin #102

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Changes include adding information after second note in Section 2.1.2.

| ### ERCOT Procedure to Allow Unit Testing |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | ERCOT will make every reasonable effort to accommodate testing. |
| 1 | In the event that a facility requires a specific unit output level for required testing, equipment testing, ERCOT required capability testing, and other mandatory testing; ERCOT shall make every reasonable effort to accommodate this testing, including constraining other resources prior to requesting the testing unit to halt their test. |
| NOTE: | The specific protocol allowing such treatment is section 5.4.3: - All Dispatch Instructions to Resources – whether for dispatch of Ancillary Services, System Emergencies or any other reason – shall be directed to the QSE responsible for the affected Resource. ERCOT shall refrain from issuing dispatch instructions to generation units and other Resources undergoing testing with the exception of: - Dispatch instructions that are a part of the testing; or - During conditions when the generating unit is the only alternative for solving a transmission constraint; or - During Force Majeure Events that threaten the reliability of the system. |
|  | Accommodation shall include: - Removing the unit from the MCE by placing it in the Test mode in the MOI. - The following reasons are acceptable to place a unit in Test mode. - New unit startup - Post Outage startup (Forced or Planned) - Seasonal testing as required by ERCOT (section 6.10 of the Protocols). This includes Generation Capacity Testing requirements and Ancillary qualifications as well. - OOME other units to control local congestion in place of the testing unit. |

A copy of the procedure, Shift Supervisor V2R8 – CN#140, is attached.