# ERCOT Operations Bulletin 30

Expected QSE response when ERCOT issues a “Hold Previous Interval” dispatch instruction.

On occasion it becomes necessary for ERCOT to issue a verbal instruction to QSEs to have them hold the previous interval’s deployments. This can happen when the balancing instructions were either not generated or not sent for the upcoming operating interval. The expected QSE response to this instruction is as follows:

- Follow your base power schedule.

- Hold any portfolio balancing, unit specific and OOME deployments from the specified interval and add these to your base schedule for the upcoming operating interval.

- Continue to deploy Regulation, Responsive and Non-Spin as instructed.

The net result is that the QSE will follow it’s schedule, any Regulation, Responsive and Non-Spin deployments as usual and use the Balancing, Unit Specific and OOME instructions from the previous interval in determining where to operate their units for the upcoming operating interval.