# ERCOT Operations Bulletin 201

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Changes include updating section 2.1.2 “ERCOT Procedure for Unit Testing.”

| - ERCOT Procedure for Unit Testing |  |
| --- | --- |
| - Step # | - Procedural Steps |
| NOTE: | ERCOT will make every reasonable effort to accommodate testing, including constraining other resources prior to requesting the testing unit to halt their test. The specific protocol allowing such treatment is section 5.4.3: - All Dispatch Instructions to Resources – whether for dispatch of Ancillary Services, System Emergencies or any other reason – shall be directed to the QSE responsible for the affected Resource. ERCOT shall refrain from issuing dispatch instructions to generation units and other Resources undergoing testing with the exception of: - Dispatch instructions that are a part of the testing; or - During conditions when the generating unit is the only alternative for solving a transmission constraint; or During Force Majeure Events that threaten the reliability of the system. This procedure includes NDC on wind generation. |
| 1 | QSEs desiring test accommodation shall make the request via e-mail to shiftsupv@ercot.com or fax to (512) 248-6858, using the form/affidavit as shown in “Attachment A: Request for Unit Testing,” (located in the Appendices). QSEs may also make testing arrangement through Operations Planning Engineer. |
| 2 | The Shift Supervisor will approve and monitor all testing. |
| 3 | Accommodation shall include: - Removing the unit from the MCE by placing it in the Test mode in the MOI. - The following reasons are acceptable to place a unit in Test mode. i. New unit startup ii. Post Outage startup (Forced or Planned) iii. Seasonal testing as required by ERCOT (section 6.10 of the Protocols). This includes Generation Capacity Testing requirements and Ancillary qualifications as well. OOME other units to control local congestion in place of the testing unit. |
| 4 | If ERCOT is incapable of maintaining system security without deployment of the testing unit; ERCOT will direct that the test be canceled and the unit deployed as needed. |
| 5 | ERCOT will reply with one of the following responses (either step 6 or step 7). |
| 6 | If ERCOT believes that it can accommodate the request; it will respond via EMAIL or FAX as follows: ERCOT recognizes the need of ___________ to test its unit on _________ and (company name) (date/time) intends to make system adjustments to accommodate this testing to the extent possible consistent with system security. This intent is NOT a guarantee of accommodation. __________ ______ (Approving supervisor) __________________ Date/time |
| 7 | If ERCOT believes that the request cannot be accommodated for one of the following reasons it will refuse with the script below: - No combination of other units can maintain transmission security - The request exceeds 14 days - A pattern of repeated requests for the same unit(s) indicate to ERCOT that abuse of the testing privilege may be taking place. If ERCOT refuses any request for this reason it will notify ROS and the Market Oversight Section of the Public Utility Commission. ERCOT cannot accommodate the request of ____________________ for a unit test on (company) the dates __________________ for the following reason: Where the reason is one of the three listed below. - No combination of other units can maintain transmission security - The request exceeds 14 days. A pattern of repeated request for the same unit(s) indicate(s) to ERCOT that abuse of the testing privilege may be taking place. |
| 8 | One of these two responses shall be emailed or faxed to the requesting party. |
| 9 | If the testing is approved direct the Transmission and Security/Frequency Desk Operator(s) to enter the information regarding the testing of this resource into the MOI. |

A copy of the updated procedure, Shift Supervisor Desk V3R17, is available at

http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.