# Power Operations Bulletin #106

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes include adding new section for Pre-Contingency Action Plans, now Section 2.2.4.

| ### 2.2.4 Pre-Contingency Action Plans |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| CAUTION | Pre-Contingency Action Plans are designed to be enacted before the contingency occurs. These actions may be taken to increase transmission transfer capability. |
| 1 | When RTCA shows a post-contingency overload and a Pre-contingency Action Plan exists to resolve it, - REVIEW the Pre-contingency Action Plan with the associated Transmission Operator. - Notify the Shift Supervisor of the anticipated actions. |
| 2 | Direct the execution of the pre-contingency action plan with the associated transmission operator. |
| 3 | If necessary, use congestion management methods for post contingency loading after initiating the Pre-contingency Action Plan. |

A copy of the procedure, Transmission & Security Desk V3R25, is attached.