# ERCOT Operations Bulletin 207

ERCOT has revised the Operating Period Desk section of the Operating Procedure Manual.

Changes include updating section 2.1.2 “OOME Respond to Hour-Ahead Study Results” – added note prior to step 5, updated step 5.

| ### 2.1.2 Respond To Hour-Ahead Study Results |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | Once the maximum procurement level has been reached, increasing the offset will not lead to additional balancing deployment. The maximum procurement level is reached once “Net Gen” and “Net Load” no longer match in the Real-Time Balancing Market window. Should Balancing Energy become depleted, over compensating with excessive offsets can potentially cause unexpected deployments when more balancing becomes available. |
| 5 | Verify UBES Remaining. - If >20% of available UBES remains undeployed, no further action is necessary. - If <20% of available UBES remains undeployed, verify that UBES is available in real-time before entering the offset. - If >5% of available UBES remains undeployed, no further action is necessary. - If <5% remaining proceed. |

A copy of the updated procedure, Operating Period Desk V2R11, is available at

http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.