# ERCOT Operations Bulletin 209

ERCOT has revised the Operating Period Desk section of the Operating Procedure Manual.

Changes include

V2R12		Updating section 2.1.1 “Execute the Hour Ahead Study” and updating 		Appendix 4

| NOTE: | Before executing the Hour-Ahead Study, verify the Balancing Energy Market has cleared for the top of the hour’s interval. (For example, before executing the Hour-Ahead Study at 0737, ensure the 0800 interval has cleared). Reference Displays: ERCOT MOI Real-Time Balancing Window If the market for the interval has cleared, the top line will state “Approved” |
| --- | --- |

## Appendix 4:  How to Deploy NSRS

DEPLOY NSRS

Reference Displays:  Non-Spin Deployment display in MOI.

NOTE:  NSRS MUST be deployed for at least one hour.

To open the Non-Spin window in the ERCOT Market Operator Interface (MOI) the operator may:

    - SELECT WINDOW on control bar at the top of the MOI and select the “Non-Spin Deployment” from the scroll down menu

OR

    - If non-spin window has already been opened, operator may maximize the window located inside the MOI

SELECT the appropriate date:

    - RIGHT CLICK in the DATE box.  POP-UP calendar will appear, LEFT CLICK the desired date and SELECT “OK” to accept date or CANCEL to close POP-UP box

OR

    - Use the up and down arrow keys to select desired date.

CLICK on the GREEN PLUS (+) on the tool bar at top of page.  This will add a blank row.

RIGHT CLICK on “Zone” field and select appropriate zone.

Appropriate zone is a zone where non-spin was deployed, and now has to be recalled.

SELECT desired START DATE, right click.

SELECT desired START TIME, right click.

SELECT desired END DATE, right click.

SELECT desired END TIME, right click.

END TIME for the recall MUST EQUAL the END TIME of the non-spin deployment in order to recall non-spin for all intervals.

ENTER 0 MW right click.

ENTER COMMENT:  e.g. “Recall NSRS”

PRESS F10 to save/recall NSRS.

V2R13		Updating section 2.1.2 “Respond to Hour Ahead Study Results”

| 7 | IF NSRS is scheduled and available, deploy hourly to resolve UBES deficiency. IF NSRS is unscheduled or unavailable, communicate with the Frequency Control Desk and/or Shift Supervisor. |
| --- | --- |
| 8 | Recall NSRS when the percent UBES used plus NSRS deployed </= 90% |
| NOTE: | Deployment of NSRS is based on the Hour Ahead Study along with a reasonable load forecast. Recall is based on real-time UBES availability. |

A copy of the procedure, Operating Period Desk V2R13, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.