# ERCOT Operations Bulletin 210

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes include incorporating new section 2.6.8 “Emergency Site Failover.”

| ### 2.6.8 Emergency Site Failover |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | This procedure addresses the loss of the primary control center. In the absence of the Shift Supervisor, the Frequency Operator shall enact this procedure. |
| NOTE: | If communication with the primary control center is lost during normal business hours and support is unavailable from the primary control center, personnel at the backup site will serve as support. If communication with the primary control center is lost during non business hours, on-call personnel should be notified. |
| 1 | If the primary control center becomes inoperative, determine if the EMMS infrastructure is running. - A & B indicate the EMMS is running from Taylor - C & D indicate the EMMS is running from Austin |
| 2 | Communicate the EMMS status to EMMS Production Support. |
| 3 | If you are unable to contact EMMS Production Support, contact Development. |
| 4 | IF frequency control is lost, initiate FCD Procedure 2.5.1 step 2 and go on Constant Frequency Control. |
| 5 | - Ensure the backup control center is staffed with the following personnel: - 1 Shift Supervisor - 2 Frequency Control - 2 Transmission Security - 1 Operating Period - 1 Day Ahead - Notify: - IT support - The Chief System Operator - Engineering Support - ERCOT Security - SPP ISO - If possible, post a message on the MIS - Place a hotline call to the QSEs advising them of ERCOT’s control center status. Typical Script: “This is the ERCOT system operator. ERCOT is in the process of transferring system operation from the primary control center to the backup control center due to <define the situation>. Further information will be forthcoming as necessary.” |
| 6 | Once the Shift Supervisor arrives at the backup site, he/she will coordinate the restoration of any MOS & POS systems that have been lost. |

A copy of the procedure is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.