# ERCOT Operations Bulletin 213

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

Changes include

V3R31		Update section 2.1.2, 2.1.5, 2.5.3, and Appendix 4

Adding new section 2.1.3 and 2.1.4

| ### 2.1.2 Verbal Dispatch Instructions (VDI) |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| PURPOSE: | ISSUE a VDI to QSE(s) specifying changes to generation. |
| NOTE: | VDIs should not re-iterate an instruction given via XML. |
| 1 | Communicate required information to QSE(s) via telephone (using the hotline if appropriate). |
| 2 | As time permits, complete the VDI form (Appendix 3). The following are required fields on all VDIs: - VDI # - EXAMPLE: For the 4th VDI issued by the Frequency Control Desk on July 22, 2005 the number would appear as “F-072205-4” - Operating Day - ERCOT System Operator - Receiving Entity - Receiving Entity’s Operator - Time & Date - Instruction - Reason for Instructions - Intervals - ERCOT Operator’s signature Blank fields are to have N/A entered. |
| 3 | As time permits, forward the VDI form to the Shift Supervisor. |
| 4 | LOG in the Operator’s Log the VDI information and information relevant to the decision for issuing the VDI. |

| ### 2.1.5 Replacing Responsive Reserve through a Bilateral Trade |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | When a resource (either LAAR or generation) that is supplying RRS becomes unavailable to a QSE (QSE A) and needs to be replaced through a Bilateral Trade with another QSE (QSE B), then QSE A is expected to inform ERCOT. |
| 1 | Obtain the following information from QSE A: - Intervals affected - Name of QSE with whom the bilateral trade was made |
| 2 | IF the resource that became unavailable was a LAAR, then ENSURE QSE B maintains an amount of Responsive Reserve Service equal to their own obligations PLUS the amount of the bilateral agreement with QSE A for the affected intervals. |
| 3 | LOG in the Operator’s Log the conversation, including the information provided. |
| 4 | Email the information RRS@Ercot.com. |

| ### 2.5.3 Actions When Frequency Telemetry is Incorrect |  |
| --- | --- |
| ###### PURPOSE: | ###### To prevent the unnecessary deployment of Responsive Reserve due to incorrect telemetry frequency data. |
| 1 | IF frequency is BELIEVED to be incorrect VERIFY that the frequency is correct by comparing the data from multiple sources. - Go to the “Generation Area Status” display, and click on the “i” to the left of “Current Frequency” to view “OPA Measurement” window. |
| 2 | In “OPA Measurement” window, under “Status” column, check “Prime” on a valid frequency source. |
| 3 | LOG in the Operator’s Log any relevant information on this event. |

## Appendix 4:  Telephone Hotline

| QSEs |  |
| --- | --- |
| AEN | FULCRUM |
| AEP Ohio | Garland |
| ANP | LCRA |
| APX Equinix | Merrill Lynch |
| BEPC (Brazos) | MIRANT |
| British Petro | MPower |
| BTU | Reliant RES |
| Calpine | Sempra |
| Constellation | STEC |
| Coral | SUEZ |
| CPSB | Tenaska |
| Dynegy | TXGNQSER (Texas Genco) |
| Exelon | TXU |
| FP&L Juno Beach | XTend |

| TOs |  |
| --- | --- |
| AEN | Denton |
| AEP | Garland |
| BEPC | LCRA |
| BPUB | Magic Valley |
| BTU | Rayburn |
| Centerpoint | STEC |
| College Station | TNMP |
| CPSB | TXU Electric Delivery |

| ### 2.1.3 VDI for Hold Last Interval |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | If market control mechanisms have failed, or ERCOT has issued erroneous instructions THEN, using the hotline, issue an instruction: Typical script: “VDI # ______. Hold the [interval ending time] interval Balancing Energy Service deployment until further notice.” |
| 2 | Print Hotline Log, ensure printout has date and time and forward to Shift Supervisor. |
| 3 | As time permits complete the VDI form (Appendix 3), using these instructions: - Issued a Verbal Dispatch Instruction to – write in “All QSEs” - Receiving entity’s operator – write in “All QSEs” - The instruction was as follows – write in “Hold interval ending <time>, resume with interval <time> ALL fields are to have either pertinent data or N/A entered. Leave NO field(s) blank. |
| 4 | As time permits, forward VDI form to Shift Supervisor. |
| 5 | LOG in the Operator’s Log the VDI information and reason(s) why interval was held. |

| ### 2.1.4 VDI for RMR Unit Availability Change |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| PURPOSE: | If an RMR unit availability changes, the QSE is NOT obligated to replace the energy deficiency of that RMR unit. If requested by the QSE a VDI shall be issued for the amount of the MW mismatch and will be valid for the period of time that the QSE cannot update their schedule. |
| 1 | If an RMR unit availability changes AND the QSE requests a VDI: - Issue the VDI for the amount of MW mismatch - Issue the VDI for the period of time that the QSE cannot update its schedule - Have the QSE submit an updated availability plan - Have the QSE submit an update schedule |
| 2 | As time permits, complete the VDI form (Appendix 3). |
| 3 | As time permits, forward the VDI form to the Shift Supervisor. |
| 4 | LOG in the Operator’s Log the VDI information and any other pertinent information relating to this event. |

V3R32		Update section 2.1.2 to reflect Release 4 changes – VDI Numbering

| ### Verbal Dispatch Instructions (VDI) |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| PURPOSE: | ISSUE a VDI to QSE(s) specifying changes to generation. |
| NOTE: | VDIs should not re-iterate an instruction given via XML. |
| 1 | Communicate required information to QSE(s) via telephone (using the hotline if appropriate). |
| NOTE: | VDI numbers as described here are only required for VDIs documented using the paper form. VDIs documented using the OOM tool (post EMMS Release 4) will not have a number available to the operator. |

A copy of the procedure, Frequency Control Desk V3R32, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.