# REMOVED FROM OPERATING PROCEDURES 10/11/05

# ERCOT Operations Bulletin 216

CN#280 – Procedure to Cancel OOMEs

## Effective Date:  October 1, 2005

## Expiration Date:  December 15, 2005

Extending CN#222

# Transmission & Security Desk

Purpose

To withdraw an OOME that has been not yet been deployed by SPD.

Procedure

When an OOME needs to be canceled, place the unit in “Test” for the periods you wish to cancel.

Request the QSE adjust the Min and Max Limits on other units to accommodate the loss of balancing capacity for the unit in “Test”.

Notes

When a unit is placed in “Test”, the Mw value will automatically populate with the value from the Resource Plan.

You can make modifications to any future OOMEs that are in “Test” or to units that have OOME deployments scheduled as long as the SPD has not run for the interval in question.

A unit in “Test” is restricted from participating in the Balancing Energy market.  If a QSE is counting on the unit that has been placed in “Test” to provide a portion of its Balancing Energy bid,

the QSE will have to adjust the Min and Max Caps on other units in its zonal portfolio so that the amount bid will match the amount available as shown by the High and Low Operating limits. In other words, the QSE will have to make room on other units, where feasible, because SPD will calculate the correct amount of Balancing Energy but the Min and Max Caps set the boundaries of the Balancing Energy amounts awarded.

With the EMMS Release 3 implementation, an OOME deployment is processed through the Balancing Energy market clearing in SPD.  When the OOME deployment is processed, SPD performs the following actions:

- Calculates the amount that the unit is instructed to deviate from its planned level (instructed deviation).

- Calculates the QSE’s available capacity for Balancing Energy using non-OOMEd units in the portfolio.  Balancing Energy is not allocated to OOMEd units.

- Includes the instructed deviation in the QSE’s portfolio deployment.

- Purchases Balancing Energy to cover the instructed deviation.