# Power Operations Bulletin #115

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

Changes include:  	Updated section 2.2.4 steps 4 and 5

Updated section 2.2.5 steps 5 and 6

Updated section 2.2.6 step 5

Added two bullets in first Note section 2.5.3

| 2.2.4 Pre-Contingency Action Plans |  |
| --- | --- |
| 4 | Between 0000 and 0300 each day, the Transmission & Security Desk Operator should review the logs and verify that no Pre-Contingency Action Plans remain in effect from the preceding day. IF any Pre-Contingency Action Plans are still in use, evaluate system conditions to determine if: - The Pre-Contingency Action Plan should remain in effect, OR - The system Action Plan should be returned to a normal status. |
| 5 | IF the ERCOT System Operator and Shift Supervisor agree that the Pre-Contingency Action Plan is no longer needed, the Transmission & Security Desk Operator shall direct the TO to: - Return the system to its’ normal status, OR - Return the system to its’ status prior to implementation of the Pre-Contingency Action Plan Which ever state is applicable. |

| ### 2.2.5 Remedial Action Plans |  |
| --- | --- |
| 5 | Between 0000 and 0300, the Transmission & Security Desk Operator should review the logs and verify that no RAPs remain in effect from the preceding day. IF any RAPs are still in use, evaluate system conditions to determine if: - The RAPs should remain in effect, OR - The system Action plan should be returned to a normal status. |
| 6 | IF the ERCOT System Operator and Shift Supervisor agree that the RAP is no longer needed, the Transmission and Security Desk Operator shall direct the TO to: - Return the system to its’ normal status, OR - Return the system to its’ status prior to implementation of the RAP Which ever state is applicable. |

| ### 2.2.6 Mitigation Plans |  |
| --- | --- |
| 5 | Between 0000 and 0300, the Transmission & Security Desk Operator should review the logs and verify that no MPs remain in effect from the preceding day. IF any MPs are still in use, evaluate system conditions to determine if: - The MPs should remain in effect, OR - The system Action Plan should be returned to a normal status. |

### 2.5.3 Issue an Alert

###### Procedural Steps

General guidelines: Knowledge of an emerging/existing situation that could/does require intervention to ensure system reliability requires ERCOT to notify Market Participants through the use of Emergency Notification Messaging System. - Include the geographical location of the existing overload when notifying Market Participants and TOs. - Before issuing a Transmission Alert, notify the TO and discuss the actions you are about to take.

A copy of the procedure, Transmission & Security Desk V3R27, is located at

http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.