# ERCOT Operations Bulletin 221

ERCOT has revised the Shift Supervisor Desk section of the Operating Procedure Manual.

Changes include removing step prior to Step 1 in section 2.4.1.

| ### Monitor QSEs Which Under Perform in Supplying Ancillary Services |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | If a VDI is issued to a QSE to relieve them from the Ancillary Service obligation, as time permits, fax a copy of the VDI to ERCOT Compliance at 225-7165. |
| 1 | REVIEW the QSEs real time performance summary display periodically or when ERCOT ACE deviation is significant. Reference Displays: “ERCOT Responsive, Regulation, and SCE” display (POS) |
| 2 | CONSULT with the System Operator on the Frequency Control Desk when abnormal real time performance by one or more QSEs is detected. |
| 3 | VERIFY if the System Operator on the Frequency Control Desk has established communication with the QSE or QSEs and the cause for the under performance is known. |
| 4 | VERIFY that the System Operator Transmission & Security desk is informed of the situation. |
| 5 | ALLOW the System Operator Transmission and Security desk and System Operator on the Frequency Control Desk to RESOLVE the problem and RESTORE ERCOT ACE performance to normal. |
| 6 | If a QSE or QSEs continue to under perform in real time, and ERCOT’s NERC performance requirements are not being met, THEN ASSIGN a System Operator to collect performance data on the QSE or QSEs affecting ERCOT NERC performance. |

A copy of the procedure is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.