# ERCOT Operations Bulletin 51

ERCOT has revised the Day Ahead Desk section of the Operating Procedure Manual.

The change is located in section 2.10 on pages 17 – 18.

### V2R9

| 9 | IF there are no shortages, GO TO Step 13, otherwise continue. |
| --- | --- |
| 10 | IF there are shortages, then click notify. This sends an electronic message to deficit QSE(s) detailing the hours, amounts, and services in which the resource plan is deficit. Then telephone the QSE(s) with shortage(s) and request resolution. If necessary, repeat steps 1-8 by about 1700. |
| 11 | a. If Ancillary Service shortages are <10 MW, ERCOT will assume the discrepancy is due to rounding error, NOTE the discrepancy in the logs and go on to step 12. b. If Ancillary Service shortages are >10 MW but <100 MW, then EVALUATE if a real shortage exists or, if a QSE has failed to submit a valid resource plan. Log the results and go to step 12. c. If Ancillary Service shortages are over 100 MW at the 1700 validation, then evaluate, log results, then REQUEST that the Transmission & Security Operator ISSUE an OCN worded as follows: - Post on MIS: “The resource plans for some Ancillary Service providers do not support their Ancillary Service obligations for <DATE> at this time. Please review and correct your resource plans as soon as possible.” - Post on ENS: “OCN-AS short-<mm/dd/yy> see MIS” |
| 12 | INFORM the Shift Supervisor. |
| 13 | CLICK the “Notify” button. |

### V2R10

| 9 | IF there are no shortages, GO TO Step 12, otherwise continue. |
| --- | --- |
| 10 | IF there are shortages, then click notify. This sends an electronic message to deficit QSE(s) detailing the hours, amounts, and services in which the resource plan is deficit. Then telephone the QSE(s) with shortage(s) and request resolution. If necessary, repeat steps 1-8 by about 1700. |
| 11 | a. If Ancillary Service shortages are <10 MW, ERCOT will assume the discrepancy is due to rounding error, NOTE the discrepancy in the logs and go on to step 12. b. If Ancillary Service shortages are >10 MW but <100 MW, then EVALUATE if a real shortage exists or, if a QSE has failed to submit a valid resource plan. Log the results and go to step 12. c. If Ancillary Service shortages are over 100 MW at the 1700 validation, then evaluate, log results, then REQUEST that the Transmission & Security Operator ISSUE an OCN worded as follows: - Post on MIS: “The resource plans for some Ancillary Service providers do not support their Ancillary Service obligations for <DATE> at this time. Please review and correct your resource plans as soon as possible.” - Post on ENS: “OCN-AS short-<mm/dd/yy> see MIS” - INFORM the Shift Supervisor. |
| 12 | CLICK the “Notify” button. |

- A copy of the Day Ahead Desk V2R10 is attached.