# ERCOT Operations Bulletin 223

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

V3R61		Incorporate CN#283 into procedure as a Note preceding step 1

| ### 2.4.1 Respond to Voltage Issues |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| NOTE: | When a VDI is given requesting voltage support from a QSE, the VDI will direct the QSE to maintain a specified voltage set point. The VDI shall not attempt to quantify the change in generator MW output necessary to maintain the voltage set point. |
| 1 | When indicated, by SCADA alarms or when notified by a Transmission Operator of voltage concerns, TAKE appropriate action. |
| 2 | Use displays that contain voltages that are of concern. |
| 3 | INITIATE a solution in collaboration with appropriate Transmission Operator and/or QSEs. |

V3R62		Update Appendix 2

## 3.2	Appendix 2:  Contingency Analysis Using a Powerflow Base Case – Typical

The following steps are recommended for performing a contingency analysis:

Basic process for using a RTNET (SE) solved case in STNET and STCA

- In STNET, select Stop Process

- Go to Data Retrieval, select “Savecase Directory”

- Go to Case Type DPFOPTS, retrieve the file SOLVE_RTNET_SNAPSHOT

- Go to Case Type PFOPTS, retrieve the file SOLVE_RTNET_SNAPSHOT

- Go to Case Type CTGS, retrieve the latest CTG_PRODUCTION_MMDDYY_XXX.

- Go to Case Type SPSMOM, retrieve the latest SPS_PRODUCTION_MMDDYY_XXX.

- Once all of these savecases have been retrieved, click on the Done button on the Data Retrieval

pop-up menu.

- The above steps do not need to be repeated as long as no other studies are loaded into this clone.  You can do the following set of steps again and again throughout the day

- In STNET, start the process

- Go to Data Retrieval again, select Network Copy

- In Copy Options, verify that the “Copy Clone of Database” field is set to NETMOM, “from Application” field is set to RTNET, and “of Family” field is set to EMS (these fields will only have to be entered once).  If not, change the fields accordingly.

- Select COPY (This copies the SE Network Model directly into STNET)

- Click the Done button.

- Close the Data Retrieval pop up menu by clicking the Done button.

- When the “Solution State:” message says, “Completed Database Copy,” click on

“Run Solution Sequence” to solve the power flow.

A copy of the procedure is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.