# Power Operations Bulletin #118

ERCOT has revised the DC Ties section of the Operating Procedure Manual.

The updated is located in section 3.3.3.

### ERCOT Post-Approval Process for NERC Tags

- ERCOT monitors the OATI tagging system and when the NERC tag changes to IMPLEMENT status, the ERCOT System Operator prints the tag and attaches it to the faxed schedule.

- The schedule is then manually entered into the DC Tie Inadvertent Excel Spreadsheet under the appropriate QSE/Sub-QSE responsible for the schedule.  The ERCOT System Operator verifies the totals match with the MOI schedules, and the fax sheet from the QSE matches with the current tag(s) printed out from the OATI

- The following duties are performed hourly each day:

- The schedules for the current day in the MOI for each DC Tie are totaled and compared to the totals in the DC Tie spreadsheet.  This ensures that the schedules in the MOI agree with what is actually being scheduled across the ties.  If there is a discrepancy, then the ERCOT System Operator will investigate the discrepancy and contact the appropriate parties, if applicable, to determine and resolve the problem.

- The ERCOT System Operator calls the AEP DC Tie Operator for hourly actual flows on the DC Ties and enters them into the DC Tie Inadvertent spreadsheet.

- The ERCOT System Operator checks the status of the actual flows on the “Generation Tie Line Status” screen.  The ERCOT System Operator then compares the actual flows for the current hour to the scheduled flows for the current hour.  If there is a discrepancy, then the ERCOT System Operator will check with the AEP DC Tie Scheduler to determine and resolve the problem.

- The ERCOT System Operator calls the AEP DC Tie Operator periodically throughout the day for scheduled hourly flows on the DC Ties and enters them into the DC Tie Inadvertent spreadsheet.

- After midnight, the total flows across the DC Ties are compared with the AEP DC Tie Operator.

- The ERCOT System Operator prints the DC Tie Total tab from the DC Tie Inadvertent Spreadsheet and faxes it to the Shift Supervisor for entry into the daily log folder.

- The tags and fax sheets are grouped by QSE, stapled together with a copy of the totals tab from the DC Tie Inadvertent spreadsheet and then placed into the monthly DC Tie folder.

- A copy of the procedure, DC Ties V1 R5, is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.