# ERCOT Operations Bulletin 224

ERCOT has revised the Transmission & Security Desk section of the Operating Procedure Manual.

- Changes include adding new Note after step 6 in section 2.1.2.

| ### RTCA Congestion Management |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 6 | IF the SCADA is of similar magnitude to the pre-contingency value and no SPS, RAP, PCAP, TOAP, or MP exist, go to step 8. |
| NOTE: | When making any changes to the SCADA database, document the changes using the SCADA tagging function. Refer to Addendum 1 for guidance. |

- Adding a Note to Appendix 3

## Appendix 3:  Conduct Future Security Analysis

The following steps are recommended for performing a Contingency Analysis.

NOTE:  When making any changes to the SCADA database, document the changes using the SCADA tagging function.  Refer to Addendum 1 for guidance.

- Adding a new Addendum

## 3.10	Addendum 1:  SCADA Tagging Maintenance, Creating SCADA Tags

SCADA Tagging Maintenance

Creating SCADA Tags

Any time a manual entry is made into the SCADA database, either a status change or an analog change, a SCADA Tag must be entered into the system as record of that manual override.  The SCADA Tag provides all users an explanation of why a measurement has been overridden in the database.  The process for creating a SCADA Tag is as follows:

Status points

- Click on the device on the SCADA oneline to bring up the SCADA_SWITCH_POPUP message box.

- Click on the button labeled Tag…

The picture below shows the pop-up for disconnect 6974 at PUTN station.

- After clicking on the Tag… button, another message box will appear, TG_POINT_POPUP.  Click on the INFO button.

- Another message box will appear.  This is the SCADA Tag itself.  Fill out the SCADA Tag accordingly:

  - Order # = Operator’s EMS Clone Name (TRANSMSN, SUPV, etc)

  - Created By = Operator’s first initial, last name (CTHOMPSON, JGAYTAN, etc)

  - Created For = may be left blank

  - Comments = add all necessary comments related to why the manual change has been made

- When finished, click on the Process List button.

- If at any time, you wish to cancel the action, click on the Cancel Tag Creation button.  Closing the window will still leave the SCADA Tag marked.

Below is an example of a completed SCADA Tag.  Note that this is a STATE ESTIMATOR Tag, which is orange.  INFO tags are white.

Analog Points

- From the Substation Tabular display, scroll to the measurement in question, and click on the button to the right of that analog measurement that looks like a baggage tag:

- After clicking on the Tag button, another message box will appear, TG_ANALOG_POPUP.  Click on the INFO button.

- Another message box will appear.  This is the SCADA Tag itself.  Fill out the SCADA Tag accordingly:

  - Order # = Operator’s EMS Clone Name (TRANSMSN, SUPV, etc)

  - Created By = Operator’s first initial, last name (CTHOMPSON, JGAYTAN, etc)

  - Created For = may be left blank

  - Comments = add all necessary comments related to why the manual change has been made

- When finished, click on the Process List button.

- If at any time, you wish to cancel the action, click on the Cancel Tag Creation button.  Closing the window will still leave the SCADA Tag marked.

Editing an Existing SCADA Tag

- From the SCADA Tabular display, click on the Tag symbol (looks like a baggage tag).  On the pop-up menu, click on the View… button.  See below.

- This will bring up the TG_POINT_LIST window, shown below.  To edit the SCADA Tag, click on the button under the “Details” column.

- An adjusted version of the SCADA Tag will show up.  Make changes to the SCADA Tag as necessary, making sure to fill out the following fields:

- Modified By = Operator’s EMS Clone Name (TRANSMSN, SUPV, etc)

- Modified for = may be left blank

- Comments = add all necessary comments related to why the manual change has been made

- When finished, click on the Done button.

The completed, updated, SCADA Tag should look as follows:

Removing an existing SCADA Tag

- From the SCADA Tabular display, click on the Tag symbol (looks like a baggage tag).  On the pop-up menu, click on the Remove button.  See below.

- This will bring up the TG_POINT_LIST window, shown below.  To edit the SCADA Tag, click on the button under the “Details” column.

- An adjusted version of the SCADA Tag will show up. (see below)  Make changes to the SCADA Tag as necessary, making sure to fill out the following fields:

- Removed By = Operator’s EMS Clone Name (TRANSMSN, SUPV, etc)

- Removed for = may be left blank

- Comments = add all necessary comments related to why the SCADA Tag is to be removed.

- When finished, click on the Process List button.  DO NOT click on the Remove button.

A copy of the procedure is available at http://www.ercot.com/AboutERCOT/PublicDisclosure/ERCOTProcedures.htm.

The SCADA Tag to be removed should look as follows: