# ERCOT Operations Bulletin 53

ERCOT has revised the Day Ahead Desk section of the Operating Procedure Manual.

The change is located in section 2.11 on page 19.

### V2R11

| ## Initiate ERCOT Day Ahead Replacement Capacity Market |  |
| --- | --- |
| ###### Step # | ###### Procedural Steps |
| 1 | - Before 0600, INITIATE the “Studies” display with tomorrow’s date and intervals to be studied. - SELECT the “Execute SPD”. - SELECT “Replacement” in the “Study Mode” area. - SELECT the “Savecase” model to be used for the study. - SAVE the study with the appropriate name. - SELECT “PUBLISH” - At the prompt for market type, SELECT “Day Ahead” - On the Day Ahead Market Management Display, CONFIRM a Replacement Market was created for tomorrow for hours 0100 to 2400. |
| NOTE: | The naming convention for Replacement Capacity studies is “RCVmmddyyflr”, where: mmddyy = the month, day and year for which the study is conducted, fl = the first and last initial of the person conducting the study, r = the revision number of the study (no number for the original study, 1 for revision 1, etc.). |
| 2 | After Replacement Plan is validated - Deactivate all active constraints for the next day. - RUN a replacement analysis by clicking the “EXECUTE” button. |
| 3 | OOMC Units for local congestion. - Check load forecast by weather zone and refer to the Generation Capacity Procurement Recommendation Book for suggested generators to serve loads at various load levels and the time of day loads are expected to be at those levels. - Check RMR unit requirements. - Refer to Outage Coordinator Notes for unit requirements due to outages. - Confer with the Transmission and Security Operator as necessary on current constraint problems. If the constraints are expected to recur, discuss obtaining generators that would help manage these constraints. Notify the Shift Supervisor. - Review Transmission Operator security requirements as received. Request confirmation of the requirements by Operations Engineering if something new or a significant change is received. |

### V2R12

## Initiate ERCOT Day Ahead Replacement Capacity Market

###### Procedural Steps

- Before 0600, INITIATE the “Studies” display with tomorrow’s date and intervals to be studied. - SELECT the “Execute SPD”. - SELECT “Replacement” in the “Study Mode” area. - SELECT the “Savecase” model to be used for the study. - SAVE the study with the appropriate name. - SELECT “PUBLISH” - At the prompt for market type, SELECT “Day Ahead” - On the Day Ahead Market Management Display, CONFIRM a Replacement Market was created for tomorrow for hours 0100 to 2400.

The naming convention for Replacement Capacity studies is “RCVmmddyyflr”, where: mmddyy = the month, day and year for which the study is conducted, fl = the first and last initial of the person conducting the study, r = the revision number of the study (no number for the original study, 1 for revision 1, etc.).

After Replacement Plan is validated - Deactivate all active constraints for the next day. - RUN a replacement analysis by clicking the “EXECUTE” button.

OOMC Units for local congestion. - Check load forecast by weather zone and refer to the Generation Capacity Procurement Recommendation Book for suggested generators to serve loads at various load levels and the time of day loads are expected to be at those levels. - Check RMR unit requirements. - OOMC units needed to solve local congestion. For units that require lead times greater than allowed by the Resource Plan in the Day-Ahead process, issue a VDI for the unit(s) needed to be online at a time and date needed to solve local congestion. Refer to Outage Coordinator Notes for unit requirements due to outages. - Confer with the Transmission and Security Operator as necessary on current constraint problems. If the constraints are expected to recur, discuss obtaining generators that would help manage these constraints. Notify the Shift Supervisor. - Review Transmission Operator security requirements as received. Request confirmation of the requirements by Operations Engineering if something new or a significant change is received.

- A copy of the Day Ahead Desk V2R12 is attached.