# ERCOT Operations Bulletin 56

ERCOT has revised the Frequency Control Desk section of the Operating Procedure Manual.

This change is a temporary change and expires on July 31, 2004.

# TC # 86 Replace RRS Through A Bilateral Trade

## Expiration Date:  July 31, 2004

When a Resource (either LAAR or GEN) that is supplying RRS becomes unavailable to a QSE (QSE–A) and needs to be replaced through a Bilateral Trade with another QSE (QSE–B), then, QSE-A is expected to inform ERCOT.

- Obtain the following information from QSE-A:  Intervals affected and the name of the QSE with whom the bilateral trade was made.

- Log this converstation and the information provided and forward the information by email to Mark Henry (Compliance) and Mark Patterson (Market Operations Support).

- IF the resource that became unavailable was a LAAR, then:

- Ensure that QSE-B maintains an amount of Responsive Reserve Service equal to their own obligations plus the amount of the bilateral agreement with QSE-A for the affected intervals.

Background information:  (not to be included in the TC)

In this situation, a QSE providing RRS “loses” a LAAR and it cannot be substituted for with another LAAR in its own portfolio.

Now, two barriers become important.

First, The EMMS does not allow the QSE carrying the RRS obligation to replaceUFR with GEN.

Second, scheduling the GEN without first replacing the UFR with GEN as noted above would result in a schedule in excess of the QSE’s obligations.  This is not allowed by the EMMS either.

Therefore, when they purchase the necessary RRS from another QSE, the bilateral transaction cannot be scheduled.  Subsequently, the purchased RRS cannot be seen by ERCOT, and the QSE originally responsible for the RRS will continue to show a shortage of RRS.

ERCOT needs to know whether the QSEs are meeting their obligations, so, this manual workaround is necessary for QSE performance monitoring.

A system change is planned for about May 2004 that will allow this temporary change to be terminated.